3GPP TSG-RAN WG2#85bis	R2-141602

31 March – 4 April 2014, Valencia, Spain.

Agenda Item:	 7.2

Source: 	Motorola Mobility

Title:  	Small Cell Enhancements – Issues to consider

Document for:	Discussion and Decision

# Introduction

A work item to define on/off procedures for (small) cells was agreed at RAN#62 [1]. This work item seeks to enable switching an eNBs state from on to off (and vice versa) to conserve energy and reduce interference. RAN1 is having detailed discussions regarding the feature and in particular the eNB transmissions during on vs. off state.

An LS was received from RAN1[2] describing the various agreements that RAN1 has made on this topic. This document gives a brief overview of the areas of potential impact to RAN2 specifications and procedures. It also provides our view on the next steps needed.

# Discussion

The following is a summary of the current status in RAN1 of the small cell on/off design [2]:

- A Discovery reference signal (DRS) will be introduced. However, DRS will not be a new signal, it will be some combination of PSS/SSS, CRS, CSI-RS and PRS.

- RSRP and RSRQ measurements based on DRS will be supported. The intention is to achieve measurement performance similar to legacy RSRP and RSRQ.

- DRS will be transmitted when cell is off. It is FFS whether DRS will be transmitted when cell is on.

RAN1 has indicated that there may be potential impact to RAN2 specifications to support the use of DRS based measurements for handover.

## DRS based measurements for Mobility

Handover of a UE is based on identifying a cell that is better than the current cell. Included in the measurements procedure related to handover is a comparison of measurement of two cells. For example the A3 event compares the difference in measurements of two cells to a “delta threshold”. A new cell with a measurement that is better than the current cell by at least the delta threshold is considered to be a candidate for a potential handover.

Design of the DRS is in progress in RAN1 and there are still some significant unresolved questions. It is agreed that there will be RSRP and RSRQ measurements based on DRS. However, it is not clear whether DRS based measurements can be directly compared to the legacy CRS based measurements. For example, if a cell is off and UE performs DRS based measurements of the cell that is off, in order to determine whether to turn on the cell and perform a handover from a legacy cell, it is necessary to be able to be able to perform comparison of the DRS based and CRS based measurements. Another case is where there are two cells that the UE measures, one of which is in an off state and the other is an on state. A comparison of the measurements of the two cells is necessary to be able to determine the correct cell to handover to. In other words, the measurement quantity for the DRS based measurements has to be the same as (or equivalent to) the CRS based measurements if the two types of measurements have to be comparable.

It is of course possible to perform handovers without performing such comparisons (e.g., inter-RAT handovers always involve different measurement quantities). Such handovers are based on absolute thresholds rather than delta thresholds (for example if the measurement of a new cell is better than an absolute threshold and the measurement of the serving cell is lower than another absolute threshold, the new cell could be considered a candidate for a potential handover). However, selecting appropriate values for absolute thresholds is operationally challenging, and for handovers within a RAT, comparing measurements of the same quantity is more efficient.

The question of whether the measurement quantities for the DRS based measurements will be equivalent to (and comparable to) legacy CRS based measurements needs to be discussed and concluded in RAN1 first. Without this first step, it is difficult to see how RAN2 can make any progress. Depending on RAN1’s input, RAN2 needs to discuss whether new measurement events or measurement procedures are needed to support small cell on/off. If new measurement quantities are to be introduced, new measurement event definitions with corresponding signalling of parameters will be needed. If existing measurement quantities are to be used, we may be able to reuse existing measurement events with minor modifications and signal some additional parameters.

If new measurement quantities are to be introduced, RAN4’s involvement is needed (probably extensively). Even if existing measurement quantities are used, RAN4’s involvement may be needed to define performance requirements for the DRS based measurements. For example, if DRS consists of sparse CRS transmission, RAN4 may need to define UE requirements for measurement performance under different CRS densities. In our view, designing DRS as a sparse transmission of CRS makes the overall feature design simpler. However, this needs to be discussed in RAN1.

Proposal 1: RAN2 should await information from RAN1 regarding the details of DRS and whether DRS based measurements can be compared to CRS based measurements.

# Summary

We have discussed the status of the work for Small Cell Enhancements and the potential impacts to RAN2 specifications. The primary impact is expected to be in defining measurements to support mobility in the presence of cells that support on/off.  Based on the discussion, we propose:

Proposal 1: RAN2 should await information from RAN1 regarding the details of DRS and whether DRS based measurements can be compared to CRS based measurements.

# References

- RP-132073, “Small Cell Enhancements – Physical Layer aspects”

- R2-140965, “LS on small cell on/off and discovery”, RAN1