3GPP TSG-RAN WG2 #91bis	Tdoc R2-154929

revised R2-154744

Malmö, Sweden, 5h – 9th October 2015

Agenda Item:	07.10.2

Source:	Ericsson (Rapporteur)

Title:	TR Text Proposal capturing outcome of [91#29][LTE/Latency] and [91bis#04][LTE/LATRED]

Document for:	Discussion, Decision

# Introduction

In this document a text proposal is presented aiming to include evaluation results including a preliminary RAN2 conclusion text for the protocol evaluation part of the SI: Study on Latency reduction techniques for LTE. The text proposal is based on the results from the e-mail discussion ‘[91#29][LTE/Latency] Evaluation results’ and [91bis#04][LTE/LATRED], and is proposed to be included in the SI TR 36.881 v0.3.0 in its next version pending discussion and agreement in RAN2.

Note: In this revision, editorial only changes for TP implementation in 36.881 v0.3.1 is not captured (section 9.1.x).

Changes compared to R2-154744:

- Removal of Simulation 8

- Removal of Simulation 8 related Annex A1.8

- Correction of values in Table 4, Average delay to next SR opportunity

- Addition of missing simulation assumptions in 9.1.x2

- New conclusion text based on e-mail discussion outcome

--- BEGIN Text Proposal ---

-- For inclusion in section 2, References --

[6]	R2-154743, Email discussion report for [91#29][LTE/Latency] Evaluation results, RAN2#91, August 2015

-- For inclusion in 36.881, Section 9.1 --

### 9.1.x1	Simulation x1

Latency evaluation results for TTI reduction and Fast UL [6]

#### 9.1.x1.1	Simulation assumptions

The performance of a single FTP download over TCP for file sizes between 100 kB and 5 MB is evaluated; in addition to a radio network delay a fixed delay of 10 ms from core, transport and Internet is assumed.

For the LTE radio part a system with a 10 TTI PUCCH cycle, an 8 TTI SR to grant period, and an 8 TTI HARQ round trip time (allowing for 8 HARQ processes) is assumed. For the LTE baseline, a TTI length of 1ms (14 OFDM/SC-FDMA symbols) is assumed.

With the technique of TTI shortening the TTI length is scaled to 7, 2, and 1 symbol. The TTI scaling in this evaluation also directly scales the HARQ RTT, so that a proportional scaling of the transport block size is achieved. For simplicity, it is further assume that a reduction of the TTI length also scales the SR to grant period, as well as the PUCCH cycle. For the shorter TTIs it is assumed that an additional DL and uplink overhead due to increased signalling applies, see Table 1. For the SR period one standard case (10 TTI) and one short case (1 TTI) is studied for comparison.

With a faster first UL grant, the UE is assumed to not perform a legacy PUCCH cycle nor SR to grant period, i.e. the grant acquisition delay is assumed to be 0ms.

In Table 1 below we summarize the simulation parameter setup used in this evaluation.

Table x1-1. TCP download simulation setup

| Simulation scenario | Static UE in a cell |
| --- | --- |
| DL and UL throughput | DL 150 Mb/s, UL 50 Mb/s |
| Number of OFDM/SC-FDMA symbols per TTI, same in both DL and UL | 1, 2, 7, 14 corresponding to - HARQ RTT: of 0.57ms, 1.14 ms, 4 ms, 8 ms - SR to grant period: 0.57ms, 1.14, 4 ms, 8 ms - PUCCH cycle: 0.7ms, 1.4 ms, 5 ms, 10 ms |
| HARQ error probability | 0.3 for 1st transmission, 0.1 for 2nd, 0.05 for 3rd, 0.01 for 4th, … |
| Traffic model | FTP download over TCP (initial window: 10segments) |
| Core, transport and internet network delay | 10ms |
| FTP file size | [100 kB, 500 kB, 1 MB, 5 MB] |
| SR period | [1 TTI, 10 TTI] |
| DL PHY overhead | 7%, 7%, 25%, 25% for TTI length of 14, 7, 2, 1 symbols respectively |
| UL PHY overhead | 14%, 14%, 33%, 50% for TTI length of 14, 7, 2, 1 symbols respectively |

#### 9.1.x1.2	TTI shortening simulations

Figure x1-1: Download speed and delay, as well as relative gains in speed and delay compared to baseline for different file sizes and latency reduction techniques in LTE. The baseline is 1 ms TTI and SR. SR period is 10 TTI.

Figure x1-2: Download speed and delay, as well as relative gains in speed and delay compared to baseline for different file sizes and latency reduction techniques in LTE. The baseline is 1 ms TTI and SR. SR period is 1 TTI.

From the results in Figure 1, for a SR period of 10 TTI, it is clear that significant gains in download delay and FTP object bit rate can be achieved at smaller FTP file sizes with the two studied latency reduction techniques. Generally, the smaller the FTP file size is (for a given link level throughput), the larger the fraction of the TCP slow start phase on the overall download time. During the TCP slow start phase the maximum achievable throughput is not fully utilized, instead the performance depends on the RTT, i.e. directly on the latency.

The same trend can be seen in Figure 2, for a SR period of 1 TTI, but compared to the 10 TTI case the baseline is somewhat improved, and therefore the gains with shorter TTI and Fast UL are slightly reduced.

It becomes evident that a high gain in terms of overall download time reduction can be achieved with a faster first UL grant (green line); compared to the baseline the download time is reduced by 20% for smaller file sizes. A similar and somewhat higher reduction can be achieved with a 7 symbol TTI (0.5ms), i.e. 25% reduction (red line). It is noteworthy that these techniques can complement each other. With a combined use, reductions of about 35% can be achieved for the download time (cyan line).

Further improvements are achievable with further reduction of the TTI length. In the case of 2 symbol TTI (1/7ms) and with the use of fast UL grant as well (orange line), the total download time reduces by 50% at the smaller file sizes. For a 0.5 MB FTP download this corresponds to a reduction from a 0.35 s to 0.18 s download time. Reducing even further to 1 symbol TTI (1/14ms) provides only a minor additional benefit compared to the 2 symbol TTI. This is due to the assumed comparably large transport/internet/core delay of 10ms compared to the reducible RAN delay.

For file sizes below 5MB an improvement in performance can be achieved from shortening the TTI length further than 1/2ms, while at 5MB file size a TTI length of 1/2ms gives the best performance, due to the impact of higher DL overhead for TTI reductions beyond 1/2ms.

Reducing TTI length down to 1 or 2 symbols gives largest latency reduction for file sizes up to 1MB, and for larger file sizes a TTI length of 7 gives best latency performance.

Adding Fast UL gives an improved performance with both legacy and short TTI length in all studied cases.

The gain in both download speed and time is larger for smaller file sizes with all TTI lengths compared to the 5MB file size. Further, for the small file sizes, these gains are largest with 1 symbol TTI. These observations are due to the proportional length of TCP slow start compared to the total file download time.

#### 9.1.x1.3	Fast UL system simulations

The Fast UL grant mechanism was simulated in a system level simulator tool with FTP traffic and its performance was compared to that of SR grants and SPS grants. The simulator parameters are given in Table 2. Figure 3 shows the user FTP throughput for the different grant mechanisms. As can be seen in the figure, a gain of 30-40% can be achieved with fast UL grants compared to SR grants with a period of 10 TTI, and gains of ~20% compared to SR grants with a period of 1 TTI. The FTP throughput is similar to that of SPS grants.

The resource utilization is given in Figures 4 for DL and in Figure 5 for UL.

Table x1-2. Simulation parameters

| Network | 7 sites with 3-sector cells, 500m ISD |
| --- | --- |
| Bandwidth | 10 MHz in all cells |
| Duplex | FDD |
| Traffic model | Single FTP file download |
| File size | 1 MB |
| TCP initial window size | 10 |
| SR period | [1 TTI, 10 TTI] |
| UE arrival rate | Poisson Distribution, [2.82, 6.58, 9.3975, 12.22, 15.04] per second |
| Radio clock power consumption | 10mW |
| RF RX mixer and LNA power consumption | 72mW |
| RF RX filter and ADC power consumption per MHz | 14mW/MHz |
| RX baseband power consumption (mW) | 25 + 7 x MbpsRX |
| TX mixer power consumption | 80mW |
| TX DAC power consumption per Mhz | 16mW/MHz |
| TX baseband power consumption (mW) | 11 + 1 x MbpsTX |
| Power consumption of PA (mW) | 72 + 17.5 PTX0.784 |

Figure x1-3. FTP Download Rate for fast UL grant (magenta) compared to SR (blue and black) and SPS (red)

In Figure 6 the energy consumption of the different grant mechanisms is shown. Here, we can see that the Fast UL grant solution exhibits the lowest energy consumption among the studied cases, about 10% lower than for SR grants. Compared to SPS, no padding is sent unless there is UL data, which saves battery. It should be noted that no DRX functionality has been assumed in these simulation. If DRX could be applied in the UE during the “inactive” phase with fast UL grant, additional reduction in power consumption can be expected.

Figure x1-4: DL Resource Utilization for fast UL grant (magenta) compared to SR (blue and black) and SPS (red).

Figure x1-5: UL Resource Utilization for fast UL grant (magenta) compared to SR (blue and black) and SPS (red).

Figure x1-6. Energy consumption for fast UL grant (magenta) compared to SR (blue and black) and SPS (red).

Fast UL reduces the energy consumption compared to SR and SPS.

### 9.1.x2	Simulation x2

System Performance with TTI shortening [6]

#### 9.1.x2.1	General information

For the service transmission, the whole latency includes backhaul latency (the transmission delay between eNB and application server) and RAN latency. Although the SI is only intended to shorten RAN latency, when we analysis the performance of RAN latency reduction, the impact of backhaul latency should not be neglected, and backhaul delay should be considered as an evaluation factor.

For FTP download application, since the improvement on the file download time brought by latency reduction would mainly on the TCP slow start phase, the improvement would be different and depend on the proportion of TCP slow start phase in the whole file download time. Hence, the downloaded file size should be considered as an evaluation factor.

Besides the file size, the throughput in Uu interface provided to UE (i.e. MCS, PRBs) also brings the impact on the benefit brought in the TCP slow start phase. Hence, the Uu throughput should also be considered as an evaluation factor.

As discussed in meeting, we should also take the L1 control and RS overhead into account.

For RAN latency, it includes data transmission time (i.e. TTI), RTT, and UL access latency (in UL direction, means the period from UL data arrival in UE to data transmission based on the receiving UL grant). Currently, TTI is 1ms; RTT is 8TTI (i.e. 8ms for FDD). UL access latency is about 14.5ms (in D-SR case).  To reduce the latency further, for TTI, the smaller granularity should be evaluated, 1 OFDM symbol, 3 OFDM symbol and 7 OFDM symbols.

For extra UL latency, two existing solutions, i.e. Short D-SR and pre-allocation, with shorter UL latency are evaluated.

#### 9.1.x2.2	Simulation assumptions and parameters

The evaluation assumptions are given as below:

1	FTP download application is used in evaluation, and Downloading Response Time (DRT, sec) is regarded as the metric of performance evaluation. Wherein, DRT includes the time period from UE requesting FTP service to finishing downloading successfully;

2	Evaluation is based on single UE system with specific block error rate at air interface (i.e. 10% initial BLER);

3	Simulation parameters in Uu interface are given in table below.

-	For backward compatibility, we assume at least half of bandwidth should be reserved for legacy UEs;

-	To evaluate the impact on various backhaul latency, FTP file size and UL access, we considers the sufficient resource allocation with 50 PRBs and assumes the good channel quality with MCS 20;

-	To evaluate the impact on the various Uu throughput, we also consider the limited resource allocation with 10 PRBs and assume middle channel quality with MCS 12.

4	L1 overhead (i.e. L1 control channel and RS) is considered as 10%, 20%, 30% and 50%, which is reflected into the TBS calculation. To unify the TBS calculation, we also use the same TBS calculation for the base line case (i.e. 1ms TTI), and the 20% L1 overhead is assumed. The detail on TBS calculation is given in Annex <A1.x2>.

5	L2 overhead is not taken into account.

Table x2-0: Simulation parameters

| Parameters | Assumptions |
| --- | --- |
| APP Server <--> eNB delay (ms) | 0, 5, 10, 20, 30, 50 |
| FTP File Size | 10KByte, 100KByte, 1MByte, 2MByte, 5MByte |
| Uu throughput (Mbps) | 27.907Mbps (MCS=20, PRB_Num=50) 2.848 (MCS=12, PRB_Num=10) |
| TTI Duration | 1ms (as baseline) |
| Available PRB number for LR UE | 50 (for 20MHz BW) |
| Target Packet Error Rate | 0.1 |
| Max Transmission times | 4 |
| HARQ RTT | 8*TTI |

#### 9.1.x2.3	Performance results

##### 9.1.x2.3.1	Various backhaul latency

Assuming FTP file size 100KByte, Table 1 and Figure 1 give the improvement of each TTI length for various backhaul latency, with taking various L1 overhead (L1_OH) into account.

Table x2-1: backhaul latency

| TTI length (in OFDM symbol number) | L1_OH |  | AppServer <-> eNB latency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ##### 0ms | ##### 5ms | ##### 10ms | ##### 20ms | ##### 30ms | ##### 50ms |
| 14 symbol (baseline) | 20% (Baseline) | DRT | 0.183 | 0.263 | 0.342 | 0.500 | 0.660 | 0.977 |
| 7 symbol | 10% | DRT | 0.097 | 0.171 | 0.251 | 0.410 | 0.571 | 0.891 |
|  |  | ##### Gain | ##### 46.99% | ##### 34.98% | ##### 26.61% | ##### 18.00% | ##### 13.48% | ##### 8.80% |
|  | ##### 20% | DRT | 0.102 | 0.176 | 0.255 | 0.413 | 0.574 | 0.894 |
|  |  | ##### Gain | 44.26% | 33.08% | 25.44% | 17.40% | 13.03% | 8.50% |
|  | ##### 30% | DRT | 0.105 | 0.177 | 0.256 | 0.414 | 0.574 | 0.895 |
|  |  | ##### Gain | ##### 42.62% | ##### 32.70% | ##### 25.15% | ##### 17.20% | 13.03% | 8.39% |
|  | ##### 50% | DRT | 0.120 | 0.186 | 0.258 | 0.416 | 0.577 | 0.896 |
|  |  | ##### Gain | ##### 34.43% | ##### 29.28% | ##### 24.56% | ##### 16.80% | ##### 12.58% | ##### 8.29% |
| 3 symbol | 10% | DRT | 0.058 | 0.124 | 0.203 | 0.363 | 0.523 | 0.842 |
|  |  | ##### Gain | 68.31% | 52.85% | 40.64% | 27.40% | 20.76% | 13.82% |
|  | ##### 20% | DRT | 0.062 | 0.127 | 0.203 | 0.363 | 0.524 | 0.843 |
|  |  | ##### Gain | ##### 66.12% | ##### 51.71% | ##### 40.64% | ##### 27.40% | 20.61% | 13.72% |
|  | ##### 30% | DRT | 0.067 | 0.130 | 0.203 | 0.364 | 0.524 | 0.843 |
|  |  | ##### Gain | ##### 63.39% | ##### 50.57% | ##### 40.64% | ##### 27.20% | ##### 20.61% | ##### 13.72% |
|  | ##### 50% | DRT | 0.085 | 0.143 | 0.213 | 0.368 | 0.529 | 0.848 |
|  |  | ##### Gain | ##### 53.55% | ##### 45.63% | ##### 37.72% | ##### 26.40% | ##### 19.85% | ##### 13.20% |
| 1 symbol | 10% | DRT | 0.042 | 0.103 | 0.179 | 0.339 | 0.498 | 0.818 |
|  |  | ##### Gain | ##### 77.05% | ##### 60.84% | 47.66% | 32.20% | 24.55% | 16.27% |
|  | ##### 20% | DRT | 0.047 | 0.106 | 0.179 | 0.340 | 0.499 | 0.819 |
|  |  | ##### Gain | ##### 74.32% | ##### 59.70% | 47.66% | 32.00% | 24.39% | 16.17% |
|  | ##### 30% | DRT | 0.052 | 0.110 | 0.180 | 0.340 | 0.500 | 0.820 |
|  |  | ##### Gain | ##### 71.58% | ##### 58.17% | ##### 47.37% | ##### 32.00% | 24.24% | 16.07% |
|  | ##### 50% | DRT | 0.071 | 0.124 | 0.192 | 0.344 | 0.503 | 0.823 |
|  |  | ##### Gain | ##### 61.20% | ##### 52.85% | ##### 43.86% | ##### 31.20% | ##### 23.79% | ##### 15.76% |
|  | Note: 1) DRT (sec) is the statistic metric, means downloading response time; 2) The gain is calculated by (1-DRT/Baseline_DRT), where, >0 means improvement, <0 means performance loss; |  |  |  |  |  |  |  |

TTI = 7 OFDM Symbols			                                         TTI = 3 OFDM Symbols

TTI = 1 OFDM Symbol

Figure 1:	Gain for various backhaul latency

From Table 1, it can be seen that with the backhaul delay increasing the DRT difference of different TTI length is decreasing, and in case of backhaul delay 50ms, we can regard there is no much difference of different TTI length. And we think the reason is that TTI shortening only brings the improvement on Uu transmission, and the benefit is negligible compare to the long backhaul delay.

The longer the backhaul latency, the less the gain on DRT brought by shorter TTI.

##### 9.1.x2.3.2	Various FTP file size

Assuming backhaul delay 5ms, Table 2 and Figure 2 gives the improvement of each TTI length for various FTP file size.

Table x2-2:	Various file size

| TTI length (in OFDM symbol number) | L1_OH |  | FTP File Size (Byte) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ##### 10K | ##### 100K | ##### 1M | ##### 2M | 5M |
| 14 symbol (baseline) | 20% | DRT | 0.157 | 0.263 | 0.634 | 1.036 | 2.263 |
| 7 symbol | 10% | DRT | 0.104 | 0.171 | 0.498 | 0.863 | 1.954 |
|  |  | ##### Gain | ##### 33.76% | ##### 34.98% | ##### 21.45% | ##### 16.70% | 13.65% |
|  | ##### 20% | ##### DRT | ##### 0.106 | ##### 0.176 | ##### 0.540 | ##### 0.954 | ##### 2.178 |
|  |  | ##### Gain | ##### 32.48% | ##### 33.08% | ##### 14.83% | ##### 7.92% | ##### 3.76% |
|  | ##### 30% | ##### DRT | ##### 0.107 | 0.177 | 0.596 | 1.062 | 2.464 |
|  |  | Gain | 31.85% | 32.70% | 5.99% | -2.51% | -8.88% |
|  | ##### 50% | ##### DRT | ##### 0.107 | ##### 0.186 | ##### 0.774 | ##### 1.428 | ##### 3.381 |
|  |  | ##### Gain | ##### 31.85% | ##### 29.28% | -22.08% | -37.84% | -49.40% |
| 3 symbol | 10% | DRT | 0.074 | 0.124 | 0.450 | 0.813 | 1.905 |
|  |  | ##### Gain | ##### 52.87% | 52.85% | 29.02% | 21.53% | 15.82% |
|  | ##### 20% | ##### DRT | ##### 0.074 | ##### 0.127 | ##### 0.494 | ##### 0.902 | ##### 2.129 |
|  |  | ##### Gain | ##### 52.87% | ##### 51.71% | ##### 22.08% | ##### 12.93% | 5.92% |
|  | ##### 30% | ##### DRT | ##### 0.074 | ##### 0.130 | ##### 0.551 | ##### 1.016 | ##### 2.417 |
|  |  | ##### Gain | ##### 52.87% | ##### 50.57% | ##### 13.09% | ##### 1.93% | ##### -6.81% |
|  | ##### 50% | ##### DRT | ##### 0.075 | 0.143 | 0.731 | 1.384 | 3.342 |
|  |  | ##### Gain | ##### 52.23% | ##### 45.63% | ##### -15.30% | ##### -33.59% | ##### -47.68% |
| 1 symbol | 10% | DRT | 0.059 | 0.103 | 0.430 | 0.793 | 1.883 |
|  |  | ##### Gain | ##### 62.42% | ##### 60.84% | ##### 32.18% | ##### 23.46% | ##### 16.79% |
|  | ##### 20% | ##### DRT | ##### 0.059 | ##### 0.106 | ##### 0.474 | ##### 0.882 | ##### 2.108 |
|  |  | ##### Gain | ##### 62.42% | 59.70% | 25.24% | 14.86% | 6.85% |
|  | ##### 30% | ##### DRT | ##### 0.060 | ##### 0.110 | ##### 0.529 | ##### 0.997 | ##### 2.396 |
|  |  | ##### Gain | ##### 61.78% | ##### 58.17% | ##### 16.56% | ##### 3.76% | ##### -5.88% |
|  | 50% | DRT | 0.060 | 0.124 | 0.712 | 1.366 | 3.323 |
|  |  | ##### Gain | ##### 61.78% | ##### 52.85% | ##### -12.30% | ##### -31.85% | ##### -46.84% |

TTI = 7 OFDM Symbols				                                       TTI = 3 OFDM Symbols

TTI = 1 OFDM Symbol

Figure x2-2:	Gain for various file size

From Table 2, it can be seen that the gain becomes smaller even negative with the larger file size and higher L1 overhead: for 10Kbyte and 100Kbyte file size, there is significant gain with shorter TTI, and better in 10KB than in 100KB; however, with larger file size, the gain is decreased and even to negative due to higher L1 overhead. We think main factors, which lead to the performance decrease, are listed as below:

1	The throughput in Uu interface is significantly affected by physical control channel and RS overhead, especially for larger files, and lack of Uu capacity leads to longer latency in RAN side;

2	The latency reduction mainly speeds the TCP slow start phase. With file size increasing, the total download time is increasing, and the ratio of TCP slow start phase in entire download period would be smaller.

For FTP download application, the larger the download file size, the less (even negative) the gain on DRT brought by shorter TTI.

With the same TTI length, the higher the L1 overhead, the less (even negative) the gain on DRT.

##### 9.1.x2.3.3	Various Uu throughput

Assuming backhaul delay 5ms and FTP file size 100KByte, Table x gives the improvement of each TTI length for different Uu throughput.

Table x2-3:	Various air interface condition

| TTI length (in OFDM symbol number) | L1_OH |  | Uu Rate = 27.907Mbps | Uu Rate = 2.848Mbps |
| --- | --- | --- | --- | --- |
|  |  |  | ##### MCS=20 ##### PRB_Num=50 | ##### MCS=12 ##### PRB_Num=10 |
| 14 symbol (baseline) | ##### 20% | DRT | 0.263 | 0.536 |
| ##### 7 symbol | ##### 10% | DRT | 0.171 | 0.438 |
|  |  | Gain | 34.98% | 18.28% |
|  | ##### 20% | DRT | 0.176 | 0.479 |
|  |  | Gain | 33.08% | 10.63% |
|  | ##### 30% | DRT | 0.177 | 0.536 |
|  |  | Gain | 32.70% | 0.00% |
|  | ##### 50% | DRT | 0.186 | 0.713 |
|  |  | Gain | 29.28% | -33.02% |
| ##### 3 symbol | ##### 10% | DRT | 0.124 | 0.409 |
|  |  | Gain | 52.85% | 23.69% |
|  | ##### 20% | DRT | 0.127 | 0.453 |
|  |  | Gain | 51.71% | 15.49% |
|  | ##### 30% | DRT | 0.13 | 0.509 |
|  |  | Gain | 50.57% | 5.04% |
|  | ##### 50% | DRT | 0.143 | 0.69 |
|  |  | Gain | 45.63% | -28.73% |
| ##### 1 symbol | ##### 10% | DRT | 0.103 | 0.397 |
|  |  | Gain | 60.84% | 25.93% |
|  | 20% | DRT | 0.106 | 0.44 |
|  |  | Gain | 59.70% | 17.91% |
|  | 30% | DRT | 0.11 | 0.497 |
|  |  | Gain | 58.17% | 7.28% |
|  | 50% | DRT | 0.124 | 0.676 |
|  |  | Gain | 52.85% | -26.12% |

TTI = 7 OFDM Symbols				                                       TTI = 3 OFDM Symbols

TTI = 1 OFDM Symbol

Figure x2-3:	Various Uu Throughput

Figure x2-4:	Various Uu Throughput

From Table 3 and Figure 3, it can be seen that the gain on DRT brought by shorter TTI is less when the provided Uu throughput is smaller; and in case of L1 overhead 50% and 1 symbol TTI, there is negative gain in case of Uu throughput 2.848Mbps. We think that the main reason is that the smaller Uu throughput brings the bottleneck for FTP downloading.

The smaller the Uu throughput, the less the gain on DRT brought by shorter TTI.

Considering the network backward compatibility, for a cell, since the network resource is shared for legacy TTI and for shorten TTI, reserving more PRBs for shorten TTI would lead to the less resource for legacy TTI, which would bring bad impact on the throughput on legacy UEs. In addition, since there are many UEs in one cell, allocating resource with 50 PRBs to a UE in every TTI can bring the low latency to that UE, but the UE number supported in that cell would be dramatically decreased. Hence, it is not reasonable to allocate resource with 50 PRBs to each UE each TTI. Comparing to that, the resource allocation with 10 PRB is more reasonable.

From Figure 4, it can be seen that in case of smaller Uu throughput, considering the same L1 overhead, there are not so much difference on the gain for each shorter TTI (i.e. 10.63% Vs. 15.49% Vs. 17.91%). In addition, Since the existence of RS signal and L1 control channel are inevitable for efficient data transmission, the shorter TTI would bring more L1 overhead, when we compare the gain of each TTI length, it is not appropriate to compare the gain of the same L1 overhead in different TTI length. For example, when evaluating the gain brought by 7 symbol and 3 symbol TTI length, it is more reasonable to compare the gain in (7 symbols, 20% L1 overhead) and in (3 symbols, at least 30% L1 overhead), 10.63% Vs. 5.04%, and 7 symbol TTI length is a little better than 3 symbol TTI length.

When evaluating and comparing the gain of different TTI length, it is reasonable to assume shorter TTI length leads to more L1 overhead.

In case of small Uu throughput, the gain brought by 20% L1 overhead and 7 symbols TTI is more than by 30% L1 overhead and 3 symbols TTI.

##### 9.1.x2.3.4	Fast UL access

UL access latency also brings impact on downloading performance, i.e. TCP ACK will feedback slowly if UL access latency is too big, which would impact the TCP transmission window increasing.

In Rel-10, 3GPP already discussed the latency reduction topic and at least the short D-SR and Pre-allocation, as we called fast UL access solutions, were supported by standardization or implementation, here we evaluated the performance gain with different UL access means (i.e. normal D-SR, short D-SR, and pre-allocation), and also with different TTI length.

Assuming download file size 100KByte and backhaul delay 5ms, Table 4 gives the improvement of various UL access solutions in different TTI length. In Table 4, gain (C) is the gain of the various UL access solution combined with TTI shortening, compared with the normal D-SR in 14-symbol TTI length (the value in red); and gain (UL) is the individual gain for various UL access solution, compared with the normal D-SR in each TTI length.

Note: the UL access latency components for UL access solutions are stated as in Annex <A1.x2>..

Table x2-4:	Various UL latency combining with TTI shortening

| UL Access |  | TTI Length (in OFDM symbol number) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | ##### 14 symbol | ##### 7 symbol | ##### 3 symbol | ##### 1 symbol |
| ##### Normal D-SR ##### (D-SR Interval = 5TTI) | DRT | 0.263 | 0.176 | 0.127 | 0.106 |
|  | gain (C) | 0.00% | 33.08% | 51.71% | 59.70% |
|  | ##### gain(UL) | ##### 0.00% | 0.00% | 0.00% | 0.00% |
| ##### Short D-SR ##### （D-SR Interval = 1TTI） | DRT | 0.244 | 0.168 | 0.124 | 0.105 |
|  | gain (C) | 7.22% | 36.12% | 52.85% | 60.08% |
|  | ##### gain(UL) | ##### 7.22% | ##### 4.55% | ##### 2.36% | ##### 0.94% |
| ##### Pre-allocation ##### （UL grant Interval = 1TTI） | DRT | 0.219 | 0.155 | 0.119 | 0.103 |
|  | ##### gain (C) | ##### 16.73% | ##### 41.06% | ##### 54.75% | ##### 60.84% |
|  | ##### gain(UL) | ##### 16.73% | ##### 11.93% | 6.30% | 2.83% |

Figure x2-5:	Gain(C) for various UL access solutions

Figure x2-6:	Gain (UL) for various UL access solutions

Figure 5 gives the gain for the combination of TTI shortening and UL latency reduction, and base on that it can be observed that compared with legacy UL access solution (i.e. 1ms TTI and 5ms D-SR period) the DRT would be dramatically decreased when the TTI and UL latency is shortening. Compared with the individual gain for TTI shortening given in Figure 4 (in case of PRB_Num=50, MCS=20, L1 OH=20%), TTI shortening gives most contribution.

Shorter TTI and shorter UL latency leads to shorter DRT, and most contribution is given from TTI shortening.

Figure 6 gives the individual gain for UL latency reduction solutions in each TTI length, respectively. It can be seen that the shorter the UL access latency is, the more the gain is; the shorter the TTI is, the more the gain is as well. However, we can’t see an obvious gain using fast UL access solutions in shorter TTI length, e.g. 3-symbol and 1-symbol TTI, as we see in normal TTI length. We think the reason is that faster TCP ACK feedback in uplink direction will speed up the TCP slow start, which will make the downloading time decreasing. On the other hand, shorter TTI has already speed up the UL access; so with shorter TTI, we can’t see an obvious gain using fast UL access solutions as we see in normal TTI length.

With the same TTI length, the shorter the UL access latency, the shorter the DRT.

With TTI shortening, the performance gain brought by fast UL access decreased.

### 9.1.x3	Simulation x3

Evaluation results for TTI reduction [6]

##### 9.1.x3.1	Simulation assumptions

When evaluating the gains provided by one symbol length TTI, we will consider various losses in L1 data rate due to the additional overhead caused by reduced TTI length. In Annex A1.x, we provide the estimated L1 data rate loss for one symbol length TTI. Table A1.x-1 assumes that the DL channel estimation is based on CRS, and Table A1.x-2 assumes that the DL channel estimation is based on DM-RS. From Table A1.x-1 and Table A1.x-2, we can see that the loss in L1 data rate is vary from -1.21% to 30.48%, hence we will use the L1 data rate loss of 0%, 5%, 10%, 20% and 30% respectively in the system simulation.

The simulation assumptions on radio aspects, TCP parameters and FTP traffic model are provided in Annex A1.2.

This simulation will not consider the increased transmission efficiency in high-speed scenarios due to the fast channel sensing (i.e. fast CSI report caused by shortened TTI).

Simulation results on UPT gain

Figure 1 illustrates the average UPT gain provided by one symbol length TTI for different L1 data rate loss, when the FTP file size is 0.5Mbytes. The number of UEs per cell assumed in the simulation is 1, 5, 10, 20 and 35, which roughly corresponds to the system load of 3%, 12%, 25%, 50% and 87% respectively.

Figure x3-1: Average UPT gain provided by one symbol length TTI, for different L1 data rate loss

From Figure 1, it could be observed that one symbol length TTI can bring significant UPT gain in general, and more specially:

-	The UPT gain gradually decreases when the loss in L1 data rate increases, as the available PDSCH capacity for data transmission is decreased. Nevertheless, even when the loss in L1 data rate reaches 30%, the UPT gain is still considerable, e.g. beyond 20%.

-	The UPT gain gradually decreases when the system load increases (i.e. when there are more UEs in the cell), this is because the UPT gain provided by shortened TCP layer RTT will be compromised by the decreased transmission/reception data rate of the UE. In case the system is heavy loaded (i.e. 35 UEs per cell), one symbol length TTI only brings marginal UPT gain even negative UPT gain.

Figure 2 illustrates the 5% UPT gain provided by one symbol length TTI for different L1 data rate loss, when the FTP file size is 0.5Mbytes.

Figure x3-2: 5% UPT gain provided by one symbol length TTI, for different L1 data rate loss

From Figure 2, it could be observed that the 5% UPT gain provided by one symbol length TTI is negative, even if we don’t consider any L1 data rate loss (i.e. 0% loss) in the simulation. This is because for 5% of the UEs, only a few bytes could be accommodated in each TTI given that the TTI is only one symbol length and the UE is in poor channel condition. Small TB size in each TTI will cause more segments for packet delivery, and consequently will bring significant MAC and RLC overhead. In addition, the 5% of UEs might suffer from power limitation, which may prevent timely report of the TCP ACK, and this will enlarge the TCP layer RTT hence further decrease the UE throughput. For the L1 data rate loss of 30%, the negative UPT gain for 5% of the UEs is significant, i.e. beyond 40%.

One symbol length TTI can bring significant UPT gain in average, even if the loss in L1 data rate reaches 30%.

If the system is high loaded, or if the UE is in cell edge, one symbol length TTI may bring negative UPT gain.

Figure x3-3 illustrates the average UPT gain provided by one symbol length TTI for different FTP file sizes, assuming the L1 data rate loss is 0% and the number of UEs per cell is one.

Figure x3-3: UPT gain provided by one symbol length TTI, for different file sizes

From Figure x3-3, it could be observed that the average UPT gain provided by one symbol length TTI decreases as the FTP file size increases. This is because the accelerated TCP slow start phase by shortened TTI is the major contributor for the UPT gain, and for small FTP file the TCP show start phase takes a big percentage of the total FTP download time while this not the case for large FTP file. Nevertheless, the average UPT gain is still considerable when the FTP file size arrives at 8Mbytes, which is about 25%. This is because the UPT will still benefit from the shortened TCP layer RTT caused by one symbol length TTI, unless the system is high loaded.

One symbol length TTI can also bring significant UPT gain in average for large file size.

### 9.1.x4	Simulation x4

Performance evaluation of latency reduction enhancements [6]

##### 9.1.x4.1	Simulation setup

This section analyses the performance impact of shorter TTI and fast UL. System simulations have been conducted to compare different approaches for latency reduction. The simulations are run in the 3GPP macro scenario, i.e. 3GPP Case 1 macro scenario with 7 sites with 21 cells in total and wrap-around. For the UEs, we use a quasi-static model with static UE mobility and TU channel model with 3 km/h speed.

The performance evaluation is done for FTP traffic model 1 [10], where the load is varied by having different UE arrival rates for a fixed 0.5 MB file size: 26.25, 52.5, 78.75, 105 dropped per second to whole network, which translates to an offered load of 5-20 Mbps per cell. Additionally we also consider the impact of FTP file size. 0.1 MB and 2 MB file sizes are simulated as well (this affects the UE arrival rate proportionally). In the simulations the FTP traffic is transported using TCP. This enables observing the impact of TCP slow start phase, which is where the reduced latency is particularly beneficial. After the file has been transmitted, the UE is removed from the system (so new files are generated for new users). So it is assumed that each new file transfer starts a new TCP connection with slow start.

Additional L1 and L2 overhead due to the shorter TTI is modelled so that in the reference case (14 symbol TTI) the overhead is assumed to be 20% and we have additional cases with 30%, 40%, 50% overhead. We range several different values as it is not yet clear what would be a realistic assumption here.

More details of simulation assumptions can be found in table 1. Section 2.1 describes the simulated schemes and Section 2.2 and 2.3 present the performance evaluation results.

Table 1 Simulation parameters

| Parameter | Assumption |
| --- | --- |
| System bandwidth | 20 MHz |
| Duplex mode | FDD |
| Carrier frequency | 2GHz |
| Cell layout | Hexagonal grid, 7 sites, 21 cells per site, with wrap-around |
| Number of UEs | According to offered load |
| Inter-site distance | 500m |
| UE speed | 3 km/h, quasi-static model |
| Antenna configuration | 1x2 MRC |
| eNB TX power | 46 dBm |
| UE Tx power | 23dBm |
| Channel model | Typical Urban |
| Pathloss model | 25.814 |
| Lognormal shadowing, std. dev. | 8dB |
| Penetration loss | 20dB |
| CQI measurement period | 5 ms |
| SRS reporting period | 5 ms |
| SR to grant | 8ms – scaled down with shorter TTI |
| HARQ RTT | 8ms – scaled down with shorter TTI |
| SR Period | Varied 1,5,10,20 ms |
| DRX | OFF |
| Initial TCP Window | 3 x 1500 Bytes (MSS), RFC 5681, section 3.1 |
| Initial Ssthresh | 150 x 1500 Bytes (MSS) |
| Ssthresh | Dynamic according to RFC 5681 [11], sections 3.1 and 3.2 |
| FTP file size | 0.5 MB |
| User arrival rate λ | FTP model 1 with UE arrival according to Poisson process [10] |
| Scheduler | TD: PF, FD: PF |
| Maximum number of scheduled users per TTI | 10 (max) |
| L1 overhead | Varied (ref (20%), 10, 20, 30 %-units increase in overhead) |
| Core network delay | 2ms, 10ms |
| Load per macro cell | 5, 10, 15, 20 Mbps |
| TTI Length | Varied (2symbols, 7 symbols, 14 symbols) |

##### 9.1.x4.2	Simulated schemes

-	Shorter TTI (7 symbols or 2 symbols)

Shorter TTI approach refers to shortening the TTI length from the currently specified 14 symbols and 1 ms to 7 symbols i.e. 0.5 ms, or even going down to a 2 symbol TTI i.e. 0.14 ms. Having shorter length TTI and high data rates would imply that more PDUs need to be handled with higher L1 and L2 overhead, which can at least to some degree offset the benefits of the reduced round trip time.

-	Fast UL grant (i.e. pre-scheduling)

In the simulations the UE is assumed to always have resource granted (in practice this would be when the eNB expects some UL data from the UE, e.g. TCP ACK). Fast uplink scheduling can be achieved with protocol enhancements in L2 like in MAC protocol without changing physical layer principles like TTI length. However, it is applicable not only with the current 14 symbol TTI, but also with shorter TTI lengths.

-	UL scheduling based on SR (as presented in Figure 1)

SR based UL grant refers to the (baseline) approach of UE sending SR in order to get an UL grant. This scheme was used as a reference for the other cases.

Figure x4-2 SR based UL grant procedure

##### 9.1.x4.3	Simulation results for shorter TTI

The performance metric used for comparing the different schemes is the user throughput measured over the entire FTP file transfer. This is inversely proportional to the total delay of the file.

A:	Effect of shorter TTI

Performance with 2 and 7 symbol TTI compared to the 14 symbol TTI for different L1 overhead, and core network delay (CN delay) assumptions. The results are presented for SR based UL grant (i.e. without pre-scheduling enhancement).

| Figure x4- 3a: shorter TTI with 2ms CN delay, 5%-tile throughput | Figure x4-3b: shorter TTI with 10ms CN delay, 5%-tile throughput |
| --- | --- |
| Figure x4-4 shorter TTI with 2ms CN delay, 50%-tile throughput | Figure x4-5 shorter TTI with 10ms CN delay, 5%-tile throughput |

As indicated by the results, the potential gain from shorter TTI depends on how much L1/L2 overhead is assumed, the load of the cell as well as the CN delay. With L1/L2 overhead increased by 10 %-units, the difference between 2 symbol TTI and 7 symbol TTI is visible in low load situation. Increasing the load or the L1/L2 overhead the performance starts to deteriorate with shorter TTI compared to reference case. With higher load, a UE can no longer be scheduled in every (short) TTI, but may have to wait its turn. This reduces the latency benefit from the shorter TTI.

The potential gain from having shorter TTI depends on how much L1/L2 overhead is assumed and the load of the cell. It would be beneficial to control when shorter TTI are being used.

The difference between 2 symbol TTI and 7 symbol TTI is visible only in low load situation.

Increasing the load the performance starts to deteriorate resulting in negative gain with shorter TTI when the effect of scheduling delay becomes visible.

A:	Effect of shorter TTI: comparing different file sizes

| Figure 6: 100kB file size, 5%-ile UE throughput | Figure 7: 100kB file size, 50%-ile UE throughput |
| --- | --- |
| Figure x4-8 500kB file size, 5%-ile UE throughput | Figure x4-9 500kB file size, 50%-ile UE throughput |
| Figure x4-10 2MB file size, 5%-ile UE throughput | Figure x4-11 2MB file size, 50%-ile UE throughput |

With smaller file size (100kB), the shorter TTI gives more gain due to the TCP slow start phase being more emphasized. With increased file size (2 MB) the benefits on the other hand start to diminish and additional overhead dominates. Similar observations are valid for both 5%-ile and 50%-ile UE throughput.

The potential gain of shorter TTI depends on file size. The benefits of shorter TTI diminish with larger file size. It would be beneficial to control when latency-reducing techniques are being used.

#### 9.1.x4.4	Simulation results for Fast UL grant with shorter TTI

‘SR based UL grant’ is used as reference scheme for the ‘Fast UL grant’.

A:	Comparison of ‘Fast UL grant’ vs. ‘SR based UL grant’

| Figure x4-12 50%-ile throughput with 2 symbol TTI | Figure x4-13 50%-ile throughput with 7 symbol TTI |
| --- | --- |
| Figure x4-14 50%-ile throughput with 14 symbol TTI |  |

Fast UL grant approach can give a significant performance improvement over (legacy) SR based UL grant. This is especially the case when longer SR periodicities are used.

Pre-scheduling is beneficial over SR based Ul grant with shorter TTI as well.

Similar observations can be seen for 5%-ile UE throughput.

| Figure x4-15 5%-ile throughput with 2 symbol TTI | Figure x4-16 5%-ile throughput with 7 symbol TTI |
| --- | --- |
| Figure x4-17 5%-ile throughput with 14 symbol TTI |  |

### 9.1.x5	Simulation x5

TTI reduction gain with additional L1/L2 overhead [6]

#### 9.1.x5.1	Simulation assumptions

In this evaluation, it is assumed that the core network and the Internet delay is very small and is independent of the TTI values. The assumed latency between eNB and TCP Server is 1-2 ms.

The average DL data-rate is kept the same for different TTIs. This is achieved by scaling the TBS sizes according to the duration of TTI compared to the current 1ms TTI.

For the FTP based file download using TCP protocol, TCP New Reno is used.

User perceived throughput (UPT) is calculated as total TCP layer data downloaded divided by the time elapsed. Initial time for setting up the network and for the UE to attach to network is removed as offset from the results such that the FTP request is sent to the server from the UE at time 0.

A saturation data-rate of approximately 21Mbps (example scenario is 3 CA 20MHz each, cell spectral efficiency ~3.5bps/Hz, 10 UEs/cell) is assumed. MCS index of UE is set such that on an average this data-rate is achieved when TCP has fully ramped up (channel adaptation is disabled).

In addition, for this study, fixed additional L1/L2 overheads due to reduced TTI handling are assumed. The additional overheads are simulated by scaling down the TBS accordingly during scheduling, both in DL and in UL. Because of the additional overhead, TCP “saturation” data-rate will be different for different overhead values, so ramp-up time taken to reach 90% saturation rate is not a fair metric for comparison across different overhead values within the same TTI. Therefore, FTP file download time (defined as the time duration between the instant when UE sends the FTP request and when the file completes download) and user perceived throughput (UPT) during the entire file download (total file size divided by file download time) is considered. Different file sizes for download are considered.

#### 9.1.x5.2	Evaluation results

Figure x5-4 and Table x5-1 show the file download time and user perceived throughput considering different additional L1/L2 overheads for different TTI. 1ms TTI with no additional L1/L2 overhead is taken as baseline for comparison. Various file sizes are considered to cover the cases of typically small, medium and large file downloads (100KB, 500KB and 1MB).

It is observed that for small FTP file sizes (100KB), even with 40% additional L1/L2 overhead, 0.5ms and 0.1ms TTI significantly outperform the baseline 1ms TTI – for 0.5ms the gain is about 43% increase in UPT (decrease in file download time) and for 0.1ms, the UPT is doubled. This is because the system is not overloaded during the initial ramp-up time (as sender TCP has not picked up yet), thus additional overhead does not affect the initial part of the TCP slow-start for the same TTI value. However, note that the slow-start behavior is different for different TTI (as shown in earlier results). Therefore, the gain due to improvement in slow-start behavior outperforms the loss due to additional L1/L2 overhead.

For smaller FTP download using TCP, the user perceived throughput improvement from the TCP slow-start outperforms the potential degradation even with additional L1/L2 overhead in the shorter TTI.

For medium file size (500KB), similar UPT compared to 1ms baseline TTI is observed with 20-30% additional overhead in 0.5ms TTI and even when more than 30% additional L1/L2 overhead is present in 0.1ms TTI.

When FTP file size is further increased, as expected, the 1ms TTI baseline scenario also can obtain high UPT. In such case, the amount of additional overhead due to reduced TTI without losing UPT performance is lower. For example, for 1MB file size, it is observed that similar UPT as in 1ms TTI is obtained with 10-20% additional overhead in 0.5ms and 0.1ms TTI.

For higher size FTP download using TCP, the user perceived throughput may be degraded in the shorter TTI if additional L1/L2 overhead is high.

Figure x5-4 FTP file download performance with TTI reduction and different L1/L2 overhead

Table x5-1 Comparison of FTP file download performance with different L1/L2 overhead

| TTI | Additional L1/L2 overhead | FTP File size 100 KB |  | FTP File Size 500KB |  | FTP File Size 1MB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | File Download Time (ms) | UPT (Mbps) | File Download Time (ms) | UPT (Mbps) | File Download Time (ms) | UPT (Mbps) |
| 1ms | Baseline 0% | 148 | 5.4 | 308 | 13.0 | 503 | 15.9 |
| 0.5ms | 0% | 79 | 10.1 | 234 | 17.1 | 434 | 18.4 |
|  | 10% | 81 | 9.9 | 254 | 15.7 | 474 | 16.9 |
|  | 20% | 86 | 9.3 | 281 | 14.2 | 529 | 15.1 |
|  | 30% | 99 | 8.1 | 331 | 12.1 | 609 | 13.1 |
|  | 40% | 104 | 7.7 | 369 | 10.8 | 699 | 11.4 |
| 0.1ms | 0% | 46 | 17.4 | 208 | 19.2 | 410 | 19.5 |
|  | 10% | 50 | 16.0 | 230 | 17.4 | 455 | 17.6 |
|  | 20% | 56 | 14.3 | 260 | 15.4 | 515 | 15.5 |
|  | 30% | 64 | 12.5 | 299 | 13.4 | 592 | 13.5 |
|  | 40% | 73 | 11.0 | 347 | 11.5 | 692 | 11.6 |

### 9.1.x6	Simulation x6

Effect of UE and eNB processing times on TCP performance [6]

In current LTE, UE processing time for downlink data reception is determined based on PUCCH transmission timing. For example, in FDD, the UE should send HARQ-ACK in subframe n+4 where the UE receives PDSCH in subframe n. Therefore, the maximum UE processing time is considered to be 3ms (which is denoted by ∆UE). This value is same as UE processing time for uplink as the UE sends uplink data at subframe n+4 when the UE receives Uplink grant in subframe n.

Note that the actual processing time available to the UE may be reduced due to TA (timing advance) (i.e., twice the propagation delay) as shown in Figure x6-5 below.

Figure x6-5 UE's actual available processing time is reduced due to propagation delays

In the following studies, we evaluate latency reduction gain with respect to different processing time in UE and in eNB side because the UP delay can be achieved by reducing UE/eNB processing time even in the normal TTI (1ms).

We assume decreased processing times of 2ms and 1ms (i.e., eNB takes 2ms / 1ms to process SR and send grant; UE takes 2ms / 1ms from receiving DL data to sending ACK).

One way to achieve less processing delay would be TBS (Transport Block Size) restriction by the specification in order to accelerate decoding speed together with potential decoder improvement by UE implementation.  However, the exact solution can be discussed in RAN1.

#### 9.1.x6.1	Simulation assumptions

For this study, we consider 1ms and 0.5ms TTI lengths with 5TTI SR periodicity.

In this evaluation it is assumed that the core network and the Internet delay is very small and is independent of the TTI values and eNB/UE processing times. The assumed latency between eNB and TCP Server is 1-2 ms.

The average DL data-rate is kept the same for different scenarios. This is achieved by scaling the TBS sizes according to the duration of TTI compared to the current 1ms TTI.

For the FTP based file download using TCP protocol, TCP New Reno is used. File size is kept so as to achieve the saturation in data-rate so that the conditions when UE’s perceived instantaneous as well as overall data-rate reaches at least 90% of the saturation rate can be determined.

The instantaneous throughput is calculated as 25ms moving window average of TCP layer throughput. Aggregate user perceived throughput (UPT) is calculated as total TCP layer data downloaded divided by the time elapsed. Initial time for setting up the network and for the UE to attach to network is removed as offset from the results such that the FTP request is sent to the server from the UE at time 0.

A saturation data-rate of approximately 21Mbps (example scenario is 3 CA 20MHz each, cell spectral efficiency ~3.5bps/Hz, 10 UEs/cell) is assumed. MCS index of UE is set such that on an average this data-rate is achieved when TCP has fully ramped up (channel adaptation is disabled).

The results for TTI reduction in this simulation do not take into account possible overhead due to e.g. additional L1/2 overhead.

#### 9.1.x6.1	Evaluation results

Figure x6-6 and Figure x6-7 show the comparison of TCP slow-start for different node processing delays. For simplicity, the processing delays for UE and eNB (denoted by ∆UE and ∆eNB respectively) are assumed to be equal in the following results. The performance of current processing delay (3ms) is compared with reduced values (2ms and 1ms) for UE and eNB processing delays. As expected the TCP ramps up quickly with smaller node processing times.

Figure x6-6 Effect of reduced node processing times on TCP slow-start behaviour with 1ms TTI

Figure x6-7 Effect of reduced node processing times on TCP slow-start behaviour with 0.5ms TTI

Table x5-1 shows the comparison of the scenarios described above. It is observed that about 40% gains are observed in the considered metrics when node processing delays are reduced to 1ms from baseline 3ms in 1ms TTI. Keeping the existing processing time (3ms from DL to UL), reduction in TTI from 1ms to 0.5ms reduces the TCP ramp-up time by 1/3rd.  The ramp-up time can be decreased by more than half with 0.5ms TTI compared to 1ms TTI, even with a more relaxed 4TTI UE processing time.

TCP slow-start performance is improved with the reduced UE/eNB processing times.

Table x6-2 Comparison of TCP slow-start performance

| ∆UE, ∆eNB (ms) | Time for TCP ramp-up to 90% of saturation rate* | Min file size for TCP ramp-up to 90% of saturation rate | Time taken for UPT to be 90% of saturation rate* | Min file size for UPT to be 90% of saturation rate |
| --- | --- | --- | --- | --- |
| 1ms TTI |  |  |  |  |
| 3ms - baseline | 176ms | 142KB | 1.31s | 3.12MB |
| 2ms | 152ms (-14%) | 124KB (-13%) | 1.06s (-19%) | 2.53MB (-19%) |
| 1ms | 107ms (-39%) | 84KB (-41%) | 768ms (-41%) | 1.84MB (-41%) |
| 0.5ms TTI |  |  |  |  |
| 3ms (6TTI) | 116ms (-34%) | 114KB (-20%) | 813ms (-38%) | 1.93MB (-38%) |
| 2ms (4TTI) | 82ms (-53%) | 65KB (-54%) | 644ms (-51%) | 1.53MB (-51%) |
| 1ms (2TTI) | 67ms (-62%) | 63KB (-56%) | 457ms (-65%) | 1.1MB (-65%) |

It is also observed from Table x6-1 that the performance gain can be achieved by reducing processing times or by reducing TTI or both. In the studied scenarios, gains are observed to be comparable for keeping the 1ms TTI and reducing the processing time to 1ms (1TTI) or reducing the TTI to 0.5ms and keeping the same 3ms of processing time (i.e., 6TTI). On the other hand, as shown in earlier results, the gains in TCP ramp-up with reduced TTI of 0.5ms while keeping 3TTI processing time is significantly high (~60%) compared to 1ms TTI and 3TTI processing time. So, in general, TTI reduction shows more gain than simply reducing processing time without changing TTI.

TTI reduction is more effective than simply reducing processing time without TTI reduction.

It should be noted that with the change in processing time, the HARQ RTT changes (as shown in Appendix A). Therefore different number of HARQ processes can be supported based on the TTI, as shown in Table x6-3 below.

Table x6-3 Number of possible HARQ processes

| ∆UE, ∆eNB (ms) | 1ms TTI |  | 0.5ms TTI |  |
| --- | --- | --- | --- | --- |
|  | HARQ RTT (ms) | Max num of UL HARQ processes | HARQ RTT (ms) | Max num of UL HARQ processes |
| 3ms | 8 | 8 | 7 | 14 |
| 2ms | 6 | 6 | 5 | 10 |
| 1ms | 4 | 4 | 3 | 6 |

Table x6-4 below shows the average air-interface latency for UEs in connected mode. Average downlink latency and latency for the UE initiated uplink data transmission for a UE with uplink synchronization are calculated following the timing analysis of [3] as explained in Appendix A. As expected, both DL and UL average delays (SR-related steps and Data transmission steps) can be reduced with reduction in node processing times.

Table x6-4 Average Air-interface latency for different processing delays and TTI

| ∆UE, ∆eNB (ms) | Average air-interface latency |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL |  | UL (SR periodicity 5ms) |  |  |  |  |  |
|  | error free | 10% error | error free |  |  | 10% error |  |  |
|  |  |  | SR-related | Data Tx | Total | SR-related | Data Tx | Total |
| 1ms TTI |  |  |  |  |  |  |  |  |
| 3ms | 4ms | 4.8ms | 9ms | 4ms | 13ms | 9ms | 4.8ms | 13.8ms |
| 2ms | 3ms | 3.6ms | 7.5ms | 3ms | 10.5ms | 7.5ms | 3.6ms | 11.1ms |
| 1ms | 2ms | 2.4ms | 6ms | 2ms | 8ms | 6ms | 2.4ms | 8.4ms |
| 0.5ms TTI |  |  |  |  |  |  |  |  |
| 3ms | 3.5ms | 4.2ms | 6.75ms | 3.5ms | 10.25ms | 6.75ms | 4.2ms | 10.95ms |
| 2ms | 2.5ms | 3ms | 5.25ms | 2.5ms | 7.75ms | 5.25ms | 3ms | 8.25ms |
| 1ms | 1.5ms | 1.8ms | 3.75ms | 1.5ms | 5.25ms | 3.75ms | 1.8ms | 5.55ms |

It is observed from the latency values in Table x6-4 and TCP ramp-up performance results in Table x6-1 that the TCP ramp-up performance is highly correlated, as expected, to the average air-interface latency, i.e., the ramp-up is faster if the air-interface latency is lower. In addition, the same benefits in terms of TCP slow-start performance can be observed with latency reduction in general regardless of how the lower latency is achieved.

### 9.1.x7	Simulation x7

System Performance Gain with TTI reduction [6]

#### 9.1.x7.1	Faster UE Feedback and Rate Control

The current LTE TTI is 1ms and composed of two 0.5ms slots with seven symbols (in case of normal CP length) per slot. In selection of a new shorter TTI, keeping the necessary work, backwards compatibility and co-existence in mind, it should be chosen as a multiple of the symbol length. Therefore, here we assume that the shortest TTI possible is one symbol and use that to compare with the current TTI duration.

For the possible shorter TTIs, we will assume that processing and feedback times are scaled with TTI duration; for example the HARQ feedback is always after four TTIs. This is based on the RAN2#90 agreement that [1] “We assume that the UE and eNB processing time (HARQ RTT) scales with the TTI duration”.

A shorter TTI can provide faster CSI and HARQ, which can lead to more accurate rate control algorithms that better match selected data rates with rapidly changing radio conditions. This is beneficial even for low mobility UEs since radio conditions can change quickly due to bursty interference from neighbour cells. As shown in Figure x7-1, the reported CSI and adjusted value after rate control is much closer to the actual channel conditions observed by the UE.

| Release-8 LTE | Short TTI with slow feedback | Short TTI with fast feedback |
| --- | --- | --- |

Figure x7-1 Benefits of Fast Rate Adaptation (slow feedback is not scaled with TTI while fast feedback is scaled with TTI)

#### 9.1.x7.2	Simulation Assumptions:

-	3GPP D1 Configuration:

-	1 channel of 10Mhz with one LTE macro site of 3 sectors.

-	10 UEs are dropped randomly in each cell, 3 cells per site, 7 sites.

-	3 km/h slow fading.

-	FTP: Poisson arrivals with

-	Case 1: inter arrival time = 0.1/sec, PktSize=100kbits

-	Case 2: inter arrival time = 1/sec, PktSize=1Mkbits

-	Case 3: inter arrival time = 4/sec, PktSize=4Mbits

-	TTI durations: 1 symbol and 1ms

-	HARQ ACK/NAK Delay = 4*TTI

-	CQI Feedback Cycle = 4*TTI or 4ms (for TTI=1symbol, slow feedback assumes 4ms)

##### 9.1.x7.3	Simulation Results

System level simulation results under the 3GPP methodology for FTP traffic are shown in Table x7-1 . The details of simulation assumptions are provided in the Appendix. TTI duration of 1 symbol was compared to the current 1ms TTI. The results are for the UE-perceived throughput and median delay, which are calculated as the average throughput, and delay for the received FTP packets. Three file sizes chosen for the FTP packets were 100Kbits, 1Mbits, and 4Mbits.

As seen in the results, even with the same absolute CSI feedback frequency, there is significant gain for the UE perceived throughput as well as median delay since the rate control algorithm can adapt better with faster HARQ turnaround. Further gains are also seen when the CSI feedback cycle is reduced. The gains are higher for smaller FTP packet sizes. This is expected since the total transmission time increase with larger packet sizes, which allows rate adaptation for 1ms TTI to improve.

Tablex7-1 UE-perceived throughput comparison between 1 symbol and 1ms TTI

| File size (100 Kbits) | 5-th %ile tput | median tput | 95-th %ile delay | median delay |
| --- | --- | --- | --- | --- |
| ##### LTE | ##### 4.8 Mbps | ##### 8.87 Mbps | 20.8 ms | ##### 11.3 ms |
| ##### Short TTI, slow CSI | ##### 2.32x | ##### 2.27x | ##### 0.43x | ##### 0.44x |
| ##### Short TTI, fast CSI | ##### 4.39x | ##### 3.63x | ##### 0.23x | ##### 0.28x |
| ##### File size (1 Mbits) | ##### 5-th %ile tput | ##### median tput | ##### 95-th %ile delay | ##### median delay |
| ##### LTE | ##### 12.4 Mbps | ##### 22.4 Mbps | ##### 80.4 ms | ##### 44.7 ms |
| ##### Short TTI, slow CSI | 1.45x | ##### 1.44x | ##### 0.69x | ##### 0.70x |
| ##### Short TTI, fast CSI | ##### 1.98x | ##### 1.77x | ##### 0.50x | ##### 0.57x |
| ##### File size (4 Mbits) | ##### 5-th %ile tput | ##### median tput | ##### 95-th %ile delay | ##### median delay |
| ##### LTE | ##### 15.0 Mbps | ##### 28.0 Mps | ##### 267 ms | ##### 143 ms |
| ##### Short TTI, slow CSI | ##### 1.15x | ##### 1.17x | ##### 0.87x | ##### 0.85x |
| ##### Short TTI, fast CSI | ##### 1.46x | ##### 1.27x | ##### 0.69x | ##### 0.79x |

-- For inclusion in Section 11, Conclusions --

## 11.1	RAN2 Protocol Evaluations

In protocol evaluations, the potential gains in terms of increased download throughput and reduction of download time for reduced latency in LTE has been performed and evaluated.

For TCP, and TCP slow start specifically, results show that reduced UL latency, shorter RTT and HARQ RTT can have a positive impact on TCP performance depending on cell load and L1/L2 overhead. This is due to the fact that the receiver may acknowledge TCP packets faster which enables a faster increase in the TCP window size. Latency reduction has shown a positive impact on both TCP congestion avoidance mode and in TCP slow start phase.

As the initial window size for each TCP connection is very small and the increase is steeper for each size increment, the effect of latency reductions for both RTT and HARQ RTT are more considerable for the slow start phase. This impact is large for small file sizes, especially where the slow start period lasts for the entire duration of the file transfer (how long the phase lasts depends on TCP congestion control algorithm and parameters). For larger file sizes, the proportion of the slow start phase of the whole file is smaller if packet losses can be avoided.

Furthermore, the following high-level observations have been obtained:

For TTI shortening, the potential gain depends on the following aspects:

-	Delays in the core network and Internet also impact the UE perceived throughput. If these delays dominate over radio network delays, latency reduction techniques will be less efficient.

-	In high loaded cells, or in situations where the available scheduled Uu bitrate is low, e.g. due to radio conditions, user data is queued before transmitting. This has an impact on User Throughput. In these scenarios, efforts to reduce radio latency will be less important as the limited availability of UL resources may result in reduced performance as the queuing delay dominates over the radio delay.

-	By reducing the TTI length, the network can schedule the UE faster, which reduces the RTT. A reduction in RTT increases the TCP throughput. A reduction of TTI length may also increase the system capacity for small data transmission.

- 	For file transfers using TCP, the amount of UPT gain provided by shorter TTI may decrease with larger file size.

-	With reduced TTI length, the processing time may be reduced with a resulting scaling of the RTT and HARQ RTT. Shortening the TTI gives a larger latency reduction effect compared to only shortened processing time(s).

-	A reduction of TTI length may require additional L1 overhead e.g. CRC, RS, DCI, UCI and result in a relative increase of L2 headers. Increased overhead will to some extent limit the User Throughput gain from TTI reduction.

-	Shorter TTI can enable quicker HARQ and CSI feedback, which may give faster link adaptation to channel conditions. Gain can be observed for the UE perceived throughput as well as median delay .

For fast uplink access:

-	An enhancement to SPS to allow for periodic UL grants every TTI (Fast UL) reduces the latency of the first UL transmission compared to legacy intervals, and performs equally well compared with SR every 1ms with lower control channel load. Fast UL improves User Throughput also with shorter TTI, although the relative gain is smaller compared to longer TTIs.

-	An enhancement of UL grants allowing the UE to skip padding transmissions in grant if it has no UL data in the UE buffer is beneficial as it may decrease UL interference and improve UE battery efficiency.

-- For inclusion in Annex <A>, Simulation assumptions, A1 Protocol Simulations --

## Annex <A1.x3> Simulation x3 [6]

For one symbol length TTI, we assume new PDCCH will be used for data scheduling. Although the legacy PDCCH is not needed anymore, we keep one symbol length legacy PDCCH region for CSS.

As we evaluated in system simulation for 3GPP Case 1, the average PDCCH Aggregation Level is 1.42. Therefore, we assume that the average PDCCH payload for each UE is 52 REs.

Table A1.x3-1: L1 overhead analysis (CRS based DL channel estimation)

| TTI length | 1ms | 1 symbol |
| --- | --- | --- |
| Number of legacy UEs (not capable of one symbol length TTI) scheduled per ms | 20 | 0 |
| Number of new UEs (capable of one symbol length TTI) scheduled per ms | 0 | 20 |
| Number of CRS port | 2 |  |
| Number of DMRS port | 0 |  |
| System Bandwidth (MHz) | 20 |  |
| Average PDCCH payload for each UE (number of RE) | 52 |  |
| CRS overhead in non-PDCCH region (number of RE/PRB/ms) | 12 |  |
| Legacy PDCCH overhead for legacy UE scheduling (number of RE/ms) | 1040 | 0 |
| Total legacy PDCCH overhead (number of RE/ms) | 2400 (2 symbols) | 1200 (1 symbol) |
| New PDCCH overhead for new UE scheduling (number of RE/ms) | 0 | 1040 |
| Total PDCCH overhead (number of RE/ms) | 2400 | 2240 |
| CRS overhead (number of RE/ms) | 1200 | 1200 |
| Total overhead (number of RE/ms) | 3600 | 3440 |
| Overhead ratio | 21.43% | 20.48% |
| Normalized L1 data rate (compared to 1ms TTI) | 100% | 101.21% |

Table A1.x3-2: L1 overhead analysis (DM-RS based DL channel estimation)

| TTI length | 1ms | 1 symbol |
| --- | --- | --- |
| Number of legacy UEs (not capable of one symbol length TTI) scheduled per ms | 20 | 0 |
| Number of new UEs (capable of one symbol length TTI) scheduled per ms | 0 | 20 |
| Number of CRS port | 1 |  |
| Number of DMRS port | 2 |  |
| System Bandwidth (MHz) | 20 |  |
| Average PDCCH payload for each UE (number of RE) | 52 |  |
| CRS overhead in non-PDCCH region (number of RE/PRB/ms) | 6 |  |
| DMRS overhead (number of RE/PRB/TTI) | 12 | 4 |
| Legacy PDCCH overhead for legacy UE scheduling (number of RE/ms) | 1040 | 0 |
| Total legacy PDCCH overhead (number of RE/ms) | 2400 (2 symbols) | 1200 (1 symbol) |
| New PDCCH overhead for new UE scheduling (number of RE/ms) | 0 | 1040 |
| Total PDCCH overhead (number of RE/ms) | 2400 | 2240 |
| CRS overhead (number of RE/ms) | 600 | 600 |
| DMRS overhead (number of RE/ms) | 1200 | 5200 |
| Total overhead (number of RE/ms) | 4200 | 8040 |
| Overhead ratio | 25.00% | 47.86% |
| Normalized L1 data rate (compared to 1ms TTI) | 100% | 69.52% |

## Annex <A1.x6> Simulation x6 [6]

A.1 Downlink

It has been understood by RAN2 that processing delays in the different nodes form the major part of the delay in obtaining valid scheduling assignment, and reducing such processing delay could improve the latency compared to LTE Rel-8 [TR 36.912]. Following the same approach as in section B.2.1 in [TR 36.912], the LTE U-plane one-way latency for a scheduled UE consists of the fixed node processing delays and 1TTI duration for transmission, as shown in Figure B.1 below. Let us denote total processing time for eNB from UL reception to DL transmission (in ms) by ∆eNB and total processing time for UE from DL reception to UL transmission (in ms) by ∆UE. The fraction of these processing times needed for processing the reception and preparing the transmission are denoted by w, x, y and z where w + x =1 and y + z = 1.

UE

eNB

z. ∆eNB ms

y. ∆eNB ms

HARQ RTT

(∆eNB + ∆UE + 2 TTI)

x. ∆UE ms

w. ∆UE ms

TTI

1 TTI

1 TTI

Figure A.1. eNB and UE processing delays and HARQ RTT

The one way latency can be calculated as

D = y.∆eNB + 1 TTI + w.∆UE + n* (∆eNB + ∆UE + 2TTI), where n is the number of HARQ retransmissions.

Considering a typical case where there would be 0 or 1 retransmission, assuming error probability of the first transmission to be p, and w = x = y = z = 1/2, the delay is given by

D = (1/2 + p) * (∆eNB + ∆UE + 2TTI).

So, for 0% BLER,     D = 0.5 * (∆eNB + ∆UE + 2TTI),    and for 10% BLER,  D = 0.6 * (∆eNB + ∆UE + 2TTI).

A.2 UE initiated UL transmission

Assume UE is in connected/synchronized mode and wants to do UL transmission, e.g., to send TCP ACK. Following table shows the steps and their corresponding contribution to the UL transmission latency.

Table A.1. UL transmission latency calculation

| Step | Description | Delay |
| --- | --- | --- |
| 1. | Average delay to next SR opportunity | SR periodicity/2 |
| 2. | UE sends SR | 1 TTI |
| 3. | eNB decodes SR and generates scheduling grant | ∆eNB |
| 4. | Transmission of scheduling grant (assumed always error free) | 1 TTI |
| 5. | UE processing delay (decoding Scheduling grant + L1 encoding of data) | ∆UE |
| 6. | UE sends UL transmission | 1TTI + p * (∆eNB + ∆UE + 2TTI) where p is initial BLER. |
| 7. | eNB receives and decodes the UL data | z.∆eNB |

In the table above, steps 1-4 and half delay of step 5 is assumed to be due to SR, and rest is assumed for UL data transmission in values shown in Table x6-4.

## Annex <A1.x2> Simulation x2 [6]

### A1.x2.1 Baseline L1 overhead calculation

We assume 20% L1 overhead for baseline case, which is calculated as following:

- TTI = 14 OFDM symbol

  - control overhead: 2 PDCCH OFDM symbols (24 REs)

  - RS Overhead: 2 CRS antenna ports (additional 12 REs)

Total number of REs per PRB: 14*12 = 168

Overhead: 36/168 ≈ 21%

### A1.x2.1TBS calculation

L1 overhead (i.e. L1 control channel and RS) is considered as 10%, 20%, 30% and 50%, which is reflected into the TBS calculation. To unify the TBS calculation, we also use the same TBS calculation for the base line case (i.e. 1ms TTI), and the 20% L1 overhead is assumed.

TBS is calculated by following formula based on efficiency in table below (R1-081638).

- TBS = PRB_Num * RE_per_PRB * efficiency * L1_Overhead

Where,

- PRB_Num is available PRB number we set in simulation;

- RE_per_PRB is the total RE number in a PRB, e.g., for 14 OFDM Symbol TTI, RE_per_PRB is 12*14 = 168, and for 7 OFDM symbol TTI, RE_per_PRB is 12*7 = 84;

- Efficiency is for calculating the TBS table in TS 36.213, and efficiency = code rate * modulation level;

- L1_Overhead denotes the L1 signaling and RS overhead.

Note: here we didn’t consider the CRC impact.

| MCS Index | modulation | coding rate x 1024 | efficiency | Comments | Code Rate |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 | from CQI table | 0.1171875 |
| 1 | 2 | 157 | 0.3057 | Average Efficiency | 0.15332031 |
| 2 | 2 | 193 | 0.377 | from CQI table | 0.18847656 |
| 3 | 2 | 251 | 0.4893 | Average Efficiency | 0.24511719 |
| 4 | 2 | 308 | 0.6016 | from CQI table | 0.30078125 |
| 5 | 2 | 379 | 0.7393 | Average Efficiency | 0.37011719 |
| 6 | 2 | 449 | 0.877 | from CQI table | 0.43847656 |
| 7 | 2 | 526 | 1.0264 | Average Efficiency | 0.51367188 |
| 8 | 2 | 602 | 1.1758 | from CQI table | 0.58789063 |
| 9 | 2 | 679 | 1.3262 | Average Efficiency | 0.66308594 |
| 10 | 4 | 340 | 1.3262 | overlap | 0.33203125 |
| 11 | 4 | 378 | 1.4766 | from CQI table | 0.36914063 |
| 12 | 4 | 434 | 1.69535 | Average Efficiency | 0.42382813 |
| 13 | 4 | 490 | 1.9141 | from CQI table | 0.47851563 |
| 14 | 4 | 553 | 2.1602 | Average Efficiency | 0.54003906 |
| 15 | 4 | 616 | 2.4063 | from CQI table | 0.6015625 |
| 16 | 4 | 658 | 2.5684 | Average Efficiency | 0.64257813 |
| 17 | 6 | 438 | 2.5684 | overlap | 0.42773438 |
| 18 | 6 | 466 | 2.7305 | from CQI table | 0.45507813 |
| 19 | 6 | 517 | 3.0264 | Average Efficiency | 0.50488281 |
| 20 | 6 | 567 | 3.3223 | from CQI table | 0.55371094 |
| 21 | 6 | 616 | 3.6123 | Average Efficiency | 0.6015625 |
| 22 | 6 | 666 | 3.9023 | from CQI table | 0.65039063 |
| 23 | 6 | 719 | 4.21285 | Average Efficiency | 0.70214844 |
| 24 | 6 | 772 | 4.5234 | from CQI table | 0.75390625 |
| 25 | 6 | 822 | 4.8193 | Average Efficiency | 0.80273438 |
| 26 | 6 | 873 | 5.1152 | from CQI table | 0.85253906 |
| 27 | 6 | 910 | 5.33495 | Average Efficiency | 0.88867188 |
| 28 | 6 | 948 | 5.5547 | from CQI table | 0.92578125 |

### A1.x2.1 UL latency components

Normally, we assume that the UL access latency is stated in following tables:

Normal D-SR UL Latency component

| Component | Description | Time |
| --- | --- | --- |
| 1 | Average waiting time for PUCCH (5 TTI SR period) | 2.5 * TTI |
| 2 | UE sends Scheduling Request (SR) on PUCCH | 1 * TTI |
| 3 | eNB decodes Scheduling Request and generates the Scheduling Grant | 3 * TTI |
| 4 | Transmission of Scheduling Grant | 1 * TTI |
| 5 | UE Processing Delay (decoding of grant + L1 encoding of UL data) | 3 * TTI |
| 6 | Transmission of UL data | 1 * TTI |
| 7 | Data decoding and processing in eNodeB | 3 * TTI |
| 8 | Backhaul(CN/Internet) transmission (to App server) | 5ms |
| Total |  | 14.5TTI+5ms |

Short D-SR UL Latency component

| Component | Description | Time |
| --- | --- | --- |
| 1 | Average waiting time for PUCCH (1 TTI SR period) | 0.5 * TTI |
| 2 | UE sends Scheduling Request (SR) on PUCCH | 1 * TTI |
| 3 | eNB decodes Scheduling Request and generates the Scheduling Grant | 3 * TTI |
| 4 | Transmission of Scheduling Grant | 1 * TTI |
| 5 | UE Processing Delay (decoding of grant + L1 encoding of UL data) | 3 * TTI |
| 6 | Transmission of UL data | 1 * TTI |
| 7 | Data decoding and processing in eNodeB | 3 * TTI |
| 8 | Backhaul(CN/Internet) transmission (to App server) | 5ms |
| Total |  | 12.5TTI+5ms |

Pre-allocation UL Latency component

| Component | Description | Time |
| --- | --- | --- |
| 1 | Average waiting time for PDCCH | 0.5 * TTI |
| 2 | UE reads Resource Blocks on PDCCH | 1 * TTI |
| 3 | UE Processing Delay (decoding of grant + L1 encoding of UL data) | 3 * TTI |
| 4 | Transmission of UL data | 1 * TTI |
| 5 | Data decoding and processing in eNodeB | 3 * TTI |
| 6 | Backhaul(CN/Internet) transmission (to App server) | 5ms |
| Total |  | 8.5TTI+5ms |

--- END Text Proposal ---