3GPP TSG-RAN WG2 #97bis	Tdoc R2-1702754

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.1.5

Source:	Ericsson

Title:	Semi Persistent Scheduling in NR

Document for:	Discussion, Decision

# Introduction

The “autonomous/grant-free/contention based” UL transmission for NR was discussed first in RAN1. In RAN2#97, the following was agreed for UL URLLC scheduling:

- NR supports an SPS scheme similar to LTE - NR supports skipping UL grant scheme similar to LTE

This contribution discusses RAN2 aspects concerning Semi Persistent Scheduling UL transmission, also with considerations from RAN1 agreements. In RAN1#87 the following was agreed for UL URLLC scheduling:

At least an UL transmission scheme without grant is supported for URLLC - Resource may or may not be shared among one or more users - FFS: resource configuration details - FFS other details of design

In RAN1#87ah, the following was agreed for UL URLLC scheduling:

• For an UL transmission scheme without grant • at least semi-static resource (re-)configuration is supported − FFS: The resource configuration includes at least physical resource in time and frequency domain and RS parameters − Higher-layer signaling could be similar to Rel-8 LTE SPS − FFS: MCS • RS is transmitted together with data − channel structure of grant-based data transmission can be starting point

In RAN1#88, the following was agreed for UL URLLC scheduling:

- For UL transmission without grant, - The resource configuration includes at least the following - Time and frequency resources, FFS: including resources for repetitions, implicitly or explicitly - Modulation and coding scheme(s), possibly including RV, implicitly or explicitly - Reference signal parameters - FFS: Details - FFS: The number of repetitions K. Whether multiple number of K can be configured to one UE - FFS other parameters

In this contribution, we do not discuss aspects related with high-reliability, e.g., repetitions. HARQ handling is treated in [4] and a RAN1 discussion on SPS is given in [3].

# Discussion

## Resource configuration

Some of the parameters that need to be configured for semi-persistent scheduling in LTE are periodicity, address (e.g., SPS-C-RNTI), allocation and MCS to be used in the SPS. Part of these parameters (periodicity, address) are configured by RRC (SPS Configuration), and the rest are configured by PDCCH (SPS Activation). This allows for flexibility of the resource allocation and link adaptation, since a new configuration message can be sent in any PDCCH. It can be assumed that in general a similar configuration is used for NR.

Like in legacy LTE, SPS period and address are configured by RRC.

Like in legacy LTE, frequency resources, MCS, etc., for SPS are semi-statically configured by PDCCH.

It has been agreed that UE does not transmit on a grant-free resource when it has an empty buffer to reduce unnecessary interference and battery consumption. This behaviour should further extend to all granted resources, which is useful especially in a pre-scheduling scenario when the network does not always know if the UE has data.

Like in legacy LTE, UE can be configured by higher layer whether or not to transmit in a SPS grant or dynamic grants when it has an empty buffer, i.e., SkipUplink.

Since SkipUplink can be configurable by the gNB, then this configuration has to be acknowledged by the UE, to avoid the error case where gNB believes that UE already has a grant while UE missed it.

Like in legacy LTE, when SkipUplink is configured, UE should send an acknowledgement of the configured resource.

In legacy (Rel-13) LTE-SPS, periodic messages, such as CSI, BSR and PHR, are defined to be transmitted. This does not come with many additional costs since anyhow a padding is transmitted when the buffer is empty. However, in Rel-14, with the introduction of “SkipUplink”, the behaviours of these periodic messages needed to be re-defined. It has been agreed in LTE that CSI and PHR can still be transmitted periodically for link adaption. But BSR is not transmitted periodically on SPS-resources when skipUplink is configured. It can be assumed that in general a similar configuration is used for NR.

When SkipUplink is configured, periodic transmissions should follow similar operations like in LTE.

Going further, DRX behaviour in connection to SPS and configurable “SkipUplink” parameter needs to be clarified. Here, the LTE solution can be reused; a DRX timer should only start or be reset after an active UL or DL transmission.

Like in legacy LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used.

For proper network control, it is beneficial that an explicit deactivation mechanism is needed. Introducing also implicit mechanisms can further improve the resource utilization by returning unused resources to the scheduler. A timer-based deactivation could also simplify scheduling.

Both explicit and implicit deactivation of the SPS resource should be available.

## Contention-based UL access

SPS builds on the pre-allocation of resources to UEs. With large bandwidth allocations (for robust MCS) or a higher number of UEs to schedule, the UL spectrum becomes a limiting factor in the scheduler. If resources can be shared by multiple UEs, the UL capacity increases. Therefore, SPS should be configurable, for example by RRC configuration to support contention-based UL access. Our detailed views on contention-based UL access are in [5].

SPS should be configurable to support contention-based UL access.

# Conclusion

Based on the discussion in Section 2 we propose the following:

- Like in legacy LTE, SPS period and address are configured by RRC.

Like in legacy LTE, frequency resources, MCS, etc., for SPS are semi-statically configured by PDCCH.

Like in legacy LTE, UE can be configured by higher layer whether or not to transmit in a SPS grant or dynamic grants when it has an empty buffer, i.e., SkipUplink.

Like in legacy LTE, when SkipUplink is configured, UE should send an acknowledgement of the configured resource.

When SkipUplink is configured, periodic transmissions should follow similar operations like in LTE.

Like in legacy LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used.

Both explicit and implicit deactivation of the SPS resource should be available.

SPS should be configurable to support contention-based UL access.

# References

TR 36.881, Study on latency reduction techniques for LTE, v0.6.0

RP-160667, WI L2 latency reduction techniques for LTE.

R1-1702754, Semi-persistent scheduling in NR, Ericsson

R2-1702666, HARQ handling for SPS UL, Ericsson

R2-1702747, aspects for contention based UL transmission in NR, Ericsson