3GPP TSG-RAN2 Meeting #98	 R2-1704061

Hangzhou, China, 15th - 19th May 2017

Agenda Item:	10.3.1.3

Source: 	OPPO

Title:  	Discussion on MAC sub-header

Document for:	Discussion and Decision

- Introduction

In NR SI stage, the following agreement was reached on MAC sublayer. In this contribution, we further discuss the sub-header enhancement to support multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks and provide our proposals.

Agreements

1	MAC sub-headers are interleaved with MAC SDUs.

2	Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;

- Discussion

The content of MAC sub-header

When multiple MAC SDUs are multiplexed into one MAC PDU, sub-header is used to indicate the corresponding MAC SDU or MAC CE characteristics. The logical channel index field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element. The length field indicates the length of the corresponding MAC SDU or variable-sized MAC control element in bytes. Therefore, at least the logical channel index and the length field are included in the MAC sub-header.

Proposal1	At least the logical channel index and the length field are included in the MAC sub-header.

As RAN2 has already agreed that MAC sub-hearders are inerleaved with MAC SDUs. The most straight-forward way is to put one sub-header in front of each MAC SDU or MAC CE, and each sub-header only indicates the characteristics of the subsequent MAC SDU or MAC CE. The general MAC PDU can be illustrated in the following Figure 1.

Figure 1: The LCID and Length field for each MAC SDU

If taking LTE as an example, the LCID field is 4bits, and the Length field is 7bits or 15 bits. If putting one sub-header with LCID and length field for each MAC SDU, the header overhead is very large compared to some small volume of payload.

Observation 1	The sub-header with LCID and length fields of each MAC SDU brings much overhead.

The optimization of the length field

In traffic transmission, there are some specific services in which the payload sizes are same during a time of period. For the transmission of this type service, the length field is not needed for each MAC SDU. Also there are some services in which the size of the payload is fixed and the length can be semi-stasticlly configured. Besides, the transitter may segment one larger payload into multiple small payloads due to the length limitation.

Observation 2	The length field is not needed to be present in the MAC sub-header of every MAC SDU in some case, e.g. fixed payload size, same payload size for a period, and multiple segments into one MAC PDU.

For the services which has the characteristic of fixed payload size, the length can be calculated and semi-statically configured per bearer or per logical channel by RRC signalling.

Proposal 2	 The length can be semi-statically configured per bearer or per logical channel by RRC signalling.

For the case that same payload size for a period, in order to further reduce the overhead, sub-header optimisation can be considered. Some optimised methods are illustrated in Figure 2-1, Figure 2-2, Figure 3-1, Figure 3-2.

Figure 2-1: Alternative 1-1 Optimised length field with a differentiation flag

Figure 2-2: Alternative 1-2 Optimised length field with SDU number

Two optimization methods tarwards the Length field are illustrated in Figure 2-1 and Figure 2-2, in which one method is intended to indicate wheher the length of the subsequent MAC SDU is the same as the previous one and the other method is to provide the number of the consecutive MAC SDU with the same length characteristic. In detial, for alternative 1-1 in Figure 2-1, there is a flag in MAC sub-header, if the flag is set to “0”, the length field is included in the sub-header,and if set to “1”, the length field is absent and considered as the same as the previous MAC SDU. For alternative 1-2 in Figure 2-2, there is Number field in each sub-header and this field is used to indicate how many SDUs followling the sub-header has the same length.

Proposal 3	Considering the two following optimization methods for the length filed in MAC-header.

- Alt1:	Add a differentiation flag to indicate whether the length of the following MAC SDU is same as the previous one.

- Alt2:	Add a Number field in sub-header to indicate how many SDUs followling the sub-header has the length present in the sub-header.

The optimization of the LCID field

Similar with the length field, the data transmitted during a time of period may belong to the same LCID. For this case, the LCID field can be absent in some MAC sub-header.

Observation 3	The LCID field can be absent in MAC sub-header if the data transmitted during a time of period may belong to the same LCID.

The detailed optimization can be illustrated in Figure 3-1 and Figure 3-2. Please note that both of the length optimization and the LCID optimization are illustrated in the two figures.

Figure 3-1: Alternative 2-1 Optimised Length and LCID field with a differentiation flag

Figure 3-2: Alternative 2-2 Optimized Length and LCID field with SDU number

The methods of alternative 2-1 and alternative 2-2 further optimize the LCID field of the sub-header based on alternative 1. In alternative 2-1, there two flags in each sub-header, one is used to indicate whether the subsequent SDU belongs to the same LCID as the previous one and the other flag is used to indicate whether the the length of the subsequent MAC SDU is the same as the previous one. If the correspondign flag is set to “0”, the LCID/ Length field is different with the previous one and the new LCID/ Length field value is included in the sub-header; otherwise, the LCID/Length field is absent. In alternative 2-2, there two flags in sub-header which has the same definition as those in alternative 2-1; besides, there is Nubmer field in the sub-header to indicate how many SDUs following the sub-header have the same sub-header information, which avoids putting an sub-header for each MAC SDU.

Proposal 4	Considering the two following optimization methods for the LCID filed in MAC-header.

- Alt1:	Add a differentiation flag to indicate whether the LCID of the following MAC SDU is same as the previous one.

- Alt2:	Add a Number field in sub-header to indicate how many SDUs followling the sub-header belong to the LCID present in the sub-header.

Conclusion

In this contribution, we discuss the MAC sub-header and provide our observations.

Observation 1	The sub-header with LCID and length fields of each MAC SDU brings much overhead.

Observation 2	The length field is not needed to be present in the MAC sub-header of every MAC SDU in some case, e.g. fixed payload size, same payload size for a period, and multiple segments into one MAC PDU.

Observation 3	The LCID field can be absent in MAC sub-header if the data transmitted during a time of period may belong to the same LCID.

Based on the observations, we provide the following proposals.

Proposal1	At least the logical channel index and the length field are included in the MAC sub-header.

Proposal 2	 The length can be semi-statically configured per bearer or per logical channel by RRC signalling.

Proposal 3	Considering the two following optimization methods for the length filed in MAC-header.

- Alt1:	Add a differentiation flag to indicate whether the length of the following MAC SDU is same as the previous one.

- Alt2:	Add a Number field in sub-header to indicate how many SDUs followling the sub-header has the length present in the sub-header.

Proposal 4	Considering the two following optimization methods for the LCID filed in MAC-header.

- Alt1:	Add a differentiation flag to indicate whether the LCID of the following MAC SDU is same as the previous one.

- Alt2:	Add a Number field in sub-header to indicate how many SDUs followling the sub-header belong to the LCID present in the sub-header.

Reference

[1]	TS 38.804 ve00.