3GPP TSG-RAN WG2 Meeting #98						R2-1705104

Hangzhou, China, 15th – 19th May 2017

Agenda item: 10.3.2.5

Source: National Instruments

Title:	NR RLC UM Receive Procedure

Document for: Discussion & Decision

# Introduction

The 5G study item regarding the “Study on New Radio Access Technology” was approved at the 3GPP TSG RAN#71 meeting [1]. The technical report [2] contains the radio interface protocol aspects discussed and agreed during the study item. For RLC receive procedure, a main change from LTE is that in-order delivery to PDCP is not needed. RLC reassembles RLC SDUs and delivers them to the higher layer in the order they are received. The technical report listed the following item as FFS.

- It is FFS whether RLC UM needs to support t-reordering like functionality for the purpose of moving the lower edge of the receive window, or for other purposes, which could be discussed in the stage-3 work.

In RAN2#97bis [3], the following agreements regarding RLC UM receive procedure were reached:

- In NR, RLC UM has no transmit window

- RLC AM/UM receiver does not store complete RLC SDUs, just RLC SDU segments

- If a segment is detected to be missing, then all stored segments associated to the RLC SDU can be discarded.  FFS how a missing segment is detected if a timer mechanism is used (e.g T-reassembly)

- Duplicate detection functionality is kept as a baseline. FFS if duplicate detection can be removed

- RLC UM receive window operation is maintained similar to LTE.  If duplicate detection is removed from RLC UM then the need for the window will depend on the mechanism used to discard.

# Discussion

## LTE RLC UM Window Operation

As shown in Figure 1, in LTE, the RLC UM receive window operates based on the following state variables:

- VR(UH) which holds the value of the highest received SN + 1. The receive window is defined by VR(UH) – UM_Window_Size ≤ SN < VR(UH).

- VR(UR) which holds the value of the highest in-sequence delivered SN + 1. Any completely received SN below this value has been delivered to the higher layer in ascending order of SN.

- VR(UX) which holds the value of the SN following the SN that set the t-reordering timer.

The receive window is a “pull-type window” in RLC UM since the window is pulled by the upper edge whenever an SN ≥ VR(UH) is received. If an SN gap develops, i.e. VR(UR) < VR(UH), the t-reordering timer starts and the value of VR(UX) is set to VR(UH). The timer resets if all the missing RLC PDUs are received, i.e. VR(UX) ≤ VR(UR). The timer is started again if the SN gap still exists.

If the t-reordering timer expires, it means that the lower layer HARQ functionality will be unable to recover the missing PDUs and the receiver gives up on any remaining missing RLC PDUs with SN < VR(UX). The VR(UR) is updated based on the next SN ≥ VR(UX) that is yet to be fully received. Any PDUs with SN lower than that will be forwarded to the upper layer in ascending order of SN.

Hence, the concept of t-reordering timer is used in LTE RLC UM receive procedure to detect lower layer packet failures and avoid excessing reordering delays.

Observation 1: T-reordering timer functionality is used in LTE RLC UM receive procedure to avoid excessive reordering delays.

Figure 1: RLC UM Receive Window Operation in LTE

## NR RLC UM Window Operation

In NR, RLC no longer needs to reorder the SDUs before delivery to the higher layer. Hence, t-reordering timer and the state variables VR(UR) and VR(UX) in the receive window are no longer strictly needed. Any RLC SDUs that are fully reassembled (or not segmented) can be immediately provided to the higher layer. Buffering is still required in RLC for SDU segments waiting for reassembly. Any packets that cannot be reassembled will be discarded as the receive window moves.

However, relying just on the window movement may lead in some cases to such segments being in the buffer for too long beyond the time within which the lower layer HARQ can potentially recover the missing segments. In RAN2#97bis, a t-reassembly timer was introduced as FFS as a possible time-bound mechanism for discarding segments in addition to the SN-based mechanism of the lower edge of the receive window. This paper agrees with adding this technique.

If a t-reassembly timer is introduced, it can function like the t-reordering timer in LTE. For example, Figure 2 illustrates an example operation largely repurposing the window operation in LTE. In this case, the timer is set when there are RLC SDU segments awaiting reassembly and VR(UX) is set to VR(UH). Timer is reset if all the segments with SN < VR(UX) are fully assembled and forwarded to higher layer. If that does not happen and timer expires, all SN < VR(UX) awaiting segments for reassembly are discarded. VR(UR) indicates the earliest SN that is still considered for forwarding to higher layers by RLC just as in LTE. The procedure to update state variables such as VR(UR) could be different from LTE. Note that partially received SDUs are also discarded when receiver window’s lower edge moves past them.

Proposal 1: A receive window and a t-reassembly timer is used to discard RLC SDU segments in RLC UM receiver.

Figure 2: Example Receive Window Operation in NR RLC UM

## Duplicate detection

In LTE, the RLC UM detects and discards duplicates. In NR, RLC no longer needs to do in-sequence delivery and this functionality is handled by higher layers. Similarly, duplicate detection can also be handled by higher layers in combination with the reordering operation. This avoids multiple layers implementing similar functionality. However, RLC will still need to do duplicate detection in the case of RLC SDU segments waiting in the buffer for completion.

Proposal 2: Duplicate detection in the RLC UM receiver is limited to RLC SDU segments that are waiting for completion.

# Conclusion

In this paper, we discussed the operation of RLC UM receiver and make the following observation and proposals:

Observation 1: T-reordering timer functionality is used in LTE RLC UM receive procedure to avoid excessive reordering delays.

Proposal 1: A receive window and a t-reassembly timer is used to discard RLC SDU segments in RLC UM receiver.

Proposal 2: Duplicate detection in the RLC UM receiver is limited to RLC SDU segments that are waiting for completion.

# References

[1] RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DoCoMo

[2] 3GPP TR 38.804 V1.0.0, “Study on New Radio Access Technology: Radio Interface Protocol Aspects”

[3] 3GPP, Report of 3GPP TSG RAN WG2 meeting #97bis