3GPP TSG- RAN2 Meeting #99			     R2-1709086

Berlin, Germany, 21st – 25th August, 2017

Agenda item:	   10.3.4.3 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 		QoS handling in Handover

Document for:	Discussion and Decision

1.	Introduction

This document discusses some issues regarding QoS handling in handover.

2.	Discussion

gNB controls the mapping of QoS Flow to DRB in two different ways: reflective mapping and explicit configuration. At the AH#2 meeting, RAN2 made agreement that a UE follows the latest QoS flow to DRB mapping information regardless of whether it was explicit RRC or AS reflective QoS. In handover, the mapping information needs to be sent to target gNB so that the target gNB can prepare resources and avoid unnecessary DRB remapping.

Proposal 1:	At handover, source gNB should provide the latest QoS flow to DRB mapping information, which is followed by UE, to target gNB.

After receiving the mapping information from the source gNB, the target gNB may decide to remap some QoS flow(s) and then command the UE to a new DRB mapping for the QoS flow(s) through the source gNB. In that case, the UE needs to update DRB mapping for the QoS flow(s), but maintain DRB mapping for the rest QoS flow(s). In addition, the target gNB may reject the request for some DRB(s). If the rejection is notified, the source gNB releases the DRB(s). In that case, the UE needs to release DRB mapping for all QoS flow(s) which is associated with the DRB(s) to be released unless there is the new DRB mapping for the QoS flow(s). Also, the UE should not transmit UL packet for the released QoS flow(s) to the target gNB.

Proposal 2:	When receiving the release request for a DRB, UE releases the DRB mapping for each QoS flow which is mapped to the DRB except when there is a new DRB mapping for the QoS flow(s).

Proposal 3:	UE should not transmit UL packet for the released QoS flow(s) to the target gNB.

Proposal 4:	At handover, UE should maintain the latest QoS flow to DRB mapping information acquired from source gNB unless the information is updated and/or released by target or source gNB.

3.	Proposal

In this document, we present our view on QoS handling in handover. We have following proposals:

Proposal 1:	At handover, source gNB should provide the latest QoS flow to DRB mapping information, which is followed by UE, to target gNB.

Proposal 2:	When receiving the release request for a DRB, UE releases the DRB mapping for each QoS flow which is mapped to the DRB except when there is a new DRB mapping for the QoS flow(s).

Proposal 3:	UE should not transmit UL packet for the released QoS flow(s) to the target gNB.

Proposal 4:	At handover, UE should maintain the latest QoS flow to DRB mapping information acquired from source gNB unless the information is updated and/or released by target or source gNB.