3GPP TSG-RAN WG2 # 99	R2-1709447

Berlin Germany, 21-25th August 2017

Agenda Item:	8.12

Source: GEMALTO N.V., Sequans, Sierra Wireless S.A., Qualcomm, Intel, Ericsson

Title:  Release Assistance Indication for CAT-M

Document for:	Discussion and decision

# Introduction

During the RAN WG2 #96 USA, it was agreed to introduce Release assistance indication for CAT-NB in Rel-14. Following was agreed:

We introduce an AS Release assistance indication in Rel-14

During RAN 2#98 Hangzhou China, We brought a contribution to introduce Release Assistance Indication for CAT-M similar to CAT-NB. Following Agreement was recorded in Chair Notes.

- Chair: There seems to be sufficient interest to go forward, but it is not clear the complexity of the prohibit timer. Cannot agree now.

- Could be a candidate for TEI14 or Rel-15

We would like to discuss the possibility of Introducing AS RAI for Cat-M technology as well.

# Discussion

During the RAN WG2 #97 bis Spokane USA, some proposals were presented related to introducing a prohibit timer for Access Stratum Release Assistance Indication (RAI) and introducing the AS RAI feature for eMTC UEs (i.e. UE category M1 or M2) as well. There was a significant support for the proposals and some resistance as well. It was mentioned by some companies that since we already have the Power preference indicator (PPI) Feature, we do not need AS RAI for eMTC UEs (i.e. UE category M1 or M2).

During the RAN WG2#98 meeting in Hangzhou China, we brought the contribution again shedding light on differences between introduction of Release Assistance Indication(RAI) for CAT – M for signaling efficiency as compared to PPI. Many companies agreed on the point that we can introduce this for CAT-M as well, on the same lines as we introduced it for CAT-NB i.e. without a prohibit timer.

The sole reason to introduce AS RAI in NB-IoT was to reduce the signaling and the power consumption. A solution which can send the UE to idle mode quickly if it does not have more data to send and is ideal for power saving. Like CAT-NB, this solution is based on MAC BSR reporting i.e. BSR=0 indicates that UE does not have any more data to send or receive in the near future (e.g. 10s of seconds).

One more important point which should be mentioned here is, Non Access Stratum Release assistance indication (NAS-RAI) is available for the CP solution in a similar way for both CAT-NB and CAT-M, but from the perspective of deployment, it is expected that CAT-NB will mainly use the Control Plane (CP) solution. On other hand, CAT-M is expected to mainly use the User plane (UP) solution and for the UP solution, NAS-RAI procedure is not supported and UE has to wait for the NW inactivity timer to expire before releasing the connection.

One more observation is, since many of the applications are similar for eMTC UEs (i.e. UE category M1 or M2) and CAT-NB and it is up to the tech operator to decide which technology to use then there should not be an artificial power disadvantage of using one technology over the other (e.g. is similar/same solution works for both technologies then it should be prevented for non-technical reasons). In the end there is no mandatory requirement on operators to use these optimizations. RAN2 should converge to have same/similar feature to align both technologies to take advantage of power efficiency feature(s).

Observation: Cat-M UEs (i.e. UE category M1 or M2) and CAT – NB support over-lapping use cases and it is up to operator to decide which tech to use. So we think there is no reason that we cannot have same power optimisation solution for both technologies.

Given that Cat-M and Cat-NB have over-lapping use cases and any effort to introduce power optimization should be considered for both Cat-M and Cat-NB. In most of the scenarios, Cat-M and CAT-NB will be deployed together so it makes sense to have the same mechanism for both. As it was agreed to introduce the AS RAI for NB-IoT, it would be extremely beneficial for CAT-M as well.

We would like to propose that RAN 2, to introduce AS RAI for eMTC UEs (i.e. UE category M1 or M2) as well.

Proposal: Introduce AS RAI procedure for UE category M1 and M2 in REL-14.

# Conclusions

RAN2 is kindly asked to discuss following additions to AS RAI procedure

Observation: Cat-M UEs (i.e. UE category M1 or M2) and CAT – NB support over-lapping use cases and it is up to operator to decide which tech to use, so we think there is no reason that we cannot have same power optimisation solution for both technologies.

Proposal: Introduce AS RAI procedure for UE category M1 and M2 in REL-14.

# Reference

- R2-1703048 Ericsson Paper

- R2-168320 Ericsson Paper

- Chair Notes IOT RAN 2 # 96 USA

- Chair notes IOT RAN 2 #97 Athens

- Chair Notes IOT RAN 2 #97 bis. Spokane USA

- Chair Notes IOT RAN 2 # 98 Hangzhou