3GPP TSG-RAN WG2 Meeting #99-Bis	R2-1711159

Prague, Czech Republic, 09th – 13th October, 2017	Revision of R2-1709305

Agenda item:		9.14.2 (Early data transmission)

Source:	LG Electronics Inc.

Title: 	Early data transmission for Control plane CIoT optimisation

Document for:	Discussion and Decision

1. Introduction

One of the objectives in this WI is to evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure after NPRACH transmission and before the RRC connection setup is completed.

At the RAN2#99 meeting, RAN2 made following agreements.

- We intend to support early UL data transmission in Msg3 for control plane and user plane CIoT EPS optimisation. - We intend to support early DL data transmission in Msg4 for control plane and user plane CIoT EPS optimisation.

In this contribution, we discuss impacts to support early data transmission for Control plane CIoT EPS optimisation.

Compared to the previous submission (R2-1709305) for RAN2#99 meeting, Proposals 4 ~ 6 have been added in this document to clarify our views and overall description has been changed based on previous agreements and discussion.

2. Discussion

For NB-IOT UEs, saving UE power is one of major concerns. Considering the data traffic pattern of NB-IOT, Control plane CIoT EPS optimization was introduced such that the UE is allowed to transmit a NAS message containing Control plane (CP) data in MSG5 of the RRC Connection Establishment procedure as soon as the UE enters RRC_CONNECTED.

For early data transmission, the UE would need to transmit Control Plane data in MSG3 while performing the RRC Connection Establishment procedure. It is likely that both the eNB and the UE shall support the new functionality. The UE should perform early data transmission only at a cell where this functionality is supported. That is, if the UE supporting early data transmission receives a system information message indicating that early data transmission is supported at a cell, the UE can operate with early data transmission functionality.

Proposal 1. If early data transmission is supported for Control plane CIoT EPS optimisation, eNB should indicate whether early data transmission is allowed or not at a cell for Control plane CIoT EPS optimisation.

In legacy operation, the UE transmits control plane data in MSG5 in the CONNECTED state. Operating with early data transmission, there are two options as follows:

Option 1) The UE transmits a single RRC message containing RRCConnectionRequest and CP data.

Option 2) The UE transmits separate RRC messages for RRCConnectionRequest and CP data.

In Option 1, CP data is contained in the RRCConnectionRequest message. Thus, the RRCConnectionRequest message including CP data is transmitted as a single RRC message. In Option 2, CP data is transmitted in a separate RRC message from the RRCConnectionRequest message. In this case, a legacy RRC message such as UL Information Transfer or newly defined RRC message can be used to transfer CP data.

In case of Option 1, early transmission of CP data is incorporated into the RRC Connection Establishment procedure. Thus, failure of early transmission of CP data would lead to failure of the RRC Connection Establishment procedure. Meanwhile, in case of Option 2, the RRC connection establishment procedure operates independently from CP data transmission procedure. CP data transmission failure does not mean that the RRC connection establishment procedure fails. If CP data transmission fails and the UE receives the RRCConnectionSetup message, the UE can re-transmit CP data in MSG5 or after successful RRC connection establishment procedure.

We think that Option 2 is better than Option 1. Failure of early transmission of CP data should not lead to failure of the RRC Connection Establishment procedure. If early transmission of CP data fails the UE should be able to re-transmit CP data in MSG5 or after successful RRC connection establishment procedure. Thus, we propose to use a separate RRC message carrying CP data for Control plane CIoT EPS optimisation.

Proposal 2: If early data transmission is supported for Control plane CIoT EPS optimisation, the RRCConnectionRequest message and the NAS message including CP data are transmitted as separate RRC messages.

According to 36.331, the RRCConnectionSetup message is currently used to establish SRB1. When the UE sends the RRCConnectionRequest message, SRB1 is not established yet. Then, upon receiving the RRCConnectionSetup message, the UE establishes SRB1 and transmits RRCConnectionSetupComplete message over SRB1. Thus, if the UE transmits CP data before receiving the RRCConnectionSetup message, the UE should currently use SRB0 i.e. CCCH to transmit early CP data. However, all CP data is currently transmitted over DCCH with RLC AM. Since CCCH does not support RLC AM, it is not likely to transmit CP data over the existing CCCH.

Observation 1: If CP data is transmitted before reception of the RRCConnectionSetup message for Control plane CIoT EPS optimisation, SRB1 cannot be used to carry CP data because SRB1 is not established yet.

Observation 2: The existing CCCH with RLC TM cannot be used to transmit CP data which is currently transmitted with RLC AM.

Accordingly, we think that it is beneficial to define a new Signalling Radio Bearer for carrying CP data before reception of the RRCConnectionSetup message.

In addition, RRCConnectionRequest should have a higher priority than CP data during the RRC Connection Establishment procedure. If UL grant cannot accommodate both RRCConnectionRequest and a message containing CP data, RRCConnectionRequest should be transmitted at first. Hence, the new Signalling Radio Bearer should have a lower priority than the legacy CCCH (i.e. SRB0).

Proposal 3: If CP data is transmitted before reception of the RRCConnectionSetup message for Control plane CIoT EPS optimisation, a new Signalling Radio Bearer with RLC AM is defined to carry CP data.

RAN2 should decide first if early data transmission will be finished after single transmission, if RRC connection will be release after receiving MSG4 and so on. If the early data transmission procedure continues after receiving RRCConnectionSetup or indication of successful RRC connection establishment, we think that the UE should follow legacy state transition rules and transits to the RRC_CONNECTED state.

In cases that a radio condition is not good and the UE may receive a small UL grant, or the UE receives NACK to MSG3, the early data transmission could go long. If the UE does not transits to the RRC_CONNECTED state whenever the UE transmits a MAC PDU, the UE should perform random access attempts with a long UE ID and user data in the MAC PDU, which is inefficient UE operation repeating RRC Connection Setup-like procedure for transmission of each MAC PDU.

Also, early data transmission would have impact on the NAS layer. If the UE NAS layer detects UL data, it will trigger RRC Connection Establishment procedure and the Access control procedure before sending the Control Plane Service Request message. Such operation should also be affected if the UE continuously remains in the RRC_IDLE state for early data transmission.

Proposal 4. The UE follows legacy state transition rules unless the RRC connection is released upon reception of MSG4 indicating successful RRC connection establishment.

Firstly, if early data transmission is successful, MSG5 can be omitted assuming that both the UE and the eNB agreed to perform the quick release procedure. RAN2 can discuss the details in the RRC Connection Release Enhancement item. Secondly, if the RRC connection establishment is successful and user data transmission fails (i.e. when the RRC signalling message and the RRC message with user data are separately transmitted), MSG5 can be omitted if the RRC layer does not care about data retransmission. Lastly, if the RRC connection establishment fails, i.e. the eNB sends RRCConnectionReject or the UE does not receives MSG4, the UE performs legacy operation.

Proposal 4. In a release timer (i.e. dataInactivityTimer) based manner, the UE can release the RRC connection without MSG5 if there is no data to be transmitted.

The UE triggers the RRC Connection Establishment procedure for both early data transmission and legacy data transmission. If UL grant for early data transmission received in RAR is sufficient, the UE transmits a new RRC message including NAS user data. Otherwise, the UE transmits the legacy RRCConnectionRequest message.

Proposal 6. If the UL grant is not sufficient for early data transmission, the UE RRC layer transmits the legacy RRCConnectionRequest message.

3. Conclusion

In this contribution, the followings are proposed.

Proposal 1: If early data transmission is supported for Control plane CIoT EPS optimisation, eNB should indicate whether early data transmission is allowed or not at a cell for Control plane CIoT EPS optimisation.

Proposal 2: If early data transmission is supported for Control plane CIoT EPS optimisation, the RRCConnectionRequest message and the NAS message including CP data are transmitted as separate RRC messages.

Observation 1: If CP data is transmitted before reception of the RRCConnectionSetup message for Control plane CIoT EPS optimisation, SRB1 cannot be used to carry CP data because SRB1 is not established yet.

Observation 2: The existing CCCH with RLC TM cannot be used to transmit CP data which is currently transmitted with RLC AM.

Proposal 3: If CP data is transmitted before reception of the RRCConnectionSetup message for Control plane CIoT EPS optimisation, a new Signalling Radio Bearer with RLC AM is defined to carry CP data.

Proposal 4. The UE follows legacy state transition rules unless the RRC connection is released upon reception of MSG4 indicating successful RRC connection establishment.

Proposal 5. In a release timer (i.e. dataInactivityTimer) based manner, the UE can release the RRC connection without MSG5 if there is no data to be transmitted.

Proposal 6. If the UL grant is not sufficient for early data transmission, the UE RRC layer transmits the legacy RRCConnectionRequest message.