3GPP TSG-RAN WG2 Meeting #99bis	R2-1711729

Prague, Czech Republic, 9th – 13th October 2017	in revision of R2-1709151

Agenda Item	: 10.3.1.5 (NR_newRAT-Core)

Source	: LG Electronics Inc.

Title	: Multiple SR in NR

Document for	: Discussion and Decision

1.	Introduction

Regarding SR, RAN2 made following agreements:

Agreements in NR AH#2:

- In case multiple SRs are configured, for each LCH, there will be a mapping between LCHs and SR configuration and the mapping should be configured by RRC signalling. FFS if grouping is needed.

- A logical channel can be mapped to none or one SR configuration. FFS if a logical channel can be mapped to more than one SR configuration.

Agreements in RAN2#99:

- One or multiple logical channel(s) are mapped to SR configuration (e.g. not LCG)

- RAN2 understanding is that numerology of the SR transmission need not be the same as the numerology of the LCH which triggered the SR

- For the single-cell case, one single LCH is mapped to none or one SR configuration per BWP.  This agreement is pending confirmation from RAN1 that a single BWP can support multiple SR configurations and understanding of how BWP is switched.

FFS how to handle SR configuration, mapping and transmission for CA case

- sr-ProhibitTimer is independently configured per SR configuration.  Whether a single timer or multiple timers are running at the same time are FFS.

- drs-TransMax is independently configured per SR configuration.  FFS whether SR_COUNTER is maintained for each SR configuration independently

This contribution discusses the detailed SR resource configuration and SR procedure based on these agreements.

2.	Discussion

2.1	Logical channel mapping in CA

It is FFS if a logical channel can be mapped to more than one SR configuration in CA. Basically, it would be sufficient to map one SR configuration to a logical channel assuming that one SR configuration could indicate multiple numerologies or the network can assign an SR configuration indicating a preferred numerology to a logical channel.

In the meanwhile, the motivation of mapping a logical channel to more than one SR configuration seems only to support multiple SR configuration without different numerologies. In RAN2#99, it was claimed that one logical channel needs to be mapped to multiple SR configurations in order to support PUCCH offload case. In LTE R13 CA, PUCCH SCell was introduced to alleviate the PCell PUCCH overload. At that time, although multiple SR resources are configured on PUCCH SCell in addition to PCell, the UE used one SR procedure by aggregating all SR resources because all cells are considered to be of the same/similar characteristics, and hence, there was no big difference in SR resources.

In NR, PUCCH offloading would still need to be supported, but it doesn’t necessarily mean that all logical channels should be mapped to multiple SRs. PUCCH offloading could also be realized by separating SR resources between logical channels. For example, PUCCH group with PCell uses SR resource on PCell while PUCCH group with PUCCH SCell uses SR resource on PUCCH SCell. This is similar to HARQ feedback offloading in LTE R13 CA and still provides offloading gain.

Figure 1 Examples of mapping between logical channel and SR configuration

Proposal 1. In CA, one logical channel is mapped to none or one SR configuration per BWP.

2.2	SR prohibit

It was agreed that sr-ProhibitTimer is independently configured per SR configuration in order to prohibit each SR transmission properly given that the periodicity of each SR configuration would be different. However it is FFS whether a single timer or multiple timers are running at the same time.

Using single timer with multiple SR prohibit timer values would mean that the UE uses SR resource by aggregating all SR resources and selects the sr-ProhibitTimer value depending on the transmitted SR. By using one timer, SR transmission on one SR resource prohibits SR transmission on another SR resource. On the other hand, using multiple timers would mean that the UE independently manages the SR transmission on different SR resources.

Given that SR in NR aims at indicating numerology information of logical channel by using different SR resources, SR transmission on each SR resource carry different information. Therefore, it would make sense that SR transmission on one SR resource shouldn’t prohibit SR transmission on another SR resource. It should be noted that, with the same intention, it was already agreed in LTE R15 sTTI to have separate timer for separate SR configuration.

Proposal 2. Separate sr-ProhibitTimer is configured per SR configuration, and each timer prohibits SR transmission on corresponding SR resource.

2.3	SR counter

In NR, different SR resources would show different SR performance. For example, the performance of sSR might be worse than the performance of normal SR. Therefore, failure of SR transmission on sSR doesn’t necessarily mean that SR transmission on another SR will be unsuccessful. For this reason, it was agreed that dsr-TransMax is independently configured per SR configuration leaving whether SR_COUNTER is maintained for each SR configuration independently as FFS.

In order to count SR transmission on each SR resource properly, it seems essential to have a separate SR_COUNTER while it is unclear how single SR_COUNTER would work with multiple dsr-TransMax.

Proposal 3. Separate SR_COUNTER is configured per SR configuration with its own dsr-TransMax.

In LTE, the UE triggers RA procedure upon reaching the maximum SR counter. However, in NR with Proposal 2 and 3, it may not be necessary to trigger RA procedure upon reaching the maximum SR counter for an SR configuration as long as there is on-going SR transmission on another SR configuration because the UE could receive an UL grant in response to the on-going SR transmission.

Proposal 4. Upon reaching the maximum SR counter for an SR configuration, the UE triggers RA procedure only if there is no on-going SR transmission on another SR configuration.

In LTE, the UE cancels all pending SR when the UE includes a BSR in a MAC PDU or the UL grant can accommodate all pending data. The same rule can be used in NR as well because the BSR will let the gNB to know BS information for all LCG having data to transmit. In other words, the gNB can figure out which numerologies are required based on BSR. Thus, there is no reason for the UE to keep transmitting SR and BSR at the same time by cancelling only the specific SR depending on the numerologies of the received UL grant.

Proposal 5. The UE cancels all pending SR when the UE includes a BSR in a MAC PDU or the UL grant can accommodate all pending data.

2.4	BSR aspects

In differentiating SR configuration, the intention is to indicate which logical channel has triggered the SR. Given that SR is triggered by BSR, we would need to check whether the current BSR trigger condition is sufficient to support this behavior.

Today in LTE, the UE triggers a regular BSR when

UL data, for a logical channel which belongs to a LCG, becomes available for transmission in the RLC entity or in the PDCP entity and either the data belongs to a logical channel with higher priority than the priorities of the logical channels which belong to any LCG and for which data is already available for transmission, or there is no data available for transmission for any of the logical channels which belong to a LCG.

It means that the UE does not trigger a BSR when a data becomes available for a logical channel that has a lower priority than a logical channel that already has data available for transmission, even if those two LCHs are mapped to different SR configurations.

An example is depicted in Figure 2. If UL data becomes available for LCH2 with P2 while there is already UL data for LCH1 with P1, currently the UE doesn’t trigger BSR, accordingly, the UE doesn’t trigger SR. We think the UL data for LCH2 should trigger BSR even if the priority of LCH2 is lower than priority of LCH1. Otherwise, the SR configuration which is mapped to lower priority logical channels, e.g., SR2 in Figure 1, would not be used properly.

Figure 2

One possible way would be that when UL data becomes available for a logical channel and if it is the first data among the logical channels mapped to the same SR configuration, the UE triggers a regular BSR. If there is no UL resource for transmitting the regular BSR, the UE triggers a SR and transmits the SR on the associated SR configuration.

Proposal 6. In order to support SR differentiation, a new BSR trigger condition is required, i.e., when UL data becomes available for a logical channel and there is no data available for any logical channels that are mapped to the same SR configuration, the UE triggers a regular BSR.

3.	Conclusion

In this contribution, we discussed how to realize multiple SR configuration and SR procedure in detail. Our proposals are:

Proposal 1. In CA, one logical channel is mapped to none or one SR configuration per BWP.

Proposal 2. Separate sr-ProhibitTimer is configured per SR configuration, and each timer prohibits SR transmission on corresponding SR resource.

Proposal 3. Separate SR_COUNTER is configured per SR configuration with its own dsr-TransMax.

Proposal 4. Upon reaching the maximum SR counter for an SR configuration, the UE triggers RA procedure only if there is no on-going SR transmission on another SR configuration.

Proposal 5. The UE cancels all pending SR when the UE includes a BSR in a MAC PDU or the UL grant can accommodate all pending data.

Proposal 6. In order to support SR differentiation, a new BSR trigger condition is required, i.e., when UL data becomes available for a logical channel and there is no data available for any logical channels that are mapped to the same SR configuration, the UE triggers a regular BSR.