3GPP TSG-RAN WG2 #98-AH	Tdoc R2-1707169

Qingdao, P.R. of China, 27th – 29th June 2017

Agenda Item:	10.3.1.5

Source:	Ericsson

Title:	Scheduling request functionality

Document for:	Discussion, Decision

# Introduction

In #RAN2 98 some agreements related to SR were reached.

Agreements

1.	Multiple SR configurations can be configured to the UE and which SR configuration is used depends on the LCH that triggers the SR.  The granularity of SR configuration for a logical channel is FFS.

2. 	From RAN2 point of view a single bit SR with multiple SR configuration is sufficient to distinguish the “numerology/TTI length” of the logical channel that trigger the SR.  RAN2 has not identified other use cases for which multibit SR is need with sufficient support.

3.	RAN2 does not see the need to convey buffer status information.

4. 	Send LS to RAN1 to indicate to RAN1 that RAN2 doesn’t see the need to support multi-bit SR.

This contribution further analyses the possible options to enhance SR to achieve the above agreements.

# Discussion

It is agreed to introduce multiple single-bit SR configurations, where different SR resources are mapped to either different LCHs/LCGs or alternatively numerology/TTI. In this document, when mentioning a SR Configuration, it includes the parameters as in TS 36.331, i.e. sr-PUCCH-ResourceIndex, sr-ConfigIndex and dsr-TransMax.

## SR functionality in LTE

It can be worthwhile to repeat the function and purpose of the SR. Assuming the functionality is similar to LTE the purpose of the SR is to indicate to the network that the UE has data to send, but no UL resources. In more detail, when a BSR is triggered in the MAC entity (e.g. due to arriving data) the BSR triggers an SR if there are no UL resources. A triggered but not cancelled SR is a pending SR. The MAC entity will act upon this SR and if the UE does not have any SR configuration, a random access procedure will be initiated. If the UE has an SR configuration the MAC entity will order the physical layer to transmit an SR at the next opportunity. Upon reception of the SR, the eNB should reply with a grant which is at least big enough to fit the BSR. We think it is beneficial to reuse as much as possible of the LTE functionality in NR, but some things need to be revised as the UE may be configured with multiple SR configurations.

In LTE, if the UE has UL resources, no SR is triggered as the BSR will be included in the next transmission.

## SR functionality in NR

If there is a grant available, the UE is not allowed to send a SR, the BSR should instead be prioritized and included in the for the grant available.

### Mapping of LCGs and SR configurations

In NR a UE may be configured with multiple SR configurations. As there is at most one SR configuration in LTE, one can view this as all LCHs are mapped to the same SR configuration, i.e., regardless of which LCH triggers the BSR and the SR, the same SR configuration will be used. With multiple SR configurations in NR it needs to be decided which LCHs trigger which SR. In practice, there needs to be a mapping from LCH to SR.

A mapping from LCH to SR configuration is needed.

A straight-forward solution is to map the LCGs to SR configurations. It can be assumed that the gNB groups LCHs with similar properties in the same LCG, hence by mapping LCG to SR the gNB can have a rough idea on what grant to reply with. RAN1 has not concluded their discussion on the number of SR configurations the UE can have, but it is reasonable to assume the number will be less than the number of LCGs.

In LTE a logical channel can be configured with an SR mask. An LCH with an SR mask does not trigger SR. This behaviour can be implemented in NR if we allow an LCG not to be mapped to an SR configuration. Furthermore, if we allow an LCH to be mapped to multiple SR configurations we can reduce the delay from triggering an SR until transmission of an SR. That is beneficial as it reduces the overall delay. This leads to the following proposal.

An LCG is mapped to zero or more SR configurations.

This allows the NW to decide how to map each LCG to which SR configuration but if a SR configuration has a SR pending, it should not be allowed to trigger a new SR.

### SR procedure

In LTE the procedure for the pending SR is as follows, when no UL resources are available (briefly)

-	If there is no SR configuration, the UE triggers random access procedure.

-	If the UE has an SR configuration, the UE transmits SR.

There is also a prohibit timer limiting how often SR can be transmitted and a counter which limits the number of SR that can be transmitted before assuming there is no uplink and falling back to random access procedure.

In NR we think we can use the assumption from LTE and not transmit SR if the UE has UL resources. The reason behind this is that the BSR will be included in the next UL transmission.

A BSR does not trigger SR if the UE has UL resources available.

The second step is what to do with the pending SR, i.e., the triggered but not yet cancelled SR. If there is no SR configuration, then there is not much to do but trigger a random access.

If no SR configuration is available, then the pending SR triggers a random access procedure, and the pending SR is cancelled.

Otherwise, an SR should be transmitted and we need to determine which one. For reasons of simplicity we think only one SR should be transmitted, even if the LCG is mapped to more than one SR configuration. If the LCG is not mapped to any SR configuration, then an SR will not be transmitted. If the LCG is mapped to one SR configuration, then an SR corresponding to that SR configuration will be transmitted. If the LCG is mapped to more than one SR configuration we think an SR corresponding to the SR configuration which comes first in time should be transmitted. This is a simple solution and it minimizes the delay.

If the LCG which triggered the BSR is not mapped to any SR configuration, then the pending SR is cancelled (and no SR is transmitted).

If the LCG which triggered the BSR is mapped to one SR configuration, then an SR corresponding to that SR configuration is transmitted.

If the LCG which triggered the BSR is mapped to more than one SR configuration, then an SR corresponding to the SR configuration which comes first in time is transmitted.

### Limits in the procedure

The corresponding LTE procedure has two limits. How often an SR can be transmitted is controlled by sr-ProhibitTimer and the number of SRs which can be transmitted before the UL is considered lost is limited by dsr-TransMax. We think similar limitations are useful also in NR, but it is not clear to us if they are defined per SR configuration or common to all SR configurations. As dsr-TransMax is used to detect UL problems, it would make sense to have it common for all SR configurations, while sr-ProhibitTimer is used to limit the UE from sending SRs too often, it could be defined per SR configuration instead.

The functionality corresponding to sr-ProhibitTimer and dsr-TransMax is supported in NR.

RAN2 to discuss if sr-ProhibitTimer and dsr-TransMax are defined per SR configuration or common for all SR configurations.

### Physical layer parameters of the SR configuration

In LTE the physical layer parameters of the SR configuration are contained in the parameters sr-PUCCH-ResourceIndex and sr-ConfigIndex. It is up to RAN1 to determine the physical layer parameters of the SR configuration. Capturing them in the RRC specification will probably not be very complicated.

Let RAN1 define the physical layer parameters of the SR configuration.

# Conclusion

Based on the discussion in Section 2 we propose the following:

Proposal 1	An LCG is mapped to zero or more SR configurations.

Proposal 2	A BSR does not trigger SR if the UE has UL resources available.

Proposal 3	If no SR configuration is available, then the pending SR triggers a random access procedure, and the pending SR is cancelled.

Proposal 4	If the LCG which triggered the BSR is not mapped to any SR configuration, then the pending SR is cancelled (and no SR is transmitted).

Proposal 5	If the LCG which triggered the BSR is mapped to one SR configuration, then an SR corresponding to that SR configuration is transmitted.

Proposal 6	If the LCG which triggered the BSR is mapped to more than one SR configuration, then an SR corresponding to the SR configuration which comes first in time is transmitted.

Proposal 7	The functionality corresponding to sr-ProhibitTimer and dsr-TransMax is supported in NR.

Proposal 8	RAN2 to discuss if sr-ProhibitTimer and dsr-TransMax are defined per SR configuration or common for all SR configurations.

Proposal 9	Let RAN1 define the physical layer parameters of the SR configuration.

# References

R2-1704397, Logical channel prioritisation and multiple numerologies, Ericsson