3GPP TSG-RAN WG2#AH1801	Tdoc R2-1800289

Vancouver, Canada, Jan 22 – Jan 26, 2018	 (resubmission of R2-1712489)

Agenda Item:	10.4.1.6.1

Source:	Ericsson

Title:	SIBs needed for stand-alone NR deployments

Document for:	Discussion, Decision

# Introduction

The initial NR deployments using Rel-15 specifications will support a large number of use cases, including both eMBB and critical MTC deployments. Still, certain existing LTE functionality (such as D2D and operation in unlicensed bands) is currently not included in the NR work item [1]..

In this contribution, we review the LTE SIBs, and analyse which of them should be prioritized for the Rel-15 NR specifications. Note that the focus of the contribution is on the SIBs not part of the minimum system information.

# Discussion

## Functionality down-prioritized in NR Rel-15 work item

Based on the latest approved work item description in RP-171485, the following LTE functionality is not included in the Rel-15 scope:

- Waveforms above 52.6 GHz

- mMTC

- Interworking with UTRAN and GERAN

- Interworking with non-3GPP systems

- Wireless relay

- Satellite communication

- Air-to-ground and light air craft communications

- Extreme long-distance coverage

- Sidelink

- V2V and V2X

- Multimedia Broadcast/Multicast Service

- Shared spectrum and unlicensed spectrum

## Existing LTE SIBs

The list of LTE SIBs and their brief description is provided in Table 1.

Table 1: List of LTE SIBs

| SIB | Contents | Comment |
| --- | --- | --- |
| SIB1 | Information needed to access the cell and scheduling information for other SIBs | Part of minimum system information in NR. Note that both LTE SIB1 and SIB2 are needed to access the system. |
| SIB2 | Radio resource configuration common for all UEs | Part of minimum system information in NR. Note that both LTE SIB1 and SIB2 are needed to access the system. |
| SIB3 | Common cell re-selection information | Needed in NR |
| SIB4 | Neighbouring cell information for intra-frequency cell re-selection | Needed in NR |
| SIB5 | Information relevant for inter-frequency cell re-selection | Needed in NR |
| SIB6 | Information relevant for inter-RAT cell re-selection to UTRA | Not included in the Rel-15 work item scope |
| SIB7 | Information relevant for inter-RAT cell re-selection to GERAN | Not included in the Rel-15 work item scope |
| SIB8 | Information relevant for inter-RAT cell re-selection to CDMA2000 | Not included in the Rel-15 work item scope |
| SIB9 | Home eNB name | Not included in the Rel-15 work item scope |
| SIB10 | ETWS primary notification | Needed in NR |
| SIB11 | ETWS secondary notification | Needed in NR |
| SIB12 | CMAS notification | Needed in NR |
| SIB13 | MBMS control information | Not included in the Rel-15 work item scope |
| SIB14 | EAB parameters | Potentially replaced by unified access barring mechanism, but not in Rel-15 scope in general. |
| SIB15 | MBMS service area identities | Not included in the Rel-15 work item scope |
| SIB16 | GPS and UTC | Need to be evaluated based on input from chipset vendors |
| SIB17 | Traffic steering between E-UTRAN and WLAN | Not included in the Rel-15 work item scope |
| SIB18 | Sidelink communication related information | Not included in the Rel-15 work item scope |
| SIB19 | Sidelink discovery related information | Not included in the Rel-15 work item scope |
| SIB20 | MBMS transmission using SC-PTM | Not included in the Rel-15 work item scope |
| SIB21 | V2X sidelink related information | Not included in the Rel-15 work item scope |

## Proposed NR SIBs

The contents of the LTE SIB1 and SIB2 have been analysed in other papers (see e.g. [2] for an overview of SIBs for minimum system information) and will not be discussed here further.

Based on analysis in section 2.2, the information from the following LTE SIBs in addition to the minimum system information are potentially needed in NR Rel-15

- Cell reselection related system information (i.e. LTE SIBs 3, 4, 5). Note that in addition to intra- and inter-frequency cell reselection, support for idle mode mobility to LTE is include in the scope of the Rel-15 work item, and corresponding system information is needed.

- Public warning system related information (ETWS and CMAS, i.e. LTE SIBs 10-12).

- GPS and UTC information (i.e. LTE SIB16)

For extended access barring, the need for a separate SIB depends on the progress on a more unified design. It’s assumed that a separate SIB for EAB is not needed.

The need for GPS and UTC information depends on the evaluation on the usefulness of this information by chipset vendors.

The need for GPS and UTC SIB in NR Rel-15 is evaluated based on input from chipset vendors.

For the cell reselection parameters, it is proposed to follow the LTE structure, and separate common cell reselection information, intra- and inter-frequency and LTE related cell reselection information to separate SIBs. Following our proposal to only use MIB and SIB1 for the minimum system information [2], this would lead to following “other system information” SIBs being prioritized for the time being.

Similarly for the ETWS/CMAS SIBs, the simplest approach would be to follow the LTE approach, and have separate SIBs for notification and ETWS/CMAS warnings. See [3] for further discussion on public warning systems in NR.

Note also that it may be beneficial to split the SIBs into two parts (SIBXa and SIBXb) where one part is common within the SI Area and the other part is not. This would enable the UE to store information common within SI area and not having to read it in every cell.

Table 2: Proposed contents of NR SIBs

| SIB | Contents | Comment |
| --- | --- | --- |
| NR-SIB2 | Common cell re-selection information |  |
| NR-SIB3 | Neighbouring cell information for intra-frequency cell re-selection |  |
| NR-SIB4 | Information relevant for inter-frequency cell re-selection |  |
| NR-SIB5 | Information relevant for inter-RAT cell re-selection to E-UTRA |  |
| NR-SIB6 | ETWS primary notification |  |
| NR-SIB7 | ETWS secondary notification |  |
| NR-SIB8 | CMAS notification |  |

Adopt the NR SIBs in Table 2 for Rel-15 (except for minimum-system information).

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	The need for GPS and UTC SIB in NR Rel-15 is evaluated based on input from chipset vendors.

Proposal 2	Adopt the NR SIBs in Table 2 for Rel-15 (except for minimum-system information).

# References

RP-171485, “Work Item on New Radio (NR) Access Technology”

R2-1708162” System information structure and contents”, Ericsson

R2-1708157, “Public Warning System in NR”, Ericsson, NTT DOCOMO INC., SoftBank