3GPP TSG-RAN WG2 ad hoc on NR	Tdoc R2-1800303

Vancouver, Canada, Jan 22 - 26, 2017

Agenda Item:	10.4.1.6.1

Source:	Ericsson

Title:	Size of remaining minimum system information

Document for:	Discussion, Decision

# Introduction

Before completing the design of SIB structure for minimum system information for NR, it is important to evaluate whether the size of the minimum system information is small enough to be transmitted in all realistic deployments. Both the maximum size of system information that can be transmitted and the size of system information that needs to be transmitted are likely to vary significantly due to factors such as beam sweeping, network sharing, access barring etc.

In this contribution, we analyze the expected size of the minimum system information.

# Discussion

Even though the contents of the minimum system information have not been fully decided, there is a general consensus (see e.g. [1] – [4] and the RAN2 agreements referenced in those contributions) that the minimum information should be based on the contents of LTE SIB1 and SIB2, as well as a few selected NR specific information elements (such as RNA ID).

In order to make a rough estimate of the size of the minimum system information, we have calculated the size including most (if not all) of the IEs currently proposed to be included. The detailed calculation is provided in Section 5, but some of the main assumptions are

- The minimum system information will include most of the information currently included in RRC ServingCellConfigCommon

- Network sharing of up to 6 PLMNs will need to be supported

- Value tags are provided per SIB

- Access barring information is provided

- There are three SI messages and 8 SIBs, SI1 = SIB2,3,4,5 (NR and LTE reselection), SI2 = SIB6,7,8 (ETWS, CMAS), SI3 = 16 (GPS and UTC).

Our estimate for the size of the minimum system information is roughly between 400 and 1300 bits. Note that even though this estimate is unlikely to be extremely accurate, we do feel it represents the order of magnitude for remaining minimum system information reasonably well. The IEs with largest number of bits are

- Serving cell config common: between 400 and 1100 bits

- PLMN information for network sharing: ~150 bits

- Access barring: ~100 bits (note that we have not assumed PLMN specific ACB parematers)

- SI scheduling info: ~50 bits

- Value tags: ~50 bits

In our opinion, even 400 bits is a relatively large size for information that may need to be beam swept at least in some deployments. It would be possible to reduce the size by moving some of the information, such as some of the PLMN information in network sharing situations (proposed in [5]) or access barring information (proposed in [3]) to other system information. Such optimizations would increase the system access time, and would contradict the RAN2 desire to have all information needed to access the cell in the minimum system information, but might be justified in some deployments.

However, since the L1 configuration is by far the largest contributor to the SIB1 size, we would prefer to check with RAN1 if the expected minimum system information size is an issue and if yes, whether they could consider slightly less flexibility in L1 parameter configuration.

- Check with RAN1 whether remaining minimum system information size of roughly 1000 bits can be supported in all deployments.

A draft LS to RAN1 is provided in [6].

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Check with RAN1 whether remaining minimum system information size of roughly 1000 bits can be supported in all deployments.

# References

R2-1712483	System information structure and contents	Ericsson

R2-1713284	Detailed design of the contents of System Information	Huawei, HiSilicon

R2-1713433	Some SIB1 contents and clean-up of Section 7.3 in Stage-2	Samsung R&D Institute India

R2-1713636	Organization of NR System Information	Qualcomm Incorporated

R2-1800291	System information content at network sharing	Ericsson

R2-1800304	Draft LS to RAN1 on size of remaining minimum system information	Ericsson

# Estimated IE sizes

Note: in case there are sub-fields (such as the IEs of a SEQUENCE), they are listed with 2 space indentation, and the overall size is marked in bold.

| Estimated total size: 426-1364 bits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IE |  |  | IE size | L1 IE size | L2 IE size | Comment |
| ServingCellConfigCommon |  |  | 178-1110 |  |  |  |
|  | physCellId OPTIONAL |  |  | - | - | Encoded in MIB in physical layer, not explicitly needed in SIB |
|  | frequencyInfoDL FrequencyInfoDL OPTIONAL |  |  | 4-81 |  |  |
|  |  | FrequencyInfoDL ::= SEQUENCE { ssb-subcarrier-offset INTEGER (1..11) OPTIONAL, offsetToPointA INTEGER (0..2199) OPTIONAL, scs-SpecificCarriers SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrier OPTIONAL, ... } |  |  | 4-81 1-5 1-13 1-62 1 | Assuming at most 2 scs-specific carriers |
|  |  | SCS-SpecificVirtualCarrier ::= SEQUENCE { offsetToVirtualCarrier INTEGER (0..2199) OPTIONAL, subcarrierSpacing SubcarrierSpacing OPTIONAL, k0 FFS_Value OPTIONAL, carrierBandwidth INTEGER (1..maxNrofPhysicalResourceBlocks) OPTIONAL, ... } |  |  | 5-31 1-13 1-3 1-4? 1-10 1 | SubcarrierSpacing ::= ENUM {kHz15, kHz30, kHz60, kHz120} Assuming 3 bits for k0 maxNrofPhysicalResourceBlocks INTEGER ::= 275 |
|  | initialDownlinkBandwidthPart DownlinkBandwidthPart OPTIONAL, |  |  | 7-110 |  |  |
|  |  | DownlinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, pdcch-ConfigCommon PDCCH-ConfigCommon OPTIONAL } |  |  | 7-110 5-24 2-86 |  |
|  |  | BandwidthPart ::= SEQUENCE { bandwidthPartId BandwidthPartId, locationAndBandwidth INTEGER (1..65536) OPTIONAL, subcarrierSpacing ENUMERATED {n0, n1, n2, n3, n4} OPTIONAL, cyclicPrefix ENUMERATED { extended } OPTIONAL } |  |  | 5-24 2 1-17 1-4 1 | BandwidthPartId ::= INTEGER (0..maxNrofBandwidthParts-1) maxNrofBandwidthParts INTEGER ::= 4 |
|  |  | PDCCH-ConfigCommon ::= SEQUENCE { searchSpaceOtherSystemInformation FFS_Value OPTIONAL, pagingSearchSpace FFS_Value OPTIONAL } |  |  | 2-86 1-43 1-43 | Assuming standard search space configuration, which is roughly 23-42 bits + common search space specific parameters. |
|  |  | SearchSpace ::= SEQUENCE { searchSpaceId SearchSpaceId, controlResourceSetId ControlResourceSetId, monitoringSlotPeriodicityAndOffset CHOICE { sl1 NULL, sl2 INTEGER (0..1), sl5 INTEGER (0..4), sl10 INTEGER (0..9), sl20 INTEGER (0..19) } OPTIONAL, monitoringSymbolsWithinSlot BIT STRING (SIZE (14)) OPTIONAL, nrofCandidates SEQUENCE { al1 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, al2 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, al4 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, al8 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}, al16 ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8} }, searchSpaceType CHOICE { common SEQUENCE { }, ue-Specific SEQUENCE {} } OPTIONAL -- Need M } |  |  | 23-42 3 3 1-5 1-15 15 1-? | Assuming 8 search spaces Assuming 8 control resources |
|  | uplinkConfigCommon UplinkConfigCommon |  |  | 132-382 |  |  |
|  |  | UplinkConfigCommon ::= SEQUENCE { frequencyInfoUL FrequencyInfoUL OPTIONAL, initialUplinkBandwidthPart UplinkBandwidthPart OPTIONAL } |  |  | 132-382 7-109 125-273 |  |
|  |  | FrequencyInfoUL ::= SEQUENCE { absoluteFrequencyUL ARFCN-ValueNR OPTIONAL, offsetToPointA INTEGER (0..2199) OPTIONAL, scs-SpecificCarriers SEQUENCE (SIZE (1..ffsValue)) OF SCS-SpecificVirtualCarrier OPTIONAL, aSE AdditionalSpectrumEmission OPTIONAL, p-Max P-Max OPTIONAL, frequencyShift7p5khz ENUMERATED {true} OPTIONAL, ... } |  |  | 7-109 1-19 1-13 1-62 1-6 1-7 1 1 | ARFCN for NR not known, assuming 262143 (extended LTE) Assuming 2 subcarrier spacings (see downlink for values) From LTE: AdditionalSpectrumEmission ::= INTEGER (1..32) From LTE: P-Max ::= INTEGER (-30..33) |
|  |  | UplinkBandwidthPart ::= SEQUENCE { genericParameters BandwidthPart, directCurrentLocation INTEGER (0..3299) OPTIONAL, rach-ConfigCommon RACH-ConfigCommon, pusch-ConfigCommon PUSCH-ConfigCommon, pusch-Config PUSCH-Config, pucch-ConfigCommon PUCCH-ConfigCommon, pucch-Config PUCCH-Config } |  |  | 125-273 5-24 1-13 111-174 4-15 - 4-47 - | Note that the current structure may not have correct optionality Not needed for SIB1 Not needed for SIB1 |
|  |  | RACH-ConfigCommon ::= SEQUENCE { groupBconfigured SEQUENCE { ra-Msg3SizeGroupA ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800, b1000, spare7, spare6, spare5, spare4, spare3, spare2, spare1}, messagePowerOffsetGroupB ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18} } OPTIONAL, cbra-SSB-ResourceList CBRA-SSB-ResourceList, ra-ContentionResolutionTimer ENUM{sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}, ssb-Threshold RSRP-Range OPTIONAL, sul-RSRP-Threshold RSRP-Range OPTIONAL, prach-ConfigurationIndex INTEGER (0..255) OPTIONAL, prach-RootSequenceIndex CHOICE { l839 INTEGER (0..837), l139 INTEGER (0..137) } OPTIONAL, zeroCorrelationZoneConfig INTEGER(0..15), msg1-SubcarrierSpacing SubcarrierSpacing OPTIONAL, msg1-FDM BIT STRING (SIZE (2)), msg1-FrequencyStart INTEGER (0..maxNrofPhysicalResourceBlocks-1) OPTIONAL, restrictedSetConfig ENUMERATED {unrestricted, restrictedToTypeA, restrictedToTypeB}, preambleReceivedTargetPower ENUMERATED { dBm-120, dBm-118, dBm-116, dBm-114, dBm-112, dBm-110, dBm-108, dBm-106, dBm-104, dBm-102, dBm-100, dBm-98, dBm-96, dBm-94,dBm-92, dBm-90, dBm-88, dBm-86, dBm-84,dBm-82, dBm-80, dBm-78, dBm-76, dBm-74, dBm-72, dBm-70, dBm-68, dBm-66, dBm-64, dBm-62, dBm-60, dBm-58, dBm-56, dBm-54, dBm-52, dBm-50, dBm-48, dBm-46, dBm-44, dBm-42, dBm-40, dBm-38, dBm-36, dBm-34, dBm-32, dBm-30, dBm-28, dBm-26, dBm-24, dBm-22, dBm-20, dBm-18, dBm-16, dBm-14, dBm-12, dBm-10, dBm-8, dBm-6, dBm-4, dBm-2, dBm-0, dBm2, dBm4, dBm6 } OPTIONAL, powerRampingStep ENUMERATED {dB0, dB2, dB4, dB6} OPTIONAL, preambleTransMax ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}, cb-preamblesPerSSB FFS_Value OPTIONAL, ssb-perRACH-Occasion FFS_Value OPTIONAL, ra-ResponseWindow ENUMERATED {ffsTypeAndValue}, msg2-SubcarrierSpacing SubcarrierSpacing, ra-ControlResourceSet ControlResourceSetId OPTIONAL, ra-SearchSpace SearchSpace OPTIONAL, msg3-SubcarrierSpacing SubcarrierSpacing, msg3-transformPrecoding ENUMERATED {true} OPTIONAL } |  |  | 86-149 1-8 - 3 7 7 8 10-12 4 1-3 2 1-10 2 1-7 1-3 4 1-8 1-7 3 2 3 23-42 2 1 | This has been replaced by new RAN1 signalling From LTE: RSRP-Range ::= INTEGER(0..97) SubcarrierSpacing ::= ENUM {kHz15, kHz30, kHz60, kHz120} maxNrofPhysicalResourceBlocks INTEGER ::= 275 Assuming 128 preambles per beam Assuming 64 beams From LTE: ENUMERATED { sf2, sf3, sf4, sf5, sf6, sf7, sf8, sf10}, Assuming 8 control set resources Assuming 8 search spaces and 8 control resources (see DL) |
|  |  | PUSCH-ConfigCommon ::= SEQUENCE { groupHoppingEnabledTP ENUMERATED {enabled} OPTIONAL, msg3-DeltaPreamble FFS_Value OPTIONAL, p0-NominalWithGrant INTEGER (-202..24) OPTIONAL, ... } |  |  | 4-15 1 1-4 1-9 1 | From LTE: deltaPreambleMsg3 INTEGER (-1..6) |
|  |  | PUCCH-ConfigCommon ::= SEQUENCE { pucch-ResourceCommon BIT STRING (SIZE (4)) OPTIONAL, pucch-Format0-BaseSequenceHopping ENUM{true} OPTIONAL, pucch-Format1-BaseSequenceHopping ENUM{true} OPTIONAL, sequenceHoppingId BIT STRING (SIZE (10)) OPTIONAL, p0-nominal INTEGER (-202..24) OPTIONAL, deltaF-pucch-f0 FFS_Value OPTIONAL, deltaF-pucch-f1 FFS_Value OPTIONAL, deltaF-pucch-f2 FFS_Value OPTIONAL, deltaF-pucch-f3 FFS_Value OPTIONAL, deltaF-pucch-f4 FFS_Value OPTIONAL } |  |  | 10-47 1-5 1 1 1-11 1-9 1-4 1-4 1-4 1-4 1-4 | Assuming 8 values for all deltaFs |
|  | supplementaryUplink SEQUENCE { uplinkConfigCommon UplinkConfigCommon OPTIONAL } OPTIONAL, -- Cond SUL |  |  | 1-383 |  |  |
|  | subcarrierSpacingCommon SubcarrierSpacing, ssb-PositionsInBurst CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, ssb-periodicityServingCell ENUM { ms5, ms10, ms20, ms40, ms80, ms160 } OPTIONAL, dmrs-TypeA-Position ENUMERATED {pos2, pos3}, subcarrierSpacingSSB SubcarrierSpacingSSB OPTIONAL, tdd-UL-DL-configurationCommon TDD-UL-DL-ConfigCommon OPTIONAL, tdd-UL-DL-configurationCommon2 TDD-UL-DL-ConfigCommon OPTIONAL, ss-PBCH-BlockPower INTEGER (-60..50) |  |  | 2 6-66 3 1 1-3 7-36 7-36 7 |  |  |
|  |  | TDD-UL-DL-ConfigCommon ::= SEQUENCE { refereceSubcarrierSpacing SubcarrierSpacing OPTIONAL, dl-UL-TransmissionPeriodicity ENUMERATED {ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10} OPTIONAL, nrofDownlinkSlots INTEGER (0..160) OPTIONAL, nrofDownlinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL, nrofUplinkSlots INTEGER (0..160) OPTIONAL, nrofUplinkSymbols INTEGER (0..maxSymbolIndex) OPTIONAL } |  |  | 1-3 1-4 1-9 1-5 1-9 1-5 | maxSymbolIndex INTEGER ::= 13 |
| cellAccessRelatedInfo |  |  | 94-244 |  |  | From LTE |
|  | plmn-IdentityList PLMN-IdentityList, trackingAreaCode TrackingAreaCode, cellIdentity CellIdentity, ranNotificationAreaCode RANnotificationAreaCode |  |  | 6-156 16 36 36 |  | From LTE: PLMN-IdentityList ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo PLMN-IdentityInfo ::= SEQUENCE { plmn-Identity PLMN-Identity, cellReservedForOperatorUse ENUMERATED {r, notR} } PLMN-Identity ::= SEQUENCE { mcc MCC OPTIONAL, mnc MNC } MCC ::= SEQUENCE (SIZE (3)) OF MCC-MNC-Digit MNC ::= SEQUENCE (SIZE (2..3)) OF MCC-MNC-Digit MCC-MNC-Digit ::= INTEGER (0..9) maxPLMN-r11 INTEGER ::= 6 TrackingAreaCode ::= BIT STRING (SIZE (16)) Notet that TAC might need to be provided per PLMN. (R3-174229) RAN3 has reached the following agreements related to NCI and its structure: 1) The NCI has fixed length of 36 bits. 2) The leftmost bits of the NCI correspond to the gNB ID. 3) The gNB ID has flexible length between 22 bits and 32 bits in 1-bit steps (11 values), to accommodate different deployment scenarios. |
| cellSelectionInfo |  |  | 1-10 |  |  |  |
|  | SEQUENCE { q-RxLevMin Q-RxLevMin, q-RxLevMinOffset INTEGER (1..8) OPTIONAL }, |  |  | 6 1-4 |  | From LTE: Q-RxLevMin ::= INTEGER (-70..-22) |
| SchedulingInfo |  |  | 45++ |  |  |  |
|  | si-WindowLength ENUMERATED {ms1, ms2, ms5, ms10, ms15, ms20, ms40}, schedulingInfoList SchedulingInfoList, + notification of on demand availability (1 bit per SIB) |  |  | 3 42 |  | From LTE: SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo SchedulingInfo ::= SEQUENCE { si-Periodicity ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo SIB-MappingInfo } SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type SIB-Type ::= ENUMERATED {sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, sibType10, sibType11, sibType12-v920, sibType13-v920, sibType14-v1130, sibType15-v1130, sibType16-v1130, sibType17-v1250, sibType18-v1250, ..., sibType19-v1250, sibType20-v1310, sibType21-v1430} maxSIB INTEGER ::= 32 maxSIB-1 INTEGER ::= 31 maxSI-Message INTEGER ::= 32 Assuming: SI1 = SIB2,3,4,5 (NR and LTE reselection) SI2 = SIB6,7,8 (ETWS, CMAS) SI3 = 16 (GPS and UTC) |
| ValueTag |  |  |  |  |  |  |
|  | systemInfoValueTag per SIB INTEGER (0..31) |  | 40 |  |  | Assuming 8 SIBs |
| Additional parameters for requesting other SI |  |  |  | 5 |  |  |
|  | Msg1vsMsg3 preamblesGroupBConfig SEQUENCE { sizeOfRA-PreamblesGroupB SetOfPreambles, } OPTIONAL |  |  | 1 4? |  |  |
| Access Barring |  |  | 68 |  |  |  |
|  | acBarringInfo SEQUENCE (SIZE (1..64)) OF AC-BarringConfig Assuming only one bitmap and AC 0..8 |  |  | 63 5 |  | From LTE: AC-BarringConfig ::= SEQUENCE { ac-BarringFactor ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p40, p50, p60, p70, p75, p80, p85, p90, p95}, ac-BarringTime ENUMERATED {s4, s8, s16, s32, s64, s128, s256, s512}, ac-BarringForSpecialAC BIT STRING (SIZE(5)) Note that more information may need to be provided due to Unified Access Barring mechanism. |
| Service identification |  |  |  | ? |  |  |