3GPP TSG-RAN WG2 NR Ad hoc 1801	R2-1800588

Vancouver, Canada, 22 – 26 January 2018

Agenda item:	10.3.1.10

Source:	Nokia, Nokia Shanghai Bell

Title:	Confirm the working assumption on inactivity timer

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution addresses the following open issue [1]:

[Issue 5.7-5]	The following working assumption [2] needs to be implemented after having RAN2 agreement: Working assumption:  Like in LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used. This behaviour depends on outcome of DRX design.

# 2	Discussion

Current MAC specification [3], (re)start of HARQ RTT timer and inactivity timer is only done during active time when a PDCCH is received.

1> if the MAC entity is in Active Time: 2> monitor the PDCCH; 2> if the PDCCH indicates a DL transmission or if a DL assignment has been configured: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission or if a UL grant has been configured: 3> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission; 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 2> if the PDCCH indicates a new transmission (DL or UL): 3> start or restart drx-InactivityTimer.

No special handling in LTE with the assumption of SPS occasions are typically aligned with drx-onDurationTimer, so that the eNB can override with dynamic grant if needed. With very short SPS periodicity now, it is not likely to align the configured grant with drx-onDurationTimer.

Proposal 1: confirm that it is possible the configured grant is not aligned with DRX drx-onDurationTimer or active time.

The timer handling is under the condition of “if the MAC entity is in Active Time”, the timers would not be restarted which causes issue for following potential UL retransmissions and DL response.

Since it was agreed retransmission would only be done via dynamic scheduling, the only problematic issue is for the initial transmission on the configured grant. With the working assumption of restarting the inactivity timer, the issue is resolved as the inactivity timer would keep the UE awake for both following potential new transmission from DL and UL retransmissions even if the UL HARQ RTT timer is not restarted.

Proposal 2: confirm the working assumption that inactivity timer is started or restarted when UL data is transmitted on a configured grant, regardless of whether the MAC entity is in active time.

# 3	Conclusion

DRX operation for SPS is discussed in this contribution with the following proposals proposed. TP for 38.321 is provided in the Annex and CR in [3].

Proposal 1: confirm that it is possible the configured grant is not aligned with DRX drx-onDurationTimer or active time.

Proposal 2: confirm the working assumption that inactivity timer is started or restarted when UL data is transmitted on a configured grant, regardless of whether the MAC entity is in active time.

# References

- R2-1800612, List of open issues on NR MAC, Samsung (Rapporteur)

- R2-1703796, Report from LTE Break-Out Session, Vice-Chair (InterDigital)

- 3GPP TS38.321 v15.0.0, NR; Medium Access Control (MAC) protocol specification

- R2-1800589, Correction on inacitivtity timer for configured grant, Nokia, Nokia Shanghai Bell

# Annex TP for 38.321

## 5.7	Discontinuous Reception (DRX)

The MAC entity may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously. When using DRX operation, the MAC entity shall monitor PDCCH according to requirements found in this specification.

RRC controls DRX operation by configuring the following timers:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay in slots before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates an initial UL or DL user data transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycle: the Long DRX cycle;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the preamble not selected by the MAC entity (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if an drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if an UL grant has been configured and UE has data to transmit on the configured grant:

2>	start or restart the drx-InactivityTimer immediately after the first repetition of the corresponding PUSCH transmission.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN * 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if drx-SlotOffset is configured:

3>	start drx-onDurationTimer after drx-SlotOffset.

2>	else:

3>	start drx-onDurationTimer.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission or if a UL grant has been configured:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer.

1>	else (i.e. not part of the Active Time):

2>	not report CQI/PMI/RI on PUCCH.