3GPP TSG-RAN WG2 Meeting AH-1801                                                                      R2-1800662

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.1.3

Souce:	MediaTek Inc.

Title:	TP for MAC CEs to Support Beam Management and CSI Acquisition

Document for:     Discussion and Decision

# 1 Introduction

The TP for MAC CEs to support beam management and CSI acquisition based on our discussion paper [1]. It is intend to:

- Capture the new MAC CEs required by RAN1 to support beam management and CSI acquisition;

- Be used as the baseline for further discussion and subject to change based on new RAN1 agreements.

# 2 TP for new MAC CEs

### 6.1.3	MAC Control Elements (CEs)

#### 6.1.3.12	SP CSI reporting Activation/Deactivation MAC CE

The SP CSI reporting activation/deactivation MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of [X] octets containing [8X] octets SPR fields. The SP CSI reporting Activation/Deactivation MAC CE is defined as follows (Figure 6.1.3.12-1):

-	SPRi: if there is a csi-ReportConfig with reportConfigId i as specified in TS 38.331 [8] with reportConfigType set to semiPersistent, this field indicates the activation/deactivation status of the CSI-ReportConfig with reportConfigId i, else the MAC entity shall ignore the SPRi field. The SPRi field is set to "1" to indicate that the CSI-ReportConfig with reportConfigId i shall be activated. The SPRi field is set to "0" to indicate that the CSI-ReportConfig with reportConfigId i shall be deactivated;

Figure 6.1.3.12-1 SP CSI reporting Activation/Deactivation MAC CE

#### 6.1.3.13	SP CSI-RS/CSI-IM Activation/Deactivation MAC CE

The SP CSI-RS/CSI-IM activation/deactivation MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of [Y] octets containing [8X] octets defined as follows (Figure 6.1.3.13-1):

-	A/D: this field indicates the activation/deactivation status of the SP CSI-RS/CSI-IM. It is set to "1" to indicate that the SP CSI-RS/CSI-IM shall be activated. It is set to "0" to indicate that the SP CSI-RS/CSI-IM shall be deactivated;

-	TCI State ID: this field indicates TCI State ID of the corresponding SP CSI-RS/CSI-IM as specified in TS 38.331 [8]. The length of the field is [7] bits;

-	CSI-RS resource set ID: this field indicates the nzp-csi-rs-ResourceId of the corresponding SP CSI-RS as specified in TS 38.331 [8]. The length of the field is [6] bits.

-	CSI-IM resource set ID: this field indicates the csi-IM-ResourceConfigId of the corresponding SP CSI-IM as specified in TS 38.331 [8]. The length of the field is [6] bits.

-	R: reserved bit, set to "0";

Figure 6.1.3.13-1 SP CSI-RS/CSI-IM Activation/Deactivation MAC CE

#### 6.1.3.14	Aperiodic CSI trigger State Sub-selection MAC CE

The aperiodic CSI trigger State Sub-selection MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a variable size and consists of variable number of AC fields. The number of AC fields is relevant to the number of configured CSI triggering states (Sc). The aperiodic CSI trigger state sub-selection MAC CE is defined as follows (Figure 6.1.3.14-1):

-	ACi: if there is a csi-TriggerState with triggerStateId i in AperiodicReportTrigger as specified in TS 38.331 [8], this field indicates the activation/deactivation status of the CSI-TriggerState with triggerStateId i, else the MAC entity shall ignore the ACi field. The ACi field is set to "1" to indicate that the CSI-TriggerState with triggerStateId i shall be activated. The ACi field is set to "0" to indicate that the CSI-TriggerState with triggerStateId i shall be deactivated;

Figure 6.1.3.14-1 Aperiodic CSI trigger State Sub-selection MAC CE

#### 6.1.3.15	NR-PDCCH TCI Indication MAC CE

The NR-PDCCH TCI Indication MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a variable size and consists of variable number TCI fields. The number of TCI fields is relevant to the number of configured TCI states (K). The NR-PDCCH TCI Indication MAC CE is defined as follows (Figure 6.1.3.15-1):

-	TCIi: if there is a tci-State with tci-State-Id i for the ControlResourceSet as specified in TS 38.331 [8], this field indicates the activation/deactivation of the TCI-State with tci-State-Id i,  else the MAC entity shall ignore the TCIi field. The TCIi field is set to "1" to indicate that the TCI-State with tci-State-Id i shall be activated. The ACi field is set to "0" to indicate that the TCI-State with tci-State-Id i shall be deactivated.

Figure 6.1.3.15-1 NR-PDCCH TCI Indication MAC CE

#### 6.1.3.16	NR-PDSCH TCI Activation/Deactivation MAC CE

The NR-PDSCH TCI activation/deactivation MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a variable size and consists of variable number TCI fields. The number of TCI fields is relevant to the number of configured TCI states (M). The NR-PDSCH TCI activation/deactivation MAC CE is defined as follows (Figure 6.1.3.16-1):

-	TCIi: this field indicates the presence/absence of the TCI-State with tci-State-Id i in the the DCI transmitted by a PDCCH, else the MAC entity shall ignore the TCIi field. The TCIi field is set to "1" to indicate that the TCI-State with tci-State-Id i shall be present in the DCI. The ACi field is set to "0" to indicate that the TCI-State with tci-State-Id i shall be absent in the DCI.

Figure 6.1.3.16-1 NR-PDCCH TCI Indication MAC CE

#### 6.1.3.17	SP SRS Activation MAC CE

The SP SRS activation MAC CE is identified by MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of [Z] octets containing [8Z] octets defined as follows (Figure 6.1.3.17-1):

-	S/C/SR: this field indicates which type of reference signal ID is used: ‘00’ indicates SSB ID is used; ‘01’ indicates CSI-RS resource ID is used; ‘10’ indicates SRS resource ID is used; ‘11’ is reserved;

-    SRS resource set ID: this field indicates srs-ResourceSetId of the corresponding SRS-ResourceSet as specified in TS 38.331 [8]. The length of the field is [6] bits;

-	RS ID: this field indicates the ID of the reference signal indicated in S/C/SR field. The length of the field is [8] bits.

Figure 6.1.3.17-1 SP SRS Activation MAC CE

# 3 References

[1] R2-1800653 MAC CEs for Beam Management and CSI Acquisition Mediatek Inc.

[2] R1-1721661 List of MAC CE parameters for MIMO

[3] R2-1714246 LS on MAC CEs for beam management and CSI

[4]TS38.321 V2.0.0