3GPP TSG-RAN WG2 NR AH#1801	Tdoc R2-1800695

Revision of R2-1712920

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.4.4

Source:	Ericsson

Title:	Issues with the existing QoS Framework

Document for:	Discussion, Decision

# Introduction

This contribution discusses about stage 3 issues existing in the current QoS framework. Two approaches can be taken in order resolve the issues, either simplify the baseline functionality or to introduce additional solutions to resolve the issues. Stage 2 details of the QoS framework are discussed in a contribution: Analysis of the QoS framework (Stage 2) [1].

SA WG2 Meeting #123 approved an CR S2-178060 on Clarification of QoS Flows with signalled characteristics:

5.7.1.3	Control of QoS Flows

Three options to control QoS flows are supported:

1)	For non-GBR QoS flows only, and when standardized 5QIs or pre-configured 5QIs are used, the 5QI value can be used as the QFI of the QoS flow. The default ARP shall be pre-configured in the AN;

NOTE 1:	The above option is intended to be used for non-3GPP ANs (e.g. Fixed AN) scenarios when there is no need for any N1 signalling including PDU session establishment, nor any N2 signalling.

2)	For non-GBR QoS flows only, and when standardized 5QIs or pre-configured 5QIs are used, the 5QI value can be used as the QFI of the QoS flow. The default ARP and the QFI is sent to RAN over N2 at PDU Session Establishment or at PDU Session Modification and when NG-RAN is used every time the User Plane of the PDU session is activated; and

3)	For GBR and non-GBR QoS flows, a dynamically assigned QFI can be used. The 5QI value may be a standardized, pre-configured or dynamic assigned. The QoS profile and the QFI of a QoS flow is provided to the (R)AN over N2 at PDU Session establishment or at QoS flow establishment/modification and when NG-RAN is used every time the User Plane of the PDU session is activated.

NOTE 2:	The options 2 and 3 are intended to be used for 3GPP ANs.

QFI is always signaled via N2 interface according to TS 23.501

RAN3 has agreed that each QFI is associated with DRB:

RAN3 TS 38.413 NG Application Protocol: 8.2.3 PDU Session Resource Modify:

For each QoS flow included in the QoS Flows Add Or Modify Request List IE, based on the QoS Flow Level QoS Parameters IE, the NG-RAN node may establish, modify or release the Data Radio Bearer configuration and may change allocation of resources on NG or Uu accordingly. The NG-RAN node shall associate each QoS flow accepted to setup or modify with a Data Radio Bearer of the PDU session. The associated Data Radio Bearer for the QoS flow accepted to modify may not change.

RAN3 TS 38.413 specifies that each QFI is associated with a DRB inside PDU session.

# Discussion

The main concerns regarding the Access Stratum (AS) Reflective QFI to DRB mapping are following:

- UE processing of each DL packet

- Added UE implementation complexity

- Questionable signalling benefits

- SDAP header format and overhead addition

## AS reflective QoS mechanism

The Access Stratum (AS) reflective QFI to DRB mapping mechanism is used to convey QFI to DRB mapping information in SDAP header. The AS reflective QFI to DRB mapping can be used only after:

- UL and DL SDAP headers are configured with RRC configuration

- QFI filters are configured via NAS signalling

- Both, source, and target DRBs are established via RRC signalling

- AS reflective QoS is configured via RRC

The initial purpose of AS reflective QoS mechanism was to reduce Access Stratum signalling. However, the mechanism is unable to operate without RRC signaling. The AS reflective QFI to DRB mapping mechanism can only work after the DRBs have been established by RRC, and therefore, the only signaling benefit that can be achieved is in the case where source and target DRBs are both established and AS-reflective mechanism is used. Following table describes mapping cases, where AS reflective QoS could be used and analyzes benefits.

Table 1	Signaling & Overhead Comparison of AS-reflective mapping options

| Case (AS reflective QoS is enabled in all cases) | Can the mechanism work without RRC signaling? | How frequently changes are expected? | Reduction in Signalling | DL & UL overhead |
| --- | --- | --- | --- | --- |
| DL traffic: QFI is mapped to a new DRB | No, the DRB needs to be established | Non-frequently | None | Increased for each DL & UL packet |
| UL traffic: QFI is mapped to a new DRB | No, cannot be done with AS reflective QoS. | Non-frequently | None | Increased for each DL & UL packet |
| DL traffic: QFI is mapped to an existing DRB | No, initial DRB setup requires RRC signaling, AS mapping does not. | Non-frequently | None/Minimal | Increased for each DL & UL packet |
| UL traffic: QFI is mapped to an existing DRB | N/A, UE cannot make the decision. AS mapping cannot be used | N/A | N/A | N/A |
| UL traffic: QFI is mapped to default DRB | No, it is likely the QFI needs to be re-mapped to another DRB. | Non-frequently | Potential increase of signaling. | Increased for each DL & UL packet |
| DL traffic: QFI is mapped to default DRB | N/A, there is no need to send DL traffic to default DRB. | N/A | N/A. | N/A |

AS reflective QFI to DRB mapping mechanism has negative impact on DL and UL overhead.

AS reflective QFI to DRB mapping mechanism by DRB establishment & modifications always require RRC signalling.

AS reflective QFI to DRB mapping introduces reduced signaling only in a single corner case, but adds significant complexity to the UE and Network implementations.

The AS reflective QFI to DRB mapping requires UE to process every DL packet with SDAP header in order to check changes in the QFI to DRB mapping. There are two main mechanisms to achieve this. Either UE keeps track of the QFIs per DRB and reacts to the changes in the mapping for DRBs where SDAP header is configured. Alternatively, there is a bit indicating in the SDAP header that the mapping has changed. The first option requires UE to process all QFIs while the second option requires UE to react on QFI to DRB mapping change only when SDAP header indicates so. The first option may require more processing than the second option, while in both cases, UE needs to observe changes in SDAP header. This per-packet analysis of in-band signalling in high data rate scenarios will be expensive and may result worse power performance than using conventional RRC signalling (as the QFI mapping is not expected to change often (e.g. every second).

Currently, there are no use cases where QFI to DRB mapping would require extensive remapping of QoS flows. Exception for this would be the default DRB concept, which in the current form, introduces increased more remapping than explicit QFI to DRB mapping. Default DRB topic is discussed in [1].

UE is required to process each DL packet for AS reflective QFI to DRB mapping. This is expensive in high data rate scenarios.

As a solution for the in-band AS reflective QFI to DRB mapping indication, RAN2 made working assumption to bind RQI indication to AS reflective indication. However, based on LS from SA2 [2], this does not work as the RQI needs to be delivered to the UE unaltered. As a result of this, the only option with 7-bit QFI value in the header is to bind the AS QFI to DRB mapping mechanism to NAS Reflective QoS mechanism. In this case, it needs to be acknowledged that binding the NAS Reflective QoS (RQ) and AS QFI to DRB mapping hands the control of AS mechanism over to the 5GC. As the implementation of NAS reflective QoS is optional for UEs, the implementation of AS reflective mechanism will become also optional for UEs. This introduces a question of why the Access Stratum should be complicated with the AS QFI to DRB mapping that can be only used with optional UE feature.

AS reflective QoS mechanism cannot be used with 1-byte SDAP header without binding it to NAS Reflective QoS mechanism.

Binding NAS Reflective QoS mechanism with AS QFI to DRB mapping would make AS-reflective mechanism 5GC controlled and optional to implement, as NAS RQ is optional.

In LTE, the delivery of DRB configuration is done with RRC signalling. Similar mechanism can be used as an only mechanism to convey QFI to DRB mapping. The benefit of this solution is obvious, RRC signalling is an existing solution which is proven to be working. As mentioned previously, the DRB configurations are not expected to change frequently. Therefore, the changes in QFI to DRB mapping are not time-critical such as requiring in-band signalling.

RRC signalling is able to cover all the aspects of QFI to DRB mapping without negative impact on UEs QFI mapping performance. I.e. the AS reflective QFI to DRB mapping is redundant.

From the network perspective, AS reflective QFI to DRB mapping is purely optional, as the mechanism is redundant and all the cases of AS reflective QFI to DRB mapping may be covered with RRC signalling.

Configuration of AS reflective QoS mechanism is in control of the network.

As justified in sections above, current QoS framework can be simplified without removing any functionality by removing AS reflective QoS mechanism.

RAN2 is kindly requested to discuss about removing the AS reflective QFI to DRB mapping from TS 37.324.

If proposal 1 is not agreed, RAN2 is kindly requested to resolve issues described in section 2.5 and P3-P6.

## Issues required to be resolved for AS-reflective QFI to DRB mapping supported.

### RQI indication and Header Size

Based on the LS from SA2, the length of the QFI needs to be at least 7-bits. The 7-bit QFI fits well with the current working assumption of having one-bit RQI and 7-bit QFI.

One feature of the NAS reflective QoS mechanism is that N1 signalling does not exist for RQI and the CN uses RQI is used to reset reflective QoS timer. The LS from SA2 also stated that the RQI bit should be delivered to the UE unchanged. This means that the previous assumption of combining AS-reflective mapping indication with the NAS-RQI cannot work.

To resolve the issue, there are three options.

- Use only RRC signalling to deliver QFI to DRB mapping (no need for AS-reflective mapping mechanism).

- Add 1-bit AS-indication and have 6-bit QFI. This agreement adds complexity to the 5QI to QFI mapping and removes transparency of QFI from UE.

- Have alternative mechanism for indicating the AS-reflective QFI to DRB mapping change. For example, use specific range of QFI values to indicate start of the reflective mapping period.

Figure 1	Impact of the AS reflective mechanism to header format

The SDAP header format is re-designed using one of the options above.

### Mismatch of QoS configuration in Handover when reflective Mechanism is used

In previous meetings, it was decided that the UE follows latest QFI to DRB mapping, whether it is RRC or AS reflective mapping. This introduces a problem when handover is made based on RRC configuration. This issue needs to be resolved if AS reflective mechanism is maintained.

Resolve mismatch of QoS configuration during HO when AS reflective QFI to DRB mapping is used.

### Potential QFI length mismatch in N3 and SDAP header

If the SA2 selects 7-bit QFI and RAN2 selects 6-bit QFI, there is obvious mismatch between headers. This results that the mapping needs to be managed by RAN. Advantages of 7-bit QFI is that the UE is able to directly identify 5QI value from the QFI, which results that there is no need to convey QFI to 5QI mapping in RAN. This simplifies the framework.

Resolve problem of QFI length mismatch in N3/NG-U and SDAP header so that QFI to 5QI mapping in RAN is straightforward.

### Dual connectivity coordination with SDAP header.

When there are MgNB and SgNB connected to a same UE, AS reflective QoS mechanism may be used independently. This introduces a problem where SDAP header needs to be removed or packets need to be dropped when PDCP SDU is moved from source to target node. This is an issue which needs to be resolved already for EN-DC if AS reflective mechanism is not removed.

Resolve DC coordination with SDAP header between MgNB and SgNB.

# Conclusion

In section 2 we made the following observations:

Observation 1	QFI is always signaled via N2 interface according to TS 23.501

Observation 2	RAN3 TS 38.413 specifies that each QFI is associated with a DRB inside PDU session.

Observation 3	AS reflective QFI to DRB mapping mechanism has negative impact on DL and UL overhead.

Observation 4	AS reflective QFI to DRB mapping mechanism by DRB establishment & modifications always require RRC signalling.

Observation 5	AS reflective QFI to DRB mapping introduces reduced signaling only in a single corner case, but adds significant complexity to the UE and Network implementations.

Observation 6	UE is required to process each DL packet for AS reflective QFI to DRB mapping. This is expensive in high data rate scenarios.

Observation 7	AS reflective QoS mechanism cannot be used with 1-byte SDAP header without binding it to NAS Reflective QoS mechanism.

Observation 8	Binding NAS Reflective QoS mechanism with AS QFI to DRB mapping would make AS-reflective mechanism 5GC controlled and optional to implement, as NAS RQ is optional.

Observation 9	RRC signalling is able to cover all the aspects of QFI to DRB mapping without negative impact on UEs QFI mapping performance. I.e. the AS reflective QFI to DRB mapping is redundant.

Observation 10	Configuration of AS reflective QoS mechanism is in control of the network.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 is kindly requested to discuss about removing the AS reflective QFI to DRB mapping from TS 37.324.

Proposal 2	If proposal 1 is not agreed, RAN2 is kindly requested to resolve issues described in section 2.5 and P3-P6.

Proposal 3	The SDAP header format is re-designed using one of the options above.

Proposal 4	Resolve mismatch of QoS configuration during HO when AS reflective QFI to DRB mapping is used.

Proposal 5	Resolve problem of QFI length mismatch in N3/NG-U and SDAP header so that QFI to 5QI mapping in RAN is straightforward.

Proposal 6	Resolve DC coordination with SDAP header between MgNB and SgNB.

# References

R2-1712919. Analysis of the QoS framework (Stage 2). Ericsson. Vancouver, Canada, 22nd – 26th January 2018

S2-178056. Reply LS on SDAP header design. October 23 – 27, 2017, Ljubljana, Slovenia.

S2-178060 [23.501] Clarification of QoS Flows with signalled characteristics. Ericsson. 23-27 October 2017 Ljubljana.

3GPP TS 38.413. NG-RAN NG Application Protocol. V0.4.0 (2017-10)