3GPP TSG-RAN WG2 #AH NR 1801	Tdoc R2-1800709

Vancouver, Canada, Jan 22nd – 26th, 2018

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Alignment of terminology for Configured Scheduling and SPS

Document for:	Discussion, Decision

# Introduction

The terminology for the feature known as SPS in LTE has changed in NR. The new specification TS 38.321 V.15.0. includes now a range of different terminology to define the feature. In this contribution discuss a terminology alignment.

# Discussion

The current specification now use the following diverse terminology

- “Transmission and reception without dynamic scheduling” as umbrella term for uplink and downlink, as section name.

- “Semi-Persistent Scheduling (SPS)” for downlink operation.

- “Transmission without dynamic grant” for uplink operation, where the operation is further described as “configured grant Type 1” and “configured grant Type 2”.

- “CS-RNTI” referring to the RNTI used for both downlink and uplink, CS standing for configured scheduling.

- “Configured Grant Confirmation MAC CE”.

- “configured assignment” for the assignment configured as part of the SPS operation (downlink)

- “configured grant” for the configured grant as part of the operation with “configured grant Type 1” or “configured grant Type 2”.

It becomes obvious that the terminology used in NR became overly complex compared to the terminology used in LTE for SPS, which consisted only of: uplink and downlink SPS and configured grants and assignments. For a reader new to the NR specification, it will thus be very difficult to understand the different terms refer to the same feature. Furthermore, some terms appear lengthy and have no acronym (like the umbrella term), and the specification appears generally not well aligned.

While the current diverse terminology may have been a result from RAN2 intermediate discussions where parts of the LTE SPS feature and new NR proposals regarding uplink grants were merged, the NR specification do not need to reflect the intermediate discussions but rather their final outcome, which can be specified with a clear and well-aligned terminology. We propose the following change to the terminology (does not impact functionality):

- Configured Scheduling (CS) as new umbrella term

  - DL CS as term for downlink operation, using configured downlink assignments

  - UL CS Type 1 and UL CS Type 2, both using configured uplink grants

    - Type 1 and Type2 as currently defined

    - UL CS confirmation MAC CE

  - CS-RNTI for both DL CS and UL CS.

The proposed new terminology had been implemented in [1] where the reduced complexity becomes apparent.

# Conclusions

Use terminology of Configured Scheduling (CS) instead of transmission and reception without dynamic scheduling. Use DL CS instead of SPS, UL CS Type 1 and Type 2 instead of configured grant Type 1 and Type 2, as well as UL CS confirmation MAC CE instead of Configured Grant confirmation MAC CE.

# References

- R2-1800710, Alignment of terminology for Configured Scheduling and SPS, draft CR, Ericsson, RAN2 # AH NR 1801, Vancouver, Canada, Jan 22nd – 26th, 2018