3GPP TS 38.455 V17.11.0 (2025-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NG-RAN;

NR Positioning Protocol A (NRPPa)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	9

2	References	9

3	Definitions, symbols and abbreviations	10

3.1	Definitions	10

3.2	Symbols	10

3.3	Abbreviations	10

4	General	11

4.1	Procedure specification principles	11

4.2	Forwards and backwards compatibility	11

4.3	Specification notations	11

5	NRPPa services	12

5.1	NRPPa procedure modules	12

5.2	Parallel transactions	12

6	Services expected from lower layer	12

7	Functions of NRPPa	12

8	NRPPa procedures	13

8.1	Elementary procedures	13

8.2	Location Information Transfer Procedures	14

8.2.1	E-CID Measurement Initiation	14

8.2.1.1	General	14

8.2.1.2	Successful Operation	15

8.2.1.3	Unsuccessful Operation	15

8.2.1.4	Abnormal Conditions	16

8.2.2	E-CID Measurement Failure Indication	16

8.2.2.1	General	16

8.2.2.2	Successful Operation	16

8.2.2.3	Unsuccessful Operation	16

8.2.2.4	Abnormal Conditions	16

8.2.3	E-CID Measurement Report	16

8.2.3.1	General	16

8.2.3.2	Successful Operation	16

8.2.3.3	Unsuccessful Operation	17

8.2.3.4	Abnormal Conditions	17

8.2.4	E-CID Measurement Termination	17

8.2.4.1	General	17

8.2.4.2	Successful Operation	17

8.2.4.3	Unsuccessful Operation	17

8.2.4.4	Abnormal Conditions	17

8.2.5	OTDOA Information Exchange	18

8.2.5.1	General	18

8.2.5.2	Successful Operation	18

8.2.5.3	Unsuccessful Operation	18

8.2.5.4	Abnormal Conditions	18

8.2.6	Positioning Information Exchange	18

8.2.6.1	General	18

8.2.6.2	Successful Operation	19

8.2.6.3	Unsuccessful Operation	19

8.2.6.4	Abnormal Conditions	20

8.2.7	Positioning Information Update	20

8.2.7.1	General	20

8.2.7.2	Successful Operation	20

8.2.7.3	Unsuccessful Operation	20

8.2.7.4	Abnormal Conditions	20

8.2.8	TRP Information Exchange	20

8.2.8.1	General	20

8.2.8.2	Successful Operation	21

8.2.8.3	Unsuccessful Operation	21

8.2.8.4	Abnormal Conditions	21

8.2.9	Positioning Activation	22

8.2.9.1	General	22

8.2.9.2	Successful Operation	22

8.2.9.3	Unsuccessful Operation	22

8.2.9.4	Abnormal Conditions	23

8.2.10	Positioning Deactivation	23

8.2.10.1	General	23

8.2.10.2	Successful Operation	23

8.2.10.3	Unsuccessful Operation	23

8.2.10.4	Abnormal Conditions	23

8.2.11	PRS Configuration Exchange	23

8.2.11.1	General	23

8.2.11.2	Successful Operation	23

8.2.11.3	Unsuccessful Operation	24

8.2.11.4	Abnormal Conditions	24

8.2.12	Measurement Preconfiguration	24

8.2.12.1	General	24

8.2.12.2	Successful Operation	24

8.2.12.3	Unsuccessful Operation	25

8.2.12.4	Abnormal Conditions	25

8.2.13	Measurement Activation	25

8.2.13.1	General	25

8.2.13.2	Successful Operation	25

8.2.13.3	Unsuccessful Operation	25

8.2.13.4	Abnormal Conditions	26

8.3	Management Procedures	26

8.3.1	Error Indication	26

8.3.1.1	General	26

8.3.1.2	Successful Operation	26

8.3.1.3	Abnormal Conditions	26

8.4	Assistance Information Transfer Procedures	26

8.4.1	Assistance Information Control	26

8.4.1.1	General	26

8.4.1.2	Successful Operation	27

8.4.2	Assistance Information Feedback	27

8.4.2.1	General	27

8.4.2.2	Successful Operation	27

8.4.2.3	Abnormal Conditions	28

8.5	Measurement Information Transfer	28

8.5.1	Measurement	28

8.5.1.1	General	28

8.5.1.2	Successful Operation	28

8.5.1.3	Unsuccessful Operation	29

8.5.1.4	Abnormal Conditions	29

8.5.2	Measurement Report	29

8.5.2.1	General	29

8.5.2.2	Successful Operation	30

8.5.2.3	Abnormal Conditions	30

8.5.3	Measurement Update	30

8.5.3.1	General	30

8.5.3.2	Successful Operation	30

8.5.3.3	Unsuccessful Operation	31

8.5.3.4	Abnormal Conditions	31

8.5.4	Measurement Abort	31

8.5.4.1	General	31

8.5.4.2	Successful Operation	31

8.5.4.3	Unsuccessful Operation	31

8.5.4.4	Abnormal Conditions	31

8.5.5	Measurement Failure Indication	31

8.5.5.1	General	31

8.5.5.2	Successful Operation	32

8.5.5.3	Abnormal Conditions	32

9	Elements for NRPPa Communication	32

9.0	General	32

9.1	Message Functional Definition and Content	32

9.1.1	Messages for Location Information Transfer Procedures	32

9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST	32

9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE	34

9.1.1.3	E-CID MEASUREMENT INITIATION FAILURE	34

9.1.1.4	E-CID MEASUREMENT FAILURE INDICATION	34

9.1.1.5	E-CID MEASUREMENT REPORT	35

9.1.1.6	E-CID MEASUREMENT TERMINATION COMMAND	35

9.1.1.7	OTDOA INFORMATION REQUEST	35

9.1.1.8	OTDOA INFORMATION RESPONSE	36

9.1.1.9	OTDOA INFORMATION FAILURE	37

9.1.1.10	POSITIONING INFORMATION REQUEST	37

9.1.1.11	POSITIONING INFORMATION RESPONSE	37

9.1.1.12	POSITIONING INFORMATION FAILURE	38

9.1.1.13	POSITIONING INFORMATION UPDATE	38

9.1.1.14	TRP INFORMATION REQUEST	38

9.1.1.15	TRP INFORMATION RESPONSE	39

9.1.1.16	TRP INFORMATION FAILURE	39

9.1.1.17	POSITIONING ACTIVATION REQUEST	39

9.1.1.18	POSITIONING ACTIVATION RESPONSE	40

9.1.1.19	POSITIONING ACTIVATION FAILURE	40

9.1.1.20	POSITIONING DEACTIVATION	40

9.1.1.21	PRS CONFIGURATION REQUEST	41

9.1.1.22	PRS CONFIGURATION RESPONSE	41

9.1.1.23	PRS CONFIGURATION FAILURE	42

9.1.1.24	MEASUREMENT PRECONFIGURATION REQUIRED	42

9.1.1.25	MEASUREMENT PRECONFIGURATION CONFIRM	42

9.1.1.26	MEASUREMENT PRECONFIGURATION REFUSE	43

9.1.1.27	MEASUREMENT ACTIVATION	43

9.1.2	Messages for Management Procedures	44

9.1.2.1	ERROR INDICATION	44

9.1.3	Messages for Assistance Information Transfer Procedures	44

9.1.3.1	ASSISTANCE INFORMATION CONTROL	44

9.1.3.2	ASSISTANCE INFORMATION FEEDBACK	44

9.1.4	Messages for Measurement Information Transfer Procedures	45

9.1.4.1	MEASUREMENT REQUEST	45

9.1.4.2	MEASUREMENT RESPONSE	47

9.1.4.3	MEASUREMENT FAILURE	47

9.1.4.4	MEASUREMENT REPORT	48

9.1.4.5	MEASUREMENT UPDATE	48

9.1.4.6	MEASUREMENT ABORT	49

9.1.4.7	MEASUREMENT FAILURE INDICATION	49

9.2	Information Element definitions	49

9.2.0	General	49

9.2.1	Cause	50

9.2.2	Criticality Diagnostics	51

9.2.3	Message Type	51

9.2.4	NRPPa Transaction ID	52

9.2.5	E-CID Measurement Result	52

9.2.6	NG-RAN CGI	55

9.2.7	CGI EUTRA	55

9.2.8	PLMN Identity	56

9.2.9	NR CGI	56

9.2.10	NG-RAN Access Point Position	56

9.2.11	TAC	57

9.2.12	Cell Portion ID	57

9.2.13	Other-RAT Measurement Result	57

9.2.14	WLAN Measurement Result	59

9.2.15	OTDOA Cell Information	60

9.2.16	PRS Muting Configuration EUTRA	62

9.2.17	PRS Frequency Hopping Configuration EUTRA	62

9.2.18	TDD Configuration EUTRA	63

9.2.19	Assistance Information	63

9.2.20	PosSIB Segments	63

9.2.21	Assistance Information Meta Data	64

9.2.22	Positioning SIB Type	64

9.2.23	Assistance Information Failure List	65

9.2.24	TRP ID	65

9.2.25	TRP Information	65

9.2.26	Search Window Information	66

9.2.27	Requested SRS Transmission Characteristics	67

9.2.28	SRS Configuration	68

9.2.29	SRS Resource	69

9.2.30	Positioning SRS Resource	71

9.2.31	SRS Resource Set	72

9.2.32	Positioning SRS Resource Set	73

9.2.33	SRS Resource Set ID	73

9.2.34	Spatial Relation Information	73

9.2.35	SRS Resource Trigger	74

9.2.36	Relative Time 1900	74

9.2.37	TRP Measurement Result	75

9.2.38	UL Angle of Arrival	75

9.2.39	UL RTOA Measurement	75

9.2.40	gNB Rx-Tx Time Difference	76

9.2.41	Additional Path List	76

9.2.42	Time Stamp	77

9.2.43	Measurement Quality	77

9.2.44	PRS Configuration	78

9.2.45	Spatial Direction Information	79

9.2.46	Geographical Coordinates	79

9.2.47	DL-PRS Resource Coordinates	80

9.2.48	Relative Geodetic Location	81

9.2.49	NG-RAN High Accuracy Access Point Position	81

9.2.50	Relative Cartesian Location	82

9.2.51	Reference Point	82

9.2.52	Location Uncertainty	82

9.2.53	Pathloss Reference Information	83

9.2.54	SSB Information	83

9.2.55	SSB Time/Frequency Configuration	83

9.2.56	DL-PRS Muting Pattern	84

9.2.57	Measurement Beam Information	84

9.2.58	NR-PRS Beam Information	84

9.2.59	Positioning Broadcast Cells	85

9.2.60	Spatial Relation Information per SRS Resource	86

9.2.61	Requested DL PRS Transmission Characteristics	86

9.2.62	Requested DL-PRS Resource List	87

9.2.63	Start Time and Duration	87

9.2.64	PRS Transmission Off Information	87

9.2.65	On-demand PRS TRP Information	88

9.2.66	UL-AoA assistance information	90

9.2.67	Z-AoA	90

9.2.68	Response Time	90

9.2.69	LCS to GCS Translation	91

9.2.70	UE Reporting Information	91

9.2.71	Multiple UL-AoA	91

9.2.72	UL SRS-RSRPP	92

9.2.73	SRS Resource type	92

9.2.74	Extended Additional Path List	92

9.2.75	ARP ID	92

9.2.76	ARP Location Information	93

9.2.77	LoS/NLoS Information	93

9.2.78	UE Tx TEG Association List	93

9.2.79	TRP Tx TEG Association	94

9.2.80	TRP TEG Information	94

9.2.81	Measurement Characteristics Request Indicator	95

9.2.82	TRP Beam Antenna Information	95

9.2.83	TRP Beam Antenna Angles	96

9.2.84	Timing Error Margin	97

9.2.85	TRP Rx TEG Information	97

9.2.86	TRP Tx TEG Information	97

9.2.87	TRP RxTx TEG Information	98

9.3	Message and Information Element Abstract Syntax (with ASN.1)	99

9.3.1	General	99

9.3.2	Usage of Private Message Mechanism for Non-standard Use	99

9.3.3	Elementary Procedure Definitions	99

9.3.4	PDU Definitions	106

9.3.5	Information Element definitions	124

9.3.6	Common definitions	187

9.3.7	Constant definitions	188

9.3.8	Container definitions	193

9.4	Message transfer syntax	197

9.5	Timers	197

10	Handling of unknown, unforeseen and erroneous protocol data	197

Annex A (informative):	Change history	198

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the control plane radio network layer signalling procedures between a NG-RAN node and the LMF. NRPPa supports the concerned functions by signalling procedures defined in this document.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	Void.

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 37.355: " Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[15]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[16]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[17]	3GPP TS 36:214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer (PHY); Measurements".

[18]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[19]	3GPP TS 38.215: "NR; Physical layer (PHY); Measurements".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

gNB: as defined in TS 38.300 [3].

NG-RAN node: as defined in TS 38.300 [3].

ng-eNB: as defined in TS 38.300 [3].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

CG-SDT	Configured Grant Small Data Transmission

CID	Cell-ID (positioning method)

DL-PRS	Downlink Positioning Reference Signal

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

LMF	Location Management Function

LPP	LTE Positioning Protocol

MSAS	Multi-functional Satellite Augmentation System

NavIC	NAVigation with Indian Constellation

NRPPa	NR Positioning Protocol A

OTDOA	Observed Time Difference of Arrival

posSIB	Positioning SIB

PRS	Positioning Reference Signal (for E-UTRA)

QZSS	Quasi-Zenith Satellite System

RSRP	Reference Signal Received Power

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

SBAS	Space Based Augmentation System

SRS	Sounding Reference Signal

TEG	Timing Error group

TRP	Transmission-Reception Point

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL SRS-RSRPP	UL SRS reference signal received path power

WAAS	Wide Area Augmentation System

Z-AoA	Zenith Angles of Arrival

# 4	General

## 4.1	Procedure specification principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating NG-RAN Node exactly and completely. Any rule that specifies the behaviour of the originating NG-RAN Node shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which "shall" be executed

The procedure text indicates that the receiving node "shall" perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the initiating message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which "shall, if supported" be executed

The procedure text indicates that the receiving node "shall, if supported," perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see section 10.

## 4.2	Forwards and backwards compatibility

The forwards and backwards compatibility of the protocol is assured by a mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word "procedure", e.g. Handover Preparation procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word "message", e.g. ERROR INDICATION message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation "IE", e.g. Cause IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the "Value" is written as it is specified in sub clause 9.2 enclosed by quotation marks, e.g. "Value".

# 5	NRPPa services

The present clause describes the services an NG -RAN Node offers to the LMF.

## 5.1	NRPPa procedure modules

The procedures are divided into two modules as follows:

1.	NRPPa Location Information Transfer Procedures;

2.	NRPPa Management Procedures;

The NRPPa Location Information Transfer Procedures module contains procedures used to handle the transfer of positioning related information between NG-RAN Node and LMF.

The Management Procedures module contains procedures that are not related specifically to positioning, i.e. error handling.

## 5.2	Parallel transactions

Unless explicitly indicated in the procedure specification, at any instance in time one protocol peer may have more than one ongoing NRPPa procedure.

# 6	Services expected from lower layer

Within 5G RAN, NRPPa protocol uses the services provided by the NGAP protocol. An NRPPa message is carried inside an NGAP message.

NGAP signalling is described in TS 38.413 [2].

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

-	PRS Information Transfer. This function allows the LMF to exchange PRS related information with the NG-RAN node.

-	Measurement Preconfiguration Information Transfer. This function allows the LMF to request the NG-RAN node to preconfigure and activate measurement gap and/or PRS processing window.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |
| PRS Information Transfer | PRS Configuration Exchange |
| Measurement Preconfiguration Information Transfer | Measurement Preconfiguration Measurement Activation |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Measurement Report | MEASUREMENT REPORT |
| Measurement Update | MEASUREMENT UPDATE |
| Measurement Abort | MEASUREMENT ABORT |
| Measurement Failure Indication | MEASUREMENT FAILURE INDICATION |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| Measurement Activation | MEASUREMENT ACTIVATION |

## 8.2	Location Information Transfer Procedures

### 8.2.1	E-CID Measurement Initiation

#### 8.2.1.1	General

The purpose of E-CID Measurement Initiation procedure is to allow the LMF to request the NG-RAN node to report E-CID measurements used by LMF to compute the location of the UE.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-CID Measurement Initiation procedure, successful operation

The LMF initiates the procedure by sending an E-CID MEASUREMENT INITIATION REQUEST message. If the NG-RAN node is able to initiate the requested E-CID measurements, it shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT INITIATION RESPONSE message when measurement results other than the "Cell-ID" have been requested.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the result of the measurement in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the NG-RAN Access Point Position IE or the Geographical Coordinates IE in the E-CID Measurement Result IE, and the LMF shall consider that the E-CID measurements for the UE has been terminated by the NG-RAN node. If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT INITIATION RESPONSE message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement. If the Report Characteristics IE is set to "OnDemand" and the Inter-RAT Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node shall, if supported, provide the corresponding measurements, if available in the NG-RAN node, in the Inter-RAT Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message. If the Report Characteristics IE is set to "OnDemand" and the WLAN Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the NG-RAN node shall, if supported, provide the corresponding measurements, if available in the NG-RAN node, in the WLAN Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message.

If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the requested measurements and shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message without including either the E-CID Measurement Result IE or the Cell Portion ID IE in this message. The NG-RAN node shall then periodically initiate the E-CID Measurement Report procedure for the measurements, with the requested reporting periodicity.

#### 8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: E-CID Measurement Initiation procedure, unsuccessful operation

If the NG-RAN node is not able to initiate at least one of the requested E-CID measurements, the NG-RAN node shall respond with an E-CID MEASUREMENT INITIATION FAILURE message.

#### 8.2.1.4	Abnormal Conditions

Void.

### 8.2.2	E-CID Measurement Failure Indication

#### 8.2.2.1	General

The purpose of the E-CID Measurement Failure Indication procedure is for the NG-RAN node to notify the LMF that the E-CID measurements previously requested with the E-CID Measurement Initiation procedure can no longer be reported.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: E-CID Measurement Failure Indication, successful operation

Upon reception of the E-CID MEASUREMENT FAILURE INDICATION message, the LMF shall consider that the E-CID measurements for the UE have been terminated by the NG-RAN node.

#### 8.2.2.3	Unsuccessful Operation

Not applicable.

#### 8.2.2.4	Abnormal Conditions

Void.

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the NG-RAN node to provide the E-CID measurements for the UE to the LMF.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The NG-RAN node initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the NG-RAN node shall include the NG-RAN Access Point Position IE or the Geographical Coordinates IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this NG-RAN Access Point Position IE, the LMF may use the value as the geographical position of the NG-RAN access point.

If available, the NG-RAN node shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the LMF may use the value as the cell portion for the measurement.

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

#### 8.2.3.4	Abnormal Conditions

Void.

### 8.2.4	E-CID Measurement Termination

#### 8.2.4.1	General

The purpose of E-CID Measurement Termination procedure is to terminate periodical E-CID measurements for the UE performed by the NG-RAN node.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: E-CID Measurement Termination procedure, successful operation

The LMF initiates the procedure by generating an E-CID MEASUREMENT TERMINATION COMMAND message.

#### 8.2.4.3	Unsuccessful Operation

Not applicable.

#### 8.2.4.4	Abnormal Conditions

Void.

### 8.2.5	OTDOA Information Exchange

#### 8.2.5.1	General

The purpose of the OTDOA Information Exchange procedure is to allow the LMF to request the NG-RAN node to transfer OTDOA information to the LMF.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: OTDOA Information Exchange procedure, successful operation

The LMF initiates the procedure by sending an OTDOA INFORMATION REQUEST message. The NG-RAN node responds with OTDOA INFORMATION RESPONSE message that contains the available OTDOA information applicable to the relevant cells/TPs.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: OTDOA Information Exchange procedure, unsuccessful operation

If the NG-RAN node does not have any OTDOA information to report, the NG-RAN node shall respond with an OTDOA INFORMATION FAILURE message.

#### 8.2.5.4	Abnormal Conditions

Void.

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the NG-RAN node is unable to configure any SRS transmissions for the UE, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

If the NG-RAN node is unable to provide any of the requested information, the NG-RAN node shall respond with a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.6.4	Abnormal Conditions

Void.

### 8.2.7	Positioning Information Update

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS configuration or in the UE Tx TEG association. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association List IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider it as the UE Tx TEG association for the SRS resources that have changed their TEG association during the latest reporting interval.

If the SRS Transmission Status IE is included in the POSITIONING INFORMATION UPDATE message and set to "stopped", the LMF shall consider that the SRS transmission has stopped.

#### 8.2.7.3	Unsuccessful Operation

Not Applicable.

#### 8.2.7.4	Abnormal Conditions

Void.

### 8.2.8	TRP Information Exchange

#### 8.2.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the LMF to request the NG-RAN node to provide detailed information for TRPs hosted by the NG-RAN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.8.2	Successful Operation

Figure 8.2.8.2-1: TRP Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a TRP INFORMATION REQUEST message. The NG-RAN node responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the NG-RAN node should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the NG-RAN node should include the requested information for all TRPs hosted by the NG-RAN node in the TRP INFORMATION RESPONSE message

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the muting information for the given PRS resource set.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account for the given PRS resource list.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the LMF may take it into account as the DL PRS Resource Coordinates relative to the TRP coordinate.

#### 8.2.8.3	Unsuccessful Operation

Figure 8.2.8.3-1: TRP Information Exchange procedure, unsuccessful operation

If the NG-RAN node cannot provide any of the requested information for any TRP, the NG-RAN node shall respond with a TRP INFORMATION FAILURE message.

#### 8.2.8.4	Abnormal Conditions

Void.

### 8.2.9	Positioning Activation

#### 8.2.9.1	General

The Positioning Activation procedure is initiated by the LMF to request the NG-RAN node to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Positioning Activation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the NG-RAN node.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the NG-RAN node shall take the indicated value as the LMF’s requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the NG-RAN node shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the LMF shall consider that the respective information indicates the activation time of SRS transmission by the UE.

#### 8.2.9.3	Unsuccessful Operation

Figure 8.2.9.3-1: Positioning Activation procedure, unsuccessful operation

If the NG-RAN node is unable to activate UL SRS transmission in the UE, it shall respond with a POSITIONING ACTIVATION FAILURE message.

If the NG-RAN node is unable to trigger the aperiodic SRS transmission with the indicated SRS Resource Trigger IE, it shall respond with a POSITIONING ACTIVATION FAILURE message with an appropriate cause value.

#### 8.2.9.4	Abnormal Conditions

Void.

### 8.2.10	Positioning Deactivation

#### 8.2.10.1	General

The Positioning Deactivation procedure is initiated by the LMF to indicate to the NG-RAN node that UL SRS transmission should be deactivated in the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Positioning Deactivation procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING DEACTIVATION message to the NG-RAN node. This message shall include an indication of the UL SRS resource set to be deactivated or release all the related resources.

#### 8.2.10.3	Unsuccessful Operation

Not Applicable.

#### 8.2.10.4	Abnormal Conditions

Void.

### 8.2.11	PRS Configuration Exchange

#### 8.2.11.1	General

The PRS Configuration Exchange procedure is initiated by the LMF to request the NG-RAN node to configure or update (i.e., turn off) PRS transmission. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.11.2	Successful Operation

Figure 8.2.11.2-1: PRS Configuration Exchange procedure, successful operation

The LMF initiates the procedure by sending a PRS CONFIGURATION REQUEST message to the NG-RAN.

If the PRS Configuration Request Type IE is set to “configure”, the NG-RAN node should use the information in the Requested DL PRS Transmission Characteristics IE to configure DL-PRS transmission by the indicated TRP(s).

If the PRS Configuration Request Type IE is set to “off”, the NG-RAN node should, if supported, use the information in the PRS Transmission Off Information IE to turn off the DL-PRS transmission for the indicated TRP(s), PRS Resource Set(s), or PRS Resource(s).

If DL-PRS transmission is successfully configured or updated for at least one of the TRPs, the NG-RAN node shall respond with a PRS CONFIGURATION RESPONSE message.

#### 8.2.11.3	Unsuccessful Operation

Figure 8.2.11.3-1: PRS Configuration Exchange procedure, unsuccessful operation

If the NG-RAN node cannot configure or update DL-PRS transmission for any of the TRPs in the PRS TRP List IE of the PRS CONFIGURATION REQUEST message, it shall respond with a PRS CONFIGURATION FAILURE message with an appropriate cause value.

#### 8.2.11.4	Abnormal Conditions

Void.

### 8.2.12	Measurement Preconfiguration

#### 8.2.12.1	General

The Measurement Preconfiguration procedure allows the LMF to provide necessary information to the serving gNB and request the gNB to preconfigure measurement gap and/or PRS processing window for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.12.2	Successful Operation

Figure 8.2.12.2-1: Measurement Preconfiguration procedure, successful operation

The LMF initiates the procedure by sending a MEASUREMENT PRECONFIGURATION REQUIRED message.

If the NG-RAN node is able to configure measurement gap or PRS processing window, it shall reply with the MEASUREMENT PRECONFIGURATION CONFIRM message.

#### 8.2.12.3	Unsuccessful Operation

Figure 8.2.12.3-1: Measurement Preconfiguration procedure, unsuccessful operation

If the NG-RAN node cannot configure any of the measurement gap or PRS processing window, the NG-RAN node shall respond with a MEASUREMENT PRECONFIGURATION REFUSE message. Upon receiving the MEASUREMENT PRECONFIGURATION REFUSE message, the LMF shall release the reserved PPW resources.

#### 8.2.12.4	Abnormal Conditions

Void.

### 8.2.13	Measurement Activation

#### 8.2.13.1	General

The Measurement Activation procedure is initiated by the LMF to request the NG-RAN node to activate or deactivate the preconfigured measurement gap or PRS processing window for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.13.2	Successful Operation

Figure 8.2.13.2-1: Measurement Activation procedure, successful operation

The LMF initiates the procedure by sending a MEASUREMENT ACTIVATION message to the NG-RAN node.

If the PRS Measurement Info List IE is included in the MEASUREMENT ACTIVATION message, the NG-RAN node may take it into account when activating pre-configured measurement gap in the UE.

#### 8.2.13.3	Unsuccessful Operation

Not Applicable.

#### 8.2.13.4	Abnormal Conditions

Void.

## 8.3	Management Procedures

### 8.3.1	Error Indication

#### 8.3.1.1	General

The Error Indication procedure is initiated by a node to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Error Indication procedure, LMF originated, successful operation

Figure 8.3.1.2-2: Error Indication procedure, NG-RAN node originated, successful operation

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE.

#### 8.3.1.3	Abnormal Conditions

Not applicable.

## 8.4	Assistance Information Transfer Procedures

### 8.4.1	Assistance Information Control

#### 8.4.1.1	General

The purpose of the Assistance Information Control procedure is to allow the LMF to signal positioning assistance information to the NG-RAN Node for assistance information broadcasting. This procedure applies only if the NG-RAN node is a gNB.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Assistance Information Control procedure

The LMF initiates the procedure by sending an ASSISTANCE INFORMATION CONTROL message.

If the Assistance Information IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall replace any previously received assistance information with the newly received information, and use the newly received information to configure assistance information broadcasting.

If the Broadcast Priority IE is included in the Assistance Information IE, the NG-RAN Node may take it into account when configuring broadcasting for the relevant information. Assistance information having the same Broadcast Priority value should receive the same treatment (i.e. broadcast by the NG-RAN Node or not broadcast).

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", the NG-RAN Node may start broadcasting the assistance information. If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "stop", the NG-RAN Node may stop broadcasting the assistance information.

If the Positioning Broadcast Cells IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall, if supported, consider that the procedure is applicable only to the indicated cells in this IE.

#### 8.4.1.3	Abnormal Conditions

If the Broadcast IE is set to "start", and the Assistance Information IE is not included, the NG-RAN Node shall consider the procedure as failed.

If the Broadcast IE is not included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN Node shall consider the procedure as failed.

### 8.4.2	Assistance Information Feedback

#### 8.4.2.1	General

The purpose of the Assistance Information Feedback procedure is to allow the NG-RAN Node to give feedback to the LMF on assistance information broadcasting. This procedure applies only if the NG-RAN node is a gNB.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Assistance Information Feedback procedure

If the Assistance Information Failure List IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall consider that assistance information broadcasting could not be configured for the relevant information.

If the Positioning Broadcast Cells IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall consider that the feedback provided is applicable to the cells in this IE.

#### 8.4.2.3	Abnormal Conditions

Void.

## 8.5	Measurement Information Transfer

### 8.5.1	Measurement

#### 8.5.1.1	General

The Measurement procedure allows the LMF to request one or more TRPs in the NG-RAN node to perform and report positioning measurements. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Measurement Beam Information IE in the TRP Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the TRP Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account as the number of SRS measurement time occasions for a measurement instance.

Interaction with the Measurement Report procedure:

If the Report Characteristics IE is set to "Periodic" and the Measurement Amount IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, take it into account for sending the MEASUREMENT REPORT message.

#### 8.5.1.3	Unsuccessful Operation

Figure 8.5.1.3.1: Measurement procedure. Unsuccessful operation.

If the NG-RAN node cannot configure any of the requested measurements for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, it shall respond with a MEASUREMENT FAILURE message with an appropriate cause value.

#### 8.5.1.4	Abnormal Conditions

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message but the NG-RAN node is unable to provide the measurement results within the indicated time, the NG-RAN node shall, if supported, respond with a MEASUREMENT FAILURE message with an appropriate cause value.

### 8.5.2	Measurement Report

#### 8.5.2.1	General

The Measurement Report procedure allows the NG-RAN node to report positioning measurements to the LMF. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2.1: Measurement Report procedure. Successful operation.

The NG-RAN node initiates the procedure by sending a MEASUREMENT REPORT message to the LMF. The MEASUREMENT REPORT message contains the measurement results according to the associated measurement configuration.

#### 8.5.2.3	Abnormal Conditions

Void.

### 8.5.3	Measurement Update

#### 8.5.3.1	General

The Measurement Update Procedure allows the LMF to notify the NG-RAN node of a change in a previously configured measurement. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.3.2	Successful Operation

Figure 8.5.3.2.1: Measurement Update: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT UPDATE message.

If the SRS Configuration IE is included in the MEASUREMENT UPDATE message, the NG-RAN node shall overwrite the previously stored SRS configuration.

If the AoA Search Window Information IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored AoA search window information and store the newly received information.

If the Number of TRP Rx TEGs IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Number of TRP RxTx TEGs IE is included in the TRP Measurement Update List IE in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Measurement Characteristics Request Indicator IE is included in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

If the Measurement Time Occasion IE is included in the MEASUREMENT UPDATE message, the NG-RAN node shall clear any previously stored information and store the newly received information.

#### 8.5.3.3	Unsuccessful Operation

Not applicable.

#### 8.5.3.4	Abnormal Conditions

If the NG-RAN node cannot identify at least one of the previously requested measurement to be modified, it shall consider the procedure as failed and initiate local error handling.

### 8.5.4	Measurement Abort

#### 8.5.4.1	General

The purpose of the Measurement Abort Procedure is to enable the LMF to abort an on-going measurement. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.4.2	Successful Operation

Figure 8.5.4.2.1: Measurement Abort Procedure: Successful Operation.

The LMF initiates the procedure by sending a MEASUREMENT ABORT message.

Upon receiving this message, the NG-RAN node shall terminate the on-going measurement identified by the LMF Measurement ID IE and may release any resources previously allocated for the same measurement.

#### 8.5.4.3	Unsuccessful Operation

Not applicable.

#### 8.5.4.4	Abnormal Conditions

If the NG-RAN node cannot identify the previously requested measurement to be aborted, it shall ignore the MEASUREMENT ABORT message.

### 8.5.5	Measurement Failure Indication

#### 8.5.5.1	General

The Measurement Failure Indication procedure allows the NG-RAN node to notify the LMF that the measurements previously requested with the Measurement procedure can no longer be reported. This procedure applies only if the NG-RAN node is a gNB.

#### 8.5.5.2	Successful Operation

Figure 8.5.5.2.1: Measurement Report procedure. Successful operation.

Upon reception of the MEASUREMENT FAILURE INDICATION message, the LMF shall consider that the indicated measurements have been terminated by the NG-RAN node.

#### 8.5.5.3	Abnormal Conditions

Void.

# 9	Elements for NRPPa Communication

## 9.0	General

Sub clauses 9.1 and 9.2 describe the structure of the messages and information elements required for the NRPPa protocol in tabular format. Sub clause 9.3 provides the corresponding ASN.1 definition.

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [2].

NOTE:	The messages have been defined in accordance to the guidelines specified in TR 25.921 [5].

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Location Information Transfer Procedures

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by LMF to initiate E-CID measurements.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15 ,…, 16..256) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 60min applies only for ng-eNB. The codepoint “extended” is not applicable. This IE is not applicable to NR Angle of Arrival. | YES | reject |
| Measurement Quantities |  | 1 |  |  | EACH | reject |
| >Measurement Quantities Item |  | 1..<maxnoMeas> |  |  | - |  |
| >>Measurement Quantities Value | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…, SS-RSRP, SS-RSRQ, CSI-RSRP, CSI-RSRQ, NR Angle of Arrival, NR Timing Advance) |  | - | - |
| Other-RAT Measurement Quantities |  | 0 |  |  | EACH | ignore |
| >Other-RAT Measurement Quantities Item |  | 0 .. <maxnoMeas> |  |  | - |  |
| >>Other-RAT Measurement Quantities Value | M |  | ENUMERATED (GERAN, UTRAN,…, NR, EUTRA) |  | - |  |
| WLAN Measurement Quantities |  | 0 |  |  | EACH | ignore |
| >WLAN Measurement Quantities Item |  | 0 .. <maxnoMeas> |  |  | - |  |
| >>WLAN Measurement Quantities Value | M |  | ENUMERATED (WLAN, ...) |  | - |  |
| Measurement Periodicity NR-AoA | C- ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA |  | ENUMERATED (160ms, 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 40960ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA | This IE shall be present if the Report Characteristics IE is set to the value "Periodic" and the Measurement Quantities Item IE is set to the value "NR Angle of Arrival". |

#### 9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement is successfully initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Other-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.14 |  | YES | ignore |

#### 9.1.1.3	E-CID MEASUREMENT INITIATION FAILURE

This message is sent by NG-RAN node to indicate that the requested E-CID measurement cannot be initiated.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.4	E-CID MEASUREMENT FAILURE INDICATION

This message is sent by NG-RAN node to indicate that the previously requested E-CID measurement can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

#### 9.1.1.5	E-CID MEASUREMENT REPORT

This message is sent by NG-RAN node to report the results of the requested E-CID measurement.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.2.5 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |

#### 9.1.1.6	E-CID MEASUREMENT TERMINATION COMMAND

This message is sent by the LMF to terminate the requested E-CID measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1..15,…, 16..256) |  | YES | reject |

#### 9.1.1.7	OTDOA INFORMATION REQUEST

This message is sent by LMF to request OTDOA information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| OTDOA Information Type |  | 1 |  |  | EACH | reject |
| >OTDOA Information Type Item |  | 1 .. <maxnoOTDOAtypes> |  |  | - |  |
| >>OTDOA Information Item | M |  | ENUMERATED (pci, cellid, tac, earfcn, prsBandwidth, prsConfigIndex, cpLength, noDlFrames, noAntennaPorts, sFNInitTime, nG-RANAccessPointPosition, prsmutingconfiguration, prsid, tpid, tpType, crsCPlength, dlBandwidth, multipleprsConfigurationsperCell, prsOccasionGroup, prsFrequencyHoppingConfiguration, …, tddConfig) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

#### 9.1.1.8	OTDOA INFORMATION RESPONSE

This message is sent by NG-RAN node to provide OTDOA information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| OTDOA Cells |  | 1 .. <maxCellinRANnode> |  | Served cells/TPs that broadcast PRS. May be used to signal multiple PRS configurations per cell/TPs (up to 3 are supported in this release). | GLOBAL | ignore |
| >OTDOA Cell Information | M |  | 9.2.15 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellinRANnode | Maximum no. cells that can be served by a RAN Node. Value is 16384. |

#### 9.1.1.9	OTDOA INFORMATION FAILURE

This message is sent by NG-RAN node to indicate that the OTDOA information cannot be provided.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

#### 9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |

#### 9.1.1.12	POSITIONING INFORMATION FAILURE

This message is sent by the NG-RAN node to indicate that the positioning information cannot be provided.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by the NG-RAN node to indicate that a change in the SRS configuration or UE Tx TEG association has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| SRS Transmission Status | O |  | ENUMERATED (stopped, ...) |  | YES | ignore |

#### 9.1.1.14	TRP INFORMATION REQUEST

This message is sent by an LMF to request information for TRPs hosted by an NG-RAN node.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP List |  | 0 ..1 |  |  | YES | ignore |
| >TRP Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type List |  | 1 |  |  |  |  |
| >TRP Information Type Item |  | 1 .. <maxnoTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp tx teg, beam antenna info) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

#### 9.1.1.15	TRP INFORMATION RESPONSE

This message is sent by an NG-RAN node to convey TRP information to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP Information List |  | 1 |  |  | YES | ignore |
| >TRP Information Item | M | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP Information | M |  | 9.2.25 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535. |

#### 9.1.1.16	TRP INFORMATION FAILURE

This message is sent by an NG-RAN node to indicate that the requested TRP information cannot be provided to an LMF.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | YES | ignore |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED(true,…) |  | - |  |
| >>SRS Resource Trigger | O |  | 9.2.35 |  | - |  |
| Activation Time | O |  | Relative Time 1900 9.2.36 | Indicates the start time when the SRS activation is requested | YES | ignore |

#### 9.1.1.18	POSITIONING ACTIVATION RESPONSE

This message is sent by NG-RAN node to confirm successful UL SRS activation in the UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

#### 9.1.1.19	POSITIONING ACTIVATION FAILURE

This message is sent by NG-RAN node to indicate that activation of UL SRS transmission in the UE was unsuccessful.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | Ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |
| >Release ALL |  |  | NULL | the NG-RAN node configures UE to stop transmitting SRS for the positioning purpose |  |  |

#### 9.1.1.21	PRS CONFIGURATION REQUEST

This message is sent by the LMF to request the NG-RAN node to configure or update PRS transmission.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| PRS Configuration Request Type | M |  | ENUMERATED(configure, off, …) |  | YES | reject |
| PRS TRP List |  | 1 |  |  | YES | ignore |
| >PRS TRP Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Requested DL PRS Transmission Characteristics | C-ifConf |  | 9.2.61 |  | - |  |
| >>PRS Transmission Off Information | C-ifOff |  | 9.2.64 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |

| Condition | Explanation |
| --- | --- |
| ifConf | This IE shall be present if the PRS Configuration Request Type IE is set to the value "configure". |
| ifOff | This IE shall be present if the PRS Configuration Request Type IE is set to the value "off". |

#### 9.1.1.22	PRS CONFIGURATION RESPONSE

This message is sent by the NG-RAN node to acknowledge configuring or updating the PRS transmission.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| PRS Transmission TRP List |  | 0..1 |  |  | YES | ignore |
| >PRS Transmission TRP Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPs | Maximum no. of TRPs in a NG-RAN node. Value is 65535 |

#### 9.1.1.23	PRS CONFIGURATION FAILURE

This message is sent by the NG-RAN node to indicate that it cannot configure any PRS transmission.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.24	MEASUREMENT PRECONFIGURATION REQUIRED

This message is sent by the LMF to provide the PRS configuration information of multiple TRPs to the NG-RAN node and request to configure measurement gap or PRS processing window of the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| TRP PRS Information List |  | 1 |  |  | YES | ignore |
| >TRP PRS Information Item |  | 1 .. <maxnoPRSTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>NR CGI | O |  | 9.2.9 |  | - |  |
| >>PRS Configuration | M |  | 9.2.44 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPRSTRPs | Maximum no. of TRPs for on-demand PRS in a NG-RAN node. Value is 256. |

#### 9.1.1.25	MEASUREMENT PRECONFIGURATION CONFIRM

This message is sent by the NG-RAN node to the LMF to confirm successful configuration of measurement gap or PRS processing window of the UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Preconfiguration Result | M |  | BIT STRING (SIZE(8)) | Indicates what has been preconfigured in the UE. first bit: measurement gaps second bit: PRS processing windows. Other bits reserved for future use. Value '1' indicates 'has been preconfigured', Value '0' indicates 'not preconfigured'. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.26	MEASUREMENT PRECONFIGURATION REFUSE

This message is sent by the NG-RAN node to indicate that configuration of measurement gap or PRS processing window of the UE was unsuccessful.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.27	MEASUREMENT ACTIVATION

This message is sent by the LMF to request the NG-RAN node to activate or deactivate the preconfigured measurement gap or PRS processing window for the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Request Type | M |  | ENUMERATED (activate, deactivate, …) |  | YES | reject |
| PRS Measurement Info List |  | 0..1 |  |  | YES | Ignore |
| >PRS Measurement Info Item |  | 1 .. < maxFreqLayers> |  |  | - |  |
| >>Point A | M |  | INTEGER (0..3279165) |  | - |  |
| >>MeasPRS Periodicity | M |  | ENUMERATED (ms20, ms40, ms80, ms160, …) | Measurement gap periodicity in units of ms | - |  |
| >>MeasPRS Offset | M |  | INTEGER (0..159, …) | Measurement gap offset in units of subframes | - |  |
| >>Measurement PRS Length | M |  | ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20} |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxFreqLayers | Maximum no. of frequency layers. Value is 4 |

### 9.1.2	Messages for Management Procedures

#### 9.1.2.1	ERROR INDICATION

This message is used to indicate that some error has been detected in the NG-RAN node or in the LMF.

Direction: LMF  NG-RAN node and NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | – |  |
| Cause | O |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

### 9.1.3	Messages for Assistance Information Transfer Procedures

#### 9.1.3.1	ASSISTANCE INFORMATION CONTROL

This message is sent by the LMF to transfer assistance information.

Direction: LMF  NG-RAN Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information | O |  | 9.2.19 |  | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, …) |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.2.59 |  | YES | reject |

#### 9.1.3.2	ASSISTANCE INFORMATION FEEDBACK

This message is sent by the NG-RAN Node to give feedback on assistance information broadcasting.

Direction: NG-RAN Node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information Failure List | O |  | 9.2.23 |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.2.59 | The cells associated to the feedback provided in the Assistance Information Failure List IE. | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, 30min, and 60min are not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >>TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, Multiple UL-AoA, UL SRS-RSRPP) |  | - |  |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) | This IE is ignored when the Measurement Characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.2.68 | This IE is ignored when the Report Characteristics IE is set to “periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |
| Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.4.3	MEASUREMENT FAILURE

This message is sent by the NG-RAN node to report measurement failure.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.4.4	MEASUREMENT REPORT

This message is sent by the NG-RAN node to report positioning measurements for the target UE.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Response List |  | 1 |  |  | YES | reject |
| >TRP Measurement Response Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>TRP Measurement Result | M |  | 9.2.37 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.4.5	MEASUREMENT UPDATE

This message is sent by the LMF to update a previously configured measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| TRP Measurement Update List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Update Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.66 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.2.81 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.4.6	MEASUREMENT ABORT

This message is sent by the LMF to request the NG-RAN node to abort a measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |

#### 9.1.4.7	MEASUREMENT FAILURE INDICATION

This message is sent by the NG-RAN node to indicate that the previously requested measurements can no longer be reported.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

## 9.2	Information Element definitions

### 9.2.0	General

When specifying information elements which are to be represented by bit strings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bit strings from other specifications, the first bit of the bit string contains the first bit of the concerned information.

### 9.2.1	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ..., Serving NG-RAN node changed, Requested Item not Supported on Time ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message), ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The NG-RAN node does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The NG-RAN node can temporarily not provide the requested measurement object or information item. |
| Serving NG-RAN node changed | The UE has moved to another serving NG-RAN node. |
| Requested Item not Supported on Time | The NG-RAN node is unable to provide the measurement results on time. |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerned criticality indicated "reject" (see sub clause 10.3 of TS 38.413) |
| Abstract Syntax Error (Ignore and Notify) | The received message included an abstract syntax error and the concerned criticality indicated "ignore and notify" (see sub clause 10.3 of TS 38.413) |
| Abstract syntax error (falsely constructed message) | The received message contained IEs or IE groups in wrong order or with too many occurrences (see sub clause 10.3 of TS 38.413) |
| Message not Compatible with Receiver State | The received message was not compatible with the receiver state (see sub clause 10.4 of TS 38.413) |
| Semantic Error | The received message included a semantic error (see sub clause 10.4 of TS 38.413) |
| Transfer Syntax Error | The received message included a transfer syntax error (see sub clause 10.2 of TS 38.413) |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related |

| Miscellaneous cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

### 9.2.2	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the NG-RAN node or LMF when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing. The conditions for inclusion of the NRPPa Transaction ID IE are described in clause 10.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED (initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED (reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| NRPPa Transaction ID | O |  | 9.2.4 |  |
| Information Element Criticality Diagnostics |  | 0 .. <maxNrOfErrors> |  |  |
| >IE Criticality | M |  | ENUMERATED (reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value "ignore'" shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type Of Error | M |  | ENUMERATED (not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxNroOfErrors is 256. |

### 9.2.3	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | M |  | INTEGER (0..255) |  |
| Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

### 9.2.4	NRPPa Transaction ID

The NRPPa Transaction ID IE is used to associate all the messages belonging to the same procedure. Messages belonging to the same procedure shall use the same NRPPa Transaction ID.

The NRPPa Transaction ID is determined by the initiating peer of a procedure.

The NRPPa Transaction ID shall uniquely identify a procedure among all ongoing parallel procedures using the same procedure code, and initiated by the same protocol peer.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NRPPa Transaction ID | M |  | INTEGER (0..32767) |  |

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | NG-RAN CGI 9.2.6 | NG-RAN Cell Identifier of the serving cell | - |  |
| Serving Cell TAC | M |  | TAC 9.2.11 | Tracking Area Code of the serving cell | - |  |
| NG-RAN Access Point Position | O |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell. If the Geographical Coordinates IE is used, the NG-RAN Access Point Position IE shall be ignored. | - |  |
| Measured Results |  | 0..1 |  | Measurement results of the serving RAT. | - |  |
| >CHOICE Measured Results Value |  | 1 ..<maxnoMeas> |  |  | - |  |
| >>Value Angle of Arrival EUTRA |  |  | INTEGER (0..719) | According to mapping in TS 36.133 [9] |  |  |
| >>Value Timing Advance Type 1 EUTRA |  |  | INTEGER (0..7690) | As defined in TS 36.214 [17] |  |  |
| >>Value Timing Advance Type 2 EUTRA |  |  | INTEGER (0..7690) | As defined in TS 36.214 [17] |  |  |
| >>Result RSRP EUTRA |  |  |  |  |  |  |
| >>>Result RSRP EUTRA Item |  | 1 .. <maxCellReport> |  |  | - |  |
| >>>>PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>>EARFCN | M |  | INTEGER (0.. 262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>>>CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>>Value RSRP EUTRA | M |  | INTEGER (0..97, …) |  | - |  |
| >>Result RSRQ EUTRA |  |  |  |  |  |  |
| >>>Result RSRQ EUTRA Item |  | 1 . <maxCellReport> |  |  | - |  |
| >>>>PCI EUTRA | M |  | INTEGER (0..503) | Physical Cell Identifier of the reported E-UTRA cell | - |  |
| >>>>EARFCN | M |  | INTEGER (0..262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7] | - |  |
| >>>>CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |
| >>>>Value RSRQ EUTRA | M |  | INTEGER (0..34, …) |  | - |  |
| >>Result SS-RSRP |  |  |  |  | YES | ignore |
| >>>Result SS-RSRP Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>>SS-RSRP per SSB Resource |  | 0..1 |  |  | - |  |
| >>>>>SS-RSRP per SSB Resource Item |  | 1 .. <maxIndexesReport> |  |  | - |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>Result SS-RSRQ |  |  |  |  | YES | ignore |
| >>>ResultSS-RSRQ-Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>>SS-RSRQ per SSB Resource |  | 0..1 |  |  | - |  |
| >>>>>SS-RSRQ PerSSB Resource Item |  | 1 .. <maxIndexesReport> |  |  | - |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>Result CSI-RSRP |  |  |  |  | YES | ignore |
| >>>Result CSI-RSRP Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value CSI-RSRP Cell | O |  | INTEGER (0..127) | CSI-RSRP measurement aggregated at cell level | - |  |
| >>>>CSI-RSRP per CSI-RS Resource |  | 0..1 |  |  | - |  |
| >>>>>CSI-RSRP per CSI-RS Resource Item |  | 1.. <maxIndexesReport> |  |  | - |  |
| >>>>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>>>Value CSI-RSRP | M |  | INTEGER (0..127) | CSI-RSRP measurement per CSI-RS resource | - |  |
| >>Result CSI-RSRQ |  |  |  |  | YES | ignore |
| >>>Result CSI-RSRQ Item |  | 1 .. <maxCellReportNR> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>NR CGI | O |  | 9.2.9 |  | - |  |
| >>>>Value CSI-RSRQ Cell | O |  | INTEGER (0..127) | CSI-RSRQ measurement aggregated at cell level | - |  |
| >>>>CSI-RSRQ per CSI-RS Resource |  | 0..1 |  |  | - |  |
| >>>>>CSI-RSRQ per CSI-RS Resource Item |  | 1 .. <maxIndexesReport> |  |  | - |  |
| >>>>>>CSI-RS Index | M |  | INTEGER (0..95) |  | - |  |
| >>>>>>Value CSI-RSRQ | M |  | INTEGER (0..127) | CSI-RSRQ measurement per CSI-RS resource | - |  |
| >>Angle of Arrival NR |  |  | UL Angle of Arrival 9.2.38 |  | YES | ignore |
| >>Value Timing Advance NR |  |  | INTEGER (0.. 7690) | As defined in TS 38.215 [19] | YES | ignore |
| Geographical Coordinates | O |  | 9.2.46 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |
| maxCellReportNR | Maximum no. of NR cells that can be reported with one message. Value is 9. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

### 9.2.6	NG-RAN CGI

The NG-RAN Cell Global Identifier (CGI) is used to globally identify a cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PLMN identity | M |  | 9.2.8 |  |
| CHOICE NG-RAN Cell | M |  |  |  |
| >NR Cell |  |  |  |  |
| >>NR Cell Identifier | M |  | BIT STRING (36) |  |
| >E-UTRAN Cell |  |  |  |  |
| >>E-UTRAN Cell Identifier | M |  | BIT STRING (28) |  |

### 9.2.7	CGI EUTRA

The Cell Global Identifier EUTRA is used to globally identify an E-UTRA cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PLMN identity | M |  | 9.2.8 |  |
| E-UTRA Cell Identifier | M |  | BIT STRING (28) |  |

### 9.2.8	PLMN Identity

This IE indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | Digits 0 to 9 encoded 0000 to 1001, 1111 used as filler digit. Two digits per octet: - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n PLMN Identity consists of 3 digits from MCC followed by either: - a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or - 3 digits from MNC (in case of 3 digit MNC). |

### 9.2.9	NR CGI

The Cell Global Identifier NR is used to globally identify an NR cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.2.8 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

### 9.2.10	NG-RAN Access Point Position

The NG-RAN Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude "h" expressed in metres is derived from the "uncertainty code" k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER (0..100) | In percentage |

### 9.2.11	TAC

This information element is used to uniquely identify a Tracking Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAC | M |  | OCTET STRING (SIZE (3)) |  |

### 9.2.12	Cell Portion ID

This parameter gives the current Cell Portion associated with the target UE. The Cell Portion ID is the unique identifier for a cell portion within a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Portion ID | M |  | INTEGER (0..4095,…) |  |

### 9.2.13	Other-RAT Measurement Result

The purpose of the Other-RAT Measurement Result information element is to provide the measurement results of RATs other than the serving RAT.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Other-RAT Measured Results |  | 1 |  |  | - |  |
| >CHOICE Other-RAT Measured Results Value |  | 1.. <maxnoMeas> |  |  | - |  |
| >>Result GERAN |  |  |  |  |  |  |
| >>>Result GERAN Item |  | 1..<maxGERANMeas> |  |  | - |  |
| >>>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  | - |  |
| >>>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  | - |  |
| >>>>RSSI | M |  | INTEGER (0..63, ...) |  | - |  |
| >>Result UTRAN |  |  |  |  |  |  |
| >>>Result UTRAN Item |  | 1..<maxUTRANMeas> |  |  | - |  |
| >>>>UARFCN | M |  | INTEGER (0..16383, ...) |  | - |  |
| >>>>CHOICE Physical CellId UTRA | M |  |  |  | - |  |
| >>>>>Physical CellId UTRA FDD |  |  | INTEGER (0..511, ...) |  |  |  |
| >>>>>Physical CellId UTRA TDD |  |  | INTEGER (0..127, ...) |  |  |  |
| >>>>UTRA RSCP | O |  | INTEGER (-5..91, ...) |  | - |  |
| >>>>UTRA EcNo | O |  | INTEGER (0..49, ...) | This IE applies to FDD only. | - |  |
| >>Result NR |  |  |  |  | YES | ignore |
| >>>Result NR Item |  | 1..<maxNRMeas> |  |  | - |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>>SS-RSRP Cell | O |  | INTEGER (0..127) | SS-RSRP measurement aggregated at cell level | - |  |
| >>>>SS-RSRQ Cell | O |  | INTEGER (0..127) | SS-RSRQ measurement aggregated at cell level | - |  |
| >>>>SS-RSRP per SSB Resource |  | 0..1 |  |  | - |  |
| >>>>>Result SS-RSRP Per SSB Item |  | 1..<maxIndexesReport)> |  |  | - |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Value SS-RSRP | M |  | INTEGER (0..127) | SS-RSRP measurement per SSB resource | - |  |
| >>>>SS-RSRQ per SSB Resource |  | 0..1 |  |  | - |  |
| >>>>>Result SS-RSRQ Per SSB Item |  | 1..<maxIndexesReport> |  |  | - |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>>Value SS-RSRQ | M |  | INTEGER (0..127) | SS-RSRQ measurement per SSB resource | - |  |
| >>>>CGI NR | O |  | 9.2.9 | Cell Global Identifier of the reported NR cell | - |  |
| >>Result EUTRA |  |  |  |  | YES | ignore |
| >>>Result EUTRA Item |  | 1..<maxEUTRAMeas> |  |  | - |  |
| >>>>PCI EUTRA | M |  | INTEGER (0..503) |  | - |  |
| >>>>EARFCN | M |  | INTEGER (0..262143) |  | - |  |
| >>>>RSRP EUTRA | O |  | INTEGER (0..97) |  | - |  |
| >>>>RSRQ EUTRA | O |  | INTEGER (0..34) |  | - |  |
| >>>>CGI EUTRA | O |  | 9.2.7 | Cell Global Identifier of the reported E-UTRA cell | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 8. |
| maxEUTRAMeas | Maximum no. of EUTRA cells that can be reported with one message. Value is 8. |
| maxIndexesReport | Maximum no. of beam level measurement results that can be reported with one message. Value is 64. |

### 9.2.14	WLAN Measurement Result

The WLAN Measurement Result information element provides the WLAN measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| WLAN Measured Results |  | 1 |  |  |
| >WLAN Measurement Result Item |  | 1.. <maxnoMeas> |  |  |
| >>WLAN RSSI | M |  | INTEGER (0..141, ...) |  |
| >>SSID | O |  | OCTET STRING (SIZE(1..32)) | Includes the SSID field as defined in subclause 8.4.2.2 of IEEE 802.11™ [11]. |
| >>BSSID | M |  | OCTET STRING (SIZE(6)) | Includes the BSSID field as defined in subclause 8.2.4.3.4 of IEEE 802.11™ [11]. |
| >>HESSID | O |  | OCTET STRING (SIZE(6)) | Includes the HESSID field as defined in subclause 8.4.2.94 of IEEE 802.11™ [11]. |
| >>Operating Class | O |  | INTEGER (0..255) | Indicates the WLAN Operating Class as defined in IEEE 802.11™ [11]. |
| >>Country Code | O |  | ENUMERATED (unitedStates, europe, japan, global, …) | Indicates the WLAN country code as defined in IEEE 802.11™ [11]. |
| >>WLAN Channel List |  | 0..1 |  |  |
| >>>WLAN Channel List Item |  | 1..<maxWLANchannels> |  |  |
| >>>>WLAN Channel | M |  | INTEGER (0..255) | Indicates the WLAN channel number as defined in IEEE 802.11™ [11]. |
| >>WLAN Band | O |  | ENUMERATED (band2dot4, band5, …) | Indicates the WLAN band as defined in IEEE 802.11™ [11]. |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxWLANchannels | Maximum no. of WLAN channels that can be reported within one list. Value is 16. |

### 9.2.15	OTDOA Cell Information

This IE contains OTDOA information of a cell/TP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE OTDOA Cell Information item |  | 1 <maxnoOTDOAtypes> |  |  | - |  |
| >>PCI EUTRA | M |  | INTEGER (0..503, …) | Physical Cell ID of the reported E-UTRA cell. | - |  |
| >>CGI EUTRA | M |  | 9.2.7 | Cell Global Identifier of the E-UTRA cell. | - |  |
| >>TAC | M |  | 9.2.11 | Tracking Area Code | - |  |
| >>EARFCN | M |  | INTEGER (0.. 262143, …) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [7]. | - |  |
| >>PRS Bandwidth EUTRA | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | Transmission bandwidth of PRS | - |  |
| >>PRS Configuration Index EUTRA | M |  | INTEGER (0..4095, ...) | PRS Configuration Index, ref TS 36.211 [10] | - |  |
| >>CP Length EUTRA | M |  | ENUMERATED (Normal, Extended, ...) | Cyclic prefix length of the PRS | - |  |
| >>Number of DL Frames EUTRA | M |  | ENUMERATED (sf1, sf2, sf4, sf6, …) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [10] | - |  |
| >>Number of Antenna Ports EUTRA | M |  | ENUMERATED(n1-or-n2, n4, …) | Number of used antenna ports, where n1-or-n2 corresponds to 1 or 2 ports, n4 corresponds to 4 ports | - |  |
| >>SFN Initialisation Time EUTRA | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. | - |  |
| >>NG-RAN Access Point Position | M |  | 9.2.10 | The configured estimated geographical position of the antenna of the cell/TP. | - |  |
| >>PRS Muting Configuration EUTRA | M |  | 9.2.16 | The configuration of positioning reference signals muting pattern. | - |  |
| >>PRS-ID EUTRA | M |  | INTEGER (0..4095, …) | PRS ID, ref TS 36.211 [10]. | - |  |
| >>TP-ID EUTRA | M |  | INTEGER (0..4095, …) | Identity of the transmission point. This IE together with the PCI and/or PRS-ID may be used to identify the transmission point in case the same physical cell ID is shared by multiple transmission points. | - |  |
| >>TP Type EUTRA | M |  | ENUMERATED (prs-only-tp, …) | A TP which transmits PRS only. | - |  |
| >>Number of DL Frames-Extended EUTRA | M |  | INTEGER (1..160, …) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [10]. | - |  |
| >>CRS CP Length EUTRA | M |  | ENUMERATED (Normal, Extended, ...) | Cyclic prefix length of the CRS. | - |  |
| >>DL Bandwidth EUTRA | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | DL transmission bandwidth expressed in units of resource blocks NRB, ref TS 36.104 [7]. | - |  |
| >>PRS Occasion Group EUTRA | M |  | ENUMERATED (og2, og4, og8, og16, og32, og64, og128, ...) | PRS occasion group in a PRS period, ref TS 36.211 [10]. | - |  |
| >>PRS Frequency Hopping Configuration EUTRA | M |  | 9.2.17 | PRS frequency hopping configuration. | - |  |
| >>TDD Configuration EUTRA | M |  | 9.2.18 | TDD specific physical channel configuration. | YES | ignore |
| >>NR CGI | M |  | 9.2.9 | Cell Global Identifier of the NR cell. | YES | ignore |
| >>SFN Initialisation Time NR | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

### 9.2.16	PRS Muting Configuration EUTRA

The PRS Muting Configuration EUTRA IE is used to describe the configuration of PRS muting patterns for the concerned cell/TP, according to TS 36.211 [10] and TS 36.133 [9].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE PRS Muting Configuration | M |  |  |  |
| >Two |  |  | BIT STRING (2) | If a bit is set to "0", it indicates that the PRS is muted in the corresponding PRS positioning occasion (numbering from any sub frame for which SFN=0) in a periodic cycle of length equal to the length of the bit string |
| >Four |  |  | BIT STRING (4) | Same as above |
| >Eight |  |  | BIT STRING (8) | Same as above |
| >Sixteen |  |  | BIT STRING (16) | Same as above |
| >thirty-two |  |  | BIT STRING (32) | Same as above |
| >sixty-four |  |  | BIT STRING (64) | Same as above |
| >one-hundred-and-twenty-eight |  |  | BIT STRING (128) | Same as above |
| >two-hundred-and-fifty-six |  |  | BIT STRING (256) | Same as above |
| >five-hundred-and-twelve |  |  | BIT STRING (512) | Same as above |
| >one-thousand-and-twenty-four |  |  | BIT STRING (1024) | Same as above |

### 9.2.17	PRS Frequency Hopping Configuration EUTRA

The PRS Frequency Hopping Configuration EUTRA IE is used to describe the configuration of PRS frequency hopping for the concerned cell/TP, according to TS 36.211 [10].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Frequency Hopping Bands | M |  | ENUMERATED (twobands, fourbands, ...) | Number of bands for frequency hopping. |
| Band Positions |  | 1.. <maxnoFreqHoppingBandsMinusOne,...> |  |  |
| >NarrowBand Index | M |  | INTEGER (0..15, ...) | Narrowband Index |

| Range bound | Explanation |
| --- | --- |
| maxnoFreqHoppingBandsMinusOne | Maximum no. of frequency hopping bands minus one. Value is 7. |

### 9.2.18	TDD Configuration EUTRA

The TDD Configuration EUTRA IE is used to specify the TDD specific physical channel configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Subframe Assignment | M |  | ENUMERATED ( sa0, sa1, sa2, sa3, sa4, sa5, sa6, … ) | sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [6, table 4.2-2]. |

### 9.2.19	Assistance Information

This IE contains the assistance information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information | M |  |  |  |
| >System Information |  | 1..<maxNrOfPosSImessage> |  | Corresponds to the number of SI messages with posSIBs to be scheduled |
| >>Broadcast Periodicity | M |  | ENUMERATED (ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ...) | Corresponds to information provided in posSI-Periodicity contained in the PosSI-SchedulingInfo IE as defined in TS 38.331 [13] |
| >>Pos SIBs |  | 1..<maxNrOfPosSIBs> |  | Number of posSIBs in the System Information. |
| >>>PosSIB-Type | M |  | 9.2.22 |  |
| >>>PosSIB Segments | M |  | 9.2.20 |  |
| >>>Assistance Information Meta Data | O |  | 9.2.21 |  |
| >>>Broadcast Priority | O |  | INTEGER (1..16, ...) | The priority of the assistance Information where 1 represents the highest priority and 16 the lowest priority |

| Range bound | Explanation |
| --- | --- |
| maxNrOfPosSImessage | Maximum number of positioning system information messages. Value is 32. |
| maxNrOfPosSIBs | Maximum number of positioning system information blocks included in the message. Value is 32. |

### 9.2.20	PosSIB Segments

This IE provides one posSIB or two or more posSIB segments which must be scheduled in series in consecutive transmissions of the same SI message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSIB Segments |  | 1..<maxNrOfSegments> |  |  |
| >Assistance Data SIB Element | M |  | OCTET STRING | Includes the assistanceDataSIBElement IE as defined in TS 37.355 [14] |

| Range bound | Explanation |
| --- | --- |
| maxNrOfSegments | Maximum number of positioning SIB segments (in case of Assistance Information Element contains segmented data according to TS 37.355 [14]). Value is 64. |

### 9.2.21	Assistance Information Meta Data

This parameter contains meta data for an assistance information element.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encrypted | O |  | ENUMERATED (true, …) | Corresponds to information provided in encrypted contained in the PosSI-SchedulingInfo IE as defined in TS 38.331 [13] |
| GNSS ID | O |  | ENUMERATED (gps, sbas, qzss, galileo, glonass, bds, navic ...) | Corresponds to information provided in gnss-id contained in the PosSI-SchedulingInfo IE as defined in TS 38.331 [13] |
| SBAS ID | O |  | ENUMERATED (waas, egnos, msas, gagan, ...) | Corresponds to to information provided in sbas-id contained in the PosSI-SchedulingInfo IE as defined in TS 38.331 [13] |

### 9.2.22	Positioning SIB Type

This parameter defines a specific positioning SIB, as defined in TS 38.331 [13].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning SIB Type | M |  | ENUMERATED ( posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType2-24, posSibType2-25, posSibType3-1, posSibType4-1, posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3, ..., posSibType1-9, posSibType1-10, posSibType6-4, posSibType6-5, posSibType6-6, dummy-Rel-18-1, dummy-Rel-18-2, posSibType2-17a, posSibType2-18a, posSibType2-20a) | The codepoints ‘dummy-Rel-18-1’ and ‘dummy-Rel-18-2’ are ignored. |

### 9.2.23	Assistance Information Failure List

This parameter identifies the assistance information for which the NG-RAN Node failed to configure broadcasting.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information Failure List |  | 1..<maxnoAssistInfoFailureListItems> |  |  |
| >PosSIB-Type | M |  | 9.2.22 |  |
| >Outcome | M |  | ENUMERATED (failed, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoAssistInfoFailureListItems | Maximum no. of assistance information failure list items that can be signaled with one message. Value is 32. |

### 9.2.24	TRP ID

The TRP ID IE is used to identify a TRP uniquely within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Identifier | M |  | INTEGER (1..65535,…) | Identifies a TRP within an NG-RAN node |

### 9.2.25	TRP Information

The TRP Information IE contains information for one TRP within an NG-RAN node.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.2.24 |  | - |  |
| TRP Information Type |  | 1 .. <maxnoTRPInfoTypes> |  |  | - |  |
| >CHOICE TRP Information Item | M |  |  |  | - |  |
| >>NR PCI |  |  | INTEGER (0..1007) | NR Physical Cell ID |  |  |
| >>NR CGI |  |  | 9.2.9 |  |  |  |
| >>NR ARFCN |  |  | INTEGER (0..3279165) |  |  |  |
| >>PRS Configuration |  |  | 9.2.44 |  |  |  |
| >>SSB Information |  |  | 9.2.54 |  |  |  |
| >>SFN Initialisation Time |  |  | Relative Time 1900 9.2.36 |  |  |  |
| >>Spatial Direction Information |  |  | 9.2.45 |  |  |  |
| >>Geographical Coordinates |  |  | 9.2.46 |  |  |  |
| >>TRP type |  |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [18] | YES | reject |
| >>On-demand PRS TRP Information |  |  | 9.2.65 |  | YES | reject |
| >>TRP Tx TEG Association |  |  | 9.2.79 |  | YES | reject |
| >>TRP Beam Antenna Information |  |  | 9.2.82 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

### 9.2.26	Search Window Information

This information element contains search window information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Expected Propagation Delay | M |  | INTEGER (-3841..3841,…) | Indicates when the SRS is expected to arrive in time at the TRP relative to the UL RTOA Reference Time. The UL RTOA Reference Time for a target SRS is defined as T0+tSRS, where - T0 is the SFN Initialisation Time - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Granularity 4Ts, where Ts=1/(15103 2048) seconds. Centre of the search window. |
| Delay Uncertainty | M |  | INTEGER (1..246,…) | Indicates the uncertainty of the expected SRS arrival time at the TRP Granularity 4Ts, where Ts=1/(151032048) seconds. Single-sided search window. |

### 9.2.27	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of periodic SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth | M |  |  |  | - |  |
| >FR1 |  |  | ENUMERATED (5mHz, 10mHz, 20mHz, 40mHz, 50mHz, 80mHz, 100mHz, ... , rel-18-dummy1, rel-18-dummy2, 15mHz, 25mHz, 30mHz, 60mHz, 35mHz, 45mHz, 70mHz, 90mHz) | The codepoints ‘rel-18-dummy1’ and ‘rel-18-dummy2’ are ignored. |  |  |
| >FR2 |  |  | ENUMERATED (50mHz, 100mHz, 200mHz, 400mHz,… , rel-18-dummy3, 800mHz, 1600mHz, 2000mHz) | The codepoint ‘rel-18-dummy3’ is ignored. |  |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.2.34 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.2.53 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.2.60 |  | - |  |
| SSB Information | O |  | 9.2.54 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

### 9.2.28	SRS Configuration

This information element contains the SRS configuration configured by the NG-RAN node for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1 |  |  |
| >SRS Carrier List Item |  | 1..<maxnoSRS-Carriers> |  |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >>Uplink Channel BW-PerSCS-List |  | 1 |  | Corresponds to sCS-SpecificCarrierList in TS 38.331 [13] |
| >>>SCS Specific Carrier |  | 1. .<maxnoSCSs> |  |  |
| >>>>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs |
| >>>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  |
| >>>>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  |
| >>Active UL BWP | M |  |  | Only the configuration in the active UL BWP is needed. |
| >>>Location And Bandwidth | M |  | INTEGER(0..37949,…) | Corresponds to information provided in locationAndBandwidth contained in BWP IE as defined in TS 38.331 [13] |
| >>>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  |
| >>>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  |
| >>>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  |
| >>>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  |
| >>>SRS Config | M |  |  | Corresponds to information provided in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  |
| >>>>>SRS Resource | M |  | 9.2.29 | Corresponds to information provided in SRS-Resource contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>Positioning SRS Resource List |  | 0..< maxnoSRS-PosResources> |  |  |
| >>>>>Positioning SRS Resource | M |  | 9.2.30 | Corresponds to information provided in SRS-PosResource contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  |
| >>>>>SRS Resource Set | M |  | 9.2.31 | Corresponds to information provided in SRS-ResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  |
| >>>>>Positioning SRS Resource Set | M |  | 9.2.32 | Corresponds to information provided in SRS-PosResourceSet contained in SRS-Config IE as defined in TS 38.331 [13] |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets per UL BWP. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

### 9.2.29	SRS Resource

This information element contains the SRS resource.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| Number of Ports | M |  | ENUMERATED(port1, ports2, ports4) |  | - |  |
| CHOICE Transmission Comb | M |  |  |  | - |  |
| >Comb Two |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  | - |  |
| >Comb Four |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  | - |  |
| >Comb Eight | M |  |  |  | YES | reject |
| >>Comb Offset | M |  | INTEGER(0..7) |  | - | - |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  | - | - |
| Start Position | M |  | INTEGER(0..13) |  | - |  |
| Number of Symbols | M |  | ENUMERATED(n1,n2,n4) | This IE is ignored if the Number of Symbols Extended IE is included | - |  |
| Repetition Factor | M |  | ENUMERATED(r1,r2,r4) | This IE is ignored if the Repetition Factor Extended IE is included | - |  |
| Frequency Domain Position | M |  | INTEGER(0..67) |  | - |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  | - |  |
| C-SRS | M |  | INTEGER(0..63) |  | - |  |
| B-SRS | M |  | INTEGER(0..3) |  | - |  |
| B-Hop | M |  | INTEGER(0..3) |  | - |  |
| Group or Sequence Hopping | M |  | ENUMERATED(neither, groupHopping, sequenceHopping) |  | - |  |
| CHOICE Resource Type | M |  |  |  | - |  |
| >Periodic |  |  |  |  | - |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  | - |  |
| >>Offset | M |  | INTEGER(0..2559, …) |  | - |  |
| >Semi-persistent |  |  |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  | - |  |
| >>Offset | M |  | INTEGER(0..2559, …) |  | - |  |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic Resource Type | M |  | ENUMERATED(true,…) |  | - |  |
| Sequence ID | M |  | INTEGER(0..1023) |  | - |  |
| Number of Symbols Extended | O |  | ENUMERATED(n8,n10,n12, n14, …) |  | YES | ignore |
| Repetition Factor Extended | O |  | ENUMERATED(r3, r5, r6, r7, r8, r10, r12, r14, …) |  | YES | ignore |
| Start RB Hopping | O |  | ENUMERATED(enable) |  | YES | ignore |
| CHOICE Start RB Index | O |  |  |  | YES | ignore |
| >FreqScalingFactor2 |  |  | INTEGER (0..1) |  |  |  |
| >FreqScalingFactor4 |  |  | INTEGER (0..3) |  |  |  |

### 9.2.30	Positioning SRS Resource

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Transmission Comb | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  |
| >Comb Eight |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(n1,n2,n4, n8, n12} |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type Positioning | M |  |  |  |
| >periodic |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >semi-persistent |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot 1, slot 2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >aperiodic |  |  |  |  |
| >>slot offset | M |  | INTEGER(0..32) |  |
| Sequence ID | M |  | INTEGER(0..65535) |  |
| CHOICE Spatial Relation Positioning | O |  |  |  |
| >SSB |  |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  |
| >>SSB index | O |  | INTEGER(0..63) |  |
| >PRS |  |  |  |  |
| >>PRS ID | M |  | INTEGER(0..255) |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>PRS Resource ID | O |  | INTEGER(0..63) |  |

### 9.2.31	SRS Resource Set

This information element indicates an SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Set Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>periodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>semi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |
| >>Slot offset | M |  | INTEGER(0..32) | Offset in number of slots, where value 0 indicates no offset. |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourcePerSet | Maximum no of SRS resources per SRS resource set. Value is 16. |

### 9.2.32	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |
| >Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| CHOICE Resource Type | M |  |  |  |
| >periodic |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  |
| >semi-persistent |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >aperiodic |  |  |  |  |
| >>SRS Resource Trigger | M |  | INTEGER(1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

### 9.2.33	SRS Resource Set ID

This information element indicates a resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set ID | M |  | INTEGER (0..15) | Corresponds to information provided in SRS-ResourceSetId contained in SRS-Config IE as defined in TS 38.331 [13] |

### 9.2.34	Spatial Relation Information

This information element indicates a spatial relation for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Spatial Relation for Resource ID |  | 1..<maxnoSpatialRelations> |  | According to TS 38.321 [15] and corresponds to information provided in SRS-SpatialRelationInfoPos contained in SRS-Config IE as defined in TS 38.331 [13] |
| >CHOICE Reference Signal | M |  |  |  |
| >>NZP CSI-RS |  |  |  |  |
| >>>NZP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>SSB |  |  |  |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>SSB Index | O |  | INTEGER (0..63) |  |
| >>SRS |  |  |  |  |
| >>>SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>Positioning SRS |  |  |  |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>DL-PRS |  |  |  |  |
| >>>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>>DL-PRS Resource ID | O |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSpatialRelations | Maximum no. of Spatial Relations that can be configured. Value is 64. |

### 9.2.35	SRS Resource Trigger

This information element indicates a DCI code point according to a SRS resource set configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Aperiodic SRS Resource Trigger List |  | 1..<maxnoSRS-TriggerStates> |  | Corresponds to information provided in aperiodicSRS-ResourceTriggerList contained in SRS-Config IE as defined in TS 38.331 [13] |
| >Aperiodic SRS Resource Trigger |  |  | INTEGER (1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRSTriggerStates | Maximum no. of SRS trigger states. Value is 3. |

### 9.2.36	Relative Time 1900

This information element indicates the initialisation time (e.g. SFN Initalisation Time for a cell, requested time for an action, etc).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  | 9.2.38 |  |  |  |
| >>UL SRS-RSRP |  |  | INTEGER (0..126) |  |  |  |
| >>UL RTOA |  |  | 9.2.39 |  |  |  |
| >>gNB Rx-Tx Time Difference |  |  | 9.2.40 |  |  |  |
| >>Z-AoA |  |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA |  |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP |  |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

### 9.2.38	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [16] |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [16] |
| LCS to GCS Translation | O |  | 9.2.69 | If absent, the azimuth and zenith are provided in GCS. |

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |  |  |
| >k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |  |  |
| >k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |  |  |
| >k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |  |  |
| >k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |  |  |
| >k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |  |  |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.2.85 |  | YES | ignore |

### 9.2.40	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 |  |  | INTEGER (0.. 1970049) | TS 38.133 [16] |  |  |
| >k1 |  |  | INTEGER (0.. 985025) | TS 38.133 [16] |  |  |
| >k2 |  |  | INTEGER (0.. 492513) | TS 38.133 [16] |  |  |
| >k3 |  |  | INTEGER (0.. 246257) | TS 38.133 [16] |  |  |
| >k4 |  |  | INTEGER (0.. 123129) | TS 38.133 [16] |  |  |
| >k5 |  |  | INTEGER (0.. 61565) | TS 38.133 [16] |  |  |
| Additional Path List | O |  | 9.2.41 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.2.74 |  | YES | ignore |
| TRP TEG Information | O |  | 9.2.80 |  | YES | ignore |

### 9.2.41	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 |  |  | INTEGER(0..16351) |  |  |  |
| >>k1 |  |  | INTEGER(0..8176) |  |  |  |
| >>k2 |  |  | INTEGER(0..4088) |  |  |  |
| >>k3 |  |  | INTEGER(0..2044) |  |  |  |
| >>k4 |  |  | INTEGER(0..1022) |  |  |  |
| >>k5 |  |  | INTEGER(0..511) |  |  |  |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  | - |  |
| >Multiple UL-AoA | O |  | 9.2.71 |  | YES | ignore |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

### 9.2.42	Time Stamp

This information element contains the time stamp.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  | - |  |
| CHOICE Slot Index | M |  |  |  | - |  |
| >SCS-15 |  |  | INTEGER(0..9) |  | - |  |
| >SCS-30 |  |  | INTEGER(0..19) |  | - |  |
| >SCS-60 |  |  | INTEGER(0..39) |  | - |  |
| >SCS-120 |  |  | INTEGER(0..79) |  | - |  |
| >SCS-480 | M |  | INTEGER(0..319) |  | YES | reject |
| >SCS-960 | M |  | INTEGER(0..639) |  | YES | reject |
| Measurement time | O |  | Relative Time 1900 9.2.36 |  | - |  |

### 9.2.43	Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  | Corresponds to information provided in NR-TimingQuality IE as defined in TS 37.355 [14] |
| >>Measurement Quality | M |  | INTEGER(0..31) |  |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) |  |
| >Angle Measurement Quality |  |  |  |  |
| >>Azimuth Quality | M |  | INTEGER(0..255) |  |
| >>Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |

### 9.2.44	PRS Configuration

This information element contains the DL PRS configuration for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource Set List |  | 1 |  |  |
| >PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …) |  |
| >>PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >>Start PRB | M |  | INTEGER(0..2176) | Starting PRB to Point A |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >>Comb Size | M |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >>CP Type | M |  | ENUMERATED(normal, extended, …) |  |
| >>Resource Set Periodicity | M |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…, 128, 256, 512) | Slots |
| >>Resource Set Slot Offset | M |  | INTEGER(0..81919,…) |  |
| >>Resource Repetition Factor | M |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  |
| >>Resource Time Gap | M |  | ENUMERATED(tg1,tg2,tg4,tg8,tg16,tg32,…) |  |
| >>Resource Number of Symbols | M |  | ENUMERATED(n2,n4,n6,n12,…) |  |
| >>PRS Muting | O |  |  |  |
| >>>Option1 | O |  |  |  |
| >>>>Muting Pattern | M |  | DL-PRS Muting Pattern 9.2.56 | Muting pattern option 1 is used to mute the whole PRS resource set (within a period) |
| >>>>Muting Bit Repetition Factor | M |  | ENUMERATED(1,2,4,8,…) |  |
| >>>Option2 | O |  |  |  |
| >>>>Muting Pattern | M |  | DL-PRS Muting Pattern 9.2.56 | Muting pattern option 2 is used to mute the selected repetition of the resource set (within the period) |
| >>PRS Resource Transmit Power | M |  | INTEGER(-60..50) |  |
| >>PRS Resource List |  | 1 |  | Corresponds to information provided in NR-DL-PRS-Resource contained in NR-DL-PRS-Info IE as defined in TS 37.355 [14] |
| >>>PRS Resource Item |  | 1..<maxnoofPRSresources> |  |  |
| >>>>PRS Resource ID | M |  | INTEGER(0..63) |  |
| >>>>Sequence ID | M |  | INTEGER(0..4095) |  |
| >>>>RE Offset | M |  | INTEGER(0..11,…) |  |
| >>>>Resource Slot Offset | M |  | INTEGER(0..511) |  |
| >>>>Resource Symbol Offset | M |  | INTEGER(0..12) |  |
| >>>>CHOICE QCL Info | O |  |  |  |
| >>>>>SSB |  |  |  |  |
| >>>>>>NR PCI | M |  | INTEGER(0..1007) |  |
| >>>>>>SSB Index | O |  | INTEGER(0..63) |  |
| >>>>>DL-PRS |  |  |  |  |
| >>>>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) | If it is absent, the QCL source PRS resource ID is the same as the PRS resource ID |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

### 9.2.45	Spatial Direction Information

This information element contains the spatial direction information of the DL PRS resources for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information | M |  | 9.2.58 | The spatial directions of DL-PRS Resources for TRP |

### 9.2.46	Geographical Coordinates

This information element contains the geographical coordinates for the TRP and any associated ARP(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  | - |  |
| >Direct |  |  |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  | - |  |
| >>>normal accuracy |  |  |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. | - |  |
| >>>high accuracy |  |  |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.49 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. | - |  |
| >Referenced |  |  |  |  |  |  |
| >>Reference Point | M |  | 9.2.51 | The reference point is used to derive the TRP position | - |  |
| >>CHOICE Type | M |  |  |  | - |  |
| >>>Geodetic |  |  |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP | - |  |
| >>>Cartesian |  |  |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP | - |  |
| DL-PRS Resource Coordinates | O |  | 9.2.47 | DL-PRS Resource Coordinates relative to the TRP coordinate | - |  |
| ARP Location Information | O |  | 9.2.76 |  | YES | ignore |

### 9.2.47	DL-PRS Resource Coordinates

This information element contains the geographical coordinates of the antenna reference points (ARP) for the DL-PRS Resources of a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DL-PRS Resource Set ARP List | M | 1..<maxPRS-ResourceSets> |  |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >CHOICE DL-PRS Resource Set ARP Location | M |  |  | Relative to the geographical coordinates for the TRP. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource Set ID. |
| >>Geodetic |  |  |  |  |
| >>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 |  |
| >>Cartesian |  |  |  |  |
| >>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 |  |
| >DL-PRS Resource ARP List | M | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |
| >>CHOICE DL-PRS Resource ARP Location | M |  |  | Relative to the DL-PRS Resource Set ARP Location. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource ID. |
| >>>Geodetic |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 |  |
| >>>Cartesian |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 |  |

| Range bound | Explanation |
| --- | --- |
| maxPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

### 9.2.48	Relative Geodetic Location

This information element provides a location relative to some known reference location in a relative geodetic coordinate system. Corresponds to information provided in RelativeLocation IE as defined in TS 37.355 [14].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Milli-Arc-Second Units | M |  | ENUMERATED (0.03, 0.3, 3, ...) | Units and scale factor for the delta-latitude and delta-longitude fields. 0.03, 0.3, 3, milliarcseconds. |
| Height Units | M |  | ENUMERATED (mm, cm, m, ...) | Units and scale factor for the delta-height field. 10-3 metre, 10-2 metre. |
| Delta Latitude | M |  | INTEGER (-1024..1023) | Delta value in latitude in the unit provided in Milli-Arc-Second Units. |
| Delta Longitude | M |  | INTEGER (-1024..1023) | Delta value in longitude in the unit provided in Milli-Arc-Second Units. |
| Delta Height | M |  | INTEGER (-1024..1023) | Delta value in ellipsoidal height in the unit provided in Height Units. |
| Location uncertainty | M |  | 9.2.52 |  |

### 9.2.49	NG-RAN High Accuracy Access Point Position

The NG-RAN High Accuracy Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Degrees of Latitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Degrees of Longitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Altitude | M |  | INTEGER(-64000..1280000) |  |
| Uncertainty Semi Major | M |  | INTEGER (0..255) |  |
| Uncertainty Semi Minor | M |  | INTEGER (0..255) |  |
| Orientation Major Axis | M |  | INTEGER (0..179) |  |
| Horizontal Confidence | M |  | INTEGER (0..100) |  |
| Uncertainty Altitude | M |  | INTEGER (0..255) |  |
| Vertical Confidence | M |  | INTEGER (0..100) |  |

### 9.2.50	Relative Cartesian Location

This information element provides a location relative to some known reference location in a relative Cartesian coordinate system.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| XYZ unit | M |  | ENUMERATED (mm, cm, dm,..) |  |
| X value | M |  | INTEGER (-216.. 216-1) | Positive value represents easting from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-216.. 216-1) | Positive value represents northing from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-215.. 215-1) | Height with respect to reference point in units of XYZ Unit IE, where the XY-plane is horizontal and the Z-axis points up. |
| Location uncertainty | M |  | 9.2.52 |  |

### 9.2.51	Reference Point

This information element provides a reference point information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE ReferencePoint | M |  |  | Reference point to which relative location information is related to |
| >Coordinate ID |  |  |  |  |
| >>Coordinate ID | M |  | INTEGER(0.. 29-1,..) | Referential ID mapped via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>Reference Point Position | M |  | NG-RAN Access Point Position 9.2.10 |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>Reference Point High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.49 |  |

### 9.2.52	Location Uncertainty

This information element provides the location uncertainty information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Horizontal Uncertainty | M |  | INTEGER (0..255) | Horizontal uncertainty of the ARP latitude/longitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [8] |
| Horizontal Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [8]. |
| Vertical Uncertainty | M |  | INTEGER (0..255) | Vertical uncertainty of the ARP altitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [8] |
| Vertical Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [8]. |

### 9.2.53	Pathloss Reference Information

This information element indicates a pathloss reference for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Pathloss Reference Signal | M |  |  |  |
| >SSB |  |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | O |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL PRS Resource ID | O |  | INTEGER (0..63) |  |

### 9.2.54	SSB Information

This information element contains the SSB time/frequency information for the TRPs.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB Info List |  | 1 |  |  |
| >SSB Info Item |  | 1…<maxNoSSBs> |  |  |
| >>SSB Configuration | M |  | SSB Time/Frequency Configuration 9.2.55 |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  |

| Range bound | Explanation |
| --- | --- |
| maxNoSSBs | Maximum no of SSBs for which the configuration can be provided. Value is 255. |

### 9.2.55	SSB Time/Frequency Configuration

This information element contains the time and frequency configuration of an SSB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB frequency | M |  | INTEGER (0..3279165) | ARFCN |
| SSB subcarrier spacing | M |  | ENUMERATED(15kHz, 30kHz, 60kHz, 120kHz, 240kHz,... , kHz480, kHz960) | The value 60kHz is not supported in this version of the specification. |
| SSB Transmit power | M |  | INTEGER (-60..50) | EPRE of SSS |
| SSB periodicity | M |  | ENUMERATED(5ms, 10ms, 20ms, 40ms, 80ms, 160ms, …) |  |
| SSB half frame index | M |  | INTEGER(0..1) |  |
| SSB SFN offset | M |  | INTEGER(0..15) |  |
| CHOICE SSB Position in Burst | O |  |  |  |
| >Short Bitmap |  |  | BIT STRING (SIZE(4)) |  |
| >Medium Bitmap |  |  | BIT STRING (SIZE(8)) |  |
| >Long Bitmap |  |  | BIT STRING (SIZE(64)) |  |
| SFN initialisation time | O |  | Relative Time 1900 9.2.36 |  |

### 9.2.56	DL-PRS Muting Pattern

This information element contains the DL-PRS muting pattern.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE DL-PRS Muting Pattern | M |  |  |  |
| >Two |  |  | BIT STRING (SIZE(2)) |  |
| >Four |  |  | BIT STRING (SIZE(4)) |  |
| >Six |  |  | BIT STRING (SIZE(6)) |  |
| >Eight |  |  | BIT STRING (SIZE(8)) |  |
| >Sixteen |  |  | BIT STRING (SIZE(16)) |  |
| >Thirty-two |  |  | BIT STRING (SIZE(32)) |  |

### 9.2.57	Measurement Beam Information

This information element contains the receiving beam information when measuring UL signals.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource ID | O |  | INTEGER(0..63) |  |
| PRS Resource Set ID | O |  | INTEGER(0..7) |  |
| SSB Index | O |  | INTEGER(0..63) |  |

### 9.2.58	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PRS Beam Information |  | 1 |  |  | - |  |
| >NR-PRS Beam Information Item |  | 1.. < maxPRS-ResourceSets > |  |  | - |  |
| >>PRS Resource Set ID | M |  | INTEGER (0..7) | The resource set in which the resources are associated with the angle. | - |  |
| >>PRS Angle |  | 1 |  |  | - |  |
| >>>PRS Angle Item |  | 1.. ..< maxPRS-ResourcesPerSet > |  |  | - |  |
| >>>>NR PRS Azimuth | M |  | INTEGER (0..359) |  | - |  |
| >>>>NR PRS Azimuth fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>>>NR PRS Elevation | O |  | INTEGER (0..180) |  | - |  |
| >>>>NR PRS Elevation fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>>>PRS Resource ID | O |  | INTEGER(0..63) |  | YES | ignore |
| LCS to GCS Translation List |  | 0..1 |  | If absent, the azimuth and elevation are provided in GCS. | - |  |
| >LCS to GCS Translation Item |  | 1..<maxnolcs-gcs-translation> |  |  | - |  |
| >>Alpha | M |  | INTEGER (0..359) |  | - |  |
| >>Alpha-fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>Beta | M |  | INTEGER (0..359) |  | - |  |
| >>Beta-fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>Gamma | M |  | INTEGER (0..359) |  | - |  |
| >>Gamma-fine | O |  | INTEGER (0..9) | Fine angles | - |  |

| Range bound | Explanation |
| --- | --- |
| maxPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |
| maxnolcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 3. The current version of the specification supports 1. |

### 9.2.59	Positioning Broadcast Cells

This IE is used to indicate the cells that are requested to broadcast, or failed to broadcast, the associated posSIB(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning Broadcast Cells |  | 1 .. <maxnoBcastCell> |  |  |
| >NG-RAN-CGI | M |  | 9.2.6 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoBcastCells | Maximum no. of cells broadcasting a posSIB in a NG-RAN node. Value is 16384. |

### 9.2.60	Spatial Relation Information per SRS Resource

This information element indicates a spatial relation for transmission of each UL SRS resource recommended by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Spatial Relation per SRS Resource List |  | 1 |  |  |
| >Spatial Relation per SRS Resource Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >CHOICE Reference Signal | M |  |  |  |
| >NZP CSI-RS |  |  |  |  |
| >>NZP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >SSB |  |  |  |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | O |  | INTEGER (0..63) |  |
| >SRS |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER (0..63) |  |
| >Positioning SRS |  |  |  |  |
| >>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL-PRS Resource ID | O |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourcePerSet | Maximum no of SRS resources per SRS resource set. Value is 16. |

### 9.2.61	Requested DL PRS Transmission Characteristics

This IE contains the requested PRS configuration for transmission by the LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested DL-PRS Resource Set List |  | 1 |  |  |
| >Requested DL-PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>PRS bandwidth | O |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >>Comb Size | O |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >>Resource Set Periodicity | O |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…, 128, 256, 512) | Slots |
| >>Resource Repetition Factor | O |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  |
| >>Resource Number of Symbols | O |  | ENUMERATED(n2,n4,n6,n12,…) |  |
| >>Requested DL-PRS Resource List | O |  | 9.2.62 |  |
| >>Resource Set Start Time and Duration | O |  | Start Time and Duration 9.2.63 | This IE is ignored if the Start Time and Duration IE is present |
| Number of Frequency Layers | O |  | INTEGER(1..4) |  |
| Start Time and Duration | O |  | 9.2.63 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |

### 9.2.62	Requested DL-PRS Resource List

This IE contains the requested DL-PRS resource list.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested DL-PRS Resource List |  | 1 |  | Corresponds to information provided in NR-DL-PRS-Resource contained in NR-DL-PRS-Info IE as defined in TS 37.355 [14] |
| >Requested DL-PRS Resource Item |  | 1..<maxnoofPRSresource> |  |  |
| >>CHOICE QCL Info | O |  |  |  |
| >>>SSB |  |  |  |  |
| >>>>NR PCI | M |  | INTEGER(0..1007) |  |
| >>>>SSB Index | O |  | INTEGER(0..63) |  |
| >>>DL-PRS |  |  |  |  |
| >>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

### 9.2.63	Start Time and Duration

This IE contains the start time and/or duration for the on-demand DL-PRS.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Start Time | O |  | Relative Time 1900 9.2.36 |  |
| Duration | O |  | INTEGER (0..90060, …) | Unit: seconds |

### 9.2.64	PRS Transmission Off Information

This IE contains the information to turn off particular PRS transmissions.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE level | M |  |  |  |
| >TRP level |  |  | NULL |  |
| >PRS resource set level |  |  |  |  |
| >>PRS Resource Set List |  | 1 |  |  |
| >>>PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>>>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >PRS resource level |  |  |  |  |
| >>PRS Resource Set List |  | 1 |  |  |
| >>>PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>>>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>PRS Resource List |  | 1 |  |  |
| >>>>>PRS Resource Item |  | 1..<maxnoofPRSresource> |  |  |
| >>>>>>PRS Resource ID | M |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

### 9.2.65	On-demand PRS TRP Information

This IE contains on-demand PRS information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| On-demand PRS Request Allowed | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents an on-demand PRS transmission parameter: first bit: Resource Set Periodicity second bit: PRS Bandwidth third bit: Resource Repetition Factor fourth bit: Resource Number of Symbols fifth bit: Comb Size sixth bit: Number of Frequency Layers seventh bit: Start Time and Duration eighth bit: Off Indication ninth bit: QCL Information Other bits reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. |
| Allowed Resource Set Periodicity Values | O |  | BIT STRING (SIZE(24)) | This IE applies only if the first bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Set Periodicity IE defined in subclause 9.2.61, first bit = 4 and so on. Bit 24 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Set Periodicity values are allowed to be requested. |
| Allowed PRS Bandwidth Values | O |  | BIT STRING (SIZE(64)) | This IE applies only if the second bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the PRS Bandwidth IE defined in subclause 9.2.61, first bit = 1 and so on. Bit 64 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all PRS Bandwidth values are allowed to be requested. |
| Allowed Resource Repetition Factor Values | O |  | BIT STRING (SIZE(8)) | This IE applies only if the third bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Repetition Factor IE defined in subclause 9.2.61, first bit = rf1 and so on. Bit 8 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Repetition Factor values are allowed to be requested. |
| Allowed Resource Number of Symbols Values | O |  | BIT STRING (SIZE(8)) | This IE applies only if the fourth bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Number of Symbols IE defined in subclause 9.2.61, first bit = n2 and so on. Bits 5-8 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Number of Symbols values are allowed to be requested. |
| Allowed Comb Size Values | O |  | BIT STRING (SIZE(8)) | This IE applies only if the fifth bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Comb Size IE defined in subclause 9.2.61, first bit = 2 and so on. Bits 5-8 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Comb Size values are allowed to be requested. |

### 9.2.66	UL-AoA assistance information

This information element contains the expected uplink Angle of Arrival and uncertainty range.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE AngleMeasurement | M |  |  |  |
| >Expected UL Angle of Arrival |  |  |  |  |
| >>Expected Azimuth AoA |  | 1 |  | Defined as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) |
| >>>Expected Azimuth AoA Value | M |  | INTEGER(0..3599) | φAOA component of Expected Azimuth AoA |
| >>>Expected Azimuth AoA Uncertainty Range | M |  | INTEGER(0..3599) | ΔφAOA component of Expected Azimuth AoA |
| >>Expected Zenith AoA |  | 0..1 |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >>>Expected Zenith AoA Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >>>Expected Zenith AoA Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| >Expected UL Angle of Arrival Zenith Only |  |  |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >>Expected Zenith AoA Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >>Expected Zenith AoA Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| LCS to GCS Translation | O |  | 9.2.69 | If absent, the azimuth and zenith are provided in GCS. In case of zenith only, the z-axis of LCS is defined along the linear array axis. |

### 9.2.67	Z-AoA

This information element contains the Zenith Angle of Arrival information, which can correspond to linear array measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Zenith Angle of Arrival | M |  | INTEGER(0..1799) | TS 38.133 [16] |
| LCS to GCS Translation | O |  | 9.2.69 | If absent, the zenith is provided in GCS. the z-axis of LCS is defined along the linear array axis |

### 9.2.68	Response Time

This information element contains the response time of the measurement results reporting.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time | M |  | INTEGER(1..128,…) |  |
| Time Unit | M |  | ENUMERATED (second, ten-seconds, ten-milliseconds, …) |  |

### 9.2.69	LCS to GCS Translation

This information element contains the LCS to GCS Translation information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Alpha | M |  | INTEGER (0..3599) |  |
| Beta | M |  | INTEGER (0..3599) |  |
| Gamma | M |  | INTEGER (0..3599) |  |

### 9.2.70	UE Reporting Information

This IE contains the UE Reporting Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | Value 0 represents an infinite number of periodic reporting |
| Reporting Interval | M |  | ENUMERATED (none, 1, 2, 4, 8, 10, 16, 20, 32, 64) | Unit: seconds |

### 9.2.71	Multiple UL-AoA

This information element contains the list of the multiple UL-AOAs values.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL AoA List |  | 1 |  |  |
| >UL AoA item |  | 1..<maxnoofULAoAs > |  |  |
| >>CHOICE AngleMeasurement | M |  |  |  |
| >>>UL Angle of Arrival |  |  |  |  |
| >>>>UL Angle of Arrival | M |  | 9.2.38 |  |
| >>>UL Zenith Angle of Arrival |  |  |  |  |
| >>>>UL Zenith Angle of Arrival | M |  | Z-AoA 9.2.67 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofULAoAs | Maximum no of UL-AOAs values (pair of AOA & ZOA values) that can be reported. Value is 8 |

### 9.2.72	UL SRS-RSRPP

This information element contains the UL SRS RSRPP measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| First Path RSRP Power | M |  | INTEGER (0..126) |  |

### 9.2.73	SRS Resource type

This IE contains the SRS resource type.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Reference Signal | M |  |  |  | - |  |
| >SRS |  |  |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| >Positioning SRS |  |  |  |  |  |  |
| >>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| SRS Port Index | O |  | ENUMERATED(id1000, id1001, id1002, id1003, …) | This IE may be present if the SRS Resource ID IE is present, and is ignored otherwise. | YES | ignore |

### 9.2.74	Extended Additional Path List

This IE contains the extended additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxNoPathExtended> |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |
| >>k0 |  |  | INTEGER(0..16351) |  |
| >>k1 |  |  | INTEGER(0..8176) |  |
| >>k2 |  |  | INTEGER(0..4088) |  |
| >>k3 |  |  | INTEGER(0..2044) |  |
| >>k4 |  |  | INTEGER(0..1022) |  |
| >>k5 |  |  | INTEGER(0..511) |  |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  |
| >Multiple UL-AoA | O |  | 9.2.71 |  |
| >Path Power | O |  | UL SRS-RSRPP 9.2.72 |  |

| Range bound | Explanation |
| --- | --- |
| maxNoPathExtended | Maximum no. of additional path measurement. Value is 8. |

### 9.2.75	ARP ID

This IE is used to uniquely identify an ARP associated with a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| ARP Identifier | M |  | INTEGER (1..16, …) |  |

### 9.2.76	ARP Location Information

This IE contains the relative position of ARP(s) to the TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ARP Location Information |  | 1 |  |  |
| >ARP Location Information Item |  | 1..<maxnoARPs> |  |  |
| >>ARP ID | M |  | 9.2.75 |  |
| >>CHOICE ARP Location Type | M |  |  |  |
| >>>geodetic |  |  |  |  |
| >>>>ARP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 |  |
| >>>cartesian |  |  |  |  |
| >>>>ARP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoARPs | Maximum no. of ARPs associated with a TRP. Value is 16. |

### 9.2.77	LoS/NLoS Information

This IE contains the LoS/NLoS information for UL measurement.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE LoS/NLoS Indicator | M |  |  |  |
| >Soft Indicator |  |  |  |  |
| >>LoS/NLoS Indicator Soft | M |  | INTEGER (0..10) | Values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS. |
| >Hard Indicator |  |  |  |  |
| >>LoS/NLoS Indicator Hard | M |  | ENUMERATED (NLoS, LoS) |  |

### 9.2.78	UE Tx TEG Association List

This information element contains the list of UE Tx TEG associations.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UE Tx TEG Association item |  | 1..<maxnoUETEGs> |  |  | - |  |
| >UE Tx TEG ID | M |  | INTEGER (0..7) |  | - |  |
| >Positioning SRS Resource ID List |  | 1 |  |  | - |  |
| >>Positioning SRS Resource ID Item |  | 1..<maxnoSRS-PosResources> |  |  | - |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Carrier Frequency |  | 0..1 |  | Indicates the frequency of the positioning SRS resources | - |  |
| >>Point A | M |  | INTEGER (0..3279165) | NR ARFCN | - |  |
| >>Offset to Carrier | M |  | INTEGER (0.2199, …) |  | - |  |
| >UE Tx Timing Error Margin | O |  | Timing Error Margin 9.2.84 | Timing error margin associated to the UE Tx TEG ID. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoUETEGs | Maximum no of reported UE Tx TEG associations. Value is 256. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources. Value is 64. |

### 9.2.79	TRP Tx TEG Association

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP TEG item |  | 1..<maxnoTRPTEGs> |  |  |
| >TRP Tx TEG Information | M |  | 9.2.86 |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >DL-PRS Resource ID List |  | 0..1 |  |  |
| >>DL-PRS Resource ID Item |  | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPTEGs | Maximum no of reported TRP Tx TEG association. Value is 8. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

### 9.2.80	TRP TEG Information

This information element contains the TRP TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TRP TEG | M |  |  |  |
| >RxTx TEG |  |  |  |  |
| >>TRP RxTx TEG Information | M |  | 9.2.87 |  |
| >>TRP Tx TEG Information | O |  | 9.2.86 |  |
| >Rx TEG |  |  |  |  |
| >>TRP Rx TEG Information | M |  | 9.2.85 |  |
| >>TRP Tx TEG Information | M |  | 9.2.86 |  |

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: Additional Path Power Fourth Bit: Multiple UL AoA of Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG-ID information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

### 9.2.82	TRP Beam Antenna Information

The IE provides the beam antenna information of the TRP. It includes either the explicit beam antenna information, or a reference to another TRP’s signalled configuration, or the indication that no change has occurred with respect to previously signalled configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Beam Antenna Info Item | M |  |  |  |
| >Reference |  |  |  |  |
| >>Associated TRP ID | M |  | TRP ID 9.2.24 | This IE specifies the TRP ID of the associated TRP from which the beam information parameters are adopted in Local Coordinate System (LCS). |
| >Explicit |  |  |  |  |
| >>TRP Beam Antenna Angles | M |  | 9.2.83 |  |
| >>LCS to GCS Translation | O |  | 9.2.69 | Included if the azimuth and elevation are not provided in GCS. |
| >No Change |  |  | NULL | No change compared to the previously signalled configuration for this TRP. |

### 9.2.83	TRP Beam Antenna Angles

The IE provides the beam antenna information of the TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP Beam Antenna Angles Item |  | 1..< maxnoAzimuthAngles> |  |  |
| >TRP Azimuth Angle | M |  | INTEGER (0..359) | For GCS, the azimuth angle is measured counter-clockwise from geographical North. For LCS, the azimuth angle is measured counter-clockwise from the x-axis of the LCS. |
| >TRP Azimuth Angle fine | O |  | INTEGER (0..9) | Fine angle |
| >TRP Elevation Angle List |  | 1 |  |  |
| >>TRP Elevation Angle Item |  | 1..<maxnoElevationAngles> |  |  |
| >>>TRP Elevation Angle | M |  | INTEGER (0..180) | For GCS, the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For LCS, the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). |
| >>>TRP Elevation Angle fine | O |  | INTEGER (0..9) | Fine angle |
| >>>TRP Beam Power List |  | 1 |  | Relative power between DL-PRS Resources for the given Azimuth and Elevation Angle. The first Relative Power element in this list provides the peak power for this Azimuth/Elevation angle and is defined as 0dB power. All the remaining Relative Power Element's in this list provide the relative DL-PRS Resource power relative to this first element in the list. |
| >>>>TRP Beam Power Item |  | 2..< maxNumResourcesPerAngle> |  |  |
| >>>>>PRS Resource Set ID | O |  | INTEGER (0..7) | DL-PRS Resource Set ID of the DL-PRS Resource for which the Relative Power is provided. If this field is absent, the DL-PRS Resource Set ID for this instance of the Beam Power List is the same as the DL-PRS Resource Set ID of the previous instance in the Beam Power List. This field shall be included at least in the first instance of the Beam Power List. |
| >>>>>PRS Resource ID | M |  | INTEGER (0..63) | DL-PRS Resource for which the Relative Power is provided. |
| >>>>>TRP Beam Relative Power | M |  | INTEGER (0..30) | The power values span from -30 to 0dB |
| >>>>>TRP Beam Relative Power "fine" | O |  | INTEGER (0..9) | Relative Power with 0.1dB resolution. The power spans from -0.9 to 0dB |

| Range bound | Explanation |
| --- | --- |
| maxNumResourcesPerAngle | Maximum number of DL-PRS Resources per angle per TRP. Value is 24. |
| maxnoAzimuthAngles | Maximum number of azimuth angles per TRP. Value is 3600. |
| maxnoElevationAngles | Maximum number of elevation angles per azimuth angle/TRP. Value is 1801. |

### 9.2.84	Timing Error Margin

This information element contains the Timing error margin for the UE Tx TEG, TRP Rx TEG, or TRP Tx TEG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timing Error Margin | M |  | ENUMERATED(Tc0, Tc2, Tc4, Tc6, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc56, Tc64, Tc72, Tc80, …) |  |

### 9.2.85	TRP Rx TEG Information

This information element contains the TRP Rx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP Rx TEG ID | M |  | INTEGER (0..31) |  |
| TRP Rx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Rx TEG ID. |

### 9.2.86	TRP Tx TEG Information

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP Tx TEG ID | M |  | INTEGER (0..7) |  |
| TRP Tx Timing Error Margin | M |  | Timing Error Margin 9.2.84 | Timing error margin associated to the TRP Tx TEG ID. |

### 9.2.87	TRP RxTx TEG Information

This information element contains the TRP RxTx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP RxTx TEG ID | M |  | INTEGER (0..255) |  |
| TRP RxTx Timing Error Margin | M |  | ENUMERATED (Tc0dot5, Tc1, Tc2, Tc4, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc64, Tc80, Tc96, Tc128, …) | Timing error margin associated to the TRP RxTx TEG ID. |

## 9.3	Message and Information Element Abstract Syntax (with ASN.1)

### 9.3.1	General

Sub clause 9.3 presents the Abstract Syntax of the NRPPa protocol with ASN.1. In case there is contradiction between the ASN.1 definition in this sub clause and the tabular format in sub clause 9.1 and 9.2, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional elements, in which the tabular format shall take precedence.

The ASN.1 definition specifies the structure and content of NRPPa messages. NRPPa messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrence being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct an NRPPa message according to the PDU definitions module and with the following additional rules (Note that in the following, "IE" means an IE in the object set with an explicit id. If one IE needs to appear more than once in one object set, then the different occurrences have different IE ids):

-	IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

-	Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e. an IE list) then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an IE container list in which the list elements reside. The second part defines list elements. The IE container list appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list elements.

If an NRPPa message that is not constructed as defined above is received, this shall be considered as Abstract Syntax Error, and the message shall be handled as defined for Abstract Syntax Error in clause 10.

### 9.3.2	Usage of Private Message Mechanism for Non-standard Use

The private message mechanism for non-standard use may be used:

-	for special operator (and/or vendor) specific features considered not to be part of the basic functionality, i.e. the functionality required for a complete and high-quality specification in order to guarantee multivendor inter-operability.

-	by vendors for research purposes, e.g. to implement and evaluate new algorithms/features before such features are proposed for standardisation.

The private message mechanism shall not be used for basic functionality. Such functionality shall be standardised.

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NRPPA-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode,

NRPPATransactionID

FROM NRPPA-CommonDataTypes

ErrorIndication,

PrivateMessage,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

OTDOAInformationRequest,

OTDOAInformationResponse,

OTDOAInformationFailure,

AssistanceInformationControl,

AssistanceInformationFeedback,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningInformationUpdate,

MeasurementRequest,

MeasurementResponse,

MeasurementFailure,

MeasurementReport,

MeasurementUpdate,

MeasurementAbort,

MeasurementFailureIndication,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation

FROM NRPPA-PDU-Contents

id-errorIndication,

id-privateMessage,

id-e-CIDMeasurementInitiation,

id-e-CIDMeasurementFailureIndication,

id-e-CIDMeasurementReport,

id-e-CIDMeasurementTermination,

id-oTDOAInformationExchange,

id-assistanceInformationControl,

id-assistanceInformationFeedback,

id-positioningInformationExchange,

id-positioningInformationUpdate,

id-Measurement,

id-MeasurementReport,

id-MeasurementUpdate,

id-MeasurementAbort,

id-MeasurementFailureIndication,

id-tRPInformationExchange,

id-positioningActivation,

id-positioningDeactivation,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation

FROM NRPPA-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

NRPPA-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

NRPPA-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			NRPPA-ELEMENTARY-PROCEDURE.&procedureCode		({NRPPA-ELEMENTARY-PROCEDURES}),

criticality			NRPPA-ELEMENTARY-PROCEDURE.&criticality			({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

nrppatransactionID		NRPPATransactionID,

value					NRPPA-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NRPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

NRPPA-ELEMENTARY-PROCEDURES NRPPA-ELEMENTARY-PROCEDURE ::= {

NRPPA-ELEMENTARY-PROCEDURES-CLASS-1			|

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

NRPPA-ELEMENTARY-PROCEDURES-CLASS-1 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementInitiation	|

oTDOAInformationExchange	|

positioningInformationExchange	|

measurement						|

tRPInformationExchange		|

positioningActivation		|

pRSConfigurationExchange	|

measurementPreconfiguration,

...

}

NRPPA-ELEMENTARY-PROCEDURES-CLASS-2 NRPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementFailureIndication		|

e-CIDMeasurementReport					|

e-CIDMeasurementTermination				|

errorIndication							|

privateMessage							|

assistanceInformationControl			|

assistanceInformationFeedback 			|

positioningInformationUpdate			|

measurementReport						|

measurementUpdate						|

measurementAbort						|

measurementFailureIndication		|

positioningDeactivation		|

measurementActivation,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

e-CIDMeasurementInitiation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-e-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-e-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-e-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-e-CIDMeasurementTermination

CRITICALITY				reject

}

oTDOAInformationExchange NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OTDOAInformationRequest

SUCCESSFUL OUTCOME		OTDOAInformationResponse

UNSUCCESSFUL OUTCOME	OTDOAInformationFailure

PROCEDURE CODE			id-oTDOAInformationExchange

CRITICALITY				reject

}

assistanceInformationControl NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationControl

PROCEDURE CODE			id-assistanceInformationControl

CRITICALITY				reject

}

assistanceInformationFeedback NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationFeedback

PROCEDURE CODE			id-assistanceInformationFeedback

CRITICALITY				reject

}

errorIndication NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY			ignore

}

privateMessage			NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY			ignore

}

positioningInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-positioningInformationExchange

CRITICALITY				reject

}

positioningInformationUpdate	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-positioningInformationUpdate

CRITICALITY				ignore

}

measurement	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementRequest

SUCCESSFUL OUTCOME		MeasurementResponse

UNSUCCESSFUL OUTCOME	MeasurementFailure

PROCEDURE CODE			id-Measurement

CRITICALITY				reject

}

measurementReport	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementReport

PROCEDURE CODE			id-MeasurementReport

CRITICALITY				ignore

}

measurementUpdate	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementUpdate

PROCEDURE CODE			id-MeasurementUpdate

CRITICALITY				ignore

}

measurementAbort	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementAbort

PROCEDURE CODE			id-MeasurementAbort

CRITICALITY				ignore

}

measurementFailureIndication	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementFailureIndication

PROCEDURE CODE			id-MeasurementFailureIndication

CRITICALITY				ignore

}

tRPInformationExchange	NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-tRPInformationExchange

CRITICALITY				reject

}

positioningActivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-positioningActivation

CRITICALITY				reject

}

positioningDeactivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-positioningDeactivation

CRITICALITY				ignore

}

pRSConfigurationExchange NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PRSConfigurationRequest

SUCCESSFUL OUTCOME		PRSConfigurationResponse

UNSUCCESSFUL OUTCOME	PRSConfigurationFailure

PROCEDURE CODE			id-pRSConfigurationExchange

CRITICALITY				reject

}

measurementPreconfiguration NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementPreconfigurationRequired

SUCCESSFUL OUTCOME		MeasurementPreconfigurationConfirm

UNSUCCESSFUL OUTCOME		MeasurementPreconfigurationRefuse

PROCEDURE CODE			id-measurementPreconfiguration

CRITICALITY				reject

}

measurementActivation NRPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivation

PROCEDURE CODE			id-measurementActivation

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NRPPa

--

-- **************************************************************

NRPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

UE-Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

Cell-Portion-ID,

OtherRATMeasurementQuantities,

OtherRATMeasurementResult,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList,

SRSConfiguration,

TRPMeasurementQuantities,

TrpMeasurementResult,

TRP-ID,

TRPInformationTypeListTRPReq,

TRPInformationListTRPResp,

TRP-MeasurementRequestList,

TRP-MeasurementResponseList,

TRP-MeasurementUpdateList,

MeasurementBeamInfoRequest,

PositioningBroadcastCells,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

TRPList,

AbortTransmission,

SystemFrameNumber,

SlotNumber,

RelativeTime1900,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

UEReportingInformation,

UETxTEGAssociationList,

TRP-PRS-Information-List,

PRS-Measurements-Info-List,

UE-TEG-Info-Request,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

PRSConfigRequestType,

MeasurementAmount,

PreconfigurationResult,

RequestType,

UE-TEG-ReportingPeriodicity,

MeasurementPeriodicityNR-AoA,

SRSTransmissionStatus

FROM NRPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-Single-Container{},

NRPPA-PRIVATE-IES,

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-LMF-Measurement-ID,

id-LMF-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-RAN-Measurement-ID,

id-RAN-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-Cell-Portion-ID,

id-OtherRATMeasurementQuantities,

id-OtherRATMeasurementResult,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList,

id-SRSConfiguration,

id-TRPMeasurementQuantities,

id-MeasurementResult,

id-TRP-ID,

id-TRPInformationTypeListTRPReq,

id-TRPInformationListTRPResp,

id-TRP-MeasurementRequestList,

id-TRP-MeasurementResponseList,

id-TRP-MeasurementReportList,

id-TRP-MeasurementUpdateList,

id-MeasurementBeamInfoRequest,

id-PositioningBroadcastCells,

id-SRSType,

id-ActivationTime,

id-SRSResourceSetID,

id-TRPList,

id-SRSSpatialRelation,

id-AbortTransmission,

id-SystemFrameNumber,

id-SlotNumber,

id-SRSResourceTrigger,

id-SFNInitialisationTime,

id-SRSSpatialRelationPerSRSResource,

id-MeasurementPeriodicityExtended,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-UEReportingInformation,

id-UETxTEGAssociationList,

id-TRP-PRS-Information-List,

id-PRS-Measurements-Info-List,

id-UE-TEG-Info-Request,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-PRSConfigRequestType,

id-MeasurementAmount,

id-PreconfigurationResult,

id-RequestType,

id-UE-TEG-ReportingPeriodicity,

id-MeasurementPeriodicityNR-AoA,

id-SRSTransmissionStatus

FROM NRPPA-Constants;

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION REQUEST

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID			CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics				PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity				PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” --

{ ID id-MeasurementQuantities			CRITICALITY reject	TYPE MeasurementQuantities				PRESENCE mandatory}|

{ ID id-OtherRATMeasurementQuantities	CRITICALITY ignore	TYPE OtherRATMeasurementQuantities		PRESENCE optional}|

{ ID id-WLANMeasurementQuantities		CRITICALITY ignore	TYPE WLANMeasurementQuantities			PRESENCE optional}|

{ ID id-MeasurementPeriodicityNR-AoA	CRITICALITY reject	TYPE MeasurementPeriodicityNR-AoA		PRESENCE conditional},

-- The IE shall be present if the Report Characteritics IE is set to “periodic” and the MeasurementQuantities-Item IE in the MeasurementQuantities IE is set to the value "angleOfArrivalNR" --

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION RESPONSE

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID					PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID					PRESENCE optional}|

{ ID id-OtherRATMeasurementResult	CRITICALITY ignore	TYPE OtherRATMeasurementResult		PRESENCE optional}|

{ ID id-WLANMeasurementResult		CRITICALITY ignore	TYPE WLANMeasurementResult			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT INITIATION FAILURE

--

-- **************************************************************

E-CIDMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementInitiationFailure-IEs}},

...

}

E-CIDMeasurementInitiationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

E-CIDMeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementFailureIndication-IEs}},

...

}

E-CIDMeasurementFailureIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT TERMINATION

--

-- **************************************************************

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-UE-Measurement-ID		CRITICALITY reject	TYPE UE-Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION REQUEST

--

-- **************************************************************

OTDOAInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationRequest-IEs}},

...

}

OTDOAInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Group		CRITICALITY reject	TYPE OTDOA-Information-Type			PRESENCE mandatory},

...

}

OTDOA-Information-Type ::= SEQUENCE (SIZE(1..maxnoOTDOAtypes)) OF ProtocolIE-Single-Container { { OTDOA-Information-Type-ItemIEs} }

OTDOA-Information-Type-ItemIEs	NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Item		CRITICALITY reject	TYPE OTDOA-Information-Type-Item	PRESENCE mandatory},

...

}

OTDOA-Information-Type-Item ::= SEQUENCE {

oTDOA-Information-Item		OTDOA-Information-Item,

iE-Extensions					ProtocolExtensionContainer { { OTDOA-Information-Type-ItemExtIEs} } OPTIONAL,

...

}

OTDOA-Information-Type-ItemExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION RESPONSE

--

-- **************************************************************

OTDOAInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationResponse-IEs}},

...

}

OTDOAInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OTDOACells						CRITICALITY ignore	TYPE OTDOACells					PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- OTDOA INFORMATION FAILURE

--

-- **************************************************************

OTDOAInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{OTDOAInformationFailure-IEs}},

...

}

OTDOAInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ASSISTANCE INFORMATION CONTROL

--

-- **************************************************************

AssistanceInformationControl ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationControl-IEs}},

...

}

AssistanceInformationControl-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Assistance-Information	CRITICALITY reject	TYPE Assistance-Information		PRESENCE optional}|

{ ID id-Broadcast				CRITICALITY reject	TYPE Broadcast					PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ASSISTANCE INFORMATION FEEDBACK

--

-- **************************************************************

AssistanceInformationFeedback ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationFeedback-IEs}},

...

}

AssistanceInformationFeedback-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-AssistanceInformationFailureList	CRITICALITY reject	TYPE AssistanceInformationFailureList	PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells					PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs NRPPA-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION REQUEST

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationRequest-IEs}},

...

}

PositioningInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional	}|

{ ID id-UEReportingInformation					CRITICALITY ignore	TYPE UEReportingInformation							PRESENCE optional	}|

{ ID id-UE-TEG-Info-Request						CRITICALITY ignore	TYPE UE-TEG-Info-Request								PRESENCE optional	}|

{ ID id-UE-TEG-ReportingPeriodicity				CRITICALITY reject	TYPE UE-TEG-ReportingPeriodicity					PRESENCE conditional	},

-- The IE shall be present if the UE TEG Info Request IE is set to “periodic”

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION RESPONSE

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationResponse-IEs}},

...

}

PositioningInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-UETxTEGAssociationList		CRITICALITY ignore	TYPE UETxTEGAssociationList		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION FAILURE

--

-- **************************************************************

PositioningInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PositioningInformationFailure-IEs}},

...

}

PositioningInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION UPDATE

--

-- **************************************************************

PositioningInformationUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PositioningInformationUpdate-IEs}},

...

}

PositioningInformationUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-UETxTEGAssociationList		CRITICALITY ignore	TYPE UETxTEGAssociationList		PRESENCE optional}|

{ ID id-SRSTransmissionStatus		CRITICALITY ignore	TYPE SRSTransmissionStatus	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REQUEST

--

-- **************************************************************

MeasurementRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementRequest-IEs}},

...

}

MeasurementRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList		CRITICALITY reject	TYPE TRP-MeasurementRequestList	PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics		PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” –

{ ID id-TRPMeasurementQuantities		CRITICALITY reject	TYPE TRPMeasurementQuantities	PRESENCE mandatory}|

{ ID id-SFNInitialisationTime			CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-SRSConfiguration				CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest		CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional}|

{ ID id-SystemFrameNumber				CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional}|

{ ID id-SlotNumber						CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional}|

{ ID id-MeasurementPeriodicityExtended	CRITICALITY reject	TYPE MeasurementPeriodicityExtended PRESENCE conditional}|

-- The IE shall be present the MeasurementPeriodicity IE is set to the value "extended"

{ ID id-ResponseTime					CRITICALITY ignore	TYPE ResponseTime				PRESENCE optional}|

{ ID id-MeasurementCharacteristicsRequestIndicator			CRITICALITY ignore	TYPE MeasurementCharacteristicsRequestIndicator	PRESENCE optional}|

{ ID id-MeasurementTimeOccasion			CRITICALITY ignore	TYPE MeasurementTimeOccasion	PRESENCE optional}|

{ ID id-MeasurementAmount				CRITICALITY ignore	TYPE MeasurementAmount			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT RESPONSE

--

-- **************************************************************

MeasurementResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementResponse-IEs}},

...

}

MeasurementResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementResponseList	CRITICALITY reject	TYPE TRP-MeasurementResponseList PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT FAILURE

--

-- **************************************************************

MeasurementFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{MeasurementFailure-IEs}},

...

}

MeasurementFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT REPORT

--

-- **************************************************************

MeasurementReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementReport-IEs}},

...

}

MeasurementReport-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementReportList			CRITICALITY reject	TYPE TRP-MeasurementResponseList PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT UPDATE

--

-- **************************************************************

MeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementUpdate-IEs}},

...

}

MeasurementUpdate-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-TRP-MeasurementUpdateList	CRITICALITY reject	TYPE TRP-MeasurementUpdateList	PRESENCE optional}|

{ ID id-MeasurementCharacteristicsRequestIndicator	CRITICALITY ignore	TYPE	MeasurementCharacteristicsRequestIndicator	PRESENCE optional}|

{ ID id-MeasurementTimeOccasion		CRITICALITY ignore	TYPE MeasurementTimeOccasion	PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT ABORT

--

-- **************************************************************

MeasurementAbort ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementAbort-IEs}},

...

}

MeasurementAbort-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

MeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{MeasurementFailureIndication-IEs}},

...

}

MeasurementFailureIndication-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-RAN-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION REQUEST

--

-- **************************************************************

TRPInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TRPInformationRequest-IEs}},

...

}

TRPInformationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPList							CRITICALITY ignore	TYPE TRPList				PRESENCE optional}|

{ ID id-TRPInformationTypeListTRPReq	CRITICALITY reject	TYPE TRPInformationTypeListTRPReq	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION RESPONSE

--

-- **************************************************************

TRPInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{TRPInformationResponse-IEs}},

...

}

TRPInformationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPInformationListTRPResp				CRITICALITY ignore	TYPE TRPInformationListTRPResp			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION FAILURE

--

-- **************************************************************

TRPInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{TRPInformationFailure-IEs}},

...

}

TRPInformationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING ACTIVATION REQUEST

--

-- **************************************************************

PositioningActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationRequestIEs} },

...

}

PositioningActivationRequestIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SRSType					CRITICALITY reject	TYPE SRSType					PRESENCE mandatory	} |

{ ID id-ActivationTime			CRITICALITY ignore	TYPE RelativeTime1900				PRESENCE optional	},

...

}

SRSType ::= CHOICE {

semipersistentSRS				SemipersistentSRS,

aperiodicSRS					AperiodicSRS,

choice-Extension				ProtocolIE-Single-Container { { SRSType-ExtIEs} }

}

SRSType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SemipersistentSRS ::= SEQUENCE {

sRSResourceSetID			SRSResourceSetID,

iE-Extensions				ProtocolExtensionContainer { {SemipersistentSRS-ExtIEs} } OPTIONAL,

...

}

SemipersistentSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelation					CRITICALITY ignore	EXTENSION SpatialRelationInfo					PRESENCE optional}|

{ ID id-SRSSpatialRelationPerSRSResource		CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource	PRESENCE optional},

...

}

AperiodicSRS ::= SEQUENCE {

aperiodic					ENUMERATED{true,...},

sRSResourceTrigger			SRSResourceTrigger OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {AperiodicSRS-ExtIEs} } OPTIONAL,

...

}

AperiodicSRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- POSITIONING ACTIVATION RESPONSE

--

-- **************************************************************

PositioningActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationResponseIEs} },

...

}

PositioningActivationResponseIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional }|

{ ID id-SystemFrameNumber			CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional }|

{ ID id-SlotNumber					CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING ACTIVATION FAILURE

--

-- **************************************************************

PositioningActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationFailureIEs} },

...

}

PositioningActivationFailureIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING DEACTIVATION

--

-- **************************************************************

PositioningDeactivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningDeactivationIEs} },

...

}

PositioningDeactivationIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-AbortTransmission			CRITICALITY ignore	TYPE AbortTransmission		PRESENCE mandatory	} ,

...

}

-- **************************************************************

--

#### -- PRS CONFIGURATION REQUEST

--

-- **************************************************************

PRSConfigurationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PRSConfigurationRequest-IEs}},

...

}

PRSConfigurationRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-PRSConfigRequestType	CRITICALITY reject	TYPE PRSConfigRequestType				PRESENCE mandatory}|

{ ID id-PRSTRPList				CRITICALITY ignore	TYPE PRSTRPList							PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- PRS CONFIGURATION RESPONSE

--

-- **************************************************************

PRSConfigurationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PRSConfigurationResponse-IEs}},

...

}

PRSConfigurationResponse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-PRSTransmissionTRPList	CRITICALITY ignore	TYPE PRSTransmissionTRPList			PRESENCE optional}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRS CONFIGURATION FAILURE

--

-- **************************************************************

PRSConfigurationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PRSConfigurationFailure-IEs}},

...

}

PRSConfigurationFailure-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT PRECONFIGURATION REQUIRED

--

-- **************************************************************

MeasurementPreconfigurationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ MeasurementPreconfigurationRequired-IEs}},

...

}

MeasurementPreconfigurationRequired-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRP-PRS-Information-List	CRITICALITY ignore	TYPE TRP-PRS-Information-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MEASUREMENT PRECONFIGURATION CONFIRM

--

-- **************************************************************

MeasurementPreconfigurationConfirm::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ MeasurementPreconfigurationConfirm-IEs}},

...

}

MeasurementPreconfigurationConfirm-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-PreconfigurationResult		CRITICALITY	ignore	TYPE PreconfigurationResult		PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- MEASUREMENT PRECONFIGURATION REFUSE

--

-- **************************************************************

MeasurementPreconfigurationRefuse::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ MeasurementPreconfigurationRefuse-IEs}},

...

}

MeasurementPreconfigurationRefuse-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-Cause					CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT ACTIVATION

--

-- **************************************************************

MeasurementActivation::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementActivation-IEs} },

...

}

MeasurementActivation-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-RequestType					CRITICALITY reject	TYPE RequestType 					PRESENCE mandatory}|

{ ID id-PRS-Measurements-Info-List	CRITICALITY ignore	TYPE PRS-Measurements-Info-List		PRESENCE optional},

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NRPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

maxnoofULAoAs,

maxNoPathExtended,

maxnoARPs,

maxnoTRPTEGs,

maxnoUETEGs,

maxFreqLayers,

maxnoPRSTRPs,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

id-Cell-ID,

id-TRPInformationTypeItem,

id-SrsFrequency,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-PRS-Resource-ID,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoA,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-ARPLocationInfo,

id-ARP-ID,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRP-Rx-TEGInformation,

id-TRPBeamAntennaInformation,

id-NR-TADV,

id-pathPower,

id-SRSPortIndex,

id-UETxTimingErrorMargin,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-StartRBHopping,

id-StartRBIndex,

id-transmissionCombn8,

id-SCS-480,

id-SCS-960

FROM NRPPA-Constants

Criticality,

NRPPATransactionID,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NRPPA-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

NRPPA-PROTOCOL-EXTENSION,

NRPPA-PROTOCOL-IES

FROM NRPPA-Containers;

#### -- A

AbortTransmission ::= CHOICE {

deactivateSRSResourceSetID		SRSResourceSetID,

releaseALL						NULL,

choice-extension							ProtocolIE-Single-Container { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ActiveULBWP  ::= SEQUENCE {

locationAndBandwidth		INTEGER (0..37949,...),

subcarrierSpacing           ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

cyclicPrefix				ENUMERATED {normal, extended},

txDirectCurrentLocation		INTEGER (0..3301,...),

shift7dot5kHz				ENUMERATED {true, ...} OPTIONAL,

sRSConfig					SRSConfig,

iE-Extensions				ProtocolExtensionContainer { { ActiveULBWP-ExtIEs} } OPTIONAL,

...

}

ActiveULBWP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPath)) OF AdditionalPathListItem

AdditionalPathListItem ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TrpMeasurementQuality	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPathListItem-ExtIEs} } OPTIONAL,

...

}

AdditionalPathListItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-MultipleULAoA	CRITICALITY ignore EXTENSION MultipleULAoA PRESENCE	optional}|

{ ID id-pathPower		CRITICALITY ignore EXTENSION UL-SRS-RSRPP PRESENCE	optional},

...

}

ExtendedAdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPathExtended)) OF ExtendedAdditionalPathList-Item

ExtendedAdditionalPathList-Item ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TrpMeasurementQuality	OPTIONAL,

multipleULAoA		MultipleULAoA  			OPTIONAL,

pathPower			UL-SRS-RSRPP			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAdditionalPathList-Item-ExtIEs} } OPTIONAL,

...

}

ExtendedAdditionalPathList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AoA-AssistanceInfo ::= SEQUENCE {

angleMeasurement	AngleMeasurementType,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AoA-AssistanceInfo-ExtIEs } } OPTIONAL,

...

}

AoA-AssistanceInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (1..3)

AngleMeasurementType ::= CHOICE {

expected-ULAoA		Expected-UL-AoA,

expected-ZoA		Expected-ZoA-only,

choice-extension ProtocolIE-Single-Container { { AngleMeasurementType-ExtIEs } }

}

AngleMeasurementType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

Expected-UL-AoA ::= SEQUENCE {

expected-Azimuth-AoA		Expected-Azimuth-AoA,

expected-Zenith-AoA		Expected-Zenith-AoA		OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { Expected-UL-AoA-ExtIEs } }	OPTIONAL,

...

}

Expected-UL-AoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Expected-ZoA-only ::= SEQUENCE {

expected-ZoA-only	Expected-Zenith-AoA,

iE-extensions		ProtocolExtensionContainer { { Expected-ZoA-only-ExtIEs } }	OPTIONAL,

...

}

Expected-ZoA-only-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Expected-Azimuth-AoA ::= SEQUENCE {

expected-Azimuth-AoA-value			Expected-Value-AoA,

expected-Azimuth-AoA-uncertainty		Uncertainty-range-AoA,

iE-extensions		ProtocolExtensionContainer { { Expected-Azimuth-AoA-ExtIEs } }	OPTIONAL,

...

}

Expected-Azimuth-AoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Expected-Zenith-AoA ::= SEQUENCE {

expected-Zenith-AoA-value			Expected-Value-ZoA,

expected-Zenith-AoA-uncertainty		Uncertainty-range-ZoA,

iE-extensions		ProtocolExtensionContainer { { Expected-Zenith-AoA-ExtIEs } }	OPTIONAL,

...

}

Expected-Zenith-AoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ARP-ID ::= INTEGER (1..16, ...)

ARPLocationInformation ::= SEQUENCE (SIZE (1..maxnoARPs)) OF ARPLocationInformation-Item

ARPLocationInformation-Item ::= SEQUENCE {

aRP-ID				ARP-ID,

aRPLocationType		ARPLocationType,

iE-Extensions		ProtocolExtensionContainer { { ARPLocationInformation-ExtIEs} } OPTIONAL,

...

}

ARPLocationInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ARPLocationType ::= CHOICE {

aRPPositionRelativeGeodetic			RelativeGeodeticLocation,

aRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-Single-Container { { ARPLocationType-ExtIEs } }

}

ARPLocationType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

Assistance-Information ::= SEQUENCE {

systemInformation				SystemInformation,

iE-Extensions					ProtocolExtensionContainer { { Assistance-Information-ExtIEs} } OPTIONAL,

...

}

Assistance-Information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationFailureList ::= SEQUENCE (SIZE (1..maxnoAssistInfoFailureListItems)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

outcome							Outcome,

iE-Extensions					ProtocolExtensionContainer { {AssistanceInformationFailureList-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationFailureList-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationMetaData ::= SEQUENCE {

encrypted			ENUMERATED {true, ...}	OPTIONAL,

gNSSID				ENUMERATED {gps, sbas, qzss, galileo, glonass, bds, navic, ...}	OPTIONAL,

sBASID				ENUMERATED {waas, egnos, msas, gagan, ...}					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AssistanceInformationMetaData-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationMetaData-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

BandwidthSRS ::= CHOICE {

fR1		ENUMERATED {mHz5, mHz10, mHz20, mHz40, mHz50, mHz80, mHz100, ..., rel-18-dummy1, rel-18-dummy2, mHz15, mHz25, mHz30, mHz60, mHz35, mHz45, mHz70, mHz90},

fR2		ENUMERATED {mHz50, mHz100, mHz200, mHz400, ..., rel-18-dummy3, mhz800, mHz1600, mHz2000 },

choice-extension		ProtocolIE-Single-Container { { BandwidthSRS-ExtIEs } }

}

BandwidthSRS-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

BCCH ::= INTEGER (0..1023, ...)

Broadcast ::= ENUMERATED {

start,

stop,

...

}

BroadcastPeriodicity ::= ENUMERATED {

ms80,

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

...

}

PositioningBroadcastCells ::= SEQUENCE (SIZE (1..maxnoBcastCell)) OF NG-RAN-CGI

BSSID ::= OCTET STRING (SIZE(6))

#### -- C

CarrierFreq ::= SEQUENCE {

pointA				INTEGER (0..3279165),

offsetToCarrier		INTEGER (0..2199, ...),

iE-Extensions		ProtocolExtensionContainer { {CarrierFreq-ExtIEs} } OPTIONAL,

...

}

CarrierFreq-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

protocol			CauseProtocol,

misc				CauseMisc,

choice-Extension	ProtocolIE-Single-Container {{ Cause-ExtensionIE }}

}

Cause-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

requested-item-not-supported,

requested-item-temporarily-not-available,

...,

serving-NG-RAN-node-changed,

requested-item-not-supported-on-time

}

Cell-Portion-ID ::= INTEGER (0..4095,...)

CGI-EUTRA ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRAcellIdentifier			EUTRACellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-EUTRA-ExtIEs} } OPTIONAL,

...

}

CGI-EUTRA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CGI-NR ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nRcellIdentifier			NRCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {CGI-NR-ExtIEs} } OPTIONAL,

...

}

CGI-NR-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CPLength-EUTRA ::= ENUMERATED {

normal,

extended,

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality					OPTIONAL,

nrppatransactionID				NRPPATransactionID				OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

DL-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

DL-PRS ::= SEQUENCE {

prsid 					INTEGER (0..255),

dl-PRSResourceSetID		PRS-Resource-Set-ID,

dl-PRSResourceID		PRS-Resource-ID		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {DL-PRS-ExtIEs} }	OPTIONAL,

...

}

DL-PRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSMutingPattern ::= CHOICE {

two					BIT STRING (SIZE(2)),

four				BIT STRING (SIZE(4)),

six					BIT STRING (SIZE(6)),

eight				BIT STRING (SIZE(8)),

sixteen				BIT STRING (SIZE(16)),

thirty-two			BIT STRING (SIZE(32)),

choice-extension							ProtocolIE-Single-Container { { DL-PRSMutingPattern-ExtIEs } }

}

DL-PRSMutingPattern-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

DLPRSResourceCoordinates ::= SEQUENCE {

listofDL-PRSResourceSetARP		SEQUENCE (SIZE(1.. maxPRS-ResourceSets)) OF DLPRSResourceSetARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceCoordinates-ExtIEs } } OPTIONAL,

...

}

DLPRSResourceCoordinates-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceSetARP ::= SEQUENCE {

dl-PRSResourceSetID				PRS-Resource-Set-ID,

dL-PRSResourceSetARPLocation	DL-PRSResourceSetARPLocation,

listofDL-PRSResourceARP			SEQUENCE (SIZE(1.. maxPRS-ResourcesPerSet)) OF DLPRSResourceARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceSetARP-ExtIEs } } OPTIONAL,

...

}

DLPRSResourceSetARP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceSetARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-Single-Container { { DL-PRSResourceSetARPLocation-ExtIEs } }

}

DL-PRSResourceSetARPLocation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

DLPRSResourceARP ::= SEQUENCE {

dl-PRSResourceID			PRS-Resource-ID,

dL-PRSResourceARPLocation	DL-PRSResourceARPLocation,

iE-Extensions				ProtocolExtensionContainer { { DLPRSResourceARP-ExtIEs } } OPTIONAL,

...

}

DLPRSResourceARP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-Single-Container { { DL-PRSResourceARPLocation-ExtIEs } }

}

DL-PRSResourceARPLocation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

#### -- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					NG-RAN-CGI,

servingCellTAC					TAC,

nG-RANAccessPointPosition		NG-RANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL,

...

}

E-CID-MeasurementResult-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-GeographicalCoordinates	CRITICALITY ignore	EXTENSION GeographicalCoordinates PRESENCE optional},

...

}

EUTRACellIdentifier ::= BIT STRING (SIZE (28))

EARFCN ::= INTEGER (0..262143, ...)

Expected-Value-AoA ::= INTEGER (0..3599)

Expected-Value-ZoA ::= INTEGER (0..1799)

#### -- F

#### -- G

GeographicalCoordinates ::= SEQUENCE {

tRPPositionDefinitionType	TRPPositionDefinitionType,

dLPRSResourceCoordinates	DLPRSResourceCoordinates	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL,

...

}

GeographicalCoordinates-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ARPLocationInfo		CRITICALITY ignore EXTENSION ARPLocationInformation 	PRESENCE optional},

...

}

GNB-RxTxTimeDiff ::= SEQUENCE {

rxTxTimeDiff		GNBRxTxTimeDiffMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { GNB-RxTxTimeDiff-ExtIEs} }	OPTIONAL,

...

}

GNB-RxTxTimeDiff-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList PRESENCE optional}|

{ ID id-TRPTEGInformation		CRITICALITY ignore EXTENSION TRPTEGInformation	PRESENCE optional },

...

}

GNBRxTxTimeDiffMeas ::= CHOICE {

k0			INTEGER (0.. 1970049),

k1			INTEGER (0.. 985025),

k2			INTEGER (0.. 492513),

k3			INTEGER (0.. 246257),

k4			INTEGER (0.. 123129),

k5			INTEGER (0.. 61565),

choice-extension		ProtocolIE-Single-Container { { GNBRxTxTimeDiffMeas-ExtIEs } }

}

GNBRxTxTimeDiffMeas-ExtIEs		NRPPA-PROTOCOL-IES ::= {

...

}

#### -- H

HESSID ::= OCTET STRING (SIZE(6))

#### -- I

#### -- J

#### -- K

#### -- L

LCS-to-GCS-Translation::= SEQUENCE {

alpha				INTEGER (0..3599),

beta				INTEGER (0..3599),

gamma				INTEGER (0..3599),

iE-Extensions		ProtocolExtensionContainer { { LCS-to-GCS-Translation-ExtIEs} } OPTIONAL,

...

}

LCS-to-GCS-Translation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

LCS-to-GCS-TranslationItem::= SEQUENCE {

alpha				INTEGER (0..359),

alphaFine			INTEGER (0..9) 		OPTIONAL,

beta				INTEGER (0..359),

betaFine			INTEGER (0..9) 		OPTIONAL,

gamma				INTEGER (0..359),

gammaFine			INTEGER (0..9) 		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LCS-to-GCS-TranslationItem-ExtIEs} } OPTIONAL,

...

}

LCS-to-GCS-TranslationItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

LocationUncertainty ::= SEQUENCE {

horizontalUncertainty		INTEGER (0..255),

horizontalConfidence		INTEGER (0..100),

verticalUncertainty			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { LocationUncertainty-ExtIEs} } OPTIONAL,

...

}

LocationUncertainty-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

LoS-NLoSIndicatorHard ::= ENUMERATED {nlos, los}

LoS-NLoSIndicatorSoft ::= INTEGER (0..10)

LoS-NLoSInformation ::= CHOICE {

loS-NLoSIndicatorSoft		LoS-NLoSIndicatorSoft,

loS-NLoSIndicatorHard		LoS-NLoSIndicatorHard,

choice-Extension		ProtocolIE-Single-Container {{ LoS-NLoSInformation-ExtIEs}}

}

LoS-NLoSInformation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

#### -- M

Measurement-ID ::= INTEGER (1.. 65536, ...)

MeasurementAmount  ::= ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64}

MeasurementBeamInfoRequest ::= ENUMERATED {true, ...}

MeasurementBeamInfo ::= SEQUENCE {

pRS-Resource-ID				PRS-Resource-ID		OPTIONAL,

pRS-Resource-Set-ID			PRS-Resource-Set-ID	OPTIONAL,

sSB-Index					SSB-Index			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { MeasurementBeamInfo-ExtIEs} } OPTIONAL,

...

}

MeasurementBeamInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

ms2048,

ms5120,

ms10240,

min1,

min6,

min12,

min30,

min60,

...,

ms20480,

ms40960,

extended

}

MeasurementPeriodicityExtended ::= ENUMERATED {

ms160,

ms320,

ms1280,

ms2560,

ms61440,

ms81920,

ms368640,

ms737280,

ms1843200,

...

}

MeasurementPeriodicityNR-AoA ::= ENUMERATED {

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

ms10240,

ms20480,

ms40960,

ms61440,

ms81920,

ms368640,

ms737280,

ms1843200,

...

}

MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF ProtocolIE-Single-Container { {MeasurementQuantities-ItemIEs} }

MeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-MeasurementQuantities-Item	CRITICALITY reject	TYPE MeasurementQuantities-Item		PRESENCE mandatory}

}

MeasurementQuantities-Item ::= SEQUENCE {

measurementQuantitiesValue					MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { MeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

MeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

... ,

sS-RSRP,

sS-RSRQ,

cSI-RSRP,

cSI-RSRQ,

angleOfArrivalNR,

timingAdvanceNR

}

MeasurementTimeOccasion ::= ENUMERATED {o1, o4, ...}

MeasurementCharacteristicsRequestIndicator ::= BIT STRING (SIZE (16))

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival-EUTRA				INTEGER (0..719),

valueTimingAdvanceType1-EUTRA			INTEGER (0..7690),

valueTimingAdvanceType2-EUTRA			INTEGER (0..7690),

resultRSRP-EUTRA						ResultRSRP-EUTRA,

resultRSRQ-EUTRA						ResultRSRQ-EUTRA,

choice-Extension			ProtocolIE-Single-Container {{ MeasuredResultsValue-ExtensionIE }}

}

MeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultSS-RSRP		CRITICALITY ignore	TYPE ResultSS-RSRP		PRESENCE mandatory	}|

{ ID id-ResultSS-RSRQ		CRITICALITY ignore	TYPE ResultSS-RSRQ		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRP		CRITICALITY ignore	TYPE ResultCSI-RSRP		PRESENCE mandatory	}|

{ ID id-ResultCSI-RSRQ		CRITICALITY ignore	TYPE ResultCSI-RSRQ		PRESENCE mandatory	}|

{ ID id-AngleOfArrivalNR	CRITICALITY ignore	TYPE UL-AoA				PRESENCE mandatory	}|

{ ID id-NR-TADV				CRITICALITY ignore	TYPE NR-TADV			PRESENCE mandatory	},

...

}

MultipleULAoA ::= SEQUENCE {

multipleULAoA					MultipleULAoA-List,

iE-Extensions					ProtocolExtensionContainer { { MultipleULAoA-ExtIEs} } OPTIONAL,

...

}

MultipleULAoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

MultipleULAoA-List ::= SEQUENCE (SIZE(1.. maxnoofULAoAs)) OF MultipleULAoA-Item

MultipleULAoA-Item ::= CHOICE {

uL-AoA		UL-AoA,

ul-ZoA		ZoA,

choice-extension ProtocolIE-Single-Container { { MultipleULAoA-Item-ExtIEs } }

}

MultipleULAoA-Item-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

NG-RANAccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NG-RANAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NG-RANAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NGRANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100),

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NGRANHighAccuracyAccessPointPosition-ExtIEs} } OPTIONAL,

...

}

NGRANHighAccuracyAccessPointPosition-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-CGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

nG-RANcell		NG-RANCell,

iE-Extensions				ProtocolExtensionContainer { {NG-RAN-CGI-ExtIEs} } OPTIONAL,

...

}

NG-RAN-CGI-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NG-RANCell ::= CHOICE {

eUTRA-CellID	EUTRACellIdentifier,

nR-CellID		NRCellIdentifier,

choice-Extension			ProtocolIE-Single-Container {{ NG-RANCell-ExtensionIE }}

}

NG-RANCell-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

NR-ARFCN ::= INTEGER (0..3279165)

NRCellIdentifier ::= BIT STRING (SIZE (36))

NrofSymbolsExtended ::=  ENUMERATED {n8, n10, n12, n14, ...}

NR-PCI ::= INTEGER (0..1007)

NR-PRS-Beam-Information ::= SEQUENCE {

nR-PRS-Beam-InformationList SEQUENCE (SIZE(1.. maxPRS-ResourceSets)) OF NR-PRS-Beam-InformationItem,

lCS-to-GCS-TranslationList SEQUENCE (SIZE(1..maxnolcs-gcs-translation)) OF LCS-to-GCS-TranslationItem		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NR-PRS-Beam-Information-IEs} } OPTIONAL,

...

}

NR-PRS-Beam-Information-IEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NR-PRS-Beam-InformationItem ::= SEQUENCE {

pRSresourceSetID 	PRS-Resource-Set-ID,

pRSAngle 		SEQUENCE (SIZE(1..maxPRS-ResourcesPerSet)) OF PRSAngleItem,

iE-Extensions	ProtocolExtensionContainer { { NR-PRS-Beam-InformationItem-ExtIEs} } OPTIONAL,

...

}

NR-PRS-Beam-InformationItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

NR-TADV ::= INTEGER (0.. 7690)

NumberOfAntennaPorts-EUTRA ::= ENUMERATED {

n1-or-n2,

n4,

...

}

NumberOfDlFrames-EUTRA ::= ENUMERATED {

sf1,

sf2,

sf4,

sf6,

...

}

NumberOfDlFrames-Extended-EUTRA ::= INTEGER (1..160,...)

NumberOfFrequencyHoppingBands ::= ENUMERATED {

twobands,

fourbands,

...

}

NumberOfTRPRxTEG ::= ENUMERATED {two, three, four, six, eight, ...}

NumberOfTRPRxTxTEG ::= ENUMERATED {two, three, four, six, eight, ...}

NZP-CSI-RS-ResourceID::= INTEGER  (0..191)

#### -- O

OnDemandPRS-Info ::= SEQUENCE {

onDemandPRSRequestAllowed				BIT STRING (SIZE (16)),

allowedResourceSetPeriodicityValues		BIT STRING (SIZE (24))	OPTIONAL,

allowedPRSBandwidthValues				BIT STRING (SIZE (64))	OPTIONAL,

allowedResourceRepetitionFactorValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedResourceNumberOfSymbolsValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedCombSizeValues					BIT STRING (SIZE (8))	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { OnDemandPRS-Info-ExtIEs} } OPTIONAL,

...

}

OnDemandPRS-Info-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OTDOACells ::= SEQUENCE (SIZE (1.. maxCellinRANnode)) OF SEQUENCE {

oTDOACellInfo					OTDOACell-Information,

iE-Extensions					ProtocolExtensionContainer { {OTDOACells-ExtIEs} } OPTIONAL,

...

}

OTDOACells-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OTDOACell-Information ::= SEQUENCE (SIZE (1..maxnoOTDOAtypes)) OF OTDOACell-Information-Item

OTDOACell-Information-Item ::= CHOICE {

pCI-EUTRA									PCI-EUTRA,

cGI-EUTRA									CGI-EUTRA,

tAC											TAC,

eARFCN										EARFCN,

pRS-Bandwidth-EUTRA							PRS-Bandwidth-EUTRA,

pRS-ConfigurationIndex-EUTRA				PRS-ConfigurationIndex-EUTRA,

cPLength-EUTRA								CPLength-EUTRA,

numberOfDlFrames-EUTRA						NumberOfDlFrames-EUTRA,

numberOfAntennaPorts-EUTRA					NumberOfAntennaPorts-EUTRA,

sFNInitialisationTime-EUTRA					SFNInitialisationTime-EUTRA,

nG-RANAccessPointPosition					NG-RANAccessPointPosition,

pRSMutingConfiguration-EUTRA				PRSMutingConfiguration-EUTRA,

prsid-EUTRA									PRS-ID-EUTRA,

tpid-EUTRA									TP-ID-EUTRA,

tpType-EUTRA								TP-Type-EUTRA,

numberOfDlFrames-Extended-EUTRA				NumberOfDlFrames-Extended-EUTRA,

crsCPlength-EUTRA							CPLength-EUTRA,

dL-Bandwidth-EUTRA							DL-Bandwidth-EUTRA,

pRSOccasionGroup-EUTRA						PRSOccasionGroup-EUTRA,

pRSFrequencyHoppingConfiguration-EUTRA		PRSFrequencyHoppingConfiguration-EUTRA,

choice-Extension		ProtocolIE-Single-Container {{ OTDOACell-Information-Item-ExtensionIE }}

}

OTDOACell-Information-Item-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-TDD-Config-EUTRA-Item		CRITICALITY 	ignore	TYPE 	TDD-Config-EUTRA-Item			PRESENCE 	mandatory }|

{ ID id-CGI-NR						CRITICALITY 	ignore	TYPE 	CGI-NR							PRESENCE 	mandatory }|

{ ID id-SFNInitialisationTime-NR	CRITICALITY 	ignore	TYPE 	SFNInitialisationTime-EUTRA 	PRESENCE 	mandatory },

...

}

OTDOA-Information-Item ::= ENUMERATED {

pci,

cGI,

tac,

earfcn,

prsBandwidth,

prsConfigIndex,

cpLength,

noDlFrames,

noAntennaPorts,

sFNInitTime,

nG-RANAccessPointPosition,

prsmutingconfiguration,

prsid,

tpid,

tpType,

crsCPlength,

dlBandwidth,

multipleprsConfigurationsperCell,

prsOccasionGroup,

prsFrequencyHoppingConfiguration,

...,

tddConfig

}

OtherRATMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {OtherRATMeasurementQuantities-ItemIEs} }

OtherRATMeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-OtherRATMeasurementQuantities-Item	CRITICALITY reject	TYPE OtherRATMeasurementQuantities-Item PRESENCE mandatory}}

OtherRATMeasurementQuantities-Item ::= SEQUENCE {

otherRATMeasurementQuantitiesValue			OtherRATMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { OtherRATMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

OtherRATMeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

OtherRATMeasurementQuantitiesValue ::= ENUMERATED {

geran,

utran,

... ,

nR,

eUTRA

}

OtherRATMeasurementResult ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF OtherRATMeasuredResultsValue

OtherRATMeasuredResultsValue ::= CHOICE {

resultGERAN									ResultGERAN,

resultUTRAN									ResultUTRAN,

choice-Extension		ProtocolIE-Single-Container {{ OtherRATMeasuredResultsValue-ExtensionIE }}

}

OtherRATMeasuredResultsValue-ExtensionIE NRPPA-PROTOCOL-IES ::= {

{ ID id-ResultNR		CRITICALITY 	ignore	TYPE 	ResultNR		PRESENCE 	mandatory }|

{ ID id-ResultEUTRA		CRITICALITY 	ignore	TYPE 	ResultEUTRA		PRESENCE 	mandatory },

...

}

Outcome ::= ENUMERATED {

failed,

...

}

#### -- P

PathlossReferenceInformation ::= SEQUENCE {

pathlossReferenceSignal					PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { { PathlossReferenceInformation-ExtIEs } } OPTIONAL,

...

}

PathlossReferenceInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PathlossReferenceSignal ::= CHOICE {

sSB-Reference					SSB,

dL-PRS-Reference				DL-PRS,

choice-Extension				ProtocolIE-Single-Container {{ PathlossReferenceSignal-ExtensionIE }}

}

PathlossReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

PCI-EUTRA ::= INTEGER (0..503, ...)

PhysCellIDGERAN ::= INTEGER (0..63, ...)

PhysCellIDUTRA-FDD ::= INTEGER (0..511, ...)

PhysCellIDUTRA-TDD ::= INTEGER (0..127, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

PeriodicityList ::= SEQUENCE (SIZE (1.. maxnoSRS-ResourcePerSet)) OF PeriodicityItem

PeriodicityItem ::= ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms0dot625, ms1, ms1dot25, ms2, ms2dot5, ms4dot, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80m, ms160, ms320, ms640m, ms1280, ms2560, ms5120, ms10240, ...}

PosSIBs ::= SEQUENCE (SIZE (1.. maxNrOfPosSIBs)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

posSIB-Segments					PosSIB-Segments,

assistanceInformationMetaData	AssistanceInformationMetaData	OPTIONAL,

broadcastPriority				INTEGER (1..16,...)				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosSIBs-ExtIEs} }	OPTIONAL,

...

}

PosSIBs-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Segments ::= SEQUENCE (SIZE (1.. maxNrOfSegments)) OF SEQUENCE {

assistanceDataSIBelement			OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { PosSIB-Segments-ExtIEs} }	OPTIONAL,

...

}

PosSIB-Segments-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Type ::= ENUMERATED {

posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType1-8,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType2-20,

posSibType2-21,

posSibType2-22,

posSibType2-23,

posSibType2-24,

posSibType2-25,

posSibType3-1,

posSibType4-1,

posSibType5-1,

posSibType6-1,

posSibType6-2,

posSibType6-3,

...,

posSibType1-9,

posSibType1-10,

posSibType6-4,

posSibType6-5,

posSibType6-6,

dummy-Rel-18-1,

dummy-Rel-18-2,

posSibType2-17a,

posSibType2-18a,

posSibType2-20a

}

PosSRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF PosSRSResource-Item

PosSRSResource-Item ::= SEQUENCE {

srs-PosResourceId				SRSPosResourceID,

transmissionCombPos				TransmissionCombPos,

startPosition                   INTEGER (0..13),

nrofSymbols                     ENUMERATED {n1, n2, n4, n8, n12},

freqDomainShift                 INTEGER (0..268),

c-SRS	                        INTEGER (0..63),

groupOrSequenceHopping          ENUMERATED { neither, groupHopping, sequenceHopping },

resourceTypePos					ResourceTypePos,

sequenceId                      INTEGER (0.. 65535),

spatialRelationPos				SpatialRelationPos OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResource-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceID-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF SRSPosResourceID

PosSRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourceSets)) OF PosSRSResourceSet-Item

PosSRSResourceIDPerSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourcePerSet)) OF SRSPosResourceID

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceIDPerSet-List		PosSRSResourceIDPerSet-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL,

...

}

PosSRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetType  ::= CHOICE {

periodic			PosResourceSetTypePeriodic,

semi-persistent		PosResourceSetTypeSemi-persistent,

aperiodic			PosResourceSetTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ PosResourceSetType-ExtIEs }}

}

PosResourceSetType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

PosResourceSetTypePeriodic ::= SEQUENCE {

posperiodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeSemi-persistent ::= SEQUENCE {

possemi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger		 	INTEGER(1..3),

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

PosResourceSetTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PreconfigurationResult ::= BIT STRING (SIZE(8))

PRS-Bandwidth-EUTRA ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

PRSAngleItem  ::= SEQUENCE {

nRPRSAzimuth			INTEGER (0..359),

nRPRSAzimuthFine		INTEGER (0..9) OPTIONAL,

nRPRSElevation			INTEGER (0..180) OPTIONAL,

nRPRSElevationFine		INTEGER (0..9) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PRSAngleItem-ExtIEs} } OPTIONAL,

...

}

PRSAngleItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-PRS-Resource-ID		CRITICALITY ignore EXTENSION PRS-Resource-ID		PRESENCE optional },

...

}

PRSInformationPos  ::= SEQUENCE {

pRS-IDPos					INTEGER(0..255),

pRS-Resource-Set-IDPos		INTEGER(0..7),

pRS-Resource-IDPos			INTEGER(0..63)	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSInformationPos-ExtIEs} } OPTIONAL,

...

}

PRSInformationPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSConfigRequestType ::= ENUMERATED {configure, off, ...}

PRSConfiguration ::= SEQUENCE {

pRSResourceSet-List				PRSResourceSet-List,

iE-Extensions					ProtocolExtensionContainer { { PRSConfiguration-ExtIEs} } OPTIONAL,

...

}

PRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-ConfigurationIndex-EUTRA ::= INTEGER (0..4095, ...)

PRS-ID-EUTRA	::= INTEGER (0..4095, ...)

PRSMutingConfiguration-EUTRA ::= CHOICE {

two 												BIT STRING (SIZE (2)),

four 												BIT STRING (SIZE (4)),

eight 												BIT STRING (SIZE (8)),

sixteen 											BIT STRING (SIZE (16)),

thirty-two											BIT STRING (SIZE (32)),

sixty-four											BIT STRING (SIZE (64)),

one-hundred-and-twenty-eight						BIT STRING (SIZE (128)),

two-hundred-and-fifty-six							BIT STRING (SIZE (256)),

five-hundred-and-twelve								BIT STRING (SIZE (512)),

one-thousand-and-twenty-four						BIT STRING (SIZE (1024)),

choice-Extension			ProtocolIE-Single-Container {{ PRSMutingConfiguration-EUTRA-ExtensionIE }}

}

PRSMutingConfiguration-EUTRA-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

PRSOccasionGroup-EUTRA ::= ENUMERATED {

og2,

og4,

og8,

og16,

og32,

og64,

og128,

...

}

PRSFrequencyHoppingConfiguration-EUTRA ::= SEQUENCE {

noOfFreqHoppingBands		NumberOfFrequencyHoppingBands,

bandPositions				SEQUENCE(SIZE (1..maxnoFreqHoppingBandsMinusOne)) OF NarrowBandIndex,

iE-Extensions				ProtocolExtensionContainer { { PRSFrequencyHoppingConfiguration-EUTRA-Item-IEs} } OPTIONAL,

...

}

PRSFrequencyHoppingConfiguration-EUTRA-Item-IEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-Measurements-Info-List ::= SEQUENCE (SIZE(1..maxFreqLayers)) OF PRS-Measurements-Info-List-Item

PRS-Measurements-Info-List-Item ::= SEQUENCE {

pointA						INTEGER (0..3279165),

measPRSPeriodicity			ENUMERATED {ms20, ms40, ms80, ms160, ...},

measPRSOffset				INTEGER (0..159, ...),

measurementPRSLength		ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

iE-Extensions	ProtocolExtensionContainer { { PRS-Measurements-Info-List-Item-ExtIEs} } OPTIONAL,

...

}

PRS-Measurements-Info-List-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSMuting::= SEQUENCE {

pRSMutingOption1			PRSMutingOption1		OPTIONAL,

pRSMutingOption2			PRSMutingOption2		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSMuting-ExtIEs} } OPTIONAL,

...

}

PRSMuting-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption1 ::= SEQUENCE {

mutingPattern						DL-PRSMutingPattern,

mutingBitRepetitionFactor			ENUMERATED{n1,n2,n4,n8,...},

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption1-ExtIEs} } OPTIONAL,

...

}

PRSMutingOption1-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption2 ::= SEQUENCE {

mutingPattern						DL-PRSMutingPattern,

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption2-ExtIEs} } OPTIONAL,

...

}

PRSMutingOption2-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresource)) OF PRSResource-Item

PRSResource-Item  ::= SEQUENCE {

pRSResourceID			PRS-Resource-ID,

sequenceID				INTEGER(0..4095),

rEOffset				INTEGER(0..11,...),

resourceSlotOffset		INTEGER(0..511),

resourceSymbolOffset	INTEGER(0..12),

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-Item-ExtIEs} } OPTIONAL,

...

}

PRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLInfo  ::= CHOICE {

qCLSourceSSB		PRSResource-QCLSourceSSB,

qCLSourcePRS		PRSResource-QCLSourcePRS,

choice-Extension		ProtocolIE-Single-Container {{ PRSResource-QCLInfo-ExtIEs }}

}

PRSResource-QCLInfo-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

PRSResource-QCLSourceSSB ::= SEQUENCE {

pCI-NR				INTEGER(0..1007),

sSB-Index 			SSB-Index	 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PRSResource-QCLSourceSSB-ExtIEs} } OPTIONAL,

...

}

PRSResource-QCLSourceSSB-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLSourcePRS ::= SEQUENCE {

qCLSourcePRSResourceSetID		PRS-Resource-Set-ID,

qCLSourcePRSResourceID 			PRS-Resource-ID OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-QCLSourcePRS-ExtIEs} } OPTIONAL,

...

}

PRSResource-QCLSourcePRS-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSet)) OF PRSResourceSet-Item

PRSResourceSet-Item ::= SEQUENCE {

pRSResourceSetID				PRS-Resource-Set-ID,

subcarrierSpacing				ENUMERATED{kHz15, kHz30, kHz60, kHz120, ...},

pRSbandwidth					INTEGER(1..63),

startPRB						INTEGER(0..2176),

pointA							INTEGER (0..3279165),

combSize						ENUMERATED{n2, n4, n6, n12, ...},

cPType							ENUMERATED{normal, extended, ...},

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,..., n128, n256, n512},

resourceSetSlotOffset			INTEGER(0..81919,...),

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...},

resourceTimeGap					ENUMERATED{tg1,tg2,tg4,tg8,tg16,tg32,...},

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...},

pRSMuting						PRSMuting 		OPTIONAL,

pRSResourceTransmitPower		INTEGER(-60..50),

pRSResource-List				PRSResource-List,

iE-Extensions					ProtocolExtensionContainer { { PRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

PRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-ID ::= INTEGER (0..63)

PRS-Resource-Set-ID ::= INTEGER(0..7)

PRS-ID ::= INTEGER(0..255)

PRSTransmissionOffIndication ::= CHOICE {

pRSTransmissionOffPerTRP				NULL,

pRSTransmissionOffPerResourceSet		PRSTransmissionOffPerResourceSet,

pRSTransmissionOffPerResource			PRSTransmissionOffPerResource,

choice-Extension		ProtocolIE-Single-Container {{ PRSTransmissionOffIndication-ExtIEs }}

}

PRSTransmissionOffIndication-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

PRSTransmissionOffPerResource ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSet)) OF PRSTransmissionOffPerResource-Item

PRSTransmissionOffPerResource-Item  ::= SEQUENCE {

pRSResourceSetID								PRS-Resource-Set-ID,

pRSTransmissionOffIndicationPerResourceList		SEQUENCE (SIZE(1.. maxnoofPRSresource)) OF PRSTransmissionOffIndicationPerResource-Item,

iE-Extensions		ProtocolExtensionContainer { { PRSTransmissionOffPerResource-Item-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionOffPerResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffIndicationPerResource-Item  ::= SEQUENCE {

pRSResourceID		PRS-Resource-ID,

iE-Extensions		ProtocolExtensionContainer { { PRSTransmissionOffIndicationPerResource-Item-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionOffIndicationPerResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffInformation ::= SEQUENCE {

pRSTransmissionOffIndication	PRSTransmissionOffIndication,

iE-Extensions					ProtocolExtensionContainer { { PRSTransmissionOffInformation-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionOffInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffPerResourceSet ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSet)) OF PRSTransmissionOffPerResourceSet-Item

PRSTransmissionOffPerResourceSet-Item  ::= SEQUENCE {

pRSResourceSetID		PRS-Resource-Set-ID,

iE-Extensions			ProtocolExtensionContainer { { PRSTransmissionOffPerResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionOffPerResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSTRPList ::= SEQUENCE (SIZE(1.. maxnoTRPs)) OF PRSTRPItem

PRSTRPItem ::= SEQUENCE {

tRP-ID		TRP-ID,

requestedDLPRSTransmissionCharacteristics	RequestedDLPRSTransmissionCharacteristics	OPTIONAL,

-- The IE shall be present if the PRS Configuration Request Type IE is set to “configure” --

pRSTransmissionOffInformation				PRSTransmissionOffInformation				OPTIONAL,

-- The IE shall be present if the PRS Configuration Request Type IE is set to “off” --

iE-Extensions	ProtocolExtensionContainer { { PRSTRPItem-ExtIEs} } OPTIONAL,

...

}

PRSTRPItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionTRPList ::= SEQUENCE (SIZE(1.. maxnoTRPs)) OF PRSTransmissionTRPItem

PRSTransmissionTRPItem ::= SEQUENCE {

tRP-ID				TRP-ID,

pRSConfiguration	PRSConfiguration,

iE-Extensions	ProtocolExtensionContainer { { PRSTransmissionTRPItem-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionTRPItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- Q

#### -- R

ReferenceSignal ::= CHOICE {

nZP-CSI-RS								NZP-CSI-RS-ResourceID,

sSB										SSB,

sRS										SRSResourceID,

positioningSRS							SRSPosResourceID,

dL-PRS									DL-PRS,

choice-Extension				ProtocolIE-Single-Container {{ReferenceSignal-ExtensionIE }}

}

ReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

ReferencePoint ::= CHOICE {

relativeCoordinateID			CoordinateID,

referencePointCoordinate		NG-RANAccessPointPosition,

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition,

choice-Extension				ProtocolIE-Single-Container { { ReferencePoint-ExtIEs} }

}

ReferencePoint-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

CoordinateID ::= INTEGER (0..511, ...)

RelativeGeodeticLocation ::= SEQUENCE {

milli-Arc-SecondUnits		ENUMERATED {zerodot03, zerodot3, three, ...},	heightUnits					ENUMERATED {mm, cm, m, ...},

deltaLatitude				INTEGER (-1024.. 1023),

deltaLongitude				INTEGER (-1024.. 1023),

deltaHeight					INTEGER (-1024.. 1023),

locationUncertainty			LocationUncertainty,

iE-extensions						ProtocolExtensionContainer {{RelativeGeodeticLocation-ExtIEs }}	OPTIONAL,

...

}

RelativeGeodeticLocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativeCartesianLocation ::= SEQUENCE {

xYZunit						ENUMERATED {mm, cm, dm, ...},

xvalue						INTEGER (-65536..65535),

yvalue						INTEGER (-65536..65535),

zvalue						INTEGER (-32768..32767),

locationUncertainty			LocationUncertainty,

iE-Extensions				ProtocolExtensionContainer { { RelativeCartesianLocation-ExtIEs} } OPTIONAL,

...

}

RelativeCartesianLocation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativePathDelay ::= CHOICE {

k0			INTEGER(0..16351),

k1			INTEGER(0..8176),

k2			INTEGER(0..4088),

k3		INTEGER(0..2044),

k4			INTEGER(0..1022),

k5			INTEGER(0..511),

choice-Extension		ProtocolIE-Single-Container { { RelativePathDelay-ExtIEs} }

}

RelativePathDelay-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

RepetitionFactorExtended ::=  ENUMERATED {n3, n5, n6, n7, n8, n10, n12, n14, ...}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedDLPRSTransmissionCharacteristics ::= SEQUENCE {

requestedDLPRSResourceSet-List		RequestedDLPRSResourceSet-List,

numberofFrequencyLayers				INTEGER(1..4)			OPTIONAL,

startTimeAndDuration				StartTimeAndDuration	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { RequestedDLPRSTransmissionCharacteristics-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSTransmissionCharacteristics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSet)) OF RequestedDLPRSResourceSet-Item

RequestedDLPRSResourceSet-Item ::= SEQUENCE {

pRSbandwidth					INTEGER(1..63)	OPTIONAL,

combSize						ENUMERATED{n2, n4, n6, n12, ...}	OPTIONAL,

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,..., n128, n256 ,n512}	OPTIONAL,

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...}		OPTIONAL,

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...}						OPTIONAL,

requestedDLPRSResource-List		RequestedDLPRSResource-List			OPTIONAL,

resourceSetStartTimeAndDuration	StartTimeAndDuration			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RequestedDLPRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresource)) OF RequestedDLPRSResource-Item

RequestedDLPRSResource-Item  ::= SEQUENCE {

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedDLPRSResource-Item-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500,...)							OPTIONAL,

-- The IE shall be present if the Resource Type IE is set to “periodic” --

resourceType			ENUMERATED {periodic, semi-persistent, aperiodic, ...},

bandwidth				BandwidthSRS,

listOfSRSResourceSet	SEQUENCE (SIZE (1.. maxnoSRS-ResourceSets)) OF SRSResourceSet-Item	OPTIONAL,

sSBInformation			SSBInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedSRSTransmissionCharacteristics-ExtIEs} } OPTIONAL,

...

}

RequestedSRSTransmissionCharacteristics-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SrsFrequency		CRITICALITY ignore EXTENSION SrsFrequency		PRESENCE optional },

...

}

SRSResourceSet-Item ::= SEQUENCE {

numberOfSRSResourcePerSet		INTEGER (1..16, ...)			OPTIONAL,

periodicityList					PeriodicityList					OPTIONAL,

spatialRelationInformation		SpatialRelationInfo		OPTIONAL,

pathlossReferenceInformation	PathlossReferenceInformation	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { SRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

SRSResourceSet-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelationPerSRSResource	CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource PRESENCE optional},

...

}

RequestType ::= ENUMERATED {activate, deactivate, ...}

ResourceSetType  ::= CHOICE {

periodic			ResourceSetTypePeriodic,

semi-persistent		ResourceSetTypeSemi-persistent,

aperiodic			ResourceSetTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ ResourceSetType-ExtIEs }}

}

ResourceSetType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ResourceSetTypePeriodic ::= SEQUENCE {

periodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

ResourceSetTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeSemi-persistent ::= SEQUENCE {

semi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

ResourceSetTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger		 	INTEGER(1..3),

slotoffset					INTEGER(0..32),

iE-Extensions				ProtocolExtensionContainer { { ResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

ResourceSetTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceType ::= CHOICE {

periodic						ResourceTypePeriodic,

semi-persistent					ResourceTypeSemi-persistent,

aperiodic						ResourceTypeAperiodic,

choice-extension				ProtocolIE-Single-Container {{ ResourceType-ExtIEs }}

}

ResourceType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodic ::= SEQUENCE {

periodicity		 		  ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset					INTEGER(0..2559, ...),

iE-Extensions			ProtocolExtensionContainer { { ResourceTypePeriodic-ExtIEs} }	OPTIONAL,

...

}

ResourceTypePeriodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistent ::= SEQUENCE {

periodicity			ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset				INTEGER(0..2559, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistent-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeSemi-persistent-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodic ::= SEQUENCE {

aperiodicResourceType	   ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodic-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeAperiodic-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypePos ::= CHOICE {

periodic			ResourceTypePeriodicPos,

semi-persistent		ResourceTypeSemi-persistentPos,

aperiodic			ResourceTypeAperiodicPos,

choice-extension				ProtocolIE-Single-Container {{ ResourceTypePos-ExtIEs }}

}

ResourceTypePos-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodicPos ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920, ..., slot128, slot256, slot512, slot20480},

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypePeriodicPos-ExtIEs} }	OPTIONAL,

...

}

ResourceTypePeriodicPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistentPos ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920, ..., slot128, slot256, slot512, slot20480},

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistentPos-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeSemi-persistentPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodicPos ::= SEQUENCE {

slotOffset          INTEGER (0..32),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodicPos-ExtIEs} }	OPTIONAL,

...

}

ResourceTypeAperiodicPos-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResponseTime ::= SEQUENCE {

time          INTEGER (1..128,...),

timeUnit	  ENUMERATED {second, ten-seconds, ten-milliseconds,...},

iE-Extensions		ProtocolExtensionContainer { { ResponseTime-ExtIEs} }	OPTIONAL,

...

}

ResponseTime-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRP ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultCSI-RSRP-Item

ResultCSI-RSRP-Item ::= SEQUENCE {

nR-PCI					NR-PCI,

nR-ARFCN				NR-ARFCN,

cGI-NR				CGI-NR													OPTIONAL,

valueCSI-RSRP-Cell		ValueRSRP-NR												OPTIONAL,

cSI-RSRP-PerCSI-RS		ResultCSI-RSRP-PerCSI-RS									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRP-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRP-PerCSI-RS ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultCSI-RSRP-PerCSI-RS-Item

ResultCSI-RSRP-PerCSI-RS-Item ::= SEQUENCE {

cSI-RS-Index		INTEGER (0..95),

valueCSI-RSRP		ValueRSRP-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRP-PerCSI-RS-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRP-PerCSI-RS-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRQ ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultCSI-RSRQ-Item

ResultCSI-RSRQ-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

cGI-NR			CGI-NR														OPTIONAL,

valueCSI-RSRQ-Cell	ValueRSRQ-NR													OPTIONAL,

cSI-RSRQ-PerCSI-RS		ResultCSI-RSRQ-PerCSI-RS									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRQ-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRQ-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultCSI-RSRQ-PerCSI-RS ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultCSI-RSRQ-PerCSI-RS-Item

ResultCSI-RSRQ-PerCSI-RS-Item ::= SEQUENCE {

cSI-RS-Index		INTEGER (0..95),

valueCSI-RSRQ		ValueRSRQ-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultCSI-RSRQ-PerCSI-RS-Item-ExtIEs} }	OPTIONAL,

...

}

ResultCSI-RSRQ-PerCSI-RS-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultEUTRA ::= SEQUENCE (SIZE (1.. maxEUTRAMeas)) OF ResultEUTRA-Item

ResultEUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

valueRSRP-EUTRA		ValueRSRP-EUTRA												OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA												OPTIONAL,

cGI-EUTRA			CGI-EUTRA													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultEUTRA-Item-ExtIEs} }	OPTIONAL,

...

}

ResultEUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRP-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-EUTRA-Item

ResultRSRP-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-EUTRA			CGI-EUTRA OPTIONAL,

valueRSRP-EUTRA		ValueRSRP-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRP-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ-EUTRA ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-EUTRA-Item

ResultRSRQ-EUTRA-Item ::= SEQUENCE {

pCI-EUTRA			PCI-EUTRA,

eARFCN				EARFCN,

cGI-UTRA			CGI-EUTRA OPTIONAL,

valueRSRQ-EUTRA		ValueRSRQ-EUTRA,

iE-Extensions		ProtocolExtensionContainer { { ResultRSRQ-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-EUTRA-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRP ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultSS-RSRP-Item

ResultSS-RSRP-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

cGI-NR			CGI-NR													OPTIONAL,

valueSS-RSRP-Cell	ValueRSRP-NR												OPTIONAL,

sS-RSRP-PerSSB		ResultSS-RSRP-PerSSB										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRP-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRP-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRP-PerSSB ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultSS-RSRP-PerSSB-Item

ResultSS-RSRP-PerSSB-Item ::= SEQUENCE {

sSB-Index			SSB-Index,

valueSS-RSRP		ValueRSRP-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRP-PerSSB-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRP-PerSSB-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRQ ::= SEQUENCE (SIZE (1.. maxCellReportNR)) OF ResultSS-RSRQ-Item

ResultSS-RSRQ-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

cGI-NR			CGI-NR													OPTIONAL,

valueSS-RSRQ-Cell	ValueRSRQ-NR												OPTIONAL,

sS-RSRQ-PerSSB		ResultSS-RSRQ-PerSSB										OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRQ-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRQ-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultSS-RSRQ-PerSSB ::= SEQUENCE (SIZE (1.. maxIndexesReport)) OF ResultSS-RSRQ-PerSSB-Item

ResultSS-RSRQ-PerSSB-Item ::= SEQUENCE {

sSB-Index			SSB-Index,

valueSS-RSRQ		ValueRSRQ-NR,

iE-Extensions		ProtocolExtensionContainer { { ResultSS-RSRQ-PerSSB-Item-ExtIEs} }	OPTIONAL,

...

}

ResultSS-RSRQ-PerSSB-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultNR ::= SEQUENCE (SIZE (1.. maxNRMeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nR-PCI				NR-PCI,

nR-ARFCN			NR-ARFCN,

valueSS-RSRP-Cell	ValueRSRP-NR											OPTIONAL,

valueSS-RSRQ-Cell	ValueRSRQ-NR											OPTIONAL,

sS-RSRP-PerSSB		ResultSS-RSRP-PerSSB									OPTIONAL,

sS-RSRQ-PerSSB		ResultSS-RSRQ-PerSSB									OPTIONAL,

cGI-NR				CGI-NR													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} }	OPTIONAL,

...

}

ResultNR-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

RxTxTimingErrorMargin ::= ENUMERATED {tc0dot5, tc1, tc2, tc4, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc64, tc80, tc96, tc128, ...}

#### -- S

SCS-480	::= INTEGER(0..319)

SCS-960	::= INTEGER(0..639)

SCS-SpecificCarrier ::= SEQUENCE {

offsetToCarrier                     INTEGER (0..2199,...),

subcarrierSpacing                   ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

carrierBandwidth                    INTEGER (1..275,...),

iE-Extensions					ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL,

...

}

SCS-SpecificCarrier-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Search-window-information ::= SEQUENCE {

expectedPropagationDelay		INTEGER (-3841..3841,...),

delayUncertainty				INTEGER (1..246,...),

iE-Extensions					ProtocolExtensionContainer { { Search-window-information-ExtIEs } } OPTIONAL,

...

}

Search-window-information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RelativeTime1900 ::= BIT STRING (SIZE (64))

SFNInitialisationTime-EUTRA ::= BIT STRING (SIZE (64))

SlotNumber ::= INTEGER (0..79)

SpatialDirectionInformation ::= SEQUENCE {

nR-PRS-Beam-Information			NR-PRS-Beam-Information,

iE-Extensions					ProtocolExtensionContainer { { SpatialDirectionInformation-ExtIEs } } OPTIONAL,

...

}

SpatialDirectionInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationInfo ::= SEQUENCE {

spatialRelationforResourceID					SpatialRelationforResourceID,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationInfo-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationforResourceID ::= SEQUENCE (SIZE(1..maxnoSpatialRelations)) OF SpatialRelationforResourceIDItem

SpatialRelationforResourceIDItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationforResourceIDItem-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationforResourceIDItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPerSRSResource ::= SEQUENCE {

spatialRelationPerSRSResource-List	SpatialRelationPerSRSResource-List,

iE-Extensions		ProtocolExtensionContainer { { SpatialRelationPerSRSResource-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationPerSRSResource-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPerSRSResource-List::= SEQUENCE(SIZE (1.. maxnoSRS-ResourcePerSet)) OF SpatialRelationPerSRSResourceItem

SpatialRelationPerSRSResourceItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationPerSRSResourceItem-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationPerSRSResourceItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPos ::= CHOICE {

sSBPos					SSB,

pRSInformationPos		PRSInformationPos,

choice-extension				ProtocolIE-Single-Container {{ SpatialInformationPos-ExtIEs }}

}

SpatialInformationPos-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SRSConfig  ::= SEQUENCE {

sRSResource-List			SRSResource-List OPTIONAL,

posSRSResource-List			PosSRSResource-List OPTIONAL,

sRSResourceSet-List			SRSResourceSet-List OPTIONAL,

posSRSResourceSet-List		PosSRSResourceSet-List OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSConfig-ExtIEs } } OPTIONAL,

...

}

SRSConfig-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSCarrier-List ::= SEQUENCE (SIZE(1.. maxnoSRS-Carriers)) OF SRSCarrier-List-Item

SRSCarrier-List-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165),

uplinkChannelBW-PerSCS-List		UplinkChannelBW-PerSCS-List,

activeULBWP						ActiveULBWP,

pCI-NR							INTEGER (0..1007)		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSCarrier-List-Item-ExtIEs } } OPTIONAL,

...

}

SRSCarrier-List-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,

iE-Extensions			ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL,

...

}

SRSConfiguration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SrsFrequency ::= INTEGER (0..3279165)

SRSPortIndex ::= ENUMERATED{id1000, id1001, id1002, id1003, ...}

SRSPosResourceID ::= INTEGER (0..63)

SRSResource::= SEQUENCE {

sRSResourceID					SRSResourceID,

nrofSRS-Ports					ENUMERATED {port1, ports2, ports4},

transmissionComb				TransmissionComb,

startPosition					INTEGER (0..13),

nrofSymbols						ENUMERATED {n1, n2, n4},

repetitionFactor				ENUMERATED {n1, n2, n4},

freqDomainPosition				INTEGER (0..67),

freqDomainShift					INTEGER (0..268),

c-SRS							INTEGER (0..63),

b-SRS							INTEGER (0..3),

b-hop							INTEGER (0..3),

groupOrSequenceHopping			ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType					ResourceType,

sequenceId						INTEGER (0..1023),

iE-Extensions					ProtocolExtensionContainer { { SRSResource-ExtIEs } } OPTIONAL,

...

}

SRSResource-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-nrofSymbolsExtended			CRITICALITY ignore EXTENSION NrofSymbolsExtended 			PRESENCE optional}|

{ ID id-repetitionFactorExtended		CRITICALITY ignore EXTENSION RepetitionFactorExtended 	PRESENCE optional}|

{ ID id-StartRBHopping			CRITICALITY ignore EXTENSION StartRBHopping 			PRESENCE optional}|

{ ID id-StartRBIndex			CRITICALITY ignore EXTENSION StartRBIndex 			PRESENCE optional},

...

}

SRSResourceID ::= INTEGER (0..63)

SRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-Resources)) OF SRSResource

SRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-ResourceSets)) OF SRSResourceSet

SRSResourceID-List::= SEQUENCE (SIZE (1..maxnoSRS-ResourcePerSet)) OF SRSResourceID

SRSResourceSet::= SEQUENCE {

sRSResourceSetID                INTEGER(0..15),

sRSResourceID-List				SRSResourceID-List,

resourceSetType					ResourceSetType,

iE-Extensions					ProtocolExtensionContainer { { SRSResourceSet-ExtIEs } } OPTIONAL,

...

}

SRSResourceSet-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSetID ::= INTEGER (0..15, ...)

SRSResourceTrigger ::= SEQUENCE {

aperiodicSRSResourceTriggerList					AperiodicSRSResourceTriggerList,

iE-Extensions		ProtocolExtensionContainer { {SRSResourceTrigger-ExtIEs} }	OPTIONAL,

...

}

SRSResourceTrigger-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SRSResourcetype ::= SEQUENCE {

sRSResourceTypeChoice					SRSResourceTypeChoice,

iE-Extensions		ProtocolExtensionContainer { { SRSResourcetype-ExtIEs} }	OPTIONAL,

...

}

SRSResourcetype-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSPortIndex	CRITICALITY ignore EXTENSION SRSPortIndex	PRESENCE optional},

...

}

SRSResourceTypeChoice ::= CHOICE {

sRSResourceInfo			SRSInfo,

posSRSResourceInfo		PosSRSInfo,

...

}

SRSInfo ::= SEQUENCE {

sRSResource			SRSResourceID,

...

}

SRSTransmissionStatus ::= ENUMERATED {stopped, ...}

PosSRSInfo ::= SEQUENCE {

posSRSResourceID		SRSPosResourceID,

...

}

SSBInfo ::= SEQUENCE {

listOfSSBInfo		SEQUENCE (SIZE (1..maxNoSSBs)) OF SSBInfoItem,

iE-Extensions		ProtocolExtensionContainer { {SSBInfo-ExtIEs} }	OPTIONAL,

...

}

SSBInfo-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSBInfoItem ::= SEQUENCE {

sSB-Configuration	TF-Configuration,

pCI-NR				INTEGER  (0..1007),

iE-Extensions		ProtocolExtensionContainer { { SSBInfoItem-ExtIEs} }	OPTIONAL,

...

}

SSBInfoItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSB ::= SEQUENCE {

pCI-NR				INTEGER  (0..1007),

ssb-index			SSB-Index	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SSB-ExtIEs} }	OPTIONAL,

...

}

SSB-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SSBBurstPosition ::= CHOICE {

shortBitmap			BIT STRING (SIZE(4)),

mediumBitmap		BIT STRING (SIZE(8)),

longBitmap			BIT STRING (SIZE(64)),

choice-extension		ProtocolIE-Single-Container { { SSBBurstPosition-ExtIEs} }

}

SSBBurstPosition-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

SSB-Index ::= INTEGER(0..63)

SSID ::= OCTET STRING (SIZE(1..32))

StartRBIndex  ::= CHOICE{

freqScalingFactor2  INTEGER(0..1),

freqScalingFactor4  INTEGER(0..3),

choice-extension	ProtocolIE-Single-Container { { StartRBIndex-ExtIEs} }

}

StartRBIndex-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

StartRBHopping  ::= ENUMERATED {enable}

StartTimeAndDuration ::= SEQUENCE {

startTime			RelativeTime1900				OPTIONAL,

duration			INTEGER (0..90060, ...)			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { StartTimeAndDuration-ExtIEs} }	OPTIONAL,

...

}

StartTimeAndDuration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

SystemFrameNumber ::= INTEGER (0..1023)

SystemInformation ::= SEQUENCE (SIZE (1.. maxNrOfPosSImessage)) OF SEQUENCE {

broadcastPeriodicity				BroadcastPeriodicity,

posSIBs								PosSIBs,

iE-Extensions						ProtocolExtensionContainer { { SystemInformation-ExtIEs} }	OPTIONAL,

...

}

SystemInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

#### -- T

TAC ::= OCTET STRING (SIZE(3))

TDD-Config-EUTRA-Item ::= SEQUENCE {

subframeAssignment			ENUMERATED { sa0, sa1, sa2, sa3, sa4, sa5, sa6, ... },

iE-Extensions				ProtocolExtensionContainer { { TDD-Config-EUTRA-Item-Item-ExtIEs } } OPTIONAL,

...

}

TDD-Config-EUTRA-Item-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPTEGInformation ::= CHOICE {

rxTx-TEG			RxTxTEG,

rx-TEG				RxTEG,

choice-extension		ProtocolIE-Single-Container { { TRPTEGInformation-ExtIEs} }

}

TRPTEGInformation-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

RxTxTEG ::= SEQUENCE {

tRP-RxTx-TEGInformation			TRP-RxTx-TEGInformation,

tRP-Tx-TEGInformation			TRP-Tx-TEGInformation		OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { RxTxTEG-ExtIEs } }	OPTIONAL,

...

}

RxTxTEG-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

RxTEG ::= SEQUENCE {

tRP-Rx-TEGInformation		TRP-Rx-TEGInformation,

tRP-Tx-TEGInformation		TRP-Tx-TEGInformation,

iE-extensions      ProtocolExtensionContainer { { RxTEG-ExtIEs } }	OPTIONAL,

...

}

RxTEG-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimingErrorMargin ::= ENUMERATED {tc0, tc2, tc4, tc6, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc56, tc64, tc72, tc80, ...}

TF-Configuration ::= SEQUENCE {

sSB-frequency				INTEGER (0..3279165),

sSB-subcarrier-spacing		ENUMERATED {kHz15, kHz30, kHz120, kHz240, ..., kHz60, kHz480, kHz960},

-- The value kHz60 is not supported in this version of the specification.

sSB-Transmit-power			INTEGER (-60..50),

sSB-periodicity				ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},

sSB-half-frame-offset		INTEGER(0..1),

sSB-SFN-offset				INTEGER(0..15),

sSB-BurstPosition			SSBBurstPosition	OPTIONAL,

sFN-initialisation-time		RelativeTime1900	 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { TF-Configuration-ExtIEs} }	OPTIONAL,

...

}

TF-Configuration-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL,

...

}

TimeStamp-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-Single-Container { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-SCS-480		CRITICALITY reject	TYPE SCS-480 PRESENCE mandatory}|

{ ID id-SCS-960		CRITICALITY reject	TYPE SCS-960 PRESENCE mandatory},

...

}

TP-ID-EUTRA ::= INTEGER (0..4095, ...)

TP-Type-EUTRA ::= ENUMERATED { prs-only-tp, ... }

TransmissionComb ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

choice-extension				ProtocolIE-Single-Container { { TransmissionComb-ExtIEs} }

}

TransmissionComb-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-transmissionCombn8	CRITICALITY reject TYPE TransmissionCombn8 PRESENCE mandatory},

...

}

TransmissionCombn8 ::= SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5),

iE-Extensions					ProtocolExtensionContainer { { TransmissionCombn8-ExtIEs } } OPTIONAL

}

TransmissionCombn8-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TransmissionCombPos ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

n8    SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5)

},

choice-extension				ProtocolIE-Single-Container { { TransmissionCombPos-ExtIEs} }

}

TransmissionCombPos-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPBeamAntennaInformation ::= SEQUENCE {

choice-TRP-Beam-Antenna-Info-Item		Choice-TRP-Beam-Antenna-Info-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRPBeamAntennaInformation-ExtIEs}}				OPTIONAL,

...

}

TRPBeamAntennaInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

Choice-TRP-Beam-Antenna-Info-Item ::= CHOICE {

reference					TRP-ID,

explicit					TRP-BeamAntennaExplicitInformation,

noChange					NULL,

choice-extension				ProtocolIE-Single-Container { { Choice-TRP-Beam-Info-Item-ExtIEs } }

}

Choice-TRP-Beam-Info-Item-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRP-BeamAntennaExplicitInformation ::= SEQUENCE {

trp-BeamAntennaAngles					TRP-BeamAntennaAngles,

lcs-to-gcs-translation	 				LCS-to-GCS-Translation										OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaExplicitInformation-ExtIEs}}				OPTIONAL,

...

}

TRP-BeamAntennaExplicitInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-BeamAntennaAngles ::= SEQUENCE (SIZE (1.. maxnoAzimuthAngles)) OF TRP-BeamAntennaAnglesList-Item

TRP-BeamAntennaAnglesList-Item ::= SEQUENCE {

trp-azimuth-angle					INTEGER (0..359),

trp-azimuth-angle-fine				INTEGER (0..9)	OPTIONAL,

trp-elevation-angle-list			SEQUENCE (SIZE (1.. maxnoElevationAngles)) OF TRP-ElevationAngleList-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaAnglesList-Item-ExtIEs}}			OPTIONAL,

...

}

TRP-BeamAntennaAnglesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-ElevationAngleList-Item ::= SEQUENCE {

trp-elevation-angle					INTEGER (0..180),

trp-elevation-angle-fine			INTEGER (0..9)	OPTIONAL,

trp-beam-power-list					SEQUENCE (SIZE (2..maxNumResourcesPerAngle)) OF TRP-Beam-Power-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRP-ElevationAngleList-Item-ExtIEs}}			OPTIONAL,

...

}

TRP-ElevationAngleList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-Beam-Power-Item ::= SEQUENCE {

pRSResourceSetID				PRS-Resource-Set-ID		OPTIONAL,

pRSResourceID					PRS-Resource-ID,

relativePower				INTEGER (0..30), --negative value

relativePowerFine				INTEGER (0..9)			OPTIONAL, --negative value

iE-Extensions			ProtocolExtensionContainer {{ TRP-Beam-Power-Item-ExtIEs}}					OPTIONAL,

...

}

TRP-Beam-Power-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuantities ::= SEQUENCE (SIZE (1..maxnoPosMeas)) OF TRPMeasurementQuantitiesList-Item

TRPMeasurementQuantitiesList-Item ::= SEQUENCE {

tRPMeasurementQuantities-Item		TRPMeasurementQuantities-Item,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRPMeasurementQuantitiesList-Item-ExtIEs}}		OPTIONAL,

...

}

TRPMeasurementQuantitiesList-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuantities-Item ::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

...,

multiple-UL-AoA,

uL-SRS-RSRPP

}

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-SRSResourcetype	CRITICALITY ignore EXTENSION SRSResourcetype PRESENCE optional}|

{ ID id-ARP-ID		CRITICALITY ignore EXTENSION ARP-ID 	PRESENCE optional}|

{ ID id-LoS-NLoSInformation		CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional },

...

}

TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOAMeasurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension							ProtocolIE-Single-Container { { TrpMeasuredResultsValue-ExtIEs } }

}

TrpMeasuredResultsValue-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-ZoA		CRITICALITY reject TYPE ZoA PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP PRESENCE mandatory},

...

}

TrpMeasurementQuality ::= CHOICE {

timingMeasQuality		TrpMeasurementTimingQuality,

angleMeasQuality		TrpMeasurementAngleQuality,

choice-Extension		ProtocolIE-Single-Container {{ TrpMeasurementQuality-ExtIEs}}

}

TrpMeasurementQuality-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TrpMeasurementTimingQuality ::= SEQUENCE {

measurementQuality		INTEGER (0..31),

resolution				ENUMERATED {m0dot1, m1, m10, m30, ...},

iE-extensions			ProtocolExtensionContainer { { TrpMeasurementTimingQuality-ExtIEs } } OPTIONAL,

...

}

TrpMeasurementTimingQuality-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TrpMeasurementAngleQuality ::= SEQUENCE {

azimuthQuality		INTEGER (0..255),

zenithQuality		INTEGER (0..255)	OPTIONAL,

resolution			ENUMERATED {deg0dot1, ...},

iE-extensions		ProtocolExtensionContainer { { TrpMeasurementAngleQuality-ExtIEs } } OPTIONAL,

...

}

TrpMeasurementAngleQuality-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-MeasurementRequestList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementRequestItem

TRP-MeasurementRequestItem ::= SEQUENCE {

tRP-ID							TRP-ID,

search-window-information		Search-window-information	OPTIONAL,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementRequestItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementRequestItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-Cell-ID		CRITICALITY ignore EXTENSION CGI-NR		PRESENCE optional }|

{ ID id-AoA-SearchWindow		CRITICALITY ignore EXTENSION AoA-AssistanceInfo		PRESENCE optional }|

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional },

...

}

TRP-MeasurementResponseList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementResponseItem

TRP-MeasurementResponseItem ::= SEQUENCE {

tRP-ID							TRP-ID,

measurementResult				TrpMeasurementResult,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementResponseItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementResponseItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-Cell-ID		CRITICALITY ignore EXTENSION CGI-NR		PRESENCE optional },

...

}

TRP-MeasurementUpdateList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementUpdateItem

TRP-MeasurementUpdateItem ::= SEQUENCE {

tRP-ID						TRP-ID,

aoA-window-information		AoA-AssistanceInfo	OPTIONAL,

iE-extensions					ProtocolExtensionContainer { { TRP-MeasurementUpdateItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementUpdateItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional },

...

}

TRPInformationListTRPResp ::= SEQUENCE (SIZE (1.. maxnoTRPs)) OF SEQUENCE {

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { {TRPInformationTRPResp-ExtIEs} } OPTIONAL,

...

}

TRPInformationTRPResp-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPInformation ::= SEQUENCE {

tRP-ID							TRP-ID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL,

...

}

TRPInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationTypeResponseList ::= SEQUENCE (SIZE (1..maxnoTRPInfoTypes)) OF TRPInformationTypeResponseItem

TRPInformationTypeResponseItem ::= CHOICE {

pCI-NR										INTEGER  (0..1007),

cGI-NR									CGI-NR,

aRFCN										INTEGER (0..3279165),

pRSConfiguration							PRSConfiguration,

sSBinformation								SSBInfo,

sFNInitialisationTime						RelativeTime1900,

spatialDirectionInformation					SpatialDirectionInformation,

geographicalCoordinates						GeographicalCoordinates,

choice-extension							ProtocolIE-Single-Container { { TRPInformationTypeResponseItem-ExtIEs } }

}

TRPInformationTypeResponseItem-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPType						CRITICALITY reject TYPE TRPType					PRESENCE mandatory }|

{ ID id-OnDemandPRS				CRITICALITY reject TYPE OnDemandPRS-Info		PRESENCE mandatory}|

{ ID id-TRPTxTEGAssociation			CRITICALITY reject TYPE TRPTxTEGAssociation		PRESENCE mandatory}|

{ ID id-TRPBeamAntennaInformation	CRITICALITY reject TYPE TRPBeamAntennaInformation	PRESENCE mandatory },

...

}

TRPInformationTypeListTRPReq ::= SEQUENCE (SIZE(1.. maxnoTRPInfoTypes)) OF ProtocolIE-Single-Container { {TRPInformationTypeItemTRPReq} }

TRPInformationTypeItemTRPReq NRPPA-PROTOCOL-IES ::= {

{ ID id-TRPInformationTypeItem	 CRITICALITY reject		TYPE TRPInformationTypeItem  	PRESENCE mandatory },

...

}

TRPInformationTypeItem ::= ENUMERATED {

nrPCI,

nG-RAN-CGI,

arfcn,

pRSConfig,

sSBInfo,

sFNInitTime,

spatialDirectInfo,

geoCoord,

...,

trp-type,

ondemandPRSInfo,

trpTxTeg,

beam-antenna-info

}

TRPList ::= SEQUENCE (SIZE(1.. maxnoTRPs)) OF TRPItem

TRPItem ::= SEQUENCE {

tRP-ID		TRP-ID,

iE-Extensions	ProtocolExtensionContainer { {TRPItem-ExtIEs} } OPTIONAL,

...

}

TRPItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-ID ::= INTEGER (1.. maxnoTRPs, ...)

TRPPositionDefinitionType ::= CHOICE {

direct		TRPPositionDirect,

referenced	TRPPositionReferenced,

choice-extension							ProtocolIE-Single-Container { { TRPPositionDefinitionType-ExtIEs } }

}

TRPPositionDefinitionType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPPositionDirect ::= SEQUENCE {

accuracy	TRPPositionDirectAccuracy,

iE-extensions		ProtocolExtensionContainer { { TRPPositionDirect-ExtIEs } }	OPTIONAL,

...

}

TRPPositionDirect-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDirectAccuracy ::= CHOICE {

tRPPosition			NG-RANAccessPointPosition						,

tRPHAposition		NGRANHighAccuracyAccessPointPosition	,

choice-extension			ProtocolIE-Single-Container { { TRPPositionDirectAccuracy-ExtIEs } }

}

TRPPositionDirectAccuracy-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRPPositionReferenced ::= SEQUENCE {

referencePoint					ReferencePoint,

referencePointType				TRPReferencePointType,

iE-extensions							ProtocolExtensionContainer { { TRPPositionReferenced-ExtIEs } }	OPTIONAL,

...

}

TRPPositionReferenced-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-PRS-Information-List ::= SEQUENCE (SIZE(1.. maxnoPRSTRPs)) OF TRP-PRS-Information-List-Item

TRP-PRS-Information-List-Item ::= SEQUENCE {

tRP-ID				TRP-ID,

nR-PCI				NR-PCI,

cGI-NR				CGI-NR					OPTIONAL,

pRSConfiguration							PRSConfiguration,

iE-Extensions	ProtocolExtensionContainer { { TRP-PRS-Information-List-Item-ExtIEs} } OPTIONAL,

...

}

TRP-PRS-Information-List-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPReferencePointType ::= CHOICE {

tRPPositionRelativeGeodetic			RelativeGeodeticLocation,

tRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension							ProtocolIE-Single-Container { { TRPReferencePointType-ExtIEs } }

}

TRPReferencePointType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

TRP-Rx-TEGInformation ::= SEQUENCE {

tRP-Rx-TEGID					INTEGER (0..31),

tRP-Rx-TimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-Rx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-Rx-TEGInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-RxTx-TEGInformation ::= SEQUENCE {

tRP-RxTx-TEGID					INTEGER (0..255),

tRP-RxTx-TimingErrorMargin		RxTxTimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-RxTx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-RxTx-TEGInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRP-Tx-TEGInformation ::= SEQUENCE {

tRP-Tx-TEGID					INTEGER (0..7),

tRP-Tx-TimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-Tx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-Tx-TEGInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPTxTEGAssociation ::= SEQUENCE (SIZE(1.. maxnoTRPTEGs)) OF TRPTEGItem

TRPTEGItem ::= SEQUENCE {

tRP-Tx-TEGInformation	TRP-Tx-TEGInformation,

dl-PRSResourceSetID		PRS-Resource-Set-ID,

dl-PRSResourceID-List	SEQUENCE (SIZE(1.. maxPRS-ResourcesPerSet)) OF DLPRSResourceID-Item	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { TRPTEGItem-ExtIEs } } OPTIONAL,

...

}

TRPTEGItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceID-Item ::= SEQUENCE {

dl-PRSResourceID		PRS-Resource-ID,

iE-Extensions			ProtocolExtensionContainer { { DLPRSResource-Item-ExtIEs} }	OPTIONAL,

...

}

DLPRSResource-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

TRPType ::= ENUMERATED {

prsOnlyTP,

srsOnlyRP,

tp,

rp,

trp,

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

UARFCN ::= INTEGER (0..16383, ...)

UE-Measurement-ID ::= INTEGER (1..15, ..., 16..256)

UEReportingInformation::= SEQUENCE {

reportingAmount				ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64},

reportingInterval			ENUMERATED {none, one, two, four, eight, ten, sixteen, twenty, thirty-two, sixty-four, ...},

iE-extensions			ProtocolExtensionContainer { { UEReportingInformation-ExtIEs } }	OPTIONAL,

...

}

UEReportingInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UE-TEG-ReportingPeriodicity ::= ENUMERATED {

ms160,

ms320,

ms1280,

ms2560,

ms61440,

ms81920,

ms368640,

ms737280,

...

}

UETxTEGAssociationList ::= SEQUENCE (SIZE(1.. maxnoUETEGs)) OF UETxTEGAssociationItem

UETxTEGAssociationItem ::= SEQUENCE {

uE-Tx-TEG-ID				INTEGER (0..7),

posSRSResourceID-List		PosSRSResourceID-List,

timeStamp					TimeStamp,

carrierFreq					CarrierFreq				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UETxTEGAssociationItem-ExtIEs } } OPTIONAL,

...

}

UETxTEGAssociationItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-UETxTimingErrorMargin			CRITICALITY ignore EXTENSION TimingErrorMargin 	PRESENCE optional },

...

}

SRSResourceID-Item ::= SEQUENCE {

sRSResourceID                   SRSResourceID,

iE-Extensions			ProtocolExtensionContainer { { SRSResourceID-Item-ExtIEs} }	OPTIONAL,

...

}

SRSResourceID-Item-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UE-TEG-Info-Request ::= ENUMERATED {onDemand, periodic, stop, ...}

UTRA-EcN0 ::= INTEGER (0..49, ...)

UTRA-RSCP ::= INTEGER (-5..91, ...)

UL-AoA ::= SEQUENCE {

azimuthAoA					INTEGER (0..3599),

zenithAoA					INTEGER (0..1799)				OPTIONAL,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-AoA-ExtIEs } }	OPTIONAL,

...

}

UL-AoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UL-RTOAMeasurement ::= SEQUENCE {

uLRTOAmeas			ULRTOAMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-RTOAMeasurement-ExtIEs } }	OPTIONAL,	...

}

UL-RTOAMeasurement-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRP-Rx-TEGInformation		CRITICALITY ignore EXTENSION TRP-Rx-TEGInformation		PRESENCE optional},

...

}

ULRTOAMeas::= CHOICE {

k0		INTEGER (0.. 1970049),

k1		INTEGER (0.. 985025),

k2		INTEGER (0.. 492513),

k3		INTEGER (0.. 246257),

k4		INTEGER (0.. 123129),

k5		INTEGER (0.. 61565),

choice-extension		ProtocolIE-Single-Container { { ULRTOAMeas-ExtIEs } }

}

ULRTOAMeas-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

UL-SRS-RSRP ::= INTEGER (0..126)

UL-SRS-RSRPP ::= SEQUENCE {

firstPathRSRPP				INTEGER (0..126),

iE-extensions			ProtocolExtensionContainer { { UL-SRS-RSRPP-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-RSRPP-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

UplinkChannelBW-PerSCS-List ::= SEQUENCE (SIZE (1..maxnoSCSs)) OF SCS-SpecificCarrier

Uncertainty-range-AoA ::= INTEGER (0..3599)

Uncertainty-range-ZoA ::= INTEGER (0..1799)

#### -- V

ValueRSRP-EUTRA ::= INTEGER (0..97, ...)

ValueRSRQ-EUTRA ::= INTEGER (0..34, ...)

ValueRSRP-NR ::= INTEGER (0..127)

ValueRSRQ-NR ::= INTEGER (0..127)

#### -- W

WLANMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {WLANMeasurementQuantities-ItemIEs} }

WLANMeasurementQuantities-ItemIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-WLANMeasurementQuantities-Item	CRITICALITY reject	TYPE WLANMeasurementQuantities-Item PRESENCE mandatory}}

WLANMeasurementQuantities-Item ::= SEQUENCE {

wLANMeasurementQuantitiesValue			WLANMeasurementQuantitiesValue,

iE-Extensions							ProtocolExtensionContainer { { WLANMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

WLANMeasurementQuantitiesValue-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasurementQuantitiesValue ::= ENUMERATED {

wlan,

...

}

WLANMeasurementResult ::= SEQUENCE (SIZE (1..maxNoMeas)) OF WLANMeasurementResult-Item

WLANMeasurementResult-Item ::= SEQUENCE {

wLAN-RSSI			WLAN-RSSI,

sSID				SSID					OPTIONAL,

bSSID				BSSID					OPTIONAL,

hESSID				HESSID					OPTIONAL,

operatingClass		WLANOperatingClass		OPTIONAL,

countryCode			WLANCountryCode			OPTIONAL,

wLANChannelList		WLANChannelList			OPTIONAL,

wLANBand			WLANBand				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementResult-Item-ExtIEs } }	OPTIONAL,

...

}

WLANMeasurementResult-Item-ExtIEs	NRPPA-PROTOCOL-EXTENSION ::= {

...

}

WLAN-RSSI ::= INTEGER (0..141, ...)

WLANBand ::= ENUMERATED {band2dot4, band5, ...}

WLANChannelList ::= SEQUENCE (SIZE (1..maxWLANchannels)) OF WLANChannel

WLANChannel ::= INTEGER (0..255)

WLANCountryCode ::= ENUMERATED {

unitedStates,

europe,

japan,

global,

...

}

WLANOperatingClass ::= INTEGER (0..255)

#### -- X

#### -- Y

#### -- Z

ZoA ::= SEQUENCE {

zenithAoA					INTEGER (0..1799),

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { ZoA-ExtIEs } }	OPTIONAL,

...

}

ZoA-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

END

-- ASN1STOP

### 9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

NRPPA-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

-- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

NRPPATransactionID		::= INTEGER (0..32767)

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local			INTEGER (0.. maxPrivateIEs),

global			OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NRPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-assistanceInformationControl									ProcedureCode ::= 7

id-assistanceInformationFeedback								ProcedureCode ::= 8

id-positioningInformationExchange								ProcedureCode ::= 9

id-positioningInformationUpdate									ProcedureCode ::= 10

id-Measurement													ProcedureCode ::= 11

id-MeasurementReport											ProcedureCode ::= 12

id-MeasurementUpdate											ProcedureCode ::= 13

id-MeasurementAbort												ProcedureCode ::= 14

id-MeasurementFailureIndication									ProcedureCode ::= 15

id-tRPInformationExchange										ProcedureCode ::= 16

id-positioningActivation										ProcedureCode ::= 17

id-positioningDeactivation										ProcedureCode ::= 18

id-pRSConfigurationExchange										ProcedureCode ::= 19

id-measurementPreconfiguration									ProcedureCode ::= 20

id-measurementActivation										ProcedureCode ::= 21

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellinRANnode							INTEGER ::= 3840

maxIndexesReport							INTEGER ::= 64

maxNoMeas									INTEGER ::= 64

maxCellReport								INTEGER ::= 9

maxCellReportNR								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxEUTRAMeas								INTEGER ::= 8

maxGERANMeas								INTEGER ::= 8

maxNRMeas									INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNoPath									INTEGER ::= 2

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNoOfMeasTRPs								INTEGER ::= 64

maxnoTRPs									INTEGER ::= 65535

maxnoTRPInfoTypes							INTEGER ::= 64

maxnoofAngleInfo							INTEGER ::= 65535

maxnolcs-gcs-translation					INTEGER ::= 3

maxnoBcastCell								INTEGER ::= 16384

maxnoSRSTriggerStates						INTEGER ::= 3

maxnoSpatialRelations						INTEGER ::= 64

maxnoPosMeas								INTEGER ::= 16384

maxnoSRS-Carriers							INTEGER ::= 32

maxnoSCSs									INTEGER ::= 5

maxnoSRS-Resources							INTEGER ::= 64

maxnoSRS-PosResources						INTEGER ::= 64

maxnoSRS-ResourceSets						INTEGER ::= 16

maxnoSRS-ResourcePerSet						INTEGER ::= 16

maxnoSRS-PosResourceSets					INTEGER ::= 16

maxnoSRS-PosResourcePerSet					INTEGER ::= 16

maxPRS-ResourceSets							INTEGER ::= 2

maxPRS-ResourcesPerSet						INTEGER ::= 64

maxNoSSBs									INTEGER ::= 255

maxnoofPRSresourceSet						INTEGER ::= 8

maxnoofPRSresource							INTEGER ::= 64

maxnoofULAoAs								INTEGER ::= 8

maxNoPathExtended							INTEGER ::= 8

maxnoARPs									INTEGER ::=	16

maxnoUETEGs									INTEGER ::= 256

maxnoTRPTEGs								INTEGER ::= 8

maxFreqLayers								INTEGER ::= 4

maxNumResourcesPerAngle						INTEGER ::= 24

maxnoAzimuthAngles							INTEGER ::= 3600

maxnoElevationAngles						INTEGER ::= 1801

maxnoPRSTRPs								INTEGER ::= 256

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-LMF-UE-Measurement-ID										ProtocolIE-ID ::= 2

id-ReportCharacteristics										ProtocolIE-ID ::= 3

id-MeasurementPeriodicity										ProtocolIE-ID ::= 4

id-MeasurementQuantities										ProtocolIE-ID ::= 5

id-RAN-UE-Measurement-ID										ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult										ProtocolIE-ID ::= 7

id-OTDOACells													ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group									ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item									ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item									ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics						ProtocolIE-ID ::= 12

id-Cell-Portion-ID												ProtocolIE-ID ::= 14

id-OtherRATMeasurementQuantities								ProtocolIE-ID ::= 15

id-OtherRATMeasurementQuantities-Item							ProtocolIE-ID ::= 16

id-OtherRATMeasurementResult									ProtocolIE-ID ::= 17

id-WLANMeasurementQuantities									ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item								ProtocolIE-ID ::= 20

id-WLANMeasurementResult										ProtocolIE-ID ::= 21

id-TDD-Config-EUTRA-Item										ProtocolIE-ID ::= 22

id-Assistance-Information										ProtocolIE-ID ::= 23

id-Broadcast													ProtocolIE-ID ::= 24

id-AssistanceInformationFailureList								ProtocolIE-ID ::= 25

id-SRSConfiguration												ProtocolIE-ID ::= 26

id-MeasurementResult											ProtocolIE-ID ::= 27

id-TRP-ID														ProtocolIE-ID ::= 28

id-TRPInformationTypeListTRPReq									ProtocolIE-ID ::= 29

id-TRPInformationListTRPResp									ProtocolIE-ID ::= 30

id-MeasurementBeamInfoRequest									ProtocolIE-ID ::= 31

id-ResultSS-RSRP												ProtocolIE-ID ::= 32

id-ResultSS-RSRQ												ProtocolIE-ID ::= 33

id-ResultCSI-RSRP												ProtocolIE-ID ::= 34

id-ResultCSI-RSRQ												ProtocolIE-ID ::= 35

id-AngleOfArrivalNR												ProtocolIE-ID ::= 36

id-GeographicalCoordinates										ProtocolIE-ID ::= 37

id-PositioningBroadcastCells									ProtocolIE-ID ::= 38

id-LMF-Measurement-ID											ProtocolIE-ID ::= 39

id-RAN-Measurement-ID											ProtocolIE-ID ::= 40

id-TRP-MeasurementRequestList									ProtocolIE-ID ::= 41

id-TRP-MeasurementResponseList									ProtocolIE-ID ::= 42

id-TRP-MeasurementReportList									ProtocolIE-ID ::= 43

id-SRSType														ProtocolIE-ID ::= 44

id-ActivationTime												ProtocolIE-ID ::= 45

id-SRSResourceSetID												ProtocolIE-ID ::= 46

id-TRPList														ProtocolIE-ID ::= 47

id-SRSSpatialRelation											ProtocolIE-ID ::= 48

id-SystemFrameNumber											ProtocolIE-ID ::= 49

id-SlotNumber													ProtocolIE-ID ::= 50

id-SRSResourceTrigger											ProtocolIE-ID ::= 51

id-TRPMeasurementQuantities										ProtocolIE-ID ::= 52

id-AbortTransmission											ProtocolIE-ID ::= 53

id-SFNInitialisationTime										ProtocolIE-ID ::= 54

id-ResultNR														ProtocolIE-ID ::= 55

id-ResultEUTRA													ProtocolIE-ID ::= 56

id-TRPInformationTypeItem										ProtocolIE-ID ::= 57

id-CGI-NR														ProtocolIE-ID ::= 58

id-SFNInitialisationTime-NR										ProtocolIE-ID ::= 59

id-Cell-ID														ProtocolIE-ID ::= 60

id-SrsFrequency													ProtocolIE-ID ::= 61

id-TRPType														ProtocolIE-ID ::= 62

id-SRSSpatialRelationPerSRSResource								ProtocolIE-ID ::= 63

id-MeasurementPeriodicityExtended								ProtocolIE-ID ::= 64

id-PRS-Resource-ID												ProtocolIE-ID ::= 65

id-PRSTRPList													ProtocolIE-ID ::= 66

id-PRSTransmissionTRPList										ProtocolIE-ID ::= 67

id-OnDemandPRS													ProtocolIE-ID ::= 68

id-AoA-SearchWindow												ProtocolIE-ID ::= 69

id-TRP-MeasurementUpdateList									ProtocolIE-ID ::= 70

id-ZoA															ProtocolIE-ID ::= 71

id-ResponseTime													ProtocolIE-ID ::= 72

id-UEReportingInformation										ProtocolIE-ID ::= 73

id-MultipleULAoA												ProtocolIE-ID ::= 74

id-UL-SRS-RSRPP													ProtocolIE-ID ::= 75

id-SRSResourcetype												ProtocolIE-ID ::= 76

id-ExtendedAdditionalPathList									ProtocolIE-ID ::= 77

id-ARPLocationInfo												ProtocolIE-ID ::= 78

id-ARP-ID														ProtocolIE-ID ::= 79

id-LoS-NLoSInformation											ProtocolIE-ID ::= 80

id-UETxTEGAssociationList										ProtocolIE-ID ::= 81

id-NumberOfTRPRxTEG												ProtocolIE-ID ::= 82

id-NumberOfTRPRxTxTEG											ProtocolIE-ID ::= 83

id-TRPTxTEGAssociation											ProtocolIE-ID ::= 84

id-TRPTEGInformation											ProtocolIE-ID ::= 85

id-TRP-Rx-TEGInformation										ProtocolIE-ID ::= 86

id-TRP-PRS-Information-List										ProtocolIE-ID ::= 87

id-PRS-Measurements-Info-List									ProtocolIE-ID ::= 88

id-PRSConfigRequestType											ProtocolIE-ID ::= 89

id-UE-TEG-Info-Request											ProtocolIE-ID ::= 90

id-MeasurementTimeOccasion										ProtocolIE-ID ::= 91

id-MeasurementCharacteristicsRequestIndicator					ProtocolIE-ID ::= 92

id-TRPBeamAntennaInformation									ProtocolIE-ID ::= 93

id-NR-TADV														ProtocolIE-ID ::= 94

id-MeasurementAmount											ProtocolIE-ID ::= 95

id-pathPower													ProtocolIE-ID ::= 96

id-PreconfigurationResult										ProtocolIE-ID ::= 97

id-RequestType													ProtocolIE-ID ::= 98

id-UE-TEG-ReportingPeriodicity									ProtocolIE-ID ::= 99

id-SRSPortIndex													ProtocolIE-ID ::= 100

id-procedure-code-101-not-to-be-used							ProtocolIE-ID ::= 101

id-procedure-code-102-not-to-be-used							ProtocolIE-ID ::= 102

id-procedure-code-103-not-to-be-used							ProtocolIE-ID ::= 103

id-UETxTimingErrorMargin										ProtocolIE-ID ::= 104

id-MeasurementPeriodicityNR-AoA									ProtocolIE-ID ::= 105

id-SRSTransmissionStatus										ProtocolIE-ID ::= 106

id-nrofSymbolsExtended											ProtocolIE-ID ::= 107

id-repetitionFactorExtended										ProtocolIE-ID ::= 108

id-StartRBHopping												ProtocolIE-ID ::= 109

id-StartRBIndex													ProtocolIE-ID ::= 110

id-transmissionCombn8											ProtocolIE-ID ::= 111

id-SCS-480														ProtocolIE-ID ::= 119

id-SCS-960														ProtocolIE-ID ::= 120

END

-- ASN1STOP

### 9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

NRPPA-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) nrppa (4) version1 (1) nrppa-Containers (5)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM NRPPA-CommonDataTypes;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

NRPPA-PROTOCOL-IES ::= CLASS {

&id					ProtocolIE-ID			UNIQUE,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

NRPPA-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

NRPPA-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { NRPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container { NRPPA-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { NRPPA-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				NRPPA-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		NRPPA-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			NRPPA-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, NRPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { NRPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { NRPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					NRPPA-PROTOCOL-EXTENSION.&id			({ExtensionSetParam}),

criticality			NRPPA-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		NRPPA-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { NRPPA-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { NRPPA-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				NRPPA-PRIVATE-IES.&id				({IEsSetParam}),

criticality		NRPPA-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value			NRPPA-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

## 9.4	Message transfer syntax

NRPPa shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax, as specified in ref. ITU-T Rec. X.691 [6].

## 9.5	Timers

Void.

# 10	Handling of unknown, unforeseen and erroneous protocol data

Section 10 of TS 38.413 [2] is applicable for the purposes of the present document, with the following additions:

-	In case of Abstract Syntax Error, when reporting the Criticality Diagnostics IE for not comprehended IE/IE groups or missing IE/IE groups, the NRPPa Transaction ID IE shall also be included;

-	In case of Logical Error, when reporting the Criticality Diagnostics IE, the NRPPa Transaction ID IE shall also be included.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08-23 | RAN3#97 | R3-173238 |  |  |  | TS skeleton agreed | v0.0.0 |
| 2017-08-25 | RAN3#97 | R3-173374 |  |  |  | TS 38.455 V0.1.0 | v0.1.0 |
| 2017-10-18 | RAN3#97bis | R3-173979 |  |  |  | Implemented agreed pCR from R3#97bis | V0.2.0 |
| 2017-12-04 | RAN3#98 | R3-175064 |  |  |  | Implemented agreed pCR from R3#98 | V0.3.0 |
| 2018-01-31 | RAN3 Adhoc 1801 | R3-180658 |  |  |  | Implemented agreed pCR from R3 Adhoc_1801 | V0.5.0 |
| 2018-03-15 | RAN3#99 | R3-181595 |  |  |  | Implemented agreed pCR's from R3#99 | V0.6.0 |
| 2018-05-29 | RAN3#100 | R3-183598 |  |  |  | Implemented agreed pCR's from R3#100 | V0.7.0 |
| 2018-06 | RAN#80 | RP-181147 |  |  |  | Submitted to RAN plenary for Approval | V1.0.0 |
| 2018-06 | RAN#80 | - | - | - | - | Specification approved at TSG-RAN and placed under change control | 15.0.0 |
| 2018-09 | RAN#81 | RP-181921 | 0002 | 1 | F | Rapporteur CR for TS 38.455 | 15.1.0 |
| 2018-12 | RAN#82 | RP-182446 | 0003 | 1 | F | Addition of TDD UL/DL configuration to OTDOA assistance data | 15.2.0 |
| 2019-01 | RAN#82 |  |  |  |  | Editorial Corrections: - 1 editorial correction to ASN.1 - adding "ASN1START" and "ASN1STOP" TAGs to the ASN.1 | 15.2.1 |
| 2020-07 | SA#88-e | - | - | - | - | Update to Rel-16 version (MCC) | 16.0.0 |
| 2020-09 | SA#89-e | RP-201849 | 0008 | 19 | B | Introduction of NR Positioning in NRPPa | 16.1.0 |
| 2020-12 | RAN#90-e | RP-202315 | 0014 | 2 | A | Support OTDOA assistance data for case of NR serving cell | 16.2.0 |
| 2020-12 | RAN#90-e | RP-202311 | 0015 | 2 | F | Corrections to tabular and asn.1 for NR positioning (NRPPa) | 16.2.0 |
| 2020-12 | RAN#90-e | RP-202311 | 0016 | - | F | Correction of NRPPa positioning procedures | 16.2.0 |
| 2020-12 | RAN#90-e | RP-202311 | 0021 | 1 | F | RRC alignement and various correction including ASN.1 | 16.2.0 |
| 2020-12 | RAN#90-e | RP-202311 | 0022 | 2 | F | Coupling TRP ID and Cell ID in Measurement procedures | 16.2.0 |
| 2021-03 | RAN#91-e | RP-210230 | 0024 | 1 | F | Including SRS frequency information in Positioning Information Request | 16.3.0 |
| 2021-03 | RAN#91-e | RP-210230 | 0025 | 1 | F | Corrections on NRPPa | 16.3.0 |
| 2021-03 | RAN#91-e | RP-210236 | 0026 | - | F | Correction of NRPPa section 10 | 16.3.0 |
| 2021-06 | RAN#92-e | RP-211333 | 0028 | - | A | Clarification of E-CID Measurement Result | 16.4.0 |
| 2021-06 | RAN#92-e | RP-211327 | 0029 | - | F | Correction of Spatial Relation Information | 16.4.0 |
| 2021-06 | RAN#92-e | RP-211327 | 0033 | 1 | F | Correction on SFN Initialisation Time | 16.4.0 |
| 2021-06 | RAN#92-e | RP-211327 | 0034 | - | F | Correction on relative cartesian coordinate | 16.4.0 |
| 2021-09 | RAN#93-e | RP-211883 | 0039 | 1 | F | Correction of the RAN and LMF UE measurement IDs extension | 16.5.0 |
| 2021-09 | RAN#93-e | RP-211883 | 0041 | 1 | F | Adding procedural text for System Frame Number and Slot Number | 16.5.0 |
| 2021-12 | RAN#94-e | RP-213173 | 0047 | 3 | F | Correction on PRS-only TP | 16.6.0 |
| 2021-12 | RAN#94-e | RP-212867 | 0049 | 1 | F | Support of providing spatial relation per SRS resource from LMF to gNB | 16.6.0 |
| 2022-03 | RAN#95-e | RP-220281 | 0052 | 1 | F | Correction on Measurement Periodicity | 16.7.0 |
| 2022-03 | RAN#95-e | RP-220281 | 0053 | 1 | F | Correction on PRS Beam Information | 16.7.0 |
| 2022-03 | RAN#95-e | RP-220228 | 0037 | 8 | B | Introduction of NR Positioning enhancements to NRPPa | 17.0.0 |
| 2022-03 | RAN#95-e | RP-220236 | 0042 | 2 | B | Addition of NR Timing Advance reporting for NR UL E-CID [NRTADV] | 17.0.0 |
| 2022-03 | RAN#95-e | RP-220236 | 0054 | - | D | NRPPa Rapporteur Corrections | 17.0.0 |
| 2022-06 | RAN#96 | RP-221131 | 0057 | 1 | F | NRPPa ASN.1 review for NR Positioning Enhancements | 17.1.0 |
| 2022-06 | RAN#96 | RP-221145 | 0058 | 4 | F | CR to 38.455 on Measurement Amount | 17.1.0 |
| 2022-06 | RAN#96 | RP-221145 | 0062 | 2 | D | Rapporteur Corrections to Rel-17 NRPPa | 17.1.0 |
| 2022-06 | RAN#96 | RP-221131 | 0063 | 4 | F | Positionng corrections for NRPPA | 17.1.0 |
| 2022-06 | RAN#96 | RP-221131 | 0066 | 1 | F | Corrections to Measurement Pre-configuration Information Transfer | 17.1.0 |
| 2022-06 | RAN#96 | RP-221131 | 0067 | 1 | F | Support for Multiple Measurement Instances | 17.1.0 |
| 2022-06 | RAN#96 | RP-221152 | 0069 | 2 | A | Correction for PRS Muting | 17.1.0 |
| 2022-06 | RAN#96 | RP-221152 | 0074 |  | A | Correction to SSB subcarrier spacing | 17.1.0 |
| 2022-06 |  |  |  |  |  | editorial corrections to rename the following asn.1 names as choice extension names - sRSType-extension -> choice-Extension - cause-Extension -> choice-Extension - measuredResultsValue-Extension -> choice-Extension - nG-RANCell-Extension -> hoice-Extension - oTDOACell-Information-Item-Extension -> choice-Extension - otherRATMeasuredResultsValue-Extension -> choice-Extension - pRSMutingConfiguration-EUTRA-Extension -> choice-Extension | 17.1.1 |
| 2022-09 | RAN#97-e | RP-222186 | 0075 | 1 | F | Correction for UE Tx TEG Association | 17.2.0 |
| 2022-09 | RAN#97-e | RP-222186 | 0076 | - | F | Introduction of SRS port index | 17.2.0 |
| 2022-09 | RAN#97-e | RP-222186 | 0077 | 2 | F | Support of timing error margins for TEGs in NRPPa | 17.2.0 |
| 2022-09 | RAN#97-e | RP-222543 | 0079 | 3 | A | CR to 38.455 on E-CID measurement periodicity | 17.2.0 |
| 2022-09 | RAN#97-e | RP-222186 | 0080 | 1 | F | Correction on Measurement Time Occasion | 17.2.0 |
| 2022-12 | RAN#98-e | RP-222886 | 0086 | 2 | F | Correction of TRP TEG | 17.3.0 |
| 2022-12 | RAN#98-e | RP-222886 | 0087 | 1 | F | Correction of Timing Error Margin | 17.3.0 |
| 2022-12 | RAN#98-e | RP-222886 | 0088 | 1 | F | Correction of Positioning Information Transfer function | 17.3.0 |
| 2022-12 | RAN#98-e | RP-222886 | 0089 | 2 | F | Correction to the PRS Measurement configuration procedures | 17.3.0 |
| 2022-12 | RAN#98-e | RP-222887 | 0092 | 1 | A | CR to 38.455 on SRS periodicity | 17.3.0 |
| 2022-12 | RAN#98-e | RP-222886 | 0093 | - | F | Correction on presence of timing error margin for TRP TEGs | 17.3.0 |
| 2023-03 | RAN#99 | RP-230597 | 0099 | 1 | A | Correction for SRS Configuration status in Positioning Information Update | 17.4.0 |
| 2023-03 | RAN#99 | RP-230593 | 0100 | 1 | F | NRPPA corrections of references to RRC | 17.4.0 |
| 2023-06 | RAN#100 | RP-231077 | 0103 | 1 | F | SRS Resource correction on Comb 8, Number of Symbols and Repetition Factor | 17.5.0 |
| 2023-06 | RAN#100 | RP-231077 | 0105 | 1 | F | Subcarrier Spacing correction | 17.5.0 |
| 2023-12 | RAN#102 | RP-233850 | 0116 | - | A | Correction of NR E-CID for OnDemand measurements | 17.6.0 |
| 2023-12 | RAN#102 | RP-233850 | 0118 | - | F | Correction to NRPPa for the misalignment on DL PRS | 17.6.0 |
| 2024-03 | RAN#103 | RP-240642 | 0137 | - | F | Correction on Time Stamp and FR2 | 17.7.0 |
| 2024-06 | RAN#104 | RP-241114 | 0146 | 1 | F | Addition of missing positioning SIBs | 17.8.0 |
| 2024-12 | RAN#106 | RP-243104 | 0167 | 2 | A | Positioning measurement correction related to "shall, if supported" | 17.9.0 |
| 2024-12 | RAN#106 | RP-243104 | 0172 | 3 | A | Correction onFR1 SRS Bandwidth in Requested SRS Transmission Characteristics | 17.9.0 |
| 2024-12 | RAN#106 | RP-243104 | 0174 | 3 | F | introduction of Missing bandwidths in FR1 SRS Bandwidth | 17.9.0 |
| 2025-03 | RAN#107 | RP-250141 | 0184 | 2 | A | Correction on Assistance Information Control Procedure | 17.10.0 |
| 2025-12 | RAN#110 | RP-25xxxx | 0195 | 3 | A | Correcting the IE extension definition for bandwidth SRS | 17.11.0 |
| 2025-12 | RAN#110 | RP-25xxxx | 0196 | 3 | F | Correcting the IE extension definition for SRS Pos SIB Type | 17.11.0 |