3GPP TSG-RAN WG3 #101	Tdoc R3-185017

Gothenburg, Sweden, 20th – 24th August

Agenda Item:	31.3.4.8

Source:	Ericsson

Title:	Correction of C-RNTI format

Document for:	TP for BL CR for TS 38.473

# Introduction

In TS 38.473, C-RNTI has a different type compared to TS 38.331. In the following we explain the reasons for the discrepancy and propose a simple solution.

# Discussion

In TS. 38473 we read the following definition for C-RNTI.

#### 9.3.1.32	C-RNTI

This IE contains the C-RNTI information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | M |  | BIT STRING (SIZE(16)) | C-RNTI as defined in TS 38.331 [8]. |

On the other hand, in TS 38.331 the RNTI-Value is defined as follows:

#### –	RNTI-Value

The RNTI-Value IE represents a Radio Network Temporary Identity.

RNTI-Value information element

-- ASN1START

-- TAG-RNTI-VALUE-START

RNTI-Value ::=						INTEGER (0..65535)

-- TAG-RNTI-VALUE-STOP

-- ASN1STOP

It is obvious that the type of C-RNTI is different in the two specifications. The reason for this is that when the C-RTNI was introduced in TS 38.473, the NR C-RNTI was not yet defined, so the LTE definition was used eventually.

Of course, the best approach would be to use the same type in all the specifications, so we propose to align the C-RNTI type in TS 38.473 [1] with the one in TS 38.331. Also, although it is not extensible in TS 38.331, we propose to make it extensible in TS 38.473 for future-proof reasons.

Proposal 1:	We propose to align the C-RNTI type in TS 38.473 with the one in TS 38.331 .

Proposal 2:	RAN3 is kindly requested to agree with the text proposal in Annex 1.

# Conclusion

In this contribution, we discussed C-RTNI format over the F1 interface.

Proposal 1:	We propose to align the C-RNTI type in TS 38.473 with the one in TS 38.331 .

Proposal 2:	RAN3 is kindly requested to agree with the text proposal in Annex 1.

# References

TS 38.473 V15.2.1, F1 Application Protocol.

# Annex 1: TP for the SA BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

#### 9.3.1.32	C-RNTI

This IE contains the C-RNTI information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | M |  | INTEGER (0..65335, …) | C-RNTI as defined in TS 38.331 [8]. |

Start of Text Proposal 2 for TS 38.473

C-RNTI ::= INTEGER (0..65335, ...)

CUtoDURRCInformation ::= SEQUENCE {

cG-ConfigInfo						CG-ConfigInfo						OPTIONAL,

uE-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList		OPTIONAL,

measConfig							MeasConfig							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CUtoDURRCInformation-ExtIEs} } OPTIONAL,

...

}

End of Text Proposal for TS 38.473