3GPP TSG-RAN WG3 Meeting #103	R3-190360

Athens, Greece, 25th February - 1th March 2019

Agenda Item:	13.2.5

Source:	Ericsson

Title:	Support for LTE Deployment at IAB Node Sites

Document for:	Agreement

# Introduction

At the RAN2#104 meeting it was agreed to support IP connectivity to the IAB node. One of the drivers for supporting IP connectivity was to enable operators to provide backhaul also for other IP-connected equipment at the IAB site, including LTE eNBs. This paper discusses how LTE eNBs over IAB backhaul can be supported with minimum impacts.

# Discussion

As part of the IAB node integration the IAB node will register to the network and obtain one or more IP addresses. In [1] it is argued that DHCP could be used to obtain IP addresses.

With such a solution it would be easy for the IAB node to also offer IP connectivity services to external devices e.g. using ethernet ports. IP address assignment for these external devices can be done using DHCP, where the IAB node can act as a DHCP relay. QoS support can be provided based on DSCP/IP flow labels. The IAB node and Donor DU can map the DSCP to backhaul bearers.

This solution can be used to support LTE eNBs at the IAB site. In this case the S1 and X2 interfaces for the LTE eNBs will be backhauled over the IAB backhaul RLC channels as illustrated in Figure 1. The LTE eNBs / S-GWs should (according to TS 36.414) support configurable mapping between QoS QCIs and other traffic parameters to DiffServ code points (DSCP) which can be used for mapping to backhaul RLC channels.

Figure1: Support for LTE deployment at IAB node sites

The architecture can also support backhauling of split LTE eNBs supporting W1 interface where only the DU part is terminated at the IAB site. No additional specification work is needed to support LTE backhauling.

LTE backhauling should be supported in Rel-16 by IAB nodes providing IP/Eth. connectivity to LTE eNBs (or LTE DUs). The IP anchor point for the LTE nodes can be supported in the Donor DU.

QoS support for LTE traffic should be supported using DSCP  RLC bearer mapping in the IAB node and Donor DU.

IP address management can be provided to the LTE nodes using DHCP, e.g. where the IAB node act as a DHCP relay.

# Conclusions

Based on the discussion in earlier sections we propose the following:

Proposal 1	LTE backhauling should be supported in Rel-16 by IAB nodes providing IP/Eth. connectivity to LTE eNBs (or LTE DUs). The IP anchor point for the LTE nodes can be supported in the Donor DU.

Proposal 2	QoS support for LTE traffic should be supported using DSCP  RLC bearer mapping in the IAB node and Donor DU.

Proposal 3	IP address management can be provided to the LTE nodes using DHCP, e.g. where the IAB node act as a DHCP relay.

# References

[1] R3-190352; “IP Address Assignment for IAB Nodes”, Ericsson, RAN3#103, February 2019.