3GPP TSG-RAN WG3 Meeting #105	R3-194661

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	17.2.4.2

Source:	Ericsson

Title: 	Summary of offline discussion on Stage 3 for Solution #1 of Key Issue 1: Redundant user plane paths based on dual connectivity

Document for:	Discussion and Approval

# 1	Summary of offline discussion

CB_NRIIOT #4 Stage3_Sol1

- RSN or PDU session pair information or both?

- other open issues listed above

- Stage3 CR sets, if agreeable?

(E///)

Please note that in TS 23.501, SA2 has captured the below:

NOTE 2: The following redundant network deployment aspects are within the responsibility of the operator and are not subject to 3GPP standardization: - RAN supports dual connectivity, and there is sufficient RAN coverage for dual connectivity in the target area. - UEs support dual connectivity. - The core network UPF deployment is aligned with RAN deployment and supports redundant user plane paths. - The underlying transport topology is aligned with the RAN and UPF deployment and supports redundant user plane paths. - The physical network topology and geographical distribution of functions also supports the redundant user plane paths to the extent deemed necessary by the operator. - The operation of the redundant user plane paths is made sufficiently independent, to the extent deemed necessary by the operator, e.g. independent power supplies.

In order to decide if we need RSN or PDU session pair or both, let us first answer the below question.

Do you think it is enough to use RSN for this solution to work?

A: Using RSN is good enough. Only in the case that NG-RAN decides to setup the PDU session even the RSN is not fulfilled, it may result into less redundancy, because the two redundant PDU sessions can be setup on the same NG-RAN node.

B: Using RSN does not work. PDU session pair information is needed.

C: Using RSN works. But the solution is restricting. It is not flexible at NG-RAN node handling with MR-DC or non-MR-DC operations.

| Company | Preference | Comments |
| --- | --- | --- |
| Ericsson | A | NG-RAN node sets up the PDU sessions with RSN1 in the node with user plane RSN =1; NG-RAN node sets up the PDU sessions with RSN2 in the node with user plane RSN =2; In TS 23.501, it says: “PDU Sessions associated with different RSN values shall be realized by different, redundant UP resources”. SA2 allows RAN to setup or fail when the PDU session cannot be setup at the indicated RSN, depending on the local configuration. If NR-RAN node setup the RSN 1 in the user plane RSN =2, then the two redundant PDU Sessions may not have disjoint user plane. Since the two SMFs are not coordinated, and the pairing information is not available in the CN. It is not possible for 5GC to provide the PDU session pair information to NG-RAN node. |
| Nokia | B | Depends what we call RSN. RSN shall contain indication of the paired PDU session ID in order to enable RAN node to remain in control of the dual connectivity configuration at the time it receives PDU session setup i.e. when receiving PDU session setup, RAN shall decide the DC configuration based on radio context. This is as specified by SA2: The decision to set up dual connectivity remains in NG-RAN as defined today. If the ID of the paired PDU session is not provided, the redundancy user plane requirements which should be based on RAN policy and radio conditions is not possible. |
| catt | A | The RSN solution can work well in the normal cases. In the failure case, may have minor difference with the paired solution. If two SMF used, the paired information cannot be provided by CN if no enhancement on SMF setup the PDU session procedure. |
| Samsung | C | RSN is used to inform NG-RAN the redundant user plane requirement. It can be set to “1” or “2”, it can also be set to Associated PDU Session ID. The formal approach is too restrictive and may cause isolated UP configuration failure unnecessarily. The latter approach is to the point and leave the radio resource configuration decision in RAN. RAN should know PDU session pair information for flexible radio resource configuration. |
| Huawei | C | As discussed in our paper [R3-194060], the PDU session pair information is beneficial to enable flexible handling with MR-DC or non-MR-DC operations. Though the final answer will be provided by SA2, we don’t see any obstacles that CN provides the PDU pair information to NG-RAN. |
| ZTE | C | Agree with Huawei. Since pairing information is much useful for RAN to configure the redundant PDU session with disjoint user resource (e.g., more than 2 disjoint user plane resources), we wish SA2 can provide this information. However, so far, without the pairing information, RAN still can work with RSN value, but it would bring bad way for RAN configuration in some cases. |
| LGE | C | Basically, the RSN solution can work well without the PDU session pair information. However, it seems that additional information (i.e., PDU session pair information) may be helpful for the RAN to setup redundant user plane requirements for the PDU sessions. Only a concern is that when two SMFs do not coordinate with each other, how the SMF provides the PDU session pair information to the RAN should be clarified. |

In summary:

- Two companies believe that using RSN is good enough. Only in the case that NG-RAN decides to setup the PDU session even the RSN is not fulfilled, it may result into less redundancy, because the two redundant PDU sessions can be setup on the same NG-RAN node.

- One company believes that using RSN does not work because the NG-RAN cannot always follow radio conditions when deciding MR-DC configuration or not. PDU session pair information is needed.

- Four companies believe that using RSN works. But the solution is restricting. It is not flexible at NG-RAN node handling with MR-DC or non-MR-DC operations.

# 2	Conclusion

Majority companies except one company believe that using RSN can work.

If deployed with more than two disjoint user plane paths, it is beneficial to have the additional pair information. In this case, NG-RAN can flexibly configure to make sure redundant PDU Sessions are deployed in Dual Connectivity when there is a partial failure (i.e. the PDU session cannot be set up at the requested RSN).

As a way forward, RAN3 can considers to proceed with two steps:

1.	Support the solution by using RSN;

2.	If the PDU session pair information can be provided to NG-RAN node, enhancements can be made to gain more flexibility.

But RAN3 is not sure if the 5GC is be able to provide the PDU session pair information to NG-RAN node without adding too much complexity.

A LS in R3-194757 is proposed to send to SA2 to request SA2 clarification.