3GPP TSG-RAN WG3 #107-e                                                            	R3-200442

24 – 28 Feb 2020 E-Meeting

Title:	LS to RAN2 on MRO for SN change

Response to:

Release:	Rel-16

Work Item:	          NR_SON_MDT-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name:	Liu Aijuan

E-mail Address:	Liuaijuan (at) catt (dot) cn

- Overall Description:

In case of MR-DC, the SCGFailureInformation message includes SN change failure which can be detected by the UE as one kind of SCG failure. In order to find out the cause of SN change failure and optimize inappropriate parameter setting of the triggering condition, RAN3 reached agreement on other information which is necessary for MRO in SCGFailureInformation as below:

- previousPSCellId: the source PSCell of the last SN change;

- failedPSCellId: the PSCell in which RLF is detected or the target PSCell of the failed SN change;

- timeConnFailure :the time elapsed since the last SN change initialization until connection failure;

- connectionFailureType: radio link failure or SN change failure.

- 2. Actions:

To RAN2:

ACTION: RAN3 respectfully asks RAN2 to take the above into account, to reply to the questions presented and to update their corresponding specifications.

3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #107-e	    24th- 28th   February   2020	      E-Meeting

TSG-RAN3 Meeting #107bis          20th-24th     Apr 2020                        Japan