3GPP TSG-RAN WG3 Meeting #107bis-e 	R3-202611

- E-Meeting, 20th April - 30th April, 2020

Agenda Item:	9.3.8.1

Source:	Samsung

Title:	Summary of offline discussion on CB: # 107_RemappedQoSflow

Document for:	Discussion

# Introduction

This is the summary of offline discussion on CB # 107_RemmapedOoSflow.

Our chairman summarized the content discussed.

E///: no issue with this; scenario is not relevant (releasing an empty DRB should not be a problem)

HW,Nok,ZTE: maybe needed only for E1

CB: # 107_RemappedQoSflow

-  CRs agreeable? If so, revise as needed

(SS - moderator)

Summary of offline disc R3-202611

The following chapter requests view from several companies.

# Discussion

There are two issues proposed in the discussion paper [1] at this meeting, both related to QoS flow re-mapping.

The first issue is about release timing of the DRB during QoS flow re-mapping, and the second one deals with the QoS flow re-mapping during handover. In detail, selecting the QoS flows for the UL data forwarding proposal and the Old QoS Flow List provision.

Based on the comment during online discussion, we focus on the issue on the E1 impact, related with the Old QoS Flow List, which should be provided by the source CU-UP to the source CU-CP.

| Description: As described in [1], the source node proves the Old QoS Flow List to the target node for the re-mapped QoS flows which SDAP end marker for UL has not been arrived at the source node. However, the CU-CP in the source node can’t know whether the UL end marker packet is received. Observation 3. With current specification, the CU-CP is not able to know whether the CU-UP has received SDAP end marker. Proposal 3. In response to the PDCP SN status request from the source CU-CP, the source CU-UP may provide the Old QoS Flow List with the PDCP SN Status information in case of QoS flow re-mapping. The revised draft CRs for E1AP, [2] and [3], are uploaded in the draft folder, which includes the following change: - [2], [3] propose to add the Old QoS Flow List - UL End Marker expected IE in the PDU Session Resource Modified List IE of the BEARER CONTEXT MODIFICATION RESPONSE message. Please provide company’s comment on the draft E1AP CRs. |  |  |
| --- | --- | --- |
| Company | Agreeable | Comment |
| Samsung | YES | For the source CU-CP to provide SN STATUS TRANSFER to the target CU-CP, it is essential to recognize for which QoS flows the source CU-UP has received SDAP end marker packet. The new IE is needed. |

Any other comments related to this issue if you have.

| Company | Comment |
| --- | --- |

# Conclusion

# Reference

- R3-202443, discussion, Handling re-mapped QoS flow over F1 and E1 interface, Samsung

- R3-20xxxx, Rev1 of R3-202446, Samsung

- R3-20xxxx, Rev1 of R3-202447, Samsung