3GPP TSG-RAN WG3 #108-e	R3-204180

Agenda item:		17.2.2

Source: 				CATT

Title: 	Summary of offline discussion on CB: # NRIIOT3-ULPDCPdup_ctrl

Document for:		Approval

- Introduction

CB: #  NRIIOT3-ULPDCPdup_ctrl

-  UL transmission for multiplication over up to 4 RLCs:

- MAC entity controls its own part seems agreeable, assistance information exchange needs to be supported or not? (ZTE, HW, CATT, CMCC, NN)

- What kind of assistance information needs to be exchanged between the assisting nodes, e.g., UL Radio quality index, RLC activation status, radio quality assistance information? Report granularity, e.g., per RLC? (ZTE, HW, CATT, CMCC)

- Which node configures the initial UL duplication configuration, e.g., the assisting node feedback the initial RLC activation status to hosting node in the DRB setup response message or the hosting node signalling the initial UL duplication to the assisting node in the DRB setup request message? (ZTE, HW, CATT, CMCC)

- The hosting node provides the primary path indication to the assisting node?(ZTE, HW)

- The hosting node provides the number of secondary RLCs of DRB IE to the assisting node? (ZTE)

- The hosting node to decide about the split of RLCs in the UE between the MN and the SN?(NN)

- Add Additional PDCP duplication Information for SRB duplication over F1? (HW)

-  DL transmission for multiplication over up to 4 RLCs:

- The DL activation suggestion(Per DRB or RLC):per RLC, one RLC report in one frame or per RLC, all RLC reports in one frame? (ZTE, CMCC, CATT)

- How to implement the agreement Introduce the DL radio quality assistance information( with clarification text or the Assistance Information is extended to carry an ID of the RLC entity )? (ZTE, CATT, HW, E///, NN, CMCC)

- the number of tunnels in UL and DL may be different, the hosting node must indicate to the assisting node how many addresses to open for DL multiplication? (NN)

- reply LS to RAN2 on Network Coordination for UL PDCP Duplication, if agreeable? (HW, NN)

- attempt to converge on minimum agreeable set in R16; if so, revise/merge as needed, split work

(CATT - moderator)

Summary of offline disc R3-204003

This contribution will initial the email discussion and summarize the status of this discussion during the meeting.

The email discussion owner would like to trigger email discussions as below steps:

- Summarize the contributions which are submitted in section 17.2.2 in the meeting agenda.

- Group the topic and analysis the solutions

- For the issues on which we have same view from all the contributions, directly give out the agreement proposal

- For the issues on which we have different view, list all the solutions and questions for discussion.

- Converge the different the solutions during the email discussion, if get agreement, convert to agreement proposal.

- If we cannot get the convergence for the difference, we will do the online discussion for them

In this email discussion, we try to get the agreement for the solutions for all topics. Companies are welcome to provide answer for the questions by June 3rd, 12:00 UTC. We can make the second version base one the answers. We may optimize the topic in the second version for further discussion and finish the discussion by June 5th, 8:00 UTC. And the draft summary will be uploaded to inbox for online discussion. Then we can make the agreement proposal for the TP generation and work split base on the online discussion. So we may have one day to modify the TPs base on the discussion summary before the CB deadline June 9th, 13:00 UTC.

- For the Chairman’s Notes

Propose the following:

R3-203175(ZTE) for 38.423 revised in R3-204181 is agreed

R3-203654(HW) for 38.473 revised in R3-204182 is agreed

R3-203577(CATT) for 38.425 revised in R3-204183 is agreed

R3-203283 LS reply revised in R3-204168

Propose to capture the following:

Agreement:

PDCP hosting node provides the initial activation state

PDCP hosting node informs the assisting node the primary path location

Add Additional PDCP duplication Information for SRB duplication in F1

Correct the semantics of LCID IE

Removal the EN is agreed for Introduce the DL radio quality assistance information provided per RLC as it is in BL CR, no additional clarification is needed

LS reply to RAN2 about UL duplication coordination between two nodes

Issue list:

The below issues were discussed in rel-16 but no consensus was reached in RAN3

Exchange UL PDCP duplication activation state via UP

The node of MAC entity owner independently control the RLC entities in its node

Include radio quality information for UL duplication coordination in both ASSISTANCE INFORMATION DATA and DL USER DATA with LCID

- Discussion

## Summarize the contributions

Almost all the contributions are discussing the topic based on the issue list we captured in summary of email discussion in last meeting as below:

- Which node controls the UL duplication activation of RLC entity?

- Assistance information exchange between RLC entities for UL duplication

- UL PDCP duplication activation state

- UL Radio quality index

- UL PDCP duplication activation suggestion

- Assistance information per RLC

- Initial UL duplication configuration

- The number of allowed activated RLC entities from the hosting node to assisting node, fixed number or range?

- Which node determines the initial UL activation state?

- Introduce the DL activation suggestion

- per RLC

- per DRB

- How to implement the agreement Introduce the DL radio quality assistance information provided per RLC in spec

- introduce the flag for per RLC report

- Just update the text description without flag

- Whether we use one data frame for one RLC report or combine all the report of RLC entities in one data frame

So we will categorize our discussion topic based on the above list. After check all the contributions, all the companies still state their point view same as last meeting. So we don’t need to collect the view for the solutions again. I directly give out the compromise or majority supported proposal. All of you are welcome to provide the comments on the proposal. Also some questions are raised for some issues, please provide answer for them.

#### UL Duplication

- Which node controls the UL duplication activation of RLC entity?

The proposals from companies are listed as below. Nok and ZTE support that the node of MAC entity owner control the RLC entities in its node. Other three companies don’t support this solution.

| Proposal | Company |
| --- | --- |
| Proposal 1: The node hosting the MAC entity controls the UL duplication to ensure low-latency. Proposal 2: Although single MAC CE format including both MCG and SCG RLC activation status is defined, the node hosting the MAC entity can decide the UL duplication for its own part of RLC entities and then send the MAC CE to UE independently, that is, the MAC CE only contains the valid part of RLC bitmap referred to MCG/or SCG is allowed. | ZTE |
| Proposal 1-1: It is proposed to enable signalling where the nodes coordinate which RLCs each of them controls. Proposal 1-2: It is the hosting node to decide about the split of RLCs in the UE between the MN and the SN. | Nok |
| Proposal 1: Discuss the solution of each node controls its own part of the MAC CE in future release. | CATT |
| Proposal 1: Adopt a solution which will not impact RAN2 progress for UL duplication MAC CE sent over MN and SN. | CMCC |
| Proposal 1: No need to change the MAC CE agreement agreed in RAN2. | HW |

In 38.321, the R16 MAC CE format is defined as below:

-	RLCi: This field indicates the activation/deactivation status of PDCP duplication for the RLC entity i where i is ascending order of logical channel ID of secondary RLC entities in the order of MCG and SCG, for the DRB. The RLCi field is set to 1 to indicate that the PDCP duplication for the RLC entity i shall be activated. The RLCi field is set to 0 to indicate that the PDCP duplication for the RLC entity i shall be deactivated.

Figure 6.1.3.32-1: Duplication RLC Activation/Deactivation MAC CEConsidering the RAN2 impact and WI closed at this meeting, we have the below proposal. Please provide comments

: the node of MAC entity owner independently control the RLC entities in its node is not supported in R16

| Company | Comment |
| --- | --- |
| Huawei | Agree. It is too late to change RAN2 agreements, and also the proposal (each node controls its own RLC entities) has RAN2 impact, which should be discussed in RAN2 first (We notice that the proponent company proposes papers to this RAN2#110-e meeting. Unless further notification is notified, RAN3 just stick to current RAN2 agreement). |
| ZTE | Not agreed. We are concerned about why in the case of R15, each node can control UL duplication independently, why not in R16? From our point of view, independent control solution shall be supported. |
| CMCC | Support the proposal. |
| Ericsson | We support independent control. Believe that the quoted mechanism in TS 38.321 is valid and not impacted by the RAN3 decision. |
| CATT | Support this proposal. We don’t have enough time to have this solution in R16 spec. Even though RAN 2 agrees it, the RAN3 impact should be further discussed. For example, if SN only has only one RLC entity how the SN control it, when and why active and deactivate. if SN only has only one RLC entity how the SN control it, when and why active and deactivate. So we may go this approach in R17. At R16, we may just provide assist information between node. |
| Samsung | Independent control is in line with Rel-15. Seems R16 MAC CE format can support independent control. The main impact on RAN2 is the stage 2 description. If RAN2 can agree it in this meeting. There is no reason to forbid it in RAN3. |
| Nokia | Each node should control its part of MAC CE independently. RAN2 has asked RAN3 to enable mechanism where both nodes coordinate MAC CE setting. The fact that it is not feasible, does not mean that all the benefits of dynamic MAC CE setting shall be sent to trash. |

Looks this proposal cannot be agreed now. We need further discuss. 4 companies support the independently control in R16 and 3 companies agree the proposal which means not support in R16 due to RAN2 impact.

Considering this independently control solution should be based on RAN2 decision, we may wait the RAN2 conclusion.  Please Nokia do follow and update this status of RAN2 during this meeting. If RAN2 agree that the MAC CE can be independently controlled, RAN3 may go to this approach if time is enough. Otherwise, we should postpone it to R17. And go to the assist information exchange approach.

If we go to this approach, please Nokia provide the TPs

      - Hosting node indicate the RLC Nbrs in corresponding node and which MAC CE bit is assigned for them

      - The initial active state of RLC entity in receiving node

      - If we go to this approach, it means the two nodes can only control the RLC entities in its node. We should consider why RAN2 introduce 4 legs. If the 4 legs are in two nodes, it is mostly impossible to active all the 4 legs if no coordination. So we should consider how the two nodes handle the activation if it wants to active the RLC entities more than its own.

      - If one node only includes one RLC entity, how this node handles the active/deactivate. What is the trigger?

Proposal 1: Nokia follow the RAN2 discussion on the independently control, if RAN2 agreed before June 9 13:00 UTC, provide the RAN3 TP

- Assistance information exchange between RLC entities for UL duplication

- UL PDCP duplication activation state

- UL Radio quality index

- UL PDCP duplication activation suggestion

- Assistance information per RLC

For the sub item UL PDCP duplication activation suggestion, there is no contribution provided in this meeting. We will ignore it.

For sub item UL PDCP duplication activation state, we get two companies support

| Proposal | Company |
| --- | --- |
| Proposal 3: Even the channel quality always changes rapidly, but it does not mean the RLC activation state need to change frequently. For a gNB to construct and issue a MAC CE including both MCG and SCG RLC activation status, the NW shall support RLC activation status exchange between two nodes. It is up to UE’s implementation to handle whether the received MAC CE includes whole RLC bitmap, or the part of RLC bitmap referred to MCG/or SCG. | ZTE |
| Proposal 3: Introduce the UL PDCP duplication activation state in both DL USER DATA and ASSISTANCE INFORMATION DATA | CATT |

For this topic, we would like to collect the comments on it

Question 1: Is there beneficial for exchanging UL PDCP duplication activation state for the coordination on UL duplication activation in R16

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | No | It is not feasible to signal the “MAC CE” decision over related interfaces considering the transmission latency in between and the varying radio conditions. |
| ZTE | Yes | It is very important that the NW shall support constructing a MAC CE including all RLC information as RAN2 required. In order to construct the union MCA CE structure (seen in TS38.321, MCG leg + SCG leg), MCG can forward its own RLC activate/deactivate state to SCG, then SCG inserts MCG RLC state directly into the union MAC CE as well as its control SCG RLC state, same as MCG. In this way, no latency introduced and union MAC CE structure is performed, Meanwhile, it also has robustness, for instance, MCG or SCG has bad RLC quality so that UE cannot decode MAC CE activate/deactivate state correctly, this peer SCG or MCG MAC CE can be received by UE. Furthermore, Even the channel quality always changes rapidly, but it does not mean the RLC activation state need to change frequently. So, we think it is feasible to exchange the MAC CE decision in most cases, it is up to implementation. |
| Ericsson | No | Do not see usefulness and also consider latency. |
| CATT | Yes | It follows same principle as the initial state provided via CP, and same usage. After the initial active state exchange via CP, subsequently state exchange via UP. Initial state providing plus active state exchange is complete solution |
| Samsung | No | The delay need to be evaluated. It’s not an urgent issue for Rel-16. |
| Nokia | ? | Well, agreeing each MAC CE will indeed cause delay. Instead, the MN and the SN shall agree semi-statically which are the “areas” that each controls. Once done, no further signaling is needed. |

We cannot get convergence on this issue. We may postpone it to R17

Proposal 2: exchanging UL PDCP duplication activation state via UP is postponed to R17

For sub item - UL Radio quality index and - Assistance information per RLC, we will handle them together. The proposals from companies are listed as below.

| Proposal | Company |
| --- | --- |
| Proposal 4: It is recommended to focus on RAN2's requirements for constructing MAC CE with the whole RLC bitmap. The issue on the exchange of UL Radio Quality is left to future release. | ZTE |
| Proposal 2: Introduce the UL Radio quality index per RLC in both DL USER DATA and ASSISTANCE INFORMATION DATA | CATT |
| Proposal 2: Add UL Radio Quality Index in DL USER DATA frame. | CMCC |
| Proposal 2: Reuse the radio quality assistance information for UL duplication coordination with new indicator and the LCH ID in the ASSISTANCE INFORMATION DATA (PDU Type 2). Proposal 3: Add the Radio Quality Assistance Information and the LCH ID in the DL USER DATA (PDU Type 0). | HW |

ZTE would like to have it in future release. Other three companies provide similar proposal. Considering the RAN2 LS, in which RAN2 suggests we need to exchange some information for the UL duplication activation. We have the below proposal. Please comments on it.

Include radio quality information for UL duplication coordination in both DL USER DATA and ASSISTANCE INFORMATION DATA

| Company | Comment |
| --- | --- |
| Huawei | Yes, we can just reuse the radio quality assistance information for UL duplication coordination with new indicator. |
| ZTE | No, the exchanged radio quality of one node is not always in time for other node, it is enhancement, we do not see much benefit so far, and we can introduce in future release. |
| CMCC | Yes. Reuse UL radio quality parameter in ASSISTANCE INFORMATION, and add UL radio quality parameter in DL USER DATA. |
| Ericsson | Why is the need to include in DL user data? |
| CATT | Yes, we already include this liked information for DL duplication active. For UL we may follow the same principle. Answer to E///: Because the Hosting node also need provide the information of it to corresponding node via DL USER DATA |
| Samsung | Ok. Similar information for DL can also be included for UL. |
| Nokia | Why is the need to include in DL user data? |

Looks majority companies agree to have this proposal. So it turns to agreements

Proposal 3: Include radio quality information for UL duplication coordination in both DL USER DATA and ASSISTANCE INFORMATION DATA

If we agree the above proposal, there is still one open issue. For the radio quality assistance information in ASSISTANCE INFORMATION DATA, do we need to identify each RLC data frame by LCID like as HW’s proposal in R3-203656? Could we use the corresponding tunnel to differentiate the information belongs to. Please provide your comments on this question.

Question 2: For the radio quality assistance information in ASSISTANCE INFORMATION DATA, do we need to identify each RLC data frame by LCID?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | This is a clearer approach. As the following figure shows, in case of CU/DU architecture, there may need to transfer multiple radio quality assistance information per RLC entity in a single GTP-U tunnel. For example, the radio quality assistance information of two secondary RLC entities at CG1 need to be transferred from the DU1 to the DU2. Hence the logical ID should be added |
| Ericsson |  | Related to Q6. See no need. |
| CATT | No | My thinking is the CU may add the LCID for the assist information from different tunnel. So we may just add the LCID in DL user DATA is enough. Please correct me if my thinking is not reasonable |
| Samsung | No |  |

If the LCID included in assist information is not agreed. We may further discuss. Please companies provide your comments on it.

- Initial UL duplication configuration

- The number of allowed activated RLC entities from the hosting node to assisting node, fixed number or range?

- Which node determines the initial UL activation state?

| Proposal | Company |
| --- | --- |
| Proposal 5: It is already supported to signal the initial stage (true, or false) of CA and DC based duplication for the DRB over XnAP and F1AP. There is no need to introduce a new initial RLC duplication state IE sent by hosting node. Proposal 6: The assisting node shall apply the duplication according the initial states of CA and/or DC duplication for the DRB, furthermore, in the case of the DRB configured with more than two RLC entities, the assisting node shall determine how many and which RLC entity shall be activated and response the initial secondary RLC activation status to hosting node. Proposal7: It is proposed to introduce a new RLC activation status IE over XnAP/F1AP sent by assisting node to inform the initial secondary RLC activation status to hosting node. Proposal 8: Hosting node shall inform the assisting node the primary RLC entity location and the total number of the secondary RLC entities for the purpose of initial UL duplication configuration and MAC CE constructing. To introduce the new Primary RLC Indication IE and Number of secondary RLCs of DRB IE over XnAP/F1AP. Proposal 9: For the solution for node hosting PDCP signal the initial UL activation to assisting node, the primary RLC indication IE is also proposed to introduce over XnAP/F1AP. | ZTE |
| Proposal 5: MN sends the initial UL duplication activation state of each configured RLC to SN | CATT |
| Proposal 3: Follow R15 principle, i.e. the hosting node configure the initial UL activation state. | CMCC |
| Proposal 1: The hosting node determines the initial UL activation state and the primary path. Proposal 2: Add initial RLC duplication state of the DRB and primary path indication from the hosting node to the assisting node in TS 38.423. Proposal 3: For F1, add the initial RLC duplication state of the DRB and primary path indication in the UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATON REQUEST message for DRB duplication. Proposal 4: For E1, add initial RLC duplication state in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION RQUEST message. Proposal 5: For F1, add Additional PDCP duplication Information for SRB duplication. Proposal 6: Update the semantics for LCID as “LCID for split Secondary Path to split bearer”. | HW |

ZTE and HW provide the detail spec modification. CMCC and CATT just provide the general proposal. For the primary path indication, all the companies have the same view. i.e.  Hosting node shall inform the assisting node the primary RLC entity location. We have proposal as below. Please comments on it.

PDCP hosting node inform the assisting node the primary path location

| Company | Comment |
| --- | --- |
| Huawei | Agree, this is the same as Rel-15 operation. |
| ZTE | Agree, it is based on RAN2 progress.we suggest to introduce the Primary RLC Indication IE to indicate whether the primary RLC entity locates at the assisting node, e.g., the IE could be defined as ENUMERATED (True, False, …). |
| CMCC | Yes. |
| Ericsson | Do not we have this already related to LCID in the spec? |
| CATT | Yes |
| Samsung | Yes |
| Nokia | Yes, this is needed to indicate which RLC supports the primary path (the problem did not exist in Rel.15). |

Looks this proposal is agreed.

Proposal 4: PDCP hosting node inform the assisting node the primary path location

For which node determines the initial UL activation state, ZTE and HW have different view. The situation of this topic is same as last meeting. HW thinks that the PDCP hosting node determines initial UL activation state. CMCC and CATT have same view as HW on this topic. But ZTE thinks that the hosting node just informs the assisting node about Secondary RLC Number of DRB and the assisting node provide the initial UL activation state to hosting node. Even though we have it in R15, we may need to reconsider it in R16. What is the usage in the state information receiving node? If we cannot identify the usage, we may ignore this topic in R16.

We would like to raise some questions for this topic.

Question 3: Do we need to transfer the initial UL activation state over interface in R16? What is the usage of initial UL activation state in hosting node (ZTE proposal) or assisting node (HW proposal)?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | Yes to transfer the initial UL activation state over interface in R16. With this information, on one hand, the UE and RAN node can have the same understanding. On the other hand, the MAC CE either at the hosting node or the assisting node can be correctly implemented by each node. |
| ZTE | YES | The information provided by assisting node is needed to construct RRC message to UE. The RAN node also need to know the configured initial RLC(s) state, then after the configuration, the RAN node can dynamically control duplication via MAC CE. |
| CMCC | Yes | The initial UL activation state can help UE and RAN to have the same understanding, so that to perform appropriate initial scheduling. |
| Ericsson | Yes |  |
| CATT |  | I don’t think this initial active state providing via CP is different from the subsequently active state exchange via UP. If we only have the once initial active state at the DC setup, I don’t think it useful. Because the state will be changed quickly, the node still doesn’t know each other and the UE state with on time exchange. We should consider the complete solution. Not just follow the R15. |
| Samsung | Yes |  |
| Nokia | ? | Not sure, but if this “initial activation state” will enable splitting the MAC CE between the SN and MN, then it is needed (as commented above). |

Majority Companies think it is needed.

Question 4: If we agree to transfer the initial UL activation state over interface in R16, which one do you support (1. ZTE proposal; 2.HW proposal)?

| Company | 1/2 | Comment |
| --- | --- | --- |
| Huawei | 2 | This is aligned with Rel-15 principle. For example, the hosting node can decide: - whether the duplication is needed; - how many RLC entities can be setup at its side; - how many RLC entities can be at the assisting node (via the tunnel numbers); - the initial activation state; - the primary path is located in which node For option 1, it is not clear to us how this works. For example, for MN-terminated bearer, only the SN provides its own initial state to the MN (including the initial state for all secondary RLC entities? Even including the RLC entities at the MN side? ). How about the initial state of those RLC entities at the MN side? Does the MN need to signal its own initial state to the SN as well? //ZTE: SN only provides initial state of RLCs at SN side, SN does not need to know the initial state at MN side. |
| ZTE | 1 | We prefer 1, we think the assisting node can know the specific resource allocation and initialization of each RLC entity, so it is more feasible for the assisting node to configure the initial state of the RLC. we can accept 2, but the Primary RLC location and the Number of secondary RLCs of DRB shall be indicated to assisting node, and PDCP hosting node does not need to signal its own initial state to assisting node, for example, for MN-terminated bearer, the MN only signals the initial state of those RLC entities at the SN side to SN, the SN does not need to know the initial state at MN side . |
| CMCC | 2 | Both can work, but prefer to follow R15. |
| Ericsson |  | In Rel 15, it is the PDCP hosting node to provide. We think this can be reused; and provided per DRB level. |
| CATT | 1 and 2 | I would like suggest include both two. The hosting node may provide the intending active state and the corresponding may change it. And then exchange the active state via UP |
| Samsung | 2 | Prefer to follow R15. |
| Nokia |  | It would be more logical for the MN to decide on the multiplication activation and the number of RLCs in UL assigned for the SN. |

Majority companies support the hosting node provide the initial activation state

Proposal 5: the PDCP hosting node provide the initial activation state

Another issue raised by HW and described as below:

In Rel-16, up to 4 RLC entities can be set up for the SRB duplication transmission. The gNB-CU sends an additional IE indicating the duplication RLC number to gNB-DU. The duplication RLC number is up to 4.  The gNB-DU can set up the same number of the RLC entities based on this indicating number for the SRB duplication transmission. This function is missing in TS 38.473, and should be added in UE setup procedure and UE context modification procedure.

Question 5: Do we need to add Additional PDCP duplication Information for SRB duplication for F1?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | This should be supported. |
| ZTE | YES | We support it. |
| Ericsson |  | Should align with RAN2 agreement on this. |
| CATT |  | Should align with RAN2 agreement on this. |
| Samsung |  | Should align with RAN2 agreement. |

Looks all company support it if RAN2 agreed

Proposal 6: HW provides RAN2 agreement on add Additional PDCP duplication Information for SRB duplication for F1 and include it if RAN already agreed

#### DL Duplication

- Introduce the DL activation suggestion

- per RLC

- per DRB

| Proposal | Company |
| --- | --- |
| Proposal 1: The DL activation suggestion is provided per RLC, only one DL suggestion for the concerned RLC is included in an ASSISTANCE INFORMATION DATA frame. | ZTE |
| Proposal 6: Introduce the DL activation suggestion per RLC and carry all the RLCs’ suggestion of the whole DRB in one data frame of any RLC entities | CATT |
| Proposal 4: Support DL activation suggestion per RLC entity. | CMCC |

All the contributions agree that DL activation suggestion per RLC. But there is still one difference among these proposals. If the suggestion of each the RLC entity carry in one Data frame or different data frame.

Question 6: Does the DL suggestion of each the RLC entity carry in 1) one Data frame or 2) in data frame of each RLC entity?

| Company | 1/2 | Comment |
| --- | --- | --- |
| Huawei | none | We think the current radio quality assistance information is enough. There is no need to introduce the DL duplication suggestion per RLC entity in this release. |
| ZTE | 2 | We can reuse Rel-15 mechanism and extend it to per RLC leg. |
| CMCC | 2 | Prefer similar behavior as DL radio quality per RLC. |
| Ericsson |  | Not clear about the question. The data tunnel is between PDCP entity and the RLC entity. Could we confirm this understanding? |
| CATT |  | Both 1 and 2 acceptable. Clarify to E///: this question is to checking if the DL suggestion for each RLC entity included in one data frame for all the RLC entities from anyone of the RLC entities. Or include its own RLC entity data frame separately Question to HW: could you clarify what is your exactly mean? If we don’t do any change for the spec, it implicitly support DL suggestion per RLC because assistant information will provide per RLC via different tunnel |
| Samsung | none | Agree with Huawei |

Proposal 7: DL suggestion of each the RLC entity carried in data frame of each RLC entity

- How to implement the agreement Introduce the DL radio quality assistance information provided per RLC in spec

- introduce the flag for per RLC report

- Just update the text description without flag

- Whether we use one data frame for one RLC report or combine all the report of RLC entities in one data frame

| Proposal | Company |
| --- | --- |
| Proposal 2: Only one radio quality assistance information for the concerned RLC is included in an ASSISTANCE INFORMATION DATA frame. Proposal 3: There is no need to introduce a flag to indicate the radio quality assistance information or DL suggestion is per RLC or per DRB. To add some text clarification is benefit. | ZTE |
| Updating the text ( which is already implemented) is enough to carry the DL radio quality assistance information per DRB/RLC over the user data. | E/// |
| Proposal 2-2: Since the number of tunnels in UL and DL may be different, the hosting node must indicate to the assisting node how many addresses to open for DL multiplication. | Nok |
| Proposal 7: Just update the text description without flag for introducing the DL radio quality assistance information provided per RLC | CATT |
| Proposal 5: Prefer just to update the text without flag for DL radio quality. | CMCC |
| Proposal 1: Remove the editor’s note, i.e. no new flag is introduced to indicate the assistant information per RLC entity in the ASSISTANCE INFORMATION DATA frame (PDU Type 2). Proposal 2: Add a note to indicate the RLC entity level is applied to more than two RLC entities configured. | HW |

Except Nokia, all other companies agree that only adopt the solution updating the text (which is already implemented). So we have the below proposal

Introduce the DL radio quality assistance information provided per RLC as is modification in spec, and remove the EN added in last meeting.

| Company | Comment |
| --- | --- |
| Huawei | Agree. But we think a note is needed to clearly indicate under what condition the per RLC information is used, .e.g. “Note: The corresponding node reports the Radio Quality Assistance Information for the concerned RLC entity when more than one secondary RLC entity are configured for at least one DRB for packet duplication as described in 3GPP TS 38.300 [9].” |
| ZTE | Agreed. The text updating is needed, e.g., Note: The corresponding node provides UL/DL Radio Quality Index of each RLC entity in case of data radio bearer configured with more than two RLC entities, otherwise, the corresponding node provides UL/DL Radio Quality Index of the data radio bearer. |
| CMCC | Share similar view as HW. |
| Ericsson | See no need for the Note. |
| CATT | Share same view as E///. No need for the note. |
| Samsung | We also don’t understand why the note is needed. |
| Nokia | The question is still here: is the number of UL tunnels always the same as for DL? If not, then the Assistance Information must carry some DL RLC ID. If not, then handling of the tunnels must be defined (e.g. always max number – but how the SN knows how many RLCs it may use?). |

Majority companies agree this proposal. It turns to agreement

Proposal 8: Introduce the DL radio quality assistance information provided per RLC as is modification in spec, and remove the EN added in last meeting.

#### Other issues

| Company | Comment |
| --- | --- |
| Huawei | For Split secondary path, we propose to update that the LCID IE can be used to indicate the primary path or split secondary path for fallback to split bearer as follows, in order to correctly reflect TS 38.331. >>LCID; O; 9.2.3.70; LCID for primary path or LCID for split secondary path for fallback to split bearer if PDCP duplication is applied. The primary path is also used for fallback to split bearer operation.. |
| ZTE | If assisting node is aware of the primary RLC location, the correction is ok for us. |
| CMCC | OK for the correction. |
| Ericsson | The existing text seems fine. |
| CATT | Share with E/// |

No convergence for this correction, we may keep it as is.

Proposal 9: keep the semantics of LCID as is

## Proposal for second round discussion

Proposals are listed below based on the first round discussion. Please comments on them

Proposal 1: Nokia follows the RAN2 discussion on the independently control, if RAN2 agreed before June 9 13:00 UTC, provide the RAN3 TP

Considering this independently control solution should be based on RAN2 decision, we may wait the RAN2 conclusion.  Please Nokia do follow and update this status of RAN2 during this meeting. If RAN2 agree that the MAC CE can be independently controlled, RAN3 may go to this approach if time is enough. Otherwise, we should postpone it to R17. And go to the assist information exchange approach.

If we go to this approach, please Nokia provide the TPs

      - Hosting node indicate the RLC Nbrs in corresponding node and which MAC CE bit is assigned for them

      - The initial active state of RLC entity in receiving node

      - If we go to this approach, it means the two nodes can only control the RLC entities in its node. We should consider why RAN2 introduce 4 legs. If the 4 legs are in two nodes, it is mostly impossible to active all the 4 legs if no coordination. So we should consider how the two nodes handle the activation if it wants to active the RLC entities more than its own.

      - If one node only includes one RLC entity, how this node handles the active/deactivate. What is the trigger?

Proposal 2: exchanging UL PDCP duplication activation state via UP is postponed to R17

Proposal 3: Include radio quality information for UL duplication coordination in both DL USER DATA and ASSISTANCE INFORMATION DATA

The Hosting node also need provide the information of it to corresponding node via DL USER DATA

Proposal 4: PDCP hosting node inform the assisting node the primary path location

Proposal 5: the PDCP hosting node provide the initial activation state

Proposal 6: HW provides RAN2 agreement on add Additional PDCP duplication Information for SRB duplication for F1 and include it if RAN already agreed

Proposal 7: DL suggestion of each the RLC entity carried in data frame of each RLC entity

Proposal 8: Introduce the DL radio quality assistance information provided per RLC as is modification in BL CR, and remove the EN added in last meeting.

Proposal 9: keep the semantics of LCID as is

| Company | Comment |
| --- | --- |
| Huawei | P1: RAN2 discussed in [045][IIoT] possibly is waiting RAN3 coordination results to make a decision, so maybe a deadlock here. So we suggest to RAN3 to agree proposal 3 to allow to two nodes to exchange assistance information. P2: agree P3: agree, also include the LC ID. P4: agree P5: agree P6: RAN2#109 meeting agreed the following: - Rel-16 PDCP duplication is applied to SRBs. - For SRBs, all secondary RLC entities are activated when configured. - MAC CE based activation/deactivation of PDCP duplication is not supported for SRBs. P7: not sure the proposal. Does this mean the DL activation suggestion? Maybe this is not needed since the radio quality assistance information is enough. P8: about the Note. First let us clarify that both DRB level and RLC level assistance information can be supported in the UP for Rel-16. This is due to RAN2 agreement that Rel-15 MAC CE can also be used in the case that only two RLC entities configured. (the following agreement comes different direction). - Rel-15 Duplication MAC CE is not used for Rel-16 Duplication configuration (with more than two RLC entities configured). So there is need to specify in our UP specification when the DRB level is used, and when the RLC level is used. P9: well, it is not fully correct to say “The primary path…” since this is the LCID IE, not the primary path IE. |
| ZTE | P1: We wonder if RAN2 can make progress on this issue in this meeting. P2: We can follow majority company’s view. However, we still think forward (other than exchange) its RLC activate/deactivate state to peer node, it will not introduce latency and the receiving node inserts the RLC status as well as its own RLC stats, then build the whole MAC CE. P3: We can follow majority company’s view. However, in our view, forwarding RLC activate/deactivate state to peer node is more useful than forwarding RLC quality to peer node. Because receiving node needs to generate peer node’s RLC activate/deactivate which may be different from that generated by sending node. P4: agreed. P5: agreed. P6: agreed. P7: agreed. P8: agreed, and some clarification as the note is useful. P9: agreed. |
| CMCC | P1-P7: Agree P8: Share the same view as HW. The clarification as a Note is helpful. P9: To align with RAN2, prefer a correction as discussed in section 3.1.3. |
| Ericsson | P1: Our view is RAN3 is not hardly depended on RAN2 on this. P2: should remove “postponed to R17”, just state “is not agreed”; P3: I am not certain what is a “majority view”, If I count right, it is 3 companies negative and 4 companies positive on “DL User Data” part. DL USER DATA” part cannot be agreed. Could be further discussed of course. CATT: please refer to this picture, the PDCP will send CG2 the DL user DATA with assist information get from CG1 P4: Unclear about the impact to the spec. Is the approach the same as in Rel 15? If so, we are ok. P5, ok, per DRB CATT: what the exactly meanings of per DRB? P6: think apply to SRB per RAN2. P7, this proposal does not imply any change to the spec, correct? If so, we are ok. CATT: yes P8, just correct the proposal as “in BL CR”. CATT: agree P9 ok. |
| CATT | Ok for P1~P9 P1, if RAN2 cannot support MAC CE independently control, RAN3 cannot do anything. P3 refer the CATT’s comments inside E///’s comments P7 will not introduce any change in spec |
| Samsung | P1: Considering the current RAN2 status, maybe RAN2 will not change their mind. In this situation, we prefer not support independent control. P2: ok P3: need to think more about DL User Data P4-P9: ok |
| Nokia |  |

## Proposal for online discussion

Agreements:

1:  Include radio quality information for UL duplication coordination in ASSISTANCE INFORMATION DATA with LCID

2: PDCP hosting node informs the assisting node the primary path location

3: PDCP hosting node provides the initial activation state

4: add Additional PDCP duplication Information for SRB duplication in F1

5: Introduce the DL radio quality assistance information provided per RLC as is in BL CR, and remove the EN added in last meeting.

6: keep the semantics of LCID as is

Issue for online discussion:

1: The solution of MAC CE independently control may be discussed in R17 due to RAN2 agree not to change their spec

2: Whether include radio quality information for UL duplication coordination in DL USER DATA

The Hosting node also need provide the information of it to corresponding node via DL USER DATA

TP for the agreements:

R3-203175(ZTE) rev for 38.423 include agreement 2, 3

R3-203654(HW) rev for 38.473 include agreement 2, 3, 4

R3-203577(CATT) rev for 38.425 include agreement 1, 5

## Discussion after online

Agreements online:

PDCP hosting node provides the initial activation state

Add Additional PDCP duplication Information for SRB duplication in F1

Removal the EN is agreed for Introduce the DL radio quality assistance information provided per RLC as it is in BL CR

Open issue for further discussion:

a).  Include radio quality information for UL duplication coordination in ASSISTANCE INFORMATION DATA with LCID

b). whether include radio quality information for UL duplication coordination in DL USER DATA with LCID

Need to be further discussed on above two issues for replying RAN2 LS. Actually the issue may be descripted as below:

            - The LCID is needed

            - Whether information in the DL USER DATA is needed

As we discussed online, the radio quality information for UL duplication coordination need to be exchanged between two nodes and go to MAC entity for MAC generated. Not only from corresponding node to hosting node, but also from hosting node to corresponding node. If we only include in ASSISTANCE INFORMATION DATA, the solution doesn’t work. The CU doesn’t have responsibility for MAC CE generated.

CASE 1

CASE 2

The above two picture are the example for DC+CA duplication. In any case, as I mentioned, the information exchange should be bi-direction.

In case 1, the information of RLCa  in CG1 is carried in ASSISTANCE INFORMATION DATA from DU to CU/PDCP. Then the CU/PDCP should forward the information in DL USER DATA to DU via tunnel-b or tunnel-c and go to the MAC2. The MAC2 makes MAC CE for this DRB.

In  other hand, The information for RLCb in CG2 is carried in ASSISTANCE INFORMATION DATA from DU to CU/PDCP. Then the CU/PDCP should forward the information in DL USER DATA to DUvia tunnel-a or tunnel-p and go to the MAC1. The MAC1 makes MAC CE for this DRB. The RLCc is same as RLCb

In case 2, the procedure is similar as case1. The DU send the information carried in ASSISTANCE INFORMATION DATA to CU/PDCP. Then the CU/PDCP should forward the information in DL USER DATA via any tunnel and go to the MAC.

So we should support

Carry the radio quality information for UL duplication coordination in both ASSISTANCE INFORMATION DATA and DL USER DATA

Please provide your comments

| Company | Comment |
| --- | --- |
| CATT | It is mandatory in both directions if we agree transfer this information. |
| Nokia | All right |
| Huawei | Agree |
| Ericsson | Still do not see how the RLCs in DU2 make use of the radio Quality information sent from RLCs in DU1. This need to be further discussed. We are fine to have radio quality information in ASSISTANCE INFORMATION DATA, without LCID. Do not see benefit and need to send in DL USER DATA |
| ZTE | Agree |

For the LCID issue, because at beginning, the CU/PDCP doesn’t know the mapping of the tunnel and RLC/LCID entity, the CU/PDCP cannot link the information to the LCID and cannot add the LCID for this information in DL USER DATA which will be sent to other node. If no LCID include, the information receiving node doesn’t know the information belongs to which RLC/LCID

So I have the below suggestion:

            - Include LCID in both ASSISTANCE INFORMATION DATA and DL USER DATA

OR

            - Indicate the mapping of Tunnel and RLC/LCID at setup in control Plane

OR

            - Assign the RLC No. for each RLC entity

Please provide your comments

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |
| CATT | Option 1 | It is simple |
| Nokia | Not needed at all? | In another discussion, it was clarified that the number of tunnels in DL shall be the same as in UL. Therefore, even if not identified, the receiver will be able to build the mapping on it own, won’t it? |
| Huawei | Option 1 | Currently the CU is not aware of the LCIDs used for the packet duplication used at the DU1 and DU2 which is contained in the cell group information agnostic to the CU (except the LC ID for the primary path). And also the CU is not aware of the tunnel/LCID mapping as the moderator describes. So anyhow some kind of information is needed. Among three options, we prefer option 1. |
| Ericsson |  | See no need to add. |
| ZTE | Prefer Option 2, Option 1 is also acceptable. | I agree with moderator/HW that the CU is not aware of the tunnel/LCID mapping. In my view, LCID is static and is simultaneously configured with the tunnels. So it is better to configured by CP signaling other than UP, however, we can also follow majority company’s view. |

c). PDCP hosting node informs the assisting node the primary path location

E/// has concern on what is the difference from R15. Need more check?

A HW comments, This is only in the response message in R15, here the proposal is that the hosting node to tell the assisting node where the primary path during the request.

Please provide your comments

| Company | Comment |
| --- | --- |
| CATT | Agree to have it |
| Nokia | Yes, it seems needed. |
| Huawei | Agree. Based on the primary path indicator, - If true, the assisting node can know to allocate a LCID back to the hosting node, and the LCID is never deactivated. If false, the assisting node can allocate LCID as the fallback path to the split bearer. |
| ZTE | Agree, follow with the RAN2 progress. |

d). whether the additional clarification needed is still open for Introduce the DL radio quality assistance information provided per RLC as it is in BL CR

HW suggests add the below clarification:

Note: The corresponding node reports the Radio Quality Assistance Information for the concerned RLC entity when more than one secondary RLC entity are configured for at least one DRB for packet duplication as described in 3GPP TS 38.300 [9].

Please provide your comments

| Company | Comment |
| --- | --- |
| CATT | Don’t think it is necessary. Either Hosting or corresponding node knows how many RLC have. |
| Huawei | It is useful since both the DRB level and RLC entity can be supported in Rel-16. Note that we agree to follow majority’s views. |
| Ericsson | Don’t think it is necessary |
| ZTE | Agree with CATT |

f). keep the semantics of LCID as it is

HW suggests the below changing. Please check if it needed.

| >>LCID | O |  | 9.2.3.70 | LCID for primary path or LCID for split secondary path for fallback to split bearer if PDCP duplication is applied. The primary path is also used for fallback to split bearer operation. |
| --- | --- | --- | --- | --- |

Please provide your comments

| Company | Comment |
| --- | --- |
| CATT | Keep it as is in BLCR will not introduce confusion But I am fine with this correction. |
| Nokia | We like think the proposed clarification is helpful. |
| Huawei | Prefer to have it and update e.g. as. - LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied |
| Ericsson | Wording in BL CR is fine. |
| ZTE | Support to make it clear, same view with HW |

g) The following agreement is abstracted from RAN2 chairman note

 The presence of pdcp-Duplication indicates the PDCP duplication configuration (i.e. pdcp-Duplication is always used to indicate the PDCP duplication configuration for both DRBs and SRBs). The 38.331 and 38.323 specifications need to be changed accordingly.  The UE just follows the received MAC CE, even if the RLCi field belongs to the other node. No specification change is required.  PDCP duplication with more than two RLC entities is supported only by NR. It needs to be clarified in 37.340 and 38.331.  Clarify DC+CA duplication in 38.300. 3+1 duplication scenario also needs to be considered. CA duplication may need clarification. Wording to be worked on.  In the description of duplicationState in 38.331, remove “initial” and use “at the time of receiving this IE”.

ZTE thinks the 2nd bullet impacts on RAN3 previous agreement, i.e.

Proposal 2: exchanging UL PDCP duplication activation state via UP is postponed to R17

Form RAN2 agreement, the UE will not judge the MAC CE that which part is controlled by own part and other part is controlled by other part. For instance, for an MAC CE generated by MCG, the UE will not use the MCG part of the MAC CE and discard the SCG part of the MAC CE, although the UE can know which part is controlled by MCG and other part is controlled by SCG.

| Company | Comment |
| --- | --- |
| ZTE | Based on the RAN2 agreement, the UE will not judge the received MAC CE from both MN and SN, the UE will not discard the useless part of the MAC CE. RAN3 shall ask RAN2 to change the UE behavior (i.e., ignore the useless part of MAC CE), or revisit the previous agreement (i.e., RLC state exchange between MN and SN is needed). I still think forwarding its own RLC activate/deactivate state to peer node, in fact, will not introduce latency, The receiving node inserts the RLC status as well as its own RLC stats, then builds the whole MAC CE. |

TP for the agreements:

R3-203175(ZTE) rev for 38.423

R3-203654(HW) rev for 38.473

R3-203577(CATT) rev for 38.425

R3-203283 LS reply revised in R3-204168

## Summary of discussion

Agreements:

PDCP hosting node provides the initial activation state

PDCP hosting node informs the assisting node the primary path location

Add Additional PDCP duplication Information for SRB duplication in F1

Correct the semantics of LCID IE

Removal the EN is agreed for Introduce the DL radio quality assistance information provided per RLC as it is in BL CR, no additional clarification is needed

Include radio quality information for UL duplication coordination in both ASSISTANCE INFORMATION DATA and DL USER DATA with LCID

Reply LS to RAN2 about UL duplication coordination between two nodes

TP for the agreements:

R3-203175(ZTE) for 38.423 revised in R3-204181

R3-203654(HW) for 38.473 revised in R3-204182

R3-203577(CATT) for 38.425 revised in R3-204183

R3-203283 LS reply revised in R3-204168

- Conclusion, Recommendations

Refer to section 2

- References

- R3-203174	Consideration on UL PDCP duplication	ZTE

- R3-203175	(TP for Introduction of NR_IIOT support to TS 38.423) Initial UL duplication configuration	ZTE

- R3-203176	(TP for Introduction of NR_IIOT support to TS 38.473) Initial UL duplication configuration	ZTE

- R3-203177	(TP for Introduction of NR_IIOT support to TS 38.425) UL PDCP duplication	ZTE

- R3-203178	(TP for Introduction of NR_IIOT support to TS 38.425) Open issues on DL PDCP duplication	ZTE

- R3-203280	Coordination of UL and DL transmission for multiplication over up to 4 RLCs	Nokia, Nokia Shanghai Bell

- R3-203281	(TPs for NR_IIoT BL CRs for TS 38.423): Enabling coordination for DL and UL multiplication over up to 4 RLCs	Nokia, Nokia Shanghai Bell

- R3-203282	(TPs for NR_IIoT BL CRs for TS 38.425): Enabling assistance information for DL coordination of the multiplication over up to 4 RLCs	Nokia, Nokia Shanghai Bell

- R3-203283	Response LS on Network Coordination for UL PDCP Duplication	Nokia, Nokia Shanghai Bell

- R3-203415	Remaining issues on Dynamic Control	Ericsson

- R3-203576	Discussion on Dynamic control of PDCP Duplication	CATT

- R3-203577	(TP for [NR_IIOT] BL CR for TS 38.425)Dynamic control of PDCP Duplication	CATT

- R3-203653	(TP for NR_IIOT BL CR for TS 38.423): PDCP duplication with more than 2 entities	Huawei

- R3-203654	(TP for NR_IIOT BL CR for TS 38.473): PDCP duplication with more than 2 entities	Huawei

- R3-203655	(TP for NR_IIOT BL CR for TS 38.425): Assistance information for DL PDCP duplication with more than 2 entities	Huawei, CMCC

- R3-203656	(TP for NR_IIOT BL CR for TS 38.425): Dynamic control of UL duplication	Huawei

- R3-203657	[DRAFT] Reply LS on coordination for UL duplication	Huawei

- R3-203901	Remaining issues of dynamic control	CMCC