3GPP TSG-RAN3 Meeting #108-e	R3-203646

E-meeting, 01 - 11 June 2020

Title: 	(TP for WWC BL CR for TS 29.413) Proposed rapporteur changes to the BL CR

Source: 	Huawei

Agenda item:	21.1

Document for:	Other

1. Introduction

This contribution intends to propose the rapporteur changes to the BL CR for TS 29.413 [1]. Several new Non-3GPP access network nodes and related IEs are introduced.

-	Trusted Non-3GPP Gateway Function (TNGF) for the trusted non-3GPP access network interfacing to the 5GC.

-	Wireline Access Gateway Function (W-AGF) for the wireline 5G access network interfacing to the 5GC.

-	Trusted WLAN Interworking Function (TWIF) for the trusted WLAN access network interfacing to the 5GC.

As a result, some IEs are repeated multiple times for the different scenarios, e.g. Global RAN Node ID, User Location Information. Hence there is a need to re-organize these IEs for simplicity and readability.

Update the procedural texts for Global RAN Node ID, User Location for non-3GPP access.

3. Conclusion

Based on the discussion in this paper, the following proposals are made.

- Propose rapporteur changes to the BL CR.

4. Reference

- 3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

5. Annex – TP

Change Begins

## 5.3	Exceptions for NGAP message contents and information element coding when used for non-3GPP access [Highlighted in green]

<Unchanged Text Omited>

NG SETUP REQUEST message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

RAN CONFIGURATION UPDATE message:

-	the following IEs shall be ignored, when received:

-	Default Paging DRX IE

The Global RAN Node ID IE in the applicable NGAP messages includes the following IEs as specified in TS 38.413 [2]:

-	Global N3IWF ID IE for the untrusted non-3GPP access.

-	Global TNGF ID IE for the trusted non-3GPP access.

-	Global TWIF ID IE for the trusted WLAN access.

-	Global W-AGF ID IE for the wireline 5G access.

The User Location Information IE in the applicable NGAP messages includes the following IEs as specified in TS 38.413 [2]:

-	IP address IE and port number IE for the untrusted non-3GPP access.

-	TNGF User Location Information IE for the trusted non-3GPP access.

-	TWIF User Location Information IE for the trusted WLAN access.

-	W-AGF User Location Information IE for the wireline 5G access.

The Security Key IE in the applicable NGAP messages includes the N3IWF key as specified in TS 33.501 [5].

The RAN UE NGAP ID IE in the applicable NGAP messages identifies the UE association over the NG interface within the N3IWF node, or the TNGF node, or the TWIF node, or the W-AGF node, as specified in TS 38.413 [2].

End Change