3GPP TSG-RAN WG3 #108-e	R3-20xxxx

Online, 1-11 June 2020

Agenda Item:	9.3.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on PWS Cancel

Document for:	Approval

# Introduction

CB: # 81_PWScancel

- 3 options still on the table?

- DU shall not omit cell broadcast cancel list in reply

- Nok/SS could be considered equivalent? Slight preference for Nok? Any clarification in e.g. semantics needed?

- need to address case where no IEs are present?

- no “shall” statements in semantics

- if consensus on a solution, revise as needed; check details

(Nok - moderator)

Summary of offline disc R3-204050

# For the Chairman’s Notes

The following is proposed:

R3-20xxx1 – agreed (Rel-15 CR for TS 38.473)

R3-20xxx2 – agreed (Rel-16 CR for TS 38.473)

# Discussion

## Background

The purpose of the PWS Cancel procedure is to cancel an already ongoing broadcast of a warning message.  The procedure over NGAP was essentially copy/pasted from S1AP, while a corresponding F1AP procedure was added to support CU-DU split.

The PWS CANCEL REQUEST message enables cancelling of either a single warning message or all warning messages. The relevant IEs and their presence are summarized in the table below.

| Information Element | NGAP | F1AP |
| --- | --- | --- |
| Cancel-All Warning Messages Indicator IE | Optional | Optional |
| Message Identifier IE and Serial Number IE | Mandatory | Optional (Note 1) |

Note 1: In F1AP, the Message Identifier IE and Serial Number IE are contained in the Notification Information IE, introduced as a correction in v15.8.0 (December 2019)

As shown in the table, the Cancel-all Warning Messages Indicator IE and the Notification Information IE are both optional in the PWS CANCEL REQUEST message over F1AP. The problem which has been acknowledged by RAN3 is as follows:

- If neither IE is included, it is not clear how the gNB-DU responds.

Note that PWS Cancel is a class 1 procedure which does not have a message to report an unsuccessful outcome. The PWS CANCEL RESPONSE message can only indicate whether cancellation was performed successfully (i.e. cell included in Cell Broadcast Cancelled List IE), or no broadcast was ongoing (i.e. cell omitted from Cell Broadcast Cancelled List IE, or Cell Broadcast Cancelled List IE itself is omitted).

## Solutions

Three solutions have been proposed. Companies are invited to add any additional advantages or drawbacks.

Solution A: Change the presence of the Notification Information IE from optional to mandatory [1].

The Rel-15 CR in [2] proposes to make the Notification Information IE mandatory in the F1AP: PWS CANCEL REQUEST message. The PWS CANCEL REQUEST received by the gNB-CU over NGAP will always contain at least the Message Identifier IE and Serial Number IE (since these IEs are mandatory over NGAP), so they can just be copied to the Notification Information IE sent over F1AP.  Then, the DU can handle the PWS CANCEL REQUEST MESSAGE using the same logic that already exists in the CU.

Advantages:

- Solves the problem in an ASN.1 backwards compatible way.

- Aligns F1AP with NGAP.

Drawbacks:

-	The solution is not functionally backwards compatible. In particular, if a (new) DU implementing this change receives a PWS CANCEL REQUEST message from a (old) CU that does not implement this change, then the Notification Information IE may be absent which would trigger an abstract syntax error.

Solution B: Keep the presence of the Notification Information IE as optional and enhance the semantics description [3] and/or procedural text.

The Rel-15 CR in [4] proposes to keep the presence of the Notification Information IE as optional but add the following to the semantics description: “This IE shall be always present from this release.”

Advantages:

- The Notification Information IE remains optional in the tabular and ASN.1.

Drawbacks:

- The semantics description is not allowed to contain “shall” statements.

- Even if the “shall” statement is moved to procedural text, it seems to have the same drawback as Solution A (the only difference is that a logical error is triggered rather than an abstract syntax error?).

Solution C: Keep the presence of the Notification Information IE as optional and specify in the procedural text that the DU sends the PWS CANCEL RESPONSE message without the Cell Broadcast Cancelled List IE [5].

The Rel-15 CR in [6] proposes to keep the presence of the Notification Information IE as optional but add in the procedural text that the DU sends the PWS CANCEL REQUEST message without the Cell Broadcast Cancelled List IE.

Advantages:

- The Notification Information IE remains optional in the tabular and ASN.1.

Drawbacks:

- In the current specification, if the Cell Broadcast Cancelled List IE is absent then the gNB-CU considers that the gNB-DU had no ongoing broadcast to stop. But this is likely untrue and would create a status mismatch between the CBC and RAN.

## Company Views

Please provide your views on the proposed solutions and suggest possible way forwards.

| Company | Comment |
| --- | --- |
| Nokia | We believe that Solution A is the only one discussed so far that solves the problem in a way that is robust/reliable to satisfy regulatory requirements. Although functionally NBC, it is simple and aligned with existing logic in NGAP (and S1AP). Solution B does not improve upon Solution A, and there is no apparent benefit to mandate inclusion of an IE in procedural text rather than ASN.1 when there is consensus that the ASN.1 change is backwards compatible. Solution C does not address the problem. |
| One2many | We believe the problem is a bit theoretical. In practice it shouldn't happen that neither IE is present. Solution A solves the problem and Solution B is not allowed under the drafting rules (a "shall" in informative text) and Solution C creates a new problem. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-203306, PWS Cancel, Nokia, Nokia Shanghai Bell, Ericsson

- R3-203308, Correction to the Notification Information IE in PWS CANCEL REQUEST message, Nokia, Nokia Shanghai Bell, Ericsson

- R3-203606, Correction to PWS Cancellation, Samsung, LGU+

- R3-203607, CR on PWS Cancellation corrections over F1 (Rel-15), Samsung, LGU+

- R3-203259, Further Discussion on PWS Cancel, ZTE Corporation

- R3-203260, Correction on PWS Cancel, ZTE Corporation