3GPP TSG-RAN WG3 Meeting #109e	R3-204922

# Online, 17-28 August 2020

Agenda Item:	22.3.1

Source:	Lenovo, Motorola Mobility

Title: 	Support of Loss-less handover for 5G MBS

Document for:	Discussion and Approval

1	Discussion

A Rel-17 Work Item on NR support of Multicast and Broadcast Services was agreed in RP-201038 , and one of objectives is:

- Specify support for basic mobility with service continuity [RAN2, RAN3]

In this contribution, PDCP Count Value misalignment issue is discussed and potential solutions to support loss-less handover are discussed.

2	Discussion

In LTE, service continuity is supported between source and target eNBs for handover. In order to support lossless data transmission, upon handover, the source eNB may forward in order to the target eNB all downlink PDCP SDUs with their SN that have not been acknowledged by the UE. In addition, the source eNB may also forward without a PDCP SN fresh data arriving over S1 to the target eNB. The PDCP SN of forwarded SDUs is carried in the "PDCP PDU number" field of the GTP-U extension header. The target eNB shall use the PDCP SN if it is available in the forwarded GTP-U packet. Since in-sequence delivery during handover is based on a continuous PDCP SN, PDCP SN allocation should be aligned between source gNB and target gNB. Same principle are applied to NR as well.

Observation 1: PDCP Count Value allocation should be aligned and continuous between source gNB and target gNB to support loss-less handover.

The 5G MBS needs to support PTM mode. In PTM mode, the 5G MBS service is multicast over one or multiple cells. The source gNB and target gNB may allocate independent PDCP Count Value for the same packet from CN. One use case is the target gNB may joint the multicast IP group later than the source gNB. The PDCP Count value misalignment will cause packet lost during handover from source to target if the existing data forwarding and SN STATUS TRANSFER message are used.

- RAN3 to confirm the issue on PDCP Count Value misalignment which may cause packet lost during handover if the existing data forwarding and SN STATUS TRANSFER message are used.

There could be multiple solutions to solve the PDCP Count Value misalignment issue and enable loss-less handover.

Solution 1: PDCP Count Value Coordination based a SN from CN

In this solution, the PDCP count value is aligned with sequence number from CN, e.g. the gNBs assign same PDCP count value for a packet with same SN from CN. The gNBs may coordinate the mapping between PDCP PDCP Count value and the SN from CN over Xn. Based on the coordination, the neighbour gNB uses the same PDCP Count value for the packets with same SN from CN. If the SYNC protocol is used in 5G MBS, the gNBs may allocated PDCP count value associated with one or more SYNC header information. The GTP-U header may also include two bytes sequence number. The anchor gNB may allocated PDCP count value associated with GTP-U SN. A new NG SN can also be defined in the ‘RAN container’ in the GTP-U extension header.

Solution 2: Anchor PDCP based solution (DC-liked)

In this solution, in order to keep count value alignment between gNBs, an anchor PDCP concept is proposed:

- The PDCP count value assignment function is only located in a single gNB. An anchor gNB is defined with PDCP count value assignment function. The anchor gNB is selected in a specific area. Other gNBs in the area will connects to the anchor gNB for 5G MBS data transmission.

- The anchor gNB has 5GS MBS Session connection with CN. The data is processed by PDCP layer in the anchor gNB. The anchor gNB distributes the PDCP PDU to neighbor gNBs via Xn interface.

One issue of this solution, the service continuity can only be supported in a small area, due to a remote gNB may not have Xn interface with the anchor gNB.

Figure 1. Overview of PDCP anchor based solution

Solution 3: End marker and dedicated bearer based solution

As shown in the Figure 2, during handover procedure, the target gNB receives forwarded data and an End Marker indication from the source gNB. The target gNB sends the forwarded data via a dedicated bearer. During handover, the UE receives the forwarded data via the dedicated bearer and the data from the PTM MRB (multicast radio bearer) simultaneously. One drawback of this solution is there are two PDCP entities in UE side one for dedicated bearer, the other for MRB. The in-order delivery to upper layer in UE is not guaranteed.

Figure 2. Overview of end marker and dedicated bearer based solution

- RAN3 to discuss the penitential solutions if the issue on PDCP Count Value misalignment is confirmed.

3	Conclusion

In this contribution, PDCP Count Value misalignment issue is discussed and potential solutions to support loss-less handover are discussed.

Observation 1: PDCP Count Value allocation should be aligned and continuous between source gNB and target gNB to support loss-less handover.

- RAN3 to confirm the issue on PDCP Count Value misalignment which may cause packet lost during handover if the existing data forwarding and SN STATUS TRANSFER message are used.

- RAN3 to discuss the penitential solutions if the issue on PDCP Count Value misalignment is confirmed.