3GPP TSG-RAN3 Meeting #107bis-e	R3-204968

E-Meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | TP | rev | 7 | Current version: | BLCR |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (TP for BL CR for TS 38.470) Positioning support over F1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of support for positioning |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the procedure and introduction of new agreed procedure. FFS remains on procedure under discussion |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for positioning over F1AP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.X, 6.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... . |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 BL CR submission based on endorsed R3-197693 at RAN3#63 Rev2 Inclusion of agreed R3-201021 and remove change on change Rev3 update against last version of specifications Rev4 Inclusion of agreed R3-202777at RAN3#107bises, remove change on change, tracking change anonymisation Rev5 correction of typos remove change on change in last line Rev6: merge R3-204196 Rev7: BL CR submission for the meeting against last version of the specification |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

### 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of NR-MIB. In case broadcast of SIB1 and other SI messages is needed, the gNB-DU is responsible for the encoding of SIB1 and SIB10 and the gNB-CU is responsible for the encoding of other SI messages. The gNB-DU may re-encode SIB9.

The gNB-DU is responsible for the encoding of SIB12, SIB13 and SIB14 and broadcasting them over the air. The gNB-DU provides SIB12, SIB13 and SIB14 to the gNB-CU via the F1AP signalling.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g the positioning related SIBs. The gNB-CU notifies gNB-DU about the SIBs, and the gNB-DU signals them directly.

To support Msg3 based on-demand SI as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested other SIs.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

### 5.2.X	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows to transfer the positioning assistance data from gNB-CU to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>

### 6.1.x	Positioning procedures

The F1 Positioning procedures are listed below:

- Positioning Measurement procedure;

- Positioning Information Exchange procedure;

- TRP Information Exchange procedure;

- Positioning Activation procedure;

- E-CID Measurement Initiation;

- Positioning Information Exchange procedure;

- Positioning Assistance Information Control procedure;

- Positioning Assistance Information Feedback procedure;

- Positioning Measurement Report procedure;

- Positioning Measurement Abort procedure;

- Positioning Measurement Failure Indication procedure;

- Positioning Measurement Update procedure;

- Positioning Deactivation procedure;

- E-CID Measurement Failure Indication procedure;

- E-CID Measurement Report procedure;

- E-CID Measurement Termination procedure;

- Positioning Information Update; - FFS

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>