3GPP TSG-RAN WG3 #109-e	R3-205621

Online, 17-27 August 2020

Agenda Item:	31.3.6

Source:	Ericsson (moderator)

Title:	CB: # 88_X2mappingRACS - Summary of email discussion

Document for:	Approval

# Introduction

CB: # 88_X2mappingRACS

- consensus to introduce a new procedure?

- check details

(E/// - moderator)

Summary of offline disc R3-205621

# For the Chairman’s Notes

Propose the following:

1.	With one company objecting, it is proposed to follow the majority view and agreed to introduce a UE Radio Capability ID Mapping procedure, which is initiated by the en-gNB on X2.

2.	The CR for 36.300 is agreed in R3-205676 (revision of R3-205363 [10]).

3.	The CR for 37.340 is agreed in R3-205675 (revision of R3-204994 [9]).

5.	The CR for 36.423 is agreed in R3-205752 (revision of R3-205068 [13]).

# Discussion

## Consensus to introduce the mapping procedure on X2 for EN-DC

From what I sensed during the meeting, it appeared, that we can ask right away for agreement on introducing the mapping procedure on X2 for EN-DC. So the following agreement text is proposed:

1.	It is agreed to introduce an UE Radio Capability ID Mapping procedure, which is initiated by the en-gNB on X2.

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Huawei | agree |
| NEC | Agreed. Further explanation: The ue-CapabilityInfo is mandatory presence in CG-ConfigInfo for EN-DC during the SgNB Addition procedure. However when RACS is supported, this can be handled similar with the X2 handover case, so in the SgNB Addition Request message the UE Radio Capability ID is sent without including the ue-CapabilityInfo. With this assumption, we then think in order to complete the RACS function, and consider the en-gNB does not have C-plane connection with the CN, the UE Radio Capability ID Mapping procedure can be introduced in X2 for EN-DC only. Otherwise we would not need the RACS function in EN-DC, i.e. take the UE Radio Capability ID IE out from SgNB Addition Request message . The other way as commented, i.e. the SN can reject the SgNN Addition Request if the ue-CapabilityInfo is not included, then the MN can insert ue-CapabilityInfo in next SgNB Addition Request message, this will be the way to do when the SN is legacy node. But if SN support RACS, we would think to have the normal handling is better than abnormal handling. |
| Samsung | agree |
| Nokia | Disagree. It is unclear why cannot use a simple solution. As NEC commented, the SgNB can reject the Addition procedure. Right. The MeNB can provide the full radio capability and the capability ID. The SgNB can save the capability ID and the UE radio capability, and use them later if same capability ID is received. It will be a same behavior for the MeNB, no matter whether the SgNB does not support RACS, or support RACS but do not have the mapping. This is a very simple solution, without need the new X2AP procedure. Could you please clarify the benefit for introducing the new procedure? |
| CATT | Agree As we discussed the last meeting, the implementation based solution is workable. Just like the NG/S1, the mapping procedure could also be replaced by implementation based solution. By introduction of the new procedure, the behavior of the receiving node is much clear when it receives a UE Radio Capability ID and not local cached the corresponding UE Radio Capability information. |

## Stage 2 CR 36.300

Please don’t ask for reasons ;-) - the moderator [14] proposes the following:

2.	The CR for 36.300 is agreed in R3-205363 [10].

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Huawei | agree |
| NEC | agree |
| Samsung | agree |
| Nokia | No. please refer to comment in 3.1 |
| CATT | Agree |

## Stage 2 CR 37.340

The moderator proposes the following:

3.	The CR for 37.340 is agreed in R3-204994 [9].

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Huawei | agree |
| NEC | agree |
| Samsung | agree |
| Nokia | No. please refer to comment in 3.1 |
| CATT | Agree, but it’s suggested to change “UE radio capability ID” to “UE Radio Capability ID”, to align with the terminology used in the other specs. |

## Stage 2 CR 36.420

The moderator proposes the following:

4.	The CR for 36.420 is agreed in R3-205070 [12].

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Huawei | It seems to us that the CR is not quite needed since the UE Radio Capability ID Mapping is a function for EN-DC, and should not put in parallel with EN-DC. |
| NEC | It to provide the CR: - The Work item code should be “RACS-RAN” - the category should be “F” |
| Samsung | This CR doesn’t seem to be needed. Agree with Huawei’s comment. |
| Nokia | No. please refer to comment in 3.1 |
| CATT | Share the view with Huawei, the X2 mapping function is just one part of EN-DC function in TS 36.420. 5.2.1.6 should be enough. |

## Stage 3 CR 36.423

The moderator proposes the follwing:

5.	The CR for 36.423 is agreed in R3-205068 [13].

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Huawei | Agree. In section 7, some procedural texts (e.g. this function is used by the en-gNB to request a connected eNB to provide the UE Radio Capability information that maps to a specific UE Radio Capability ID) is needed after UE Radio Capability ID Mapping. |
| NEC | - the Work item code should be “RACS-RAN” - the category should be “F” - in Table 7-1, the added “UE Radio Capability ID Mapping” should be placed under the “E-UTRA-NR Dual Connectivity” since this is only for EN-DC. (but I also see the existing “EN-DC Configuration Transfer” is not under “E-UTRA-NR Dual Connectivity”) |
| Samsung | agree |
| Nokia | No. please refer to comment in 3.1 |
| CATT | Agree |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-204876 "RACS in X2/Xn and UE Radio ID Capability Mapping procedure in X2" (NEC) discussion

- R3-204877 "UE Radio ID Capability Mapping procedure in X2" (NEC) CR1514r, TS 36.423 v16.2.0, Rel-16, Cat. F

- R3-205135 "Discussion on X2 mapping procedure for RACS" (CATT) discussion

- R3-205136 "CR for TS36.423 support of X2 mapping for RACS" (CATT) CR1535r, TS 36.423 v16.2.0, Rel-16, Cat. F

- R3-205137 "CR for TS37.340 support of X2 mapping for RACS" (CATT) draftCR

- R3-205138 "CR for TS36.300 support of X2 mapping for RACS" (CATT) draftCR

- R3-204992 "Introducing UE Radio Capability Mapping procedure for EN-DC" (Huawei) discussion

- R3-204993 "Introducing UE Radio Capability Mapping procedure for EN-DC" (Huawei) CR1519r, TS 36.423 v16.2.0, Rel-16, Cat. F

- R3-204994 "Introducing UE Radio Capability Mapping procedure for EN-DC" (Huawei) draftCR

- R3-205363 "Introducing UE Radio Capability Mapping procedure for EN-DC" (Huawei) draftCR

- R3-205069 "Introducing X2AP UE Radio Capability ID Mapping procedure" (Ericsson) discussion

- R3-205070 "Introducing X2AP UE Radio Capability ID Mapping procedure" (Ericsson) CR0022r, TS 36.420 v16.0.0, Rel-16, Cat. B

- R3-205068 "Introducing UE Radio Capability ID Mapping procedure" (Ericsson) CR1532r, TS 36.423 v16.2.0, Rel-16, Cat. B

- moderator: (1) a person whose job is to help the two sides in a disagreement to reach an agreement (2) a person whose job is to make sure that a discussion or debate is fair (Oxford’s Advanced Learner’s Dictionary)