3GPP TSG-RAN WG3 #111-e						draft R3-211104

25 January – 4 February 2021

Online

Agenda Item:	9.3.8.1.

Source:	Samsung (moderator)

Title:	Summary of Offline Discussion on CB: # 96_SNmodAck

Document for:	Approval

# Introduction

CB: # 96_SNmodAck

- check usage: whether to allow multiple modifications (removal+setup etc.) in same procedure in this way?

- IE handling to allow this, if agreeable?

- commonality with similar issue discussed for NG at RAN3 #110-e?

- further details?

(SS - moderator)

Summary of offline disc R3-211104

# For the Chairman’s Notes

Propose the following:

R3-21xxxa, R3-21xxxc merged

R3-21xxxc rev [in xxxg] – agreed

R3-21xxxd rev [in xxxh] – agreed

R3-21xxxe rev [in xxxi] – agreed

R3-21xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion [if needed]

## Whether allow multiple modifications

In some cases, the sending node may decide to include both Setup, Modifiy or Release action in the same message, in order to save the signaling in the interface. It is the behavior used since 3G. We can find the corresponding description about this multiple modification in standard specifications. As showed in below as example.

In TS36.423: If the en-gNB receives a SGNB MODIFICATION REQUEST message containing multiple E-RAB ID IEs (in the E-RABs To Be Added List IE and/or the E-RABs To Be Modified List IE) set to the same value, the en-gNB shall not admit the action requested for the corresponding E-RABs. In TS38.413 (based on the R3-207163/4 agreed in RAN3#110e If the NG-RAN node receives a PDU SESSION RESOURCE MODIFY REQUEST message containing a PDU Session in the PDU Session Resource Modify Request List IE with the same QoS flow included in both the QoS Flow Add or Modify Request List IE and the QoS Flow to Release List IE,

Therefore we propose to agree:

- Multiple modifications (removal + setup etc) are allowed in the same procedure.

## How to acknowledge the multiple modifications

If multiple modifications are included in the same message, the addition/modification/release are for different bearers. The general principle used in dual connectivity is the SN should accept the release (i.e. there is no DRB not release List in the response message) and can reject the addition and modification. In X2, the E-RABs Admitted To Be Added List and E-RABs Admitted To Be Modified List are all optional IE. It is up SN to decide whether admit addition or modification.

A comment is received that the MN meant to configure the addition and release as the related actions if configuring addition and release in the same message. We think it is one implementation way and not guaranteed since.

- The SN may reject the addition/modification due to resource limitation but SN shall accept the release in any case. MN can not force SN to accept the addition even SN can not afford it.

- MN can not get the expected action, since only in Xn the DRBs Admitted to be Setup or Modified List is MP. In F1/E1/X2 this IE is optional. It is possible that DU or CU-UP reject the addition and accept the release. So even MN meant to configure related action for addition and release, but the MN can not get the expected action.

Based on the analysis we think there is possibility that the SN partially accepts the modification.

Therefore we propose to agree:

- It is allow SN partially accepts the modification.

Q1: Do companies agree with the agreement 1 and agreement 2?

| Company | Answer | Comment |
| --- | --- | --- |
| Samsung | Yes |  |

## Which solution is used

In R3-210230 and R3-210231  it is proposed the new optional IE is in below DRBs Admitted to be Setup or Modified List, there is another solution to propose insert this optional IE parallel with DRBs Admitted to be Setup or Modified List.

Solution 1:

#### 9.2.1.12	PDU Session Resource Modification Response Info – MN terminated

This IE contains the PDU session resource related result of an M-NG-RAN node initiated modification of DRBs configured with an MN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRBs Admitted to be Setup or Modified List |  | 1 |  |  |
| >DRBs Admitted to be Setup or Modified Item |  | 1 .. <maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs. |
| >>secondary SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs in case of PDCP duplication. |
| >>LCID | O |  | 9.2.3.70 | LCID for primary path if PDCP duplication is applied |
| >>No DRB admitted to be setup or modified | O |  | ENUMERATED (True, …) | If included, the DRB ID IE in DRBs Admitted to be Setup or Modified List IE shall be ignored. |
| DRBs Released List | O |  | DRB List 9.2.1.29 |  |
| DRBs Not Admitted To Be Setup or Modified List | O |  | DRB List with Cause 9.2.1.28 |  |

Solution 2:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRBs Admitted to be Setup or Modified List |  | 1 |  |  |
| >DRBs Admitted to be Setup or Modified Item |  | 1 .. <maxnoofDRBs> |  |  |
| >>DRB ID | M |  | 9.2.3.33 |  |
| >>SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs. |
| >>secondary SN DL SCG UP TNL Information | O |  | UP Transport Parameters 9.2.3.76 | S-NG-RAN node GTP-U tunnel endpoint(s) of the DRB’s Xn transport at its Lower Layer SCG resource. For delivery of DL PDUs in case of PDCP duplication. |
| >>LCID | O |  | 9.2.3.70 | LCID for primary path if PDCP duplication is applied |
| DRBs Released List | O |  | DRB List 9.2.1.29 |  |
| DRBs Not Admitted To Be Setup or Modified List | O |  | DRB List with Cause 9.2.1.28 |  |
| No DRB admitted to be setup or modified | O |  | ENUMERATED (True, …) | If included, the DRB ID IE in DRBs Admitted to be Setup or Modified List IE shall be ignored. |

Q2: Which solution is preferred? Or is there any suggestion on the solutions?

| Company | Answer | Comment |
| --- | --- | --- |
| Samsung | Solution 1 |  |

# Conclusion, Recommendations [if needed]

If needed

# References