3GPP TSG-RAN WG3 #111-e   			                                      R3-211109

25 January – 4 February 2021

Online

Agenda Item:	9.3.8.1

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on MaxAMFregions

Document for:	Approval

# Introduction

CB: # 99_MaxAMFregions

- clarify usage and scenario

- may be solved via proper selection in NG-RAN node?

(ZTE - moderator)

Summary of offline disc R3-211109

Note:

The first round email discussion plan to be end at end of Friday of the first week.(Friday 17:00 UTC 2021-1-29)

The second round email discussion plan to be end 2 hours before the on-line session (Thursday 11:00 UTC).

# For the Chairman’s Notes

Propose the following:

To have a chairman notes:

16 AMF regions already cover a very very large area. It is not practical to deploy gNB serving area more than 16 AMF regions. If an operator really have such deployment, they should accept the compromise that only 16 AMF regions are exchanged over Xn.

With this the CB can be closed

Propose to capture the following:

# Discussion

## Clarification of the usage and scenario

In [1], the company identify an issue for Max number of supported AMF Regions exceeded at NG-RAN node. The general description of the issue has been explained in [1]. In this part provide further clarification on the issue.

As shown in the figure, assume actually 20 AMF region acknowledged in NGAP, then the issue is how to transfer these AMF region to the new setup neighbor gNB in XnAP?

As defined in TS 38.423, copied below for easy reference , it is clear the the specification mandatory to ask source gNB provide complete AMF region list to the neighbor.

With limitation of only 16 items in current specification, how does source gNB provide complete AMF region list to the neighbor.

TS 38.423

The AMF Region Information IE in the XN SETUP REQUEST message shall contain a complete list of Global AMF Region IDs to which the NG-RAN node1 belongs. The AMF Region Information IE in the XN SETUP RESPONSE message shall contain a complete list of Global AMF Region IDs to which the NG-RAN node2 belongs.

Consequences not provide complete list:

1: first of all, break the law of specification , which is clearer.

2: can not fulfil the handover type decision: AMF region exchange is design for HO type decision, e.g. NG based HO or Xn based HO. If one gNB has 20 AMF region but only provide 16 items to the neighbour, it is highly possible the gNB will make wrong decision in HO type.

The possibility of the issue

First of all, this issue was discovered during the field test and was a practical problem. Secondly, this problem exists in theory, because the AMF region that an AMF can support can reach 256. When a NG RAN node  is connected to multiple AMFs, the possibility of the issue will exist more higher. The problem is that when the issue happen, there is no solution to deal with the problem reasonably.

Q1: Provide you view on identify the issue of Max number of supported AMF Regions exceeded at NG-RAN node.

| Company | Yes/no | Comment |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei |  | We agree that this issue exists in theory, at least according to the allowed maximum number in our spec. But we would like to consider whether there are real deployments where a single NG-RAN node can connect with more than 16 AMF regions. Up to 256 GUAMIs do not mean that more than 16 AMF regions are indeed signaled. In general we think first to confirm the real deployments before discussing the potential solutions. |
| Nokia |  | Share the same view as Huawei. 16 AMF regions already cover a very very large area. we doubt the real deployment that an gNB serving area >16 AMF regions. If an operator really have such deployment, they should accept the compromise that only 16 AMF regions are exchanged over Xn. |
| Ericsson |  | Share the view that with 16 AMF regions, it is very big area for a NG-RAN node to server. Also to point out that in NGAP we turn to define big numbers for future proof. |

Conclusion : No consensus.

## Candidate approaches for the issue

In [1], the company provides three candidate approaches.

Approach 1 [2]: Introduce a new cause value “Max number of supported AMF Regions exceeded at NG-RAN node” in NGAP.

After a successful NG SETUP procedure, NG-RAN node identifies that the number of new received AMF region ID contained in GUAMI(s) served by new AMF exceed 16. Then NG-RAN node sends an NGAP Error indication message to the AMF with new cause value as proposed in this contribution. In implementation, AMF and NG-RAN node suspends to server UE(s) in this connection and inform the situation to its OAM independently.

Approach 2: Expand the XnAP range of maxnoofAMFRegions from 16 to e.g. 256.

Approach 3: Other view ?e.g solved via proper selection in NG-RAN node ?

Q2: provide your view on approaches?

| Company | Approach ? | Comment |
| --- | --- | --- |
| ZTE | Approach 1 | For approach 3, multiple vendor scenario can not rule out. Then how does NG-RAN node do when facing the situation? For example, beak the law of specification ? |
| Nokia |  | No of the solution, before the scenario is confirmed. |

## Any other stuff?

Please provide your view on the Proposal.

| Company | Yes/no/other view | Comment |
| --- | --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-210571 Max number of supported AMF Regions exceeded at NG-RAN node (ZTE, China Unicom ,China Telecom)

- R3-210572 Rel-15 CR Max number of supported AMF Regions exceeded at NG-RAN node (ZTE, China Unicom ,China Telecom)

- R3-210573 Rel-16 CR Max number of supported AMF Regions exceeded at NG-RAN node (ZTE, China Unicom ,China Telecom)