3GPP TSG-RAN WG3 Meeting #112-e	R3-212058

# E-meeting, 17-28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0759 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SRB ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CATT, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 38.473, SRB ID IE that corresponds to SRB-Identity is defined with the value range of 0 – 3. However, the SRB-Identity specified in TS 38.331 is with the value range of 1 – 3, as SRB0 is not needed to be explicitly indicated over the radio. Regarding the above misalignment in the value range, the value 0 cannot be derived from the current Semantics description; therefore, it is needed to update the Semantics description of the SRB ID IE to enable it. - SRB ID IE in TS 38.473: IE/Group Name; Presence; Range; IE type and reference; Semantics description. SRB ID; M; INTEGER (0..3, ...); Corresponds to the SRB-Identity defined in TS 38.331 [8].. - SRB-Identity IE in TS 38.331: -- ASN1START -- TAG-SRB-IDENTITY-START SRB-Identity ::= INTEGER (1..3) -- TAG-SRB-IDENTITY-STOP |
| Summary of change: |  | Clearify the SRB ID IE corresponds to the SRB Identities defined in TS 38.331, instead of the SRB-Identity IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because the change only affects the description of SRB ID IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The value 0 of F1AP SRB ID IE cannot be derived based on current Semantics description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Start of Changes>>>>>>>>>>>>>>>>>>>>>>>>>>

#### 9.3.1.7	SRB ID

This IE uniquely identifies a SRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRB ID | M |  | INTEGER (0..3, ...) | Corresponds to the identities of SRB as defined in TS 38.331 [8]. Value 0 indicates SRB0, value 1 indicates SRB1, etc. |