3GPP TSG-RAN WG3 Meeting #112-e						R3-212275

Online, 17th – 27th May 2021

Title:	Resolve conflicts in the SN initiated SN modification procedure for EN-DC

Source:	Ericsson, NTT DOCOMO, INC.

Agenda Item:	9.3.8.1

Document for:	Discussion

# 1	Introduction

In EN-DC, the SN initiated SN modification procedure would be failed by sending SGNB MODIFICATION REFUSE message if any of the E-RABs is not successfully modified, i.e., partial success is not allowed for SN initiated SN modification with for SN terminated split bearers. Yet, there is remaining contradiction in the stage-3 specification. We would figure out how to solve the issue in this contribution.

# 2	Discussion

During SgNB initiated SgNB modification procedure, since only full success/failure is allowed, no E-RABs list had been included in the SGNB MODIFICATION CONFIRM message until RAN3#100 meeting. In [1]to support PDCP duplication with CA, a E-RABs Admitted To Be Modified List IE was introduced to the SGNB MODIFICATION CONFIRM message with the use of following procedural text.

| E-RABs Admitted To Be Modified List |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >E-RABs To Be Modified Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>EN-DC Resource Configuration | M |  | EN-DC Resource Configuration 9.2.108 | Indicates the PDCP and Lower Layer MCG/SCG configuration. | – |  |
| >>CHOICE Resource Configuration | M |  |  |  |  |  |
| >>>PDCP not present in SN |  |  |  | This choice tag is used if the PDCP at SgNB IE in the EN-DC Resource Configuration IE is set to the value "not present". |  |  |
| >>>>>Secondary MeNB UL GTP TEID at PDCP | O |  | GTP Tunnel Endpoint 9.2.1 | MeNB endpoint of the X2-U transport bearer at the PDCP. For delivery of UL PDCP PDUs for PDCP duplication. | – |  |

Concern was raised that the above change potentially brings different understanding whether the partial success/failure is possible in the SN initiated SN modification procedure. Companies in RAN3 agreed to follow legacy conclusion and clarified in [2] by adding the sentence below.

The MeNB shall include only E-RABs with the following IE in E-RABs Admitted To Be Modified List IE:

-	the Secondary MeNB UL GTP Tunnel Endpoint at PDCP IE.

However, in section 8.7.7.2, one paragraph remains.

For each E-RAB successfully modified or released as requested by the en-gNB, the MeNB shall inform the en-gNB, in the SGNB MODIFICATION CONFIRM message, the same value in the EN-DC Resource Configuration IE as received in the SGNB MODIFICATION REQUIRED message.

This is not aligned with the intention of the E-RABs Admitted To Be Modified List IE, also the success case for SN initiated SN modification procedure for EN-DC. Either we would restrict the scenario to PDCP duplication or simple remove it. The latter is slightly preferred.

Remove the procedural text above for partial success in the SN initiated SN modification procedure for EN-DC.

# Conclusion

We propose to solve the issue by proposing:

Proposal 1	Remove the procedural text for partial success in the SN initiated SN modification procedure for EN-DC.

CRs to X2 for R15 and R16 are provided in [3][4].

# References

R3-182939, Correction for PDCP Duplication, Ericsson

R3-185222, CR on clarification of partial success on SgNB initiated SgNB Modification over X2, NTT DOCOMO, INC.

R3-212276, Correction on SN initiated SN modification procedure for EN-DC, R15, Ericsson, NTT DOCOMO, INC.

R3-212277, Correction on SN initiated SN modification procedure for EN-DC, R16, Ericsson, NTT DOCOMO, INC.