3GPP TSG-RAN WG3 Meeting #112-e	R3-212377

E-meeting, 17 May – 27 May, 2021

Agenda item:	10.2.1.7

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for SON BL CR for TS 38.423) Enhancement of RACH Conflict Resolution

Document for:	Text Proposal

# 1	Introduction

This TP includes updates to TS 38.423, by introducing support for transfer of RACH assistance information on the Xn interface, comprising RACH Success and RACH Failure Information originating at a gNB-DU according to the discussion in [1].

# References

[1]		R3-212376, RACH Optimization Further Discussion, Nokia, Nokia Shanghai Bell

# Annex	- TP for TR 38.423

<<< start of changes >>>

#### 9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1 ® NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| RACH Conflict Resolution Info | O |  |  |  |  |  |
| >DU-CU RACH Failure and Success Information | O |  | 9.2.3.X |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

<<< next change >>>

#### 9.2.3.X	DU-CU RACH Failure and Success Information

The DU-CU RACH Failure and Success Information IE contains information about RACH successes and failures extracted at the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell To Report List |  | 1 |  | Cell ID list |  |  |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | 9.3.1.12 |  |  |  |
| >>SSB Item |  | 1..< maxnoofSSBIds > |  |  |  |  |
| >>>SSB-Index |  |  | INTEGER (0.. maxnoofSSBIds-1) |  |  |  |
| >>>Number of RACH Failures on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Successes on NUL Carrier | M |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the NUL carrier |  |  |
| >>>Number of RACH Failures on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of failed attempted RACH accesses, for the SSB, on the SUL carrier |  |  |
| >>>Number of RACH Successes on SUL Carrier | O |  | INTEGER (0.. 65535) | Number of successful attempted RACH accesses, for the SSB, on the SUL carrier |  |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBIds | Maximum no. of SSB items. Value is 64 |
| maxnoofSSBIds-1 | Maximum no. of SSB items minus 1. Value is 63 |

<<< end of changes >>>