3GPP TSG-RAN WG3 Meeting #112-e 	R3-212968

# E-Meeting, 17th May – 27th May, 2021

Agenda Item:	14.3

Source:	Samsung

Title:	(TP to TS 38.423, LTE_NR_DC_enh2-Core) Adding CPAC Procedure

Document for:	Approval

Introduction

This contribution provides TP for the CPAC to be included in the XnAP BL CR.

The following are WAs and agreements in RAN3#112-e.

Initiating node provides upper limit for the number of PSCells to be prepared (i.e. maximum number of PSCells). WA: initiating node provides suggested number of PSCells to be prepared. For CPA and MN initiated inter-SN CPC, initiating node should be informed of the number of prepared PSCells (i.e. via the prepared PSCell IDs). FFS for SN initiated inter-SN CPC. Introduce “List of Prepared PSCell IDs” in SN Addition Request ACK. Introduce “CPAC initiation Indication” in SN Addition Request, and SN Change Required. FFS whether to introduce “List of Prepared PSCell IDs” in SN Change Confirm.

This should be reflected in

- the S-NODE ADDITION REQUEST message

- the S-NODE ADDITION REQUEST ACKNOWLEDGE message

- the S-NODE CHANGE REQUIRED message

- the S-NODE CHANGE CONFIRM message

in the XnAP BL CR.

TP to 38.423 BL CR

- 8.3.1	S-NG-RAN node Addition Preparation

- 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

- 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

When the M-NG-RAN node sends the S-NODE ADDITION REQUEST message, it shall start the timer TXnDCprep.

The allocation of resources according to the values of the Allocation and Retention Priority IE included in the QoS Flow Level QoS Parameters IE for each QoS flow shall follow the principles specified for the PDU Session Resource Setup procedure in TS 38.413 [5].

The S-NG-RAN node shall choose the ciphering algorithm based on the information in the UE Security Capabilities IE and locally configured priority list of AS encryption algorithms and apply the key indicated in the S-NG-RAN node Security Key IE as specified in TS 33.501 [28].

If the TSC Traffic Characteristics IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

If the Additional QoS Flow Information IE is included for a QoS flow in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [5].

For each GBR QoS flow, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE, the S-NG-RAN node shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure specified in TS 38.413 [5].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and the Common Network Instance IE is not present, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

For each GBR QoS flow, if the Offered GBR QoS Flow Information IE is included in the QoS Flows To Be Setup List IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node may request the M-NG-RAN node to configure the DRB to which that QoS flow is mapped with MCG resources.

For each PDU session, if the Non-GBR Resources Offered IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE and set to "true", the S-NG-RAN node may request the M-NG-RAN node to configure DRBs to which non-GBR QoS flows of the PDU session are mapped with MCG resources.

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE contained in the PDU Session Resources To Be Added List IE, the S-NG-RAN node shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [7].

Redundant transmission:

-	For each PDU session, if the Redundant UL NG-U UP TNL Information at UPF IE is included in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall, if supported, use it as the uplink termination point for the user plane data for this PDU session for the redundant transmission and it shall include the Redundant DL NG-U UP TNL Information at NG-RAN IE in the PDU Session Resource Setup Response Info – SN terminated IE as described in TS 23.501 [9].

-	For each PDU session, if the Redundant Common Network Instance IE is included in the PDU Session Resource Setup Info – SN terminated IE the S-NG-RAN node shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [7].

-	For each PDU session for which the Redundant QoS Flow Indicator IE is include in QoS Flows To Be Setup List IE contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store and use it as specified in TS 23.501 [7].

-	For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource Setup Info - SN terminated IE in the S-NODE ADDITION REQUEST message, the S-NODE-RAN node shall, if supported, store the received information in the UE context and setup the redundant user plane resources for the concerned PDU session, as specified in TS 23.501 [7].

-	For each PDU session resource successfully setup for which the Redundant PDU Session Information IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, include the Used RSN Information IE in the PDU Session Resource Setup Response Info – SN terminated IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the S-NODE ADDITION REQUEST message contains the Selected PLMN IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the Expected UE Behaviour IE, the S-NG-RAN node shall, if supported, store this information and may use it to optimize resource allocation.

If the S-NODE ADDITION REQUEST message contains the Mobility Restriction List IE, the S-NG-RAN node, if supported, shall store this information and use it to select an appropriate SCG.

If the S-NODE ADDITION REQUEST message contains the Index to RAT/Frequency Selection Priority IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NG-RAN node is a gNB and the S-NODE ADDITION REQUEST message contains the PCell ID IE, the S-NG-RAN node shall search for the target NR cell among the NR neighbour cells of the PCell indicated, as specified in the TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node PDU Session Aggregate Maximum Bit Rate IE, the S-NG-RAN node may use it for RRM purposes.

If the S-NODE ADDITION REQUEST message contains the MR-DC Resource Coordination Information IE, the S-NG-RAN node should forward it to lower layers and it may use it for the purpose of resource coordination with the M-NG-RAN node. The S-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The S-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the S-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the S-NG-RAN node and the M-NG-RAN node.

If the S-NODE ADDITION REQUEST message contains the NE-DC TDM Pattern IE, the S-NG-RAN node should forward it to lower layers and use it for the purpose of single uplink transmission. The S-NG-RAN node shall consider the value of the received NE-DC TDM Pattern IE valid until reception of a new update of the IE for the same UE.

If the S-NODE ADDITION REQUEST message contains the QoS Flow Mapping Indication IE, the S-NG-RAN node may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

For each bearer for which allocation of the PDCP entity is requested at the S-NG-RAN node:

-	the M-NG-RAN node may propose to apply forwarding of downlink data by including the DL Forwarding IE within PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. For each bearer that it has decided to admit, the S-NG-RAN node may include the DL Forwarding GTP Tunnel Endpoint IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding of downlink data for this bearer.

-	the S-NG-RAN node may include for each bearer in the PDU Session Resource Setup Response Info – SN terminated IE the UL Forwarding GTP Tunnel Endpoint IE to indicates it request data forwarding of uplink packets to be performed for that bearer.

-	the M-NG-RAN node shall include RLC Mode IE for each bearer offloaded from M-NG-RAN node to S-NG-RAN node in the DRBs to QoS Flow Mapping List IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDTION REQUEST message, and the RLC Mode IE indicates the mode that the M-NG-RAN used for the DRB when it was hosted at the M-NG-RAN node.

For each bearer for which the PDCP entity is at the M-NG-RAN node:

-	the M-NG-RAN node shall include the RLC mode IE for each bearer in the DRBs To Be Setup List IE within the PDU Session Resource Setup Info – MN terminated IE of the S-NODE ADDTION REQUEST message to indicate the RLC mode has been configured at the M-NG-RAN node, so that the S-NG-RAN node shall configure the same RLC mode for this MN terminated split bearer.

The M-NG-RAN node may also propose to apply forwarding of UL data when offloading QoS flows for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message. The S-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Setup Response Info – SN terminated IE of the S-NODE ADDITION REQUEST ACKNOWLEDGE message to indicate that it accepts the proposed forwarding.

If the Masked IMEISV IE is contained in the S-NODE ADDITION REQUEST message the S-NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the UE Radio Capability ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

The S-NG-RAN node shall report to the M-NG-RAN node, in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the result for all the requested PDU session resources in the following way:

-	A list of PDU session resources which are successfully established shall be included in the PDU Session Resources Admitted To Be Added List IE.

-	A list of PDU session resources which failed to be established shall be included in the PDU Session Resources Not Admitted List IE.

Upon reception of the S-NODE ADDITION REQUEST ACKNOWLEDGE message the M-NG-RAN node shall stop the timer TXnDCprep.

If the S-NODE ADDITION REQUEST ACKNOWLEDGE message contains the MR-DC Resource Coordination Information IE, the M-NG-RAN node may use it for the purpose of resource coordination with the S-NG-RAN node. The M-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The M-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the M-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the M-NG-RAN node and the S-NG-RAN node.

The S-NG-RAN node may include for each bearer in the DRBs To Be Setup List IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message the PDCP SN Length IE to indicate the PDCP SN length for that DRB.

If the S-NG-RAN node UE XnAP ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, store this information and use it as defined in TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the PDCP SN Length IE, the S-NG-RAN node shall, if supported, store this information and use it for lower layer configuration of the concerned MN terminated bearer.

If the S-NODE ADDITION REQUEST message contains the SN Addition Trigger Indication IE, the S-NG-RAN node shall include the RRC config indication IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message to inform the M-NG-RAN node if the S-NG-RAN node applied full or delta configuration, as specified in TS 37.340 [8].

If the S-NODE ADDITION REQUEST message contains the S-NG-RAN node Maximum Integrity Protected Data Rate Uplink IE or the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE, the S-NG-RAN node shall use the received information when enforcing the maximum integrity protected data rate for the UE.

If the Security Indication IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message, the behaviour of the S-NG-RAN node shall be the same as specified for the same IE in the PDU Session Resources To Be Setup List IE in the Handover Preparation procedure, for the concerned PDU session, and the S-NG-RAN node shall include the Security Result IE in the PDU Session Resource Setup Response Info – SN terminated IE.

If the Security Result IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message, the S-NG-RAN node may take the information into account when deciding whether to perform user plane integrity protection or ciphering for the DRBs that it establishes for the concerned PDU session, except if the Split Session Indicator IE is included in the PDU Session Resource Setup Info – SN terminated IE and set to "split", in which case it shall perform user plane integrity protection or ciphering according to the information in the Security Result IE. If the S-NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required" as specified in TS 33.501 [28]. If either the S-NG-RAN node or the M-NG-RAN node is an ng-eNB, the S-NG-RAN node shall behave according to clause 6.10.4 of TS 33.501 [28] for PDU sessions for which the Integrity Protection Indication IE is set to "preferred".

The S-NG-RAN node may include the Location Information at S-NODE IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message, if respective information is available at the S-NG-RAN node.

If the Location Information at S-NODE Reporting IE set to "pscell" is included in the S-NODE ADDITION REQUEST, the S-NG-RAN node shall, start providing information about the current location of the UE. If the Location Information at S-NODE IE is included in the S-NODE ADDITION REQUEST ACKNOWLEDGE, the M-NG-RAN node shall store the included information so that it may be transferred towards the AMF.

If the Default DRB Allowed IE is included in the PDU Session Resource Setup Info – SN terminated IE of the S-NODE ADDITION REQUEST message and set to "true", the S-NG-RAN node may configure the default DRB for the PDU session.

If the S-NODE ADDITION REQUEST ACKNOWLEDGE message includes the DRB IDs taken into use IE, the M-NG-RAN node, if applicable, shall act as specified in TS 37.340 [8].

If Trace Activation IE has previously been received for this UE, it shall be included in the S-NODE ADDITION REQUEST message. If the Trace Activation IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [23].

If the Requested Fast MCG recovery via SRB3 IE set to "true" is included in the S-NODE ADDITION REQUEST message and the S-NG-RAN node decides to configure fast MCG link recovery via SRB3 as specified in TS 37.340 [8], the S-NG-RAN shall, if supported, include the Available fast MCG recovery via SRB3 IE set to "true" in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE for a QoS flow contained in the DRBs To Be Setup List IE of the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall, if supported, use it for RAN part delay reporting.

For each QoS flow which has been successfully established in the S-NG-RAN node, if the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, perform delay measurement and QoS monitoring as specified in TS 23.501 [7]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the PDU Session Resource Setup Info – SN terminated IE, the S-NG-RAN node shall store this information, and, if supported, use it for RAN part delay reporting. In case such a QoS flow is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it to configure lower layers for the purpose of delay measurement and QoS monitoring. If the QoS Monitoring Reporting Frequency IE is included in the DRBs To Be Setup List IE of the PDU Session Resource Setup Response Info – SN terminated IE, the M-NG-RAN node shall, if supported, use it for RAN part delay reporting.

If the Conditional PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall consider that the request concerns CPAC, as described in TS 37.340 [8]. Accordingly, the S-NG-RAN node shall include the Conditional PSCell Addition Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

*** Skip unchanged ***

- 8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

- 8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

- 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE CHANGE REQUIRED message may contain

-	the S-NG-RAN node to S-NG-RAN node Container IE.

If the M-NG-RAN node is able to perform the change requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE CHANGE CONFIRM message to the S-NG-RAN node. For DRBs configured with the PDCP entity in the S-NG-RAN node, the M-NG-RAN node may include data forwarding related information in the Data Forwarding Info from target NG-RAN node IE.

If the S-NODE CHANGE CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8].

The S-NG-RAN node may start data forwarding and stop providing user data to the UE and shall stop the timer TXnDCoverall upon reception of the S-NODE CHANGE CONFIRM message.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall consider that the requirement concerns CPAC, as described in TS 37.340 [8]. Accordingly, the M-NG-RAN node shall include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message.

*** Skip unchanged ***

- 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >CPAC Indicator | M |  | ENUMERATED (CPAC-initiation, ...) |  |  |  |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..FFS, …) | Indicates the maximum number of PSCells that the target SN may prepare. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

- 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE is present in a PDU Session Resources Admitted to be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Setup Response Info – SN terminated | O |  | 9.2.1.6 |  | – |  |
| >>PDU Session Resource Setup Response Info – MN terminated | O |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Available fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates the fast MCG recovery via SRB3 isenabled. | YES | ignore |
| Conditional PSCell Addition Information Acknowledge | O |  |  |  |  |  |
| >Candidate PSCell ID List |  | 1 |  |  |  |  |
| >>Candidate PSCell ID Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  |  |
| >>>PSCell ID | M |  | Target Cell Global ID 9.2.3.25 |  |  |  |
| >>>RRC Container | M |  | OCTET STRING | Includes RRCReconfiguration message as defined in subclause 6.2.2 of TS 38.331[10]. FFS whether single or multiple RRC containers. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is FFS |

Skip unchanged

- 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | reject |
| Conditional PSCell Change Information Required | O |  |  |  | YES | reject |
| >CPAC Indicator | M |  | ENUMERATED (CPAC-initiation, ...) |  |  |  |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..FFS, …) | Indicates the maximum number of PSCells that the target SN may prepare. |  |  |
| >RRC Container | M |  | OCTET STRING | Includes CPCExecutionCondition as defined in subclause 6.2.x of TS 38.331 [10]. FFS |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

- 9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node change was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session SN Change Confirm List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Confirm Item |  | 1 .. <maxnoof PDUsessions> |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Confirm Info – SN terminated | O |  | 9.2.1.19 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Conditional PSCell Change Information Confirm | O |  |  | FFS if the SN that initiated CPC is informed of the candidate PSCell IDs | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

End of the change