3GPP TSG-RAN WG3 Meeting #113-e 	R3-213623

# E-meeting, -, 16. - 27.08.2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0646 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG Event Indication over Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 5.08.2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Dual Connectivity scenarios with disaggregated architecture at the Master Node, the gNB-DU considers the set of capabilities at the UE, MN and SN for band combination selection. For this purpose, it can be beneficial for the gNB-DU at the Master Node to have an explicit indication when an SCG has been added at the Secondary Node to allow the gNB-DU to optimize its own band selection. For that purpose, the information about SCG addition needs to be transferred over the Xn interface from the SN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new codepoint to SCG Inidication IE to signal that an SCG has been added. Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol and functional point of view. The impact can be considered isolated because the change is limited to UE Context Modification procedure. This CR is ASN.1 backwards compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The gNB-DU at the Master Node may not be able to optimize band selection to for SN capabilities after an SCG has been added. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.4.2, 9.1.2.8, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 0783 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: S-NG-RAN node initiated S-NG-RAN node Modification, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUIRED message to the M-NG-RAN node.

When the S-NG-RAN node sends the S-NODE MODIFICATION REQUIRED message, it shall start the timer TXnDCoverall.

The S-NODE MODIFICATION REQUIRED message may contain

-	the S-NG-RAN node to M-NG-RAN node Container IE.

-	PDU session resources to be modified within the PDU Session Resources To Be Modified Item IE;

-	PDU session resources to be released within the PDU Session Resources To Be Released Item IE;

-	the PDCP Change Indication IE;

-	the Spare DRB IDs IE;

-	the Required Number of DRB IDs IE;

-	the QoS Flow Mapping Indication IE;

-	the MR-DC Resource Coordination Information IE.

If the M-NG-RAN node receives a S-NODE MODIFICATION REQUIRED message containing the PDCP Change Indication IE, the M-NG-RAN node shall act as specified in TS 37.340 [8].

If the S-NODE MODIFICATION REQUIRED message contains the MR-DC Resource Coordination Information IE, the M-NG-RAN node may use it for the purpose of resource coordination with the S-NG-RAN node. The M-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE.  The M-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the M-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the M-NG-RAN node and the S-NG-RAN node.

If the M-NG-RAN node receives an S-NODE MODIFICATION REQUIRED message containing the Spare DRB IDs IE, the M-NG-RAN node may take those into consideration to be used for MN-terminated bearers.

If the M-NG-RAN node receives an S-NODE MODIFICATION REQUIRED message containing the Required Number of DRB IDs IE, the M-NG-RAN node shall provide new DRB IDs to be used by the S-NG-RAN node for SN-terminated bearers , if such DRB IDs are available, in the Additional DRB IDs IE included in the S-NODE MODIFICATION CONFIRM message.

If the M-NG-RAN node is able to perform the modifications requested by the S-NG-RAN node, the M-NG-RAN node shall send the S-NODE MODIFICATION CONFIRM message to the S-NG-RAN node. The S-NODE MODIFICATION CONFIRM message may contain the M-NG-RAN node to S-NG-RAN node Container IE.

If the PDCP Duplication Configuration IE in the PDU Session Resource Modification Required Info – SN terminated IE is contained in the S-NODE MODIFICATION REQUIRED message and set to "configured", the M-NG-RAN node shall, if supported, add the RLC entity of secondary path for the indicated DRB. And if the S-NODE MODIFICATION REQUIRED message contains the Duplication Activation IE, the M-NG-RAN node shall, if supported, store this information and use it for the purpose of PDCP duplication.

If the PDCP Duplication Configuration IE in the PDU Session Resource Modification Required Info – SN terminated IE is contained in the S-NODE MODIFICATION REQUIRED message and set to "de-configured", the M-NG-RAN node shall, if supported, delete the RLC entity of secondary path for the indicated DRB.

The S-NG-RAN node may include for each DRB in the DRBs To Be Modified List IE in the S-NODE MODIFICATION REQUIRED message the RLC Status IE to indicate that RLC has been reestablished at the S-NG-RAN node and the M-NG-RAN node may trigger PDCP data recovery.

If the S-NODE MODIFICATION REQUIRED message contains the QoS flows To Be Released List within the PDU Session Resource Modification Info – SN terminated IE, the S-NG-RAN node may also propose to apply forwarding of UL data for which in-order delivery is requested by including the UL Forwarding Proposal IE in the Data Forwarding and Offloading Info from source NG-RAN node IE within the PDU Session Resource Modification Required Info – SN terminated IE of the S-NODE MODIFICATION REQUIRED message. The M-NG-RAN node may include the PDU Session Level UL Data Forwarding UP TNL Information IE in the Data Forwarding Info from target NG-RAN node IE within the PDU Session Resource Modification Confirm Info – SN terminated IE of the S-NODE MODIFICATION CONFIRM message to indicate that it accepts the proposed forwarding.

Upon reception of the S-NODE MODIFICATION CONFIRM message the S-NG-RAN node shall stop the timer TXnDCoverall.

If the S-NODE MODIFICATION CONFIRM message contains the MR-DC Resource Coordination Information IE, the S-NG-RAN node should forward it to lower layers and it may use it for the purpose of resource coordination with the M-NG-RAN node. The S-NG-RAN node shall consider the value of the received UL Coordination Information IE valid until reception of a new update of the IE for the same UE. The S-NG-RAN node shall consider the value of the received DL Coordination Information IE valid until reception of a new update of the IE for the same UE. If the E-UTRA Coordination Assistance Information IE or the NR Coordination Assistance Information IE is contained in the MR-DC Resource Coordination Information IE, the S-NG-RAN node shall, if supported, use the information to determine further coordination of resource utilisation between the S-NG-RAN node and the M-NG-RAN node.

If the S-NODE MODIFICATION REQUIRED message contains a PDU session resource to be released which is configured with the SCG bearer option within the PDU sessions to be released List – SN terminated IE, the S-NG-RAN node shall include the RLC Mode IE within the DRBs To Be Released List IE in the PDU Session to be released List – SN terminated IE in the S-NODE MODIFICATION REQUIRED message. The RLC Mode IE indicates the RLC mode used in the S-NG-RAN node for the DRB.

If the Location Information at S-NODE IE is included in the S-NODE MODIFICATION REQUIRED, the M-NG-RAN node shall store the included information so that it may be transferred towards the AMF.

If the QoS Flows Mapped To DRB List IE is included in the S-NODE MODIFICATION REQUIRED message for a DRB to be modified, the M-NG-RAN node shall replace any existing QoS flow mapping for that DRB with the one received.

If the S-NG-RAN node applied a full configuration or delta configuration, e.g., as part of mobility procedure involving a change of DU, the S-NG-RAN node shall inform the M-NG-RAN node by including the RRC config indication IE in the S-NODE MODIFICATION REQUIRED message.

If the S-NODE MODIFICATION CONFIRM message includes the DRB IDs taken into use IE, the S-NG-RAN node shall, if applicable, act as specified in TS 37.340 [8]

If the SCG Indicator IE is contained in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, take the information into account for L1/L2 configuration.

##### Interaction with the M-NG-RAN node initiated S-NG-RAN node Modification Preparation procedure:

If applicable, as specified in TS 37.340 [8], the S-NG-RAN node may receive, after having initiated the S-NG-RAN node initiated S-NG-RAN node Modification procedure, the S-NODE MODIFICATION REQUEST message including the measGapConfig IE as defined in TS 38.331 [10] within the M-NG-RAN node to S-NG-RAN node Container IE.

Next change, ommited text not changed

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.3.74 |  | YES | ignore |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Modification Required Info – SN terminated IE nor the PDU Session Resource Modification Required Info – MN terminated IE is present in a PDU Session Resources To Be Modified Item IE, abnormal conditions as specified in clause 8.3.4.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Modification Required Info – SN terminated | O |  | 9.2.1.20 |  | – |  |
| >>PDU Session Resource Modification Required Info – MN terminated | O |  | 9.2.1.22 |  | – |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | – |  |
| >PDU sessions to be released List – SN terminated | O |  | PDU session List with data forwarding request info 9.2.1.24 |  | – |  |
| >PDU sessions to be released List – MN terminated | O |  | PDU session List with Cause 9.2.1.26 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |
| Spare DRB IDs | O |  | DRB List 9.2.1.29 | Indicates the list of unnecessary DRB IDs that had been used by the S-NG-RAN node. | YES | ignore |
| Required Number of DRB IDs | O |  | Number of DRBs 9.2.3.78 | Indicates the number of DRB IDs that the S-NG-RAN node requests more. | YES | ignore |
| Location Information at S-NODE | O |  | Target Cell Global ID 9.2.3.25 | Contains information to support localisation of the UE | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | Ignore |
| RRC Config Indication | O |  | 9.2.3.72 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED (released, ..., added) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

Next change, ommited text not changed

SCGIndicator	::=	ENUMERATED{released, ..., added}

Remaining text not changed