3GPP TSG-RAN WG3 Meeting #113-e	R3-213935

E-meeting, 16-26 Aug 2021

Title: 	Discussion on IAB congestion mitigation

Source: 	Huawei

Agenda item:	13.3.1

Document Type:	Discussion

1. Introduction

In the last RAN3 meeting, issues related to congestion migration were discussed, and the following agreements were achieved [1]:

- The following two types of congestion indication are supported in CP-based congestion mitigation: 1) per child link; 2) per BH RLC CH ID. Which type of congestion indication to be reported could be up to implementation. FFS on per BAP routing ID.

- The trigger for sending the CP-based congestion indication is up to implementation.

- The congestion level is not introduced for CP-based congestion indication report.

- The handling with respect to simultaneous presence of IAB Congestion Indication IE and the gNB-DU Overload Information IE is up to implementation.

- The “do nothing” option, i.e. use current DDDS as it is, is selected for IAB DL end-to-end UP-based flow control.

In this contribution, the discussion is mainly about the remaining issue of the CP-based congestion mitigation.

2. Discussion

2.1 CP-based congestion migration

According to the agreements from the last RAN3 meeting, per child link and per BH RLC CH ID granularity of congestion indication are supported by the Rel-17 IAB. Whether the per BAP routing ID level congestion indication is supported is FFS.

For the BAP routing ID level indication, the analysis is as follows,

Figure 1. Example of per BAP routing ID feedback

In some cases, the feedback overhead indicated by per BAP routing ID is significant. As shown in Figure 1, if the link between IAB-node 2 and IAB-node 1 is congested, all paths through IAB-node 2 are unavailable. Therefore, the congestion indication includes the BAP routing ID 1 and the BAP routing ID 2. The more downstream nodes of IAB-node 2, the higher the feedback overhead. On the other hand, since the per link level congestion indication has been agreed to introduced, and the CU-CP is aware of that the routing path indicated by a BAP routing ID consists of which links, so it can deduce the congestion status of each routing path based on the per child link congestion feedback.

Therefore, it is proposed,

Proposal 1: No need to support per BAP routing ID congestion indication.

3. Conclusion

This paper is mainly about the remaining issues of CP-based congestion migration, and we get the following proposal:

Proposal 1: No need to support per BAP routing ID congestion indication.

4. Reference

- Chairman notes of 3GPP TSG-RAN WG3 meeting #112-e.