3GPP TSG-RAN WG3 Meeting #113-e	R3-214038

- 16-26 Aug 2021

- Online

Title: 	(TP for SON BL CR 38.423) Coverage and Capacity Optimization

Source: 	ZTE, China Telecom, China Unicom

Agenda item:	10.2.2

Document for:	Agreement

1. Introduction

This contribution is a TP for the CCO based on the agreed SON BL CR 38.423 [1].

Text Proposal for 38.423

<<<<<<<<<<<<<<<<<<<< Start of the First Change >>>>>>>>>>>>>>>>>>>>

8.4.2	NG-RAN node Configuration Update

8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

If the TAI Support List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving node shall replace the previously provided TAI Support List IE by the received TAI Support List IE.

If the Cell Assistance Information NR IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served NR Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Cell Assistance Information LTE IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served LTE Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Partial List Indicator NR IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served NR Cells IE  in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Partial List Indicator E-UTRA IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served E-UTRA Cells IE  in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Cell and Capacity Assistance Information NR IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

If the Cell and Capacity Assistance Information E-UTRA IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

Upon reception of the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall update the information for NG-RAN node1 as follows:

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information NR IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE is contained in the Served Cells NR To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

-	If the Intended TDD DL-UL Configuration NR IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 should take this information into account for cross-link interference management and/or NR-DC power coordination with the NG-RAN node1. The NG-RAN node2 shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of a new update of the IE for the same NG-RAN node2.

-	If the NR Cell PRACH Configuration IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

Update of Served Cell Information E-UTRA:

-	If Served Cells E-UTRA To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information E-UTRA IE.

-	If Served Cells E-UTRA To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information E-UTRA IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information E-UTRA IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE is contained in the Served Cells E-UTRA To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If the Served Cells E-UTRA To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old ECGI IE.

-	If the Protected E-UTRA Resource Indication IE is included into the NG-RAN NODE CONFIGURATION UPDATE (inside the Served Cell Information E-UTRA IE), the receiving gNB should take this into account for cell-level resource coordination with the ng-eNB. The gNB shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same ng-eNB. The protected resource pattern indicated in the Protected E-UTRA Resource Indication IE is not valid in subframes indicated by the Reserved Subframes IE (contained in E-UTRA - NR CELL RESOURCE COORDINATION REQUEST messages), as well as in the non-control region of the MBSFN subframes i.e. it is valid only in the control region therein. The size of the control region of MBSFN subframes is indicated in the Protected E-UTRA Resource Indication IE.

-	If the PRACH Configuration IE is contained in the Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

Update of TNL addresses for SCTP associations:

If the TNL Association to Add List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to establish the TNL association(s) with the NG-RAN node1. The NG-RAN node2 shall report to the NG-RAN node1, in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the NG-RAN node1 as follows:

-	A list of successfully established TNL associations shall be included in the TNL Association Setup List IE;

-	A list of TNL associations that failed to be established shall be included in the TNL Association Failed to Setup List IE.

If the TNL Association to Remove List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message the NG-RAN node2 shall, if supported, initiate removal of the TNL association(s) indicated by the received Transport Layer information towards the NG-RAN node1.

If the TNL Association to Update List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message the NG-RAN node2 shall, if supported, update the TNL association(s) indicated by the received Transport Layer information towards the NG-RAN node1.

Update of AMF Region Information:

-	If AMF Region Information To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall add the AMF Regions to its AMF Region List.

-	If AMF Region Information To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall remove the AMF Regions from its AMF Region List.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for IPSec establishment.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall take this IE into account for IPSec establishment.

If the CSI-RS Transmission Indication IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for neighbour cell’s CSI-RS measurement.

The NG-RAN NODE CONFIGURATION UPDATE message may contain for each cell served by NG-RAN node1 NPN related broadcast information. The NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message may contain for each cell served by NG-RAN node2 NPN related broadcast information.

Editor's Note: the details for the procedure text below is FFS

Update of Cell Coverage:

If the Coverage Modification List IE is present, the NG-RAN node2 may use the information in the Cell Coverage State IE to identify the cell deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the cell(s) indicated by the Global NG-RAN Cell Identity IE, as described in TS 38.300 [9]. If the Cell Deployment Status Indicator IE is present in the Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the cell to be modified as the next planned configuration and shall remove any planned configuration stored for this cell. If the Cell Deployment Status Indicator IE is present and the Cell Replacing Info IE contains non-empty cell list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the cells in the list as possible alternative handover targets. If the Cell Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the cell deployment configuration of cell to be modified as activated and replace any previous configuration for the cells indicated in the Coverage Modification List IE. If the SSB Area Coverage Modification List IE is present in the Coverage Modification List IE, the NG-RAN node2 may use the information in the Beam Coverage State IE or the Beam Coverage Settings IE to identify the beam deployment configuration enabled by the NG-RAN node1 and for configuring the mobility towards the beam(s) indicated by the SSB Index IE, as described in TS 38.300 [9]. If the Beam Deployment Status Indicator IE is present in the SSB Area Coverage Modification List IE, the NG-RAN node2 shall consider the cell deployment configuration of the beam to be modified as the next planned configuration and shall remove any planned configuration stored for this beam. If the Beam Deployment Status Indicator IE is present and the Beam Replacing Info IE contains non-empty beam list, the NG-RAN node2 may use this list to avoid connection or re-establishment failures during the reconfiguration, e.g. consider the beams in the list as possible alternative handover targets. If the Beam Deployment Status Indicator IE is not present, the NG-RAN node2 shall consider the beam deployment configuration of beam to be modified as activated and replace any previous configuration for the beams indicated in the SSB Area Coverage Modification List IE.

8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: NG-RAN node Configuration Update, unsuccessful operation

If the NG-RAN node2 cannot accept the update it shall respond with the NG-RAN NODE CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the NG-RAN NODE CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the NG-RAN node1 shall wait at least for the indicated time before reinitiating the NG-RAN Node Configuration Update procedure towards the same NG-RAN node2. Both nodes shall continue to operate the Xn with their existing configuration data.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

8.4.2.4	Abnormal Conditions

If the NG-RAN node1 after initiating NG-RAN node Configuration Update procedure receives neither NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message nor NG-RAN NODE CONFIGURATION UPDATE FAILURE message, the NG-RAN node1 may reinitiate the NG-RAN node Configuration Update procedure towards the same NG-RAN node2, provided that the content of the new NG-RAN NODE CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged NG-RAN NODE CONFIGURATION UPDATE message.

<<<<<<<<<<<<<<<<<<<< End of the First Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Start of the Second Change >>>>>>>>>>>>>>>>>>>>

9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name |  | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List |  | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType |  | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |  |
| >>Served Cells To Update NR |  | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA |  | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR |  | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA |  | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | M |  |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | O |  |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  |  | 0 .. <maxnoofCellsinNG-RAN node> |  | List of cells with modified coverage. | GLOBAL | reject |
| >Global NG-RAN Cell Identity | M |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be modified. | - |  |
| >Cell Coverage State | M |  |  | INTEGER (0..FFS, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >Cell Deployment Status Indicator | O |  |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. |  |  |
| >Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  |  |  |  |
| >>Replacing Cells |  |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  |  |  |
| >>>Global NG-RAN Cell Identity |  |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. |  |  |
| >SSB Area Coverage Modification List |  |  | 0..1 |  | List of beams with modified coverage. |  |  |
| >>SSB Area Coverage Modification Item |  |  | 1..<maxnoofSSBAreas> |  |  |  |  |
| >>>SSB Index | O |  |  | INTEGER (0..63) | Identifier of a single beam or a set of beams to be modified. |  |  |
| >>>CHOICE Beam Coverage Configuration Type | M |  |  |  |  |  |  |
| >>>> State of Beam Coverage Configuration |  |  |  |  | Indicates the status of beam coverage configuration. |  |  |
| >>>>>Beam Coverage State | O |  |  | NTEGER (0..FFS, …) | Value '0' indicates that the beam is inactive. Other values Indicates that the beam is active and also indicates the coverage configuration of the concerned beam. |  |  |
| >>>> Details of Beam Coverage Configuration |  |  |  |  |  |  |  |
| >>>>>Beam Coverage Settings | O |  |  |  | Indicates the detailed expression of beam coverage configuration. |  |  |
| >>>>>>Azimuth Angle | O |  |  | FFS | Indicates the azimuth angle of the beam to be modified. |  |  |
| >>>>>>Tilt Angle | O |  |  | FFS | Indicates the tilt angle of the beam to be modified. |  |  |
| >>>>>>Horizontal Beam Width | O |  |  | FFS | Indicates the horizontal width of the beam to be modified. |  |  |
| >>>>>>Vertical Beam Width | O |  |  | FFS | Indicates the vertical width of the beam to be modified. |  |  |
| >>>Beam Deployment Status Indicator | O |  |  | ENUMERATED(pre-change-notification, ...) | Indicates the Beam Coverage State or the Beam Coverage Setting is planned to be used at the next reconfiguration. |  |  |
| >>>Beam Replacing Info | C-ifBeamDeploymentStatusIndicatorPresent |  |  |  |  |  |  |
| >>>>Replacing Beams |  |  | 0 .. <maxnoofSSBAreas> |  |  |  |  |
| >>>>>SSB Index |  |  |  | INTEGER (0..63) | Identifier of a beam or a set of beams that may replace all or part of the coverage of the beam to be modified. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the NG RAN nodes. Value is 32. |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |
| ifBeamDeploymentStatusIndicatorPresent | This IE shall be present if the Beam Deployment Status Indicator IE is present. |

<<<<<<<<<<<<<<<<<<<< End of the Second Change >>>>>>>>>>>>>>>>>>>>

References

- R3-213181 BLCR to 38.423_Addition of SON features enhancement  Samsung