3GPP TSG-RAN WG3 Meeting #113-e	R3-214474

# Online, August 16th – 26th 2021

Title:	LS on BAP- and RRC-related agreements from RAN3#113-e

Response to:	-

Release:	Rel-17

Study Item:	NR_IAB_enh-Core

Source:	RAN3

To:	RAN2

Cc:	-

Contact Person:

Name: 		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description:

At the RAN3#113-e meeting, RAN3 reached the following agreements related to BAP and RRC layers:

Regarding the processing at the boundary node:

- RAN3 prefers that the boundary node processes access traffic in the same manner as the non-boundary access IAB-node.

- RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic.

- FFS: In addition to BAP header rewriting, performs routing and bearer mapping in the same manner as the non-boundary intermediate IAB-node.

RAN3 assumes that the boundary node has only one BAP address in each topology.

RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers.

For partial inter-donor migration, the IP addresses, BAP address, BH RLC CHs and default mapping used by the boundary node for traffic in a particular topology are assigned by the CU of that topology, and they are configured via RRC.

2. Actions:

RAN2 is respectfully asked to take the above agreements into account in their work.

3. Date of next TSG RAN WG3 meetings:

RAN3#114-e                         1st November - 11th November 2021	Online