3GPP TSG-RAN WG3 #113-e	R3-214169

Online, 16-26 Aug, 2021

Agenda Item:	10.2.1.6

Source:	Samsung (moderator)

Title:	Summary of Offline Discussion on MRO for SN Change Failure

Document for:	Approval

# Introduction

CB: # SONMDT5_MROSNchange

- Which IEs should be included in the new XnAP message for carrying SCGfailureinformation?

- Whether to include the Mobility Information in the S-NODE ADDITION REQUEST ACKNOWLEDGE message?

- How to support intra-SN PSCell change failures without MN involvement?

- SN change failure for pre-Rel-17 UEs?

- Other scenarios to be considered? E.g. EN-DC and NR-DC? LS to RAN2?

- Whether the source SN can keep the UE context till the timer to detect SN change failure expires?

- Any other topic based on contributions submitted

- Start with summary of offline, proceed to TPs if there are agreements

(Samsung - moderator)

Summary of offline disc in R3-214169

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: xxx

# Discussion

## Intra-SN PSCell change after MN/SN initiated SN change for pre-R17 UEs

Three options were proposed in the contributions:

Option 1: MN always forward SCG failure report to last serving SN. If no intra-SN PSCell change, last serving SN indicates it to MN, so it’s SN change failure. A class-1 procedure should be defined. [1][2][7]

Option 2: MN asks first the last serving SN whether intra-SN PSCell change occurs. If yes, MN forward SCG failure report to last serving SN; Otherwise it’s SN change failure. A class-1 procedure should be defined. [9]

Option 3: No enhancement is needed for this specific case. The MN can depend on the measurement results and other information to decide the node that caused the SCG failure. [5][11]

Q1: Which option do you support?

| Company | Comment |
| --- | --- |
| Samsung | Option 3 MN has overall information, therefore can make a right decision on the node that caused the SCG failure. E.g. SCG failure just after successful SN1->SN2 change. According to UE measurement report, if the PScell in SN1 is a good cell while no good cell in SN2, it could be too early PScell change no matter there is intra-PScell change in SN2 or not. It doesn’t mean MRO issue can be excluded if there is intra-PScell change. If the MN simply sends SCGFailureInformation to the SN2, SN2 reply that there is an intra-SN PScell change, the MN may exclude MRO for SN change, which is not right. |

## Information other than SCGfailureinformation in new XnAP message

In the last meeting, it’s still FFS whether include the following IEs in the new XnAP message for carrying SCGfailureinformation:

a) PSCell failure type

b) Source PSCell CGI

c) Failed PSCell CGI

d) Suitable PSCell CGI

e) Mobility Information

f) PSCell selection assistant information, e.g. UE history information

g) Initiating node type i.e. MN or SN

h) S-NG-RAN node UE X2AP ID

i) M-NG-RAN node UE XnAP ID

Q2: which information should be included in the new XnAP message other than SCGfailureinformation?

| Company | Comment |
| --- | --- |
| Samsung | - b) c) d) e) f) can be considered later For a), in order to identify the node which bring the problem, the MN already makes the initial analysis on the failure type. Similar like HO case, the last serving node make initial analysis and sends the failure type to the node which bring problem, the MN can sends the failure type to the source SN for information. It’s the source SN decision how to use it. There is a comment that the MN cannot differentiate the too early and wrong PScell change. Based on the definition agreed for TS37.340, the MN can differentiate this based on the received information from the UE and the UE context. - and c) are for pre-Rel-17 UEs. For d), in order to identify the node which bring the problem, the MN should know which is a suitable cell based on the UE measurement. Similar reason as a), d) is beneficial. For e), in case of SCG failure just after successful SN1->SN2 change, it’s possible that SN has released the UE context. Similar as HO case, mobility information is useful. |

## Other proposals of MRO issues for SN change failure

Scenarios to be supported

In [9], it’s proposed to support EN-DC and NR-DC scenarios for SN change failure scheme but NG-EN-DC and NE-DC scenarios are not to be considered.

Q3: Are you ok with this proposal?

| Company | Comment |
| --- | --- |
| Samsung | Prefer to focus on NR case firstly. |

Ambiguity in SCG failure cases

In [10], a scenario is introduced: after RRC Reconfiguration message for SN change initiated by SN is received by a UE, a random access procedure occurs at the serving PSCell due to other reason, e.g. for timing synchronization, then it fails. So an SCG failure report is produced. It will make source SN confused whether it’s SN change failure or not. So [10] proposes to add a new cause value “random access problem” in F1 UE context release message to avoid the confusion.

Q4: Are you ok with this proposal?

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-213308, Discussion on MRO for SN Change Failure (China Telecommunication)

- R3-213392, MRO for PSCell change failure (Nokia, Nokia Shanghai Bell)

- R3-213393, (TP to SON BL CR to 38.423, NR_ENDC_SON_MDT_enh-Core) Enabling SCG MRO (Nokia, Nokia Shanghai Bell)

- R3-213394, (TP to SON BL CR to 36.423, NR_ENDC_SON_MDT_enh-Core) Enabling SCG MRO (Nokia, Nokia Shanghai Bell)

- R3-213408, (TP for SON BLCR for 38.423) MRO for SN Change Failure (Huawei)

- R3-213409, (TP for SON BLCR for 38.300) MRO for SN Change Failure (Huawei)

- R3-213500, Consideration on support of SN change failure in case of MR-DC (CATT)

- R3-213501, (TP on SN change failure for 38.423) Addition of SCG Failure procedure for SN change failure (CATT)

- R3-213659, MRO for SN Change Failure (Qualcomm Incorporated)

- R3-213817, (TP for SON BL CR for TS 38.473) MRO for SN Change Failure (Ericsson)

- R3-214001, Further consideration on MRO SN change failure (ZTE)

- R3-214002, (TP for SON BL CR for TS 38.423) MRO SN change failure (ZTE)

- R3-214008, Support of SN change failure (Samsung)

- R3-214009, TP for SON BLCR for 38.423: Support of SN change failure (Samsung)