3GPP TSG-RAN WG3 #114-e							 R3-215946

1-11 November 2021

Online

Agenda Item:	9.3.6.1

Source:	ZTE (moderator)

Title:	Summary_of_offline_disc_CB # 112_CriticalityDiag

Document for:	Approval

# Introduction

This contribution is to kick off the following discussion.

CB: # 112_CriticallityDiag - Check all the messages to include the Criticality Diagnostics - Update ASN.1 - Make it BC (ZTE - moderator) Summary of offline disc R3-215946

The deadline is Tuesday, November 9th, 12:00 UTC.

# For the Chairman’s Notes

[To be added]

# Discussion

## Correction on the IE Criticality Diagnostics over W1

According to [1], by checking the latest version of TS 37.473, the IE Criticality Diagnostics is only included in the PWS CANCEL RESPONSE message. While, by comparing with the F1AP specification, the IE Criticality Diagnostics over W1 should also be used in many response messages instead of only the PWS CANCEL RESPONSE message.

To solve this issue, there are two options for selection.

Option 1: Add the IE Criticality Diagnostics in other response messages, i.e. RESET ACKNOWLEDGE message, ERROR INDICATION message, W1 SETUP FAILURE message, NG-ENB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, NG-ENB-DU CONFIGURATION UPDATE FAILURE message, NG-ENB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, NG-ENB-CU CONFIGURATION UPDATE FAILURE message, UE CONTEXT SETUP RESPONSE message, UE CONTEXT SETUP FAILURE message, UE CONTEXT RELEASE COMPLETE message, UE CONTEXT MODIFICATION RESPONSE message, UE CONTEXT MODIFICATION FAILURE message, UE CONTEXT MODIFICATION CONFIRM message, UE CONTEXT MODIFICATION REFUSE message and WRITE-REPLACE WARNING RESONSE message.

Option 2: Add a note in the IE Criticality Diagnostics in PWS CANCEL RESPONSE message to indicate that this IE is not used in Release 16.

During the online discussion, companies agreed to apply the Option 1 for this issue, and a corresponding draft CR has been uploaded in the CB folder.

Question: Can you agree this CR? Any comment is welcomed.

| Company | Comment |
| --- | --- |
| ZTE | Yes |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-215737	Discussion on Criticality Diagnostics over W1  ZTE, China Telecom, China Unicom

- R3-215738	Correction on Criticality Diagnostics over W1  ZTE, China Telecom, China Unicom