3GPP TSG-RAN WG3 #114-e	R3-215986

Online, 1st - 11th November 2021	was R3-215888

Agenda Item:	22.2.4

Source:	Ericsson (moderator)

Title:	CB: # MBS2_BearerMgmt - Summary of email discussion - 2nd round

Document for:	Approval

# Introduction

This is the summary of offline discussions for AI 22.2.4, for which Mdme Chair summarized the papers as follows:

CB: # MBS2_BearerMgmt

- Further discussion on the open issues

- Flow control mechanism for Multicast/Broadcast

- E1AP/F1AP signalling for Multicast/Broadcast session management

- Stage2/stage3 TPs if agreeable and check details, split work, if needed

- Capture agreements and open issues

(E/// - moderator)

Summary of offline disc R3-215888

in the first round we had the following agreements:

For Broadcast and Multicast, optional use of DL flow control in the shared F1-U tunnel.

For split MRB with common PDCP, shared F1-U tunnel is used, existing NR user plane protocol functions need to be reviewed for their applicability for MBS.

For Multicast, reuse the existing UE-associated F1AP procedures to provide per UE the joined MBS Session IDs, further FFS UE specific MBS information and MBS context information (FFS).

so, the remaining topics are 3.1.2, 3.1.3, 3.1.4, 3.2.1, 3.2.2, 3.2.3, 3.3.2, which are repeated, but rephrased, take into account feedback received in the 1st round.

# For the Chairman’s Notes

Propose the following:

to be added

# 2nd round Discussion

## Shared F1-U bearer control

### resolved

### gNB-DU amd gNB CU role for F1-U bearer control.

The moderator gathers that for this item, the question was probably ambiguously worded. The following worksplit between DU and CU is proposed:

1 - The gNB-CU requests the gNB-DU to setup MBS Session Resources, providing: MRB and QoS flow information, area information.

2 - In the successful case, the gNB-DU acknowledges the request and triggers the F1-U shared bearer setup.

3 - The F1-U shared bearer setup is performed by means of a separate F1AP procedure (NOTE: this follows the scheme on NG discussed for MC and BC).

4 - The gNB-DUs decision to setup an F1-U shared bearer is based on

- 	a. MBS service area and Area Session information

-	b. in case of multicast: taking into account the presence of joined UE’s with established UE context in the gNB-DU

-	c. the gNB-DUs decision to either setup a per-cell or per-DU or any other F1-U aggregation policy. [Note that such decision is also determined by area session information, if available]

-	d. NB-, based on the gNB-DU’s decision to setup F1-U shared bearers per served cell or per DU (or any other DU specific F1-U bearer setup policy), for multicast: taking into account joined UE information, MBS service area (including Area session information).

5 - The F1-U shared bearer release setup is performed by means of a separate F1AP procedure (NOTE: this follows the scheme on NG discussed for MC and BC).

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| ZTE | Can only agree with part of the proposed (1, 4). The wording on “F1-U aggregation” in 4 might be a bit vague and needs more explanation. - The F1-U shared bearer setup, could be just simply the response message to the one from gNB-CU. Does not see the need to define a separate F1AP procedure. - per DU tunnel in case of no location dependent content is the optimum choice, but we are willing to compromise to let DU choose per DU or per cell either because companies think it is blocking the flow control or for the best flexibility. |

### postponed

### postponed

## MBS Session Resource Context Control

### E1AP: MBS Session Resource Control

Having in mind the fact that the UP resources for PDU Session Resources and MBS Session Resources are can in general not be assumed to be located within the same logical gNB-CU-UP, using UE associated procedures is not possible. The moderator proposes to consider this fact, re-consider previous positions and agree on the following: Introduce a setup of E1 procedures to control NR MBS resources in gNB-CU-UP including NG-U and F1-U terminations. The E1 procedures are common for BC and MC. FFS whether this works on MRB context or MBS Session level.

| Company | Comment |
| --- | --- |
| Ericsson | agree, we would prefer E1 procedures to work on MBS session level |
| ZTE | Agree. The QoS and bearer mapping info are per MBS session. |

### resolved

### F1AP: MBS Session Resource Control

Following up from the necessity to define MBS Session Resource specific E1AP procedures, F1AP should follow the same approach and define separate MBS Session Resource specific procedures. The following is proposed to be agreed:

Define non-UE associated signaling for MBS Session Resource Control, i.e. gNB-CU triggered for setup, modification and release of MBS Session Resources.

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| ZTE | Agree. Common signaling for common resources. |

### F1AP: dependency on RAN2 on NR MBS specific DU to CU RRC Information Container

In Rel-15, RRC was designed in a way to reflect CU/DU functional split decided by RAN3 and to allow respective DU to CU communication to construct the DU to CU RRC Information Container at the DU and provided to the UE within RRCReconfiguration from the CU. 
[NOTE: F1AP mainly transports the CellGroupInfo from DU to CU. The assumed “MBSCellGroupInfo” would need to be defined that it can be re-used for each (or at least the majority of) UE(s) in the cell.]

If RAN2 is able to provide for Rel-17 an NR MBS specific DU to CU RRC Information Container in a way that MBS Session related RRC Configuration for the Lower Layers residing in the gNB-DU so that identical content of the DU to CU RRC Information container can be added to the all the UEs that have joined the multicast session. Agree to send an LS to RAN2 asking to consider such RRC design.

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| ZTE | Agree. In the LS RAN3 motivation shall be included, e.g. efficiency and scalability on F1AP operation. |

## F1-U Flow Control for NR MBS

### resolved

### F1-U Flow control for NR MBS

Agree the following:

If F1-U flow control is applied for NR MBS

-	Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast

-	No additional protocol elements required to be specified in TS 38.425 for support of BC MBS NR

-	Discussions on additional procedure text are necessary for the DDDS procedure to clarify how to specify how the receiving node shall interpret the contained information in case DDDS is applied on an MRB.

Please provide your view

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| ZTE | Agree with all three. |

# 1st round Discussion

## Shared F1-U bearer control

### Turn the WA: For split MRB with common PDCP, shared F1-U tunnel is used into an agreement

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Nokia | Agree. |
| Qualcomm | Agree. For UE specific retransmission initiated by CU, a separate per UE F1-U tunnel may be needed. |
| Huawei | Agree |
| Lenovo, Motorola Mobility | Agree |
| LGE | Agree |
| CMCC | Agree |
| ZTE | Agree. |
| CATT | Agree with QC |
| Samsung | Agree |
| NEC | Agree |

### Agree that the gNB-DU decides how many F1-U shared bearers are established per MRB context, taking into account potential needs for location dependent content delivery.

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Nokia | Disagree. CU seems better placed to see the location dependent need. |
| Qualcomm | Disagree. Bearer establishment should be decided by CU. |
| Huawei | Disagree. Same view with Nokia. |
| Lenovo, Motorola Mobility | Disagree. Same view with Nokia. |
| LGE | Disagree. Same view with Nokia. |
| CMCC | Disagree. Same view with Nokia. |
| ZTE | If this is about location dependent content delivery then it should be CU to decide as the context about location is known to CU first. |
| CATT | Disagree |
| Samsung | Disagree. Same view with Nokia. |
| NEC | Disagree, it should be CU not DU. |

### If finally agreed by RAN2 for mobility, agree that Gnb-DU shall be enabled deciding whether retransmitted data is provided for a specific UE only (scheduled with C-RNTI) or for all Ues (scheduled with G-RNTI).

| Company | Comment |
| --- | --- |
| Ericsson | agree, and we do not want to see per-UE F1-U bearers being setup for retransmission |
| Nokia | Pending RAN2. |
| Qualcomm | For PDCP Status Report based retransmission, CU should send the retransmission packet to DU using UE specific F1-U tunnel. Typically, the retransmission is UE specific (C-RNTI). If many Ues didn’t receive the packet, G-RNTI based retransmission may be more efficient. In this case, CU sends the missed packet to DU using shared F1-U tunnel and DU decides to deliver by G-RNTI or C-RNTI. If in the future, RLC feedback is supported for MRB, DU can decide retransmission using PTP or PTM. For HARQ retransmission via C-RNTI or G-RNTI, it is up to RAN1. |
| Huawei | Disagree. For per UE retransmission and for gap-filling transmission after handover, the Gnb-DU does not have the information to decide to send the data via PTP. In these cases, the Gnb-CU can send these packets to the Gnb-DU via UE specific tunnel(s), then DU will know it should send them to the UE via PTP. |
| Lenovo, Motorola Mobility | During mobility, the PTP retransmission may be triggered by PDCP SR. Since the PDCP function is located in Gnb-CU, it would be better Gnb-CU makes the decision. For HARQ retransmission over PTP, we agree that Gnb-DU makes the decision. |
| LGE | Wait until RAN2 reaches final agreements for mobility. |
| CMCC | Prefer Gnb-CU making the decision. When focus on mobility issues, PTP retransmission is determined by PDCP function which located in Gnb-CU. |
| ZTE | Disagree. Same view with QC on the PDCP and HARQ re-tx. |
| CATT | Disagree On retransmission, similar to QC’s point of view |
| Samsung | Retransmission based on PDCP Status Report should be in Gnb-CU. |
| NEC | Disagree, wait for RAN2 final decision. |

### Support of PTP only configuration. There are two approaches:

1) although possible as per RRC configuration, F1(/E1) does not foresee explicit support of PTP only configuration

2) F1/E1 does provide explicit support of PTP only configuration.

| Company | Comment |
| --- | --- |
| Ericsson | please go for 1), reducing options, keeping the system simple and avoiding DU-CU communication if for once, “PTP-only” needs to re-configured to “PTP/PTM”. |
| Nokia | 2) I assume the question refers to the support of non-split MRB PTP bearer. Given RAN2 recent decision to not enable deactivation of the PTM leg of a split MRB bearer, there are cases where it is useful to change the bearer type for a UE from split MRB into non-split MRB PTP bearer. See tdoc R3-214791 for details. |
| Qualcomm | Based on Nokia feedback on RAN2 decision, it seems that 2) is the only option. |
| Huawei | 2) We should align with RAN2 decision. For an MRB configured with PTP leg only, whether shared F1 tunnel or UE dedicated F1 tunnel to be used should be discussed by RAN3, as mentioned in contributions [6][11]. |
| Lenovo, Motorola Mobility | 2) we should align with RAN2 agreement. |
| LGE | 2) We should align with RAN2 decision. |
| CMCC | 2) We should align with RAN2 decision. |
| ZTE | We prefer 2) because of the scenario of PTP only does exist. And we prefer a shared F1-U tunnel for whatever MRB configuration for initial transmission. |
| CATT | Based on RAN2 discussion, for MRB configured with PTP only, only the RLC entity of PTP transmission is established in DU. So 2) is the only option |
| Samsung | 2) Prefer to align with RAN2. |
| NEC | 2) We should align with RAN2 decision. |

## MBS Session Resource Context Control

### E1AP: Introduce a setup of E1 procedures to control NR MBS resources in Gnb-CU-UP including NG-U and F1-U terminations common for BC and MC. FFS whether this works on MRB context or MBS Session level.

| Company | Comment |
| --- | --- |
| Ericsson | agree, we would prefer E1 procedures to work on MBS session level |
| Nokia | Disagree. The tunnel termination could be piggy-backed together with the first UE context setup. |
| Huawei | Disagree. Better to align with the way used for NG for MC and BC. |
| Lenovo, Motorola Mobility | Disagree. We share the same view with Huawei. |
| ZTE | Agree. Common signaling for common resource management. |
| CATT | Agree |
| Samsung | Agree. For multicast, CU-CP notifies the activation of multicast to CU-UP and DU. It is per-session level message. |
| NEC | No strong view, both setup of E1 procedures or piggy back with UE context setup works. |

### F1AP: Turn the WA: For Multicast, reuse the existing UE-associated F1AP procedures to provide per UE the joined MBS Session IDs into an agreement

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Nokia | Agree. |
| Huawei | Agree. |
| Lenovo, Motorola Mobility | Agree |
| CMCC | Agree |
| ZTE | Agree. |
| CATT | Agree |
| Samsung | Agree |
| NEC | Agree |

### F1AP: For MC, shall MBS Session information be provided

1) via UE associated F1-C signaling or

2) via “shared” “MBS session associated” signaling

and hence control of shared resources be achieved

1) by UE-associated signaling or

2) by “MBS session associated” signaling?

| Company | Comment |
| --- | --- |
| Ericsson | In order to keep UE associated resource handling separate from shared MBS session resource handling, we propose to go for 1), i.e. introduce “MBS session associated” signaling procedures, which can be nicely used for MC and BC. This is a straightforward, allows to nicely separate MBS resource control from individual resource control and is the only reasonable stage-3 design choice. |
| Nokia | We prefer the UE-associated way which can piggyback the MBS session information, therefore 1). This can save the addition of non-UE associated procedure by piggybacking the MBS context and the shared tunnel addresses. Overall simpler. |
| Huawei | Prefer 1) |
| Lenovo, Motorola Mobility | - To align with NG interface. |
| CMCC | We prefer 1) |
| ZTE | 2) Again, common signaling for common resource management. |
| CATT | We prefer to use the “MBS session associated” signaling for providing MBS Session information and control of shared resources, therefore 2）. |
| Samsung | - is useful to session update procedure. E.g. if session level QoS information is changed, a session level message is efficient. |
| NEC | No strong view, both option1 and option2 works. We don’t see clear pros and cons of these two options. |

### F1AP: RAN2 shall design RRC in a way that MBS Session related RRC Configuration for the Lower Layers residing in the Gnb-DU in a way that the identical content of the DU to CU RRC Information container can be added to the all the Ues that have joined the multicast session

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Nokia | I would refrain from talking in the name of RAN2. |
| Qualcomm | Agree |
| Huawei | Same view with Nokia, it is up to RAN2. |
| Lenovo, Motorola Mobility | We tend to agree. |
| LGE | We tend to agree. |
| CMCC | Up to RAN2 decision |
| ZTE | Agree. Don’t think it is about RAN2, but RAN3 to decide how the signaling works in light of the exactly the same signaling content delivery on F1-C. Having F1 to follow the NG principle does not always work. Each interface works in its own role. |
| CATT | Pending RAN2. For multicast session, different UE may have different PTP leg configuration, why are the DU to CU RRC container the same? |
| Samsung | Up to RAN2 decision. |
| NEC | Agree, pending RAN2 discussion |

## F1-U Flow Control for NR MBS

### It is proposed to turn the WA
WA: For Broadcast and Multicast, optional use of DL flow control in the shared F1-U tunnel. 
into an agreement

Please provide your view below

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Nokia | Agree. |
| Qualcomm | Agree |
| Lenovo, Motorola Mobility | Agree |
| LGE | Agree. |
| CMCC | Agree |
| ZTE | Agree. |
| CATT | Agree |
| Samsung | Agree |
| NEC | Agree |

3.3.2: It is proposed that for BC no additional specification work is needed for F1-U flow control

please provide your view below

| Company | Comment |
| --- | --- |
| Ericsson | agree |
| Nokia | Some clarification to TS 38.425 is useful. |
| Qualcomm | Agree. No actual frame change, just some clarification. |
| Huawei | Same view with Nokia. Need some clarification. |
| Lenovo, Motorola Mobility | Same view with Nokia and Huawei. |
| LGE | Same view with Nokia. |
| CMCC | Same view with Nokia, Qualcomm and Huawei. |
| ZTE | Agree. And It is helpful to state that even it is per DU tunnel, flow control is left to implementation and no spec impact is needed. |
| CATT | Some modifications are required. Details need to be checked based on a draft CR. |
| Samsung | We also think clarification is needed to 38.425 |
| NEC | Same view with Nokia and Huawei. |

We seem to have two options on the table to progress with specification of F1-U flow control for multicast NR MBS, if applied, in case multiple RLC entities are involved:

1) minimum approach: no additional specification work, with the option to specify explicitly that behaviour of Gnb-DU is implementation matter.

2) extensive approach: specify additional details of Gnb-DU as e.g. in [2], [14], [15]

| Company | Comment |
| --- | --- |
| Ericsson | we suggest 1) |
| Nokia | We prefer 2) because some specification is needed in TS 38.425. |
| Qualcomm | Prefer 2 |
| Huawei | Agree with Option 2). Some clarifications in TS 38.425 is needed, as R3-215143. - Reuse existing PDU Type 0 and Type 1 to support flow control for multicast and broadcast. - For MRB with PTM leg, the ‘Highest transmitted/delivery NR PDCP Sequence Number’ could be reused to indicate the Highest transmitted/delivery NR PDCP Sequence Number of the slowest UE within the cell or the DU. - in case there is no PTP-AM leg, reuse the ‘Highest transmitted NR PDCP Sequence Number’, - in case there is PTP-AM leg, reuse ‘Highest delivery NR PDCP Sequence Number’. |
| Lenovo, Motorola Mobility | 2). As discussed in [11], some clarifications are needed: In 38.401, it is specified that during intra-NR mobility procedure: A Random Access procedure is performed at the target Gnb-DU. The target Gnb-DU sends a Downlink Data Delivery Status frame to inform the Gnb-CU. It is also specified in 38.425: As soon as the corresponding node detects the successful RACH access by the UE for the corresponding data radio bearer(s), the corresponding node shall send initial DL DATA DELIVERY STATUS frame to the node(s) hosting the NR PDCP entity(ies). In option 1, it is not possible for Gnb-CU sending the DDDS after RACH procedure. If we adopt option 1, above issue should be addressed |
| CMCC | We prefer option2. The approach details are explained in R3-215673. |
| ZTE | Go 1) This is the same issue to the last question about BC. - For BC it is multiple cell delivery with its own RLC entity. - For MC it is multiple per UE or per PTM delivery with its respective RLC entity. We tend to agree with HW’s intention on the scenarios, but it is also OK and good to leave DU to decide. As long as we are not flooding DU based on its report, there wont be any cross vendor impacts. No worries! |
| CATT | Prefer 2） |
| Samsung | We prefer option2. Some description is needed to TS38.425 |
| NEC | We prefer option1. |

# References

- R3-215043	Further Consideration on MBS Bearer Management over F1E1 (CATT), discussion

- R3-215044	Further Consideration on Flow Control Mechanism over F1-U (CATT), discussion

- R3-215046	TP for 38.463 on MBS Bearer Management at E1 interface (CATT), other

- R3-215141	(TPs to TS 38.401 38.470, 38.460 BL CRs) Bearer management for Broadcast (Huawei, CBN, Nokia, Nokia Shanghai Bell, China Unicom, China Telecom, CMCC), other, 38.401

- R3-215142	(TPs to TS 38.401, 470, 460 BL CRs) Bearer Management for Multicast (Huawei, CBN, China Unicom, China Telecom, CMCC), other, 38.401

- R3-215143	(TP to TS 38.425 BL CR) Flow Control for MBS (Huawei, CBN, China Unicom, China Telecom), other, 38.425

- R3-215205	On Bearer Management over E1/F1 (Ericsson), other

- R3-215206	[TP for BL CR 38.473, 38.463] On Bearer Management over E1/F1 (Ericsson), other, 38.473

- R3-215272	(TP to TS 38.460 BL CR) Support of Multicast Distribution Setup and Release (Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated), other

- R3-215273	(TP to TS 38.463 BL CR) Support of Multicast Distribution Setup and Release (Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated), other

- R3-215274	Remaining Issues on F1 Interface for MBS (Lenovo, Motorola Mobility, discussion

- R3-215372	(TP to TS 38.425 BL CR) FLow Control over F1 for NR MBS (ZTE), other

- R3-215373	(TP to TS 38.473 BL CR) Bearer Management over F1 for NR MBS (ZTE), other

- R3-215516	Flow control for MBS multicast session (Samsung), other

- R3-215673	Discussion on MBS Bearer Management (CMCC), discussion

- R3-215138	Consideration on Multicast Session Management (Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom), other (partly, proposal 9)