3GPP TSG-RAN WG3 #114-e	R3-215852

Online, 1 – 11 Nov 2021

Agenda Item:	9.3.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on MLB (CB #SONMDT3)

Document for:	Approval

# Introduction

CB: # SONMDT3_LoadBalance

- Topics to discuss:

- Per-SSB Mobility Settings Change over Xn and F1

- Per-Slice Mobility Settings Change

- NUL and SUL CAC

- Per-SSB SUL PRB

- MLB for PSCell and “Aggregated NR CAC”

- load metric for UEs in RRC Inactive

- MLB for resource aggregation

- Mechanism of controlling load balancing

- Start with summary of offline, proceed to TPs if there are agreements

(Nok - moderator)

Summary of offline disc R3-215852

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Completion of the work on per-slice PRB reporting

Three papers propose how the “FFS” in the semantics of the per-slice PRB reporting can be clarified:

Options 1 proposed in [2] and [5]:

It is observed that the semantics in the existing IEs for not define the reference point either, while the beginning of the semantics define what is actually measured. Hence, a correction is proposed to both, the new and existing IEs to make the descriptions coherent.

Option 2 proposed in [11]:

The “FFS” is simply replaced with a description of the reference for the percentage calculation.

Question 1-1: Companies are requested to comment on the proposed two options to resolve the issue with the “FFS”. Please, indicate which is preferred and if there is any technical problem with any of proposed options?

| Company | Preferred option | Any technical problem? Please, indicate if you cannot accept one of the options and why. |
| --- | --- | --- |
| Nokia | 1 | Both options are technically all right. Preference is based on the fact that option 1 resolves some logical discrepancy in the semantics of the existing IEs (apparently relevant, because two companies noticed this independently). |
| Deutsche Telekom | 1 | Both options are feasible, but Option 1 solves any ambiguity for both existing and new IEs. |

## Mobility Setting Change enhancements

At RAN3 #113, the support for beam-based MSC has been nearly agreed. At this meeting, the solution is proposed in [7], [8] and [11].

In [5], it is argued that per-beam MSC is not needed. Instead per-slice MSC is more beneficial. Also [7] supports per-slice MSC. This is commented to be unnecessary in [8] and [11].

Question 2-1: Companies are requested to comment on the proposed two enhancements for the Mobility Setting Change.

| Company | Shall RAN3 enable per-beam MSC? | Shall RAN3 enable per-slice MSC? |
| --- | --- | --- |
| Nokia | We consider it useful. One question concerns CHO: how the per-beam MSC would work with CHO, if the triggering condition is not defined per-beam and the UE executes it on its own? | We consider it possibly useful, but we’d like to have it once again explained briefly, how the nodes shall handle the MSC requests if the slice is not supported in the neighbour node? |
| Deutsche Telekom | We are fine with enabling per-beam MSC as it would allow more granular settings based on M-MIMO deployments. | In principle, we see it useful at least for slices carrying services with homogenous characteristics. But there is the need to think about scenarios where slices may have services with different characteristics and where several slices are used by a UE in parallel. |

In addition, in [1], the need to provide the HO reference point per each beam is further explained. It is proposed either as a supplementary solution for the per-beam MSC, or its substitute in case per-beam MSC is not to be supported.

Question 2-2: Companies are requested to comment if they see any benefit (or technical problem) with the information exchange on the per-beam HO reference point?

| Company | Shall RAN3 enable information on per-beam HO reference point on F1AP? |
| --- | --- |
| Nokia | We propose the solution and consider it useful. Especially, if the per-beam MSC is not accepted (there are numerous objections to enable it), the information on the per-beam triggering point from the DU can help. |

## Information on other cells in the load reporting

There are two proposals related to reporting additional resources (of other cells):

- In [6], reporting of possible SgNB’s CAC is proposed (when EN-DC could possibly be configured). Alternatively, an accumulated CAC is provided.

- In [8], it is proposed to provide plain list of cells that can be configured with CA for the UE (“can be configured” means the CAC there is above configured threshold).

Question 3-1: Companies are requested to comment on the proposed two enhancements related to reporting additional resources of other cells.

| Company | Shall RAN3 enable information on the possible SgNB cells? | Shall RAN3 enable information on the possible CA cells? |
| --- | --- | --- |
| Nokia | We’re fine to keep working on this. The biggest problem is the selection of the cells to report: if full list of provided (which may be very long!), the overloaded MN will not know which ones may be relevant for the UE. The alternative, i.e. the aggregated CAC will be built including possibly irrelevant SgNB. Neither of the solutions, as proposed, seem to offer benefits, if there is not mechanism to select only the relevant possible SgNBs. | We’re fine to keep working on this. This is clearer mechanism. However, we’re somewhat surprised the mechanism offers only a list of possible cells instead of CAC value there. If only the list is offered, obvious question arises: how the threshold is configured? |
| Deutsche Telekom | We see it generally beneficial to have such information, but as Nokia pointed out the suitability of proposed solutions is still vague. | Same view as for possible SgNB cells. |

## Additional metric

Further enhancements are proposed for the load metrics to be reported and the reporting mechanism:

- In [8], it is proposed to add the information on stored contexts for inactive UEs.

- In [5], separate reporting of SUL and NUL CAC is proposed (addition of NUL to the existing and agreed enhancements).

- In [3], SUL part of the per-beam resource utilisation is proposed to be added.

Question 4-1: Companies are requested to comment on the information related to the stored contexts of inactive UE.

| Company | Comment |
| --- | --- |
| Nokia | No, we do not think this brings any benefit. Stored contexts have no impact on the radio resources. Also, it is unclear how to map UE contexts to cells, considering that memory is used at the CU. If it is to be the last cell that served the UE, it tells nothing about the “potential load”, because the UE may activate in another cell than the one where it was pushed to inactive. |
| Deutsche Telekom | Generally, we see it beneficial to have such information about potential inactive UEs that may require cell resources in near future, but we acknowledge that benefits may vary according to the underlying deployment scenario (e.g. more static UE locations or dynamic change of UEs between cells). |

Question 4-2: Companies are requested to comment on the separated reporting of SUL and NUL CAC (addition of NUL to the existing and agreed enhancements).

| Company | Comment |
| --- | --- |
| Nokia | No, this is not necessary. When the SUL was added, it was clarified that the legacy information addresses the whole cell, i.e. the SUL and NUL. If now NUL is considered separately, the existing CAC seem unnecessary. |
| Deutsche Telekom | If the calculation of UL CAC is a simple addition of SUL and NUL CACs, then the addition of NUL CAC is unnecessary. Otherwise, we are ok with that extension. |

Question 4-3: Companies are requested to comment on adding the SUL resource utilisation to the per-beam resource reporting.

| Company | Comment |
| --- | --- |
| Nokia | We leave it up to further discussion in RAN3. However, if enabled it creates quite a complication, because SUL is already enabled to be reported as CAC (not beam-specific). So the proposal would have to be reworked to align it with other agreed changes. |
| Deutsche Telekom | No strong opinion. It makes sense to have a consistent representation of metrics, but the benefits for per-beam SUL are questionable due to typically lower frequency band usage. |

## Reporting enhancements

In [8], a mechanism to enable stopping, pausing and resuming of the reporting from the reporting node side is proposed.

Question 5-1: Companies are requested to comment on enabling the proposed additional control (stopping of measurement, pausing or resuming it) from the reporting node’s side.

| Company | Comment |
| --- | --- |
| Nokia | If I remember all right, in LTE, it was made possible that the reporting node could use the Resource Status Reporting Initiation procedure, with the known measurement IDs to stop given reporting session, couldn’t it? So ‘stop’ does not seem needed. Pausing and resuming may be considered, if RAN3 acknowledges that resource status reporting may be a real burden for the reporting node. This is doubtful to us. |
| Deutsche Telekom | We are generally ok to introduce such mechanism but see still details that should be discussed. In e-mail discussion Huawei already raised the issue that also the measurement type, not only the number of cells could be a reason for overload. If the reporting is not fully stopped, which node makes the decision on cells and measurements to be continued on lower load level. Can there be any preference by the requesting node? Will such approach only be used for cell resource status reporting or also for other reportings under discussion in parallel CBs? |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-214812, [SON, TP to BL CR to TS 38.473]  Further discussion on the CU-DU impacts of the per-beam mobility setting change (Nokia, Nokia Shanghai Bell)

- R3-214813, [TP to SON BL CR to 38.423, NR_ENDC_SON_MDT_enh]  Semantics for the PRB per slice description (Nokia, Nokia Shanghai Bell)

- R3-214946, (TP for SON BLCR for 38.423) Load Balancing Enhancements: SUL reporting in Radio Resource Status (NEC)

- R3-214947, (TP for SON BLCR for 38.473) Load Balancing Enhancements: SUL reporting in Radio Resource Status (NEC)

- R3-214957, (TP for SON BLCR for 38.423, 38.473) Load Balancing Enhancements (Huawei)

- R3-215040, Discussion on PSCell MLB (CATT)

- R3-215176, Discussion on per-SSB and per-slice Mobility Settings Change (NEC)

- R3-215445, (TP for SON BL CR for TS 38.423) MLB enhancements (Ericsson)

- R3-215446, (TP for SON BL CR for TS 36.423) MLB enhancements (Ericsson)

- R3-215769, Further Discussion on Load Balancing Enhancements in NR (ZTE,China Telecom, China Unicom)

- R3-215770, TP for BL CR 38.423 on Load Balancing Enhancements (ZTE,China Telecom, China Unicom)

- R3-215771, TP for BL CR 38.473 on Load Balancing Enhancements (ZTE,China Telecom, China Unicom)