3GPP TSG-RAN WG3 #114bis-e											R3-220228

Online, 17 Jan – 26 Jan 2022

Agenda item:		21.2

Source:			ZTE

Title:	(TP for Introduction of Enhanced IIoT support over F1) Time Synchronization enhancements

Document for:		Approval

- Introduction

This TP is based on the following proposals and observations.

Observation 1: For unicast, the gNB-CU can send the minimum Uu time synchronization error budget (e.g. the most rigorous budget) of multiple UEs to the gNB-DU, and gNB-DU reports the reference time with the minimum budget based on the gNB-CU request.

Observation 2: For broadcast, the Uu time synchronization error budget can be decided in gNB-DU by OAM.

Proposal 1: For F1AP, the gNB-CU notifies the gNB-DU of the Uu time synchronization error budget information through the REFERENCE TIME INFORMATION REPORTING CONTROL message.

Proposal 2:  RAN3 confirms whether the target gNB needs to provide the UE with configuration information for RTT measurement during the handover.

Detailed analysis can be seen in the related discussion paper (R3-220227).

- Text Proposal for TS38.473

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

  - 8.12	Reference Time Information Reporting procedures

  - 8.12.1	Reference Time Information Reporting Control

  - 8.12.1.1	General

The purpose of the Reference Time Information Reporting Control procedure is to command the gNB-DU to send the requested accurate reference time information to the gNB-CU. The procedure uses non-UE associated signalling.

  - 8.12.1.2	Successful Operation

Figure 8.12.1.2-1: Reference Time Information Reporting Control

The gNB-CU initiates the procedure by sending REFERENCE TIME INFORMATION REPORTING CONTROL message to the gNB-DU. Upon reception of the REFERENCE TIME INFORMATION REPORTING CONTROL message, the gNB-DU shall, if supported, perform the requested reference time information reporting action.

The Report Type IE indicates to the gNB-DU whether:

-	to report on demand;

-	to report periodic, with a frequency as specified by the Report Periodicity IE;

-	to stop periodic reporting.

If the Uu time synchronization error budget IE is included in the REFERENCE TIME INFORMATION REPORTING CONTROL message, the gNB-DU node may use it as described in TS 23.501 [21].

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

  - 9.2.11.1	REFERENCE TIME INFORMATION REPORTING CONTROL

This message is sent by the gNB-CU and is used to request the gNB-DU to deliver the accurate reference time information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Reporting Request Type | M |  | 9.3.1.147 |  | YES | reject |
| Uu Time Synchronization Error Budget | O |  | 9.3.1.yyy |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.yyy	Uu time synchronization error budget

This IE indicates the time synchronization error budget for the time synchronization service as specified in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uu Time Synchronization Error Budget | M |  | INTEGER (1..1000000, …) | Expressed in units of 1ns. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>