3GPP TSG-RAN WG3#114bis-e                                             	                     R3-220369

E-Meeting, 17 – 26 January 2022

Agenda item:	9.3.4

Source:	Nokia, Nokia Shanghai Bell, Orange, Verizon Wireless

Title: 	Correction of NAS PDU Delivery

Document for:	Approval

# Introduction

At the last RAN3#114 meeting the NAS PDU delivery for the PDU Session Modify case was discussed without final conclusion:

RAN3#114e:

Choose one of the following text proposals for PDU Session modification corrections in the next meeting:

Opt1: If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE.

Opt2: The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE.

# When the NG-RAN receives a PDU Session Resource Modify Request message which only includes the PDU Session AMBR IE, NG-RAN shall send NAS-PDU to UE when the PDU Session AMBR IE is modified successfully?

# This paper continues the discussion.

# Discussion

It was clearly shown at RAN3#114 that the NAS PDU contained in the PDU Session Modify request transfer IE can contain three types of modifications (see the annex of this document and section 8.3.9 of TS 24.501):

- NAS Modification containing parameters transparent to the gNB i.e. modification of RQI timer (type 1).

- NAS Modification containing parameters linked to a modification requested to gNB in the PDU Session Modify request transfer IE e.g. addition/release of QoS Flow (type 2).

- NAS Modification containing both parameters transparent to the gNB and parameters linked to a modification requested to gNB in the PDU Session Modify request transfer IE (type 3).

Obviously for modifications of type 1 and 3 the gNB shall always pass the NAS PDU so that the UE receives the transparent NAS request. For type 2 we could envision some optimizations i.e. to not send the NAS PDU when the modification in the gNB fails or when the part of the gNB modification related to DRB configuration fails.

However, the point is that there is no way for gNB to know what is in the NAS PDU included in the PDU Session Modify request transfer IE and therefore whether the modification is of type 1, type 2 or type 3.

As a consequence, the gNB needs to always transfer to UE the NAS PDU regardless of the outcome of the gNB modification (success or failure), because it can be a modification of type 1 or type 3.

This also means that in case of partial success or failure of the gNB modification, this is reported back in the PDU Session Resource Modify Response Transfer container (partial gNB modification success) or the PDU Session Modify Failure Transfer container (gNB modification total failure) to the SMF and the SMF sends a second PDU Session Resource Modify Request including a new NAS PDU which updates the UE based on the result of the first modification.

# Conclusion and Proposal

The handling of the NAS PDU in the PDU Session Modify Request message is done in simple way: because the gNB has no way to determine what is in the NAS PDU received in the PDU Session Modify request transfer IE, which may contain updates which are transparent to the gNB, therefore the gNB has no other choice than always transfer the NAS PDU to the UE, regardless of the PDU session resource modification result in the gNB.

Proposal 1: agree to update the procedural text as per above conclusion:

-	If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE regardless of the outcome of the PDU Session Resource Modification.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

# Annex : content of the NAS PDU Session Modification Command

### 8.3.9 PDU session modification command #### 8.3.9.1 Message definition The PDU SESSION MODIFICATION COMMAND message is sent by the SMF to the UE to indicate a modification of a PDU session. See table 8.3.9.1.1 Message type: PDU SESSION MODIFICATION COMMAND Significance: dual Direction: network to UE Table 8.3.9.1.1: PDU SESSION MODIFICATION COMMAND message content IEI; Information Element; Type/Reference; Presence; Format; Length. Extended protocol discriminator; Extended protocol discriminator 9.2; M; V; 1. PDU session ID; PDU session identity 9.4; M; V; 1. PTI; Procedure transaction identity 9.6; M; V; 1. PDU SESSION MODIFICATION COMMAND message identity; Message type 9.7; M; V; 1. 59; 5GSM cause; 5GSM cause 9.11.4.2; O; TV; 2. 2A; Session AMBR; Session-AMBR 9.11.4.14; O; TLV; 8. 56; RQ timer value; GPRS timer 9.11.2.3; O; TV; 2. 8-; Always-on PDU session indication; Always-on PDU session indication 9.11.4.3; O; TV; 1. 7A; Authorized QoS rules; QoS rules 9.11.4.13; O; TLV-E; 7-65538. 75; Mapped EPS bearer contexts; Mapped EPS bearer contexts 9.11.4.8; O; TLV-E; 7-65538. 79; Authorized QoS flow descriptions; QoS flow descriptions 9.11.4.12; O; TLV-E; 6-65538. 7B; Extended protocol configuration options; Extended protocol configuration options 9.11.4.6; O; TLV-E; 4-65538. 77; ATSSS container; ATSSS container 9.11.4.22; O; TLV-E; 3-65538. 66; IP header compression configuration; IP header compression configuration 9.11.4.24; O; TLV; 5-257. 74; Port management information container; Port management information container 9.11.4.27; O; TLV-E; 4-65538. 1E; Serving PLMN rate control; Serving PLMN rate control 9.11.4.20; O; TLV; 4. 1F; Ethernet header compression configuration; Ethernet header compression configuration 9.11.4.28; O; TLV; 3.