3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220615

# Online Meeting, 17th– 26th January 2022

Agenda Item:	9.3.4

Source:	Ericsson

Title: 	Discussion on PDU session related NAS-PDU delivery

Document for:	Discussion and Approval

# 1	Information

At RAN3#114-e, the topic is discussed.

RAN3#114e: Choose one of the following text proposals for PDU Session modification corrections in the next meeting: Opt1: If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE. Opt2: The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE. When the NG-RAN receives a PDU Session Resource Modify Request message which only includes the PDU Session AMBR IE, NG-RAN shall send NAS-PDU to UE when the PDU Session AMBR IE is modified successfully?

This paper discusses further on this topic, taking the SA2 reply into consideration.

# 2	Discussion

PDU session modify procedure can modify one or muliple of the following items with one message:

- TNL is modified;

- AMBR is modified;

- QFI x is added/modified/deleted;

There is no failure message, but in the response message, there are two different IEs, the “PDU Session Resource Modify Response List” will contain the PDU sessions that have been successfully modified and the “PDU Session Resource Failed to Modify List” will contain the PDU sessions that have been failed to be modified.

Observation 1: In the modification response, NG-RAN node reports the PDU sessions being successfully modified and being failed in different IEs.

If all the Items are to be modified fails, the PDU session modification is failed.

If at least one of the Items is modificed successfully, the PDU session modification is sucessful.

In PDU session setup procedure, it is specified that:

For each PDU session successfully established the NG-RAN node shall pass to the UE the PDU Session NAS-PDU IE, if included, and the value contained in the PDU Session ID IE received for the PDU session. The NG-RAN node shall not send to the UE the PDU Session NAS PDUs associated to the failed PDU sessions.

Observation 2: It is specified in the PDU session resource setup procedure that the PDU session NAS-PDU is sent to the UE only when the PDU session is setup successfully.

The above principle should apply to PDU session modification.

PDU session NAS-PDU is optionally included in the PDU session resource modification message.

NG-RAN node does not know and should not need to know what are included in the PDU session NAS-PDU. But there should be no use case that when the PDU session is not successfully modified, the PDU session NAS-PDU needs to be sent to the UE. In our view, such implementation would cause inconsistence between NG-RAN node and UE.

Proposal 1: RAN3 to discuss and agree that the PDU session NAS PDU is sent only when the PDU session management procedure is successful.

# 3	Proposals

Observation 1: In the modification response, NG-RAN node reports the PDU sessions being successfully modified and being failed in different IEs.

Observation 2: It is specified in the PDU session resource setup procedure that the PDU session NAS-PDU is sent to the UE only when the PDU session is setup successfully.

Proposal 1: RAN3 to discuss and agree that the PDU session NAS PDU is sent only when the PDU session management procedure is successful.

# 4	Reference

[1]	R3-220613	Discussion on PDU session related NAS-PDU delivery, Ericsson

[2]	R3-220614	Discussion on PDU session related NAS-PDU delivery, Ericsson