3GPP TSG-RAN WG3 #114bis-e						                                                                        R3-220871

17-26 Jan 2022

Online

Agenda item:	9.3.4

Source:	CMCC

Title:	Discussion on PDU session resource modify

Document for:	Discussion

1	Introduction

Last meeting primarily discusses the issue of NAS-PDU deliver during PDU session resource modify procedure, and the following open issues are captured,

Choose one of the following text proposals for PDU Session modification corrections in the next meeting:

Opt1: If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE.

Opt2: The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE.

When the NG-RAN receives a PDU Session Resource Modify Request message which only includes the PDU Session AMBR IE, NG-RAN shall send NAS-PDU to UE when the PDU Session AMBR IE is modified successfully?

This contribution provides further discussions on NAS-PDU delivery during PDU session resource modify procedure.

2	Discussion

Last meeting the issue on whether PDU session NAS-PDU contained in PDU Session Resource Modify Request should be delivered to UE under different cases was raised, and the following 4 cases have been discussed [1],

- Case 1: PDU session modification without QoS flow to be added or modified or released.

Considering the following case at NG-RAN:

- Request: Only the PDU Session Aggregate Maximum Bit Rate IE is included.

- Case 2:  Only partial QoS flows to be added or modified are failed.

Considering the following case at NG-RAN:

- Request: QoS flow 1 and QoS flow 2 are to be added.

- NG-RAN Result: QoS flow 1 is added, while QoS flow 2 is failed.

- Case 3:  QoS flow to be added is failed, and QoS flow to be released is included.

Considering the following case at NG-RAN:

- Request: QoS flow 1 to be added, QoS flow2  to be released

- NG-RAN Result: QoS flow 1 is failed to be added, while QoS flow 2 release is always successful.

- Case 4:  All QoS flows to be added or modified are failed, and PDU Session AMBR IE is included.

Considering the following case at NG-RAN:

- Request: QoS flow 1 & 2 to be added, and PDU Session AMBR IE to be modified

- NG-RAN Result: All QoS flows to be added are failed, while update of PDU Session AMBR IE is always successful.

Additionally, two more cases are discussed during last meeting,

- Case 5:  QoS flows to be added or modified are all failed, and NG-U UP TNL modification is successful.

Considering the following case at NG-RAN:

- Request: QoS flow 1 and QoS flow 2 are to be added, and NG-U UP TNL is modified.

- NG-RAN Result: QoS flow 1 and QoS flow 2 are failed to be added, while NG-U UP TNL is modified successfully.

- Case 6:  QoS flows to be added or modified are all failed.

Considering the following case at NG-RAN:

- Request: QoS flow 1 and QoS flow 2 are to be added.

- NG-RAN Result: QoS flow 1 and QoS flow 2 are failed to be added.

As proposed by some company, we need to firstly identify under which cases the NAS-PDU should/shouldn’t be sent to UE, then we can figure out an appropriate correction on the current spec text.

For Case 1, according to the online discussion during last meeting, all companies agreed that the NAS-PDU should be sent to UE; however, companies still have different understandings on whether PDU Session AMBR IE will always be successfully modified or not. Based on the current spec text on the modification of PDU Session AMBR IE [2],

------------------------------------------------------------Start of quotation------------------------------------------------------------

-	If the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall store and use the received PDU Session Aggregate Maximum Bit Rate value when enforcing traffic policing for Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

------------------------------------------------------------End of quotation-------------------------------------------------------------

The text above gives us an impression that as long as PDU Session AMBR IE is included, the PDU Session AMBR will always be accepted by NG-RAN, namely the modification on PDU Session AMBR at NG-RAN will always be successful, so there’s no need to emphasize whether the PDU Session AMBR IE is modified ‘successfully’ at NG-RAN.

Proposal 1: Agree that when the NG-RAN receives a PDU Session Resource Modify Request message which only includes the PDU Session AMBR IE, the NG-RAN shall send NAS-PDU to UE.

Similarly, as indicated by the current spec below [2], if the QoS Flow to Release List IE is included, the NG-RAN will always release the QoS flows indicated in the list.

------------------------------------------------------------Start of quotation------------------------------------------------------------

-	For each QoS flow included in the QoS Flow to Release List IE, the NG-RAN node shall de-associate the QoS flow with the previously associated DRB.

------------------------------------------------------------End of quotation-------------------------------------------------------------

For Case 2 and Case 3, all companies agreed that the NAS-PDU should be delivered to UE.

Proposal 2: Agree that when only partial QoS flows to be added or modified are failed, NG-RAN shall send NAS-PDU to UE.

Proposal 3: Agree that when QoS flows to be added or modified are all failed while some QoS flow(s) is/are released, NG-RAN shall send NAS-PDU to UE.

For Case 4-6, the first issue we need to clarify is whether the PDU session is modified successfully or unsuccessfully under such cases, and different interpretations will lead to using different IEs in the response message, more specifically, as specified in TS 38.413 [2],

------------------------------------------------------------Start of quotation------------------------------------------------------------

#### 9.2.1.6	PDU SESSION RESOURCE MODIFY RESPONSE

This message is sent by the NG-RAN node and is used to report the outcome of the request from the PDU SESSION RESOURCE MODIFY REQUEST message.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| PDU Session Resource Modify Response List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resource Modify Response Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Modify Response Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Modify Response Transfer IE specified in subclause 9.3.4.4. | - |  |
| PDU Session Resource Failed to Modify List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resource Failed to Modify Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session Resource Modify Unsuccessful Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Modify Unsuccessful Transfer IE specified in subclause 9.3.4.17. | - |  |
| User Location Information | O |  | 9.3.1.16 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

------------------------------------------------------------End of quotation-------------------------------------------------------------

------------------------------------------------------------Start of quotation------------------------------------------------------------

#### 9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | Identifies the NG-U transport bearer at the 5GC node. | - |  |
| QoS Flow Add or Modify Response List |  | 0..1 |  |  | - |  |
| >QoS Flow Add or Modify Response Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | Ignore |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. | - |  |
| QoS Flow Failed to Add or Modify List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |

------------------------------------------------------------End of quotation-------------------------------------------------------------

As we can notice that even the PDU Session Resource Modify Response Item can optionally contain QoS Flow Add or Modify Response List and QoS Flow Failed to Add or Modify List, so if all QoS flows to be added or modified associated with a specific PDU Session are failed, it is still unclear whether we use PDU Session Response or Failed Item as a response under Case 3-6.

From our understanding, for Case 6, it is natural for the NG-RAN to use PDU Session Failed Item as a response. And for Case 5, since if UL TNL information is modified successfully, the NG-RAN shall at least respond with DL TNL information, it is natural for the NG-RAN to use PDU Session Response Item as a response.

For Case 4 as well as Case 3, i.e. if QoS flows to release IE or/and PDU Session AMBR IE is/are also included (these IEs will always be modified successfully by the NG-RAN as mentioned above), and QoS flows to be added or modified are all failed, since such case should also be regarded as partial success of PDU session resource modify, it would be more appropriate to use PDU Session Response Item as a response.

Proposal 4: Agree that when QoS flows to be added or modified are all failed and QoS flows to release IE or/and PDU Session AMBR IE is/are also included, NG-RAN shall respond to SMF with PDU Session Resource Modify Response Item.

Next is to examine whether the NAS-PDU shall be sent to UE under Case 4-6.

For Case 4, since it is similar to Case 3, the operation of NG-RAN could also be reused, namely NAS-PDU shall be sent to UE under Case 4.

Proposal 5: Agree that when QoS flows to be added or modified are all failed while PDU Session AMBR IE is modified, NG-RAN shall send NAS-PDU to UE.

For Case 5 and Case 6, it is still controversial whether to send NAS-PDU according to the email discussion last meeting, and one company raised the situation of the UE-initiated PDU Session Modify at the NAS level, which can be basically generalised as follows,

If QoS flows to be added or modified are all failed, and only TNL info is successfully modified if included (Case 5 or 6), generally there are two options on the table,

Option 1: NG-RAN sends PDU Session NAS-PDU to UE, and responds to SMF with N2 Response message. And after the reception of N2 Response message, SMF identifies that all QoS flows are failed to be added/modified; however, the original command has been sent to UE, and there’s already misalignment between SMF and UE, so SMF has to send a second command to align the understanding among SMF, NG-RAN and UE.

Option 2: NG-RAN does not send PDU Session NAS-PDU to UE, and responds to SMF with N2 Response message. And after the reception of N2 Response message, SMF identifies that all QoS flows are failed to be added/modified; since NG-RAN does not deliver NAS-PDU which contains NAS modify command to UE, SMF is able to send NAS modify reject message to UE in the following step. Additionally, if the original NAS-PDU also contains IEs other than QoS flow related ones, in order to configure these IEs, SMF may send another NAS modify command to UE.

These two options in fact corresponds to the following two text proposals as discussed during last meeting,

Choose one of the following text proposals for PDU Session modification corrections in the next meeting:

Opt1: If the NAS-PDU IE is received for the PDU session, the NG-RAN node shall pass it to the UE.

Opt2: The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE.

Compared to Option 1, Option 2 will not cause any misalignment between SMF and UE throughout the whole procedure, and Option 2 may only need to send one NAS message over Uu interface which saves overhead over Uu, so we prefer Option 2.

Proposal 6: Agree that when only NG-U UP TNL modification is successful, NG-RAN shall not send NAS-PDU to UE.

Proposal 7: Agree that when PDU Session Resource Modify is totally failed (i.e. NG-RAN reports the PDU Session Resource Failed to Modify List IE contained in the PDU SESSION RESOURCE MODIFY RESPONSE message for the concerned PDU session), NG-RAN shall not send NAS-PDU to UE.

3	Conclusion

This contribution discusses NAS-PDU delivery during PDU Session Resource Modify procedure, and provides following proposals,

Proposal 1: Agree that when the NG-RAN receives a PDU Session Resource Modify Request message which only includes the PDU Session AMBR IE, the NG-RAN shall send NAS-PDU to UE.

Proposal 2: Agree that when only partial QoS flows to be added or modified are failed, NG-RAN shall send NAS-PDU to UE.

Proposal 3: Agree that when QoS flows to be added or modified are all failed while some QoS flow(s) is/are released, NG-RAN shall send NAS-PDU to UE.

Proposal 4: Agree that when QoS flows to be added or modified are all failed and QoS flows to release IE or/and PDU Session AMBR IE is/are also included, NG-RAN shall respond to SMF with PDU Session Resource Modify Response Item.

Proposal 5: Agree that when QoS flows to be added or modified are all failed while PDU Session AMBR IE is modified, NG-RAN shall send NAS-PDU to UE.

Proposal 6: Agree that when only NG-U UP TNL modification is successful, NG-RAN shall not send NAS-PDU to UE.

Proposal 7: Agree that when PDU Session Resource Modify is totally failed (i.e. NG-RAN reports the PDU Session Resource Failed to Modify List IE contained in the PDU SESSION RESOURCE MODIFY RESPONSE message for the concerned PDU session), NG-RAN shall not send NAS-PDU to UE.

References

- R3-215832, SOD on PDU session modification corrections, CMCC

- TS 38.413 v16.7.0