3GPP TSG-RAN3 #114bis-e	R3-221344

17-26 Jan 2022

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1862 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | S1AP Rapporteur Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2021-12-21 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Rapporteur Corrections |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Section 8.7.1.2, 8.10.1.2, 8.10.2.2, 8.11.2.2, change “eNB” to “eNB”, and change “MME” to “MME” in the figures - Section 8.9.2, correct “thie” - Section 8.9.3, correct the font color. - Section 9.1.7.1, correct the reference, i.e. change it to TS 36.331 [16]. - Section 9.2.3.27, correct the typo “NTL” - ASN.1 section: Add ASN1START and ASN1STOP to align with other RAN3 specifications; replace non-ASCII characters in ASN.1, e.g. change “–” to “-” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.1.2.1, 8.7.1.2.2, 8.7.2.2, 8.9.2, 8.9.3, 8.10.1.2, 8.10.2.2, 8.11.2.2, 9.1.7.1, 9.2.3.27, 9.3.2, 9.3.3, 9.3.4, 9.3.5, 9.3.6, 9.3.7, B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: updated with more corrections discovered during RAN3#114bis-e meeting. |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGE >>>>>>

#### 8.7.1.2	Successful Operation

##### 8.7.1.2.1	Reset Procedure Initiated from the MME

Figure 8.7.1.2.1-1: Reset procedure initiated from the MME. Successful operation.

In the event of a failure at the MME, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the eNB.

At reception of the RESET message the eNB shall release all allocated resources on S1 and Uu related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated UE contexts including S1AP ID.

After the eNB has released all assigned S1 resources and the UE S1AP IDs for all indicated UE associations which can be used for new UE-associated logical S1-connections over the S1 interface, the eNB shall respond with the RESET ACKNOWLEDGE message. The eNB does not need to wait for the release of radio resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical S1-connection list IE, then:

-	The eNB shall use the MME UE S1AP ID IE and/or the eNB UE S1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The eNB shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE. The UE-associated logical S1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical S1-connections. Empty UE-associated logical S1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the MME UE S1AP ID IE is included in the UE-associated logical S1-connection Item IE for a UE association, the eNB shall include the MME UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the eNB UE S1AP ID IE is included in the UE-associated logical S1-connection Item IE for a UE association, the eNB shall include the eNB UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same S1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

##### 8.7.1.2.2	Reset Procedure Initiated from the E-UTRAN

Figure 8.7.1.2.2-1: Reset procedure initiated from the E-UTRAN. Successful operation.

In the event of a failure at the eNB, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the MME.

At reception of the RESET message the MME shall release all allocated resources on S1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the S1AP ID for the indicated UE associations.

After the MME has released all assigned S1 resources and the UE S1AP IDs for all indicated UE associations which can be used for new UE-associated logical S1-connections over the S1 interface, the MME shall respond with the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical S1-connection list IE, then:

-	The MME shall use the MME UE S1AP ID IE and/or the eNB UE S1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The MME shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE. The UE-associated logical S1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical S1-connections. Empty UE-associated logical S1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the MME UE S1AP ID IE is included in the UE-associated logical S1-connection Item IE for a UE association, the MME shall include the MME UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the eNB UE S1AP ID IE is included in a UE-associated logical S1-connection Item IE for a UE association, the MME shall include the eNB UE S1AP ID IE in the corresponding UE-associated logical S1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same S1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

#### 8.7.1.3	Abnormal Conditions

##### 8.7.1.3.1	Abnormal Condition at the EPC

If the RESET message includes the UE-associated logical S1-connection list IE, but neither the MME UE S1AP ID IE nor the eNB UE S1AP ID IE is present for a UE-associated logical S1-connection Item IE, then the MME shall ignore the UE-associated logical S1-connection Item IE. The MME may return the empty UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE in the RESET ACKNOWLEDGE message.

##### 8.7.1.3.2	Abnormal Condition at the E-UTRAN

If the RESET message includes the UE-associated logical S1-connection list IE, but neither the MME UE S1AP ID IE nor the eNB UE S1AP ID IE is present for a UE-associated logical S1-connection Item IE, then the eNB shall ignore the UE-associated logical S1-connection Item IE. The eNB may return the empty UE-associated logical S1-connection Item IE in the UE-associated logical S1-connection list IE in the RESET ACKNOWLEDGE message.

##### 8.7.1.3.3	Crossing of Reset Messages

If a Reset procedure is ongoing in the eNB and the eNB receives a RESET message from the peer entity on the same S1 interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received RESET message, the eNB shall respond with the RESET ACKNOWLEDGE message as described in 8.7.1.2.1.

If a Reset procedure is ongoing in the MME and the MME receives a RESET message from the peer entity on the same S1 interface related to one or several UE associations previously requested to be reset, indicated explicitly or implicitly in the received RESET message, the MME shall respond with the RESET ACKNOWLEDGE message as described in 8.7.1.2.2.

### 8.7.2	Error Indication

#### 8.7.2.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.7.2.2	Successful Operation

Figure 8.7.2.2-1: Error Indication procedure, MME originated. Successful operation.

Figure 8.7.2.2-2: Error Indication procedure, eNB originated. Successful operation.

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the MME UE S1AP ID IE and the eNB UE S1AP ID IE shall be included in the ERROR INDICATION message. If one or both of MME UE S1AP ID IE and the eNB UE S1AP ID IE are not correct, the cause shall be set to appropriate value, e.g., “Unknown or already allocated MME UE S1AP ID”, “Unknown or already allocated eNB UE S1AP ID” or “Unknown or inconsistent pair of UE S1AP ID”.

<<<<<< NEXT CHANGE >>>>>>

### 8.9.2	Successful Operation

Figure 8.9.2-1: UE Capability Info Indication procedure. Successful operation.

The eNB controlling a UE-associated logical S1-connection initiates the procedure by sending a UE CAPABILITY INFO INDICATION message to the MME including the UE capability information. The UE CAPABILITY INFO INDICATION message may also include paging specific UE capability information within the UE Radio Capability for Paging IE. The UE capability information received by the MME shall replace previously stored corresponding UE capability information in the MME for the UE, as described in TS 23.401 [11].

If UE CAPABILITY INFO INDICATION message contains the LTE-M indication IE, the MME shall, if supported, store this information in the UE context and use it according to TS 23.401 [11].

If the UE indicates the support for UE Application Layer Measurement, the eNB shall if supported include the UE Application Layer Measurement Capability IE in the UE CAPABILITY INFO INDICATION message. The MME shall, if supported, store and use this information when initiating UE Application Layer Measurement.

If UE CAPABILITY INFO INDICATION message contains the UE Radio Capability – NR Format IE, the MME shall, if supported, use it according to TS 23.401 [11].

If the UE RADIO CAPABILITY INFO INDICATION message includes the UE Radio Capability for Paging IE and the UE Radio Capability for Paging – NR Format IE, the MME shall, if supported, use it according to TS 23.401 [11].

### 8.9.3	Abnormal Conditions

If the UE RADIO CAPABILITY INFO INDICATION message includes the UE Radio Capability for Paging – NR Format IE without the UE Radio Capability for Paging IE, the MME shall consider it as a logical error and act as described in subclause 10.4.

<<<<<< NEXT CHANGE >>>>>>

## 8.10	Trace Procedures

### 8.10.1	Trace Start

#### 8.10.1.1	General

The purpose of the Trace Start procedure is to allow the MME to request the eNB to initiate a trace function for a UE. The procedure uses UE-associated signalling. If no UE-associated logical S1-connection exists, the UE-associated logical S1-connection shall be established as part of the procedure.

#### 8.10.1.2	Successful Operation

Figure 8.10.1.2-1: Trace Start procedure.

The MME initiates the procedure by sending a TRACE START message. On receipt of a TRACE START message, the eNB shall initiate the requested trace function as described in TS 32.422 [10].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT and Trace”, the eNB shall if supported, initiate the requested trace session and MDT session as described in TS 32.422 [10].

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT”, the target eNB shall, if supported, initiate the requested MDT session as described in TS 32.422 [10] and the target eNB shall ignore Interfaces To Trace IE, and Trace Depth IE.

If the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, the eNB shall, if supported, store this information and take it into account in the requested MDT session.

If the Trace Activation IE is included in the TRACE START message which includes the MDT Activation IE set to “Immediate MDT Only”, “Logged MDT only” or “Logged MBSFN MDT” and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, the eNB may use it to propagate the MDT Configuration as described in TS 37.320 [31].

If the Trace Activation IE includes the MBSFN-ResultToLog IE, within the MDT Configuration IE, the eNB shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [31].

If the Trace Activation IE includes the MBSFN-AreaId IE in the MBSFN-ResultToLog IE, within the MDT Configuration IE, the eNB shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [31].

If the Trace Activation IE includes the UE Application layer measurement configuration IE, the eNB shall, if supported, initiate the requested trace session and QoE Measurement Collection function as described in TS 36.300 [14].

If the Trace Activation IE includes the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, the eNB shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [31].

If the Trace Activation IE includes the WLAN Measurement Configuration IE, within the MDT Configuration IE, the eNB shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [31].

If the Trace Activation IE includes the MDT Configuration NR IE, the eNB shall, if supported, store and forward MDT Configuration NR IE to the SgNB, if the eNB has configured EN-DC for the UE.

Interactions with other procedures:

If the eNB is not able to initiate the trace session due to ongoing handover of the UE to another eNB, the eNB shall initiate a Trace Failure Indication procedure with the appropriate cause value.

### 8.10.2	Trace Failure Indication

#### 8.10.2.1	General

The purpose of the Trace Failure Indication procedure is to allow the eNB to inform the MME that a Trace Start procedure or a Deactivate Trace procedure has failed due to an interaction with a handover procedure. The procedure uses UE-associated signalling.

#### 8.10.2.2	Successful Operation

Figure 8.10.2.2-1: Trace Failure Indication procedure.

The eNB initiates the procedure by sending a TRACE FAILURE INDICATION message. Upon reception of the TRACE FAILURE INDICATION message, the MME shall take appropriate actions based on the failure reason indicated by the Cause IE.

<<<<<< NEXT CHANGE >>>>>>

#### 8.11.2.2	Successful Operation

Figure 8.11.2.2-1: Location Report Failure Indication procedure.

Upon reception of the LOCATION REPORT FAILURE INDICATION message the MME shall take appropriate actions based on the failure reason indicated by the Cause IE.

#### <<<<<< NEXT CHANGE >>>>>>

#### 9.1.7.1	INITIAL UE MESSAGE

This message is sent by the eNB to transfer the initial layer 3 message to the MME over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| TAI | M |  | 9.2.3.16 | Indicating the Tracking Area from which the UE has sent the NAS message. | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 | Indicating the E-UTRAN CGI from which the UE has sent the NAS message. | YES | ignore |
| RRC Establishment Cause | M |  | 9.2.1.3a |  | YES | ignore |
| S-TMSI | O |  | 9.2.3.6 |  | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| GUMMEI | O |  | 9.2.3.9 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | reject |
| GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating GW Transport Layer Address if the GW is collocated with eNB. | YES | ignore |
| Relay Node Indicator | O |  | 9.2.1.79 | Indicating a relay node. | YES | reject |
| GUMMEI Type | O |  | ENUMERATED (native, mapped, …, mappedFrom5G) |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating SIPTO L-GW Transport Layer Address if the SIPTO L-GW is collocated with eNB. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |
| MME Group ID | O |  | 9.2.3.44 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |
| CE-mode-B Support Indicator | O |  | 9.2.1.118 |  | YES | ignore |
| DCN ID | O |  | INTEGER (0..65535) |  | YES | ignore |
| Coverage Level | O |  | ENUMERATED (extendedcoverage, …) |  | YES | ignore |
| UE Application Layer Measurement Capability | O |  | BIT STRING (SIZE(8)) | Each bit in the bitmap indicates an UE Application layer measurement capability, refer to TS 36.331 [16]. Bit 0 = QoE Measurement for streaming service Bit 1 = QoE Measurement for MTSI service Value ‘1’ indicates “Capable” and value ‘0’ indicates “not Capable”. Unused bits are reserved for future use. | YES | ignore |
| EDT Session | O |  | ENUMERATED (true, …) |  | YES |  |
| IAB Node Indication | O |  | ENUMERATED (true, ...) | Indication of an IAB-node. | YES | reject |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.3.27	SON Information

This IE identifies the nature of the configuration information transferred, i.e., a request, a reply or a report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE SON Information | M |  |  |  |  |  |
| >SON Information Request |  |  |  |  |  |  |
| >>SON Information Request | M |  | ENUMERATED(X2 TNL Configuration Info, …, Time synchronisation Info, Activate Muting, Deactivate Muting) | In the current version of the specification only "X2 TNL Configuration Info" is applicable for EN-DC. | - |  |
| >SON Information Reply |  |  |  |  |  |  |
| >>SON Information Reply | M |  | 9.2.3.28 |  | - |  |
| >SON Information Report |  |  |  |  |  |  |
| >>SON Information Report | M |  | 9.2.3.39 |  | YES | ignore |

<<<<<< NEXT CHANGE >>>>>>

### 9.3.2	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

S1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Descriptions (0)}

<Unaffected part is omitted>

mMEEarlyStatusTransfer S1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MMEEarlyStatusTransfer

PROCEDURE CODE			id-MMEEarlyStatusTransfer

CRITICALITY				ignore

}

END

-- ASN1STOP

<<<<<< NEXT CHANGE >>>>>>

### 9.3.3	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

<Unaffected part is omitted>

##### -- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{PagingIEs}},

...

}

PagingIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue			CRITICALITY ignore	TYPE UEIdentityIndexValue				PRESENCE mandatory}|

{ ID id-UEPagingID						CRITICALITY ignore	TYPE UEPagingID							PRESENCE mandatory}|

{ ID id-pagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional}|

{ ID id-CNDomain						CRITICALITY ignore	TYPE CNDomain							PRESENCE mandatory}|

{ ID id-TAIList							CRITICALITY ignore	TYPE TAIList							PRESENCE mandatory}|

{ ID id-CSG-IdList						CRITICALITY ignore	TYPE CSG-IdList							PRESENCE optional}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional}|

-- Extension for Release 13 to support Paging Optimisation and Coverage Enhancement paging --

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional}|

{ ID id-Paging-eDRXInformation			CRITICALITY ignore	TYPE Paging-eDRXInformation				PRESENCE optional}|

{ ID id-extended-UEIdentityIndexValue	CRITICALITY ignore	TYPE Extended-UEIdentityIndexValue	PRESENCE optional}|

{ ID id-NB-IoT-Paging-eDRXInformation	CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInformation	PRESENCE optional}|

{ ID id-NB-IoT-UEIdentityIndexValue		CRITICALITY ignore	TYPE NB-IoT-UEIdentityIndexValue		PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted					PRESENCE optional}|

{ ID id-DataSize						CRITICALITY ignore	TYPE DataSize							PRESENCE optional}|

{ ID id-WUS-Assistance-Information		CRITICALITY ignore	TYPE WUS-Assistance-Information			PRESENCE optional}|

{ ID id-NB-IoT-PagingDRX				CRITICALITY ignore	TYPE NB-IoT-PagingDRX					PRESENCE optional},

...

}

<Unaffected part is omitted>

UEContextReleaseComplete-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-MME-UE-S1AP-ID								CRITICALITY ignore	TYPE MME-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-eNB-UE-S1AP-ID								CRITICALITY ignore	TYPE ENB-UE-S1AP-ID						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional}|

-- Extension for Release 12 to support User Location Information --

{ ID id-UserLocationInformation						CRITICALITY ignore	TYPE UserLocationInformation				PRESENCE optional}|

-- Extension for Release 13 to support Paging Optimisation

{ ID id-InformationOnRecommendedCellsAndENBsForPaging	CRITICALITY ignore	TYPE InformationOnRecommendedCellsAndENBsForPaging		PRESENCE optional}|

-- Extension for Release 13 to support coverage enhancement paging --

{ ID id-CellIdentifierAndCELevelForCECapableUEs		CRITICALITY ignore	TYPE CellIdentifierAndCELevelForCECapableUEs	PRESENCE optional}|

{ ID id-SecondaryRATDataUsageReportList				CRITICALITY ignore	TYPE SecondaryRATDataUsageReportList		PRESENCE optional }|

{ ID id-TimeSinceSecondaryNodeRelease				CRITICALITY ignore	TYPE TimeSinceSecondaryNodeRelease				PRESENCE optional },

...

}

<Unaffected part is omitted>

UERadioCapabilityIDMappingResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UERadioCapabilityID				CRITICALITY reject	TYPE UERadioCapabilityID				PRESENCE mandatory	}|

{ ID id-UERadioCapability					CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

END

-- ASN1STOP

<<<<<< NEXT CHANGE >>>>>>

### 9.3.4	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

<Unaffected part is omitted>

EN-DCSONConfigurationTransfer ::= SEQUENCE {

transfertype				EN-DCSONTransferType,

sONInformation					SONInformation,

x2TNLConfigInfo					X2TNLConfigurationInfo 	OPTIONAL,

-- This IE shall be present if the SON Information IE contains the SON Information Request IE and the SON Information Request IE is set to "X2TNL Configuration Info" --

iE-Extensions			ProtocolExtensionContainer { {EN-DCSONConfigurationTransfer-ExtIEs} }			OPTIONAL,

...

}

<Unaffected part is omitted>

E-RABQoSParameters-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extended for introduction of downlink and uplink packet loss rate for enhanced Voice performance --

{ ID id-DownlinkPacketLossRate		CRITICALITY ignore	EXTENSION Packet-LossRate		PRESENCE optional}|

{ ID id-UplinkPacketLossRate			CRITICALITY ignore	EXTENSION Packet-LossRate		PRESENCE optional},

...

}

<Unaffected part is omitted>

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6delay-threshold	M6delay-threshold		OPTIONAL,

-- This IE shall be present if the M6 Links to log IE is set to "uplink" or to "both-uplink-and-downlink" --

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

<Unaffected part is omitted>

SONConfigurationTransfer-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Release 10 to transfer the IP addresses of the eNB initiating the ANR action --

{ID id-x2TNLConfigurationInfo		CRITICALITY ignore	EXTENSION X2TNLConfigurationInfo				PRESENCE conditional

-- This IE shall be present if the SON Information IE contains the SON Information Request IE and the SON Information Request IE is set to "X2TNL Configuration Info" --}|

-- Extension for Release 12 to transfer information concerning the source cell of synchronisation and the aggressor cell --

{ID id-Synchronisation-Information	CRITICALITY ignore	EXTENSION SynchronisationInformation			PRESENCE conditional

-- This IE shall be present if the SON Information IE contains the SON Information Request IE set to " Activate Muting " --},

...

}

<Unaffected part is omitted>

TraceActivation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

-- Extension for Rel-10 to support MDT --

{ ID id-MDTConfiguration	CRITICALITY ignore	EXTENSION MDT-Configuration		PRESENCE optional }|

-- Extension for Rel-15 to support QMC --

{ ID id-UEAppLayerMeasConfig	CRITICALITY ignore	EXTENSION UEAppLayerMeasConfig		PRESENCE optional }|

{ ID id-MDTConfigurationNR		CRITICALITY ignore	EXTENSION MDT-ConfigurationNR		PRESENCE optional }|

{ ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URI-Address			PRESENCE optional	},

...

}

<Unaffected part is omitted>

#### -- Y

#### -- Z

END

-- ASN1STOP

<<<<<< NEXT CHANGE >>>>>>

### 9.3.5	Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

S1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-CommonDataTypes (3) }

<Unaffected part is omitted>

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessfull-outcome }

END

-- ASN1STOP

<<<<<< NEXT CHANGE >>>>>>

### 9.3.6	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

<Unaffected part is omitted>

id-UERadioCapabilityForPaging-NR-Format 			ProtocolIE-ID ::= 327

END

-- ASN1STOP

<<<<<< NEXT CHANGE >>>>>>

### 9.3.7	Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

S1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Containers (5) }

<Unaffected part is omitted>

PrivateIE-Field {S1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					S1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			S1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				S1AP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

-- ASN1STOP

<<<<<< NEXT CHANGE >>>>>>

# B.2	ASN.1 definition

-- ASN1START

-- **************************************************************

--

-- IE definitions for the SON Transfer application

-- The IEs in this ASN.1 module shall be defined and encoded

-- using the same rules as applicable for the S1AP-IEs module.

--

-- **************************************************************

SonTransfer-IEs

<Unaffected part is omitted>

maxnoofIRATReportingCells					INTEGER ::= 128

maxnoofcandidateCells						INTEGER ::= 16

maxnoofCellineNB							INTEGER ::= 256

END

-- ASN1STOP

<<<<<< END OF CHANGE >>>>>>