# 3GPP TSG-RAN WG3 Meeting #115-e														R3-222763

# E-meeting, 21 February – 3 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR | 0009 | rev | 5 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of dynamic ACL during dual connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current ACL function may cause data discarding at the receiving node in case of dual connectivity if the source IP address in the IP packet header of forwarding packets is unknown to the ng-eNB-DU, i.e. in case of MN-terminated SCG bearers and SN-terminated MCG bearers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the source IP address information in case of MN-terminated SCG bearers and SN-terminated MCG bearers. . Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has limited impact under funtional point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data packets for MN-terminated SCG bearers and SN-terminated MCG bearers will be discarded by the ng-eNB-DU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 9.3.1.34, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS38.423CR0691 TS36.423CR1639 TS38.413CR0619 TS36.413CR1838 TS38.473CR0778 TS38.463CR0651 |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin>>>>>>>>>>>>>>>>>>>>

8.3.1	UE Context Setup

8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, and DRB configuration. The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The ng-eNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the ng-eNB-DU. If the ng-eNB-DU succeeds to establish the UE context, it replies to the ng-eNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical W1-connection exists, the UE-associated logical W1-connection shall be established as part of the procedure.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the ng-eNB-DU shall act as specified in TS 38.401 [4].

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the ng-eNB-DU shall use the provided value from the ng-eNB-CU.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the ng-eNB-DU shall act as specified in TS 38.401 [4].

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the ng-eNB-DU shall act as specified in TS 38.401 [4].

The ng-eNB-DU shall report to the ng-eNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs and SRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

When the ng-eNB-DU reports the unsuccessful establishment of a DRB or SRB, the cause value should be precise enough to enable the ng-eNB-CU to know the reason for the unsuccessful establishment.

The ng-eNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the ng-eNB-CU shall regard the corresponding SCell(s) failed to be established with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the ng-eNB-CU shall consider that the C-RNTI has been allocated by the ng-eNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the ng-eNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the ng-eNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [8].

The ng-eNB-DU shall store the received ng-eNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the  PDCP Terminating Node DL Transport Layer Address IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, then the ng-eNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

9.3.1.34	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow or to a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.38 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.36 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.37 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.35 | This IE shall be present for GBR QoS Flows only. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [8]. This IE applies to non-GBR flows only and shall be ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [8]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| PDCP Terminating Node DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers and SN-terminated MCG bearers. |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change>>>>>>>>>>>>>>>>>>>>

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

W1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM W1AP-CommonDataTypes

Reset,

ResetAcknowledge,

W1SetupRequest,

W1SetupResponse,

W1SetupFailure,

NGENBDUConfigurationUpdate,

NGENBDUConfigurationUpdateAcknowledge,

NGENBDUConfigurationUpdateFailure,

NGENBCUConfigurationUpdate,

NGENBCUConfigurationUpdateAcknowledge,

NGENBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

NGENBDUResourceCoordinationRequest,

NGENBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

NGENBDUStatusIndication,

UEContextModificationRefuse

FROM W1AP-PDU-Contents

id-Reset,

id-W1Setup,

id-ngeNBDUConfigurationUpdate,

id-ngeNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-NGENBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-NGENBDUStatusIndication

FROM W1AP-Constants

ProtocolIE-SingleContainer{},

W1AP-PROTOCOL-IES

FROM W1AP-Containers;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

W1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

W1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome

}

InitiatingMessage ::= SEQUENCE {

procedureCode	W1AP-ELEMENTARY-PROCEDURE.&procedureCode		({W1AP-ELEMENTARY-PROCEDURES}),

criticality		W1AP-ELEMENTARY-PROCEDURE.&criticality			({W1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			W1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({W1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	W1AP-ELEMENTARY-PROCEDURE.&procedureCode		({W1AP-ELEMENTARY-PROCEDURES}),

criticality		W1AP-ELEMENTARY-PROCEDURE.&criticality			({W1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			W1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({W1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	W1AP-ELEMENTARY-PROCEDURE.&procedureCode		({W1AP-ELEMENTARY-PROCEDURES}),

criticality		W1AP-ELEMENTARY-PROCEDURE.&criticality			({W1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			W1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({W1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

W1AP-ELEMENTARY-PROCEDURES W1AP-ELEMENTARY-PROCEDURE ::= {

W1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

W1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

W1AP-ELEMENTARY-PROCEDURES-CLASS-1 W1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

w1Setup							|

ngeNBDUConfigurationUpdate		|

ngeNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning			|

pWSCancel					|

ngeNBDUResourceCoordination		,

...

}

W1AP-ELEMENTARY-PROCEDURES-CLASS-2 W1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication					|

uEContextReleaseRequest			|

dLRRCMessageTransfer			|

uLRRCMessageTransfer			|

uEInactivityNotification		|

privateMessage					|

initialULRRCMessageTransfer		|

paging							|

notify							|

pWSRestartIndication				|

pWSFailureIndication			|

ngeNBDUStatusIndication			,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

reset W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

w1Setup W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		W1SetupRequest

SUCCESSFUL OUTCOME		W1SetupResponse

UNSUCCESSFUL OUTCOME	W1SetupFailure

PROCEDURE CODE			id-W1Setup

CRITICALITY				reject

}

ngeNBDUConfigurationUpdate W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGENBDUConfigurationUpdate

SUCCESSFUL OUTCOME		NGENBDUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME		NGENBDUConfigurationUpdateFailure

PROCEDURE CODE			id-ngeNBDUConfigurationUpdate

CRITICALITY				reject

}

ngeNBCUConfigurationUpdate W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGENBCUConfigurationUpdate

SUCCESSFUL OUTCOME		NGENBCUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME		NGENBCUConfigurationUpdateFailure

PROCEDURE CODE			id-ngeNBCUConfigurationUpdate

CRITICALITY				reject

}

uEContextSetup W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSetupRequest

SUCCESSFUL OUTCOME		UEContextSetupResponse

UNSUCCESSFUL OUTCOME		UEContextSetupFailure

PROCEDURE CODE			id-UEContextSetup

CRITICALITY				reject

}

uEContextRelease W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextModification W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME		UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextModificationRequired W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequired

SUCCESSFUL OUTCOME		UEContextModificationConfirm

UNSUCCESSFUL OUTCOME		UEContextModificationRefuse

PROCEDURE CODE			id-UEContextModificationRequired

CRITICALITY				reject

}

writeReplaceWarning W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

pWSCancel W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

errorIndication W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

uEContextReleaseRequest W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

initialULRRCMessageTransfer W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialULRRCMessageTransfer

PROCEDURE CODE			id-InitialULRRCMessageTransfer

CRITICALITY				ignore

}

dLRRCMessageTransfer W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLRRCMessageTransfer

PROCEDURE CODE			id-DLRRCMessageTransfer

CRITICALITY				ignore

}

uLRRCMessageTransfer W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULRRCMessageTransfer

PROCEDURE CODE			id-ULRRCMessageTransfer

CRITICALITY				ignore

}

uEInactivityNotification  W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEInactivityNotification

PROCEDURE CODE			id-UEInactivityNotification

CRITICALITY				ignore

}

ngeNBDUResourceCoordination W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGENBDUResourceCoordinationRequest

SUCCESSFUL OUTCOME		NGENBDUResourceCoordinationResponse

PROCEDURE CODE			id-NGENBDUResourceCoordination

CRITICALITY				reject

}

privateMessage W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

paging W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

notify W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Notify

PROCEDURE CODE			id-Notify

CRITICALITY				ignore

}

pWSRestartIndication W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

pWSFailureIndication W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

ngeNBDUStatusIndication 	W1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NGENBDUStatusIndication

PROCEDURE CODE			id-NGENBDUStatusIndication

CRITICALITY				ignore

}

END

-- ASN1STOP

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for W1AP.

--

-- **************************************************************

W1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

C-RNTI,

CriticalityDiagnostics,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DUtoCURRCInformation,

EUTRANQoS,

NGENB-CU-UE-W1AP-ID,

NGENB-DU-UE-W1AP-ID,

NGENB-DU-ID,

NGENB-DU-Served-Cells-Item,

NGENB-DU-System-Information,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

NotificationControl,

EUTRANCGI,

EUTRANPCI,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TransactionID,

TransmissionActionIndicator,

DUtoCURRCContainer,

PagingCell-Item,

UEIdentityIndexValue,

UE-associatedLogicalW1-ConnectionItem,

PagingDRX,

PagingIdentity,

PagingOrigin,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

E-UTRAN-CGI-List-For-Restart-Item,

PWS-Failed-E-UTRAN-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

NotificationInformation,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

BitRate,

NGENBDUOverloadInformation,

ResourceCoordinationTransferInformation,

IgnoreResourceCoordinationRequestContainer,

SCGIndicator

FROM W1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

W1AP-PRIVATE-IES,

W1AP-PROTOCOL-EXTENSION,

W1AP-PROTOCOL-IES,

W1AP-PROTOCOL-IES-PAIR

FROM W1AP-Containers

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-NotificationInformation,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Deactivated-List,

id-ConfirmedUEID,

id-C-RNTI,

id-CUtoDURRCInformation,

id-CriticalityDiagnostics,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-List,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-List,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ngeNB-CU-UE-W1AP-ID,

id-ngeNB-DU-UE-W1AP-ID,

id-ngeNB-DU-ID,

id-ngeNB-DU-Served-Cells-List,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-oldngeNB-DU-UE-W1AP-ID,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetupMod-List,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-List,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-SpCell-ID,

id-SRBID,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-List,

id-SRBs-Setup-List,

id-SRBs-SetupMod-List,

id-TransactionID,

id-TransmissionActionIndicator,

id-UE-associatedLogicalW1-ConnectionListResAck,

id-DUtoCURRCContainer,

id-EUTRANCGI,

id-PagingCell-List,

id-PagingDRX,

id-UEIdentityIndexValue,

id-PagingIdentity,

id-PagingOrigin,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-Broadcast-Completed-List,

id-Broadcast-To-Be-Cancelled-List,

id-Cells-Broadcast-Cancelled-List,

id-E-UTRAN-CGI-List-For-Restart-List,

id-PWS-Failed-E-UTRAN-CGI-List,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-RequestType,

id-ServingPLMN,

id-NGENB-DU-UE-AMBR-UL,

id-NGENBDUOverloadInformation,

id-ResourceCoordinationTransferInformation,

id-IgnoreResourceCoordinationRequestContainer,

id-SCGIndicator,

maxCellinngeNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualW1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs

FROM W1AP-Constants;

-- **************************************************************

--

-- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID	CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-Cause			CRITICALITY ignore	TYPE Cause				PRESENCE mandatory	}|

{ ID id-ResetType		CRITICALITY reject	TYPE ResetType			PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

w1-Interface					ResetAll,

partOfW1-Interface				UE-associatedLogicalW1-ConnectionListRes,

choice-extension				ProtocolIE-SingleContainer { { ResetType-ExtIEs} }

}

ResetType-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalW1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualW1ConnectionsToReset)) OF UE-associatedLogicalW1-ConnectionItem

-- **************************************************************

--

-- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-UE-associatedLogicalW1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalW1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalW1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualW1ConnectionsToReset)) OF UE-associatedLogicalW1-ConnectionItem

-- **************************************************************

--

-- ERROR INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Error Indication

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ErrorIndicationIEs}},

...

}

ErrorIndicationIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory}|

{ ID id-ngeNB-CU-UE-W1AP-ID	CRITICALITY ignore	TYPE NGENB-CU-UE-W1AP-ID	PRESENCE optional	}|

{ ID id-ngeNB-DU-UE-W1AP-ID	CRITICALITY ignore	TYPE NGENB-DU-UE-W1AP-ID	PRESENCE optional	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- W1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- W1 Setup Request

--

-- **************************************************************

W1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {W1SetupRequestIEs} },

...

}

W1SetupRequestIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-ngeNB-DU-ID			CRITICALITY reject	TYPE NGENB-DU-ID		PRESENCE mandatory	}|

{ ID id-ngeNB-DU-Served-Cells-List		CRITICALITY reject	TYPE NGENB-DU-Served-Cells-List	PRESENCE optional	},

...

}

NGENB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU)) OF NGENB-DU-Served-Cells-Item

-- **************************************************************

--

-- W1 Setup Response

--

-- **************************************************************

W1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {W1SetupResponseIEs} },

...

}

W1SetupResponseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List	CRITICALITY reject	TYPE Cells-to-be-Activated-List		PRESENCE optional	},

...

}

Cells-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Cells-to-be-Activated-List-Item

-- **************************************************************

--

-- W1 Setup Failure

--

-- **************************************************************

W1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {W1SetupFailureIEs} },

...

}

W1SetupFailureIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- NGENB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- NGENB-DU CONFIGURATION UPDATE

--

-- **************************************************************

NGENBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {NGENBDUConfigurationUpdateIEs} },

...

}

NGENBDUConfigurationUpdateIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List	CRITICALITY reject	TYPE Served-Cells-To-Add-List		PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List	CRITICALITY reject	TYPE Served-Cells-To-Modify-List					PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List	CRITICALITY reject	TYPE Served-Cells-To-Delete-List					PRESENCE optional	}|

{ ID id-Cells-Status-List	CRITICALITY reject	TYPE Cells-Status-List	PRESENCE optional	}|

{ ID id-ngeNB-DU-ID			CRITICALITY reject	TYPE NGENB-DU-ID		PRESENCE optional	},

...

}

Served-Cells-To-Add-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Served-Cells-To-Add-Item

Served-Cells-To-Modify-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Served-Cells-To-Modify-Item

Served-Cells-To-Delete-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Served-Cells-To-Delete-Item

Cells-Status-List	::= SEQUENCE (SIZE(0.. maxCellinngeNBDU))	OF Cells-Status-Item

-- **************************************************************

--

-- NGENB-DU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

NGENBDUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {NGENBDUConfigurationUpdateAcknowledgeIEs} },

...

}

NGENBDUConfigurationUpdateAcknowledgeIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE Cells-to-be-Activated-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List		CRITICALITY reject	TYPE Cells-to-be-Deactivated-List			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- NGENB-DU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

NGENBDUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {NGENBDUConfigurationUpdateFailureIEs} },

...

}

NGENBDUConfigurationUpdateFailureIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- NGENB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- NGENB-CU CONFIGURATION UPDATE

--

-- **************************************************************

NGENBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { NGENBCUConfigurationUpdateIEs} },

...

}

NGENBCUConfigurationUpdateIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE	 Cells-to-be-Activated-List				PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List	CRITICALITY reject	TYPE	 Cells-to-be-Deactivated-List			PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Cells-to-be-Deactivated-List-Item

-- **************************************************************

--

-- NGENB-CU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

NGENBCUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { NGENBCUConfigurationUpdateAcknowledgeIEs} },

...

}

NGENBCUConfigurationUpdateAcknowledgeIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-Cells-Failed-to-be-Activated-List	CRITICALITY reject	TYPE Cells-Failed-to-be-Activated-List			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

Cells-Failed-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Cells-Failed-to-be-Activated-List-Item

-- **************************************************************

--

-- NGENB-CU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

NGENBCUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { NGENBCUConfigurationUpdateFailureIEs} },

...

}

NGENBCUConfigurationUpdateFailureIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- NGENB-DU RESOURCE COORDINATION REQUEST

--

-- **************************************************************

NGENBDUResourceCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{NGENBDUResourceCoordinationRequest-IEs}},

...

}

NGENBDUResourceCoordinationRequest-IEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID	CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-RequestType	CRITICALITY reject	TYPE RequestType			PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReq-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReq-Container		PRESENCE mandatory}|

{ ID id-IgnoreResourceCoordinationRequestContainer	CRITICALITY reject	TYPE IgnoreResourceCoordinationRequestContainer		PRESENCE optional },

...

}

-- **************************************************************

--

-- NGENBDU RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

NGENBDUResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{NGENBDUResourceCoordinationResponse-IEs}},

...

}

NGENBDUResourceCoordinationResponse-IEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReqAck-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReqAck-Container		PRESENCE mandatory},

...

}

-- **************************************************************

--

-- UE Context Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID						CRITICALITY ignore	TYPE NGENB-DU-UE-W1AP-ID								PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE EUTRANCGI											PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex									PRESENCE mandatory	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation							PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle										PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer									PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional	}|

{ ID id-NGENB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate										PRESENCE conditional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	},

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF Candidate-SpCell-Item

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF SCell-ToBeSetup-Item

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-ToBeSetup-Item

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-ToBeSetup-Item

-- **************************************************************

--

-- UE CONTEXT SETUP RESPONSE

--

-- **************************************************************

UEContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupResponseIEs} },

...

}

UEContextSetupResponseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE mandatory }|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-DRBs-Setup-List							CRITICALITY ignore	TYPE DRBs-Setup-List								PRESENCE optional	}|

{ ID id-SRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE SRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-SCell-FailedtoSetup-List				CRITICALITY ignore	TYPE SCell-FailedtoSetup-List					PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-SRBs-Setup-List							CRITICALITY ignore	TYPE SRBs-Setup-List									PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

DRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-Setup-Item

SRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-FailedToBeSetup-Item

DRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-FailedToBeSetup-Item

SCell-FailedtoSetup-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF SCell-FailedtoSetup-Item

SRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-Setup-Item

-- **************************************************************

--

-- UE CONTEXT SETUP FAILURE

--

-- **************************************************************

UEContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupFailureIEs} },

...

}

UEContextSetupFailureIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID			CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID			PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID			CRITICALITY ignore	TYPE NGENB-DU-UE-W1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-Potential-SpCell-List		CRITICALITY ignore	TYPE Potential-SpCell-List		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

Potential-SpCell-List::= SEQUENCE (SIZE(0..maxnoofPotentialSpCells)) OF Potential-SpCell-Item

-- **************************************************************

--

-- UE Context Release Request ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Context Release Request

--

-- **************************************************************

UEContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEContextReleaseRequestIEs}},

...

}

UEContextReleaseRequestIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID	CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID	PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID	CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID	PRESENCE mandatory	}|

{ ID id-Cause				CRITICALITY ignore	TYPE Cause			PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- UE Context Release (ngeNB-CU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE CONTEXT RELEASE COMMAND

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCommandIEs} },

...

}

UEContextReleaseCommandIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-RRCContainer						CRITICALITY ignore	TYPE RRCContainer					PRESENCE optional	}|

{ ID id-SRBID							CRITICALITY ignore	TYPE SRBID							PRESENCE optional		}|

{ ID id-oldngeNB-DU-UE-W1AP-ID			CRITICALITY ignore	TYPE NGENB-DU-UE-W1AP-ID				PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCompleteIEs} },

...

}

UEContextReleaseCompleteIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID			CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID			PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID			CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE EUTRANCGI											PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle										PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation							PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator		PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer									PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 						PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-NGENB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate										PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-SCGIndicator								CRITICALITY ignore	TYPE SCGIndicator									PRESENCE optional },

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF SCell-ToBeSetupMod-Item

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF SCell-ToBeRemoved-Item

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-ToBeSetupMod-Item

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-ToBeSetupMod-Item

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-ToBeModified-Item

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-ToBeReleased-Item

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-ToBeReleased-Item

-- **************************************************************

--

-- UE CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE optional}|

{ ID id-DRBs-SetupMod-List						CRITICALITY ignore	TYPE DRBs-SetupMod-List								PRESENCE optional}|

{ ID id-DRBs-Modified-List						CRITICALITY ignore	TYPE DRBs-Modified-List								PRESENCE optional}|

{ ID id-SRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE SRBs-FailedToBeSetupMod-List				PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE DRBs-FailedToBeSetupMod-List				PRESENCE optional	}|

{ ID id-SCell-FailedtoSetupMod-List				CRITICALITY ignore	TYPE SCell-FailedtoSetupMod-List				PRESENCE optional	}|

{ ID id-DRBs-FailedToBeModified-List			CRITICALITY ignore	TYPE DRBs-FailedToBeModified-List				PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-SRBs-SetupMod-List						CRITICALITY ignore	TYPE SRBs-SetupMod-List								PRESENCE optional	}|

{ ID id-SRBs-Modified-List						CRITICALITY ignore	TYPE SRBs-Modified-List								PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

DRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-SetupMod-Item

DRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-Modified-Item

SRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-SetupMod-Item

SRBs-Modified-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-Modified-Item

DRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-FailedToBeModified-Item

SRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-FailedToBeSetupMod-Item

DRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-FailedToBeSetupMod-Item

SCell-FailedtoSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF SCell-FailedtoSetupMod-Item

-- **************************************************************

--

-- UE CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

UEContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationFailureIEs} },

...

}

UEContextModificationFailureIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- UE Context Modification Required (ngeNB-DU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

UEContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequiredIEs} },

...

}

UEContextModificationRequiredIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID							CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID							CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID								PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer		CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-DUtoCURRCInformation						CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE optional}|

{ ID id-DRBs-Required-ToBeModified-List				CRITICALITY reject	TYPE DRBs-Required-ToBeModified-List				PRESENCE optional}|

{ ID id-SRBs-Required-ToBeReleased-List				CRITICALITY reject	TYPE SRBs-Required-ToBeReleased-List				PRESENCE optional}|

{ ID id-DRBs-Required-ToBeReleased-List				CRITICALITY reject	TYPE DRBs-Required-ToBeReleased-List				PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause												PRESENCE mandatory	},

...

}

DRBs-Required-ToBeModified-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-Required-ToBeModified-Item

DRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-Required-ToBeReleased-Item

SRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF SRBs-Required-ToBeReleased-Item

-- **************************************************************

--

-- UE CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

UEContextModificationConfirm::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationConfirmIEs} },

...

}

UEContextModificationConfirmIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID							CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID									PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID							CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID									PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer		CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-DRBs-ModifiedConf-List						CRITICALITY ignore	TYPE DRBs-ModifiedConf-List							PRESENCE optional}|

{ ID id-RRCContainer								CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferInformation		CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

DRBs-ModifiedConf-List	::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRBs-ModifiedConf-Item

-- **************************************************************

--

-- UE CONTEXT MODIFICATION REFUSE

--

-- **************************************************************

UEContextModificationRefuse	::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRefuseIEs} },

...

}

UEContextModificationRefuseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

-- WRITE-REPLACE WARNING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Write-Replace Warning Request

--

-- **************************************************************

WriteReplaceWarningRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {WriteReplaceWarningRequestIEs} },

...

}

WriteReplaceWarningRequestIEs W1AP-PROTOCOL-IES	::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-PWSSystemInformation 				CRITICALITY reject	TYPE PWSSystemInformation 						PRESENCE mandatory }|

{ ID id-RepetitionPeriod 					CRITICALITY reject	TYPE RepetitionPeriod 							PRESENCE mandatory }|

{ ID id-NumberofBroadcastRequest 			CRITICALITY reject	TYPE NumberofBroadcastRequest 					PRESENCE mandatory }|

{ ID id-Cells-To-Be-Broadcast-List			CRITICALITY reject	TYPE Cells-To-Be-Broadcast-List					PRESENCE optional	},

...

}

Cells-To-Be-Broadcast-List	::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Cells-To-Be-Broadcast-Item

-- **************************************************************

--

-- Write-Replace Warning Response

--

-- **************************************************************

WriteReplaceWarningResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {WriteReplaceWarningResponseIEs} },

...

}

WriteReplaceWarningResponseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID	CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-Cells-Broadcast-Completed-List			CRITICALITY reject	TYPE Cells-Broadcast-Completed-List					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

Cells-Broadcast-Completed-List		::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Cells-Broadcast-Completed-Item

-- **************************************************************

--

-- PWS CANCEL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- PWS Cancel Request

--

-- **************************************************************

PWSCancelRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {PWSCancelRequestIEs} },

...

}

PWSCancelRequestIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-Broadcast-To-Be-Cancelled-List	CRITICALITY reject TYPE Broadcast-To-Be-Cancelled-List	PRESENCE optional	}|

{ ID id-Cancel-all-Warning-Messages-Indicator	CRITICALITY reject TYPE Cancel-all-Warning-Messages-Indicator	PRESENCE optional	}|

{ ID id-NotificationInformation					CRITICALITY reject TYPE NotificationInformation						PRESENCE mandatory	},

...

}

Broadcast-To-Be-Cancelled-List		::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Broadcast-To-Be-Cancelled-Item

-- **************************************************************

--

-- PWS Cancel Response

--

-- **************************************************************

PWSCancelResponse	::= SEQUENCE {

protocolIEs ProtocolIE-Container { {PWSCancelResponseIEs} },

...

}

PWSCancelResponseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-Cells-Broadcast-Cancelled-List	CRITICALITY reject	TYPE Cells-Broadcast-Cancelled-List	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

Cells-Broadcast-Cancelled-List		::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF Cells-Broadcast-Cancelled-Item

-- **************************************************************

--

-- UE Inactivity Notification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UE Inactivity Notification

--

-- **************************************************************

UEInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEInactivityNotificationIEs}},

...

}

UEInactivityNotificationIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID							CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID							PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID							CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID							PRESENCE mandatory	}|

{ ID id-DRB-Activity-List							CRITICALITY reject	TYPE DRB-Activity-List							PRESENCE mandatory	},

...

}

DRB-Activity-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Activity-Item

-- **************************************************************

--

-- Initial UL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- INITIAL UL RRC Message Transfer

--

-- **************************************************************

InitialULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ InitialULRRCMessageTransferIEs}},

...

}

InitialULRRCMessageTransferIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-DU-UE-W1AP-ID		CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID	PRESENCE mandatory	}|

{ ID id-EUTRANCGI				CRITICALITY reject	TYPE EUTRANCGI		PRESENCE mandatory	}|

{ ID id-C-RNTI					CRITICALITY reject	TYPE C-RNTI			PRESENCE mandatory	}|

{ ID id-RRCContainer			CRITICALITY reject	TYPE RRCContainer	PRESENCE mandatory	}|

{ ID id-DUtoCURRCContainer		CRITICALITY reject	TYPE DUtoCURRCContainer		PRESENCE optional	}|

{ ID id-TransactionID			CRITICALITY ignore	TYPE TransactionID	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- DL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- DL RRC Message Transfer

--

-- **************************************************************

DLRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DLRRCMessageTransferIEs}},

...

}

DLRRCMessageTransferIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID	CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID	PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID	CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID	PRESENCE mandatory	}|

{ ID id-oldngeNB-DU-UE-W1AP-ID						CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID							PRESENCE optional	}|

{ ID id-SRBID										CRITICALITY reject	TYPE SRBID											PRESENCE mandatory	}|

{ ID id-RRCContainer								CRITICALITY reject	TYPE RRCContainer									PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- UL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- UL RRC Message Transfer

--

-- **************************************************************

ULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ULRRCMessageTransferIEs}},

...

}

ULRRCMessageTransferIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID				CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID				PRESENCE mandatory	}|

{ ID id-SRBID							CRITICALITY reject	TYPE SRBID							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY reject	TYPE RRCContainer					PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs W1AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

-- Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PagingIEs}},

...

}

PagingIEs W1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue	CRITICALITY reject	TYPE UEIdentityIndexValue		PRESENCE mandatory	}|

{ ID id-PagingIdentity			CRITICALITY reject	TYPE PagingIdentity				PRESENCE mandatory	}|

{ ID id-PagingDRX				CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-PagingCell-List			CRITICALITY ignore	TYPE PagingCell-list			PRESENCE mandatory	}|

{ ID id-PagingOrigin			CRITICALITY ignore	TYPE PagingOrigin				PRESENCE optional	},

...

}

PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF PagingCell-Item

-- **************************************************************

--

-- Notify

--

-- **************************************************************

Notify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ NotifyIEs}},

...

}

NotifyIEs W1AP-PROTOCOL-IES ::= {

{ ID id-ngeNB-CU-UE-W1AP-ID					CRITICALITY reject	TYPE NGENB-CU-UE-W1AP-ID						PRESENCE mandatory	}|

{ ID id-ngeNB-DU-UE-W1AP-ID					CRITICALITY reject	TYPE NGENB-DU-UE-W1AP-ID						PRESENCE mandatory	}|

{ ID id-DRB-Notify-List						CRITICALITY reject	TYPE DRB-Notify-List						PRESENCE mandatory	},

...

}

DRB-Notify-List::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Notify-Item

-- **************************************************************

--

-- PWS RESTART INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- PWS Restart Indication

--

-- **************************************************************

PWSRestartIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { PWSRestartIndicationIEs} },

...

}

PWSRestartIndicationIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-E-UTRAN-CGI-List-For-Restart-List	CRITICALITY reject	TYPE E-UTRAN-CGI-List-For-Restart-List	PRESENCE mandatory	},

...

}

E-UTRAN-CGI-List-For-Restart-List		::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF E-UTRAN-CGI-List-For-Restart-Item

-- **************************************************************

--

-- PWS FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- PWS Failure Indication

--

-- **************************************************************

PWSFailureIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { PWSFailureIndicationIEs} },

...

}

PWSFailureIndicationIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-PWS-Failed-E-UTRAN-CGI-List	CRITICALITY reject	TYPE	 PWS-Failed-E-UTRAN-CGI-List	PRESENCE optional	},

...

}

PWS-Failed-E-UTRAN-CGI-List		::= SEQUENCE (SIZE(1.. maxCellinngeNBDU))	OF PWS-Failed-E-UTRAN-CGI-Item

-- **************************************************************

--

-- ngeNB-DU STATUS INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- ngeNB-DU Status Indication

--

-- **************************************************************

NGENBDUStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {NGENBDUStatusIndicationIEs} },

...

}

NGENBDUStatusIndicationIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-NGENBDUOverloadInformation	CRITICALITY reject	TYPE NGENBDUOverloadInformation		PRESENCE mandatory	},

...

}

END

-- ASN1STOP

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

W1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-NotificationInformation,

id-AdditionalSIBMessageList,

id-SourceTNLAddrInfo,

maxEARFCN,

maxnoofAdditionalSIBs,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofDLUPTNLInformation,

maxnoofE-UTRANCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxBandsEutra

FROM W1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM W1AP-CommonDataTypes

ProtocolExtensionContainer{},

W1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

W1AP-PROTOCOL-IES

FROM W1AP-Containers;

-- A

AdditionalSIBMessageList ::= SEQUENCE (SIZE(1..maxnoofAdditionalSIBs)) OF AdditionalSIBMessageList-Item

AdditionalSIBMessageList-Item ::= SEQUENCE {

additionalSIB			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { AdditionalSIBMessageList-Item-ExtIEs} } OPTIONAL

}

AdditionalSIBMessageList-Item-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL,

...

}

AvailablePLMNList-Item-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow  ::= INTEGER (0..4095, ...)

-- B

BitRate ::= INTEGER (0..4000000000000,...)

BearerTypeChange ::= ENUMERATED {true, ...}

BPLMN-ID-Info-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BPLMN-ID-Info-Item

BPLMN-ID-Info-Item ::= SEQUENCE {

pLMN-Identity-List			AvailablePLMNList,

fiveGS-TAC					FiveGS-TAC					OPTIONAL,

eUTRA-Cell-ID				EUTRA-Cell-ID,

ranac						RANAC						OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BPLMN-ID-Info-ItemExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-ItemExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

Broadcast-To-Be-Cancelled-Item ::= SEQUENCE {

eUTRANCGI			EUTRANCGI,

iE-Extensions		ProtocolExtensionContainer { { Broadcast-To-Be-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Broadcast-To-Be-Cancelled-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

-- C

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			EUTRANCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer { { Cause-ExtIEs} }

}

Cause-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

rl-failure-rlc,

unknown-or-already-allocated-enb-cu-ue-w1ap-id,

unknown-or-already-allocated-enb-du-ue-w1ap-id,

unknown-or-inconsistent-pair-of-ue-w1ap-id,

interaction-with-other-procedure,

not-supported-qci-Value,

action-desirable-for-radio-reasons,

no-radio-resources-available,

procedure-cancelled,

normal-release,

cell-not-available,

rl-failure-others,

ue-rejection,

resources-not-available-for-the-slice,

amf-initiated-abnormal-release,

release-due-to-pre-emption,

multiple-drb-id-instances,

unknown-drb-id,

...

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...

}

Cells-Failed-to-be-Activated-List-Item ::= SEQUENCE {

eUTRANCGI			EUTRANCGI,

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { { Cells-Failed-to-be-Activated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Failed-to-be-Activated-List-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Status-Item ::= SEQUENCE {

eUTRANCGI			EUTRANCGI,

service-status		Service-Status,

iE-Extensions		ProtocolExtensionContainer { { Cells-Status-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Status-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-To-Be-Broadcast-Item ::= SEQUENCE {

eUTRANCGI				EUTRANCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-To-Be-Broadcast-ItemExtIEs } }	OPTIONAL,

...

}

Cells-To-Be-Broadcast-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Completed-Item ::= SEQUENCE {

eUTRANCGI				EUTRANCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Completed-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Completed-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Cancelled-Item ::= SEQUENCE {

eUTRANCGI				EUTRANCGI,

numberOfBroadcasts		NumberOfBroadcasts,

iE-Extensions			ProtocolExtensionContainer { { Cells-Broadcast-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Cancelled-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Activated-List-Item ::= SEQUENCE {

eUTRANCGI						EUTRANCGI,

eUTRANPCI						EUTRANPCI							OPTIONAL,

ngeNB-CUSystemInformation		NGENB-CUSystemInformation			OPTIONAL,

availablePLMNList				AvailablePLMNList					OPTIONAL,

extendedAvailablePLMN-List		ExtendedAvailablePLMN-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Cells-to-be-Activated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-Activated-List-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

eUTRANCGI					EUTRANCGI,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Cell-Type ::= ENUMERATED {

verysmall,

small,

medium,

large,

...

}

CNUEPagingIdentity ::= CHOICE {

fiveG-S-TMSI				BIT STRING (SIZE(48)),

choice-extension			ProtocolIE-SingleContainer { { CNUEPagingIdentity-ExtIEs } }

}

CNUEPagingIdentity-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode				ProcedureCode									OPTIONAL,

triggeringMessage			TriggeringMessage								OPTIONAL,

procedureCriticality		Criticality										OPTIONAL,

transactionID					TransactionID								OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1.. maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError 			TypeOfError,

iE-Extensions			ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}}	OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= INTEGER (0..65535, ...)

CUtoDURRCInformation ::= SEQUENCE {

cG-ConfigInfo						CG-ConfigInfo						OPTIONAL,

uE-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList		OPTIONAL,

measConfig							MeasConfig							OPTIONAL,

handoverPreparationInformation		HandoverPreparationInformation		OPTIONAL,

radioResourceConfigDedicated		RadioResourceConfigDedicated		OPTIONAL,

measurementTimingConfiguration		MeasurementTimingConfiguration		OPTIONAL,

uEAssistanceInformation				UEAssistanceInformation				OPTIONAL,

requestedP-MaxFR1					OCTET STRING						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CUtoDURRCInformation-ExtIEs} } OPTIONAL,

...

}

CUtoDURRCInformation-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

-- D

DLUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDLUPTNLInformation)) OF DLUPTNLInformation-ToBeSetup-Item

DLUPTNLInformation-ToBeSetup-Item ::= SEQUENCE {

dLUPTNLInformation	UPTransportLayerInformation	,

iE-Extensions	ProtocolExtensionContainer { { DLUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DLUPTNLInformation-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity-Item ::= SEQUENCE {

dRBID			DRBID,

dRB-Activity	DRB-Activity		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity ::= ENUMERATED {active, not-active}

DRBID ::= INTEGER (1..32, ...)

DRBs-FailedToBeModified-Item	::= SEQUENCE {

dRBID		DRBID,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeModified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetup-Item	::= SEQUENCE {

dRBID	DRBID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

dRBID		DRBID,

cause		Cause			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID									DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

rLC-Status								RLC-Status								OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }		OPTIONAL,

...

}

DRBs-Modified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ModifiedConf-Item	::= SEQUENCE {

dRBID							DRBID,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ModifiedConf-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Notify-Item ::= SEQUENCE {

dRBID			DRBID,

notification-Cause	Notification-Cause,

iE-Extensions	ProtocolExtensionContainer { { DRB-Notify-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Notify-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Required-ToBeModified-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

rLC-Status								RLC-Status							OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeModified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Required-ToBeReleased-Item	::= SEQUENCE {

dRBID		DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeReleased-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Setup-Item ::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Setup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-SetupMod-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-SetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeModified-Item	::= SEQUENCE {

dRBID								DRBID,

qoSInformation						QoSInformation							OPTIONAL,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List,

dLPDCPSNLength						PDCPSNLength							OPTIONAL,

uLPDCPSNLength						PDCPSNLength							OPTIONAL,

bearerTypeChange					BearerTypeChange						OPTIONAL,

rLCMode								RLCMode									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DRBs-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeModified-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeReleased-Item	::= SEQUENCE {

dRBID	DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeReleased-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeSetup-Item ::= SEQUENCE	{

dRBID								DRBID,

qoSInformation						QoSInformation,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List,

rLCMode								RLCMode,

dLPDCPSNLength						PDCPSNLength,

uLPDCPSNLength						PDCPSNLength								OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeSetupMod-Item	::= SEQUENCE {

dRBID									DRBID,

qoSInformation							QoSInformation,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List,

rLCMode									RLCMode,

dLPDCPSNLength							PDCPSNLength							OPTIONAL,

uLPDCPSNLength							PDCPSNLength							OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRXCycle	::= SEQUENCE {

longDRXCycleLength		LongDRXCycleLength,

shortDRXCycleLength		ShortDRXCycleLength		OPTIONAL,

shortDRXCycleTimer		ShortDRXCycleTimer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { DRXCycle-ExtIEs} }	OPTIONAL,

...

}

DRXCycle-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

DRX-Config ::= OCTET STRING

DRX-LongCycleStartOffset ::= INTEGER (0..10239)

DUtoCURRCContainer ::= OCTET STRING

DUtoCURRCInformation ::= SEQUENCE {

radioResourceConfigDedicated		RadioResourceConfigDedicated,

measGapConfig						MeasGapConfig					OPTIONAL,

requestedP-MaxFR1					OCTET STRING					OPTIONAL,

dRX-LongCycleStartOffset			DRX-LongCycleStartOffset		OPTIONAL,

selectedBandCombinationIndex		SelectedBandCombinationIndex	OPTIONAL,

selectedFeatureSetEntryIndex		SelectedFeatureSetEntryIndex	OPTIONAL,

ph-InfoSCG							Ph-InfoSCG						OPTIONAL,

requestedBandCombinationIndex		RequestedBandCombinationIndex	OPTIONAL,

requestedFeatureSetEntryIndex		RequestedFeatureSetEntryIndex	OPTIONAL,

dRX-Config							DRX-Config						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DUtoCURRCInformation-ExtIEs} } OPTIONAL,

...

}

DUtoCURRCInformation-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

{ID id-MeasGapSharingConfig		CRITICALITY ignore	EXTENSION MeasGapSharingConfig		PRESENCE optional},

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel			INTEGER (1..127),

packetDelayBudget			PacketDelayBudget,

packetErrorRate				PacketErrorRate,

fiveQI						INTEGER (0..255, ...)					OPTIONAL,

delayCritical					ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

averagingWindow 			AveragingWindow										OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume									OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL,

...

}

Dynamic5QIDescriptor-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

-- E

Endpoint-IP-address-and-port ::=SEQUENCE {

endpointIPAddress	TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL,

...

}

Endpoint-IP-address-and-port-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Cell-ID ::= BIT STRING (SIZE(28))

EUTRA-Coex-FDD-Info ::= SEQUENCE {

uL-EUTRAARFCN						ExtendedEARFCN					OPTIONAL,

dL-EUTRAARFCN						ExtendedEARFCN,

uL-Transmission-Bandwidth			EUTRA-Transmission-Bandwidth	OPTIONAL,

dL-Transmission-Bandwidth			EUTRA-Transmission-Bandwidth,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-Coex-FDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-Coex-FDD-Info-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Coex-Mode-Info ::= CHOICE {

fDD		EUTRA-Coex-FDD-Info,

tDD		EUTRA-Coex-TDD-Info,

...

}

EUTRA-Coex-TDD-Info ::= SEQUENCE {

eARFCN							ExtendedEARFCN,

transmission-Bandwidth			EUTRA-Transmission-Bandwidth,

subframeAssignment				EUTRA-SubframeAssignment,

specialSubframe-Info			EUTRA-SpecialSubframe-Info,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-Coex-TDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-Coex-TDD-Info-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-CyclicPrefixDL ::= ENUMERATED {

normal,

extended,

...

}

EUTRA-CyclicPrefixUL ::= ENUMERATED {

normal,

extended,

...

}

EUTRA-PRACH-Configuration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							BOOLEAN,

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)		OPTIONAL,  -- present for TDD --

iE-Extensions							ProtocolExtensionContainer { {EUTRA-PRACH-Configuration-ExtIEs} }	OPTIONAL,

...

}

EUTRA-PRACH-Configuration-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-SpecialSubframe-Info ::= SEQUENCE {

specialSubframePatterns		EUTRA-SpecialSubframePatterns,

cyclicPrefixDL				EUTRA-CyclicPrefixDL,

cyclicPrefixUL				EUTRA-CyclicPrefixUL,

iE-Extensions				ProtocolExtensionContainer { { EUTRA-SpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-SpecialSubframe-Info-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-SpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

ssp10,

...

}

EUTRA-SubframeAssignment ::= ENUMERATED {

sa0,

sa1,

sa2,

sa3,

sa4,

sa5,

sa6,

...

}

EUTRA-Transmission-Bandwidth ::= ENUMERATED {

bw1,

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

EUTRANQoS	::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EUTRANQoS-ExtIEs} }	OPTIONAL,

...

}

EUTRANQoS-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedEARFCN ::= INTEGER (0..maxEARFCN)

EUTRA-NR-CellResourceCoordinationReq-Container	::= OCTET STRING

EUTRA-NR-CellResourceCoordinationReqAck-Container	::= OCTET STRING

EUTRAFreqInfo ::=  SEQUENCE {

eARFCN			INTEGER (0..maxEARFCN),

freqBandListEutra	SEQUENCE (SIZE(1.. maxnoofE-UTRANCellBands)) OF FreqBandEutraItem,

iE-Extensions	ProtocolExtensionContainer { { EUTRAFreqInfoExtIEs} } OPTIONAL,

...

}

EUTRAFreqInfoExtIEs		W1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRANCGI ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

eUTRACellIdentity		E-UTRACellIdentity,

iE-Extensions			ProtocolExtensionContainer { {EUTRANCGI-ExtIEs} } OPTIONAL,

...

}

EUTRANCGI-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

E-UTRAN-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { E-UTRAN-Mode-Info-ExtIEs} }

}

E-UTRAN-Mode-Info-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

E-UTRACellIdentity ::= BIT STRING (SIZE(28))

EUTRANPCI ::= INTEGER (0..503)

E-UTRAN-CGI-List-For-Restart-Item ::= SEQUENCE {

eUTRANCGI				EUTRANCGI,

iE-Extensions		ProtocolExtensionContainer { { E-UTRAN-CGI-List-For-Restart-ItemExtIEs } }	OPTIONAL,

...

}

E-UTRAN-CGI-List-For-Restart-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedAvailablePLMN-List ::= SEQUENCE (SIZE(1..maxnoofExtendedBPLMNs)) OF ExtendedAvailablePLMN-Item

ExtendedAvailablePLMN-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAvailablePLMN-Item-ExtIEs} } OPTIONAL

}

ExtendedAvailablePLMN-Item-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

-- F

FDD-Info ::= SEQUENCE {

uL-EUTRAFreqInfo						EUTRAFreqInfo,

dL-EUTRAFreqInfo						EUTRAFreqInfo,

uL-Transmission-Bandwidth		Transmission-Bandwidth,

dL-Transmission-Bandwidth		Transmission-Bandwidth,

iE-Extensions					ProtocolExtensionContainer { {FDD-Info-ExtIEs} } OPTIONAL,

...

}

FDD-Info-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

Flows-Mapped-To-DRB-List	::=	SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF Flows-Mapped-To-DRB-Item

Flows-Mapped-To-DRB-Item 	::= SEQUENCE {

qoSFlowIdentifier						QoSFlowIdentifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoSFlowMappingIndication    OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Flows-Mapped-To-DRB-ItemExtIEs} } OPTIONAL,

...

}

Flows-Mapped-To-DRB-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

FreqBandEutraItem ::= SEQUENCE {

freqBandIndicatorEutra 		INTEGER (1.. maxBandsEutra),

iE-Extensions				ProtocolExtensionContainer { {FreqBandEutraItem-ExtIEs} } OPTIONAL,

...

}

FreqBandEutraItem-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

-- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

NGENB-CUSystemInformation::= SEQUENCE {

sibtypetobeupdatedlist	SEQUENCE (SIZE(1.. maxnoofSIBTypes)) OF SibtypetobeupdatedListItem,

iE-Extensions					ProtocolExtensionContainer { { NGENB-CUSystemInformation-ExtIEs} } OPTIONAL,

...

}

NGENB-CUSystemInformation-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

NGENB-CU-UE-W1AP-ID	::= INTEGER (0..4294967295)

NGENB-DU-UE-W1AP-ID	::= INTEGER (0..4294967295)

NGENB-DU-ID	::= INTEGER (0..68719476735)

NGENB-DU-Served-Cells-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

ngeNB-DU-System-Information	NGENB-DU-System-Information	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { NGENB-DU-Served-Cells-ItemExtIEs} }	OPTIONAL,

...

}

NGENB-DU-Served-Cells-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

NGENB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

sIB2-message		SIB2-message,

sIB3-message		SIB3-message,

sIB8-message		SIB8-message,

sIB16-message		SIB16-message,

iE-Extensions					ProtocolExtensionContainer { { NGENB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

NGENB-DU-System-Information-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

NGENBDUOverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTunnel				::= SEQUENCE {

transportLayerAddress		TransportLayerAddress,

gTP-TEID		GTP-TEID,

iE-Extensions					ProtocolExtensionContainer { { GTPTunnel-ExtIEs } } OPTIONAL,

...

}

GTPTunnel-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

-- H

HandoverPreparationInformation ::= OCTET STRING

-- I

IgnoreResourceCoordinationRequestContainer ::= ENUMERATED { true,...}

InactivityMonitoringRequest ::= ENUMERATED { true,...}

InactivityMonitoringResponse ::= ENUMERATED { not-supported,...}

-- J

-- K

-- L

LongDRXCycleLength ::= 	ENUMERATED

{ms10, ms20, ms32, ms40, ms60, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560,...}

-- M

MaxDataBurstVolume  ::= INTEGER (0..4095,...)

MaxPacketLossRate ::= INTEGER (0..1000)

MIB-message ::= OCTET STRING

MeasConfig ::= OCTET STRING

MeasGapConfig ::= OCTET STRING

MeasGapSharingConfig ::= OCTET STRING

MeasurementTimingConfiguration ::= OCTET STRING

MessageIdentifier ::= BIT STRING (SIZE (16))

-- N

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

NGRANAllocationAndRetentionPriority-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

NonDynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255,...),

qoSPriorityLevel			INTEGER (1..127)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { NonDynamic5QIDescriptor-ExtIEs } } OPTIONAL,

...

}

NonDynamic5QIDescriptor-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

Notification-Cause ::= ENUMERATED {fulfilled, not-fulfilled,...}

NotificationControl ::= ENUMERATED {active, not-active,...}

NotificationInformation ::= SEQUENCE {

message-Identifier	MessageIdentifier,

serialNumber		SerialNumber,

iE-Extensions		ProtocolExtensionContainer { { NotificationInformationExtIEs} } OPTIONAL,

...

}

NotificationInformationExtIEs		W1AP-PROTOCOL-EXTENSION ::= {

...

}

NumberOfBroadcasts ::= INTEGER (0..65535)

NumberofBroadcastRequest ::= INTEGER (0..65535)

-- O

-- P

PacketDelayBudget ::= INTEGER (0..1023,...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9,...)

PER-Exponent ::= INTEGER (0..9,...)

PagingCell-Item ::= SEQUENCE {

eUTRANCGI		EUTRANCGI,

iE-Extensions	ProtocolExtensionContainer { { PagingCell-ItemExtIEs } }	OPTIONAL,

...

}

PagingCell-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingIdentity ::=	CHOICE {

rANUEPagingIdentity	RANUEPagingIdentity,

cNUEPagingIdentity	CNUEPagingIdentity,

choice-extension			ProtocolIE-SingleContainer { { PagingIdentity-ExtIEs } }

}

PagingIdentity-ExtIEs W1AP-PROTOCOL-IES::= {

...

}

PagingOrigin ::= ENUMERATED { non-3gpp,	...}

PDCP-SN ::= INTEGER (0..4095)

PDCPSNLength	::= ENUMERATED { twelve-bits,eighteen-bits,...}

PDUSessionID ::= INTEGER (0..255)

Ph-InfoSCG  ::= OCTET STRING

PLMN-Identity ::= OCTET STRING (SIZE(3))

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

Potential-SpCell-Item ::= SEQUENCE {

potential-SpCell-ID			EUTRANCGI	,

iE-Extensions	ProtocolExtensionContainer { { Potential-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Potential-SpCell-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

PWS-Failed-E-UTRAN-CGI-Item ::= SEQUENCE {

eUTRANCGI			EUTRANCGI,

iE-Extensions		ProtocolExtensionContainer { { PWS-Failed-E-UTRAN-CGI-ItemExtIEs } }	OPTIONAL,

...

}

PWS-Failed-E-UTRAN-CGI-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

PWSSystemInformation ::= SEQUENCE {

sIBtype 				SIBType-PWS,

sIBmessage			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { PWSSystemInformationExtIEs } }	OPTIONAL,

...

}

PWSSystemInformationExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

{ID id-NotificationInformation		CRITICALITY ignore	EXTENSION NotificationInformation		PRESENCE mandatory}|

{ID id-AdditionalSIBMessageList		CRITICALITY reject	EXTENSION AdditionalSIBMessageList		PRESENCE optional},

...

}

-- Q

QCI ::= INTEGER (0..255)

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				NonDynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer { { QoS-Characteristics-ExtIEs } }

}

QoS-Characteristics-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

QoSFlowIdentifier ::= INTEGER (0..63)

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

pDUSessionID							PDUSessionID								OPTIONAL,

uLPDUSessionAggregateMaximumBitRate		BitRate										OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL,

...

}

QoSFlowLevelQoSParameters-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SourceTNLAddrInfo		CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	},

...

}

QoSFlowMappingIndication ::= ENUMERATED {ul,dl,...}

QoSInformation	::=	CHOICE {

eUTRANQoS				EUTRANQoS,

dRB-Information			DRB-Information,

choice-extension		ProtocolIE-SingleContainer { { QoSInformation-ExtIEs} }

}

QoSInformation-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

-- R

RadioResourceConfigDedicated	::= OCTET STRING

RANAC ::= INTEGER (0..255)

RANUEPagingIdentity ::= SEQUENCE	{

iRNTI				BIT STRING (SIZE(40)),

iE-Extensions		ProtocolExtensionContainer { { RANUEPagingIdentity-ExtIEs } }	OPTIONAL,

...

}

RANUEPagingIdentity-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

RAT-FrequencyPriorityInformation::= CHOICE {

eNDC		SubscriberProfileIDforRFP,

nGRAN		RAT-FrequencySelectionPriority,

choice-extension			ProtocolIE-SingleContainer { { RAT-FrequencyPriorityInformation-ExtIEs} }

}

RAT-FrequencyPriorityInformation-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

RAT-FrequencySelectionPriority::= INTEGER (1.. 256, ...)

Reestablishment-Indication	::=	ENUMERATED  {

reestablished,

...

}

RequestedBandCombinationIndex	::= OCTET STRING

RequestedFeatureSetEntryIndex	::= OCTET STRING

RequestType	::= ENUMERATED {offer, execution, ...}

ResourceCoordinationEUTRACellInfo ::= SEQUENCE {

eUTRA-Mode-Info 						EUTRA-Coex-Mode-Info,

eUTRA-PRACH-Configuration 				EUTRA-PRACH-Configuration,

iE-Extensions	ProtocolExtensionContainer { { ResourceCoordinationEUTRACellInfo-ExtIEs } }	OPTIONAL,

...

}

ResourceCoordinationEUTRACellInfo-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordinationTransferInformation ::= SEQUENCE {

meNB-Cell-ID							EUTRA-Cell-ID,

resourceCoordinationEUTRACellInfo		ResourceCoordinationEUTRACellInfo	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { ResourceCoordinationTransferInformation-ExtIEs } }	OPTIONAL,

...

}

ResourceCoordinationTransferInformation-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordinationTransferContainer ::= OCTET STRING

RepetitionPeriod ::= INTEGER (0..131071, ...)

RLCMode ::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication	Reestablishment-Indication,

iE-Extensions				ProtocolExtensionContainer { { RLC-Status-ExtIEs } } OPTIONAL,

...

}

RLC-Status-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

RRCContainer ::= OCTET STRING

RRCReconfigurationCompleteIndicator	::= ENUMERATED {

true,

failure,

...

}

-- S

SCell-FailedtoSetup-Item	::= SEQUENCE {

sCell-ID			EUTRANCGI,

cause				Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-FailedtoSetupMod-Item	::= SEQUENCE {

sCell-ID			EUTRANCGI,

cause				Cause			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeRemoved-Item	::= SEQUENCE {

sCell-ID			EUTRANCGI,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeRemoved-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeRemoved-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeSetup-Item ::= SEQUENCE {

sCell-ID			EUTRANCGI,

sCellIndex			SCellIndex,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeSetupMod-Item	::= SEQUENCE {

sCell-ID			EUTRANCGI,

sCellIndex			SCellIndex,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SCellIndex ::=INTEGER (1..31, ...)

SCGIndicator	::= ENUMERATED {released, ...}

SIBType-PWS ::=INTEGER (6..8,...)

SelectedBandCombinationIndex ::= OCTET STRING

SelectedFeatureSetEntryIndex ::= OCTET STRING

SerialNumber ::= BIT STRING (SIZE (16))

CG-ConfigInfo ::= OCTET STRING

ServCellIndex ::= INTEGER (0..31,...)

Served-Cell-Information ::= SEQUENCE {

eUTRANCGI							EUTRANCGI,

eUTRANPCI							EUTRANPCI,

fiveGS-TAC							FiveGS-TAC						OPTIONAL,

servedPLMNs							ServedPLMNs-List,

eUTRAN-Mode-Info					E-UTRAN-Mode-Info,

measurementTimingConfiguration		OCTET STRING,

rANAC								RANAC							OPTIONAL,

cell-Type							Cell-Type						OPTIONAL,

bPLMN-ID-Info-List					BPLMN-ID-Info-List				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Add-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

ngeNB-DU-System-Information	NGENB-DU-System-Information	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Add-ItemExtIEs} }	OPTIONAL,

...

}

Served-Cells-To-Add-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Delete-Item ::= SEQUENCE {

oldEUTRANCGI					EUTRANCGI,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Delete-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Delete-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Modify-Item ::= SEQUENCE {

oldEUTRANCGI					EUTRANCGI,

served-Cell-Information			Served-Cell-Information,

ngeNB-DU-System-Information		NGENB-DU-System-Information 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Served-Cells-To-Modify-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Modify-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

Service-State ::= ENUMERATED {

in-service,

out-of-service,

...

}

Service-Status ::= SEQUENCE {

service-state				Service-State,

switchingOffOngoing			ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Service-Status-ExtIEs } }	OPTIONAL,

...

}

Service-Status-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

ServedPLMNs-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedPLMNs-Item

ServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

tAISliceSupportList			SliceSupportList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ServedPLMNs-ItemExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

ShortDRXCycleLength ::=  ENUMERATED {ms2, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ...}

ShortDRXCycleTimer ::= INTEGER (1..16)

SIB1-message ::= OCTET STRING

SIB2-message ::= OCTET STRING

SIB3-message ::= OCTET STRING

SIB8-message ::= OCTET STRING

SIB16-message ::= OCTET STRING

SibtypetobeupdatedListItem ::= SEQUENCE {

sIBtype 			INTEGER (2..32,...),

sIBmessage			OCTET STRING,

valueTag			INTEGER (0..31,...),

iE-Extensions	ProtocolExtensionContainer { { SibtypetobeupdatedListItem-ExtIEs } }	OPTIONAL,

...

}

SibtypetobeupdatedListItem-ExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { SliceSupportItem-ExtIEs } }	OPTIONAL,

...

}

SliceSupportItem-ExtIEs	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST			OCTET STRING (SIZE(1)),

sD			OCTET STRING (SIZE(3))	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-ExtIEs } }	OPTIONAL,

...

}

SNSSAI-ExtIEs	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBID ::= INTEGER (0..3, ...)

SRBs-FailedToBeSetup-Item	::= SEQUENCE {

sRBID		SRBID,

cause		Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-FailedToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

sRBID		SRBID,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-FailedToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Modified-Item ::= SEQUENCE {

sRBID							SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Modified-ItemExtIEs	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Required-ToBeReleased-Item	::= SEQUENCE {

sRBID	SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Required-ToBeReleased-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Setup-Item ::= SEQUENCE {

sRBID							SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Setup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-SetupMod-Item ::= SEQUENCE {

sRBID						SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-SetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeReleased-Item	::= SEQUENCE {

sRBID			SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeReleased-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeSetup-Item ::= SEQUENCE {

sRBID			SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeSetupMod-Item	::= SEQUENCE {

sRBID	SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeSetupMod-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

-- T

FiveGS-TAC ::= OCTET STRING (SIZE(3))

TDD-Info ::= SEQUENCE {

eUTRAFreqInfo							EUTRAFreqInfo,

transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

TransportLayerAddress		::= BIT STRING (SIZE(1..160,...))

TransactionID				::= INTEGER (0..255,...)

Transmission-Bandwidth ::= SEQUENCE {

eUTRANRB	EUTRA-Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { { Transmission-Bandwidth-ExtIEs} } OPTIONAL,

...

}

Transmission-Bandwidth-ExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionActionIndicator ::= ENUMERATED {stop, ..., restart }

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

-- U

UE-associatedLogicalW1-ConnectionItem ::= SEQUENCE {

ngeNB-CU-UE-W1AP-ID		NGENB-CU-UE-W1AP-ID	 OPTIONAL,

ngeNB-DU-UE-W1AP-ID		NGENB-DU-UE-W1AP-ID	 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { UE-associatedLogicalW1-ConnectionItemExtIEs} } OPTIONAL,

...

}

UE-associatedLogicalW1-ConnectionItemExtIEs W1AP-PROTOCOL-EXTENSION ::= {

...

}

UEAssistanceInformation ::= OCTET STRING

UE-CapabilityRAT-ContainerList::= OCTET STRING

UEIdentityIndexValue ::= CHOICE {

indexLength10			BIT STRING (SIZE (10)),

choice-extension		ProtocolIE-SingleContainer { {UEIdentityIndexValueChoice-ExtIEs} }

}

UEIdentityIndexValueChoice-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

ULUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofULUPTNLInformation)) OF ULUPTNLInformation-ToBeSetup-Item

ULUPTNLInformation-ToBeSetup-Item ::=SEQUENCE {

uLUPTNLInformation		UPTransportLayerInformation,

iE-Extensions			ProtocolExtensionContainer { { ULUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

ULUPTNLInformation-ToBeSetup-ItemExtIEs 	W1AP-PROTOCOL-EXTENSION ::= {

...

}

UPTransportLayerInformation		::= CHOICE {

gTPTunnel					GTPTunnel,

choice-extension			ProtocolIE-SingleContainer { { UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs W1AP-PROTOCOL-IES ::= {

...

}

-- V

-- W

-- X

-- Y

-- Z

END

-- ASN1STOP

9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

W1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0..65535),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..65535)

ProtocolIE-ID		::= INTEGER (0..65535)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessfull-outcome }

END

-- ASN1STOP

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

W1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM W1AP-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-W1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-ngeNBDUConfigurationUpdate				ProcedureCode ::= 3

id-ngeNBCUConfigurationUpdate				ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-NGENBDUResourceCoordination				ProcedureCode ::= 16

id-Paging									ProcedureCode ::= 17

id-Notify									ProcedureCode ::= 18

id-WriteReplaceWarning						ProcedureCode ::= 19

id-PWSCancel								ProcedureCode ::= 20

id-PWSRestartIndication						ProcedureCode ::= 21

id-PWSFailureIndication						ProcedureCode ::= 22

id-NGENBDUStatusIndication 					ProcedureCode ::= 23

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxEARFCN								INTEGER ::= 262143

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualW1ConnectionsToReset	INTEGER ::= 65536

maxCellinngeNBDU						INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofE-UTRANCellBands					INTEGER ::= 8

maxnoofSIBTypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxBandsEutra							INTEGER ::= 256

maxnoofAdditionalSIBs					INTEGER ::= 63

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause													ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List						ProtocolIE-ID ::= 1

id-Cells-to-be-Activated-List							ProtocolIE-ID ::= 2

id-Cells-to-be-Deactivated-List							ProtocolIE-ID ::= 3

id-CriticalityDiagnostics								ProtocolIE-ID ::= 4

id-CUtoDURRCInformation									ProtocolIE-ID ::= 5

id-DRBs-FailedToBeModified-List							ProtocolIE-ID ::= 6

id-DRBs-FailedToBeSetup-List								ProtocolIE-ID ::= 7

id-DRBs-FailedToBeSetupMod-List							ProtocolIE-ID ::= 8

id-DRBs-ModifiedConf-List								ProtocolIE-ID ::= 9

id-DRBs-Modified-List									ProtocolIE-ID ::= 10

id-DRBs-Required-ToBeModified-List						ProtocolIE-ID ::= 11

id-DRBs-Required-ToBeReleased-List						ProtocolIE-ID ::= 12

id-DRBs-Setup-List										ProtocolIE-ID ::= 13

id-DRBs-SetupMod-List									ProtocolIE-ID ::= 14

id-DRBs-ToBeModified-List								ProtocolIE-ID ::= 15

id-DRBs-ToBeReleased-List								ProtocolIE-ID ::= 16

id-DRBs-ToBeSetup-List									ProtocolIE-ID ::= 17

id-DRBs-ToBeSetupMod-List								ProtocolIE-ID ::= 18

id-DRXCycle												ProtocolIE-ID ::= 19

id-DUtoCURRCInformation									ProtocolIE-ID ::= 20

id-ngeNB-CU-UE-W1AP-ID									ProtocolIE-ID ::= 21

id-ngeNB-DU-UE-W1AP-ID									ProtocolIE-ID ::= 22

id-ngeNB-DU-ID											ProtocolIE-ID ::= 23

id-ngeNB-DU-Served-Cells-List							ProtocolIE-ID ::= 24

id-oldngeNB-DU-UE-W1AP-ID								ProtocolIE-ID ::= 25

id-RAT-FrequencyPriorityInformation						ProtocolIE-ID ::= 26

id-ResetType											ProtocolIE-ID ::= 27

id-ResourceCoordinationTransferContainer				ProtocolIE-ID ::= 28

id-RRCContainer											ProtocolIE-ID ::= 29

id-SCell-ToBeRemoved-List								ProtocolIE-ID ::= 30

id-SCell-ToBeSetup-List									ProtocolIE-ID ::= 31

id-SCell-ToBeSetupMod-List								ProtocolIE-ID ::= 32

id-Served-Cells-To-Add-List								ProtocolIE-ID ::= 33

id-Served-Cells-To-Delete-List							ProtocolIE-ID ::= 34

id-Served-Cells-To-Modify-List							ProtocolIE-ID ::= 35

id-SpCell-ID											ProtocolIE-ID ::= 36

id-SRBID												ProtocolIE-ID ::= 37

id-SRBs-FailedToBeSetup-List							ProtocolIE-ID ::= 38

id-SRBs-FailedToBeSetupMod-List							ProtocolIE-ID ::= 39

id-SRBs-Required-ToBeReleased-List						ProtocolIE-ID ::= 40

id-SRBs-ToBeReleased-List								ProtocolIE-ID ::= 41

id-SRBs-ToBeSetup-List									ProtocolIE-ID ::= 42

id-SRBs-ToBeSetupMod-List								ProtocolIE-ID ::= 43

id-TransactionID										ProtocolIE-ID ::= 44

id-TransmissionActionIndicator							ProtocolIE-ID ::= 45

id-UE-associatedLogicalW1-ConnectionListResAck			ProtocolIE-ID ::= 46

id-SCell-FailedtoSetup-List								ProtocolIE-ID ::= 47

id-SCell-FailedtoSetupMod-List							ProtocolIE-ID ::= 48

id-RRCReconfigurationCompleteIndicator 					ProtocolIE-ID ::= 49

id-Cells-Status-List									ProtocolIE-ID ::= 50

id-Candidate-SpCell-List								ProtocolIE-ID ::= 51

id-Potential-SpCell-List								ProtocolIE-ID ::= 52

id-C-RNTI												ProtocolIE-ID ::= 53

id-InactivityMonitoringRequest							ProtocolIE-ID ::= 54

id-InactivityMonitoringResponse							ProtocolIE-ID ::= 55

id-DRB-Activity-List									ProtocolIE-ID ::= 56

id-EUTRA-NR-CellResourceCoordinationReq-Container 		ProtocolIE-ID ::= 57

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 58

id-RequestType 											ProtocolIE-ID ::= 59

id-ServCellIndex										ProtocolIE-ID ::= 60

id-EUTRANCGI											ProtocolIE-ID ::= 61

id-PagingCell-List										ProtocolIE-ID ::= 62

id-PagingDRX											ProtocolIE-ID ::= 63

id-UEIdentityIndexValue									ProtocolIE-ID ::= 64

id-PagingIdentity										ProtocolIE-ID ::= 65

id-DUtoCURRCContainer									ProtocolIE-ID ::= 66

id-DRB-Notify-List										ProtocolIE-ID ::= 67

id-NotficationControl									ProtocolIE-ID ::= 68

id-PWSSystemInformation									ProtocolIE-ID ::= 69

id-RepetitionPeriod										ProtocolIE-ID ::= 70

id-NumberofBroadcastRequest								ProtocolIE-ID ::= 71

id-Cells-To-Be-Broadcast-List							ProtocolIE-ID ::= 72

id-Cells-Broadcast-Completed-List 						ProtocolIE-ID ::= 73

id-Broadcast-To-Be-Cancelled-List 						ProtocolIE-ID ::= 74

id-Cells-Broadcast-Cancelled-List 						ProtocolIE-ID ::= 75

id-E-UTRAN-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 76

id-PWS-Failed-E-UTRAN-CGI-List 							ProtocolIE-ID ::= 77

id-ConfirmedUEID										ProtocolIE-ID ::= 78

id-Cancel-all-Warning-Messages-Indicator				ProtocolIE-ID ::= 79

id-NGENB-DU-UE-AMBR-UL									ProtocolIE-ID ::= 80

id-ServingPLMN											ProtocolIE-ID ::= 81

id-NGENBDUOverloadInformation							ProtocolIE-ID ::= 82

id-ResourceCoordinationTransferInformation				ProtocolIE-ID ::= 83

id-SRBs-Setup-List										ProtocolIE-ID ::= 85

id-SRBs-SetupMod-List									ProtocolIE-ID ::= 86

id-SRBs-Modified-List									ProtocolIE-ID ::= 87

id-IgnoreResourceCoordinationRequestContainer			ProtocolIE-ID ::= 88

id-NotificationInformation								ProtocolIE-ID ::= 89

id-AdditionalSIBMessageList								ProtocolIE-ID ::= 90

id-MeasGapSharingConfig									ProtocolIE-ID ::= 91

id-SCGIndicator											ProtocolIE-ID ::= 92

id-PagingOrigin											ProtocolIE-ID ::= 93

id-SourceTNLAddrInfo									ProtocolIE-ID ::= xx

END

-- ASN1STOP

9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

W1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

Presence,

PrivateIE-ID,

ProtocolExtensionID,

ProtocolIE-ID

FROM W1AP-CommonDataTypes

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs

FROM W1AP-Constants;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

W1AP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 					UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

W1AP-PROTOCOL-IES-PAIR ::= CLASS {

&id					ProtocolIE-ID 				UNIQUE,

&firstCriticality	Criticality,

&FirstValue,

&secondCriticality	Criticality,

&SecondValue,

&presence			Presence

}

WITH SYNTAX {

ID				&id

FIRST CRITICALITY		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

W1AP-PROTOCOL-EXTENSION ::= CLASS {

&id				ProtocolExtensionID			UNIQUE,

&criticality	Criticality,

&Extension,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

EXTENSION		&Extension

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

W1AP-PRIVATE-IES ::= CLASS {

&id				PrivateIE-ID,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {W1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {W1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {W1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				W1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		W1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			W1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {W1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {W1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					W1AP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	W1AP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			W1AP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	W1AP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			W1AP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {W1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {W1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					W1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			W1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		W1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {W1AP-PRIVATE-IES : IEsSetParam } ::=

SEQUENCE (SIZE (1.. maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {W1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					W1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			W1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				W1AP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>