3GPP TSG-RAN WG3 Meeting #115-e	R3-222586

# Online, 21th February - 3rd March 2022	was R3-222061

Agenda Item:	22.2.4

Source:	Ericsson

Title:	[TP for BL CR 38.463] Multicast and Broadcast E1AP functions

Document for:	Discussions & Approval

# 1	Introduction

This TP follows stage 2 description as provided in R3-222060, a TP for the BL CR for TS 38.401.

The principles outlined there are repeated for the readers convenience:

-	separate sets of procedures for multicast and broadcast

-	for broadcast,

-	the setup of shared F1-U and NG-U tunnels is preformed by the same procedure that established/modifies the broadcast context at the DU/gNB, E1 follows accordingly.

-	In case of multiple F1-U/NG-U tunnels (e.g. location dependent sessions), the broadcast context procedures are able to establish all F1-U/NG-U bearers with a single context procedure.

-	for multicast,

-	the setup of shared F1-U and NG-U tunnels is performed by a DU/gNB triggered “distribution” procedure.

-	In case of multiple F1-U/NG-U tunnels (e.g. location dependent sessions), only a single F1-U/NG-U tunnel is established by means of those “distribution” procedures.

-	in case of shared NG-U terminations,

-	the 5GC may provide information about one or several already established NG-U termination that can be shared among gNBs. In return, gNBs may offer NG-U terminations to be shared. This has impact on E1

-	alternatively, the gNB-CU-UP may provide information for which TMGIs pre-configured NG-U terminations are available to be used.

-	the MBS QoS Flow to MRB mapping proposed by the gNB-CU-CP may be overwritten by the gNB-CU-UP in case an NG-U termination is already existing, if the gNB-CU-CP allows it.

# ^2	Text Proposal for TS 38.460 (based on R3-220009)

Start of the First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specificaiton”.

[11]	3GPP TS 23.401: “General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[12]	3GPP TS 23.203: “Policy and Charging Control Architecture”.

[13]	3GPP TS 33.501: “Security Architecture and Procedures for 5G System”.

[14]	IETF RFC 5905: “Network Time Protocol Version 4: Protocol and Algorithms Specification”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 38.414: “NG-RAN; NG Data Transport”.

[17]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TS 38.462: "NG-RAN; E1 Signalling Transport".

[19]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[20]	3GPP TS 23.501: "System Architecture for the 5G System".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[22]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 32.422: "Trace control and configuration management".

[25]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[26]	3GPP TS 32.425: "Performance measurements; Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[27]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[28]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[29]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[x]	3GPP TS 23.247: "5G multicast-broadcast services; Stage 2".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Elementary Procedure: E1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU-CP and gNB-CU-UP. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several E1AP EPs together is specified in stage 2 specifications (e.g., TS 38.460 [3]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

Conditional handover: as defined in TS 38.300 [4].

Conditional PSCell Change: as defined in TS 37.340 [19].

DAPS Handover: as defined in TS 38.300 [4].

gNB: as defined in TS 38.300 [4].

gNB-CU: as defined in TS 38.401 [2].

gNB-DU: as defined in TS 38.401 [2].

gNB-CU-CP: as defined in TS 38.401 [2].

gNB-CU-UP: as defined in TS 38.401 [2].

PDU Session Resource: as defined in TS 38.401 [2].

UE-associated signalling: When E1AP messages associated to one UE uses the UE-associated logical E1-connection for association of the message to the UE in gNB-CU-UP and gNB-CU-CP.

UE-associated logical E1-connection: The UE-associated logical E1-connection uses the identities GNB-CU-CP UE E1AP ID and GNB-CU-UP UE E1AP ID according to the definition in TS 38.401 [2]. For a received UE associated E1AP message the gNB-CU-CP identifies the associated UE based on the GNB-CU-CP UE E1AP ID IE and the gNB-CU-UP identifies the associated UE based on the GNB-CU-UP UE E1AP ID IE.

Public Network Integrated NPN: as defined in TS 23.501 [20].

Stand-alone Non-Public Network: as defined in TS 23.501 [20].

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DL	Downlink

DRB	Data Radio Bearer

E1AP	E1 Application Protocol

IP	Internet Protocol

MBS	Multicast/Broadcast Service

PTP	Point to Point

PTM	Point to Multipoint

SCTP	Stream Control Transmission Protocol

TNL	Transport Network Layer

Start of the Next Change

### 5.1.x	E1 bearer context management function for NR MBS

The E1 bearer context management function for NR MBS consists of two sub-sets for functions, one for NR MBS broadcast, one for NR MBS multicast.

Both sets follow the principles of the E1 bearer context management functions, with the following differences

-	E1 NR MBS procedure concerns a single MBS Session Resource only.

-	E1 NR MBS procedures concern the control of MRB resources in gNB-CU-UP

Editor’s Note:	Specification of gNB-CU-UP initiated procedures for both, multicast and broadcast, are FFS and requires specification text if not defined.

-	E1 NR MBS procedures do not contol security information, as for NR MBS, PDCP does not apply security as specified in TS 38.300 [4].

-	QoS flow to MRB mapping is either determined by the gNB-CU-CP or, in case of shared NR-U terminations, the gNB-CU-UP may be notified about the QoS flow to MRB mapping already determined in the bearer context for the shared NR-U termination. The gNB-CU-CP may inform the gNB-CU-UP whether it is contended with the already determined mapping decision.

NOTE:	Not all QoS flow parameters are applicable for NR MBS, as specified in TS 23.247 [x].

Editor’s Note:	Some specification text for data forwarding, where applicable, to be inserted here.

Editor’s Note:	Header Compression text to be inserted here as well. So far only schemes w/o feedback schemes assumed.

-	DL data arrival detection is not applicable for NR MBS.

-	Data volume reporting is not applicable for NR MBS.

-	Suspension and resumption of bearer contexts is not applicable for NR MBS.

-	CA based packet duplication is not applicable for NR MBS.

Start of the Next Change

## 6.x	NR MBS procedures

The E1 MBS procedures are listed below:

-	Broadcast E1AP MBS procedures

-	BC Bearer Context Setup

-	BC Bearer Context Modification (gNB-CU-CP inititated)

-	BC Bearer Context Modification (gNB-CU-UP inititated) [FFS]

-	BC Bearer Context Release (gNB-CU-CP inititated)

-	BC Bearer Context Release (gNB-CU-UP inititated)

-	Multicast E1AP MBS procedures

-	MC Bearer Context Setup

-	MC Bearer Context Modification (gNB-CU-CP inititated)

-	MC Bearer Context Modification (gNB-CU-UP inititated) [FFS]

-	MC Bearer Context Release (gNB-CU-CP inititated)

-	MC Bearer Context Release (gNB-CU-UP inititated)

End of the Changes

# ^3	Text Proposal for TS 38.463

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specificaiton”.

[11]	3GPP TS 23.401: “General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[12]	3GPP TS 23.203: “Policy and Charging Control Architecture”.

[13]	3GPP TS 33.501: “Security Architecture and Procedures for 5G System”.

[14]	IETF RFC 5905: “Network Time Protocol Version 4: Protocol and Algorithms Specification”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 38.414: “NG-RAN; NG Data Transport”.

[17]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TS 38.462: "NG-RAN; E1 Signalling Transport".

[19]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[20]	3GPP TS 23.501: "System Architecture for the 5G System".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[22]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 32.422: "Trace control and configuration management".

[25]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[26]	3GPP TS 32.425: "Performance measurements; Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[27]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[28]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[29]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[xx]	3GPP TS 23.247: "5G multicast-broadcast services; Stage 2".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Elementary Procedure: E1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU-CP and gNB-CU-UP. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several E1AP EPs together is specified in stage 2 specifications (e.g., TS 38.460 [3]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

Conditional handover: as defined in TS 38.300 [4].

Conditional PSCell Change: as defined in TS 37.340 [19].

DAPS Handover: as defined in TS 38.300 [4].

gNB: as defined in TS 38.300 [4].

gNB-CU: as defined in TS 38.401 [2].

gNB-DU: as defined in TS 38.401 [2].

gNB-CU-CP: as defined in TS 38.401 [2].

gNB-CU-UP: as defined in TS 38.401 [2].

PDU Session Resource: as defined in TS 38.401 [2].

UE-associated signalling: When E1AP messages associated to one UE uses the UE-associated logical E1-connection for association of the message to the UE in gNB-CU-UP and gNB-CU-CP.

UE-associated logical E1-connection: The UE-associated logical E1-connection uses the identities GNB-CU-CP UE E1AP ID and GNB-CU-UP UE E1AP ID according to the definition in TS 38.401 [2]. For a received UE associated E1AP message the gNB-CU-CP identifies the associated UE based on the GNB-CU-CP UE E1AP ID IE and the gNB-CU-UP identifies the associated UE based on the GNB-CU-UP UE E1AP ID IE.

Public Network Integrated NPN: as defined in TS 23.501 [20].

Stand-alone Non-Public Network: as defined in TS 23.501 [20].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

CAG	Closed Access Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CN	Core Network

CP	Control Plane

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

EHC	Ethernet Header Compression

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IAB	Integrated Access and Backhaul

MCG	Master Cell Group

NID	Network Identifier

NPN	Non-Public Network

PNI-NPN	Public Network Integrated Non-Public Network

NSSAI	Network Slice Selection Assistance Information

RANAC	RAN Area Code

SCG	Secondary Cell Group

SDAP	Service Data Adaptation Protocol

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

TNLA	Transport Network Layer Association

# 4	General

## 4.1	Procedure specification principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating node exactly and completely. Any rule that specifies the behaviour of the originating node shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which "shall" be executed.

The procedure text indicates that the receiving node "shall" perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the REQUEST message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which "shall, if supported" be executed.

The procedure text indicates that the receiving node "shall, if supported," perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see clause 10.

## 4.2	Forwards and backwards compatibility

The forwards and backwards compatibility of the protocol is assured by mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word "procedure", e.g. Handover Preparation procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word "message", e.g. HANDOVER REQUEST message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation "IE", e.g. E-RAB ID IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the "Value" is written as it is specified in the specification enclosed by quotation marks, e.g. "Value".

# 5	E1AP services

E1AP provides the signalling service between the gNB-CU-CP and the gNB-CU-UP that is required to fulfil the E1AP functions described in clause 7. E1AP services are divided into two groups:

Non UE-associated services:	They are related to the whole E1 interface instance between the gNB-CU-CP and gNB-CU-UP utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. E1AP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

MBS-associated services:	They are related to one MBS session. E1AP functions that provide these services are associated with an MBS-associated signalling connection that is maintained for the MBS sessioni in question.

Unless explicitly indicated in the procedure specification, at any instance in time one protocol endpoint shall have a maximum of one ongoing E1AP procedure related to a certain UE.

# 6	Services expected from signalling transport

The signalling connection shall provide in sequence delivery of E1AP messages. E1AP shall be notified if the signalling connection breaks.

# 7	Functions of E1AP

The functions of E1AP are described in TS 38.460 [3].

# 8	E1AP procedures

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| BC Bearer Context Setup |  | BC BEARER CONTEXT SETUP REQUEST |  | BC BEARER CONTEXT SETUP RESPONSE |  | BC BEARER CONTEXT SETUP FAILURE |  |
| BC Bearer Context Modification (gNB-CU-CP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUEST |  | BC BEARER CONTEXT MODIFICATION RESPONSE |  | BC BEARER CONTEXT MODIFICATION FAILURE |  |
| BC Bearer Context Modification Required (gNB-CU-UP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUIRED |  | BC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| BCBearer Context Release (gNB-CU-CP initiated) |  | BC BEARER CONTEXT RELEASE COMMAND |  | BC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| MC Bearer Context Setup |  | MC BEARER CONTEXT SETUP REQUEST |  | MC BEARER CONTEXT SETUP RESPONSE |  | BC BEARER CONTEXT SETUP FAILURE |  |
| MC Bearer Context Modification (gNB-CU-CP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUEST |  | MC BEARER CONTEXT MODIFICATION RESPONSE |  | BC BEARER CONTEXT MODIFICATION FAILURE |  |
| MC Bearer Context Modification Required (gNB-CU-UP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUIRED |  | MC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| MC Bearer Context Release (gNB-CU-CP initiated) |  | MC BEARER CONTEXT RELEASE COMMAND |  | MC BEARER CONTEXT RELEASE COMPLETE |  |  |  |

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |
| BC Bearer Context Release (gNB-CU-UP inititated) |  |  | BC BEARER CONTEXT RELEASE REQUEST |  |  |
| BC Bearer Context Release (gNB-CU-UP inititated) |  |  | MC BEARER CONTEXT RELEASE REQUEST |  |  |

## 8.2	Interface Management procedures

### 8.2.1	Reset

#### 8.2.1.1	General

The purpose of the Reset procedure is to initialise or re-initialise the E1AP UE-related contexts, in the event of a failure in the gNB-CU-CP or gNB-CU-UP. This procedure does not affect the application level configuration data exchanged during, e.g., the E1 Setup procedure.

The procedure uses non-UE associated signalling.

#### 8.2.1.2	Successful Operation

##### 8.2.1.2.1	Reset Procedure Initiated from the gNB-CU-CP

Figure 8.2.1.2.1-1: Reset procedure initiated from the gNB-CU-CP. Successful operation.

In the event of a failure at the gNB-CU-CP, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the gNB-CU-UP.

At reception of the RESET message the gNB-CU-UP shall release all allocated resources on E1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated bearer contexts including E1AP ID.

After the gNB-CU-UP has released all assigned E1 resources and the UE E1AP IDs for all indicated UE associations which can be used for new UE-associated logical E1-connections over the E1 interface, the gNB-CU-UP shall respond with the RESET ACKNOWLEDGE message. The gNB-CU-UP does not need to wait for the release of bearer resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical E1-connection list IE, then:

-	The gNB-CU-UP shall use the gNB-CU-CP UE E1AP ID IE and/or the gNB-CU-UP UE E1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The gNB-CU-UP shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical E1-connection Item IE in the UE-associated logical E1-connection list IE. The UE-associated logical E1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical E1-connections. Empty UE-associated logical E1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the gNB-CU-CP UE E1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the gNB-CU-UP shall include the gNB-CU-CP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the gNB-CU-UP UE E1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the gNB-CU-UP shall include the gNB-CU-UP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same E1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

##### 8.2.1.2.2	Reset Procedure Initiated from the gNB-CU-UP

Figure 8.2.1.2.2-1: Reset procedure initiated from the gNB-CU-UP. Successful operation.

In the event of a failure at the gNB-CU-UP, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the gNB-CU-CP.

At reception of the RESET message the gNB-CU-CP shall release all allocated resources on E1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the E1AP ID for the indicated UE associations.

After the gNB-CU-CP has released all assigned E1 resources and the UE E1AP IDs for all indicated UE associations which can be used for new UE-associated logical E1-connections over the E1 interface, the gNB-CU-CP shall respond with the RESET ACKNOWLEDGE message. The gNB-CU-CP does not need to wait for the release of bearer resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical E1-connection list IE, then:

-	The gNB-CU-CP shall use the gNB-CU-CP UE E1AP ID IE and/or the gNB-CU-UP UE E1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The gNB-CU-CP shall in the RESET ACKNOWLEDGE message include, for each UE association to be reset, the UE-associated logical E1-connection Item IE in the UE-associated logical E1-connection list IE. The UE-associated logical E1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical E1-connections. Empty UE-associated logical E1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the gNB-CU-CP UE E1AP ID IE is included in the UE-associated logical E1-connection Item IE for a UE association, the gNB-CU-CP shall include the gNB-CU-CP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the gNB-CU-UP UE E1AP ID IE is included in a UE-associated logical E1-connection Item IE for a UE association, the gNB-CU-CP shall include the gNB-CU-UP UE E1AP ID IE in the corresponding UE-associated logical E1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same E1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

#### 8.2.1.3	Abnormal Conditions

Not applicable.

### 8.2.2	Error Indication

#### 8.2.2.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: Error Indication procedure, gNB-CU-CP originated. Successful operation.

Figure 8.2.2.2-2: Error Indication procedure, gNB-CU-UP originated. Successful operation.

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the gNB-CU-CP UE E1AP ID IE and gNB-CU-UP UE E1AP ID IE shall be included in the ERROR INDICATION message. If one or both of the gNB-CU-CP UE E1AP ID IE and the gNB-CU-UP UE E1AP ID IE are not correct, the cause shall be set to appropriate value, e.g., "Unknown or already allocated gNB-CU-CP UE E1AP ID", "Unknown or already allocated gNB-CU-UP UE E1AP ID" or "Unknown or inconsistent pair of UE E1AP ID".

#### 8.2.2.3	Abnormal Conditions

Not applicable.

### 8.2.3	gNB-CU-UP E1 Setup

#### 8.2.3.1	General

The purpose of the gNB-CU-UP E1 Setup procedure is to exchange application level data needed for the gNB-CU-UP and the gNB-CU-CP to correctly interoperate on the E1 interface. If the gNB-CU-UP initiates the first TNL association, it shall also initiate the gNB-CU-UP E1 Setup procedure. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: gNB-CU-UP E1 Setup procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-CP. The gNB-CU-CP responds with a GNB-CU-UP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-UP E1 SETUP REQUEST message contains the gNB-CU-UP Name IE the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP. If the GNB-CU-UP E1 SETUP REQUEST message contains the Extended gNB-CU-UP Name IE, the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP and shall ignore the gNB-CU-UP Name IE if included.

If the GNB-CU-UP E1 SETUP RESPONSE message contains the gNB-CU-CP Name IE, the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP. If the GNB-CU-UP E1 SETUP RESPONSE message contains the Extended gNB-CU-CP Name IE, the GNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP and shall ignore the gNB-CU-CP Name IE if included.

If the Slice Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NR CGI Support List or the Extended NR CGI Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the QoS Parameters Support List IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NPN Support Information IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP E1 SETUP REQUEST message, the gNB-CU-CP shall take this IE into account.

If the GNB-CU-UP E1 SETUP REQUEST message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-UP E1 SETUP RESPONSE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: gNB-CU-UP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the setup, it shall respond with a GNB-CU-UP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-UP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-UP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-CP.

#### 8.2.3.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-CP E1 SETUP REQUEST, GNB-CU-UP E1 SETUP RESPONSE, or GNB-CU-UP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-UP does not receive either GNB-CU-UP E1 SETUP RESPONSE message or GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP may reinitiate the gNB-CU-UP E1 Setup procedure towards the same gNB-CU-CP, provided that the content of the new GNB-CU-UP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-UP E1 SETUP REQUEST message.

If the gNB-CU-UP receives a GNB-CU-CP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-UP E1 SETUP FAILURE message, the gNB-CU-UP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.3.3.

-	In case the gNB-CU-UP answers with a GNB-CU-CP E1 SETUP FAILURE message and receives a subsequent GNB-CU-UP E1 SETUP RESPONSE message, the gNB-CU-UP shall ignore the GNB-CU-UP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

### 8.2.4	gNB-CU-CP E1 Setup

#### 8.2.4.1	General

The purpose of the gNB-CU-CP E1 Setup procedure is to exchange application level data needed for the gNB-CU-CP and the gNB-CU-UP to correctly interoperate on the E1 interface. If the gNB-CU-CP initiates the first TNL association, it shall also initiate the gNB-CU-CP E1 Setup procedure.The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the E1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-CU-CP E1 Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP E1 SETUP REQUEST message including the appropriate data to the gNB-CU-UP. The gNB-CU-UP responds with a GNB-CU-CP E1 SETUP RESPONSE message including the appropriate data.

If the GNB-CU-CP E1 SETUP REQUEST message contains the gNB-CU-CP Name IE the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP. If the GNB-CU-CP E1 SETUP REQUEST message contains the Extended gNB-CU-CP Name IE, the gNB-CU-UP may use this IE as a human readable name of the gNB-CU-CP and shall ignore the gNB-CU-CP Name IE if included.

If the GNB-CU-CP E1 SETUP RESPONSE message contains the gNB-CU-UP Name IE, the gNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP. If the GNB-CU-CP E1 SETUP RESPONSE message contains the Extended gNB-CU-UP Name IE, the GNB-CU-CP may use this IE as a human readable name of the gNB-CU-UP and shall ignore the gNB-CU-UP Name IE if included.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the E1 interface is operational and other E1 messages can be exchanged.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall take this IE into account.

If the GNB-CU-CP E1 SETUP REQUEST message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-CP E1 SETUP RESPONSE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the NPN Support Information IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

If the NR CGI Support List or the Extended NR CGI Support List IE is contained in the GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall store the corresponding information and it may take it into account for bearer context establishment.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-CU-CP E1 Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot accept the setup, it shall respond with a GNB-CU-CP E1 SETUP FAILURE and appropriate cause value.

If the GNB-CU-CP E1 SETUP FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the E1 setup towards the same gNB-CU-UP.

#### 8.2.4.4	Abnormal Conditions

If the first message received for a specific TNL association is not a GNB-CU-UP E1 SETUP REQUEST, GNB-CU-CP E1 SETUP RESPONSE, or GNB-CU-CP E1 SETUP FAILURE message then this shall be treated as a logical error.

If the gNB-CU-CP does not receive either GNB-CU-CP E1 SETUP RESPONSE message or GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP may reinitiate the gNB-CU-CP E1 Setup procedure towards the same gNB-CU-UP, provided that the content of the new GNB-CU-CP E1 SETUP REQUEST message is identical to the content of the previously unacknowledged GNB-CU-CP E1 SETUP REQUEST message.

If the gNB-CU-CP receives a GNB-CU-UP E1 SETUP REQUEST message from the peer entity on the same E1 interface:

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP RESPONSE message and receives a subsequent GNB-CU-CP E1 SETUP FAILURE message, the gNB-CU-CP shall consider the E1 interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.2.4.3.

-	In case the gNB-CU-CP answers with a GNB-CU-UP E1 SETUP FAILURE message and receives a subsequent GNB-CU-CP E1 SETUP RESPONSE message, the gNB-CU-CP shall ignore the GNB-CU-CP E1 SETUP RESPONSE message and consider the E1 interface as non operational.

### 8.2.5	gNB-CU-UP Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU-UP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-UP and the gNB-CU-CP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU-UP Configuration Update procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a GNB-CU-UP CONFIGURATION UPDATE message to the gNB-CU-CP including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU-CP responds with GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall interpret that the corresponding configuration data is not changed and shall continue to operate with the existing related configuration data.

If the Supported PLMNs IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall overwrite the whole list of information and store the corresponding information.

-	If the Slice Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the NR CGI Support List or the Extended NR CGI Extended Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the QoS Parameters Support List IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

-	If the NPN Support Information IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall store the corresponding information and replace any existing information.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-UP Capacity IE is contained in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall take this IE into account.

If the gNB-CU-UP ID IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall associate the TNLA to the E1 interface instance using the gNB-CU-UP ID.

If the gNB-CU-UP Name IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU-UP. If the Extended gNB-CU-UP Name IE is included in the GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU-UP and shall ignore the gNB-CU-UP Name IE if also included.

If the GNB-CU-UP CONFIGURATION UPDATE message includes gNB-CU-UP TNLA To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-UP TNLA To Remove List IE, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by both received TNL endpoints will be removed by the gNB-CU-UP. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU-UP TNLA To Remove List IE in GNB-CU-UP CONFIGURATION UPDATE message, the gNB-CU-CP shall, if supported, consider that the TNL association(s) indicated by the received endpoint IP address(es) will be removed by the gNB-CU-UP.

If the GNB-CU-UP CONFIGURATION UPDATE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU-UP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-CP cannot accept the update, it shall respond with a GNB-CU-UP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-UP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-UP shall wait at least for the indicated time before reinitiating the GNB-CU-UP CONFIGURATION UPDATE message towards the same gNB-CU-CP.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

### 8.2.6	gNB-CU-CP Configuration Update

#### 8.2.6.1	General

The purpose of the gNB-CU-CP Configuration Update procedure is to update application level configuration data needed for the gNB-CU-CP and the gNB-CU-UP to interoperate correctly on the E1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: gNB-CU-CP Configuration Update procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP CONFIGURATION UPDATE message to the gNB-CU-UP including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU-UP responds with GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP shall interpret that the corresponding configuration data is not changed and shall continue to operate with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If the gNB-CU-CP Name IE is included in the GNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU-CP. If the Extended gNB-CU-CP Name IE is included in the GNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU-CP and shall ignore the gNB-CU-CP Name IE if also included.

If the gNB-CU-CP TNLA To Add List IE is contained in the gNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP shall, if supported, use it to establish the TNL association(s) with the gNB-CU-CP. The gNB-CU-UP shall report to the gNB-CU-CP, in the gNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU-CP as follows:

-	A list of TNL address(es) with which the gNB-CU-UP successfully established the TNL association shall be included in the gNB-CU-CP TNLA Setup List IE;

-	A list of TNL address(es) with which the gNB-CU-UP failed to establish the TNL association shall be included in the gNB-CU-CP TNLA Failed To Setup List IE.

If the GNB-CU-CP CONFIGURATION UPDATE message includes gNB-CU-CP TNLA To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU-CP TNLA To Remove List IE, the gNB-CU-UP shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU-CP. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU-CP TNLA To Remove List IE, the gNB-CU-UP shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU-CP TNLA To Update List IE is contained in the gNB-CU-CP CONFIGURATION UPDATE message the gNB-CU-UP shall, if supported, overwrite the previously stored information for the related TNL association.

If the TNLA Usage IE is included in the gNB-CU-CP TNLA To Add List IE or the gNB-CU-CP TNLA To Update List IE in the gNB-CU-CP CONFIGURATION UPDATE message, the gNB-CU-UP shall, if supported, use it as described in TS 38.462 [18].

If the GNB-CU-CP CONFIGURATION UPDATE message includes the Transport Network Layer Address Info IE, the gNB-CU-UP shall, if supported, take this IE into account for IPSec tunnel establishment.

If the GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE message includes the Transport Network Layer Address Info IE, the gNB-CU-CP shall, if supported, take this IE into account for IPSec tunnel establishment.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: gNB-CU-CP Configuration Update procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot accept the update, it shall respond with a GNB-CU-CP CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU-CP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated time before reinitiating the GNB-CU-CP CONFIGURATION UPDATE message towards the same gNB-CU-UP.

#### 8.2.6.4	Abnormal Conditions

Not applicable.

### 8.2.7	E1 Release

#### 8.2.7.1	General

The purpose of the E1 Release procedure is to release all existing signalling connections and related application level data. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.7.2	Successful Operation

##### 8.2.7.2.1	E1 Release Procedure Initiated from the gNB-CU-CP

Figure 8.2.7.2.1-1: E1 Release procedure initiated from the gNB-CU-CP. Successful operation.

The gNB-CU-CP initiates the procedure by sending the E1 RELEASE REQUEST message to the gNB-CU-UP.

Upon reception of the E1 RELEASE REQUEST message, the gNB-CU-UP shall release any existing resources related to the E1 interface. The gNB-CU-UP shall respond with a E1 RELEASE RESPONSE message to confirm that it has initiated the release of the resources, if existing, and that the signalling connection for the E1AP application protocol is released.

##### 8.2.7.2.2	E1 Release Procedure Initiated from the gNB-CU-UP

Figure 8.2.7.2.2-1: E1 Release procedure initiated from the gNB-CU-UP. Successful operation.

The gNB-CU-UP initiates the procedure by sending the E1 RELEASE REQUEST message to the gNB-CU-CP.

Upon reception of the E1 RELEASE REQUEST message, the gNB-CU-CP shall release any existing resources related to the E1 interface. The gNB-CU-CP shall respond with a E1 RELEASE RESPONSE message to confirm that it has initiated the release of the resources, if existing, and that the signalling connection for the E1AP application protocol is released.

#### 8.2.7.3	Abnormal Conditions

Not applicable.

### 8.2.8	gNB-CU-UP Status Indication

#### 8.2.8.1	General

The purpose of the gNB-CU-UP Status Indication procedure is to inform the gNB-CU-CP that the gNB-CU-UP is overloaded so that overload reduction actions can be applied. The procedure uses non-UE associated signalling.

#### 8.2.8.2	Successful Operation

Figure 8.3.7.2-1: DL Data Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the GNB-CU-UP STATUS INDICATION message to the gNB-CU-CP.

If the gNB-CU-UP Overload Information IE in the GNB-CU-UP STATUS INDICATION message indicates that the gNB-CU-UP is overloaded, the gNB-CU-CP shall apply overload reduction actions until informed, with a new GNB-CU-UP STATUS INDICATION message, that the overload situation has ceased.

The detailed overload reduction policy is up to gNB-CU-CP implementation.

#### 8.2.8.3	Abnormal Conditions

Not applicable.

### 8.2.9	Resource Status Reporting Initiation

#### 8.2.9.1	General

This procedure is used by an gNB-CU-CP to request the reporting of load measurements to gNB-CU-UP.

The procedure uses non UE-associated signalling.

#### 8.2.9.2	Successful Operation

Figure 8.2.9.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from gNB-CU-CP to gNB-CU-UP to start a measurement or stop a measurements.

If gNB-CU-UP is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU-CP, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the RESOURCE STATUS UPDATE message:

-	the HW Capacity Indicator IE, if the second bit, "HW Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the TNL Available Capacity Indicator IE, if the first bit, " TNL Available Capacity Ind Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

If the Reporting Periodicity IE is included in the RESOURCE STATUS REQUEST message, this indicates the periodicity for the reporting of periodic measurements. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.9.3	Unsuccessful Operation

Figure 8.2.9.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-CU-UP shall send a RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.9.4	Abnormal Conditions

If the initiating gNB-CU-CP does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU-CP may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-CU-UP, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-CU-UP shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-CU-UP receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU-CP Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-CU-UP shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

### 8.2.10	Resource Status Reporting

#### 8.2.10.1	General

This procedure is initiated by gNB-CU-UP to report the result of measurements admitted by gNB-CU-UP following a successful Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting, successful operation

The gNB-CU-UP shall report the results of the admitted measurements in RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Resource Status Reporting Initiation procedure.

#### 8.2.10.3	Unsuccessful Operation

Not applicable.

#### 8.2.10.4	Abnormal Conditions

Void.

## 8.3	Bearer Context Management procedures

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT SETUP RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	For each established PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each established PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each established DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each established DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Existing Allocated NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may re-use the indicated resources already allocated for this bearer context. If the gNB-CU-UP decides to re-use the indicated resources, it shall include the NG DL UP Unchanged IE in the BEARER CONTEXT SETUP RESPONSE message.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store and use the information for the down link traffic policing for the Non-GBR QoS flows for the concerned UE as specified in TS 23.501 [20].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT SETUP RESPONSE message.

If the DL UP Parameters IE is contained in the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall configure the corresponding information.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT SETUP RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT SETUP REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

For each PDU session, if the Data Forwarding to E-UTRAN Information List IE is included in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it for inter-system data forwarding from 5GS to EPS as specified in TS38.300 [8].

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall use this value when enforcing the maximum integrity protected data rate for the UE.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

For each requested DRB, if the PDCP Duplication IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, then the gNB-CU-UP shall include two UP Transport Layer Information IEs in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication. If only one cell group is included in the Cell Group Information IE for the concerned DRB, then the gNB-CU-UP shall consider that the first UP Transport Layer Information IE of the two UP Transport Layer Information IEs is for the primary path.

For each requested DRB, if the Additional PDCP duplication Information IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, then the gNB-CU-UP shall, if supported, include the same number of UP Transport Layer Information IEs indicated by the Additional PDCP duplication Information IE in the BEARER CONTEXT SETUP RESPONSE message to support packet duplication. If only one cell group is included in the Cell Group Information IE for the concerned DRB, then the gNB-CU-UP shall consider that the first UP Transport Layer Information IE of these UP Transport Layer Information IEs is for the primary path. If more than one cell group is included in the Cell Group Information IE, then the gNB-CU-UP shall consider that the number of duplication tunnels for each cell group is indicated by the Number of tunnels IE, and that the first UP Transport Layer Information IE for each cell group is for the primary path or the split secondary path.

If the PDCP SN Status Information IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the QoS Flow Mapping Indication IE is contained in the QoS Flows Information To Be Setup IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the QoS Flows Remapping IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message for a DRB and set to "update", the gNB-CU-UP shall, if supported, consider that QoS flows mapped for the DRB is updated to the QoS flow(s) included in the QoS Flows Information To Be Setup IE after finishing handling forwarded PDCP SDUs during an intra-system handover procedure. If the QoS Flows Remapping IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message for a DRB and set to "source configuration", the gNB-CU-UP shall, if supported, consider that no QoS flow is mapped to the DRB after finishing handling forwarded PDCP SDUs over that DRB during an intra-system handover procedure and ignore the information included in the QoS Flows Information To Be Setup IE for the concerned DRB.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Redundant NG UL UP Transport Layer Information IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it as the uplink termination point of the redundant tunnel for the user plane data of those QoS flows in this PDU session which need redundant transmission as described in TS 23.501 [20], and it shall include the Redundant NG DL UP Transport Layer Information IE in the PDU Session Resource Setup List IE in the BEARER CONTEXT SETUP RESPONSE message.

For each PDU Session Resource, if the Redundant Common Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [20].

For each PDU session, if the Redundant QoS Flow Indicator IE is included in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, consider it for the redundant transmission.

For each PDU session, if the Redundant PDU Session Information IE is included in the PDU Session Resource To Setup List IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, set up the redundant user plane resources, as specified in TS 23.501 [20] and include, if supported, the Used Redundant PDU Session Information IE in the PDU Session Resource Setup List IE in the BEARER CONTEXT SETUP RESPONSE message.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take it into account as specified in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store the information received.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT SETUP RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT SETUP REQUEST message.

If the Trace Activation IE is included in the BEARER CONTEXT SETUP REQUEST message the gNB-CU-UP shall, if supported, initiate the requested trace function as described in TS 32.422 [24]. In particular, the gNB-CU-UP shall, if supported:

-	if the MDT Activation IE is set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [24] and the gNB-CU-UP shall ignore Interfaces To Trace IE, and Trace Depth IE;

-	if the MDT Activation IE is set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [24];

If the Management Based MDT PLMN List IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store the received information, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [24].

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the BEARER CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the BEARER CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

If the TSC Traffic Characteristics IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store this information, and, if supported, use it for RAN part delay reporting.

If the BEARER CONTEXT SETUP REQUEST message contains the NPN Context Information IE the gNB-CU-UP shall, if supported, take it into account when allocating UP resources for the bearer context.

For each requested DRB, if the EHC Parameters IE is included in the PDCP Configuration IE, the gNB-CU-CP shall, if supported, also include ROHC Parameters IE in the PDCP Configuration IE in the BEARER CONTEXT SETUP REQUEST message, to enable the gNB-CU-UP to perform appropriate header compression.

If the EHC parameters IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take these parameters into account to perform appropriate header compression for the concerned DRB.

If the DAPS Request Information IE is included for a DRB to be setup in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns a DAPS handover for that DRB and, if admitted, act as specified in TS 38.300 [4].

If the CHO Initiation IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns conditional handover or conditional PSCell change and act as specified in TS 38.401 [2].

If the MCG Offered GBR QoS Flow Information IE is contained in the QoS Flows Information To Be Setup IE within the DRB To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take it into account when two cell groups are served by the gNB-CU-UP.

If the Additional Handover Information IE is included in the BEARER CONTEXT SETUP REQUEST message and set to “Discard PDCP SN”, the gNB-CU-UP shall, if supported, remove the forwarded PDCP SNs if received in the forwarded GTP-U packets, and deliver the forwarded PDCP SDUs to the UE, as specified in TS 38.300 [8].

If the Ignore Mapping Rule Indication IE is contained within the DRB To Setup List IE for a DRB in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, ignore the QoS flow mapping information indicated by the QoS Flows Information To Be Setup IE for the concerned DRB.

If the Direct Forwarding Path Availability IE set to "inter-system direct path available" is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, assign the UP Transport Layer Information for inter-system direct data forwarding from the appropriate address space, if applicable.

If the Direct Forwarding Path Availability IE set to “intra-system direct path available” is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, assign the UP Transport Layer Information for intra-system direct data forwarding from the appropriate address space, if applicable.

If the gNB-CU-UP UE E1AP ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may use it to identify the UE context as specified in TS 38.401 [2].

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Bearer Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested bearer context, or cannot even establish one bearer it shall consider the procedure as failed and respond with a BEARER CONTEXT SETUP FAILURE message and appropriate cause value.

#### 8.3.1.4	Abnormal Conditions

If the gNB-CU-UP receives a BEARER CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE in the DRB To Setup List IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-CU-UP shall report the establishment of the corresponding DRB as failed in the DRB Failed List IE of the BEARER CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT SETUP REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource To Setup List IE for a GBR QoS Flow but where the GBR QoS Flow Information IE is not present, the gNB-CU-UP shall report the establishment of the corresponding QoS Flow as failed in the corresponding  Flow Failed List IE of the BEARER CONTEXT SETUP RESPONSE message with an appropriate cause value.

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully modified shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be modified shall be included in the PDU Session Resource Failed To Modify List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Security Information IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Aggregate Maximum Bit Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2].

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDU Session Data Forwarding Information IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, consider that data forwarding is applicable for the indicated QoS flows for the concerned PDU session.

If the PDCP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information, except for the PDCP SN UL Size IE, the PDCP SN DL Size IE and the RLC mode IE which shall be ignored.

If the E-UTRAN QoS IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDCP SN Status Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall act as specified in TS 38.401 [2] and include the UL COUNT Value IE and the DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP SN Status Information IE is contained in the DRB To Setup List IE or the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the DL UP Parameters IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Cell Group To Add IE or the Cell Group To Modify IE or the Cell Group To Remove IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall add or modify or remove the corresponding cell group.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501 [20].

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the SDAP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Flow Mapping Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

For each requested DRB, if the PDCP Duplication IE or Additional PDCP duplication Information IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, then the gNB-CU-CP shall include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION REQUEST message, and the gNB-CU-UP shall, if supported, also include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION RESPONSE message to support packet duplication. If only one cell group is included in the Cell Group Information IE for the concerned DRB, then the gNB-CU-UP shall consider that the first UP Transport Layer Information IE of these UP Transport Layer Information IEs is for the primary path. If more than one cell group is included in the Cell Group Information IE, then the gNB-CU-UP shall consider that the number of duplication tunnels for each cell group is indicated by the Number of tunnels IE, and that the first UP Transport Layer Information IE for each cell group is for the primary path or the split secondary path.

For a certain DRB which was allocated with two or more GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the Bearer Context Modification (gNB-CU-CP initiated) procedure, i.e. only one UP Transport Layer Information per Cell Group ID is present in DL UP Parameters IE for the concerned DRB, then the gNB-CU-UP shall consider that PDCP duplication is deconfigured for this DRB. If such Bearer Context Modification (gNB-CU-CP initiated) procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the New UL TNL Information Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the new UP Transport Layer Information in the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE of the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List of the BEARER CONTEXT MODIFICATION REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Redundant NG UL UP Transport Layer Information IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, include the Redundant NG DL UP Transport Layer Information IE in the PDU Session Resource Setup List IE or the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Redundant Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [20].

For each PDU session for which the Redundant QoS Flow Indicator IE is included in QoS Flows Information To Be Setup IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if support, shall store and use it as specified in TS 23.501 [20].

For each PDU session, if the Redundant QoS Flow Indicator IE is set to false for all QoS flows, the gNB-CU-UP shall, if supported, stop the redundant transmission and release the redundant tunnel for the concerned PDU session as specified in TS 23.501 [20].

If the QoS Flow Mapping Indication IE is contained in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Data Discard Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to “Required”, the gNB-CU-UP shall consider that a RAN Paging Failure occurred for that UE. The gNB-CU-UP shall discard the user plane data for that UE and consider that the bearer context is still suspended.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take it into account for each DRB, as specified in TS 28.552 [22].

If the DRB QoS IE is contained within the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account for each DRB, as specifed in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message including Activity Notification Level IE and its value does not match the current bearer context, the gNB-CU-UP shall ignore the Activity Notification Level IE and also the requested modification of inactivity timer.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT MODIFICATION RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT MODIFICATION REQUEST message.

If the Old QoS Flow List - UL End Marker expected IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message for a DRB to be modified, the gNB-CU-UP shall consider that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. The gNB-CU-UP shall consider that the Old QoS Flow List - UL End Marker expected  IE only contains UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side.

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the BEARER CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the BEARER CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

If there is at least one DRB removed by the gNB-CU-UP, the gNB-CU-UP shall, if supported, include the Retainability Measurements Information IE in the BEARER CONTEXT MODIFICATION RESPONSE message, providing information on the removed DRB(s) for retainability measurements in the gNB-CU-CP, as described in TS 32.425 [26] and TS 28.552 [22].

If the TSC Traffic Characteristics IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, use it for RAN part delay reporting.

For each requested DRB, if the QoS Mapping Information IE is contained in the DL UP Parameters IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall use it to set DSCP and/or flow label fields in the downlink IP packets which are transmitted through the GTP tunnels indicated by the UP Transport Layer Information IE. The Diffserv code point (DSCP) marking is performed as specified in TS 38.474 [28].

If the Early Forwarding COUNT Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall act as specified in TS 38.401 [2] and include the requested FIRST DL COUNT Value IE or DISCARD DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Early Forwarding COUNT Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the Ignore Mapping Rule Indication IE is contained within the DRB To Setup List IE for a DRB in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, ignore the QoS flow mapping information indicated by the QoS Flows Information To Be Setup IE for the concerned DRB.

If the DAPS Request Information IE is included for a DRB to be modified in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider that the request concerns a DAPS handover for that DRB and, if admitted, act as specified in TS 38.300 [4].

If the Early Data Forwarding Indicator IE set to “stop” is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported and if already initiated, stop the early data forwarding for the concerned DRB. If the DRB Data forwarding information IE containing the DL Data Forwarding IE is included together in the DRB To Modify List IE, the gNB-CU-UP shall consider that the stop is only for the early data forwarding initiated toward that forwarding TNL.

Interaction with the Bearer Context Modification (gNB-CU-CP initiated)

If the BEARER CONTEXT MODIFICATION REQUEST message includes for a DRB in the DRB To Modify List IE the PDCP SN Status Request IE set to “requested” and if the gNB-CU-UP has not yet received a SDAP end marker packet for a QoS flow which has been previously re-configured to another DRB by means of a gNB-CU-CP initiated Bearer Context Modification procedure, the gNB-CU-UP shall includes the QoS Flow Identifier of that QoS flow in the Old QoS Flow List - UL End Marker expected IE in the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

#### 8.3.2.3	Unsuccessful Operation

Figure 8.3.2.3-1: Bearer Context Modification procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested bearer context modifications, it shall respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

#### 8.3.2.4	Abnormal Conditions

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE in the DRB To Setup List or the DRB To Modify List IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-CU-UP shall report the addition or the modification of the corresponding DRB as failed in the DRB Failed List IE or the DRB Failed To Modify List IE of the BEARER CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE for a GBR QoS Flow but where the GBR QoS Flow Information IE is not present, the gNB-CU-UP shall report the addition or the modification of the corresponding QoS Flow as failed in the corresponding  Flow Failed List IE of the BEARER CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

### 8.3.3	Bearer Context Modification Required (gNB-CU-UP initiated)

#### 8.3.3.1	General

The purpose of the Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a bearer context (e.g., due to local problems) and inform the gNB-CU-CP. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: Bearer Context Modification Required procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies with the BEARER CONTEXT MODIFICATION CONFIRM message.

If the S1 DL UP Transport Layer Information IE or the NG DL UP Transport Layer Information IE or the Redundant NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall update the corresponding information.

If the gNB-CU-UP Cell Group Related Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall try to change the cell group related configuration accordingly. If the gNB-CU-CP is not able to update the requested cell group related configuration, it shall include the Cell Group Information IE with the current cell group configuration in the DRB Modified List IE in the BEARER CONTEXT MODIFICATION CONFIRM message.

#### 8.3.3.3	Abnormal Conditions

Not applicable.

### 8.3.4	Bearer Context Release (gNB-CU-CP initiated)

#### 8.3.4.1	General

The purpose of the Bearer Context Release procedure is to allow the gNB-CU-CP to command the release of an UE-associated logical E1 connection. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: Bearer Context Release procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT RELEASE COMMAND message to the gNB-CU-UP. The gNB-CU-UP replies with the BEARER CONTEXT RELEASE COMPLETE message.

Upon reception of the BEARER CONTEXT RELEASE COMMAND message, the gNB-CU-UP shall release all related signalling and user data transport resources and reply with the BEARER CONTEXT RELEASE COMPLETE message.

The gNB-CU-UP shall, if supported, include the Retainability Measurements Information IE in the BEARER CONTEXT RELEASE COMPLETE message, providing information on the removed DRB(s) for retainability measurements in the gNB-CU-CP, as described in TS 32.425 [26] and TS 28.552 [22].

#### 8.3.4.3	Abnormal Conditions

Not applicable.

### 8.3.5	Bearer Context Release Request (gNB-CU-UP initiated)

#### 8.3.5.1	General

The purpose of the Bearer Context Release Request procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to release an UE-associated logical E1 connection. The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: Bearer Context Release Requset procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT RELEASE REQUEST message to the gNB-CU-CP.

If the DRB Status List IE is included in the BEARER CONTEXT RELEASE REQUEST message, the gNB-CU-CP shall act as specified in TS 38.401 [2].

Interactions with Bearer Context Release procedure:

The Bearer Context Release (gNB-CU-CP initiated) procedure may be initiated upon reception of a BEARER CONTEXT RELEASE REQUEST message.

Interaction with Bearer Context Modification (gNB-CU-CP initiated) procedure:

If applicable, as specified in TS 38.401 [2], the gNB-CU-UP may receive, after having performed the Bearer Context Release Request (gNB-CU-UP initiated) procedure, the BEARER CONTEXT MODIFICATION REQUEST message including the Data Forwarding Information Request IE within the DRBs To Modify List IE.

#### 8.3.5.3	Abnormal Conditions

Not applicable.

### 8.3.6	Bearer Context Inactivity Notification

#### 8.3.6.1	General

This procedure is initiated by the gNB-CU-UP to indicate the inactivity/resumption of activity related to the UE. The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: Bearer Context Inactivity Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT INACTIVITY NOTIFICATION message to the gNB-CU-CP.

If the Activity Notification Level was set to “DRB” during the Bearer Context establishment, the gNB-CU-UP shall include the DRB Activity List IE in the BEARER CONTEXT INACTIVITY NOTIFICATION message.

If the Activity Notification Level was set to “PDU Session” during the Bearer Context establishment, the gNB-CU-UP shall include the PDU Session Resource Activity List IE in the BEARER CONTEXT INACTIVITY NOTIFICATION message.

If the Activity Notification Level was set to “UE” during the Bearer Context establishment, the gNB-CU-UP shall include the UE Activity IE in the BEARER CONTEXT INACTIVITY NOTIFICATION message.

#### 8.3.6.3	Abnormal Conditions

Not applicable.

### 8.3.7	DL Data Notification

#### 8.3.7.1	General

This procedure is initiated by the gNB-CU-UP to indicate the detection of DL data arrival for the UE, or indicate that a DL packet including a QFI value in the NG-U header not configured by the QoS Flows Information To Be Setup IE or the Flow Mapping Information IE is received for the first time. The procedure uses UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: DL Data Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the DL DATA NOTIFICATION message to the gNB-CU-CP.

If the PPI IE is included in the DL DATA NOTIFICATION message, the gNB-CU-CP shall use it for paging policy differentiation.

If the PDU Session To Notify List IE is included in the DL DATA NOTIFICATION message, the gNB-CU-CP shall, if supported, either map the flow(s) included in PDU Session To Notify List IE to the existing DRB or establish a new DRB for the flow(s).

#### 8.3.7.3	Abnormal Conditions

Not applicable.

### 8.3.8	Data Usage Report

#### 8.3.8.1	General

This procedure is initiated by the gNB-CU-UP to report data volume served at the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Data Usage Report procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the DATA USAGE REPORT message to the gNB-CU-CP.

#### 8.3.8.3	Abnormal Conditions

Not applicable.

### 8.3.9	gNB-CU-UP Counter Check

#### 8.3.9.1	General

This procedure is initiated by the gNB-CU-UP to request the gNB-CU-CP to execute a counter check procedure to verify the value of the PDCP COUNTs associated with DRBs established in the gNB-CU-UP.

The procedure uses UE-associated signalling.

#### 8.3.9.2	Successful Operation

Figure 8.3.9.2-1: gNB-CU-UP Counter Check procedure, successful operation.

The gNB-CU-UP initiates the procedure by sending the gNB-CU-UP COUNTER CHECK REQUEST message to the gNB-CU-CP.

Upon reception of the gNB-CU-UP COUNTER CHECK REQUEST message, the gNB-CU-CP may perform the RRC counter check procedure as defined in TS 33.501 [13].

#### 8.3.9.3	Unsuccessful Operation

Not applicable.

#### 8.3.9.4	Abnormal Conditions

Not applicable.

### 8.3.10	UL Data Notification

#### 8.3.10.1	General

This procedure is initiated by the gNB-CU-UP to notify the gNB-CU-CP that an UL packet including a QFI value in the SDAP header not configured by the QoS Flows Information To Be Setup IE or the Flow Mapping Information IE is received for the first time at the default DRB. The procedure uses UE-associated signalling.

#### 8.3.10.2	Successful Operation

Figure 8.3.10.2-1: UL Data Notification procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the UL DATA NOTIFICATION message to the gNB-CU-CP.

#### 8.3.10.3	Abnormal Conditions

Not applicable.

### 8.3.11	MR-DC Data Usage Report

#### 8.3.11.1	General

This procedure is initiated by the gNB-CU-UP to report data volume served at the gNB-CU-UP, where the UE is connected to the 5GC. The procedure uses UE-associated signalling.

#### 8.3.11.2	Successful Operation

Figure 8.3.11.2-1: MR-DC Data Usage Report procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the MR-DC DATA USAGE REPORT message to the gNB-CU-CP.

#### 8.3.11.3	Abnormal Conditions

Not applicable.

### 8.3.12	Early Forwarding SN Transfer

#### 8.3.12.1	General

The purpose of the Early Forwarding SN Transfer procedure is to transfer, from the source gNB-CU-UP to the source gNB-CU-CP, DL COUNT of the last PDCP SDU successfully delivered or transmitted to the UE, for the purpose of discarding early forwarded downlink PDCP SDUs during Conditional Handover or conditional PSCell change.

The procedure uses UE-associated signalling.

#### 8.3.12.2	Successful Operation

Figure 8.3.12.2-1: Early Forwarding SN Transfer procedure: Successful Operation.

The source gNB-CU-UP initiates the procedure by sending the EARLY FORWARDING SN TRANSFER message.

The DRBs Subject To Early Forwarding List IE included in the EARLY FORWARDING SN TRANSFER message contains the DRB ID(s) corresponding to the DRB(s) subject to early data forwarding during Conditional Handover or conditional PSCell change.

For each DRB in the DRBs Subject To Early Forwarding List IE, the value of the DL COUNT Value IE indicates the DL COUNT of the last PDCP SDU successfully delivered in-sequence to the UE, if RLC-AM, and successfully transmitted, if RLC-UM.

#### 8.3.12.3	Unsuccessful Operation

Not applicable.

#### 8.3.12.4	Abnormal Conditions

If the source gNB-CU-CP receives this message for a UE for which no prepared Conditional Handover exists, the source gNB-CU-CP shall ignore the message.

### 8.3.13	GNB-CU-CP Measurement Results Information

#### 8.3.13.1	General

This procedure is initiated by the gNB-CU-CP to inform the measurement results received from the UE to the gNB-CU-UP.

The procedure uses UE-associated signalling.

#### 8.3.13.2	Successful Operation

Figure 8.3.x.2-1: GNB-CU-CP Measurement Results Information procedure. Successful operation.

The gNB-CU-CP initiates the procedure by sending a GNB-CU-CP MEASUREMENT RESULTS INFORMATION message.

#### 8.3.13.3	Abnormal Conditions

Not applicable.

## 8.4	Trace Procedures

### 8.4.1	Trace Start

#### 8.4.1.1	General

The purpose of the Trace Start procedure is to allow the gNB-CU-CP to request the gNB-CU-UP to initiate a trace session for a UE. The procedure uses UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Trace start procedure: Successful Operation.

Upon reception of the TRACE START message, the gNB-CU-UP shall initiate the requested trace session for the requested UE, as described in TS 32.422 [24]. In particular, the gNB-CU-UP shall, if supported:

-	if the MDT Activation IE is set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [24] and the gNB-CU-UP shall ignore Interfaces To Trace IE, and Trace Depth IE.

#### 8.4.1.3	Abnormal Conditions

Void.

### 8.4.2	Deactivate Trace

#### 8.4.2.1	General

The purpose of the Deactivate Trace procedure is to allow the gNB-CU-CP to request the gNB-CU-UP to stop the trace session for the indicated trace reference. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Deactivate trace procedure: Successful Operation.

Upon reception of the DEACTIVATE TRACE message, the gNB-CU-UP shall stop the trace session for the indicated trace reference contained in the Trace ID IE, as described in TS 32.422 [24].

#### 8.4.2.3	Abnormal Conditions

Void.

### 8.4.3	Cell Traffic Trace

#### 8.4.3.1	General

The purpose of the Cell Traffic Trace procedure is to send the allocated Trace Recording Session Reference and the Trace Reference to the gNB-CU-CP. The procedure uses UE-associated signalling.

#### 8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Cell Traffic Trace procedure. Successful operation.

The procedure is initiated with a CELL TRAFFIC TRACE message sent from the gNB-CU-UP to the gNB-CU-CP.

If the Privacy Indicator IE is included in the message, the gNB-CU-CP shall store the information so that it can be transferred towards the AMF.

#### 8.4.3.3	Abnormal Conditions

Void.

## 8.5	IAB Procedures

### 8.5.1	IAB UP TNL Address Update

#### 8.5.1.1	General

The purpose of the IAB UP TNL Address Update procedure is to allow the gNB-CU-CP to request the gNB-CU-UP to update the TNL Address(es) for all the DL F1-U GTP-U tunnels related to this (these) TNL address(es), and to allow the gNB-CU-UP to inform the gNB-CU-CP about the updated TNL Address(es) for all the UL F1-U GTP-U tunnels. The procedure uses non-UE associated signalling.

NOTE: 	This procedure is applicable for IAB-nodes, where the term "gNB-CU-CP" applies to IAB-donor-CU-CP, and the term “gNB-CU-UP” applies to IAB-donor-CU-UP.

NOTE:	Implementation shall ensure the avoidance of potential race conditions, i.e. it must ensure that the UP configuration (e.g., UL/DL UP TNL address) update is not concurrently performed using the non-UE-associated IAB UP TNL Address Update procedure and the UE-associated procedures for Bearer Context Management.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: IAB UP TNL Address Update procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the IAB UP TNL ADDRESS UPDATE message to the gNB-CU-UP. If the gNB-CU-UP succeeds to update the TNL Address(es), it replies to the gNB-CU-CP with the IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE message.

Upon reception of the IAB UP TNL ADDRESS UPDATE message, if the DL UP TNL Address to Update List IE is included therein, the gNB-CU-UP shall replace the old TNL Address(es) by the new TNL Address(es) for all the maintained DL F1-U GTP tunnels corresponding to the old TNL Address(es).

If the UL UP TNL Address to Update List IE is contained in the IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE message, the gNB-CU-CP shall consider the new TNL address(es) as replacement for the corresponding old TNL address(es).

#### 8.5.1.3	Unsuccessful Operation

Figure 8.5.1.3-1: IAB UP TNL Address Update procedure: Unsuccessful Operation.

If the gNB-CU-UP receives an IAB UP TNL ADDRESS UPDATE message, but cannot perform the update accordingly, it shall consider the update procedure as failed and respond with an IAB UP TNL ADDRESS UPDATE FAILURE message and appropriate cause value.

If the IAB UP TNL ADDRESS UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU-CP shall wait at least for the indicated amount of time before reinitiating the IAB UP TNL Address Update procedure towards the same gNB-CU-UP.

#### 8.5.1.4	Abnormal Conditions

Not Applicable.

## 8.z	MBS Procedures

### 8.z.1	MBS Procedures for Broadcast

#### 8.z.1.1	BC Bearer Context Setup

##### 8.z.1.1.1	General

The purpose of the BC Bearer Context Setup procedure is to allow the gNB-CU-CP to establish resources for a broadcast MBS session in the gNB-CU-UP. The procedure uses MBS associated signalling.

##### 8.z.1.1.2	Successful Operation

Figure 8.z.1.1.2-1: BC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BC BEARER CONTEXT SETUP RESPONSE message.

If the BC BEARER CONTEXT SETUP REQUEST message contains the Available Shared NG-U Termination Information IE, the gNB-CU-UP may use the contained information as specified in TS 38.401 [2].

##### 8.z.1.1.3	Unsuccessful Operation

Figure 8.z.1.1.3-1: BC Bearer Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested resources for the MBS session, it shall consider the procedure as failed and respond with the BC BEARER CONTEXT SETUP FAILURE message and an appropriate cause value.

##### 8.z.1.1.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

#### 8.z.1.2	BC Bearer Context Modification (gNB-CU-CP initiated)

##### 8.z.1.2.1	General

The purpose of the gNB-CU-CP initiated BC Bearer Context Modification procedure is to allow the gNB-CU-CP to modify resources for a broadcast MBS session. The procedure uses MBS associated signalling.

##### 8.z.1.2.2	Successful Operation

Figure 8.z.1.2.2-1: BC Bearer Context Modification procedure, gNB-CU-CP initiated: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to perform at least partially the requested modifications it replies to the gNB-CU-CP with the BC BEARER CONTEXT MODIFICATION RESPONSE message.

##### 8.z.1.2.3	Unsuccessful Operation

Figure 8.z.1.2.3-1: BC Bearer Context Modification procedure, gNB-CU-CP intiated: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested modifications, it shall respond with a BC BEARER CONTEXT MODIFICATION FAILURE message and an appropriate cause value.

##### 8.z.1.2.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

#### 8.z.1.3	BC Bearer Context Modification (gNB-CU-UP initiated)

##### 8.z.1.3.1	General

The purpose of the gNB-CU-UP initiated BC Bearer Context Modification procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to modify resources for a broadcast MBS session. The procedure uses MBS associated signalling.

##### 8.z.1.3.2	Successful Operation

Figure 8.z.1.3.2-1: BC Bearer Context Modification procedure, gNB-CU-UP initiated: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BC BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. If the gNB-CU-CP accepts the requested modifications, at least partially, it replies to the gNB-CU-UP with the BC BEARER CONTEXT MODIFICATION CONFIRM message.

##### 8.z.1.3.3	Unsuccessful Operation

Figure 8.z.1.3.3-1: BC Bearer Context Modification procedure, gNB-CU-UP intiated: Unsuccessful Operation.

If the gNB-CU-CP does not accept any of the requested modifications, it replies to the gNB-CU-UP with the BC BEARER CONTEXT MODIFICATION REFUSE message.

##### 8.z.1.3.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

#### 8.z.1.4	BC Bearer Context Release (gNB-CU-CP initiated)

##### 8.z.1.4.1	General

The purpose of the gNB-CU-CP initiated BC Bearer Context Release procedure is to allow the gNB-CU-CP to command the release of resources for the broadcast MBS Session. The procedure uses MBS associated signalling.

##### 8.z.1.4.2	Successful Operation

Figure 8.z.1.3.2-1: MC Bearer Context Release procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT RELEASE COMMAND message to the gNB-CU-UP. Upon reception of the BC BEARER CONTEXT RELEASE COMMAND message, the gNB-CU-UP shall release all related signalling and user data transport resources and reply with the BC BEARER CONTEXT RELEASE COMPLETE message.

##### 8.z.1.3.3	Abnormal Conditions

Not applicable.

#### 8.z.1.5	BC Bearer Context Release Request (gNB-CU-UP initiated)

##### 8.z.1.5.1	General

The purpose of the BC Bearer Context Release Request procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to trigger the release of resources for the broadcast MBS Session. The procedure uses MBS associated signalling.

##### 8.z.1.5.2	Successful Operation

Figure 8.z.1.5.2-1: BC Bearer Context Release Request procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BC BEARER CONTEXT RELEASE REQUEST message to the gNB-CU-CP.

Interactions with gNB-CU-CP intitiated BC Bearer Context Release procedure:

Upon reception of the BC BEARER CONTEXT RELEASE REQUEST message the gNB-CU-CP should initiate the BC Bearer ContextContext Release procedure.

##### 8.3.1.5.3	Abnormal Conditions

Not applicable.

### 8.z.2	MBS Procedures for Multicast

#### 8.z.2.1	MC Bearer Context Setup

##### 8.z.2.1.1	General

The purpose of the MC Bearer Context Setup procedure is to allow the gNB-CU-CP to establish resources for a multicast MBS session in the gNB-CU-UP. The procedure uses MBS associated signalling.

##### 8.z.2.1.2	Successful Operation

Figure 8.z.2.1.2-1: MC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the MC BEARER CONTEXT SETUP RESPONSE message.

##### 8.z.2.1.3	Unsuccessful Operation

Figure 8.z.2.1.3-1: MC Bearer Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested resources for the MBS session, it shall consider the procedure as failed and respond with the MC BEARER CONTEXT SETUP FAILURE message and an appropriate cause value.

##### 8.z.2.1.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

#### 8.z.2.2	MC Bearer Context Modification (gNB-CU-CP initiated)

##### 8.z.2.2.1	General

The purpose of the gNB-CU-CP initiated MC Bearer Context Modification procedure is to allow the gNB-CU-CP to modify resources for a multicast MBS session. The procedure uses MBS associated signalling.

##### 8.z.2.2.2	Successful Operation

Figure 8.z.2.2.2-1: MC Bearer Context Modification procedure, gNB-CU-CP initiated: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MC BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to perform at least partially the requested modifications it replies to the gNB-CU-CP with the MC BEARER CONTEXT MODIFICATION RESPONSE message.

##### 8.z.2.2.3	Unsuccessful Operation

Figure 8.z.2.2.3-1: MC Bearer Context Modification procedure, gNB-CU-CP intiated: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested modifications, it shall respond with a MC BEARER CONTEXT MODIFICATION FAILURE message and an appropriate cause value.

##### 8.z.2.2.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

#### 8.z.2.3	MC Bearer Context Modification (gNB-CU-UP initiated)

##### 8.z.2.3.1	General

The purpose of the gNB-CU-UP initiated MC Bearer Context Modification procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to modify resources for a multicast MBS session. The procedure uses MBS associated signalling.

##### 8.z.2.3.2	Successful Operation

Figure 8.z.2.3.2-1: MC Bearer Context Modification procedure, gNB-CU-UP initiated: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the MC BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. If the gNB-CU-CP accepts the requested modifications, at least partially, it replies to the gNB-CU-UP with the MC BEARER CONTEXT MODIFICATION CONFIRM message.

##### 8.z.2.3.3	Unsuccessful Operation

Figure 8.z.2.3.3-1: MC Bearer Context Modification procedure, gNB-CU-UP intiated: Unsuccessful Operation.

If the gNB-CU-CP does not accept any of the requested modifications, it replies to the gNB-CU-UP with the MC BEARER CONTEXT MODIFICATION REFUSE message.

##### 8.z.2.3.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

#### 8.z.2.4	MC Bearer Context Release (gNB-CU-CP initiated)

##### 8.z.2.4.1	General

The purpose of the gNB-CU-CP initiated MC Bearer Context Release procedure is to allow the gNB-CU-CP to command the release of resources for the multicast MBS Session. The procedure uses MBS associated signalling.

##### 8.z.2.4.2	Successful Operation

Figure 8.z.2.3.2-1: MC Bearer Context Release procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MC BEARER CONTEXT RELEASE COMMAND message to the gNB-CU-UP. Upon reception of the MC BEARER CONTEXT RELEASE COMMAND message, the gNB-CU-UP shall release all related signalling and user data transport resources and reply with the MC BEARER CONTEXT RELEASE COMPLETE message.

##### 8.z.2.3.3	Abnormal Conditions

Not applicable.

#### 8.z.2.5	MC Bearer Context Release Request (gNB-CU-UP initiated)

##### 8.z.2.5.1	General

The purpose of the MC Bearer Context Release Request procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to trigger the release of resources for the multicast MBS Session. The procedure uses MBS associated signalling.

##### 8.z.2.5.2	Successful Operation

Figure 8.z.2.5.2-1: MC Bearer Context Release Request procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the MC BEARER CONTEXT RELEASE REQUEST message to the gNB-CU-CP.

Interactions with gNB-CU-CP intitiated MC Bearer Context Release procedure:

Upon reception of the MC BEARER CONTEXT RELEASE REQUEST message the gNB-CU-CP should initiate the MC Bearer ContextContext Release procedure.

##### 8.3.2.5.3	Abnormal Conditions

Not applicable.

# 9	Elements for E1AP communication

## 9.1	General

Subclauses 9.2 and 9.3 present the E1AP message and IE definitions in tabular format. The corresponding ASN.1 definition is presented in subclause 9.4. In case there is contradiction between the tabular format and the ASN.1 definition, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional IEs, where the tabular format shall take precedence.

The messages have been defined in accordance to the guidelines specified in TR 25.921 [5].

When specifying IEs which are to be represented by bitstrings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bitstrings from other specifications, the first bit of the bitstring contains the first bit of the concerned information;

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [6].

## 9.2	Message Functional Definition and Content

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.1	Interface Management messages

#### 9.2.1.1	RESET

This message is sent by both the gNB-CU-CP and the gNB-CU-UP and is used to request that the E1 interface, or parts of the E1 interface, to be reset.

Direction: gNB-CU-CP  gNB-CU-UP and gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >E1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,…) |  | - |  |
| >Part of E1 interface |  |  |  |  |  |  |
| >>UE-associated logical E1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical E1-connection Item |  | 1 .. <maxnoofIndividualE1ConnectionsToReset> |  |  | EACH | reject |
| >>>>gNB-CU-CP UE E1AP ID | O |  | 9.3.1.4 |  | - |  |
| >>>>gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualE1ConnectionsToReset | Maximum no. of UE-associated logical E1-connections allowed to reset in one message. Value is 65536. |

#### 9.2.1.2	RESET ACKNOWLEDGE

This message is sent by both the gNB-CU-CP and the gNB-CU-UP as a response to a RESET message.

Direction: gNB-CU-UP  gNB-CU-CP and gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| UE-associated logical E1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical E1-connection Item |  | 1 .. <maxnoofIndividualE1ConnectionsToReset> |  |  | EACH | ignore |
| >>gNB-CU-CP UE E1AP ID | O |  | 9.3.1.4 |  | - |  |
| >>gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualE1ConnectionsToReset | Maximum no. of UE-associated logical E1-connections allowed to reset in one message. Value is 65536. |

#### 9.2.1.3	ERROR INDICATION

This message is sent by both the gNB-CU-CP and the gNB-CU-UP and is used to indicate that some error has been detected in the node.

Direction: gNB-CU-CP  gNB-CU-UP and gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.53 | This IE is ignored if received in UE associated signalling message. | YES | reject |
| gNB-CU-CP UE E1AP ID | O |  | 9.3.1.4 |  | YES | ignore |
| gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| gNB-CU-CP MBS E1AP ID | O |  | 9.3.1.x1 |  | YES | ignore |
| gNB-CU-UP MBS E1AP ID | O |  | 9.3.1.x2 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.4	GNB-CU-UP E1 SETUP REQUEST

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >Extended Slice Support List | O |  | 9.3.1.94 | Additional Supported S-NSSAIs per PLMN. | YES | reject |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >NPN Support Information | O |  | 9.3.1.83 |  | YES | reject |
| >Extended NR CGI Support List | O |  | 9.3.1.97 | Additional supported cells per PLMN. | YES | ignore |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-UP Name | O |  | 9.3.1.95 |  | YES | ignore |
| gNB-CU-UP MBS Support Information | O |  | 9.3.1.x5 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

#### 9.2.1.5	GNB-CU-UP E1 SETUP RESPONSE

This message is sent by the gNB-CU-CP to transfer information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP. | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-CP Name | O |  | 9.3.1.96 |  | YES | ignore |

#### 9.2.1.6	GNB-CU-UP E1 SETUP FAILURE

This message is sent by the gNB-CU-CP to indicate E1 Setup failure.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.7	GNB-CU-CP E1 SETUP REQUEST

This message is sent by the gNB-CU-CP to transfer information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP. | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-CP Name | O |  | 9.3.1.95 |  | YES | ignore |

#### 9.2.1.8	GNB-CU-CP E1 SETUP RESPONSE

This message is sent by the gNB-CU-UP to transfer information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | M |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| CN Support | M |  | ENUMERATED (EPC. 5GC, both, …) |  | YES | reject |
| Supported PLMNs |  | 1..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >Extended Slice Support List | O |  | 9.3.1.94 | Additional Supported S-NSSAIs per PLMN. | YES | reject |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >NPN Support Information | O |  | 9.3.1.83 |  | YES | reject |
| >Extended NR CGI Support List | O |  | 9.3.1.97 | Additional supported cells per PLMN. | YES | ignore |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-UP Name | O |  | 9.3.1.95 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |

#### 9.2.1.9	GNB-CU-CP E1 SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate E1 Setup failure.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.10	GNB-CU-UP CONFIGURATION UPDATE

This message is sent by the gNB-CU-UP to transfer updated information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP ID | O |  | 9.3.1.15 |  | YES | reject |
| gNB-CU-UP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-UP. | YES | ignore |
| Supported PLMNs |  | 0..<maxnoofSPLMNs> |  | Supported PLMNs | YES | reject |
| >PLMN Identity | M |  | 9.3.1.7 |  | - | - |
| >Slice Support List | O |  | 9.3.1.8 | Supported S-NSSAIs per PLMN. | - | - |
| >Extended Slice Support List | O |  | 9.3.1.94 | Additional Supported S-NSSAIs per PLMN. | YES | reject |
| >NR CGI Support List | O |  | 9.3.1.36 | Supported cells. | - | - |
| >QoS Parameters Support List | O |  | 9.3.1.37 | Supported QoS parameters per PLMN. | - | - |
| >NPN Support Information | O |  | 9.3.1.83 |  | YES | reject |
| >Extended NR CGI Support List | O |  | 9.3.1.97 | Additional supported cells per PLMN. | YES | ignore |
| gNB-CU-UP Capacity | O |  | 9.3.1.56 |  | YES | ignore |
| gNB-CU-UP TNLA To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-CU-UP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-CP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-UP Name | O |  | 9.3.1.96 |  | YES | ignore |
| gNB-CU-UP MBS Support Information | O |  | 9.3.1.x5 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-UP and the gNB-CU-CP. Value is 32. |

#### 9.2.1.11	GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU-CP to a gNB-CU-UP to acknowledge update of information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |

#### 9.2.1.12	GNB-CU-UP CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-CU-CP to indicate gNB-CU-UP Configuration Update failure.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.13	GNB-CU-CP CONFIGURATION UPDATE

This message is sent by the gNB-CU-CP to transfer updated information for a TNL association.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Name | O |  | PrintableString(SIZE(1..150,…)) | Human readable name of the gNB-CU-CP | YES | ignore |
| gNB-CU-CP TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Usage | M |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNLA is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.462 [18]. | - | - |
| gNB-CU-CP TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Remove Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Transport Layer Address gNB-CU-UP | O |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-UP. | YES | reject |
| gNB-CU-CP TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.2 | Transport Layer Address of the gNB-CU-CP. | - | - |
| >>TNLA Usage | O |  | ENUMERATED (ue, non-ue, both, …) | Indicates whether the TNLA is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.462 [18]. | - | - |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |
| Extended gNB-CU-CP Name | O |  | 9.3.1.95 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-CP and the gNB-CU-UP. Value is 32. |

#### 9.2.1.14	GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU-UP to a gNB-CU-CP to acknowledge update of information for a TNL association.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP TNLA Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP | - | - |
| gNB-CU-CP TNLA Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU-CP TNLA Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | - | - |
| >>TNLA Transport Layer Address | M |  | CP Transport Layer Information 9.3.2.2 | Transport Layer Address of the gNB-CU-CP | - | - |
| >>Cause | M |  | 9.3.1.2 |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Transport Network Layer Address Info | O |  | 9.3.2.7 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU-CP and the gNB-CU-UP. Value is 32. |

#### 9.2.1.15	GNB-CU-CP CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-CU-UP to indicate gNB-CU-CP Configuration Update failure.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.16	E1 RELEASE REQUEST

This message is sent by both the gNB-CU-CP and the gNB-CU-UP and is used to request the release of the E1 interface.

Direction: gNB-CU-CP  gNB-CU-UP and gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.1.17	E1 RELEASE RESPONSE

This message is sent by both the gNB-CU-CP and the gNB-CU-UP as a response to an E1 RELEASE REQUEST message.

Direction: gNB-CU-UP  gNB-CU-CP and gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |

#### 9.2.1.18	GNB-CU-UP STATUS INDICATION

This message is sent by the gNB-CU-UP to indicate to the gNB-CU-CP its status of overload.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-UP Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |

#### 9.2.1.19	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU-CP to gNB-CU-UP to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |  |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |  |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |  |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |  |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the resource status is required. | YES | ignore |  |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-CU-UP is requested to report. First Bit = TNL Available Capacity Ind Periodic, Second Bit = HW Capacity Ind Periodic. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, 20000ms, 30000ms, 40000ms, 50000ms, 60000ms, 70000ms, 80000ms, 90000ms, 100000ms, 110000ms, 120000ms, …) | Periodicity that can be used for reporting. Also used as the averaging window length for all measurement object if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.2.1.20	RESOURCE STATUS RESPONSE

This message is sent by the gNB-CU-UP to indicate that the requested measurement, for all the measurement objects included in the measurement is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.21	RESOURCE STATUS FAILURE

This message is sent by the gNB-CU-UP to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  |  | 9.3.1.2 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.3.1.3 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop” |

#### 9.2.1.22	RESOURCE STATUS UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the results of the requested measurements.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | Ignore |  |
| Transaction ID | M |  | 9.3.1.53 |  | YES | Reject |  |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | Reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | Reject |
| TNL Available Capacity Indicator | O |  | 9.3.1.72 |  |  |  |
| HW Capacity Indicator | O |  | 9.3.1.73 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSPLMNs | Maximum no. of Supported PLMN Ids. Value is 12. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

### 9.2.2	Bearer Context Management messages

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | 9.3.1.67 |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | M |  | DRB To Setup List E-UTRAN 9.3.3.1 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | Ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | M |  | 9.3.3.2 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 | Included whenever it is known by the gNB-CU-CP | YES | ignore |
| Trace Activation | O |  | 9.3.1.68 |  | YES | ignore |
| NPN Context Information | O |  | 9.3.1.84 |  | YES | reject |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.89 |  | YES | ignore |
| CHO Initiation | O |  | ENUMERATED (True, …) |  | YES | reject |
| Additional Handover Information | O |  | ENUMERATED(Discard PDCP SN, …) | If set to “Discard PDCP SN”, indicates that the forwarded PDCP SNs have to be removed | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.98 |  | YES | ignore |
| gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.2	BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List | M |  | DRB Setup List E-UTRAN 9.3.3.3 |  | YES | reject |
| >>DRB Failed List | O |  | DRB Failed List E-UTRAN 9.3.3.4 |  | YES | reject |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List | M |  | 9.3.3.5 |  | YES | reject |
| >>PDU Session Resource Failed List | O |  | 9.3.3.6 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.3	BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| New UL TNL Information Required | O |  | ENUMERATED (required, …) | Indicates that new UL TNL information has been requested to be provided. | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Data Discard Required | O |  | ENUMERATED (required, …) | Indicate to discard the DL user data in case of RAN paging failure. | YES | ignore |
| CHOICE System | O |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List | O |  | DRB To Setup Modification List E-UTRAN 9.3.3.7 |  | YES | reject |
| >>DRB To Modify List | O |  | DRB To Modify List E-UTRAN 9.3.3.8 |  | YES | reject |
| >>DRB To Remove List | O |  | DRB To Remove List E-UTRAN 9.3.3.9 |  | YES | reject |
| >>Subscriber Profile ID for RAT/Frequency priority | O |  | 9.3.1.69 |  | YES | ignore |
| >>Additional RRM Policy Index | O |  | 9.3.1.70 |  | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List | O |  | PDU Session Resource To Setup Modification List 9.3.3.10 |  | YES | reject |
| >>PDU Session Resource To Modify List | O |  | 9.3.3.11 |  | YES | reject |
| >>PDU Session Resource To Remove List | O |  | 9.3.3.12 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE(8)) |  | YES | ignore |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |
| Activity Notification Level | O |  | 9.3.1.67 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.5	BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | O |  |  |  | YES | ignore |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List | O |  | DRB Setup Modification List E-UTRAN 9.3.3.13 |  | YES | ignore |
| >>DRB Failed List | O |  | DRB Failed Modification List E-UTRAN 9.3.3.14 |  | YES | ignore |
| >>DRB Modified List | O |  | DRB Modified List E-UTRAN 9.3.3.15 |  | YES | ignore |
| >>DRB Failed To Modify List | O |  | DRB Failed To Modify List E-UTRAN 9.3.3.16 |  | YES | ignore |
| >>Retainability Measurements Information | O |  | 9.3.1.71 | Provides information on all the removed DRB(s), needed for retainability measurements in the gNB-CU-CP | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List | O |  | PDU Session Resource Setup Modification List 9.3.3.17 |  | YES | reject |
| >>PDU Session Resource Failed List | O |  | PDU Session Resource Failed Modification List 9.3.3.18 |  | YES | reject |
| >>PDU Session Resource Modified List | O |  | 9.3.3.19 |  | YES | reject |
| >>PDU Session Resource Failed To Modify List | O |  | 9.3.3.20 |  | YES | reject |
| >>Retainability Measurements Information | O |  | 9.3.1.71 | Provides information on all the removed DRB(s), needed for retainability measurements in the gNB-CU-CP | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.6	BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the bearer context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.7	BEARER CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to inform the gNB-CU-CP that a modification of a bearer context is required (e.g., due to local problems at the gNB-CU-UP).

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Modify List | O |  | DRB Required To Modify List E-UTRAN 9.3.3.21 |  | YES | reject |
| >>DRB To Remove List | O |  | DRB Required To Remove List 9.3.3.22 |  | YES | reject |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Modify List | O |  | PDU Session Resource Required To Modify List 9.3.3.23 |  | YES | reject |
| >>PDU Session Resource To Remove List | O |  | 9.3.3.12 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.8	BEARER CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU-CP to confirm the modification of the requested bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | O |  |  |  | YES | ignore |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Modified List | O |  | DRB Confirm Modified List E-UTRAN 9.3.3.24 |  | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Modified List | O |  | PDU Session Resource Confirm Modified List 9.3.3.25 |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.9	BEARER CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release an UE-associated logical E1 connection.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.2.10	BEARER CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of the UE-associated logical E1 connection.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Retainability Measurements Information | O |  | 9.3.1.71 | Provides information on all the removed DRB(s) and QoS Flow(s), needed for retainability measurements in the gNB-CU-CP | YES | ignore |

#### 9.2.2.11	BEARER CONTEXT RELEASE REQUEST

This message is sent by the gNB-CU-UP to request the release of an UE-associated logical E1 connection.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Status List |  | 0.. 1 |  |  | YES | ignore |
| >DRB Status Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>PDCP DL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for next DL packet to be assigned. | - | - |
| >>PDCP UL Count | O |  | PDCP Count 9.3.1.35 | PDCP count for first un-acknowledged UL packet. | - | - |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.2.2.12	BEARER CONTEXT INACTIVITY NOTIFICATION

This message is sent by the gNB-CU-UP to provide information about the UE activity to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE Activity Information | M |  |  |  | YES | reject |
| >DRB Activity List |  | 1 |  | Used if the Activity Notification Level IE is set as “DRB” in BEARER CONTEXT SETUP Request message | YES | reject |
| >>DRB Activity Item |  | 1 .. <maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>DRB Activity | M |  | ENUMERATED (Active, Not active, …) |  | - | - |
| >PDU Session Resource Activity List |  | 1 |  | Used if the Activity Notification Level IE is set as “PDU Session” in the BEARER CONTEXT SETUP Request message | YES | reject |
| >>PDU Session Resource Activity Item |  | 1 .. <maxnoofPDUSessionResource> |  |  | - | - |
| >>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>PDU Session Resource Activity | M |  | ENUMERATED (Active, Not active, …) |  | - | - |
| >UE Activity | M |  | ENUMERATED (Active, Not active, …) | Used if the Activity Notification Level IE is set as “UE” in the BEARER CONTEXT SETUP Request message | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB for a UE, the maximum value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.13	DL DATA NOTIFICATION

This message is sent by the gNB-CU-UP to provide information about the DL data detection to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | YES | ignore |
| PDU Session To Notify List | O |  |  |  | YES | ignore |
| >PDU Session To Notify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>QoS Flow List | M |  | 9.3.1.12 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.14	DATA USAGE REPORT

This message is sent by the gNB-CU-UP to report data volumes.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Data Usage Report List | M |  | 9.3.1.44 |  | YES | ignore |

#### 9.2.2.15	GNB-CU-UP COUNTER CHECK REQUEST

This message is sent by the gNB-CU-UP to request the verification of the value of the PDCP COUNTs associated with the DRBs established in the gNB-CU-UP.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRBs Subject to Counter Check List |  | 1 |  |  | YES | ignore |
| >>>DRBs Subject to Counter Check Item |  | 1 .. <maxnoof DRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP UL Count | M |  | PDCP Count 9.3.1.35 | Indicates the value of uplink COUNT associated to this DRB, as specified in TS 38.331 [8]. | - | - |
| >>>>PDCP DL Count | M |  | PDCP Count 9.3.1.35 | Indicates the value of downlink COUNT associated to this DRB, as specified in TS 38.331 [8]. | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>DRBs Subject to Counter Check List |  | 1 |  |  | YES | ignore |
| >>>DRBs Subject to Counter Check Item |  | 1 .. <maxnoof DRBs> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP UL Count | M |  | PDCP Count 9.3.1.35 | Indicates the value of uplink COUNT associated to this DRB, as specified in TS 38.331 [8]. | - | - |
| >>>>PDCP DL Count | M |  | PDCP Count 9.3.1.35 | Indicates the value of downlink COUNT associated to this DRB, as specified in TS 38.331 [8]. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.2.2.16	UL DATA NOTIFICATION

This message is sent by the gNB-CU-UP to provide information about the UL data detection to the gNB-CU-CP.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PDU Session To Notify List |  | 1 |  |  | YES | reject |
| >PDU Session To Notify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>QoS Flow List | M |  | 9.3.1.12 |  | - | - |

#### 9.2.2.17	MR-DC DATA USAGE REPORT

This message is sent by the gNB-CU-UP to report data volumes when the UE is connected to the 5GC.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PDU Session Resource Data Usage List |  | 1 |  |  | YES | ignore |
| >PDU Session Resource Data Usage Item |  | 1 .. <maxnoof PDUsessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.3.1.21 |  | – |  |
| >>MR-DC Usage Information | M |  | 9.3.1.63 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

#### 9.2.2.18	EARLY FORWARDING SN TRANSFER

This message is sent by the source gNB-CU-UP to the source gNB-CU-CP to transfer the COUNT value(s) related to early forwarded downlink PDCP SDUs during Conditional Handover or conditional PSCell change.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRBs Subject To Early Forwarding List | M | 1 |  |  | YES | reject |
| >DRBs Subject To Early Forwarding Item |  | 1 .. <maxnoofDRBs> |  |  | - | - |
| >>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>DL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper frame number of the last DL SDU successfully delivered in sequence to the UE, if RLC-AM, and successfully transmitted, if RLC-UM. | - | - |

#### 9.2.2.19	GNB-CU-CP MEASUREMENT RESULTS INFORMATION

This message is sent to the gNB-CU-UP to provide the measurement result received by the gNB-CU-CP.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Measurement Results Information List |  | 1 |  |  | YES | reject |
| >DRB Measurement Results Information Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >> UL D1 Result | O |  | INTEGER (0 .. 10000,…) | The unit is: 0.1ms | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

### 9.2.3	Trace Messages

#### 9.2.3.1	TRACE START

This message is sent by the gNB-CU-CP to initiate a trace session for a UE.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace Activation | M |  | 9.3.1.68 |  | YES | ignore |

#### 9.2.3.2	DEACTIVATE TRACE

This message is sent by the gNB-CU-CP to deactivate a trace session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | As per Trace ID in Trace Activation IE | YES | ignore |

#### 9.2.3.3	CELL TRAFFIC TRACE

This message is sent by the gNB-CU-UP to initiate a trace session for a UE.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | The Trace ID IE is composed of the following: Trace Reference defined in TS 32.422 [24] (leftmost 6 octets, with PLMN information coded as in 9.2.3.8), and Trace Recording Session Reference defined in TS 32.422 [24] (last 2 octets). | YES | ignore |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.2.2.1 | For File based Reporting. Defined in TS 32.422 [24]. Should be ignored if URI is present. | YES | ignore |
| Privacy Indicator | O |  | ENUMERATED (Immediate MDT, Logged MDT, ...) |  | YES | ignore |
| Trace Collection Entity URI | O |  | 9.3.2.8 | For Streaming based Reporting. Defined in TS 32.422 [24] Replaces Trace Collection Entity IP Address if present. | YES | ignore |

### 9.2.4	IAB Messages

#### 9.2.4.1	IAB UP TNL ADDRESS UPDATE

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to update the TNL address(es) of the DL F1-U GTP tunnel information.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| DL UP TNL Address To Update List |  | 0..1 |  |  | YES | reject |
| > DL UP TNL Address To Update Item IEs |  | 1..<maxnoofTNLAddresses> |  |  | - | - |
| >>Old TNL Address | M |  | 9.3.2.4 | The old Transport Layer Address of IAB-DU for DL F1-U GTP tunnel. | - | - |
| >>New TNL Address | M |  | 9.3.2.4 | The new Transport Layer Address of IAB-DU for DL F1-U GTP tunnel. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAddresses | Maximum no. of TNL addresses to be updated in one E1AP procedure. Value is 8. |

#### 9.2.4.2	IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE

This message is sent by the gNB-CU-UP to the gNB-CU-CP to acknowledge the update of TNL address in DL F1-U GTP tunnel information, or provide the updated TNL address(es) of the UL F1-U GTP tunnel information.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UL UP TNL Address to Update List |  | 0..1 |  |  | YES | ignore |
| > UL UP TNL Address Updated Item IEs |  | 1..<maxnoofTNLAddresses> |  |  | - | - |
| >>Old TNL Address | M |  | 9.3.2.4 | The old Transport Layer Address of CU-UP for UL F1-U GTP tunnel. | - | - |
| >>New TNL Address | M |  | 9.3.2.4 | The new Transport Layer Address of CU-UP for UL F1-U GTP tunnel. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofTNLAddresses | Maximum no. of TNL addresses updated in one E1AP procedure. Value is 8. |

#### 9.2.4.3	IAB UP TNL ADDRESS UPDATE FAILURE

This message is sent by the gNB-CU-UP to indicate IAB UP TNL address Update failure.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time To wait | O |  | 9.3.1.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### 9.2.y	MBS Messages

### 9.2.y.1	MBS Messages for Broadcast

#### 9.2.y.1.1	BC BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup resources for a broadcast MBS Session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| Global MBS Session ID | M |  | 9.3.1.x3 |  | YES | reject |
| BC Bearer Context To Setup | M |  | 9.3.3.y1 |  | YES | reject |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y1	BC Bearer Context To Setup

This IE contains MBS session resource related information used to request BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| BC Bearer Context NG-U TNL Info at 5GC | M |  | 9.3.1.y1x1 |  |
| BC MRB To Setup List | M |  | BC MRB Setup Configuration 9.3.1.y1.x3 |  |
| Available Shared NG-U Termination Information | O |  | 9.3.1.y1a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y1x1	BC Bearer Context NG-U TNL Info at 5GC

This IE contains TNL information for an MBS Session as provided by the 5GC for both, shared NG-U multicast and unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type |  |  |  |  |
| >location independent |  |  |  |  |
| >>MBS NG-U Information at 5GC | M |  | 9.3.1.y1x2 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS NG-U Information at 5GC |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.x6 |  |
| >>MBS NG-U Information at 5GC | M |  | 9.3.1.y1x2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.y1x2	MBS NG-U Information at 5GC

This IE contains TNL information for a single shared NG-U tunnel as provided by the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS NG-U Transport |  |  |  |  |
| >multicast |  |  |  |  |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |
| >unicast |  |  |  |  |
| >>Unicast NG-U UL Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.y1.x3	BC MRB Setup Configuration

This IE contains MRB configuration information for a BC Bearer Context Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC MRB To Setup List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >SDAP Configuration | M |  | 9.3.1.39 |  |
| >MBS PDCP Configuration | M |  | PDCP Configuration 9.3.1.38 | Editor’s Note: along running RRC CR |
| >MBS QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

#### 9.3.1.y1a	Available Shared MBS Session NG-U Termination Information

This IE contains information from the 5GC about NG-U terminations already established for the MBS-Session for other gNBs that can be shared, for both, location dependent and location independent MBS Sessions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Available MBS Session Type Info | M |  |  |  |
| >location independent |  |  |  |  |
| >>Available Shared NG-U Termination Information | M |  | 9.3.1.y1b |  |
| >location dependent |  |  |  |  |
| >>Location dependent Shared NG-U Information |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.x6 |  |
| >>>Available Shared NG-U Termination Information | M |  | 9.3.1.y1b |  |
| Consent to Apply Available Shared UP MBS QoS flow mapping | O |  | 9.3.1.y1c |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.y1b	Available Shared NG-U Termination Information

This IE contains information from the 5GC about NG-U terminations already established for the MBS-Session for other gNBs that can be shared.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available Shared NG-U Termination Information |  | 1..<maxnoofsharedNG-UTerminations> |  |  |
| >Shared NG-U Termination DL Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSharedNG-UTerminations | Maximum no. shared NG-U Terminations. Value is 8. |

#### 9.3.1.y1b	Consent to Apply Available Shared UP MBS QoS flow mapping

This IE provides information whether the gNB-CU-CP consents to the gNB-CU-UP’s decision to apply the MBS QoS flow to MRB mapping already decided at the shared NG-U termination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Consent to Apply Available Shared UP Termination MBS QoS flow mapping | M |  | ENUMERATED (allowed, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.y.1.2	BC BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested resources for a broadcast MBS Session.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| BC Bearer Context To Setup Response | M |  | 9.3.3.y2 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y2	BC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm BC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | C-ifunicast |  | 9.3.1.y2x1 |  |
| BC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.y2x3 |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |
| Offered Shared MBS Session NG-U Termination Information | O |  | 9.3.1.y2a |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.y1.x3 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifunicast | This IE shall be present if the MBS NG-U Information at NG-RAN IE within the BC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information. |

#### 9.3.1.y2x1	BC Bearer Context NG-U TNL Info at NG-RAN

This IE contains NG-RAN NG-U TNL information for an MBS Session for both, shared NG-U multicast and unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.y2x2 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS NG-U Information at NG-RAN |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID |  |  | 9.3.1.x6 |  |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.y2x2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.y2x2	MBS NG-U Information at NG-RAN

This IE contains NG-RAN TNL information for a single shared NG-U tunnel.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS NG-U Transport | M |  |  |  |
| >unicast |  |  |  |  |
| >>Shared NG-U UL Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.y2x3	BC Bearer Context F1-U TNL Info at CU

This IE contains CU F1-U TNL information for an MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information at CU | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information at CU |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.x6 |  |
| >>MBS F1-U Information at CU | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.y2a	Offered Shared MBS Session NG-U Termination Information

This IE contains information for the 5GC about offered NG-U terminations shareable with other gNBs, for both, location dependent and location independent MBS Sessions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>Offered Shared NG-U Termination DL Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent Shared NG-U Termination Information |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID |  |  | 9.3.1.x6 |  |
| >>>Offered Shared NG-U Termination DL Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.y.1.3	BC BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the broadcast MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | 9.3.1.x2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.y.1.4	BC BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a broadcast MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| BC Bearer Context To Modify | M |  | 9.3.3.y4 |  | YES | reject |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y4	BC Bearer Context To Modify

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify | O |  | BC Bearer Context NG-U TNL Info at 5GC 9.3.1.y1x1 |  |
| BC MRB To Setup List |  | O |  |  |  | BC MRB Setup Configuration 9.3.1.y1.x3 |  |  |  |
| BC MRB To Modify List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.y4x1 |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 | Editor’s Note: along running RRC CR |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |
| BC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| Available Shared NG-U Termination Information | O |  | 9.3.1.y1a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y4x1	BC Bearer Context F1-U TNL Info at DU

This IE contains CU F1-U TNL information for an MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information at DU |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID |  |  | 9.3.1.x6 |  |
| >>MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.y.1.5	BC BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested broadcast MBS Session Resource context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| BC Bearer Context To Modify Response | M |  | 9.3.3.y5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y5	BC Bearer Context To Modify Response

This IE contains MBS session resource related information used to confirm a BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.y2x1 |  |
| BC MRB Setup or Modify Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >MBS QoS Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >BC Bearer Context F1-U TNL Info at CU | O |  | 9.3.1.y2x3 |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |
| Offered Shared MBS Session NG-U Termination Information | O |  | 9.3.1.y2a |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.y1.x3 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.y.1.6	BC BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the broadcast MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.1.4a	BC BEARER CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to request the gNB-CU-CP to request the modification of a broadcast MBS Session Resource context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| BC Bearer Context To Modify Required | M |  | 9.3.3.y4a |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y4a	BC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.1.5a	BC BEARER CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU-CP to confirm the modification of the requested modification of a broadcast MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| BC Bearer Context To Modify Confirm | M |  | 9.3.3.y5a |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y5a	BC Bearer Context To Modify Confirm

This IE contains MBS session resource related information used to confirm a BC Bearer Context Modification.

NOTE:	In the current version of this specification, this IE does not contain any information.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.y.1.7	BC BEARER CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release a broadcast MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.y.1.8	BC BEARER CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of the broadcast MBS Session Resource Context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.y.1.9	BC BEARER CONTEXT RELEASE REQUEST

This message is sent by the gNB-CU-UP to request the release of a broadcast MBS Session Resource Context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.y.2	MBS Messages for Multicast

##### 9.2.y.2.1	MC BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup resources for a multicast MBS Session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| Global MBS Session ID | M |  | 9.3.1.x3 |  | YES | reject |
| MC Bearer Context To Setup | M |  | 9.3.3.ym1 |  | YES | reject |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.ym1	MC Bearer Context To Setup

This IE contains MBS session resource related information used to request MC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| MC MRB To Setup List | M |  | MC MRB Setup Configuration 9.3.1.ym1.x3 |  |
| Available Shared NG-U Termination Information | O |  | 9.3.1.y1a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

#### 9.3.1.ym1.x3	MC MRB Setup Configuration

This IE contains MRB configuration information for a MC Bearer Context Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC MRB To Setup List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >SDAP Configuration | M |  | 9.3.1.39 |  |
| >MBS PDCP Configuration | M |  | PDCP Configuration 9.3.1.38 | Editor’s Note: along running RRC CR |
| >MBS QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.2	MC BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested resources for a multicast MBS Session.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | reject |
| MC Bearer Context To Setup Response | M |  | 9.3.3.ym2 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.ym2	MC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm MC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN | C-ifunicast |  | 9.3.1.ym2x1 |  |
| MC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |
| Offered Shared MBS Session NG-U Termination Information | O |  | 9.3.1.y2a |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.ym1.x3 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifunicast | This IE shall be present if the MBS NG-U Information at NG-RAN IE within the MC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.ym2x1	MC Bearer Context NG-U TNL Info at NG-RAN

This IE contains NG-RAN NG-U TNL information for an MBS Session for both, shared NG-U multicast and unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.y2x2 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS NG-U Information at NG-RAN |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.x6 |  |
| >>>MBS NG-U Information at NG-RAN | M |  | 9.3.1.y2x2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.3	MC BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the multicast MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | 9.3.1.x2 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

##### 9.2.y.2.4	MC BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a multicast MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| MC Bearer Context To Modify | M |  | 9.3.3.ym4 |  | YES | reject |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.ym4	MC Bearer Context To Modify

This IE contains MBS session resource related information used to request a modification of a multicast MC Bearer Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.ym4x1 |  |
| MC Bearer Context NG-U TNL Info at NG-RAN Request | O |  | 9.3.1.ym4.x2 | To request NG-U TNL information from the gNB-CU-UP, if not yet available at gNB-CU-CP |
| MBS Multicast F1-U Context Descriptor | C-ifSetupOrRemove |  | 9.3.1.ym4x4 |  |
| MC MRB To Setup or Modify List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >MC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.ym4x3 |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 | Editor’s Note: along running RRC CR |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |
| MC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| Available Shared NG-U Termination Information | O |  | 9.3.1.y1a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifSetupOrRemove | This IE shall be present if either the MC MRB To Setup or Modify List IE or the MC MRB To Remove List IE or both IEs are included. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.ym4x1	MC Bearer Context NG-U TNL Info at 5GC

This IE contains TNL information for a multicast MBS Session as provided by the 5GC for both, shared NG-U multicast and unicast transport. It may also contain an MBS Area Session ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS NG-U Information at 5GC | M |  | 9.3.1.y1x2 |  |
| MBS Area Session ID | O |  | 9.3.1.x6 | For a location dependent multicast MBS Session |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.ym4x2	MC Bearer Context NG-U TNL Info at NG-RAN Request

This IE is used to request NG-U TNL information from the gNB-CU-UP, if not yet available at gNB-CU-CP and may contain an MBS Area Session ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG-RAN NG-U TNL requested. | M |  | ENUMERATED (requested, ...) |  |
| MBS Area Session ID | O |  | 9.3.1.x6 |  |

#### 9.3.1.ym4x3	MC Bearer Context F1-U TNL Info at DU

This IE contains CU F1-U TNL information for a multicast MBS Session. It may also contain per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS F1-U Information at DU | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.ym4x4	MBS Multicast F1-U Context Descriptor

This IE contains information about the multicast F1-U tunnel type, providing multicast data for either the whole DU, or for a single cell or for an MBS Area Session ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MC F1-U tunnel type | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >DU |  |  |  |  |
| >MBS Area Session |  |  |  |  |
| >>MBS Area Session ID | M |  | 9.3.1.x6 | To support per MBS Session Area F1-U tunnels and being able to refer to it. |
| >Cell |  |  |  |  |
| >>DU Cell Reference | M |  | 9.3.1.x4 | To support per cell F1-U tunnels and being able to refer to it. |
| >ptp retransmission |  |  |  |  |
| >>MBS PTP UE Reference | M |  | 9.3.1.ym4x5 | UE reference for a F1-U bearer established for issuing a PDCP Status Report and subsequent retransmission |
| >ptp-only MRB |  |  |  |  |
| >>MBS PTP UE Reference | M |  | 9.3.1.ym4x5 | UE reference for a F1-U bearer established for support of an ptp-only MRB configuration |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.2.ym4x5	MBS PTP UE Reference

This IE contains information to associate an F1-U bearer established for a UE for either retransmitting PDCP PDUs or to support a ptp-only MRB configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | M |  | INTEGER (0..65535, ...) | C-RNTI as defined in TS 38.331 [10]. |
| DU Cell Index | M |  | INTEGER (1..512) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.5	BC BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested multicast MBS Session Resource context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| MC Bearer Context To Modify Response | M |  | 9.3.3.ym5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.ym5	MC Bearer Context To Modify Response

This IE contains MBS session resource related information used to confirm a MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN Modify Response | O |  | 9.3.1.ym5x1 |  |
| MBS Multicast F1-U Context Descriptor | C-ifSetupOrFailed |  | 9.3.1.ym4x4 |  |
| MC MRB Setup or Modify Response List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |
| >MBS QoS Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >MC Bearer Context F1-U TNL Info at CU | O |  | UP Transport Layer Information 9.3.2.1 |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.ym1.x3 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |
| Offered Shared MBS Session NG-U Termination Information | O |  | 9.3.1.y2a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifSetupOrFailed | This IE shall be present if either the MC MRB Setup or Modify Response List IE or the MC MRB Failed List IE or both IEs are included. |

#### 9.3.1.ym5x1	MC Bearer Context NG-U TNL Info at NG-RAN Modify Response

This IE contains NG-RAN NG-U TNL information for an MBS Session for both, shared NG-U multicast and unicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.y2x2 |  |
| >>MBS Area Session ID | O |  | 9.3.1.x6 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.6	MC BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the multicast MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| MBS Multicast F1-U Context Descriptor | O |  | 9.3.1.ym4x4 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.4a	MC BEARER CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to request the gNB-CU-CP to request the modification of a multicast MBS Session Resource context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| MC Bearer Context To Modify Required | M |  | 9.3.3.ym4a |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.ym4a	MC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast F1-U Context Descriptor | C-ifRemoved |  | 9.3.1.ym4x4 |  |
| MC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | DRB ID 9.3.1.16 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

| Condition | Explanation |
| --- | --- |
| ifRemove | This IE shall be present if either the MC MRB To Remove List Required IE is included. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.5a	MC BEARER CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU-CP to confirm the modification of the requested modification of a multicast MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| MC Bearer Context To Modify Confirm | M |  | 9.3.3.ym5a |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.ym5a	MC Bearer Context To Modify Confirm

This IE contains MBS session resource related information used to confirm a MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Multicast F1-U Context Descriptor | C-ifRemoved |  | 9.3.1.ym4x4 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

##### 9.2.y.2.7	MC BEARER CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release a multicast MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

##### 9.2.y.2.8	MC BEARER CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of the multicast MBS Session Resource Context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

##### 9.2.y.2.9	MC BEARER CONTEXT RELEASE REQUEST

This message is sent by the gNB-CU-UP to request the release of the multicast MBS Session Resource Context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.x1 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.x2 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.1	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

#### 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the E1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Unknown or already allocated gNB-CU-CP UE E1AP ID, Unknown or already allocated gNB-CU-UP UE E1AP ID, Unknown or inconsistent pair of UE E1AP ID, Interaction with other procedure, PDCP Count Wrap Around, Not supported QCI value, Not supported 5QI value, Encryption algorithms not supported, Integrity protection algorithms not supported, UP integrity protection not possible, UP confidentiality protection not possible, Multiple PDU Session ID Instances, Unknown PDU Session ID, Multiple QoS Flow ID Instances, Unknown QoS Flow ID, Multiple DRB ID Instances, Unknown DRB ID, Invalid QoS combination, Procedure cancelled, Normal release, No radio resources available, Action desirable for radio reasons, Resources not available for the slice, PDCP configuration not supported, …, UE DL maximum integrity protected data rate reason, UP integrity protection failure, Release due to Pre-Emption, RSN not available for the UP, NPN not supported, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available Measurement not Supported For The Object) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, …, Unknown TNL address for IAB) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, …) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| Unknown or already allocated gNB-CU-CP UE E1AP ID | The action failed because the gNB-CU-CP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-CU-UP UE E1AP ID | The action failed because the gNB-CU-UP UE E1AP ID is either unknown, or (for a first message received at the gNB-CU-UP) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE E1AP ID | The action failed because both UE E1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| PDCP COUNT wrap around | PDCP COUNT approaches the maximum value. |
| Not supported QCI value | The action failed because the requested QCI is not supported. |
| Not supported 5QI value | The action failed because the requested 5QI is not supported. |
| Encryption algorithms not supported | The gNB-CU-UP is unable to support the selected encryption algorithm for the UE. |
| Integrity protection algorithms not supported | The gNB-CU-UP is unable to support the selected integrity protection algorithm for the UE. |
| UP integrity protection not possible | The PDU Session cannot be accepted according to the required user plane integrity protection policy. |
| UP confidentiality protection not possible | The PDU Session cannot be accepted according to the required user plane confidentiality protection policy |
| Multiple PDU Session ID Instances | The action failed because multiple instances of the same PDU Session had been provided. |
| Unknown PDU Session ID | The action failed because the PDU Session ID is unknown. |
| Multiple QoS Flow ID Instances | The action failed because multiple instances of the same QoS flow had been provided. |
| Unknown QoS Flow ID | The action failed because the QoS Flow ID is unknow. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Invalid QoS combination | The action was failed because of invalid QoS combination |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| No radio resources available | The requested node doesn’t have sufficient radio resources available. |
| Action desirable for radio reasons | The reason for requesting the action is radio related. |
| Resources not available for the slice | The requested resources are not available for the slice. |
| PDCP configuration not supported, | The gNB-CU-UP is unable to support the selected PDCP configuration for the UE. |
| UE DL maximum integrity protected data rate reason | The request is not accepted in order to comply with the maximum downlink data rate for integrity protection supported by the UE. |
| UP integrity protection failure | The gNB-CU-UP detects an integrity protection failure in the UL PDU. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| RSN not available for the UP | The redundant user plane resources indicated by RSN are not available. |
| NPN not supported | The action failed because the indicated SNPN is not supported in the node. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The gNB-CU-UP can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |
| Transport Resource Unavailable | The required transport resources are not available. |
| Unknown TNL address for IAB | The action failed because the TNL address is unknown. This cause value is applicable for IAB only. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer, NAS or Protocol. |

#### 9.3.1.3	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the gNB-CU-UP or the gNB-CU-CP when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing. The conditions for inclusion of the Transaction ID IE are described in clause 10.

For further details on how to use the Criticality Diagnostics IE, (see clause 10).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED(initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Transaction ID | O |  | 9.3.1.53 |  |
| Information Element Criticality Diagnostics |  | 0 .. <maxnoof Errors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value 'ignore' is not applicable. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnoofErrors is 256. |

#### 9.3.1.4	gNB-CU-CP UE E1AP ID

The gNB-CU-CP UE E1AP ID uniquely identifies the UE association over the E1 interface within the gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU-CP UE E1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.5	gNB-CU-UP UE E1AP ID

The gNB-CU-UP UE E1AP ID uniquely identifies the UE association over the E1 interface within the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU-UP UE E1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.6	Time To wait

This IE defines the minimum allowed waiting times.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time To wait | M |  | ENUMERATED(1s, 2s, 5s, 10s, 20s, 60s) |  |

#### 9.3.1.7	PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

#### 9.3.1.8	Slice Support List

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofSliceItems> |  |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.3.1.9	S-NSSAI

This IE indicates the S-NSSAI as defined in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SST | M |  | OCTET STRING (SIZE(1)) |  |
| SD | O |  | OCTET STRING (SIZE(3)) |  |

#### 9.3.1.10	Security Information

This IE provides the information for configuring UP ciphering and/or integrity protection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Security Algorithm | M |  | 9.3.1.31 |  |
| User Plane Security Keys | M |  | 9.3.1.32 |  |

#### 9.3.1.11	Cell Group Information

This IE provides information about the cell group(s) (i.e., radio leg(s)) that are part of the DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cell Group List |  | 1 |  |  | - | - |
| >Cell Group Item |  | 1..<maxnoofCellGroups> |  |  | - | - |
| >>Cell Group ID | M |  | INTEGER (0..3, …) | Cell group ID as defined in TS 38.331 [10] (0=MCG, 1=SCG). In this version of the specification, values “2” and “3” are not used. For E-UTRA Cell Groups, the same encoding is used as for NR Cell Groups. NOTE: There is no corresponding IE defined in TS 36.331 [21]. | - | - |
| >>UL Configuration | O |  | 9.3.1.33 | Indicates whether the Cell Group is used for UL traffic. | - | - |
| >>DL TX Stop | O |  | ENUMERATED (stop, resume, …) |  | - | - |
| >>RAT Type | O |  | ENUMERATED (E-UTRA, NR, …) | Indicates the RAT. | - | - |
| >>Number of tunnels | O |  | INTEGER (1..4, …) | Indicates the tunnel number of PDCP duplication for this cell group. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellGroups | Maximum no. of cell groups for a DRB. Value is 4. |

#### 9.3.1.12	QoS Flow List

This IE includes a list of QoS Flows that are identified by the QoS Flow Identifier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow List |  | 1 |  |  | - | - |
| >QoS Flow Item |  | 1..<maxnoofQoSflows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | 9.3.1.24 |  | - | - |
| >>QoS Flow Mapping Indication | O |  | 9.3.1.60 | Indicates that only the uplink or downlink QoS flow is mapped to the DRB | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

#### 9.3.1.13	UP Parameters

This IE provides information related to a DRB configured in the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Parameters List |  | 1 |  |  | - | - |
| >UP Parameters Item |  | 1..<maxnoofUPParameters> |  |  | - | - |
| >>UP Transport Layer Information | M |  | 9.3.2.1 |  | - | - |
| >>Cell Group ID | M |  | INTEGER (0..3, …) | Cell group ID as defined in TS 38.331 [10] (0=MCG, 1=SCG). In this version of the specification, values “2” and “3” are not used. | - | - |
| >>QoS Mapping Information | O |  | 9.3.1.81 | This IE is only used for IAB. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPParameters | Maximum no. of UP parameters (e.g., GTP tunnels) for a DRB. Value is 8 |

#### 9.3.1.14	NR CGI

The NR Cell Global Identifier (NR CGI) is used to globally identify a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.1.7 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

#### 9.3.1.15	gNB-CU-UP ID

The gNB-CU-UP ID uniquely identifies the gNB-CU-UP at least within a gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU-UP ID | M |  | INTEGER (0 .. 236-1) |  |

#### 9.3.1.16	DRB ID

This IE uniquely identifies a DRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB ID | M |  | INTEGER (1.. 32, ...) | Corresponds to the DRB-Identity defined in TS 38.331 [10]. |

#### 9.3.1.17	E-UTRAN QoS

This IE defines the QoS to be applied to a DRB for EN-DC case.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QCI | M |  | INTEGER (0..255) | QoS Class Identifier defined in TS 23.401 [11]. Logical range and coding specified in TS 23.203 [12]. | – | – |
| E-UTRAN Allocation and Retention Priority | M |  | 9.3.1.18 | E-UTRAN Allocation and Retention Priority | – | – |
| GBR QoS Information | O |  | 9.3.1.19 | This IE applies to GBR bearers only and is ignored otherwise. | – | – |

#### 9.3.1.18	E-UTRAN Allocation and Retention Priority

This IE specifies the relative importance compared to other E-RABs for allocation and retention of the E-UTRAN Radio Access Bearer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (0..15) | Desc.: This IE should be understood as "priority of allocation and retention" (see TS 23.401 [11]). Usage: Value 15 means "no priority". Values between 1 and 14 are ordered in decreasing order of priority, i.e. 1 is the highest and 14 the lowest. Value 0 shall be treated as a logical error if received. |
| Pre-emption Capability | M |  | ENUMERATED(shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other E-RABs Usage: The E-RAB shall not pre-empt other E-RABs or, the E-RAB may pre-empt other E-RABs The Pre-emption Capability indicator applies to the allocation of resources for an E-RAB and as such it provides the trigger to the pre-emption procedures/processes of the eNB. |
| Pre-emption Vulnerability | M |  | ENUMERATED(not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the E-RAB to pre-emption of other E-RABs. Usage: The E-RAB shall not be pre-empted by other E-RABs or the E-RAB may be pre-empted by other RABs. Pre-emption Vulnerability indicator applies for the entire duration of the E-RAB, unless modified, and as such indicates whether the E-RAB is a target of the pre-emption procedures/processes of the eNB. |

#### 9.3.1.19	GBR QoS Information

This IE indicates the maximum and guaranteed bit rates of a GBR E-RAB for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-RAB Maximum Bit Rate Downlink | M |  | Bit Rate 9.3.1.20 | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [11]. | – | – |
| E-RAB Maximum Bit Rate Uplink | M |  | Bit Rate 9.3.1.20 | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [11]. | – | – |
| E-RAB Guaranteed Bit Rate Downlink | M |  | Bit Rate 9.3.1.20 | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [11]. | – | – |
| E-RAB Guaranteed Bit Rate Uplink | M |  | Bit Rate 9.3.1.20 | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [11]. | – | – |

#### 9.3.1.20	Bit Rate

This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0.. 4,000,000,000,000,…) | The unit is: bit/s |

#### 9.3.1.21	PDU Session ID

This IE identifies a PDU Session for a UE. The definition and use of the PDU Session ID is specified in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session ID | M |  | INTEGER (0 ..255) |  |

#### 9.3.1.22	PDU Session Type

This IE indicates the PDU Session Type as specified in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Type | M |  | ENUMERATED (IPv4, IPv6, IPv4v6, ethernet, unstructured, ...) |  |

#### 9.3.1.23	Security Indication

This IE contains the user plane integrity protection indication and confidentiality protection indication which indicates the requirements on UP integrity protection and ciphering for corresponding PDU Session Resources, respectively.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Integrity Protection Indication | M |  | ENUMERATED (required, preferred, not needed, …) | Indicates whether UP integrity protection shall apply, should apply or shall not apply for the concerned PDU Session Resource. |
| Confidentiality Protection Indication | M |  | ENUMERATED (required, preferred, not needed, …) | Indicates whether UP ciphering shall apply, should apply or shall not apply for the concerned PDU Session Resource. |
| Maximum Integrity Protected Data Rate | C-ifIntegrityProtectionrequiredorpreferred |  | 9.3.1.57 | If present, this is the value received from the CN for the overall UE capability. This IE is ignored when enforcing the maximum IP data rate. |

| Condition | Explanation |
| --- | --- |
| ifIntegrityProtectionrequiredorpreferred | This IE shall be present if the Integrity Protection Indication IE within the Security Indication IE is set to “required” or “preferred”. |

#### 9.3.1.24	QoS Flow Identifier

This IE identifies a QoS Flow within a PDU Session. Definition and use of the QoS Flow Identifier is specified in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0 ..63) |  |

#### 9.3.1.25	QoS Flow QoS Parameters List

This IE contains a list of QoS Flows including the QoS Flow parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow List |  | 1 |  |  | - | - |
| >QoS Flow Item |  | 1..<maxnoofQoSFlows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | 9.3.1.24 |  | - | - |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.26 |  | - | - |
| >>QoS Flow Mapping Indication | O |  | 9.3.1.60 | Indicates that only the uplink or downlink QoS flow is mapped to the DRB. For MBS, this IE is associated with an MRB and always set to "dl". | - | - |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.74 | This IE indicates that this QoS flow is requested for the redundant transmission. | YES | ignore |
| >>TSC Traffic Characteristics | O |  | 9.3.1.75 | Traffic pattern information associated with the QFI. Details in TS 23.501 [20]. | YES | ignore |
| >>MCG Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.3.1.30 | This IE contains M-Node offered GBR QoS Flow Information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

#### 9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.29 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. | - |  |
| Paging Priority Indicator (PPI) | O |  | 9.3.1.55 |  | - |  |
| RDI | O |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1..1800, …) | Indicates the Reporting Frequency for RAN part delay for Qos monitoring. Units: second | YES | ignore |
| QoS Monitoring Disabled | O |  | ENUMERATED (true, ...) | Indicates to stop the QoS monitoring. | YES | ignore |

#### 9.3.1.27	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255, …) | This IE contains the standardized or pre-configured 5QI as specified in TS 23.501 [20]. | - | - |
| Priority Level | O |  | 9.3.1.51 | For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. | - | - |
| Averaging Window | O |  | 9.3.1.49 | This IE applies to GBR QoS Flows only. For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. | - | - |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. When included overrides standardized or pre-configured value. | - | - |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.79 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.79 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

#### 9.3.1.28	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Priority Level | M |  | 9.3.1.51 | For details see TS 23.501 [20]. | - | - |
| Packet Delay Budget | M |  | 9.3.1.47 | For details see TS 23.501 [20]. This IE is ignored if the Extended Packet Delay Budget IE is present. | - | - |
| Packet Error Rate | M |  | 9.3.1.48 | For details see TS 23.501 [20]. | - | - |
| 5QI | O |  | INTEGER (0..255,…) | This IE contains the dynamically assigned 5QI as specified in TS 23.501 [20]. | - | - |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical) | For details see TS 23.501 [20]. | - | - |
| Averaging Window | C-ifGBRflow |  | 9.3.1.49 | For details see TS 23.501 [20]. | - | - |
| Maximum Data Burst Volume | O |  | 9.3.1.50 | For details see TS 23.501 [20]. This IE shall be included if the Delay Critical IE is set to “delay critical” and is ignored otherwise. | - | - |
| Extended Packet Delay Budget | O |  | Extended Packet Delay Budget 9.3.1.79 | Packet Delay Budget is specified in TS 23.501 [9] | YES | ignore |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.79 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.79 | Core Network Packet Delay Budget is specified in TS 23.501 [9]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

#### 9.3.1.29	NG-RAN Allocation and Retention Priority

This IE specifies the relative importance of a QoS flow compared to other QoS flows for allocation and retention of NG-RAN resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (1..15) | Desc.: This IE defines the relative importance of a resource request (see TS 23.501 [20]). Usage: Values are ordered in decreasing order of priority, i.e., with 1 as the highest priority and 15 as the lowest priority. Further usage is defined in TS 23.501 [20]. |
| Pre-emption Capability | M |  | ENUMERATED (shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other QoS flows. Usage: The QoS flow shall not pre-empt other QoS flows or, the QoS flow may pre-empt other QoS flows. Specified in TS 23.501 [20] NOTE: The Pre-emption Capability indicator applies to the allocation of resources for a QoS flow and as such it provides the trigger to the pre-emption procedures/processes of the NG-RAN node. |
| Pre-emption Vulnerability | M |  | ENUMERATED (not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the QoS flow to pre-emption of other QoS flows. Usage: The QoS flow shall not be pre-empted by other QoS flows or the QoS flow may be pre-empted by other QoS flows. Specified in TS 23.501 [20] NOTE: The Pre-emption Vulnerability indicator applies for the entire duration of the QoS flow, unless modified and as such indicates whether the QoS flow is a target of the pre-emption procedures/processes of the NG-RAN node. |

#### 9.3.1.30	GBR QoS Flow Information

This IE indicates QoS parameters for a GBR QoS flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.20 | Maximum Bit Rate in DL. Details in TS 23.501 [20]. | - |  |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.20 | Maximum Bit Rate in UL. Details in TS 23.501 [20]. | - |  |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.20 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [20]. | - |  |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.20 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [20]. | - |  |
| Maximum Packet Loss Rate Downlink | O |  | Packet Loass Rate 9.3.1.46 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [20]. | - |  |
| Maximum Packet Loss Rate Uplink | O |  | Packet Loss Rate 9.3.1.46 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [20]. | - |  |
| Alternative QoS Parameters Set List | O |  | 9.3.1.93 | Indicates alternative sets of QoS Parameters for the QoS flow. | YES |  |

#### 9.3.1.31	Security Algorithm

This IE defines the type of ciphering algorithm and/or integrity protection used for the DRBs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Ciphering Algorithm | M |  | ENUMERATED (NEA0, 128-NEA1, 128-NEA2, 128-NEA3) | As defined in TS 33.501 [13]. |
| Integrity Protection Algorithm | O |  | ENUMERATED (NIA0, 128-NIA1, 128-NIA2, 128-NIA3) | As defined in TS 33.501 [13] for NG-RAN. |

#### 9.3.1.32	User Plane Security Keys

This IE contains the ciphering and/or integrity protection keys generated by the gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encryption Key | M |  | OCTET STRING | As defined in TS 33.501 [13]. |
| Integrity Protection Key | O |  | OCTET STRING | As defined in TS 33.501 [13] for NG-RAN. |

#### 9.3.1.33	UL Configuration

This IE includes the UL configuration for the DRB and the corresponding Cell Groups.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Configuration | M |  | ENUMERATED (no-data, shared, only, ..) | Indicates the UL configuration for a Cell Group that is part of a DRB. “no data” means that the Cell Group is not used for UL data. “shared” means that the Cell Group is used for UL data together with at least another Cell Group. “only” means that only this Cellg Group is used for UL data. |

#### 9.3.1.34	gNB-CU-UP Cell Group Related Configuration

This IE provides information related to a cell group that the gNB-CU-UP is allowed to change.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP Parameters List |  | 1 |  |  | - | - |
| >UP Parameters Item |  | 1..<maxnoofUPParameters> |  |  | - | - |
| >>Cell Group ID | M |  | INTEGER (0..3, …) | Cell group ID as defined in TS 38.331 [10] (0=MCG, 1=SCG). Used to identify the Cell Group to modify. In this version of the specification, values “2” and “3” are not used. | - | - |
| >>UP Transport Layer Information | M |  | 9.3.2.1 |  | - | - |
| >>UL Configuration | O |  | 9.3.1.33 | Indicates whether the Cell Group is used for UL traffic. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPParameters | Maximum no. of UP parameters (e.g., GTP tunnels) for a DRB. Value is 8. |

#### 9.3.1.35	PDCP Count

This IE include the PDCP Count information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| >PDCP SN | M |  | INTEGER (0 .. ..2PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |
| >HFN | M |  | INTEGER (0 .. 232-PDCP_SN_Size-1) | The PDCP SN Size is provided in the PDCP Configuration IE. |

#### 9.3.1.36	NR CGI Support List

This IE indicates the list of supported NR CGIs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR CGI Support Item IEs |  | 1..<maxnoofNRCGI> |  |  |
| >NR CGI | M |  | 9.3.1.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNRCGI | Maximum no. of supported NR CGIs. Value is 512. This range may be redefined. |

#### 9.3.1.37	QoS Parameters Support List

This IE indicates the list of supported QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-UTRAN QoS Support List | O |  |  |  |
| >E-UTRAN QoS Support Item |  | 1..<maxnoofEUTRNQOSParameters> |  |  |
| >>E-UTRAN QoS | M |  | 9.3.1.17 |  |
| NG-RAN QoS Support List | O |  |  |  |
| >NG-RAN QoS Support Item |  | 1..<maxnoofNGRANQOSParameters> |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEUTRANQOSParameters | Maximum no. of supported E-UTRAN QoS parameters. Value is 256. This range may be redefined. |
| maxnoofNGRANQOSParameters | Maximum no. of supported NG-RAN QoS parameters. Value is 256. This range may be redefined. |

#### 9.3.1.38	PDCP Configuration

This IE carries the PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP SN UL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN UL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| PDCP SN DL Size | M |  | PDCP SN Size 9.3.1.61 | Indicates the PDCP SN DL size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| RLC mode | M |  | ENUMERATED (RLC-TM, RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, …) | Indicates the RLC mode for the DRB. For more information see PDCP-Config IE in TS 38.331 [10]. Is ignored if received through DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message. | - | - |
| ROHC Parameters | O |  | 9.3.1.40 |  | - | - |
| T-Reordering Timer | O |  | 9.3.1.41 |  | - | - |
| Discard Timer | O |  | 9.3.1.42 |  | - | - |
| UL Data Split Threshold | O |  | 9.3.1.43 |  | - | - |
| PDCP Duplication | O |  | ENUMERATED (True, …) | Indicates whether PDCP duplication is to be configured for the DRB. This IE is ignored when the “Additional PDCP duplication Information” IE is present. | - | - |
| PDCP Re-establishment | O |  | ENUMERATED (true,…) | Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17] | - | - |
| PDCP Data Recovery | O |  | ENUMERATED (true,…) | Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17] | - | - |
| Duplication Activation | O |  | ENUMERATED ( Active, Inactive, …) | Information on the initial state of DL PDCP duplication | - | - |
| Out Of Order Delivery | O |  | ENUMERATED (true,…) | Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured. Out of order delivery is configured only when the radio bearer is established. | - | - |
| PDCP Status Report Indication | O |  | ENUMERATED (downlink, uplink, both, …) | For AM DRB, “downlink” indicates that the PDCP entity is configured to send PDCP status report(s) to the UE, and “uplink” indicates that the UE is configured to send PDCP status report(s), as specified in TS 38.323 [17]. “both” indicates that both “downlink” and “uplink” should be applied. | YES | ignore |
| Additional PDCP duplication Information | O |  | ENUMERATED (three, four, …) | Indicates the number of PDCP duplication configured when it is more than 2 for the DRB | YES | ignore |
| EHC Parameters | O |  | 9.3.1.90 |  | YES | ignore |
| MBS Initial HFN and Reference PDPC SN | O |  | BIT STRING (32) | applicable for NR MBS only | YES | ignore |

#### 9.3.1.39	SDAP Configuration

This IE carries the SDAP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Default DRB | M |  | ENUMERATED (True, False, …) | Indicates whether or not this is the default DRB for the PDU Session Resource. For more information see SDAP-Config IE in TS 38.331 [10]. |
| SDAP Header UL | M |  | ENUMERATED (Present, Absent, …) | Indicates whether or not a SDAP header is present for UL data on this DRB. For more information see SDAP-Config IE in TS 38.331 [10]. |
| SDAP Header DL | M |  | ENUMERATED (Present, Absent, …) | Indicates whether or not a SDAP header is present for DL data on this DRB. For more information see SDAP-Config IE in TS 38.331 [10]. |

#### 9.3.1.40	ROHC Parameters

This IE carries the ROCH parameters for header compressions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Choice ROHC Parameters | M |  |  | For more information see PDCP-Config IE in TS 38.331 [10]. |
| >ROHC |  |  |  |  |
| >>max CID | M |  | INTEGER (0..16383) | See description of maxCID inTS 38.331 [10] |
| >>ROHC Profiles | M |  | INTEGER (0..511) | Bitmap with supported UE profiles, bit 0 (LSB 0) = profile0x0001, bit 1 = profile0x0002, bit 2 = profile0x0003, bit 3 = profile0x0004, bit 4 = profile0x0006, bit 5 = profile0x0101, bit 6 = profile0x0102, bit 7 = profile0x0103, bit 8 = profile0x0104. See description of supportedROHC-Profiles in PDCP-Parameters in TS 38.331 [10]. |
| >>Continue ROHC | O |  | ENUMERATED (true, …) | See description of drb-ContinueROHC inTS 38.331 [10] |
| >uplinkOnlyROHC |  |  |  |  |
| >>max CID | M |  | INTEGER (0..16383) | See description of maxCID inTS 38.331 [10] |
| >>ROHC Profiles | M |  | INTEGER (0..511) | Bitmap with supported UE profiles, bit 4 = profile0x0006. See description of supportedROHC-Profiles in PDCP-Parameters in TS 38.331 [10]. |
| >>Continue ROHC | O |  | ENUMERATED (true, …) | See description of drb-ContinueROHC inTS 38.331 [10] |

#### 9.3.1.41	T-Reordering Timer

This IE indicates the t-Reordering timer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| T-Reordering Timer | M |  | ENUMERATED (0, 1, 2, 4, 5, 8, 10, 15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 160, 180, 200, 220, 240, 260, 280, 300, 500, 750, 1000, 1250, 1500, 1750, 2000, 2250, 2500, 2750, 3000, …) | Indicates the t-Reordering UL timer. The values are expressed in ms. For more information see PDCP-Config IE in TS 38.331 [10]. |

#### 9.3.1.42	Discard Timer

This IE indicates PDCP discard timer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Discard Timer |  |  | ENUMERATED (10, 20, 30, 40, 50, 60, 75, 100, 150, 200, 250, 300, 500, 750, 1500, Infinity) | Indicates the PDCP discard timer. The values are expressed in ms. For more information see PDCP-Config IE in TS 38.331 [10]. |

#### 9.3.1.43	UL Data Split Threshold

This IE indicates UL data split threshold.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL Data Split Threshold |  |  | ENUMERATED (0, 100, 200, 400, 800, 1600, 3200, 6400, 12800, 25600, 51200, 102400, 204800, 409600, 819200, 1228800, 1638400, 2457600, 3276800, 4096000, 4915200, 5734400, 6553600, Infinity, …) | Indicates the UL data split threshold. The values are expressed in bytes. For more information see PDCP-Config IE in TS 38.331 [10]. |

#### 9.3.1.44	Data Usage Report List

This IE provides information on the data usage for the UE, e.g., secondary NR RAT in EN-DC as specified in TS 37.340 [19].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Data usage report Item |  | 1 .. <maxnoofDRBs> |  |  | - | - |
| >DRB ID | M |  | 9.3.1.16 |  | - | - |
| > RAT Type | M |  | ENUMERATED (NR, …) |  | - | - |
| >DRB Usage Report List |  | 1 |  |  | - | - |
| >>DRB Usage Report Item |  | 1.. <maxnooftimeperiods> |  |  | - | - |
| >>>Start timestamp | M |  | OCTET STRING (SIZE(4)) | Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [14]. It indicates the UTC time when the recording of the Data Volume was started. | - | - |
| >>>End timestamp | M |  | OCTET STRING (SIZE(4)) | Encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [14]. It indicates the UTC time when the recording of the Data Volume was ended. | - | - |
| >>>Usage count UL | M |  | INTEGER (0..264-1) | The unit is: octets. | - | - |
| >>>Usage count DL | M |  | INTEGER (0..264-1) | The unit is: octets. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs. Value is 32. |
| Maxnooftimeperiods | Maximum no. of time reporting periods. Value is 2. |

#### 9.3.1.45	 Flow Failed List

This IE contains a list of QoS flows with a cause value. It is used for example to indicate failed QoS flow(s) or QoS flow(s) to be released.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - | - |
| >QoS Flow Identifier | M |  | 9.3.1.24 |  | - | - |
| >Cause | M |  | 9.3.1.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

#### 9.3.1.46	Packet Loss Rate

This IE indicates the Packet Loss Rate for a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Loss Rate | M |  | INTEGER (0..1000, …) | Ratio of lost packets per number of packets sent, expressed in tenth of percent. |

#### 9.3.1.47	Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Delay Budget | M |  | INTEGER (0..1023, …) | Upper bound value for the delay that a packet may experience expressed in unit of 0.5ms. |

#### 9.3.1.48	Packet Error Rate

This IE indicates the Packet Error Rate for a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Scalar | M |  | INTEGER (0..9, …) | The packet error rate is expressed as Scalar x 10-k where k is the Exponent. |
| Exponent | M |  | INTEGER (0..9, …) |  |

#### 9.3.1.49	Averaging Window

This IE indicates the Averaging Window for a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Averaging Window | M |  | INTEGER (0..4095, …) | Unit: ms. The default value is 2000ms. |

#### 9.3.1.50	Maximum Data Burst Volume

This IE indicates the Maximum Data Burst Volume for a QoS Flow and applies to delay critical GBR QoS flows only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Data Burst Volume | M |  | INTEGER (0..4095, ..., 4096.. 2000000) | Unit: byte. |

#### 9.3.1.51	Priority Level

This IE indicates the Priority Level for a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (1..127, …) | Values ordered in decreasing order of priority i.e. with 1 as the highest priority and 127 as the lowest priority. |

#### 9.3.1.52	Security Result

This IE indicates whether the security policy indicated as "preferred" in the Security Indication IE is performed or not.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Integrity Protection Result | M |  | ENUMERATED (performed, not performed, …) | Indicates whether UP integrity protection is performed or not for the concerned PDU Session Resource. |
| Confidentiality Protection Result | M |  | ENUMERATED (performed, not performed, …) | Indicates whether UP ciphering is performed or not for the concerned PDU Session Resource. |

#### 9.3.1.53	Transaction ID

The Transaction ID IE uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer. Messages belonging to the same procedure shall use the same Transaction ID. The Transaction ID is determined by the initiating peer of a procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | INTEGER (0..255, …) |  |

#### 9.3.1.54	Inactivity timer

This IE indicates the inactivity timer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Inactivity Timer | M |  | INTEGER (1.. 7200, …) | Indicates the inactivity timer. The values are expressed in seconds. |

#### 9.3.1.55	Paging Priority Indicator (PPI)

The Paging Policy Indicator is used for paging policy differentiation (see details in TS 23.501 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PPI | M |  | INTEGER (0.. 7, …) |  |

#### 9.3.1.56	gNB-CU-UP Capacity

This IE indicates the relative processing capacity of an gNB-CU-UP with respect to other gNB-CU-UPs in order to load-balance among different gNB-CU-UPs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| gNB-CU-UP Capacity | M |  | INTEGER(0..255) |  | - | - |

#### 9.3.1.57	Maximum Integrity Protected Data Rate

This IE indicates the maximum aggregate data rate for integrity protected DRBs for a UE as defined in TS 38.300 [8].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum IP rate | M |  | ENUMERATED (64kbps, max-UErate, …) | Defines the upper bound of the aggregated data rate of user plane integrity protected data. This limit applies to both UL and DL independently. |

#### 9.3.1.58	PDCP SN Status Information

This IE contains information about PDCP PDU transfer status of a DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDCP Status Transfer UL |  | 1 |  |  | – |  |
| >Receive Status Of PDCP SDU | O |  | BIT STRING (SIZE(1.. 131072)) | The first bit indicates the status of the SDU after the First Missing UL PDCP SDU. The Nth bit indicates the status of the UL PDCP SDU in position (N + First Missing SDU Number) modulo (1 + the maximum value of the PDCP-SN). 0: PDCP SDU has not been received. 1: PDCP SDU has been received correctly. | – |  |
| >UL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper Frame Number of the first missing UL SDU | – |  |
| PDCP Status Transfer DL |  | 1 |  |  | – |  |
| >DL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper Frame Number that the target NG-RAN node (handover) or the NG-RAN node to which the DRB context is transferred (dual connectivity) should assign for the next DL SDU not having an SN yet. | – |  |

#### 9.3.1.59	QoS Flow Mapping List

This IE contains a list of DRBs containing information about the mapped QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Mapping Item |  | 1..<maxnoofQoSFlows> |  |  | – |  |
| >QoS Flow Identifier | M |  | 9.3.1.24 |  | – |  |
| >QoS Flow Mapping Indication | O |  | 9.3.1.60 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU Session. Value is 64. |

#### 9.3.1.60	QoS Flow Mapping Indication

This IE is used to indicate whether only the uplink or only the downlink of a QoS flow is mapped to a DRB. For MBS this IE is applied to an MRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Mapping Indication | M |  | ENUMERATED (ul, dl, ...) | Indicates that only the uplink or downlink QoS flow is mapped to the DRB. If applied to an MRB, the IE is always set to "dl". |

#### 9.3.1.61	PDCP SN Size

This IE carries the PDCP SN Size.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDCP SN Size | M |  | ENUMERATED (s-12, s-18, …) | Indicates the PDCP SN size in bits. For more information see PDCP-Config IE in TS 38.331 [10]. |

#### 9.3.1.62	Network Instance

This IE provides the network instance to be used by the NG-RAN node when selecting a particular transport network resource as described in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Network Instance | M |  | INTEGER (1..256, …) |  |

#### 9.3.1.63	MR-DC Usage Information

This IE provides information on the data usage for the UE connected to 5GC, e.g., secondary RAT in MR-DC as specified in TS 37.340 [19].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Data Usage per PDU Session Report | O |  |  |  | - |  |
| >Secondary RAT Type | M |  | ENUMERATED (nR, e-UTRA…) |  |  |  |
| >PDU session Timed Report List | M |  | MR-DC Data Usage Report List 9.3.1.64 |  |  |  |
| Data Usage per QoS Flow List | O |  |  |  |  |  |
| >Data Usage per QoS Flow Item |  | 1..<maxnoofQoSFlows> |  |  | – |  |
| >>QoS Flow Indicator | M |  | 9.3.1.24 |  | – |  |
| >>Secondary RAT Type | M |  | ENUMERATED (nR, e-UTRA…) |  | – |  |
| >>QoS Flow Timed Report List | M |  | MR-DC Data Usage Report List 9.3.1.64 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.3.1.64	MR-DC Data Usage Report List

This IE provides information on the data usage.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MR-DC Data Usage Report Item |  | 1.. <maxnooftimeperiods> |  |  |
| >Start timestamp | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [14]. It indicates the start time of the collecting period of the included Usage Count UL IE and Usage Count DL IE. |
| >End timestamp | M |  | OCTET STRING (SIZE(4)) | UTC time encoded in the same format as the first four octets of the 64-bit timestamp format as defined in section 6 of IETF RFC 5905 [14]. It indicates the end time of the collecting period of the included Usage Count UL IE and Usage Count DL IE. |
| >Usage count UL | M |  | INTEGER (0..264-1) | The unit is: octets. |
| >Usage count DL | M |  | INTEGER (0..264-1) | The unit is: octets. |

| Range bound | Explanation |
| --- | --- |
| maxnooftimeperiods | Maximum no. of time reporting periods. Value is 2. |

#### 9.3.1.65	gNB-DU ID

The gNB-DU ID uniquely identifies a gNB-DU at least within a gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU ID | M |  | INTEGER (0 .. 236-1) | The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers. |

#### 9.3.1.66	Common Network Instance

This IE provides the common network instance to be used by the NG-RAN node when selecting a particular transport network resource as described in TS 23.501 [9] in a format common with 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Common Network Instance | M |  | OCTET STRING | The octets of OCTET STRING are encoded as the Network Instance field of the Network Instance IE specified in TS 29.244 [29] |

#### 9.3.1.67	Activity Notification Level

This IE contains information on which level activity notification shall be performed..

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Activity Notification Level | M |  | ENUMERATED (DRB, PDU Session, UE, …) |  |

#### 9.3.1.68	Trace Activation

This IE defines parameters related to a trace session activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [24] (leftmost 6 octets, with PLMN information encoded as in 9.3.1.7), and Trace Recording Session Reference defined in TS 32.422 [24] (last 2 octets). | - | - |
| Interfaces To Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an NG-RAN node interface: first bit = NG-C, second bit = Xn-C, third bit = Uu, fourth bit = F1-C, fifth bit = E1: other bits reserved for future use. Value '1' indicates 'should be traced'. Value '0' indicates 'should not be traced'. | - | - |
| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, minimumWithoutVendorSpecificExtension, mediumWithoutVendorSpecificExtension, maximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [24]. | - | - |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.4 | For File based Reporting. Defined in TS 32.422 [24]. Should be ignored if URI is present. | - | - |
| Trace Collection Entity URI | O |  | 9.3.2.8 | For Streaming based Reporting. Defined in TS 32.422 [24] Replaces Trace Collection Entity IP Address if present. | YES | ignore |
| MDT Configuration | O |  | 9.3.1.85 |  | YES | ignore |

#### 9.3.1.69	Subscriber Profile ID for RAT/Frequency priority

This parameter is used to define local configuration for RRM strategies.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subscriber Profile ID for RAT/Frequency priority | M |  | INTEGER (1.. 256, ...) |  |

#### 9.3.1.70	Additional RRM Policy Index

The Additional RRM Policy Index IE is used to provide additional information as specified in TS 36.300 [25].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional RRM Policy Index | M |  | BIT STRING (SIZE(32)) |  |

#### 9.3.1.71	Retainability Measurements Information

This IE contains information on removed DRB(s) and QoS Flow(s) which are needed to perform retainability measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRB Removed List |  | 1 |  |  | - |  |
| >DRB Removed Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>DRB Released In Session | O |  | ENUMERATED (released in session, not released in session, …) | Indicates if the DRB was “in session” or not (as defined in TS 32.425 [26] and TS 28.552 [22]) when released | - |  |
| >>DRB Accumulated Session Time | O |  | OCTET STRING (SIZE(5)) | Accumulated “in session” time for the DRB, as defined in TS 32.425 [26] and TS 28.552 [22], in milliseconds | - |  |
| >>QoS Flow Removed List |  | 0..1 |  |  | - |  |
| >>>QoS Flow Removed Item |  | 1..< maxnoofQoSFlows > |  |  | - |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.24 |  | - |  |
| >>>>QoS Flow Released In Session | O |  | ENUMERATED (released in session, not released in session, …) | Indicates if the QoS Flow was “in session” or not (as defined in TS 28.552 [22]), when released | - |  |
| >>>>QoS Flow Accumulated Session Time | O |  | OCTET STRING (SIZE(5)) | Accumulated “in session” time for the QoS Flow, as defined in TS 28.552 [22], in milliseconds | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

#### 9.3.1.72	TNL Available Capacity Indicator

The TNL Available Capacity Indicator IE indicates offered and available capacity of the Transport Network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Offered Capacity | M |  | INTEGER (0.. 16777216,...) | Maximum capacity in kbps |
| DL TNL Available Capacity | M |  | INTEGER (0.. 100,...) | Available capacity. Value 100 corresponds to the offered capacity. |
| UL TNL Offered Capacity | M |  | INTEGER (0.. 16777216,...) | Maximum capacity in kbps |
| UL TNL Available Capacity | M |  | INTEGER (0.. 100,...) | Available capacity. Value 100 corresponds to the offered capacity. |

#### 9.3.1.73	HW Capacity Indicator

The HW Capacity Indicator IE indicates offered and available throughput experienced by the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Offered Throughput | M |  | INTEGER (1.. 16777216,...) | Maximum capacity offered by the gNB-CU-UP in kbps |
| Available Throughput | M |  | INTEGER(0..100, …) | Average available capacity at the gNB-CU-UP. Value 100 corresponds to the offered throughput. |

#### 9.3.1.74	Redundant QoS Flow Indicator

This IE provides the Redundant QoS Flow Indicator for a QoS flow as specified in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Redundant QoS Flow Indicator | M |  | ENUMERATED (true, false) | This IE indicates that this QoS flow is requested for the redundant transmission. Value “true” indicates that redundant transmission is requested for this QoS flow. Value “false” indicates that redundant transmission is requested to be stopped if started. |

#### 9.3.1.75	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.76 |  |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.76 |  |

#### 9.3.1.76	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [20]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.77 |  |
| Burst Arrival Time | O |  | 9.3.1.78 |  |

#### 9.3.1.77	Periodicity

This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..640000, …) | Periodicity expressed in units of 1 us. |

#### 9.3.1.78	Burst Arrival Time

This IE indicates the Burst Arrival Time of the TSC QoS flow as defined in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time | M |  | OCTET STRING | Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [10]. The value is truncated to 1 us granularity. |

#### 9.3.1.79	Extended Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Packet Delay Budget | M |  | INTEGER (0..65535, …) | Upper bound value for the delay that a packet may experience expressed in unit of 0.01ms. |

#### 9.3.1.80	Redundant PDU Session Information

This IE defines Redundancy information to be applied to a PDU Session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RSN | M |  | ENUMERATED (v1, v2, …) |  |

#### 9.3.1.81	QoS Mapping Information

This IE indicates the DSCP and/or IPv6 Flow Label field(s) of IP packet which is sent through the GTP-U tunnel of a requested DRB. This IE is only used for IAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DSCP | O |  | BIT STRING (SIZE(6)) |  |
| Flow Label | O |  | BIT STRING (SIZE(20)) |  |

#### 9.3.1.82	NID

This IE contains the Network Identifier of an SNPN, as specified in TS 23.501 [20]. The NID is specified in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | BIT STRING (SIZE(44)) |  |

#### 9.3.1.83	NPN Support Information

This IE provides NPN related information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Support Information | M |  |  |  |
| >NPN Support Information -SNPN |  |  |  |  |
| >>NID | M |  | 9.3.1.82 | This IE is associated with the PLMN Identity and the Slice Support List contained in the Supported PLMNs IE. Together with the PLMN Identity it identifies the SNPN supported by the gNB-CU-UP. |

#### 9.3.1.84	NPN Context Information

This IE provides bearer context related NPN information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Context Information | M |  |  |  |
| >SNPN Information |  |  |  |  |
| >>NID | M |  | 9.3.1.82 | This IE is associated with Serving PLMN information contained in bearer context related E1AP message. Together with the Serving PLMN identity it identifies the serving SNPN. |

#### 9.3.1.85	MDT Configuration

The IE defines the NR MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Immediate MDT and Trace,…) |  |
| CHOICE MDT Mode | M |  |  |  |
| >Immediate MDT |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [27]. Fourth Bit = M4, Seventh Bit = M6, Eighth Bit = M7. Value “1” indicates “activate” and value “0” indicates “do not activate”. This version of the specification does not use bits 1, bit 2, bit 3, bit 5 and bit 6. |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.86 |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.87 |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.88 |  |

| Condition | Explanation |
| --- | --- |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to “1”. |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to “1”. |

#### 9.3.1.86	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M4 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.3.1.87	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640,ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30, …) |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  |

#### 9.3.1.88	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) |  |
| M7 Links to log | M |  | ENUMERATED(uplink, …) |  |

#### 9.3.1.89	MDT PLMN List

The purpose of the MDT PLMN List IE is to provide the list of PLMN allowed for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT PLMN List |  | 1..<maxnoofMDTPLMNs> |  |  |
| >PLMN Identity | M |  | 9.3.1.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |

#### 9.3.1.90	EHC Parameters

This IE carries the EHC parameters for ethernet header compression.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| EHC Common | M |  |  |  | - | - |
| >EHC-CID-Length | M |  | ENUMERATED { bits7, bits15, … } | See description of ehc-CID-Length in TS 38.331 [10] | - | - |
| EHC Downlink | O |  |  |  | - | - |
| >drb-ContinueEHC-DL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-DL in TS 38.331 [10] | - | - |
| >maxCID-EHC-DL | O |  | INTEGER(1..32767, …) | Indicate the maximum number of DL EHC contexts that can be established for the DRB. The total value of maxCID-EHC-DL plus maxCID-EHC-UL(as specified in TS 38.331) across all bearers for the UE should be less than or equal to the value of maxNumberEHC-Contexts parameter as indicated by the UE. | YES | ignore |
| EHC Uplink | O |  |  |  | - | - |
| >drb-ContinueEHC-UL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-UL in TS 38.331 [10] | - | - |

#### 9.3.1.91	DAPS Request Information

The DAPS Indicator IE indicates that DAPS HO is requested for the concerned DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DAPS Indicator | M |  | ENUMERATED (DAPS HO required, …) | Indicates that DAPS HO is requested |

#### 9.3.1.92	Early Forwarding COUNT Information

This IE contains DL COUNT value related to early data forwarding during DAPS Handover or Conditional Handover or conditional PSCell change.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Early Forwarding | M |  |  |  |
| >First DL COUNT |  |  |  |  |
| >>FIRST DL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper frame number of the first DL SDU that the source NG-RAN node forwards to the target NG-RAN node |
| >DL Discarding |  |  |  |  |
| >>DISCARD DL COUNT Value | M |  | PDCP Count 9.3.1.35 | PDCP-SN and Hyper frame number for which the target NG-RAN node should discard forwarded DL SDUs associated with lower values. |

#### 9.3.1.93	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the NG-RAN node can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Item |  | 1..<maxnoofQoSParaSets> |  |  |
| >Alternative QoS Parameters Index | M |  | INTEGER (1..8,..) |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.20 |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.20 |  |
| >Packet Delay Budget | O |  | 9.3.1.47 |  |
| >Packet Error Rate | O |  | 9.3.1.48 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSParaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS under Notification Control. Value is 8. |

#### 9.3.1.94	Extended Slice Support List

This IE indicates a list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofExtSliceItems> |  |  | - |  |
| >S-NSSAI | M |  | 9.3.1.9 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtSliceItems | Maximum no. of signalled slice support items. Value is 65535. |

#### 9.3.1.95	Extended gNB-CU-CP Name

This IE provides extended human readable name of the gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| gNB-CU-CP Name Visible | O |  | VisibleString (SIZE(1..150, …)) |  | - |  |
| gNB-CU-CP Name UTF8 | O |  | UTF8String (SIZE(1..150, …)) |  | - |  |

#### 9.3.1.96	Extended gNB-CU-UP Name

This IE provides extended human readable name of the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| gNB-CU-UP Name Visible | O |  | VisibleString (SIZE(1..150, …)) |  | - |  |
| gNB-CU-UP Name UTF8 | O |  | UTF8String (SIZE(1..150, …)) |  | - |  |

#### 9.3.1.97	Extended NR CGI Support List

This IE indicates the list of supported NR CGIs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended NR CGI Support Item IEs |  | 0..<maxnoofExtNRCGI> |  |  |
| >NR CGI | M |  | 9.3.1.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtNRCGI | Maximum no. of extended NR CGIs supported. Value is 16384. |

#### 9.3.1.98	Direct Forwarding Path Availability

This IE indicates whether a direct forwarding path is available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Direct Forwarding Path Availability | M |  | ENUMERATED (inter-system direct path available, …, intra-system direct path available) |  |

#### 9.3.1.x1	gNB-CU-CP MBS E1AP ID

The gNB-CU-CP UE E1AP ID uniquely identifies the MBS association over the E1 interface within the gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU-CP MBS E1AP ID | M |  | INTEGER (0 .. 216 -1) |  |

#### 9.3.1.x2	gNB-CU-UP MBS E1AP ID

The gNB-CU-UP UE E1AP ID uniquely identifies the MBS association over the E1 interface within the gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU-UP MBS E1AP ID | M |  | INTEGER (0 .. 216 -1) |  |

#### 9.3.1.x3	Global MBS Session ID

This IE indicates the TMGI uniquely identifies the MBS Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Encoded as defined in TS 23.003. [FFS whether the same TMGI applies for 4G or 5G, as per current 23.003] |
| NID | O |  | 9.3.1.82 | Defined in TS 23.003 [23]. |

#### 9.3.1.x4	DU Cell Reference

This IE indicates the index of an NR CGI within a DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DU Cell Index | M |  | INTEGER (1..512) | To support per cell F1-U tunnels and being able to refer to it. |
| NR CGI | M |  | 9.3.1.14 |  |

#### 9.3.1.x5	gNB-CU-UP MBS Support Information

This IE includes MBS related support information for the E1 Setup procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Support Information To Add List |  | 0..1 |  |  |
| >MBS Support Information To Add Item |  | 1..<maxnoofMBSSessionIDs> |  |  |
| >>Global MBS Session ID | M |  | 9.3.1.x3 |  |
| MBS Support Information To Remove List |  | 0..1 |  |  |
| >MBS Support Information To Remove Item |  | 1..<maxnoofMBSSessionIDs> |  |  |
| >>Global MBS Session ID | M |  | 9.3.1.x3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessionIDs | Maximum no. of MBS Session IDs. Value is 512 [FFS]. |

#### 9.3.1.x6	MBS Area Session ID

This IE indicates the TMGI uniquely identifies the MBS Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session Area ID | M |  | INTEGER (0 .. 255, …) [FFS] |  |

### 9.3.2	Transport Network Layer Related IEs

#### 9.3.2.1	UP Transport Layer Information

The UP Transport Layer Information IE identifies an transport bearer associated to a DRB. It contains a Transport Layer Address and a GTP Tunnel Endpoint Identifier. The Transport Layer Address is an IP address to be used for the user plane transport.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Transport Layer Information | M |  |  |  |
| >GTP Tunnel |  |  |  |  |
| >>Transport Layer Address | M |  | 9.3.2.4 |  |
| >>GTP-TEID | M |  | 9.3.2.3 |  |

#### 9.3.2.2	CP Transport Layer Information

This IE is used to provide the E1 control plane transport layer information associated with an gNB-CU-CP and gNB-CU-UP pair.

| IE/Group Name | Presence | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CHOICE CP Transport Layer Information |  |  |  |  |  |  |  |
| >Endpoint-IP-address |  |  |  |  |  | - | - |
| >> Endpoint IP address | M |  |  | Transport Layer Address 9.3.2.4 |  | - | - |
| >Endpoint-IP-address-and-port |  |  |  |  |  | YES | reject |
| >>Endpoint IP address | M |  |  | Transport Layer Address 9.3.2.4 |  | - | - |
| >>Port Number | M |  |  | BIT STRING (SIZE(16)) |  | - | - |

#### 9.3.2.3	GTP-TEID

The GTP-TEID IE is the GTP Tunnel Endpoint Identifier to be used for the user plane transport.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GTP-TEID | M |  | OCTET STRING (SIZE(4)) | For details and range, see TS 29.281 [15]. |

#### 9.3.2.4	Transport Layer Address

This Transport Layer Address IE is an IP address.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (SIZE(1..160, …)) | The Radio Network Layer is not supposed to interpret the address information. It should pass it to the Transport Layer for interpretation. For details, see TS 38.414 [16]. |

#### 9.3.2.5	Data Forwarding Information Request

This IE offers the possibility for the gNB-CU-CP to request data forwarding addresses to the gNB-CU-UP. It also offers the possibility for the gNB-CU-CP to provide a list of QoS flows subject to PDU Session level or DRB level data forwarding to the gNB to which DRBs or QoS flows have been offloaded.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Forwarding Request | M |  | ENUMERATED (UL, DL, both, …) |  |
| QoS Flows forwarded on the forwarding tunnel(s) | O |  | QoS Flow Mapping List 9.3.1.59 | This IE contains information for which QoS flows forwarded data packets are sent on: - either the PDU Session forwarding tunnel (UL and DL) - or the DRB forwarding tunnel (UL and DL). |

#### 9.3.2.6	Data Forwarding Information

This IE provides the data forwarding information when performing handover or data offloading.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL Data Forwarding | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| DL Data Forwarding | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| Data Forwarding to NG-RAN QoS Flow Information List |  | 0..1 |  | Providing QoS flows accepted for data forwarding to the source gNB-CU-UP. | YES | ignore |
| >Data Forwarding to NG-RAN QoS Flow Information List Item |  | 1..<maxnoofQoSflows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.24 |  | - | - |

#### 9.3.2.7	Transport Network Layer Address Info

This IE is used for signalling TNL address information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport UP Layer Addresses Info to Add List |  | 0..1 |  |  |
| >Transport UP Layer Addresses Info to Add Item |  | 1..<maxnoofTLAs> |  |  |
| >>IPsec Transport Layer Address | M |  | Transport Layer Address 9.3.2.4 | Transport Network Layer address for IPsec endpoint. |
| >>GTP Transport Layer Addresses To Add List |  | 0..1 |  |  |
| >>>GTP Transport Layer Addresses To Add Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | Transport Layer Address 9.3.2.4 | GTP Transport Layer Addresses for GTP end-points. |
| Transport UP Layer Addresses Info to Remove List |  | 0..1 |  |  |
| >Transport UP Layer Addresses Info to Remove Item |  | 1..<maxnoofTLAs> |  |  |
| >>IPsec Transport Layer Address | M |  | Transport Layer Address 9.3.2.4 | Transport Network Layer address for IPsec endpoint. |
| >>GTP Transport Layer Addresses To Remove List |  | 0..1 |  |  |
| >>>GTP Transport Layer Addresses To Remove Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | Transport Layer Address 9.3.2.4 | GTP Transport Layer Addresses for GTP end-points. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAs | Maximum no. of Transport Layer Addresses in the message. Value is 16. |
| maxnoofGTPTLAs | Maximum no. of GTP Transport Layer Addresses for a GTP end-point in the message. Value is 16. |

#### 9.3.2.8	URI

This IE is defined to contain a URI  address.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| URI | M |  | VisibleString | String representing URI (Uniform Resource Identifier) |

### 9.3.3	Container and List IE definitions

#### 9.3.3.1	DRB To Setup List E-UTRAN

This IE contains DRB related information used at Bearer Context Setup Request in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB To Setup Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >PDCP Configuration | M |  | 9.3.1.38 |  |
| >E-UTRAN QoS | M |  | 9.3.1.17 |  |
| >S1 UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >Data Forwarding Information Request | O |  | 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. |
| >Cell Group Information | M |  | 9.3.1.11 |  |
| >DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  |
| >DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. |
| >Existing Allocated S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | This IE is not used in this version of the specification. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when at least one Non-GBR QoS Flows is being setup. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Existing Allocated NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB. | YES | ignore |
| >>>DAPS Request Information | O |  | 9.3.1.91 |  | YES | ignore |
| >>>Ignore Mapping Rule Indication | O |  | ENUMERATED (True, …) | Included if the QoS flow mapping rule for the DRB has not been decided by gNB-CU-CP. | YES | reject |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | Indicates that the target gNB-CU-CP requests QoS flow remapping during an intra-system lossless handover as specified in TS 38.300 [4]. | YES | reject |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |
| >Redundant PDU Session Information | O |  | 9.3.1.80 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.3	DRB Setup List E-UTRAN

This IE contains setup DRB related information at Bearer Context Setup Response in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Setup Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >S1 DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. |
| >UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  |
| >S1 DL UP Unchanged | O |  | ENUMERATED (True, …) | This IE is not used in this version of the specification. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.4	DRB Failed List E-UTRAN

This IE contains failed to setup DRB related information at Bearer Context Setup Response in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Failed Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.5	PDU Session Resource Setup List

This IE contains setup PDU session resource related information used at Bearer Context Setup Response

| IE/Group Name |  | Presence | Range |  | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup Item |  |  | 1..<maxnoofPDUSessionResource> |  |  |  | - | - |  |
| >PDU Session ID |  | M |  |  | 9.3.1.21 |  | - | - |  |
| >Security Result |  | O |  |  | 9.3.1.52 |  | - | - |  |
| >NG DL UP Transport Layer Information |  | M |  |  | UP Transport Layer Information 9.3.2.1 |  | - | - |  |
| >PDU Session Data Forwarding Information Response |  | O |  |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |  |
| >NG DL UP Unchanged |  | O |  |  | ENUMERATED (True, …) |  | - | - |  |
| >DRB Setup List |  |  | 1 |  |  |  | - | - |  |
| >>DRB Setup Item |  |  | 1..<maxnoofDRBs> |  |  |  | - | - |  |
| >>>DRB ID |  | M |  |  | 9.3.1.16 |  | - | - |  |
| >>>DRB Data forwarding information Response |  | O |  |  | Data Forwarding Information 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |  |
| >>>UL UP Parameters |  | M |  |  | UP Parameters 9.3.1.13 |  | - | - |  |
| >>>Flow Setup List |  | M |  |  | QoS Flow List 9.3.1.12 |  | - | - |  |
| >>>Flow Failed List |  | O |  |  | Flow Failed List 9.3.1.45 |  | - | - |  |
| >DRB Failed List |  |  | 0.. 1 |  |  |  | - | - |  |
| >>DRB Failed Item |  |  | 1..<maxnoofDRBs> |  |  |  | - | - |  |
| >>>DRB ID |  | M |  |  | 9.3.1.16 |  | - | - |  |
| >>>Cause |  | M |  |  | 9.3.1.2 |  | - | - |  |
| >Redundant NG DL UP Transport Layer Information |  | O |  |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |  |
| >Used Redundant PDU Session Information |  | O |  |  | 9.3.1.80 |  | YES | ignore |  |
| Range bound |  |  | Explanation |  |  |  |  |
| maxnoofDRBs |  |  | Maximum no. of DRBs for a UE. Value is 32. |  |  |  |  |
| maxnoofPDUSessionResource |  |  | Maximum no. of PDU Sessions for a UE. Value is 256. |  |  |  |  |

#### 9.3.3.6	PDU Session Resource Failed List

This IE contains failed PDU session resource related information used at Bearer Context Setup Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.7	DRB To Setup Modification List E-UTRAN

This IE contains DRB to setup related information used at Bearer Context Modification Request in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB To Setup Modification Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >PDCP Configuration | M |  | 9.3.1.38 |  |
| >E-UTRAN QoS | M |  | 9.3.1.17 |  |
| >S1 UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >Data Forwarding Information Request | O |  | 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. |
| >Cell Group Information | M |  | 9.3.1.11 |  |
| >DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  |
| >DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.8	DRB To Modify List E-UTRAN

This IE contains DRB to modify related information used at Bearer Context Modification Request in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB To Modify Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >PDCP Configuration | O |  | 9.3.1.38 |  |
| >E-UTRAN QoS | O |  | 9.3.1.17 |  |
| >S1 UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |
| >Data Forwarding Information | O |  | 9.3.2.6 | Providing forwarding info to the source gNB-CU-UP. |
| >PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. |
| >PDCP SN Status Information | O |  | 9.3.1.58 | Providing SN Status information to the target gNB-CU-UP. |
| >DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  |
| >Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  |
| >Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  |
| >Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  |
| >DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.9	DRB To Remove List E-UTRAN

This IE contains DRB to remove related information used at Bearer Context Modification Request in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB To Remove Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.10	PDU Session Resource To Setup Modification List

This IE contains PDU session resource to setup related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when Non-GBR QoS Flows are setting up. | - | - |
| >NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 |  | - | - |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>Ignore Mapping Rule Indication | O |  | ENUMERATED (True, …) | Included if the QoS flow mapping rule for the DRB has not been decided by gNB-CU-CP. | YES | reject |
| >>>DAPS Request Information | O |  | 9.3.1.91 | This IE is not used in this version of the specification. | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Indication | O |  | 9.3.1.23 | This IE is not used in this release. | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >PDU Session Data Forwarding Information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flow Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>Ignore Mapping Rule Indication | O |  | ENUMERATED (True, …) | Included if the QoS flow mapping rule for the DRB has not been decided by gNB-CU-CP. | YES | reject |
| >>>DAPS Request Information | O |  | 9.3.1.91 | Used to request intra-gNB-CU-UP DAPS HO | YES | ignore |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | This IE is not used in this version of the specification. | YES | reject |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>DRB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | Indicates that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. | YES | reject |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>Early Forwarding COUNT Request | O |  | ENUMERATED (First DL count, DL discarding, …) | Requests early data forwarding information from the source gNB-CU-UP | YES | reject |
| >>>Early Forwarding COUNT Information | O |  | 9.3.1.92 | Provides early data forwarding information to the target gNB-CU-UP. | YES | reject |
| >>>DAPS Request Information | O |  | 9.3.1.91 | Used to request intra-gNB-CU-UP DAPS HO | YES | ignore |
| >>>Early Data Forwarding Indicator | O |  | ENUMERATED (stop, …) |  | YES | ignore |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |
| >Data Forwarding to E-UTRAN Information List |  | 0.. 1 |  | Contains a list of DL Data Forwarding tunnels and the associated QoS Flows to be forwarded on each tunnel | YES | ignore |
| >>Data Forwarding to E-UTRAN Information List Item |  | 1..<maxnoofDataForwardingTunneltoE-UTRAN> |  |  | - | - |
| >>>Data forwarding tunnel information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>QoS Flows to be forwarded List |  | 1 |  |  | - | - |
| >>>>QoS Flows to be forwarded Item |  | 1..<maxnoofQoSflows> |  |  | - | - |
| >>>>>QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.24 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |
| maxnoofDataForwardingTunneltoE-UTRAN | Maximum no. of Data Forwarding Tunnels to E-UTRAN for a UE. Value is 256. |
| maxnoofQoSflows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

#### 9.3.3.12	PDU Session Resource To Remove List

This IE contains PDU session resource to remove related information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Cause | O |  | 9.3.1.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.13	DRB Setup Modification List E-UTRAN

This IE contains setup DRB related information at Bearer Context Modification Response in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Setup Modification Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >S1 DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >Data Forwarding Information Response | O |  | 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. |
| >UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.14	DRB Failed Modification List E-UTRAN

This IE contains failed to setup DRB related information at Bearer Context Modification Response in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Failed Modification Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.15	DRB Modified List E-UTRAN

This IE contains modified DRB related information at Bearer Context Modification Response in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Modified Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |
| >PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status from the source gNB-CU-UP. |
| >UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.16	DRB Failed To Modify List E-UTRAN

This IE contains failed to modify DRB related information at Bearer Context Modification Response in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Failed To Modify Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.17	PDU Session Resource Setup Modification List

This IE contains setup PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Result | O |  | 9.3.1.52 |  | - | - |
| >NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >DRB Setup List |  | 1 |  |  | - | - |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.18	PDU Session Resource Failed Modification List

This IE contains failed to setup PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource Failed Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.19	PDU Session Resource Modified List

This IE contains modified PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  | - |  |
| >PDU Session ID | M |  | 9.3.1.21 |  | - |  |
| >NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - |  |
| >Security Result | O |  | 9.3.1.52 |  | - |  |
| >PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information 9.3.2.6 |  | - |  |
| >DRB Setup List |  | 0.. 1 |  |  | - |  |
| >>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>>DRB Data forwarding information Response | O |  | Data Forwarding Information 9.3.2.6 |  | - |  |
| >>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - |  |
| >>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - |  |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - |  |
| >DRB Failed List |  | 0.. 1 |  |  | - |  |
| >>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>>Cause | M |  | 9.3.1.2 |  | - |  |
| >DRB Modified List |  | 0.. 1 |  |  | - |  |
| >>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters. | - |  |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides PDCP SN Status to the target gNB-CU-UP. | - |  |
| >>>Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  | - |  |
| >>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - |  |
| >>>Early Forwarding COUNT Information | O |  | 9.3.1.92 | Provides early data forwarding information from the source gNB-CU-UP. | - |  |
| >>> Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | Indicates the QoS flow(s) for which the gNB-CU-UP has not yet received SDAP end markers after the gNB-CU-CP reconfigured those QoS flow(s) to another DRB. | Yes | ignore |
| >DRB Failed To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.20	PDU Session Resource Failed To Modify List

This IE contains failed to modify PDU session resource related information used at Bearer Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource Failed To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.21	DRB Required To Modify List E-UTRAN

This IE contains DRB to modify related information used at Bearer Context Modification Required in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Required To Modify Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |
| >gNB-CU-UP Cell Group Related Configuration | O |  | 9.3.1.34 |  |
| >Cause | O |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.22	DRB Required To Remove List E-UTRAN

This IE contains DRB to remove related information used at Bearer Context Modification Required in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Required To Remove Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.23	PDU Session Resource Required To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Required

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource Required To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>gNB-CU-UP Cell Group Related Configuration | O |  | 9.3.1.34 |  | - | - |
| >>>Flow To Remove | O |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>Cause | O |  | 9.3.1.2 |  | - | - |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>Cause | M |  | 9.3.1.2 |  | - | - |
| >Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.24	DRB Confirm Modified List E-UTRAN

This IE contains modified DRB related information at Bearer Context Modification Confirm in E-UTRAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB Confirm Modified Item E-UTRAN |  | 1..<maxnoofDRBs> |  |  |
| >DRB ID | M |  | 9.3.1.16 |  |
| >Cell Group Information | O |  | 9.3.1.11 | Included if the gNB-CU-CP was unable to change cell group related information as requested in the gNB-CU-UP Cell Group Related Configuration IE (e.g., UL Configuration). |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.3.25	PDU Session Resource Confirm Modified List

This IE contains modified PDU session resource related information used at Bearer Context Modification Confirm

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >PDU Session ID | M |  | 9.3.1.21 |  |
| >DRB Modified List |  | 0.. 1 |  |  |
| >>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  |
| >>>DRB ID | M |  | 9.3.1.16 |  |
| >>>Cell Group Information | O |  | 9.3.1.11 | Included if the gNB-CU-CP was unable to change cell group related information as requested in the gNB-CU-UP Cell Group Related Configuration IE (e.g., UL Configuration). |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

### 9.4.1	General

E1AP ASN.1 definition conforms to ITU-T Rec. X.691 [7], ITU-T Rec. X.680 [8] and ITU-T Rec. X.681 [9].

The ASN.1 definition specifies the structure and content of E1AP messages. E1AP messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrence being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct an E1AP message according to the PDU definitions module and with the following additional rules:

-	IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

-	Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e., an IE list) then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an IE container list where the list elements reside. The second part defines list elements. The IE container list appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list elements.

NOTE:	In the above “IE” means an IE in the object set with an explicit ID. If one IE needs to appear more than once in one object set, then the different occurrences will have different IE IDs.

If an E1AP message that is not constructed as defined above is received, this shall be considered as Abstract Syntax Error, and the message shall be handled as defined for Abstract Syntax Error in clause 10.

### 9.4.2	Usage of private message mechanism for non-standard use

The private message mechanism for non-standard use may be used:

-	for special operator- (and/or vendor) specific features considered not to be part of the basic functionality, i.e., the functionality required for a complete and high-quality specification in order to guarantee multivendor interoperability;

-	by vendors for research purposes, e.g., to implement and evaluate new algorithms/features before such features are proposed for standardisation.

The private message mechanism shall not be used for basic functionality. Such functionality shall be standardised.

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM E1AP-CommonDataTypes

Reset,

ResetAcknowledge,

ErrorIndication,

GNB-CU-UP-E1SetupRequest,

GNB-CU-UP-E1SetupResponse,

GNB-CU-UP-E1SetupFailure,

GNB-CU-CP-E1SetupRequest,

GNB-CU-CP-E1SetupResponse,

GNB-CU-CP-E1SetupFailure,

GNB-CU-UP-ConfigurationUpdate,

GNB-CU-UP-ConfigurationUpdateAcknowledge,

GNB-CU-UP-ConfigurationUpdateFailure,

GNB-CU-CP-ConfigurationUpdate,

GNB-CU-CP-ConfigurationUpdateAcknowledge,

GNB-CU-CP-ConfigurationUpdateFailure,

BCBearerContextSetupRequest,

BCBearerContextSetupResponse,

BCBearerContextSetupFailure,

BCBearerContextModificationRequest,

BCBearerContextModificationResponse,

BCBearerContextModificationFailure,

BCBearerContextModificationRequired,

BCBearerContextModificationConfirm,

BCBearerContextReleaseCommand,

BCBearerContextReleaseComplete,

BCBearerContextReleaseRequest,

BearerContextSetupRequest,

BearerContextSetupResponse,

BearerContextSetupFailure,

BearerContextModificationRequest,

BearerContextModificationResponse,

BearerContextModificationFailure,

BearerContextModificationRequired,

BearerContextModificationConfirm,

BearerContextReleaseCommand,

BearerContextReleaseComplete,

BearerContextReleaseRequest,

BearerContextInactivityNotification,

DLDataNotification,

ULDataNotification,

DataUsageReport,

E1ReleaseRequest,

E1ReleaseResponse,

GNB-CU-UP-CounterCheckRequest,

GNB-CU-UP-StatusIndication,

MCBearerContextSetupRequest,

MCBearerContextSetupResponse,

MCBearerContextSetupFailure,

MCBearerContextModificationRequest,

MCBearerContextModificationResponse,

MCBearerContextModificationFailure,

MCBearerContextModificationRequired,

MCBearerContextModificationConfirm,

MCBearerContextReleaseCommand,

MCBearerContextReleaseComplete,

MCBearerContextReleaseRequest,

MRDC-DataUsageReport,

DeactivateTrace,

TraceStart,

PrivateMessage,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

IAB-UPTNLAddressUpdate,

IAB-UPTNLAddressUpdateAcknowledge,

IAB-UPTNLAddressUpdateFailure,

CellTrafficTrace,

EarlyForwardingSNTransfer,

GNB-CU-CPMeasurementResultsInformation

FROM E1AP-PDU-Contents

id-reset,

id-errorIndication,

id-gNB-CU-UP-E1Setup,

id-gNB-CU-CP-E1Setup,

id-gNB-CU-UP-ConfigurationUpdate,

id-gNB-CU-CP-ConfigurationUpdate,

id-e1Release,

id-bearerContextSetup,

id-bearerContextModification,

id-bearerContextModificationRequired,

id-bearerContextRelease,

id-bearerContextReleaseRequest,

id-bearerContextInactivityNotification,

id-dLDataNotification,

id-uLDataNotification,

id-dataUsageReport,

id-gNB-CU-UP-CounterCheck,

id-gNB-CU-UP-StatusIndication,

id-mRDC-DataUsageReport,

id-DeactivateTrace,

id-TraceStart,

id-privateMessage,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-iAB-UPTNLAddressUpdate,

id-CellTrafficTrace,

id-earlyForwardingSNTransfer,

id-gNB-CU-CPMeasurementResultsInformation,

id-BCBearerContextSetup,

id-BCBearerContextModification,

id-BCBearerContextModificationRequired,

id-BCBearerContextRelease,

id-BCBearerContextReleaseRequest,

id-MCBearerContextSetup,

id-MCBearerContextModification,

id-MCBearerContextModificationRequired,

id-MCBearerContextRelease,

id-MCBearerContextReleaseRequest

FROM E1AP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

E1AP-PDU ::= CHOICE {

initiatingMessage		InitiatingMessage,

successfulOutcome		SuccessfulOutcome,

unsuccessfulOutcome		UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode			E1AP-ELEMENTARY-PROCEDURE.&procedureCode		({E1AP-ELEMENTARY-PROCEDURES}),

criticality				E1AP-ELEMENTARY-PROCEDURE.&criticality			({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value					E1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({E1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

E1AP-ELEMENTARY-PROCEDURES E1AP-ELEMENTARY-PROCEDURE ::= {

E1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

E1AP-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-1 E1AP-ELEMENTARY-PROCEDURE ::= {

reset										|

gNB-CU-UP-E1Setup							|

gNB-CU-CP-E1Setup							|

gNB-CU-UP-ConfigurationUpdate				|

gNB-CU-CP-ConfigurationUpdate				|

e1Release									|

bearerContextSetup							|

bearerContextModification					|

bearerContextModificationRequired				|

bearerContextRelease							|

resourceStatusReportingInitiation				|

iAB-UPTNLAddressUpdate						|

bCBearerContextSetup							|

bCBearerContextModification					|

bCBearerContextModificationRequired			|

bCBearerContextRelease						|

mCBearerContextSetup							|

mCBearerContextModification					|

mCBearerContextModificationRequired			|

mCBearerContextRelease						,

...

}

E1AP-ELEMENTARY-PROCEDURES-CLASS-2 E1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

bearerContextReleaseRequest				|

bearerContextInactivityNotification		|

dLDataNotification						|

uLDataNotification						|

dataUsageReport							|

gNB-CU-UP-CounterCheck					|

gNB-CU-UP-StatusIndication			|

mRDC-DataUsageReport					|

deactivateTrace							|

traceStart								|

privateMessage							|

cellTrafficTrace						|

resourceStatusReporting					|

earlyForwardingSNTransfer				|

gNB-CU-CPMeasurementResultsInformation	|

bCBearerContextReleaseRequest			|

mCBearerContextReleaseRequest			,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

reset E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-reset

CRITICALITY				reject

}

errorIndication E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

gNB-CU-UP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-UP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-UP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-UP-E1Setup

CRITICALITY				reject

}

gNB-CU-CP-E1Setup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-E1SetupRequest

SUCCESSFUL OUTCOME		GNB-CU-CP-E1SetupResponse

UNSUCCESSFUL OUTCOME	GNB-CU-CP-E1SetupFailure

PROCEDURE CODE			id-gNB-CU-CP-E1Setup

CRITICALITY				reject

}

gNB-CU-UP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-UP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-UP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-UP-ConfigurationUpdate

CRITICALITY				reject

}

gNB-CU-CP-ConfigurationUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CP-ConfigurationUpdate

SUCCESSFUL OUTCOME		GNB-CU-CP-ConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNB-CU-CP-ConfigurationUpdateFailure

PROCEDURE CODE			id-gNB-CU-CP-ConfigurationUpdate

CRITICALITY				reject

}

e1Release E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E1ReleaseRequest

SUCCESSFUL OUTCOME		E1ReleaseResponse

PROCEDURE CODE			id-e1Release

CRITICALITY				reject

}

bearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextSetupRequest

SUCCESSFUL OUTCOME		BearerContextSetupResponse

UNSUCCESSFUL OUTCOME	BearerContextSetupFailure

PROCEDURE CODE			id-bearerContextSetup

CRITICALITY				reject

}

bearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequest

SUCCESSFUL OUTCOME		BearerContextModificationResponse

UNSUCCESSFUL OUTCOME	BearerContextModificationFailure

PROCEDURE CODE			id-bearerContextModification

CRITICALITY				reject

}

bearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextModificationRequired

SUCCESSFUL OUTCOME		BearerContextModificationConfirm

PROCEDURE CODE			id-bearerContextModificationRequired

CRITICALITY				reject

}

bearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseCommand

SUCCESSFUL OUTCOME		BearerContextReleaseComplete

PROCEDURE CODE			id-bearerContextRelease

CRITICALITY				reject

}

bearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextReleaseRequest

PROCEDURE CODE			id-bearerContextReleaseRequest

CRITICALITY				ignore

}

bearerContextInactivityNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BearerContextInactivityNotification

PROCEDURE CODE			id-bearerContextInactivityNotification

CRITICALITY				ignore

}

dLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLDataNotification

PROCEDURE CODE			id-dLDataNotification

CRITICALITY				ignore

}

uLDataNotification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULDataNotification

PROCEDURE CODE			id-uLDataNotification

CRITICALITY				ignore

}

dataUsageReport E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DataUsageReport

PROCEDURE CODE			id-dataUsageReport

CRITICALITY				ignore

}

gNB-CU-UP-CounterCheck E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-CounterCheckRequest

PROCEDURE CODE			id-gNB-CU-UP-CounterCheck

CRITICALITY				ignore

}

gNB-CU-UP-StatusIndication 	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-UP-StatusIndication

PROCEDURE CODE			id-gNB-CU-UP-StatusIndication

CRITICALITY			ignore

}

privateMessage E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

gNB-CU-CPMeasurementResultsInformation E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNB-CU-CPMeasurementResultsInformation

PROCEDURE CODE			id-gNB-CU-CPMeasurementResultsInformation

CRITICALITY				ignore

}

mRDC-DataUsageReport	E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MRDC-DataUsageReport

PROCEDURE CODE			id-mRDC-DataUsageReport

CRITICALITY				ignore

}

deactivateTrace E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

traceStart E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

resourceStatusReportingInitiation E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusRequest

SUCCESSFUL OUTCOME		ResourceStatusResponse

UNSUCCESSFUL OUTCOME	ResourceStatusFailure

PROCEDURE CODE			id-resourceStatusReportingInitiation

CRITICALITY				reject

}

resourceStatusReporting E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

iAB-UPTNLAddressUpdate E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IAB-UPTNLAddressUpdate

SUCCESSFUL OUTCOME		IAB-UPTNLAddressUpdateAcknowledge

UNSUCCESSFUL OUTCOME	IAB-UPTNLAddressUpdateFailure

PROCEDURE CODE			id-iAB-UPTNLAddressUpdate

CRITICALITY				reject

}

cellTrafficTrace E1AP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE CellTrafficTrace

PROCEDURE CODE		id-CellTrafficTrace

CRITICALITY			ignore

}

earlyForwardingSNTransfer E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		EarlyForwardingSNTransfer

PROCEDURE CODE			id-earlyForwardingSNTransfer

CRITICALITY				ignore

}

bCBearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BCBearerContextSetupRequest

SUCCESSFUL OUTCOME		BCBearerContextSetupResponse

UNSUCCESSFUL OUTCOME		BCBearerContextSetupFailure

PROCEDURE CODE			id-BCBearerContextSetup

CRITICALITY				reject

}

bCBearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BCBearerContextModificationRequest

SUCCESSFUL OUTCOME		BCBearerContextModificationResponse

UNSUCCESSFUL OUTCOME		BCBearerContextModificationFailure

PROCEDURE CODE			id-BCBearerContextModification

CRITICALITY				reject

}

bCBearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BCBearerContextModificationRequired

SUCCESSFUL OUTCOME		BCBearerContextModificationConfirm

PROCEDURE CODE			id-BCBearerContextModificationRequired

CRITICALITY				reject

}

bCBearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BCBearerContextReleaseCommand

SUCCESSFUL OUTCOME		BCBearerContextReleaseComplete

PROCEDURE CODE			id-BCBearerContextRelease

CRITICALITY				reject

}

bCBearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BCBearerContextReleaseRequest

PROCEDURE CODE			id-BCBearerContextReleaseRequest

CRITICALITY				reject

}

mCBearerContextSetup E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextSetupRequest

SUCCESSFUL OUTCOME		MCBearerContextSetupResponse

UNSUCCESSFUL OUTCOME		MCBearerContextSetupFailure

PROCEDURE CODE			id-MCBearerContextSetup

CRITICALITY				reject

}

mCBearerContextModification E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextModificationRequest

SUCCESSFUL OUTCOME		MCBearerContextModificationResponse

UNSUCCESSFUL OUTCOME		MCBearerContextModificationFailure

PROCEDURE CODE			id-MCBearerContextModification

CRITICALITY				reject

}

mCBearerContextModificationRequired E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextModificationRequired

SUCCESSFUL OUTCOME		MCBearerContextModificationConfirm

PROCEDURE CODE			id-MCBearerContextModificationRequired

CRITICALITY				reject

}

mCBearerContextRelease E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextReleaseCommand

SUCCESSFUL OUTCOME		MCBearerContextReleaseComplete

PROCEDURE CODE			id-MCBearerContextRelease

CRITICALITY				reject

}

mCBearerContextReleaseRequest E1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MCBearerContextReleaseRequest

PROCEDURE CODE			id-MCBearerContextReleaseRequest

CRITICALITY				reject

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-MBS-E1AP-ID,

GNB-CU-UP-MBS-E1AP-ID,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

Extended-GNB-CU-UP-Name,

GNB-CU-CP-Name,

Extended-GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Measurement-Results-Information-List,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

ExtendedSliceSupportList,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

GNB-CU-UP-TNLA-To-Remove-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RANUEID,

GNB-DU-ID,

TraceID,

TraceActivation,

SubscriberProfileIDforRFP,

AdditionalRRMPriorityIndex,

RetainabilityMeasurementsInfo,

Transport-Layer-Address-Info,

HW-CapacityIndicator,

RegistrationRequest,

ReportCharacteristics,

ReportingPeriodicity,

TNL-AvailableCapacityIndicator,

DLUPTNLAddressToUpdateItem,

ULUPTNLAddressToUpdateItem,

NPNContextInfo,

NPNSupportInfo,

MDTPLMNList,

PrivacyIndicator,

URIaddress,

DRBs-Subject-To-Early-Forwarding-List,

CHOInitiation,

ExtendedSliceSupportList,

TransportLayerAddress,

AdditionalHandoverInfo,

Extended-NR-CGI-Support-List,

DirectForwardingPathAvailability,

GlobalMBSSessionID,

BCBearerContextToSetup,

BCBearerContextToSetupResponse,

BCBearerContextToModify,

BCBearerContextToModifyResponse,

BCBearerContextToModifyRequired,

BCBearerContextToModifyConfirm,

MCBearerContextToSetup,

MCBearerContextToSetupResponse,

MCBearerContextToModify,

MCBearerContextToModifyResponse,

MCBearerContextToModifyRequired,

MCBearerContextToModifyConfirm,

MBSMulticastF1UContextDescriptor

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-Extended-GNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-Extended-GNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-NPNSupportInfo,

id-NPNContextInfo,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-GNB-CU-UP-TNLA-To-Remove-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Measurement-Results-Information-List,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RANUEID,

id-GNB-DU-ID,

id-TraceID,

id-TraceActivation,

id-SubscriberProfileIDforRFP,

id-AdditionalRRMPriorityIndex,

id-RetainabilityMeasurementsInfo,

id-Transport-Layer-Address-Info,

id-gNB-CU-CP-Measurement-ID,

id-gNB-CU-UP-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-ReportingPeriodicity,

id-TNL-AvailableCapacityIndicator,

id-HW-CapacityIndicator,

id-DLUPTNLAddressToUpdateList,

id-ULUPTNLAddressToUpdateList,

id-ManagementBasedMDTPLMNList,

id-TraceCollectionEntityIPAddress,

id-PrivacyIndicator,

id-URIaddress,

id-DRBs-Subject-To-Early-Forwarding-List,

id-CHOInitiation,

id-ExtendedSliceSupportList,

id-AdditionalHandoverInfo,

id-Extended-NR-CGI-Support-List,

id-DirectForwardingPathAvailability,

id-GNB-CU-CP-MBS-E1AP-ID,

id-GNB-CU-UP-MBS-E1AP-ID,

id-GlobalMBSSessionID,

id-BCBearerContextToSetup,

id-BCBearerContextToSetupResponse,

id-BCBearerContextToModify,

id-BCBearerContextToModifyResponse,

id-BCBearerContextToModifyRequired,

id-BCBearerContextToModifyConfirm,

id-MCBearerContextToSetup,

id-MCBearerContextToSetupResponse,

id-MCBearerContextToModify,

id-MCBearerContextToModifyResponse,

id-MCBearerContextToModifyRequired,

id-MCBearerContextToModifyConfirm,

id-MBSMulticastF1UContextDescriptor,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset,

maxnoofTNLAddresses

FROM E1AP-Constants;

-- **************************************************************

--

#### -- RESET

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

e1-Interface					ResetAll,

partOfE1-Interface				UE-associatedLogicalE1-ConnectionListRes,

choice-extension				ProtocolIE-SingleContainer	{{ResetType-ExtIEs}}

}

ResetType-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalE1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualE1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalE1-ConnectionItemRes } }

UE-associatedLogicalE1-ConnectionItemRes E1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalE1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalE1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-UE-associatedLogicalE1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalE1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalE1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualE1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalE1-ConnectionItemResAck } }

UE-associatedLogicalE1-ConnectionItemResAck 	E1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalE1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalE1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY ignore	TYPE GNB-CU-CP-UE-E1AP-ID		PRESENCE optional}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID		PRESENCE optional}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- GNB-CU-UP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Request

--

-- **************************************************************

GNB-CU-UP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupRequestIEs} },

...

}

GNB-CU-UP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-UP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-UP-Name		PRESENCE optional	},

...

}

SupportedPLMNs-List	::=	SEQUENCE (SIZE (1..maxnoofSPLMNs)) OF SupportedPLMNs-Item

SupportedPLMNs-Item ::= SEQUENCE {

pLMN-Identity					PLMN-Identity,

slice-Support-List				Slice-Support-List												OPTIONAL,

nR-CGI-Support-List				NR-CGI-Support-List												OPTIONAL,

qoS-Parameters-Support-List		QoS-Parameters-Support-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SupportedPLMNs-ExtIEs } }		OPTIONAL,

...

}

SupportedPLMNs-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NPNSupportInfo					CRITICALITY reject	EXTENSION NPNSupportInfo					PRESENCE optional}|

{ ID id-ExtendedSliceSupportList		CRITICALITY reject	EXTENSION ExtendedSliceSupportList 		PRESENCE optional}|

{ ID id-Extended-NR-CGI-Support-List	CRITICALITY ignore	EXTENSION Extended-NR-CGI-Support-List	PRESENCE optional},

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Response

--

-- **************************************************************

GNB-CU-UP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupResponseIEs} },

...

}

GNB-CU-UP-E1SetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-CP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-CP-Name			PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-UP E1 Setup Failure

--

-- **************************************************************

GNB-CU-UP-E1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-E1SetupFailureIEs} },

...

}

GNB-CU-UP-E1SetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-CP E1 SETUP

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Request

--

-- **************************************************************

GNB-CU-CP-E1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupRequestIEs} },

...

}

GNB-CU-CP-E1SetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-CP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-CP-Name			PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Response

--

-- **************************************************************

GNB-CU-CP-E1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupResponseIEs} },

...

}

GNB-CU-CP-E1SetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-CNSupport							CRITICALITY reject	TYPE CNSupport						PRESENCE mandatory	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-UP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-UP-Name		PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-CP E1 Setup Failure

--

-- **************************************************************

GNB-CU-CP-E1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-E1SetupFailureIEs} },

...

}

GNB-CU-CP-E1SetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-UP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-UP Configuration Update

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-ID						CRITICALITY reject	TYPE GNB-CU-UP-ID					PRESENCE mandatory }|

{ ID id-gNB-CU-UP-Name						CRITICALITY ignore	TYPE GNB-CU-UP-Name					PRESENCE optional	}|

{ ID id-SupportedPLMNs						CRITICALITY reject	TYPE SupportedPLMNs-List			PRESENCE optional	}|

{ ID id-gNB-CU-UP-Capacity					CRITICALITY ignore	TYPE GNB-CU-UP-Capacity			   PRESENCE optional	}|

{ ID id-GNB-CU-UP-TNLA-To-Remove-List		CRITICALITY reject	TYPE GNB-CU-UP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-UP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-UP-Name		PRESENCE optional	},

...

}

GNB-CU-UP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-UP-TNLA-To-Remove-Item

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-CU-UP Configuration Update Failure

--

-- **************************************************************

GNB-CU-UP-ConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-UP-ConfigurationUpdateFailureIEs} },

...

}

GNB-CU-UP-ConfigurationUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU-CP CONFIGURATION UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU-CP Configuration Update

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-gNB-CU-CP-Name						CRITICALITY ignore	TYPE GNB-CU-CP-Name						PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Add-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Add-List		PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Remove-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Remove-List	PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-To-Update-List		CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-To-Update-List	PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	}|

{ ID id-Extended-GNB-CU-CP-Name				CRITICALITY ignore	TYPE Extended-GNB-CU-CP-Name			PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-To-Add-List       ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Add-Item

GNB-CU-CP-TNLA-To-Remove-List 	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Remove-Item

GNB-CU-CP-TNLA-To-Update-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-To-Update-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Acknowledge

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Setup-List			CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Setup-List					PRESENCE optional	}|

{ ID id-GNB-CU-CP-TNLA-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-CP-TNLA-Failed-To-Setup-List		PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info	PRESENCE optional	},

...

}

GNB-CU-CP-TNLA-Setup-List				::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Setup-Item

GNB-CU-CP-TNLA-Failed-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF GNB-CU-CP-TNLA-Failed-To-Setup-Item

-- **************************************************************

--

-- GNB-CU-CP Configuration Update Failure

--

-- **************************************************************

GNB-CU-CP-ConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNB-CU-CP-ConfigurationUpdateFailureIEs} },

...

}

GNB-CU-CP-ConfigurationUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- E1 RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- E1 Release Request

--

-- **************************************************************

E1ReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E1ReleaseRequestIEs} },

...

}

E1ReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- E1 Release Response

--

-- **************************************************************

E1ReleaseResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {E1ReleaseResponseIEs} },

...

}

E1ReleaseResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Setup Request

--

-- **************************************************************

BearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupRequestIEs} },

...

}

BearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-SecurityInformation					CRITICALITY reject	TYPE SecurityInformation							PRESENCE mandatory }|

{ ID id-UEDLAggregateMaximumBitRate			CRITICALITY reject	TYPE BitRate										PRESENCE mandatory }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate	CRITICALITY reject		TYPE BitRate								PRESENCE optional	 }|

{ ID id-Serving-PLMN						CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE mandatory }|

{ ID id-ActivityNotificationLevel			CRITICALITY reject	TYPE ActivityNotificationLevel					PRESENCE mandatory }|

{ ID id-UE-Inactivity-Timer					CRITICALITY reject	TYPE Inactivity-Timer								PRESENCE optional  }|

{ ID id-BearerContextStatusChange			CRITICALITY reject	TYPE BearerContextStatusChange					PRESENCE optional  }|

{ ID id-System-BearerContextSetupRequest	CRITICALITY reject	TYPE System-BearerContextSetupRequest				PRESENCE mandatory }|

{ ID id-RANUEID								CRITICALITY ignore	TYPE RANUEID										PRESENCE optional }|

{ ID id-GNB-DU-ID							CRITICALITY ignore	TYPE GNB-DU-ID										PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation								PRESENCE optional }|

{ ID id-NPNContextInfo						CRITICALITY reject	TYPE NPNContextInfo									PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY	ignore	TYPE MDTPLMNList									PRESENCE optional}|

{ ID id-CHOInitiation						CRITICALITY reject	TYPE CHOInitiation									PRESENCE optional }|

{ ID id-AdditionalHandoverInfo				CRITICALITY ignore	TYPE AdditionalHandoverInfo							PRESENCE optional }|

{ ID id-DirectForwardingPathAvailability	CRITICALITY ignore	TYPE DirectForwardingPathAvailability			PRESENCE optional }|

{ ID id-gNB-CU-UP-UE-E1AP-ID				CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE optional },

...

}

System-BearerContextSetupRequest	::=	CHOICE {

e-UTRAN-BearerContextSetupRequest		ProtocolIE-Container		 	{{EUTRAN-BearerContextSetupRequest}},

nG-RAN-BearerContextSetupRequest		ProtocolIE-Container		 	{{NG-RAN-BearerContextSetupRequest}},

choice-extension						ProtocolIE-SingleContainer		{{System-BearerContextSetupRequest-ExtIEs}}

}

System-BearerContextSetupRequest-ExtIEs E1AP-PROTOCOL-IES::= {

...

}

EUTRAN-BearerContextSetupRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-To-Setup-List-EUTRAN		CRITICALITY reject	 TYPE DRB-To-Setup-List-EUTRAN		PRESENCE mandatory }|

{ ID id-SubscriberProfileIDforRFP		CRITICALITY ignore	 TYPE SubscriberProfileIDforRFP		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex		CRITICALITY ignore	 TYPE AdditionalRRMPriorityIndex	PRESENCE optional },

...

}

NG-RAN-BearerContextSetupRequest E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-To-Setup-List		CRITICALITY reject	 TYPE PDU-Session-Resource-To-Setup-List		PRESENCE mandatory },

...

}

-- **************************************************************

--

-- Bearer Context Setup Response

--

-- **************************************************************

BearerContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupResponseIEs} },

...

}

BearerContextSetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory	}|

{ ID id-System-BearerContextSetupResponse	CRITICALITY ignore	TYPE System-BearerContextSetupResponse				PRESENCE mandatory  },

...

}

System-BearerContextSetupResponse::=	CHOICE {

e-UTRAN-BearerContextSetupResponse		ProtocolIE-Container 			{{EUTRAN-BearerContextSetupResponse}},

nG-RAN-BearerContextSetupResponse		ProtocolIE-Container 			{{NG-RAN-BearerContextSetupResponse}},

choice-extension						ProtocolIE-SingleContainer		{{System-BearerContextSetupResponse-ExtIEs}}

}

System-BearerContextSetupResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextSetupResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-List-EUTRAN 		CRITICALITY ignore	 TYPE DRB-Setup-List-EUTRAN			PRESENCE mandatory }|

{ ID id-DRB-Failed-List-EUTRAN 		CRITICALITY ignore	 TYPE DRB-Failed-List-EUTRAN		PRESENCE optional  },

...

}

NG-RAN-BearerContextSetupResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-List 	CRITICALITY ignore	 TYPE PDU-Session-Resource-Setup-List		PRESENCE mandatory }|

{ ID id-PDU-Session-Resource-Failed-List 	CRITICALITY ignore	 TYPE PDU-Session-Resource-Failed-List		PRESENCE optional  },

...

}

-- **************************************************************

--

-- Bearer Context Setup Failure

--

-- **************************************************************

BearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextSetupFailureIEs} },

...

}

BearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY ignore	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Modification Request

--

-- **************************************************************

BearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequestIEs} },

...

}

BearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-SecurityInformation						CRITICALITY reject	TYPE SecurityInformation									PRESENCE optional  }|

{ ID id-UEDLAggregateMaximumBitRate				CRITICALITY reject	TYPE BitRate												PRESENCE optional  }|

{ ID id-UEDLMaximumIntegrityProtectedDataRate		CRITICALITY reject	TYPE BitRate											PRESENCE optional	 }|

{ ID id-BearerContextStatusChange				CRITICALITY reject	TYPE BearerContextStatusChange							PRESENCE optional  }|

{ ID id-New-UL-TNL-Information-Required			CRITICALITY reject	TYPE New-UL-TNL-Information-Required					PRESENCE optional  }|

{ ID id-UE-Inactivity-Timer						CRITICALITY reject	TYPE Inactivity-Timer										PRESENCE optional  }|

{ ID id-DataDiscardRequired					CRITICALITY ignore	TYPE DataDiscardRequired											PRESENCE optional  }|

{ ID id-System-BearerContextModificationRequest	CRITICALITY reject	TYPE System-BearerContextModificationRequest		PRESENCE optional  }|

{ ID id-RANUEID										CRITICALITY ignore	TYPE RANUEID												PRESENCE optional }|

{ ID id-GNB-DU-ID									CRITICALITY ignore	TYPE GNB-DU-ID												PRESENCE optional }|

{ ID id-ActivityNotificationLevel					CRITICALITY ignore	TYPE ActivityNotificationLevel							PRESENCE optional },

...

}

System-BearerContextModificationRequest	::=	CHOICE {

e-UTRAN-BearerContextModificationRequest		ProtocolIE-Container 			{{EUTRAN-BearerContextModificationRequest}},

nG-RAN-BearerContextModificationRequest			ProtocolIE-Container 			{{NG-RAN-BearerContextModificationRequest}},

choice-extension								ProtocolIE-SingleContainer		{{System-BearerContextModificationRequest-ExtIEs}}

}

System-BearerContextModificationRequest-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-To-Setup-Mod-List-EUTRAN		CRITICALITY reject	 TYPE DRB-To-Setup-Mod-List-EUTRAN		PRESENCE optional }|

{ ID id-DRB-To-Modify-List-EUTRAN			CRITICALITY reject	 TYPE DRB-To-Modify-List-EUTRAN			PRESENCE optional }|

{ ID id-DRB-To-Remove-List-EUTRAN			CRITICALITY reject	 TYPE DRB-To-Remove-List-EUTRAN			PRESENCE optional }|

{ ID id-SubscriberProfileIDforRFP			CRITICALITY ignore	 TYPE 	SubscriberProfileIDforRFP		PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex			CRITICALITY ignore	 TYPE 	AdditionalRRMPriorityIndex		PRESENCE optional },

...

}

NG-RAN-BearerContextModificationRequest E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-To-Setup-Mod-List	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Setup-Mod-List	PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Modify-List 	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Modify-List		PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Remove-List		CRITICALITY reject	 TYPE PDU-Session-Resource-To-Remove-List		PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Response

--

-- **************************************************************

BearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationResponseIEs} },

...

}

BearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-System-BearerContextModificationResponse	CRITICALITY ignore	TYPE System-BearerContextModificationResponse		PRESENCE optional  },

...

}

System-BearerContextModificationResponse	::=	CHOICE {

e-UTRAN-BearerContextModificationResponse			ProtocolIE-Container {{EUTRAN-BearerContextModificationResponse}},

nG-RAN-BearerContextModificationResponse			ProtocolIE-Container {{NG-RAN-BearerContextModificationResponse}},

choice-extension									ProtocolIE-SingleContainer {{System-BearerContextModificationResponse-ExtIEs}}

}

System-BearerContextModificationResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Setup-Mod-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Failed-Mod-List-EUTRAN				PRESENCE optional }|

{ ID id-DRB-Modified-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Modified-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-To-Modify-List-EUTRAN		CRITICALITY ignore	TYPE DRB-Failed-To-Modify-List-EUTRAN		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo			CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo				PRESENCE optional },

...

}

NG-RAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-Mod-List 		CRITICALITY reject	TYPE PDU-Session-Resource-Setup-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-Mod-List		CRITICALITY reject	TYPE PDU-Session-Resource-Failed-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Modified-List			CRITICALITY reject	TYPE PDU-Session-Resource-Modified-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Failed-To-Modify-List		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo				CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo						PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Failure

--

-- **************************************************************

BearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationFailureIEs} },

...

}

BearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Modification Required

--

-- **************************************************************

BearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationRequiredIEs} },

...

}

BearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID									PRESENCE mandatory }|

{ ID id-System-BearerContextModificationRequired	CRITICALITY reject	TYPE System-BearerContextModificationRequired			PRESENCE mandatory },

...

}

System-BearerContextModificationRequired	::=	CHOICE {

e-UTRAN-BearerContextModificationRequired		ProtocolIE-Container {{EUTRAN-BearerContextModificationRequired}},

nG-RAN-BearerContextModificationRequired		ProtocolIE-Container {{NG-RAN-BearerContextModificationRequired}},

choice-extension								ProtocolIE-SingleContainer {{System-BearerContextModificationRequired-ExtIEs}}

}

System-BearerContextModificationRequired-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationRequired E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Required-To-Modify-List-EUTRAN	CRITICALITY reject	TYPE DRB-Required-To-Modify-List-EUTRAN	PRESENCE optional }|

{ ID id-DRB-Required-To-Remove-List-EUTRAN	CRITICALITY reject	TYPE DRB-Required-To-Remove-List-EUTRAN	PRESENCE optional },

...

}

NG-RAN-BearerContextModificationRequired E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Required-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Required-To-Modify-List	PRESENCE optional }|

{ ID id-PDU-Session-Resource-To-Remove-List	CRITICALITY reject	 TYPE PDU-Session-Resource-To-Remove-List	PRESENCE optional },

...

}

-- **************************************************************

--

-- Bearer Context Modification Confirm

--

-- **************************************************************

BearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationConfirmIEs} },

...

}

BearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-System-BearerContextModificationConfirm		CRITICALITY ignore	TYPE System-BearerContextModificationConfirm		PRESENCE optional  },

...

}

System-BearerContextModificationConfirm	::=	CHOICE {

e-UTRAN-BearerContextModificationConfirm		ProtocolIE-Container {{EUTRAN-BearerContextModificationConfirm}},

nG-RAN-BearerContextModificationConfirm			ProtocolIE-Container {{NG-RAN-BearerContextModificationConfirm}},

choice-extension								ProtocolIE-SingleContainer {{System-BearerContextModificationConfirm-ExtIEs}}

}

System-BearerContextModificationConfirm-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationConfirm E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Confirm-Modified-List-EUTRAN	CRITICALITY ignore	 TYPE DRB-Confirm-Modified-List-EUTRAN	PRESENCE optional },

...

}

NG-RAN-BearerContextModificationConfirm E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Confirm-Modified-List	CRITICALITY ignore	 TYPE PDU-Session-Resource-Confirm-Modified-List	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Release Command

--

-- **************************************************************

BearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCommandIEs} },

...

}

BearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory },

...

}

-- **************************************************************

--

-- Bearer Context Release Complete

--

-- **************************************************************

BearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCompleteIEs} },

...

}

BearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	}|

{ ID id-RetainabilityMeasurementsInfo	CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo	PRESENCE optional },

...

}

-- **************************************************************

--

#### -- BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Release Request

--

-- **************************************************************

BearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseRequestIEs} },

...

}

BearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-DRB-Status-List					CRITICALITY ignore	TYPE DRB-Status-List				PRESENCE optional  }|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory },

...

}

DRB-Status-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Status-Item

-- **************************************************************

--

#### -- BEARER CONTEXT INACTIVITY NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- Bearer Context Inactivity Notification

--

-- **************************************************************

BearerContextInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextInactivityNotificationIEs } },

...

}

BearerContextInactivityNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-ActivityInformation				CRITICALITY reject	TYPE ActivityInformation			PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DL DATA NOTIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- DL Data Notification

--

-- **************************************************************

DLDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DLDataNotificationIEs } },

...

}

DLDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PPI								CRITICALITY ignore	TYPE PPI							PRESENCE optional }|

{ ID id-PDU-Session-To-Notify-List		CRITICALITY ignore	TYPE PDU-Session-To-Notify-List		PRESENCE optional },

...

}

-- **************************************************************

-- **************************************************************

--

-- UL Data Notification

--

-- **************************************************************

ULDataNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ULDataNotificationIEs } },

...

}

ULDataNotificationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-PDU-Session-To-Notify-List		CRITICALITY reject	 TYPE PDU-Session-To-Notify-List	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- DATA USAGE REPORT

--

-- **************************************************************

-- **************************************************************

--

-- Data Usage Report

--

-- **************************************************************

DataUsageReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { DataUsageReportIEs } },

...

}

DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-Data-Usage-Report-List			CRITICALITY ignore	TYPE Data-Usage-Report-List			PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- GNB-CU-UP COUNTER CHECK

--

-- **************************************************************

-- **************************************************************

--

-- gNB-CU-UP Counter Check Request

--

-- **************************************************************

GNB-CU-UP-CounterCheckRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNB-CU-UP-CounterCheckRequestIEs } },

...

}

GNB-CU-UP-CounterCheckRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory }|

{ ID id-System-GNB-CU-UP-CounterCheckRequest	CRITICALITY reject	TYPE System-GNB-CU-UP-CounterCheckRequest	PRESENCE mandatory },

...

}

System-GNB-CU-UP-CounterCheckRequest	::=	CHOICE {

e-UTRAN-GNB-CU-UP-CounterCheckRequest		ProtocolIE-Container		{{EUTRAN-GNB-CU-UP-CounterCheckRequest}},

nG-RAN-GNB-CU-UP-CounterCheckRequest		ProtocolIE-Container		{{NG-RAN-GNB-CU-UP-CounterCheckRequest}},

choice-extension							ProtocolIE-SingleContainer	{{System-GNB-CU-UP-CounterCheckRequest-ExtIEs}}

}

System-GNB-CU-UP-CounterCheckRequest-ExtIEs E1AP-PROTOCOL-IES::= {

...

}

EUTRAN-GNB-CU-UP-CounterCheckRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Subject-To-Counter-Check-List-EUTRAN	CRITICALITY ignore	 TYPE DRBs-Subject-To-Counter-Check-List-EUTRAN	PRESENCE mandatory },

...

}

NG-RAN-GNB-CU-UP-CounterCheckRequest E1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Subject-To-Counter-Check-List-NG-RAN	CRITICALITY ignore	 TYPE DRBs-Subject-To-Counter-Check-List-NG-RAN	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- gNB-CU-UP STATUS INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- gNB-CU-UP Status Indication

--

-- **************************************************************

GNB-CU-UP-StatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNB-CU-UP-StatusIndicationIEs} },

...

}

GNB-CU-UP-StatusIndicationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID				 			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-OverloadInformation	CRITICALITY reject	TYPE GNB-CU-UP-OverloadInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- gNB-CU-CP MEASUREMENT RESULTS INFORMATION

--

-- **************************************************************

GNB-CU-CPMeasurementResultsInformation ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { GNB-CU-CPMeasurementResultsInformationIEs } },

...

}

GNB-CU-CPMeasurementResultsInformationIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-DRB-Measurement-Results-Information-List	CRITICALITY ignore		TYPE DRB-Measurement-Results-Information-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- MR-DC DATA USAGE REPORT

--

-- **************************************************************

MRDC-DataUsageReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { MRDC-DataUsageReportIEs } },

...

}

MRDC-DataUsageReportIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-CP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-gNB-CU-UP-UE-E1AP-ID					CRITICALITY reject		TYPE GNB-CU-UP-UE-E1AP-ID							PRESENCE mandatory}|

{ ID id-PDU-Session-Resource-Data-Usage-List	CRITICALITY ignore		TYPE PDU-Session-Resource-Data-Usage-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation					CRITICALITY ignore	TYPE TraceActivation					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceID							CRITICALITY ignore	TYPE TraceID							PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- CELL TRAFFIC TRACE

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{ { CellTrafficTraceIEs } },

...

}

CellTrafficTraceIEs E1AP-PROTOCOL-IES ::= {

{ID id-gNB-CU-CP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID				PRESENCE mandatory	}|

{ID id-gNB-CU-UP-UE-E1AP-ID				CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID				PRESENCE mandatory	}|

{ID id-TraceID					           CRITICALITY ignore	TYPE TraceID			            PRESENCE mandatory}|

{ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress		   PRESENCE mandatory	}|

{ID id-PrivacyIndicator					CRITICALITY ignore	TYPE PrivacyIndicator			        PRESENCE optional}|

{ID id-URIaddress	                     CRITICALITY ignore	    TYPE URIaddress		            PRESENCE optional},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs E1AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS REQUEST

--

-- **************************************************************

ResourceStatusRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ResourceStatusRequestIEs } },

...

}

ResourceStatusRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject		TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID		CRITICALITY reject		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID		CRITICALITY ignore		TYPE INTEGER (1..4095, ...)		PRESENCE optional}|

{ ID id-RegistrationRequest				CRITICALITY reject		TYPE RegistrationRequest			PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject		TYPE ReportCharacteristics		PRESENCE conditional}|

{ ID id-ReportingPeriodicity			CRITICALITY reject		TYPE ReportingPeriodicity			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS RESPONSE

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ResourceStatusResponseIEs } },

...

}

ResourceStatusResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject		TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID		CRITICALITY reject		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID		CRITICALITY ignore		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore		TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS FAILURE

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ResourceStatusFailureIEs } },

...

}

ResourceStatusFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject		TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID		CRITICALITY reject		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID		CRITICALITY ignore		TYPE INTEGER (1..4095, ...)		PRESENCE optional}|

{ ID id-Cause					CRITICALITY ignore		TYPE Cause		PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore		TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

#### -- RESOURCE STATUS UPDATE

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ResourceStatusUpdateIEs } },

...

}

ResourceStatusUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject		TYPE TransactionID	PRESENCE mandatory}|

{ ID id-gNB-CU-CP-Measurement-ID		CRITICALITY reject		TYPE INTEGER (1..4095, ...)		PRESENCE mandatory}|

{ ID id-gNB-CU-UP-Measurement-ID		CRITICALITY ignore		TYPE INTEGER (1..4095, ...)		PRESENCE optional}|

{ ID id-TNL-AvailableCapacityIndicator		CRITICALITY ignore		TYPE 							TNL-AvailableCapacityIndicator	PRESENCE optional}|

{ ID id-HW-CapacityIndicator				CRITICALITY ignore		TYPE 							HW-CapacityIndicator			PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- IAB UP TNL ADDRESS UPDATE

--

-- **************************************************************

-- **************************************************************

--

-- IAB UP TNL Address Update

--

-- **************************************************************

IAB-UPTNLAddressUpdate	::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { IAB-UPTNLAddressUpdateIEs} },

...

}

IAB-UPTNLAddressUpdateIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-DLUPTNLAddressToUpdateList			CRITICALITY ignore	TYPE DLUPTNLAddressToUpdateList			PRESENCE optional	},

...

}

DLUPTNLAddressToUpdateList       ::= SEQUENCE (SIZE(1.. maxnoofTNLAddresses))	OF DLUPTNLAddressToUpdateItem

-- **************************************************************

--

-- IAB UP TNL Address Update Acknowledge

--

-- **************************************************************

IAB-UPTNLAddressUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { IAB-UPTNLAddressUpdateAcknowledgeIEs} },

...

}

IAB-UPTNLAddressUpdateAcknowledgeIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	}|

{ ID id-ULUPTNLAddressToUpdateList			CRITICALITY ignore	TYPE ULUPTNLAddressToUpdateList			PRESENCE optional	},

...

}

ULUPTNLAddressToUpdateList       ::= SEQUENCE (SIZE(1.. maxnoofTNLAddresses))	OF ULUPTNLAddressToUpdateItem

-- **************************************************************

--

-- IAB UP TNL Address Update Failure

--

-- **************************************************************

IAB-UPTNLAddressUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {IAB-UPTNLAddressUpdateFailureIEs} },

...

}

IAB-UPTNLAddressUpdateFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- EARLY FORWARDING SN TRANSFER

--

-- **************************************************************

-- **************************************************************

--

-- Early Forwarding SN Transfer

--

-- **************************************************************

EarlyForwardingSNTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { EarlyForwardingSNTransferIEs } },

...

}

EarlyForwardingSNTransferIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID			CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-DRBs-Subject-To-Early-Forwarding-List		CRITICALITY reject	 TYPE DRBs-Subject-To-Early-Forwarding-List		PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT SETUP REQUEST

--

-- **************************************************************

BCBearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextSetupRequestIEs } },

...

}

BCBearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GlobalMBSSessionID				CRITICALITY reject	TYPE 	GlobalMBSSessionID					PRESENCE mandatory	}|

{ ID id-BCBearerContextToSetup			CRITICALITY ignore	TYPE 	BCBearerContextToSetup				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT SETUP RESPONSE

--

-- **************************************************************

BCBearerContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextSetupResponseIEs } },

...

}

BCBearerContextSetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToSetupResponse	CRITICALITY ignore	TYPE 	BCBearerContextToSetupResponse		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT SETUP FAILURE

--

-- **************************************************************

BCBearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextSetupFailureIEs } },

...

}

BCBearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

BCBearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationRequestIEs } },

...

}

BCBearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToModify			CRITICALITY ignore	TYPE 	BCBearerContextToModify				PRESENCE mandatory	}|

{ ID id-GNB-DU-ID						CRITICALITY ignore	TYPE 	GNB-DU-ID							PRESENCE optional },

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

BCBearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationResponseIEs } },

...

}

BCBearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToModifyResponse	CRITICALITY ignore	TYPE 	BCBearerContextToModifyResponse	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

BCBearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationFailureIEs } },

...

}

BCBearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

BCBearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationRequiredIEs } },

...

}

BCBearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToModifyRequired	CRITICALITY ignore	TYPE 	BCBearerContextToModifyRequired	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

BCBearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationConfirmIEs } },

...

}

BCBearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-BCBearerContextToModifyConfirm	CRITICALITY ignore	TYPE 	BCBearerContextToModifyConfirm		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT RELEASE COMMAND

--

-- **************************************************************

BCBearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextReleaseCommandIEs } },

...

}

BCBearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT RELEASE COMPLETE

--

-- **************************************************************

BCBearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextReleaseCompleteIEs } },

...

}

BCBearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

BCBearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextReleaseRequestIEs } },

...

}

BCBearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT SETUP REQUEST

--

-- **************************************************************

MCBearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextSetupRequestIEs } },

...

}

MCBearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GlobalMBSSessionID				CRITICALITY reject	TYPE 	GlobalMBSSessionID					PRESENCE mandatory	}|

{ ID id-MCBearerContextToSetup			CRITICALITY ignore	TYPE 	MCBearerContextToSetup				PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT SETUP RESPONSE

--

-- **************************************************************

MCBearerContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextSetupResponseIEs } },

...

}

MCBearerContextSetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToSetupResponse	CRITICALITY ignore	TYPE 	MCBearerContextToSetupResponse		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT SETUP FAILURE

--

-- **************************************************************

MCBearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextSetupFailureIEs } },

...

}

MCBearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MCBearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationRequestIEs } },

...

}

MCBearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToModify			CRITICALITY ignore	TYPE 	MCBearerContextToModify				PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE 	MBSMulticastF1UContextDescriptor PRESENCE mandatory	}|

{ ID id-GNB-DU-ID						CRITICALITY ignore	TYPE 	GNB-DU-ID							PRESENCE optional },

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MCBearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationResponseIEs } },

...

}

MCBearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToModifyResponse	CRITICALITY ignore	TYPE 	MCBearerContextToModifyResponse	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

MCBearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationFailureIEs } },

...

}

MCBearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

MCBearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationRequiredIEs } },

...

}

MCBearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToModifyRequired	CRITICALITY ignore	TYPE 	MCBearerContextToModifyRequired	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

MCBearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationConfirmIEs } },

...

}

MCBearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-MCBearerContextToModifyConfirm	CRITICALITY ignore	TYPE 	MCBearerContextToModifyConfirm		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT RELEASE COMMAND

--

-- **************************************************************

MCBearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextReleaseCommandIEs } },

...

}

MCBearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT RELEASE COMPLETE

--

-- **************************************************************

MCBearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextReleaseCompleteIEs } },

...

}

MCBearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

MCBearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextReleaseRequestIEs } },

...

}

MCBearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AdditionalHandoverInfo ::= ENUMERATED	{

discard-pdpc-SN,

...

}

AdditionalPDCPduplicationInformation	::=	ENUMERATED	{

three,

four,

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AveragingWindow  ::= INTEGER (0..4095, ...)

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParameterIndex		INTEGER(1..8,...),

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailableSharedMBSSessionNGUTermination-Information ::= SEQUENCE {

availableMBSSessionType-Info		AvailableMBSSessionTypeInfo,

consent							ConsentApplyAvailSharedUPMBSQoSFlowMapping		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AvailableSharedMBSSessionNGUTermination-Information-ExtIEs} }	OPTIONAL,

...

}

AvailableSharedMBSSessionNGUTermination-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailableMBSSessionTypeInfo ::= CHOICE {

locationindpendent				AvailableSharedNGUTermination-Information,

locationdependent				LocationDependentAvailableMBSNGUTerminationInformationAt5GC,

choice-extension	ProtocolIE-SingleContainer	{{AvailableMBSSessionTypeInfo-ExtIEs}}

}

AvailableMBSSessionTypeInfo-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

AvailableSharedNGUTermination-Information ::= SEQUENCE (SIZE(1..maxnoofSharedNG-UTerminations)) OF AvailableSharedNGUTermination-Information-Item

AvailableSharedNGUTermination-Information-Item ::= SEQUENCE {

up-TNL-Info				UP-TNL-Information,

iE-Extensions		ProtocolExtensionContainer { {AvailableSharedNGUTermination-Information-Item-ExtIEs} }	OPTIONAL,

...

}

AvailableSharedNGUTermination-Information-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentAvailableMBSNGUTerminationInformationAt5GC ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF

LocationDependentAvailableMBSNGUTerminationInformationAt5GC-Item

LocationDependentAvailableMBSNGUTerminationInformationAt5GC-Item 	::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

availableSharedNGUTermination-Information		AvailableSharedNGUTermination-Information,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentAvailableMBSNGUTerminationInformationAt5GC-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentAvailableMBSNGUTerminationInformationAt5GC-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- B

##### -- BCBearerContextToSetup

BCBearerContextToSetup ::= SEQUENCE {

snssai								SNSSAI,

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC,

bcMRBToSetupList						BCMRBSetupConfiguration,

availablesharedNG-UInfo				AvailableSharedNGUTermination-Information		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextNGU-TNLInfoat5GC::= CHOICE {

locationindpendent				MBSNGUInformationAt5GC,

locationdependent				LocationDependentMBSNGUInformationAt5GC,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextNGU-TNLInfoat5GC-ExtIEs}}

}

BCBearerContextNGU-TNLInfoat5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupConfiguration-Item

BCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

sdap-config						SDAP-Configuration,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- BCBearerContextToSetupResponse

BCBearerContextToSetupResponse ::= SEQUENCE {

bcBearerContextNGU-TNLInfoatNGRAN			BCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

-- This IE shall be present if the MBS NG-U Information at NG-RAN IE within the BC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information

bcMRBSetupResponseList					BCMRBSetupResponseList,

bcMRBFailedList							BCMRBFailedList							OPTIONAL,

offerfedSharedNGUInfo					OfferedSharedMBSSessionNGUTerminationInformation		OPTIONAL,

availableBCMRBConfig						BCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextNGU-TNLInfoatNGRAN::= CHOICE {

locationindpendent				MBSNGUInformationAtNGRAN,

locationdependent				LocationDependentMBSNGUInformationAtNGRAN,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextNGU-TNLInfoatNGRAN-ExtIEs}}

}

BCBearerContextNGU-TNLInfoatNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBSetupResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupResponseList-Item

BCMRBSetupResponseList-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

qosflow-setup						QoS-Flow-List,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

bcBearerContextF1U-TNLInfoatCU 		BCBearerContextF1U-TNLInfoatCU,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextF1U-TNLInfoatCU ::= CHOICE {

locationindpendent				MBSF1UInformationAtCU,

locationdependent				LocationDependentMBSF1UInformationAtCU,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfoatCU-ExtIEs}}

}

BCBearerContextF1U-TNLInfoatCU-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBFailedList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBFailedList-Item

BCMRBFailedList-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {BCMRBFailedList-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBFailedList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

OfferedSharedMBSSessionNGUTerminationInformation ::= CHOICE {

locationindpendent				OfferedSharedNGUTermination-Information,

locationdependent				LocationDependentOfferedSharedNGUTermination-Information,

choice-extension	ProtocolIE-SingleContainer	{{OfferedSharedMBSSessionNGUTerminationInformation-ExtIEs}}

}

OfferedSharedMBSSessionNGUTerminationInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

OfferedSharedNGUTermination-Information ::= SEQUENCE {

up-TNL-Info				UP-TNL-Information,

iE-Extensions		ProtocolExtensionContainer { {OfferedSharedNGUTermination-Information-ExtIEs} }	OPTIONAL,

...

}

OfferedSharedNGUTermination-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentOfferedSharedNGUTermination-Information ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF

LocationDependentOfferedSharedNGUTermination-Information-Item

LocationDependentOfferedSharedNGUTermination-Information-Item 	::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

offeredSharedNGUTermination-Information		OfferedSharedNGUTermination-Information,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentOfferedSharedNGUTermination-Information-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentOfferedSharedNGUTermination-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- BCBearerContextToModify

BCBearerContextToModify ::= SEQUENCE {

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC	OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration			OPTIONAL,

bcMRBToModifyList					BCMRBModifyConfiguration			OPTIONAL,

bcMRBToRemoveList					BCMRBRemoveConfiguration			OPTIONAL,

availablesharedNG-UInfo				AvailableSharedNGUTermination-Information		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCMRBModifyConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBModifyConfiguration-Item

BCMRBModifyConfiguration-Item ::= SEQUENCE {

mrb-ID							DRB-ID,

bcBearerContextF1U-TNLInfoatDU 	BCBearerContextF1U-TNLInfoatDU,

sdap-config						SDAP-Configuration,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBModifyConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBModifyConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextF1U-TNLInfoatDU ::= CHOICE {

locationindpendent				MBSF1UInformationAtDU,

locationdependent				LocationDependentMBSF1UInformationAtDU,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfoatDU-ExtIEs}}

}

BCBearerContextF1U-TNLInfoatDU-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

BCMRBRemoveConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF DRB-ID

##### -- BCBearerContextToModifyResponse

BCBearerContextToModifyResponse ::= SEQUENCE {

bcBearerContextNGU-TNLInfoatNGRAN			BCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

bcMRBSetupModifyResponseList				BCMRBSetupModifyResponseList,

bcMRBFailedList							BCMRBFailedList							OPTIONAL,

offerfedSharedNGUInfo					OfferedSharedMBSSessionNGUTerminationInformation		OPTIONAL,

availableBCMRBConfig						BCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyResponse-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BCMRBSetupModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF BCMRBSetupModifyResponseList-Item

BCMRBSetupModifyResponseList-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

qosflow-setup						QoS-Flow-List			OPTIONAL,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

bcBearerContextF1U-TNLInfoatCU 		BCBearerContextF1U-TNLInfoatCU		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCMRBSetupModifyResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

BCMRBSetupModifyResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- BCBearerContextToModifyRequired

BCBearerContextToModifyRequired ::= SEQUENCE {

bcMRBToRemoveList	BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- BCBearerContextToModifyConfirm

BCBearerContextToModifyConfirm ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModifyConfirm-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModifyConfirm-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

BearerContextStatusChange	::= 	ENUMERATED {

suspend,

resume,

...

}

BitRate ::= INTEGER (0..4000000000000,...)

#### -- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer	{{Cause-ExtIEs}}

}

Cause-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

unknown-or-already-allocated-gnb-cu-cp-ue-e1ap-id,

unknown-or-already-allocated-gnb-cu-up-ue-e1ap-id,

unknown-or-inconsistent-pair-of-ue-e1ap-id,

interaction-with-other-procedure,

pPDCP-Count-wrap-around,

not-supported-QCI-value,

not-supported-5QI-value,

encryption-algorithms-not-supported,

integrity-protection-algorithms-not-supported,

uP-integrity-protection-not-possible,

uP-confidentiality-protection-not-possible,

multiple-PDU-Session-ID-Instances,

unknown-PDU-Session-ID,

multiple-QoS-Flow-ID-Instances,

unknown-QoS-Flow-ID,

multiple-DRB-ID-Instances,

unknown-DRB-ID,

invalid-QoS-combination,

procedure-cancelled,

normal-release,

no-radio-resources-available,

action-desirable-for-radio-reasons,

resources-not-available-for-the-slice,

pDCP-configuration-not-supported,

...,

ue-dl-max-IP-data-rate-reason,

uP-integrity-protection-failure,

release-due-to-pre-emption,

rsn-not-available-for-the-up,

nPN-not-supported,

report-characteristic-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...,

unknown-TNL-address-for-IAB

}

Cell-Group-Information	::= SEQUENCE (SIZE(1.. maxnoofCellGroups)) OF Cell-Group-Information-Item

Cell-Group-Information-Item	::=	SEQUENCE {

cell-Group-ID							Cell-Group-ID,

uL-Configuration						UL-Configuration		OPTIONAL,

dL-TX-Stop								DL-TX-Stop				OPTIONAL,

rAT-Type								RAT-Type				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Cell-Group-Information-Item-ExtIEs } }	OPTIONAL,

...

}

Cell-Group-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Number-of-tunnels	CRITICALITY ignore	EXTENSION Number-of-tunnels 	PRESENCE optional},

...

}

Cell-Group-ID	::=		INTEGER (0..3, ...)

CHOInitiation	::=		ENUMERATED {true, ...}

Number-of-tunnels  ::=		INTEGER (1..4, ...)

CipheringAlgorithm	::=	ENUMERATED {

nEA0,

c-128-NEA1,

c-128-NEA2,

c-128-NEA3,

...

}

CNSupport ::= ENUMERATED {

c-epc,

c-5gc,

both,

...

}

CommonNetworkInstance ::= OCTET STRING

ConfidentialityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

ConfidentialityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

ConsentApplyAvailSharedUPMBSQoSFlowMapping ::= ENUMERATED {

allowed,

...

}

CP-TNL-Information		::= 	CHOICE {

endpoint-IP-Address		TransportLayerAddress,

choice-extension		ProtocolIE-SingleContainer	{{CP-TNL-Information-ExtIEs}}

}

CP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

{ ID id-endpoint-IP-Address-and-Port	CRITICALITY reject	TYPE Endpoint-IP-address-and-port 	PRESENCE mandatory},

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

transactionID					TransactionID					OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxnoofErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- D

DAPSRequestInfo ::= SEQUENCE {

dapsIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DataForwardingtoNG-RANQoSFlowInformationList	CRITICALITY ignore	EXTENSION DataForwardingtoNG-RANQoSFlowInformationList	PRESENCE optional},

...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

DataForwardingtoE-UTRANInformationList ::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunneltoE-UTRAN)) OF DataForwardingtoE-UTRANInformationListItem

DataForwardingtoE-UTRANInformationListItem ::= SEQUENCE {

data-forwarding-tunnel-information							UP-TNL-Information,

qoS-Flows-to-be-forwarded-List						QoS-Flows-to-be-forwarded-List,

iE-Extensions		ProtocolExtensionContainer { { DataForwardingtoE-UTRANInformationListItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingtoE-UTRANInformationListItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-QoS-Flow-List	::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF Data-Usage-per-QoS-Flow-Item

Data-Usage-per-QoS-Flow-Item ::= SEQUENCE {

qoS-Flow-Identifier		QoS-Flow-Identifier,

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

qoS-Flow-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-QoS-Flow-Item-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-QoS-Flow-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-Report-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF Data-Usage-Report-Item

Data-Usage-Report-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

rAT-Type					RAT-Type,

dRB-Usage-Report-List		DRB-Usage-Report-List,

iE-Extensions	ProtocolExtensionContainer { { Data-Usage-Report-ItemExtIEs } }	OPTIONAL,

...

}

Data-Usage-Report-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DefaultDRB	::=	ENUMERATED	{

true,

false,

...

}

DirectForwardingPathAvailability ::= ENUMERATED {

inter-system-direct-path-available,

...,

intra-system-direct-path-available

}

DiscardTimer	::=	ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity}

DLDiscarding ::= SEQUENCE {

dLDiscardingCountVal			PDCP-Count,

iE-Extensions					ProtocolExtensionContainer { { DLDiscarding-ExtIEs } } 		OPTIONAL

}

DLDiscarding-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DLUPTNLAddressToUpdateItem	::= SEQUENCE {

oldTNLAdress						TransportLayerAddress,

newTNLAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { DLUPTNLAddressToUpdateItemExtIEs } }	OPTIONAL,

...

}

DLUPTNLAddressToUpdateItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-TX-Stop	::=	ENUMERATED	{

stop,

resume,

...

}

DRB-Activity	::= ENUMERATED {

active,

not-active,

...

}

DRB-Activity-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DRB-Activity-Item

DRB-Activity-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

dRB-Activity				DRB-Activity,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-EUTRAN

DRB-Confirm-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Confirm-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Confirm-Modified-Item-NG-RAN

DRB-Confirm-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cell-Group-Information					Cell-Group-Information	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Confirm-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Confirm-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-EUTRAN

DRB-Failed-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-EUTRAN

DRB-Failed-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Item-NG-RAN

DRB-Failed-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-Mod-Item-NG-RAN

DRB-Failed-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-EUTRAN

DRB-Failed-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Failed-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Failed-To-Modify-Item-NG-RAN

DRB-Failed-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Failed-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-ID	::=	INTEGER (1..32, ...)

DRB-Measurement-Results-Information-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Measurement-Results-Information-Item

DRB-Measurement-Results-Information-Item	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-D1-Result								INTEGER (0..10000, ...)							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Measurement-Results-Information-Item-ExtIEs } }	OPTIONAL,

...

}

DRB-Measurement-Results-Information-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-EUTRAN

DRB-Modified-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters							OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Modified-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Modified-Item-NG-RAN

DRB-Modified-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

uL-UP-Transport-Parameters					UP-Parameters							OPTIONAL,

pDCP-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

flow-Setup-List								QoS-Flow-List							OPTIONAL,

flow-Failed-List							QoS-Flow-Failed-List					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Modified-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Modified-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-EarlyForwardingCOUNTInfo				CRITICALITY reject	EXTENSION EarlyForwardingCOUNTInfo		PRESENCE optional}|

{ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY ignore	EXTENSION QoS-Flow-List						PRESENCE optional},

...

}

DRB-Removed-Item	::=	SEQUENCE {

dRB-ID									DRB-ID,

dRB-Released-In-Session					ENUMERATED {released-in-session, not-released-in-session, ...}		OPTIONAL,

dRB-Accumulated-Session-Time			OCTET STRING (SIZE(5))													OPTIONAL,

qoS-Flow-Removed-List					SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Removed-Item		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Removed-Item-ExtIEs } }			OPTIONAL,

...

}

DRB-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-EUTRAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-EUTRAN

DRB-Required-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information							OPTIONAL,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Modify-List-NG-RAN ::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Modify-Item-NG-RAN

DRB-Required-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

gNB-CU-UP-CellGroupRelatedConfiguration	GNB-CU-UP-CellGroupRelatedConfiguration		OPTIONAL,

flow-To-Remove							QoS-Flow-List								OPTIONAL,

cause									Cause		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-EUTRAN

DRB-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

s1-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-EUTRAN

DRB-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

s1-DL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters				UP-Parameters,

iE-Extensions							ProtocolExtensionContainer { { DRB-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Item-NG-RAN

DRB-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Setup-Mod-Item-NG-RAN

DRB-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

dRB-data-Forwarding-Information-Response	Data-Forwarding-Information		OPTIONAL,

uL-UP-Transport-Parameters					UP-Parameters,

flow-Setup-List								QoS-Flow-List,

flow-Failed-List							QoS-Flow-Failed-List	OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Status-Item	::= SEQUENCE {

dRB-ID						DRB-ID,

pDCP-DL-Count				PDCP-Count		OPTIONAL,

pDCP-UL-Count				PDCP-Count		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Status-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Status-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-EUTRAN

DRBs-Subject-To-Counter-Check-Item-EUTRAN	::=	SEQUENCE {

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions		ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Counter-Check-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Counter-Check-Item-NG-RAN

DRBs-Subject-To-Counter-Check-Item-NG-RAN	::=	SEQUENCE {

pDU-Session-ID				PDU-Session-ID,

dRB-ID						DRB-ID,

pDCP-UL-Count				PDCP-Count,

pDCP-DL-Count				PDCP-Count,

iE-Extensions			ProtocolExtensionContainer { { DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Counter-Check-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Subject-To-Early-Forwarding-List	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRBs-Subject-To-Early-Forwarding-Item

DRBs-Subject-To-Early-Forwarding-Item	::=	SEQUENCE {

dRB-ID										DRB-ID,

dLCountValue								PDCP-Count,

iE-Extensions							ProtocolExtensionContainer { { DRBs-Subject-To-Early-Forwarding-Item-ExtIEs } }	OPTIONAL,

...

}

DRBs-Subject-To-Early-Forwarding-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-EUTRAN

DRB-To-Modify-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration						OPTIONAL,

eUTRAN-QoS								EUTRAN-QoS								OPTIONAL,

s1-UL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request					PDCP-SN-Status-Request						OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

cell-Group-To-Add						Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify					Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove					Cell-Group-Information					OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Modify-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Modify-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Modify-Item-NG-RAN

DRB-To-Modify-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration						OPTIONAL,

pDCP-Configuration							PDCP-Configuration						OPTIONAL,

dRB-Data-Forwarding-Information		Data-Forwarding-Information		OPTIONAL,

pDCP-SN-Status-Request							PDCP-SN-Status-Request						OPTIONAL,

pdcp-SN-Status-Information					PDCP-SN-Status-Information				OPTIONAL,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

cell-Group-To-Add							Cell-Group-Information					OPTIONAL,

cell-Group-To-Modify						Cell-Group-Information					OPTIONAL,

cell-Group-To-Remove						Cell-Group-Information					OPTIONAL,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List				OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Modify-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Modify-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-OldQoSFlowMap-ULendmarkerexpected	CRITICALITY reject	EXTENSION QoS-Flow-List	PRESENCE optional}|

{ID id-DRB-QoS						CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters			PRESENCE optional}|

{ID id-EarlyForwardingCOUNTReq		CRITICALITY reject	EXTENSION EarlyForwardingCOUNTReq			PRESENCE optional}|

{ID id-EarlyForwardingCOUNTInfo		CRITICALITY reject	EXTENSION EarlyForwardingCOUNTInfo			PRESENCE optional}|

{ID id-DAPSRequestInfo				CRITICALITY ignore	EXTENSION DAPSRequestInfo					PRESENCE optional}|

{ID id-EarlyDataForwardingIndicator	CRITICALITY ignore	EXTENSION EarlyDataForwardingIndicator	PRESENCE optional},

...

}

DRB-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-EUTRAN

DRB-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-EUTRAN

DRB-Required-To-Remove-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Remove-Item-NG-RAN

DRB-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Required-To-Remove-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Required-To-Remove-Item-NG-RAN

DRB-Required-To-Remove-Item-NG-RAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { DRB-Required-To-Remove-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-Required-To-Remove-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-EUTRAN

DRB-To-Setup-Item-EUTRAN	::=	SEQUENCE {

dRB-ID									DRB-ID,

pDCP-Configuration						PDCP-Configuration,

eUTRAN-QoS								EUTRAN-QoS,

s1-UL-UP-TNL-Information				UP-TNL-Information,

data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information					Cell-Group-Information,

dL-UP-Parameters						UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer					Inactivity-Timer						OPTIONAL,

existing-Allocated-S1-DL-UP-TNL-Info	UP-TNL-Information						OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-Mod-List-EUTRAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-EUTRAN

DRB-To-Setup-Mod-Item-EUTRAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

pDCP-Configuration							PDCP-Configuration,

eUTRAN-QoS									EUTRAN-QoS,

s1-UL-UP-TNL-Information					UP-TNL-Information,

data-Forwarding-Information-Request			Data-Forwarding-Information-Request		OPTIONAL,

cell-Group-Information						Cell-Group-Information,

dL-UP-Parameters							UP-Parameters							OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-EUTRAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS						CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters		PRESENCE optional}|

{ID id-DAPSRequestInfo				CRITICALITY ignore	EXTENSION DAPSRequestInfo					PRESENCE optional}|

{ID id-ignoreMappingRuleIndication	CRITICALITY reject	EXTENSION IgnoreMappingRuleIndication	PRESENCE optional}|

{ID id-QoSFlowsDRBRemapping			CRITICALITY reject	EXTENSION QoS-Flows-DRB-Remapping			PRESENCE optional},

...

}

DRB-To-Setup-Mod-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Mod-Item-NG-RAN

DRB-To-Setup-Mod-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

flow-Mapping-Information					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer					OPTIONAL,

pDCP-SN-Status-Information				PDCP-SN-Status-Information					OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Mod-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS						CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters		PRESENCE optional}|

{ID id-ignoreMappingRuleIndication	CRITICALITY reject	EXTENSION IgnoreMappingRuleIndication	PRESENCE optional}|

{ID id-DAPSRequestInfo				CRITICALITY ignore	EXTENSION DAPSRequestInfo				PRESENCE optional},

...

}

DRB-Usage-Report-List ::= SEQUENCE (SIZE(1..maxnooftimeperiods)) OF DRB-Usage-Report-Item

DRB-Usage-Report-Item	::= SEQUENCE {

startTimeStamp					OCTET STRING (SIZE(4)),

endTimeStamp					OCTET STRING (SIZE(4)),

usageCountUL					INTEGER (0..18446744073709551615),

usageCountDL					INTEGER (0..18446744073709551615),

iE-Extensions					ProtocolExtensionContainer { { DRB-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

DRB-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Duplication-Activation	::=		ENUMERATED {

active,

inactive,

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel					QoSPriorityLevel,

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

fiveQI								INTEGER (0..255, ...)								OPTIONAL,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

averagingWindow 					AveragingWindow										OPTIONAL,

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	},

...

}

DataDiscardRequired	::= 	ENUMERATED {

required,

...

}

#### -- E

EarlyDataForwardingIndicator ::= ENUMERATED {stop, ...}

EarlyForwardingCOUNTInfo ::= CHOICE {

firstDLCount					FirstDLCount,

dLDiscardingCount				DLDiscarding,

choice-Extension				ProtocolIE-SingleContainer { { EarlyForwardingCOUNTInfo-ExtIEs} }

}

EarlyForwardingCOUNTInfo-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EarlyForwardingCOUNTReq ::= ENUMERATED { first-dl-count, dl-discarding, ...}

EHC-Common-Parameters ::= SEQUENCE {

ehc-CID-Length						ENUMERATED { bits7, bits15, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Common-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Common-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Downlink-Parameters ::= SEQUENCE {

drb-ContinueEHC-DL					ENUMERATED { true, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Downlink-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Downlink-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-MaxCIDEHCDL			CRITICALITY ignore	EXTENSION	MaxCIDEHCDL		PRESENCE optional	},

...

}

EHC-Uplink-Parameters ::= SEQUENCE {

drb-ContinueEHC-UL					ENUMERATED { true, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Uplink-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Uplink-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Parameters ::= SEQUENCE {

ehc-Common							EHC-Common-Parameters,

ehc-Downlink						EHC-Downlink-Parameters					OPTIONAL,

ehc-Uplink							EHC-Uplink-Parameters					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { EHC-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EncryptionKey	::=	OCTET STRING

Endpoint-IP-address-and-port::= SEQUENCE {

endpoint-IP-Address			TransportLayerAddress,

portNumber					PortNumber,

iE-Extensions							ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {EUTRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

ExtendedPacketDelayBudget ::= INTEGER (1..65535, ...)

EUTRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofEUTRANQOSParameters)) OF EUTRAN-QoS-Support-Item

EUTRAN-QoS-Support-Item ::= SEQUENCE {

eUTRAN-QoS	EUTRAN-QoS,

iE-Extensions			ProtocolExtensionContainer { { EUTRAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

EUTRAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRAN-QoS	::= SEQUENCE {

qCI										QCI,

eUTRANallocationAndRetentionPriority	EUTRANAllocationAndRetentionPriority,

gbrQosInformation						GBR-QosInformation										OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { EUTRAN-QoS-ExtIEs } }	OPTIONAL,

...

}

EUTRAN-QoS-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedSliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofExtSliceItems)) OF Slice-Support-Item

#### -- F

FirstDLCount ::= SEQUENCE {

firstDLCountVal					PDCP-Count,

iE-Extensions					ProtocolExtensionContainer { { FirstDLCount-ExtIEs } } 		OPTIONAL

}

FirstDLCount-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- G

GlobalMBSSessionID ::= SEQUENCE {

tmgi	OCTET STRING (SIZE(6)),

nid		NID						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { GlobalMBSSessionID-ExtIEs } } OPTIONAL,

...

}

GlobalMBSSessionID-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-Name 				::= PrintableString(SIZE(1..150,...))

Extended-GNB-CU-CP-Name	 ::= SEQUENCE {

gNB-CU-CP-NameVisibleString			GNB-CU-CP-NameVisibleString				OPTIONAL,

gNB-CU-CP-NameUTF8String			GNB-CU-CP-NameUTF8String				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { Extended-GNB-CU-CP-Name-ExtIEs } } OPTIONAL,

...

}

Extended-GNB-CU-CP-Name-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-MBS-E1AP-ID		::= INTEGER (0..65535)

GNB-CU-CP-NameVisibleString ::= VisibleString(SIZE(1..150,...))

GNB-CU-CP-NameUTF8String ::= UTF8String(SIZE(1..150,...))

GNB-CU-CP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-UP-Capacity				::= INTEGER (0..255)

GNB-CU-UP-CellGroupRelatedConfiguration	 ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF GNB-CU-UP-CellGroupRelatedConfiguration-Item

GNB-CU-UP-CellGroupRelatedConfiguration-Item ::= SEQUENCE {

cell-Group-ID				Cell-Group-ID,

uP-TNL-Information			UP-TNL-Information,

uL-Configuration			UL-Configuration		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs } }	OPTIONAL

}

GNB-CU-UP-CellGroupRelatedConfiguration-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-ID				::= INTEGER (0..68719476735)

GNB-CU-UP-MBS-E1AP-ID		::= INTEGER (0..65535)

GNB-CU-UP-Name 				::= PrintableString(SIZE(1..150,...))

Extended-GNB-CU-UP-Name	 ::= SEQUENCE {

gNB-CU-UP-NameVisibleString			GNB-CU-UP-NameVisibleString				OPTIONAL,

gNB-CU-UP-NameUTF8String			GNB-CU-UP-NameUTF8String				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { Extended-GNB-CU-UP-Name-ExtIEs } } OPTIONAL,

...

}

Extended-GNB-CU-UP-Name-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-NameVisibleString ::= VisibleString(SIZE(1..150,...))

GNB-CU-UP-NameUTF8String ::= UTF8String(SIZE(1..150,...))

GNB-CU-UP-UE-E1AP-ID		::= INTEGER (0..4294967295)

GNB-CU-CP-TNLA-Setup-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Setup-Item-ExtIEs} } OPTIONAL,

...

}

GNB-CU-CP-TNLA-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-Failed-To-Setup-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Add-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Add-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-CP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressgNBCUUP	CRITICALITY reject	EXTENSION CP-TNL-Information	PRESENCE optional},

...

}

GNB-CU-CP-TNLA-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TNL-Information,

tNLAssociationUsage						TNLAssociationUsage 	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-CP-TNLA-To-Update-Item-ExtIEs} } OPTIONAL

}

GNB-CU-CP-TNLA-To-Update-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-TNLA-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress			CP-TNL-Information,

tNLAssociationTransportLayerAddressgNBCUCP	CP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-UP-TNLA-To-Remove-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional},

...

}

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddresses				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }         OPTIONAL,

...

}

GTPTLA-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GTPTunnel				::= SEQUENCE {

transportLayerAddress				TransportLayerAddress,

gTP-TEID							GTP-TEID,

iE-Extensions						ProtocolExtensionContainer { { GTPTunnel-ExtIEs} } OPTIONAL,

...

}

GTPTunnel-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UP-OverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GNB-DU-ID	::= INTEGER (0..68719476735)

#### -- H

HFN		::=		INTEGER	(0..4294967295)

HW-CapacityIndicator ::= SEQUENCE {

offeredThroughput				INTEGER (1..16777216, ...),

availableThroughput				INTEGER (0..100, ...),

iE-Extensions		ProtocolExtensionContainer { { HW-CapacityIndicator-ExtIEs } },

...

}

HW-CapacityIndicator-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- I

IgnoreMappingRuleIndication	::=	ENUMERATED	{

true,

...

}

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionAlgorithm	::=	ENUMERATED {

nIA0,

i-128-NIA1,

i-128-NIA2,

i-128-NIA3,

...

}

IntegrityProtectionKey	::= OCTET STRING

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

Inactivity-Timer	::=	INTEGER (1..7200, ...)

InterfacesToTrace ::= BIT STRING (SIZE(8))

ImmediateMDT ::= SEQUENCE {

measurementsToActivate		MeasurementsToActivate,

measurementFour 	M4Configuration		OPTIONAL,

measurementSix 				M6Configuration		OPTIONAL,

measurementSeven 			M7Configuration	    OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ImmediateMDT-ExtIEs} } OPTIONAL,

...

}

ImmediateMDT-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- J

#### -- K

#### -- L

Links-to-log ::= ENUMERATED {

uplink,

downlink,

both-uplink-and-downlink,

...

}

#### -- M

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaximumIPdatarate ::= SEQUENCE {

maxIPrate			MaxIPrate,

iE-Extensions		ProtocolExtensionContainer { {MaximumIPdatarate-ExtIEs} }	OPTIONAL,

...

}

MaximumIPdatarate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MaxIPrate ::= ENUMERATED {

bitrate64kbs,

max-UErate,

...

}

MaxPacketLossRate ::= INTEGER (0..1000, ...)

MaxCIDEHCDL ::= INTEGER (1..32767, ...)

MBSAreaSessionID ::= INTEGER (0..255)  -- FFS

MBSNGUInformationAt5GC ::= CHOICE {

multicast 				MBSNGUInformationAt5GC-Multicast,

unicast					UP-TNL-Information,

choice-extension	ProtocolIE-SingleContainer	{{MBSNGUInformationAt5GC-ExtIEs}}

}

MBSNGUInformationAt5GC-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MBSNGUInformationAt5GC-Multicast ::= SEQUENCE {

ipmcAddress		TransportLayerAddress,

ipsourceAddress	TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {MBSNGUInformationAt5GC-Multicast-ExtIEs} }	OPTIONAL,

...

}

MBSNGUInformationAt5GC-Multicast-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentMBSNGUInformationAt5GC ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSNGUInformationAt5GC-Item

LocationDependentMBSNGUInformationAt5GC-Item ::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

mbsNGUInformationAt5GC						MBSNGUInformationAt5GC,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentMBSNGUInformationAt5GC-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSNGUInformationAt5GC-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSNGUInformationAtNGRAN ::= CHOICE {

unicast					UP-TNL-Information,

choice-extension	ProtocolIE-SingleContainer	{{MBSNGUInformationAtNGRAN-ExtIEs}}

}

MBSNGUInformationAtNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

LocationDependentMBSNGUInformationAtNGRAN ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSNGUInformationAtNGRAN-Item

LocationDependentMBSNGUInformationAtNGRAN-Item ::= SEQUENCE {

mbsAreaSession-ID							MBSAreaSessionID,

mbsNGUInformationAt5GC						MBSNGUInformationAtNGRAN,

iE-Extensions							ProtocolExtensionContainer	{ { LocationDependentMBSNGUInformationAtNGRAN-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSNGUInformationAtNGRAN-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformationAtCU ::= SEQUENCE {

mbs-f1u-info-at-CU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformationAtCU-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformationAtCU-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentMBSF1UInformationAtCU ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformationAtCU-Item

LocationDependentMBSF1UInformationAtCU-Item ::= SEQUENCE {

mbsAreaSession-ID				MBSAreaSessionID,

mbs-f1u-info-at-CU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformationAtCU-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformationAtCU-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformationAtDU ::= SEQUENCE {

mbs-f1u-info-at-DU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformationAtDU-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformationAtDU-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationDependentMBSF1UInformationAtDU ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformationAtDU-Item

LocationDependentMBSF1UInformationAtDU-Item ::= SEQUENCE {

mbsAreaSession-ID				MBSAreaSessionID,

mbs-f1u-info-at-DU				UP-TNL-Information,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformationAtDU-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformationAtDU-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToSetup

MCBearerContextToSetup ::= SEQUENCE {

snssai								SNSSAI,

mcMRBToSetupList						MCMRBSetupConfiguration,

availablesharedNG-UInfo				AvailableSharedNGUTermination-Information		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToSetup-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToSetup-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupConfiguration-Item

MCMRBSetupConfiguration-Item ::= SEQUENCE {

mrb-ID							DRB-ID,

sdap-config						SDAP-Configuration,

mbs-pdcp-config					PDCP-Configuration,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List,

qoSFlowLevelQoSParameters		QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupConfiguration-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupConfiguration-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToSetupResponse

MCBearerContextToSetupResponse ::= SEQUENCE {

mcBearerContextNGU-TNLInfoatNGRAN			MCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

-- This IE shall be present if the MBS NG-U Information at NG-RAN IE within the BC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information

mcMRBSetupResponseList					MCMRBSetupResponseList,

mcMRBFailedList							MCMRBFailedList							OPTIONAL,

offerfedSharedNGUInfo					OfferedSharedMBSSessionNGUTerminationInformation		OPTIONAL,

availableMCMRBConfig						MCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGU-TNLInfoatNGRAN::= CHOICE {

locationindpendent				MBSNGUInformationAtNGRAN,

locationdependent				LocationDependentMBSNGUInformationAtNGRAN,

choice-extension	ProtocolIE-SingleContainer	{{MCBearerContextNGU-TNLInfoatNGRAN-ExtIEs}}

}

MCBearerContextNGU-TNLInfoatNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MCMRBSetupResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupResponseList-Item

MCMRBSetupResponseList-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

qosflow-setup						QoS-Flow-List,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBFailedList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBFailedList-Item

MCMRBFailedList-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {MCMRBFailedList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBFailedList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModify

MCBearerContextToModify ::= SEQUENCE {

mcBearerContextNGUTNLInfoat5GC					MCBearerContextNGUTNLInfoat5GC				OPTIONAL,

mcBearerContextNGUTnlInfoatNGRANRequest			MCBearerContextNGUTnlInfoatNGRANRequest		OPTIONAL,

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB To Setup or Modify List IE or the MC MRB To Remove List IE or both IEs are included.

mcMRBToSetupModifyList							MCMRBSetupModifyConfiguration				OPTIONAL,

mcMRBToRemoveList								MCMRBRemoveConfiguration						OPTIONAL,

availablesharedNG-UInfo							AvailableSharedNGUTermination-Information		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGUTNLInfoat5GC ::= SEQUENCE {

mbsNGUInformationAt5GC			MBSNGUInformationAt5GC,

mbsAreaSession-ID				MBSAreaSessionID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGUTNLInfoat5GC-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGUTNLInfoat5GC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGUTnlInfoatNGRANRequest ::= SEQUENCE {

ngRANNGUTNLRequested				ENUMERATED {requested, ...},

mbsAreaSession-ID				MBSAreaSessionID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGUTnlInfoatNGRANRequest-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGUTnlInfoatNGRANRequest-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupModifyConfiguration ::= SEQUENCE {

mrb-ID							DRB-ID,

f1uTNLatDU						MCBearerContextF1UTNLInfoatDU		OPTIONAL,

sdap-config						SDAP-Configuration				OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

mrbQoS							QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyConfiguration-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyConfiguration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextF1UTNLInfoatDU ::= SEQUENCE {

mbsF1UInfoatDU		UP-TNL-Information,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextF1UTNLInfoatDU-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextF1UTNLInfoatDU-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastF1UContextDescriptor ::= CHOICE {

du					NULL,

mbsAreaSession		MBSAreaSessionID,

cell				MCDUCellReference,

ptp-retransmission	MBSPTPUEReference,

ptp-only-MRB			MBSPTPUEReference,

choice-extension		ProtocolIE-SingleContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}}

}

MBSMulticastF1UContextDescriptor-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MCDUCellReference ::= SEQUENCE {

du-CellIndex			INTEGER (1..512),

nr-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {MCDUCellReference-ExtIEs} }	OPTIONAL,

...

}

MCDUCellReference-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSPTPUEReference::= SEQUENCE {

c-RNTI				INTEGER (0..65535, ...),

du-CellIndex			INTEGER (1..512),

iE-Extensions		ProtocolExtensionContainer { {MBSPTPUEReference-ExtIEs} }	OPTIONAL,

...

}

MBSPTPUEReference-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBRemoveConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF DRB-ID

##### -- MCBearerContextToModifyResponse

MCBearerContextToModifyResponse ::= SEQUENCE {

mcBearerContextNGU-TNLInfoatNGRANModifyResponse	MCBearerContextNGU-TNLInfoatNGRANModifyResponse		OPTIONAL,

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB Setup or Modify Response List IE or the MC MRB Failed List IE or both IEs are included.

mcMRBModifySetupResponseList						MCMRBSetupModifyResponseList							OPTIONAL,

mcMRBFailedList									MCMRBFailedList										OPTIONAL,

offeredSharedNGUInfo								OfferedSharedMBSSessionNGUTerminationInformation		OPTIONAL,

availableMCMRBConfig								MCMRBSetupConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGU-TNLInfoatNGRANModifyResponse ::= SEQUENCE {

mbs-NGU-InfoatNGRAN	MBSNGUInformationAtNGRAN,

mbsAreaSession		MBSAreaSessionID					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGU-TNLInfoatNGRANModifyResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGU-TNLInfoatNGRANModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupModifyResponseList-Item

MCMRBSetupModifyResponseList-Item ::= SEQUENCE {

mrb-ID								DRB-ID,

qosflow-setup						QoS-Flow-List			OPTIONAL,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

mcBearerContextF1UTNLInfoatCU		UP-TNL-Information		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModifyRequired

MCBearerContextToModifyRequired ::= SEQUENCE {

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

-- This IE shall be present if either the MC MRB To Remove List Required IE is included.

mcMRBToRemoveRequiredList						MCMRBRemoveConfiguration						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModifyConfirm

MCBearerContextToModifyConfirm ::= SEQUENCE {

mbsMulticastF1UContextDescriptor					MBSMulticastF1UContextDescriptor				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyConfirm-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyConfirm-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRDC-Data-Usage-Report-Item ::= SEQUENCE {

startTimeStamp				OCTET STRING (SIZE(4)),

endTimeStamp				OCTET STRING (SIZE(4)),

usageCountUL				INTEGER (0..18446744073709551615),

usageCountDL				INTEGER (0..18446744073709551615),

iE-Extensions				ProtocolExtensionContainer { { MRDC-Data-Usage-Report-Item-ExtIEs} } OPTIONAL,

...

}

MRDC-Data-Usage-Report-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRDC-Usage-Information ::= SEQUENCE {

data-Usage-per-PDU-Session-Report		Data-Usage-per-PDU-Session-Report			OPTIONAL,

data-Usage-per-QoS-Flow-List			Data-Usage-per-QoS-Flow-List				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MRDC-Usage-Information-ExtIEs} } OPTIONAL,

...

}

MRDC-Usage-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

M4Configuration ::= SEQUENCE {

m4period			M4period,

m4-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M4Configuration-ExtIEs} } OPTIONAL,

...

}

M4Configuration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

M4period ::= ENUMERATED {ms1024, ms2048, ms5120, ms10240, min1, ... }

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480 ,ms40960, min1, min6, min12, min30, ... }

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

M7period ::= INTEGER(1..60, ...)

MDT-Activation 	::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

...

}

MDT-Configuration ::= SEQUENCE {

mdt-Activation				MDT-Activation,

mDTMode			MDTMode,

iE-Extensions				ProtocolExtensionContainer { { MDT-Configuration-ExtIEs} } OPTIONAL,

...

}

MDT-Configuration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MDTMode ::= CHOICE {

immediateMDT 				ImmediateMDT,

choice-extension	ProtocolIE-SingleContainer	{{MDTMode-ExtIEs}}

}

MDTMode-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MeasurementsToActivate ::= BIT STRING (SIZE (8))

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

#### -- N

NetworkInstance ::= INTEGER (1..256, ...)

New-UL-TNL-Information-Required::= 	ENUMERATED {

required,

...

}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

NG-RAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNGRANQOSParameters)) OF NG-RAN-QoS-Support-Item

NG-RAN-QoS-Support-Item ::= SEQUENCE {

non-Dynamic5QIDescriptor	Non-Dynamic5QIDescriptor,

iE-Extensions				ProtocolExtensionContainer { { NG-RAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

NG-RAN-QoS-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NID	::= BIT STRING (SIZE (44))

Non-Dynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			QoSPriorityLevel				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Non-Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Non-Dynamic5QIDescriptor-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION 	ExtendedPacketDelayBudget		PRESENCE optional	},

...

}

NPNSupportInfo ::= CHOICE {

sNPN				NPNSupportInfo-SNPN,

choice-extension	ProtocolIE-SingleContainer	{{NPNSupportInfo-ExtIEs}}

}

NPNSupportInfo-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

NPNSupportInfo-SNPN ::= SEQUENCE {

nID						NID,

iE-Extensions			ProtocolExtensionContainer { { NPNSupportInfo-SNPN-ExtIEs } }	OPTIONAL

}

NPNSupportInfo-SNPN-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NPNContextInfo ::= CHOICE {

sNPN				NPNContextInfo-SNPN,

choice-extension	ProtocolIE-SingleContainer	{{NPNContextInfo-ExtIEs}}

}

NPNContextInfo-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

NPNContextInfo-SNPN ::= SEQUENCE {

nID						NID,

iE-Extensions			ProtocolExtensionContainer { {NPNContextInfo-SNPN-ExtIEs } }	OPTIONAL

}

NPNContextInfo-SNPN-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-Cell-Identity	::=		BIT STRING (SIZE(36))

NR-CGI ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

nR-Cell-Identity		NR-Cell-Identity,

iE-Extensions			ProtocolExtensionContainer { { NR-CGI-ExtIEs } }	OPTIONAL

}

NR-CGI-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-CGI-Support-List ::= SEQUENCE (SIZE(1.. maxnoofNRCGI)) OF NR-CGI-Support-Item

NR-CGI-Support-Item ::= SEQUENCE {

nR-CGI	NR-CGI,

iE-Extensions				ProtocolExtensionContainer { { NR-CGI-Support-Item-ExtIEs } }	OPTIONAL

}

NR-CGI-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

Extended-NR-CGI-Support-List ::= SEQUENCE (SIZE(1.. maxnoofExtNRCGI)) OF Extended-NR-CGI-Support-Item

Extended-NR-CGI-Support-Item ::= SEQUENCE {

nR-CGI	NR-CGI,

iE-Extensions				ProtocolExtensionContainer { { Extended-NR-CGI-Support-Item-ExtIEs } }	OPTIONAL

}

Extended-NR-CGI-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- O

OutOfOrderDelivery	::=	ENUMERATED	{

true,

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PDCP-Configuration	::=	SEQUENCE {

pDCP-SN-Size-UL							PDCP-SN-Size,

pDCP-SN-Size-DL							PDCP-SN-Size,

rLC-Mode								RLC-Mode,

rOHC-Parameters							ROHC-Parameters			OPTIONAL,

t-ReorderingTimer						T-ReorderingTimer		OPTIONAL,

discardTimer							DiscardTimer			OPTIONAL,

uLDataSplitThreshold					ULDataSplitThreshold	OPTIONAL,

pDCP-Duplication						PDCP-Duplication		OPTIONAL,

pDCP-Reestablishment					PDCP-Reestablishment	OPTIONAL,

pDCP-DataRecovery						PDCP-DataRecovery		OPTIONAL,

duplication-Activation				Duplication-Activation		OPTIONAL,

outOfOrderDelivery						OutOfOrderDelivery		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Configuration-ExtIEs } }	OPTIONAL,

...

}

PDCP-Configuration-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-PDCP-StatusReportIndication					CRITICALITY ignore	EXTENSION PDCP-StatusReportIndication					PRESENCE	optional}|

{ ID id-AdditionalPDCPduplicationInformation		CRITICALITY ignore	EXTENSION AdditionalPDCPduplicationInformation	PRESENCE optional	}|

{ ID id-EHC-Parameters								CRITICALITY ignore	EXTENSION EHC-Parameters									PRESENCE optional},

...

}

PDCP-Count	::=	SEQUENCE {

pDCP-SN				PDCP-SN,

hFN					HFN,

iE-Extensions							ProtocolExtensionContainer	{ { PDCP-Count-ExtIEs } }	OPTIONAL,

...

}

PDCP-Count-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN-Status-Request	::=		ENUMERATED {

requested,

...

}

PDCP-DataRecovery	::=	ENUMERATED	{

true,

...

}

PDCP-Duplication	::=	ENUMERATED	{

true,

...

}

PDCP-Reestablishment	::=	ENUMERATED	{

true,

...

}

PDU-Session-Resource-Data-Usage-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Data-Usage-Item

PDU-Session-Resource-Data-Usage-Item	::= SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

mRDC-Usage-Information						MRDC-Usage-Information,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Data-Usage-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Data-Usage-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCP-SN		::=		INTEGER	(0..262143)

PDCP-SN-Size	::=	ENUMERATED	{

s-12,

s-18,

...

}

PDCP-SN-Status-Information ::= SEQUENCE {

pdcpStatusTransfer-UL	DRBBStatusTransfer,

pdcpStatusTransfer-DL	PDCP-Count,

iE-Extension		ProtocolExtensionContainer { { PDCP-SN-Status-Information-ExtIEs} }	OPTIONAL,

...

}

PDCP-StatusReportIndication	::=	ENUMERATED	{

downlink,

uplink,

both,

...

}

PDCP-SN-Status-Information-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBBStatusTransfer ::= SEQUENCE {

receiveStatusofPDCPSDU	BIT STRING (SIZE(1..131072))										OPTIONAL,

countValue				PDCP-Count,

iE-Extension			ProtocolExtensionContainer { {DRBBStatusTransfer-ExtIEs} }	OPTIONAL,

...

}

DRBBStatusTransfer-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-ID	::=	INTEGER (0..255)

PDU-Session-Resource-Activity	::= ENUMERATED {

active,

not-active,

...

}

PDU-Session-Resource-Activity-List ::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Activity-Item

PDU-Session-Resource-Activity-Item	::= SEQUENCE {

pDU-Session-ID									PDU-Session-ID,

pDU-Session-Resource-Activity					PDU-Session-Resource-Activity,

iE-Extensions	ProtocolExtensionContainer 		{ { PDU-Session-Resource-Activity-ItemExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Activity-ItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Confirm-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Confirm-Modified-Item

PDU-Session-Resource-Confirm-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

dRB-Confirm-Modified-List-NG-RAN		DRB-Confirm-Modified-List-NG-RAN	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Confirm-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Confirm-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Item

PDU-Session-Resource-Failed-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-Mod-Item

PDU-Session-Resource-Failed-Mod-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Failed-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Failed-To-Modify-Item

PDU-Session-Resource-Failed-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Failed-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Resource-Modified-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Modified-Item

PDU-Session-Resource-Modified-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

securityResult							SecurityResult							OPTIONAL,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN					OPTIONAL,

dRB-Failed-List-NG-RAN				DRB-Failed-List-NG-RAN				OPTIONAL,

dRB-Modified-List-NG-RAN				DRB-Modified-List-NG-RAN				OPTIONAL,

dRB-Failed-To-Modify-List-NG-RAN		DRB-Failed-To-Modify-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Modified-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Modified-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-Required-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Required-To-Modify-Item

PDU-Session-Resource-Required-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

nG-DL-UP-TNL-Information				UP-TNL-Information						OPTIONAL,

dRB-Required-To-Modify-List-NG-RAN		DRB-Required-To-Modify-List-NG-RAN		OPTIONAL,

dRB-Required-To-Remove-List-NG-RAN				DRB-Required-To-Remove-List-NG-RAN				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Required-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Required-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Item

PDU-Session-Resource-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

securityResult							SecurityResult				OPTIONAL,

nG-DL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

nG-DL-UP-Unchanged						ENUMERATED {true, ...}		OPTIONAL,

dRB-Setup-List-NG-RAN					DRB-Setup-List-NG-RAN,

dRB-Failed-List-NG-RAN					DRB-Failed-List-NG-RAN		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	}|

{ ID id-RedundantPDUSessionInformation-used		CRITICALITY ignore	EXTENSION 	RedundantPDUSessionInformation	PRESENCE optional	},

...

}

PDU-Session-Resource-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-Setup-Mod-Item

PDU-Session-Resource-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID											PDU-Session-ID,

securityResult											SecurityResult								OPTIONAL,

nG-DL-UP-TNL-Information								UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Response		Data-Forwarding-Information		OPTIONAL,

dRB-Setup-Mod-List-NG-RAN								DRB-Setup-Mod-List-NG-RAN,

dRB-Failed-Mod-List-NG-RAN								DRB-Failed-Mod-List-NG-RAN					OPTIONAL,

iE-Extensions											ProtocolExtensionContainer	{ { PDU-Session-Resource-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-redundant-nG-DL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information	PRESENCE optional	},

...

}

PDU-Session-Resource-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Modify-Item

PDU-Session-Resource-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

securityIndication									SecurityIndication						OPTIONAL,

pDU-Session-Resource-DL-AMBR						BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information						OPTIONAL,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Data-Forwarding-Information	Data-Forwarding-Information	OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

networkInstance										NetworkInstance							OPTIONAL,

dRB-To-Setup-List-NG-RAN							DRB-To-Setup-List-NG-RAN				OPTIONAL,

dRB-To-Modify-List-NG-RAN							DRB-To-Modify-List-NG-RAN				OPTIONAL,

dRB-To-Remove-List-NG-RAN					DRB-To-Remove-List-NG-RAN		OPTIONAL,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SNSSAI		CRITICALITY reject	EXTENSION SNSSAI		PRESENCE optional}|

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

{ID id-redundant-nG-UL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information			PRESENCE optional	}|

{ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	EXTENSION 	CommonNetworkInstance		PRESENCE optional	}|

{ID id-DataForwardingtoE-UTRANInformationList				CRITICALITY ignore	EXTENSION 	DataForwardingtoE-UTRANInformationList		PRESENCE optional	},

...

}

PDU-Session-Resource-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Remove-Item

PDU-Session-Resource-To-Remove-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Remove-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Remove-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Cause		CRITICALITY ignore	EXTENSION Cause		PRESENCE optional},

...

}

PDU-Session-Resource-To-Setup-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Item

PDU-Session-Resource-To-Setup-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

pDU-Session-Type						PDU-Session-Type,

sNSSAI									SNSSAI,

securityIndication						SecurityIndication,

pDU-Session-Resource-DL-AMBR			BitRate						OPTIONAL,

nG-UL-UP-TNL-Information				UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer			Inactivity-Timer	OPTIONAL,

existing-Allocated-NG-DL-UP-TNL-Info	UP-TNL-Information		OPTIONAL,

networkInstance							NetworkInstance		OPTIONAL,

dRB-To-Setup-List-NG-RAN				DRB-To-Setup-List-NG-RAN,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

{ ID id-redundant-nG-UL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information					PRESENCE optional	}|

{ ID id-RedundantCommonNetworkInstance			CRITICALITY ignore	EXTENSION 	CommonNetworkInstance				PRESENCE optional	}|

{ ID id-RedundantPDUSessionInformation			CRITICALITY ignore	EXTENSION 	RedundantPDUSessionInformation	PRESENCE optional	},

...

}

PDU-Session-Resource-To-Setup-Mod-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Setup-Mod-Item

PDU-Session-Resource-To-Setup-Mod-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

pDU-Session-Type									PDU-Session-Type,

sNSSAI												SNSSAI,

securityIndication									SecurityIndication,

pDU-Session-Resource-AMBR							BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

dRB-To-Setup-Mod-List-NG-RAN						DRB-To-Setup-Mod-List-NG-RAN,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Setup-Mod-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-NetworkInstance		CRITICALITY ignore	EXTENSION NetworkInstance		PRESENCE optional}|

{ID id-CommonNetworkInstance	CRITICALITY ignore	EXTENSION CommonNetworkInstance	PRESENCE optional}|

{ID id-redundant-nG-UL-UP-TNL-Information		CRITICALITY ignore	EXTENSION 	UP-TNL-Information		PRESENCE optional	}|

{ID id-RedundantCommonNetworkInstance			CRITICALITY ignore	EXTENSION 	CommonNetworkInstance	PRESENCE optional	},

...

}

PDU-Session-To-Notify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-To-Notify-Item

PDU-Session-To-Notify-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

qoS-Flow-List							QoS-Flow-List,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-To-Notify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-To-Notify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

PDU-Session-Type ::= ENUMERATED {

ipv4,

ipv6,

ipv4v6,

ethernet,

unstructured,

...

}

PLMN-Identity ::= OCTET STRING (SIZE(3))

PortNumber ::= BIT STRING (SIZE(16))

PPI ::= INTEGER (0..7, ...)

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

PrivacyIndicator ::= ENUMERATED {

immediate-MDT,

logged-MDT,

...

}

#### -- Q

QCI ::= INTEGER (0..255)

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				Non-Dynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer	{{QoS-Characteristics-ExtIEs}}

}

QoS-Characteristics-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

QoS-Flow-Identifier	::=	INTEGER (0..63)

QoS-Flow-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Item

QoS-Flow-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoS-Flow-Mapping-Indication		PRESENCE optional},

...

}

QoS-Flow-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Failed-Item

QoS-Flow-Failed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Mapping-Item

QoS-Flow-Mapping-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowMappingIndication						QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Mapping-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Mapping-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-Indication ::= ENUMERATED {ul, dl, ...}

QoS-Flows-DRB-Remapping ::= ENUMERATED {update, source-configuration, ...}

QoS-Parameters-Support-List	::= SEQUENCE {

eUTRAN-QoS-Support-List			EUTRAN-QoS-Support-List			OPTIONAL,

nG-RAN-QoS-Support-List			NG-RAN-QoS-Support-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { QoS-Parameters-Support-List-ItemExtIEs} } OPTIONAL,

...

}

QoS-Parameters-Support-List-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSPriorityLevel ::= INTEGER (0..127, ...)

QoS-Flow-QoS-Parameter-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-QoS-Parameter-Item

QoS-Flow-QoS-Parameter-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-QoS-Parameter-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-QoS-Parameter-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-RedundantQosFlowIndicator		CRITICALITY ignore 	EXTENSION RedundantQoSFlowIndicator			PRESENCE optional}|

{ID id-TSCTrafficCharacteristics 		CRITICALITY ignore 	EXTENSION TSCTrafficCharacteristics 			PRESENCE optional},

...

}

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest				CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-MCG-OfferedGBRQoSFlowInfo		CRITICALITY ignore	EXTENSION GBR-QoSFlowInformation				PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency	CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency	PRESENCE optional}|

{ID id-QoSMonitoringDisabled			CRITICALITY ignore	EXTENSION QosMonitoringDisabled					PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QosMonitoringReportingFrequency ::= INTEGER (1..1800, ...)

QosMonitoringDisabled ::= ENUMERATED {true, ...}

QoS-Flow-Removed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoS-Flow-Released-In-Session			ENUMERATED {released-in-session, not-released-in-session, ...}			OPTIONAL,

qoS-Flow-Accumulated-Session-Time		OCTET STRING (SIZE(5))														OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flow-Removed-Item-ExtIEs } }		OPTIONAL,

...

}

QoS-Flow-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flows-to-be-forwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flows-to-be-forwarded-Item

QoS-Flows-to-be-forwarded-Item ::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flows-to-be-forwarded-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flows-to-be-forwarded-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Mapping-Information ::= SEQUENCE {

dscp							BIT STRING (SIZE(6))		OPTIONAL,

flow-label						BIT STRING (SIZE(20))	OPTIONAL,

...

}

DataForwardingtoNG-RANQoSFlowInformationList		::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF DataForwardingtoNG-RANQoSFlowInformationList-Item

DataForwardingtoNG-RANQoSFlowInformationList-Item	::=	SEQUENCE {

qoS-Flow-Identifier					QoS-Flow-Identifier,

iE-Extensions						ProtocolExtensionContainer	{ { DataForwardingtoNG-RANQoSFlowInformationList-Item-ExtIEs} }	OPTIONAL,

...

}

DataForwardingtoNG-RANQoSFlowInformationList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- R

RANUEID ::= OCTET STRING (SIZE (8))

RAT-Type	::=	ENUMERATED	{

e-UTRA,

nR,

...

}

RedundantQoSFlowIndicator::= ENUMERATED {true,false}

RedundantPDUSessionInformation ::= SEQUENCE {

rSN					RSN,

iE-Extensions		ProtocolExtensionContainer { {RedundantPDUSessionInformation-ExtIEs} }	OPTIONAL,

...

}

RedundantPDUSessionInformation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

RSN ::= ENUMERATED {v1, v2, ...}

RetainabilityMeasurementsInfo	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Removed-Item

RegistrationRequest	::=	ENUMERATED	{

start,

stop,

...

}

ReportCharacteristics	::=	BIT STRING (SIZE(36))

ReportingPeriodicity	::=	ENUMERATED	{

ms500, ms1000, ms2000, ms5000, ms10000, ms20000, ms30000, ms40000, ms50000, ms60000, ms70000, ms80000, ms90000, ms100000, ms110000, ms120000,

...

}

RLC-Mode	::=	ENUMERATED	{

rlc-tm,

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

ROHC-Parameters	::= CHOICE {

rOHC					ROHC,

uPlinkOnlyROHC			UplinkOnlyROHC,

choice-Extension		ProtocolIE-SingleContainer { { ROHC-Parameters-ExtIEs} }

}

ROHC-Parameters-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ROHC	::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { ROHC-ExtIEs } } 		OPTIONAL

}

ROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- S

SecurityAlgorithm	::= SEQUENCE {

cipheringAlgorithm				CipheringAlgorithm,

integrityProtectionAlgorithm	IntegrityProtectionAlgorithm	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SecurityAlgorithm-ExtIEs } }	OPTIONAL,

...

}

SecurityAlgorithm-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityIndication ::= SEQUENCE {

integrityProtectionIndication			IntegrityProtectionIndication,

confidentialityProtectionIndication		ConfidentialityProtectionIndication,

maximumIPdatarate							MaximumIPdatarate							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SecurityIndication-ExtIEs} }	OPTIONAL,

...

}

SecurityIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityInformation ::= SEQUENCE {

securityAlgorithm			SecurityAlgorithm,

uPSecuritykey				UPSecuritykey,

iE-Extensions				ProtocolExtensionContainer { { SecurityInformation-ExtIEs } }	OPTIONAL,

...

}

SecurityInformation-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityResult ::= SEQUENCE {

integrityProtectionResult			IntegrityProtectionResult,

confidentialityProtectionResult		ConfidentialityProtectionResult,

iE-Extensions						ProtocolExtensionContainer { {SecurityResult-ExtIEs} }	OPTIONAL,

...

}

SecurityResult-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Slice-Support-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF Slice-Support-Item

Slice-Support-Item ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { Slice-Support-Item-ExtIEs } }	OPTIONAL

}

Slice-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST			OCTET STRING (SIZE(1)),

sD			OCTET STRING (SIZE(3)) 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-ExtIEs } }	OPTIONAL,

...

}

SNSSAI-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SDAP-Configuration ::= SEQUENCE {

defaultDRB				DefaultDRB,

sDAP-Header-UL			SDAP-Header-UL,

sDAP-Header-DL			SDAP-Header-DL,

iE-Extensions			ProtocolExtensionContainer { { SDAP-Configuration-ExtIEs } }	OPTIONAL,

...

}

SDAP-Configuration-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SDAP-Header-DL	::=	ENUMERATED	{

present,

absent,

...

}

SDAP-Header-UL	::=	ENUMERATED	{

present,

absent,

...

}

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

#### -- T

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TNL-AvailableCapacityIndicator ::= SEQUENCE {

dL-TNL-OfferedCapacity				INTEGER (0..16777216, ...),

dL-TNL-AvailableCapacity			INTEGER (0..100, ...),

uL-TNL-OfferedCapacity				INTEGER (0..16777216, ...),

uL-TNL-AvailableCapacity			INTEGER (0..100, ...),

iE-Extensions		ProtocolExtensionContainer { { TNL-AvailableCapacityIndicator-ExtIEs } },

...

}

TNL-AvailableCapacityIndicator-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficCharacteristics		::= SEQUENCE {

tSCTrafficCharacteristicsUL				TSCTrafficInformation			OPTIONAL,

tSCTrafficCharacteristicsDL				TSCTrafficInformation			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TSCTrafficCharacteristics-ExtIEs } }	OPTIONAL

}

TSCTrafficCharacteristics-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TSCTrafficInformation			::= SEQUENCE {

periodicity						Periodicity,

burstArrivalTime						BurstArrivalTime				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { TSCTrafficInformation-ExtIEs } }	OPTIONAL

}

TSCTrafficInformation-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

Periodicity					::= INTEGER	(1..640000,	...)

BurstArrivalTime			::= OCTET STRING

TraceActivation ::= SEQUENCE {

traceID								TraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL,

...

}

TraceActivation-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MDTConfiguration	CRITICALITY ignore	EXTENSION MDT-Configuration		PRESENCE 	optional }|

{ ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URIaddress	PRESENCE    optional},

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TraceID ::= OCTET STRING (SIZE(8))

TransportLayerAddress		::= 	BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

T-Reordering	::=	ENUMERATED	{ms0, ms1, ms2, ms4, ms5, ms8, ms10, ms15, ms20, ms30, ms40, ms50, ms60, ms80, ms100, ms120, ms140, ms160, ms180, ms200, ms220, ms240, ms260, ms280, ms300, ms500, ms750, ms1000, ms1250, ms1500, ms1750, ms2000, ms2250, ms2500, ms2750, ms3000, ...}

T-ReorderingTimer ::= SEQUENCE {

t-Reordering				T-Reordering,

iE-Extensions				ProtocolExtensionContainer { { T-ReorderingTimer-ExtIEs } }	OPTIONAL,

...

}

T-ReorderingTimer-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Addresses-Info-To-Add-List	Transport-UP-Layer-Addresses-Info-To-Add-List	OPTIONAL,

transport-UP-Layer-Addresses-Info-To-Remove-List	Transport-UP-Layer-Addresses-Info-To-Remove-List	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs} }	OPTIONAL,

...

}

Transport-Layer-Address-Info-ExtIEs	E1AP-PROTOCOL-EXTENSION	::=	{

...

}

Transport-UP-Layer-Addresses-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Add-Item

Transport-UP-Layer-Addresses-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Add-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Addresses-Info-To-Remove-Item

Transport-UP-Layer-Addresses-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressesToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs } }    OPTIONAL,

...

}

Transport-UP-Layer-Addresses-Info-To-Remove-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- U

UE-Activity	::=	ENUMERATED	{

active,

not-active,

...

}

UE-associatedLogicalE1-ConnectionItem ::= SEQUENCE {

gNB-CU-CP-UE-E1AP-ID		GNB-CU-CP-UE-E1AP-ID	 OPTIONAL,

gNB-CU-UP-UE-E1AP-ID		GNB-CU-UP-UE-E1AP-ID	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-associatedLogicalE1-ConnectionItemExtIEs} }	OPTIONAL,

...

}

UE-associatedLogicalE1-ConnectionItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-Configuration	::=	ENUMERATED	{

no-data,

shared,

only,

...

}

ULUPTNLAddressToUpdateItem	::= SEQUENCE {

oldTNLAdress						TransportLayerAddress,

newTNLAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { ULUPTNLAddressToUpdateItemExtIEs } }	OPTIONAL,

...

}

ULUPTNLAddressToUpdateItemExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

ULDataSplitThreshold	::=	ENUMERATED {b0, b100, b200, b400, b800, b1600, b3200, b6400, b12800, b25600, b51200, b102400, b204800, b409600, b819200, b1228800, b1638400, b2457600, b3276800, b4096000, b4915200, b5734400, b6553600, infinity, ...}

UP-Parameters ::= SEQUENCE (SIZE(1.. maxnoofUPParameters)) OF UP-Parameters-Item

UP-Parameters-Item ::= SEQUENCE {

uP-TNL-Information			UP-TNL-Information,

cell-Group-ID				Cell-Group-ID,

iE-Extensions				ProtocolExtensionContainer { { UP-Parameters-Item-ExtIEs } }	OPTIONAL,

...

}

UP-Parameters-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoS-Mapping-Information	CRITICALITY reject	EXTENSION QoS-Mapping-Information	PRESENCE optional},

...

}

UPSecuritykey	::= SEQUENCE {

encryptionKey				EncryptionKey,

integrityProtectionKey		IntegrityProtectionKey		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UPSecuritykey-ExtIEs } }	OPTIONAL,

...

}

UPSecuritykey-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

UP-TNL-Information		::= 	CHOICE {

gTPTunnel		GTPTunnel,

choice-extension		ProtocolIE-SingleContainer	{{UP-TNL-Information-ExtIEs}}

}

UP-TNL-Information-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

UplinkOnlyROHC ::= SEQUENCE {

maxCID							INTEGER (0..16383, ...),

rOHC-Profiles					INTEGER (0..511, ...),

continueROHC					ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UplinkOnlyROHC-ExtIEs } } 		OPTIONAL

}

UplinkOnlyROHC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

URIaddress ::= VisibleString

#### -- V

#### -- W

#### -- X

#### -- Y

#### -- Z

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

#### -- Common definitions

--

-- **************************************************************

E1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

#### -- Common Data Types

--

-- **************************************************************

Criticality		::= 	ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global			OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..maxProtocolExtensions)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

id-resourceStatusReportingInitiation							ProcedureCode ::= 22

id-resourceStatusReporting										ProcedureCode ::= 23

id-iAB-UPTNLAddressUpdate										ProcedureCode ::= 24

id-CellTrafficTrace												ProcedureCode ::= 25

id-earlyForwardingSNTransfer									ProcedureCode ::= 26

id-gNB-CU-CPMeasurementResultsInformation						ProcedureCode ::= 27

id-BCBearerContextSetup											ProcedureCode ::= 90 -- to be assigned

id-BCBearerContextModification									ProcedureCode ::= 91 -- to be assigned

id-BCBearerContextModificationRequired							ProcedureCode ::= 92 -- to be assigned

id-BCBearerContextRelease										ProcedureCode ::= 93 -- to be assigned

id-BCBearerContextReleaseRequest								ProcedureCode ::= 94 -- to be assigned

id-MCBearerContextSetup											ProcedureCode ::= 95 -- to be assigned

id-MCBearerContextModification									ProcedureCode ::= 96 -- to be assigned

id-MCBearerContextModificationRequired							ProcedureCode ::= 97 -- to be assigned

id-MCBearerContextRelease										ProcedureCode ::= 98 -- to be assigned

id-MCBearerContextReleaseRequest								ProcedureCode ::= 99 -- to be assigned

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAddresses							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofExtNRCGI 							INTEGER	::= 16384

maxnoofMBSAreaSessionIDs					INTEGER ::= 256

maxnoofSharedNG-UTerminations				INTEGER ::= 8

maxnoofMRBs								INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info									ProtocolIE-ID ::= 86

id-QoSMonitoringRequest											ProtocolIE-ID ::= 87

id-PDCP-StatusReportIndication									ProtocolIE-ID ::= 88

id-gNB-CU-CP-Measurement-ID										ProtocolIE-ID ::= 89

id-gNB-CU-UP-Measurement-ID										ProtocolIE-ID ::= 90

id-RegistrationRequest											ProtocolIE-ID ::= 91

id-ReportCharacteristics										ProtocolIE-ID ::= 92

id-ReportingPeriodicity											ProtocolIE-ID ::= 93

id-TNL-AvailableCapacityIndicator								ProtocolIE-ID ::= 94

id-HW-CapacityIndicator											ProtocolIE-ID ::= 95

id-RedundantCommonNetworkInstance								ProtocolIE-ID ::= 96

id-redundant-nG-UL-UP-TNL-Information							ProtocolIE-ID ::= 97

id-redundant-nG-DL-UP-TNL-Information							ProtocolIE-ID ::= 98

id-RedundantQosFlowIndicator									ProtocolIE-ID ::= 99

id-TSCTrafficCharacteristics									ProtocolIE-ID ::= 100

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= 101

id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= 102

id-ExtendedPacketDelayBudget									ProtocolIE-ID ::= 103

id-AdditionalPDCPduplicationInformation							ProtocolIE-ID ::= 104

id-RedundantPDUSessionInformation								ProtocolIE-ID ::= 105

id-RedundantPDUSessionInformation-used							ProtocolIE-ID ::= 106

id-QoS-Mapping-Information										ProtocolIE-ID ::= 107

id-DLUPTNLAddressToUpdateList									ProtocolIE-ID ::= 108

id-ULUPTNLAddressToUpdateList									ProtocolIE-ID ::= 109

id-NPNSupportInfo												ProtocolIE-ID ::= 110

id-NPNContextInfo												ProtocolIE-ID ::= 111

id-MDTConfiguration												ProtocolIE-ID ::= 112

id-ManagementBasedMDTPLMNList									ProtocolIE-ID ::= 113

id-TraceCollectionEntityIPAddress								ProtocolIE-ID ::= 114

id-PrivacyIndicator												ProtocolIE-ID ::= 115

id-TraceCollectionEntityURI										ProtocolIE-ID ::= 116

id-URIaddress													ProtocolIE-ID ::= 117

id-EHC-Parameters												ProtocolIE-ID ::= 118

id-DRBs-Subject-To-Early-Forwarding-List						ProtocolIE-ID ::= 119

id-DAPSRequestInfo												ProtocolIE-ID ::= 120

id-CHOInitiation												ProtocolIE-ID ::= 121

id-EarlyForwardingCOUNTReq										ProtocolIE-ID ::= 122

id-EarlyForwardingCOUNTInfo										ProtocolIE-ID ::= 123

id-AlternativeQoSParaSetList									ProtocolIE-ID ::= 124

id-ExtendedSliceSupportList									ProtocolIE-ID ::= 125

id-MCG-OfferedGBRQoSFlowInfo									ProtocolIE-ID ::= 126

id-Number-of-tunnels											ProtocolIE-ID ::= 127

id-DRB-Measurement-Results-Information-List						ProtocolIE-ID ::= 128

id-Extended-GNB-CU-CP-Name										ProtocolIE-ID ::= 129

id-Extended-GNB-CU-UP-Name										ProtocolIE-ID ::= 130

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-AdditionalHandoverInfo										ProtocolIE-ID ::= 134

id-Extended-NR-CGI-Support-List									ProtocolIE-ID ::= 135

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-MaxCIDEHCDL													ProtocolIE-ID ::= 137

id-ignoreMappingRuleIndication									ProtocolIE-ID ::= 138

id-DirectForwardingPathAvailability								ProtocolIE-ID ::= 139

id-EarlyDataForwardingIndicator									ProtocolIE-ID ::= 140

id-QoSFlowsDRBRemapping											ProtocolIE-ID ::= 141

id-GNB-CU-CP-MBS-E1AP-ID											ProtocolIE-ID ::= 500 -- to be allocated

id-GNB-CU-UP-MBS-E1AP-ID											ProtocolIE-ID ::= 501 -- to be allocated

id-GlobalMBSSessionID											ProtocolIE-ID ::= 502 -- to be allocated

id-BCBearerContextToSetup										ProtocolIE-ID ::= 503 -- to be allocated

id-BCBearerContextToSetupResponse									ProtocolIE-ID ::= 504 -- to be allocated

id-BCBearerContextToModify										ProtocolIE-ID ::= 505 -- to be allocated

id-BCBearerContextToModifyResponse								ProtocolIE-ID ::= 506 -- to be allocated

id-BCBearerContextToModifyRequired								ProtocolIE-ID ::= 507 -- to be allocated

id-BCBearerContextToModifyConfirm									ProtocolIE-ID ::= 508 -- to be allocated

id-MCBearerContextToSetup										ProtocolIE-ID ::= 509 -- to be allocated

id-MCBearerContextToSetupResponse									ProtocolIE-ID ::= 510 -- to be allocated

id-MCBearerContextToModify										ProtocolIE-ID ::= 511 -- to be allocated

id-MCBearerContextToModifyResponse								ProtocolIE-ID ::= 512 -- to be allocated

id-MCBearerContextToModifyRequired								ProtocolIE-ID ::= 513 -- to be allocated

id-MCBearerContextToModifyConfirm									ProtocolIE-ID ::= 514 -- to be allocated

id-MBSMulticastF1UContextDescriptor									ProtocolIE-ID ::= 515 -- to be allocated

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

#### -- Container definitions

--

-- **************************************************************

E1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Class Definition for Protocol IEs

--

-- **************************************************************

E1AP-PROTOCOL-IES ::= CLASS {

&id					ProtocolIE-ID			UNIQUE,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Protocol Extensions

--

-- **************************************************************

E1AP-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Class Definition for Private IEs

--

-- **************************************************************

E1AP-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

#### -- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { E1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer { E1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { E1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				E1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		E1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			E1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

#### -- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, E1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

-- **************************************************************

--

#### -- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { E1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { E1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					E1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			E1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		E1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

#### -- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { E1AP-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { E1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				E1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality		E1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value			E1AP-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

## 9.5	Message Transfer Syntax

E1AP shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax, as specified in ITU-T Recommendation X.691 [7].

## 9.6	Timers

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>