3GPP TSG-RAN WG3 Meeting #114bis-e	R3-222540

# Online, 17th – 26th January, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0620 | rev | 8 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Enhanced IIoT support over Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung, Huawei, ZTE,CATT, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2022-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New QoS related parameters are agreed in Rel 17 for the further enhanced NR-IIoT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the new QoS related parameters over XnAP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no support for the new Rel 17 QoS related parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.115, 9.2.x.y, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0598 TS 38.463 CR 0609 TS 38.473 CR 0751 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 3: Including the agreed TP in R3-213774 to correct Rev 4: rebase to 16.7.0 Rev 5: Including TP from R3-215954 and R3-215979 Rev 8: Correct the max value of the Survival Time IE has an extra ‘0’ in both tabular and asn.1 |  |  |  |  |  |  |  |  |

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

********************************

Skip to the next change

********************************

If the Estimated Arrival Probability IE is contained in the Conditional Handover Information Request IE included in the HANDOVER REQUEST message, then the target NG-RAN node may use the information to allocate necessary resources for the incoming CHO.

If the IAB Node Indication IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the handover is for an IAB node.

If the UE Radio Capability ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the Time Synchronisation Assistance Information IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

********************************

Skip to the next change

********************************

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

********************************

Skip to the next change

********************************

In case of RRC Re-establishment, the old NG-RAN may include the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message. Upon reception of the UE History Information IE or the UE History Information from the UE IE in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store the collected information and use it for future handover preparations.

If the UE Radio Capability ID IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG- RAN node shall, if supported store this information in the UE context and use it as defined in TS 23.501 [7] and TS 23.502 [13].

If the Time Synchronisation Assistance Information IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall, if supported, store this information in the UE context and use it as defined in TS 23.501 [7].

********************************

Skip to the next change

********************************

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| Time Synchronisation Assistance Information | O |  | 9.2.3.aaa |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

********************************

Skip to the next change

********************************

#### 9.1.1.9	RETRIEVE UE CONTEXT RESPONSE

This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node.

Direction: old NG-RAN node  new NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information – Retrieve UE Context Response | M |  | 9.2.1.13 |  | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | Ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.aaa |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | PLMNs in the Management Based MDT PLMN list. Value is 16. |

********************************

Skip to the next change

********************************

#### 9.2.3.115	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.2.3.116 | Periodicity as specified in TS 23.501 [7]. | _ |  |
| Burst Arrival Time | O |  | 9.2.3.117 | Burst Arrival Time as specified in TS 23.501 [7]. | _ |  |
| Survival Time | O |  | 9.2.3.xxx |  | YES | ignore |

********************************

Skip to the next change

********************************

#### 9.2.3.xxx	Survival Time

This IE provides the Survival Time for a TSC QoS flow (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Expressed in units of 1 us. |

#### 9.2.3.aaa	Time Synchronisation Assistance Information

This IE indicates the 5G access stratum time distribution parameters as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time Distribution indication | M |  | ENUMERATED (enabled, disabled, …) |  |
| Uu Time Synchronization Error Budget | C-ifEnabled |  | INTEGER (0..1000000, …) | Expressed in units of 1 ns. |

| Condition | Explanation |
| --- | --- |
| C-ifEnabled | This IE shall be present if the Time Distribution Indication IE is set to “enabled”. |

********************************

Skip to the next change

********************************

### 9.3.4	PDU Definitions

********************************

Skip to the next change

********************************

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

TimeSynchronizationAssistanceInformation

********************************

Skip to the next change

********************************

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-TimeSynchronizationAssistanceInformation,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows

FROM XnAP-Constants;

-- **************************************************************

--

#### -- HANDOVER REQUEST

--

-- **************************************************************

HandoverRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{HandoverRequest-IEs}},

...

}

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters					PRESENCE optional }|

{ ID id-MobilityInformation				CRITICALITY ignore	TYPE MobilityInformation								PRESENCE optional}|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional}|

{ ID id-TimeSynchronizationAssistanceInformation		CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional },

...

}

********************************

Skip to the next change

********************************

-- **************************************************************

--

#### -- RETRIEVE UE CONTEXT RESPONSE

--

-- **************************************************************

RetrieveUEContextResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ RetrieveUEContextResponse-IEs}},

...

}

RetrieveUEContextResponse-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-newNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-oldNG-RANnodeUEXnAPID					CRITICALITY ignore		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-GUAMI									CRITICALITY reject		TYPE GUAMI											PRESENCE mandatory}|

{ ID id-UEContextInfoRetrUECtxtResp				CRITICALITY reject		TYPE UEContextInfoRetrUECtxtResp				PRESENCE mandatory}|

{ ID id-TraceActivation							CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-MaskedIMEISV							CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-LocationReportingInformation			CRITICALITY ignore		TYPE LocationReportingInformation				PRESENCE optional }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore		TYPE CriticalityDiagnostics						PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized			CRITICALITY ignore	TYPE NRV2XServicesAuthorized		PRESENCE optional}|

{ ID id-LTEV2XServicesAuthorized		CRITICALITY ignore	TYPE LTEV2XServicesAuthorized		PRESENCE optional}|

{ ID id-PC5QoSParameters				CRITICALITY ignore	TYPE PC5QoSParameters			PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE optional}|

{ ID id-UEHistoryInformationFromTheUE	CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-MDTPLMNList								CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional },

...

}

********************************

Skip to the next change

********************************

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

********************************

Skip to the next change

********************************

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-SurvivalTime,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-Cause,

********************************

Skip to the next change

********************************

#### -- S

SecondarydataForwardingInfoFromTarget-Item::= SEQUENCE {

secondarydataForwardingInfoFromTarget				DataForwardingInfoFromTargetNGRANnode,

iE-Extensions		ProtocolExtensionContainer { { SecondarydataForwardingInfoFromTarget-Item-ExtIEs} }	OPTIONAL,

...

}

SecondarydataForwardingInfoFromTarget-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

********************************

Skip to the next change

********************************

SUL-Information ::= SEQUENCE {

sulFrequencyInfo			NRARFCN,

sulTransmissionBandwidth	NRTransmissionBandwidth,

iE-Extensions				ProtocolExtensionContainer { {SUL-Information-ExtIEs} } OPTIONAL,

...

}

SUL-Information-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

SupportedSULBandList ::= SEQUENCE (SIZE(1..maxnoofNRCellBands)) OF SupportedSULBandItem

SupportedSULBandItem ::= SEQUENCE {

sulBandItem					SUL-FrequencyBand,

iE-Extensions				ProtocolExtensionContainer { {SupportedSULBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SymbolAllocation-in-Slot ::= CHOICE {

allDL				SymbolAllocation-in-Slot-AllDL,

allUL				SymbolAllocation-in-Slot-AllUL,

bothDLandUL			SymbolAllocation-in-Slot-BothDLandUL,

choice-extension	ProtocolIE-Single-Container { {SymbolAllocation-in-Slot-ExtIEs} }

}

SymbolAllocation-in-Slot-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

SymbolAllocation-in-Slot-AllDL ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllDL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-AllDL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot-AllUL ::= SEQUENCE {

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-AllUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-AllUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

SymbolAllocation-in-Slot-BothDLandUL ::= SEQUENCE {

numberofDLSymbols	INTEGER (0..13),

numberofULSymbols	INTEGER (0..13),

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-BothDLandUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-BothDLandUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

********************************

Skip to the next change

********************************

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

TABasedMDT-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TAIBasedMDT ::= SEQUENCE {

tAIListforMDT			TAIListforMDT,

iE-Extensions			ProtocolExtensionContainer { {TAIBasedMDT-ExtIEs} } OPTIONAL,

...

}

********************************

Skip to the next change

********************************

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDownlink	TSCAssistanceInformation OPTIONAL,

tSCAssistanceInformationUplink		TSCAssistanceInformation OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} } OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity			INTEGER (0.. 640000, ...),

burstArrivalTime	OCTET STRING				OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { { TSCAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::=

{	ID id-SurvivalTime			CRITICALITY ignore	EXTENSION SurvivalTime			PRESENCE optional},

...

}

********************************

Skip to the next change

********************************

Threshold-RSRQ ::= INTEGER(0..34)

Threshold-RSRP ::= INTEGER(0..97)

Threshold-SINR ::= INTEGER(0..127)

TimeSynchronizationAssistanceInformation ::= SEQUENCE {

timeDistributionIndication					ENUMERATED {enabled, disabled, ...},

uuTimeSynchronizationErrorBudget			INTEGER (0..1000000, ...)				OPTIONAL,

-- The above IE shall be present if the Time Distribution Indication IE is set to the value “enabled”

ie-Extension								ProtocolExtensionContainer { { TimeSynchronizationAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TimeSynchronizationAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

********************************

Skip to the next change

********************************

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

********************************

Skip to the next change

********************************

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																			ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-SurvivalTime																						ProtocolIE-ID ::= 2xx

id-TimeSynchronizationAssistanceInformation															ProtocolIE-ID ::= 2xx+1

END

-- ASN1STOP